# 3GPP TSG CT WG3 Meeting #143	C3-254397

# Sophia-Antipolis, France, 13 – 17 October, 2025					 (Revision of C3-254250)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.519 | CR | 0625 | rev | 1 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections for handling when FinerGranUEs feature is supported |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Nokia, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19, EDGE_Ph2 |  |  |  |  |  | Date: |  |  | 2025-9-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | When the FinerGranUEs feature is supported, the UDR stores an influence data resource associated with at least 2 internal group IDs within "interGroupIdList" attribute. When the NEF would like to retrieve the influence data through the GET request towards the UDR, it supports retrieving one and more than one group Ids as: internal-group-ids-Add (NOTE 2); array(GroupId); O; 1..N; Each element identifies an internal group id. The UDR shall return only Individual Influence Data resource(s) that match all the provided internal group IDs within the "interGroupIdList" attribute. (NOTE 1). The current description clarifies that even when one GroupId is provided within the internal-group-ids-Add, the UDR still returns the resources that match all the provided internal group IDs within the "interGroupIdList" attribute. Since the "interGroupIdList" attribute at least contains 2 elements, the retrieval for one element with internal-group-ids-Add can never success even in fact the single group ID would be within "interGroupId" attribute. A clarification on how the UDR shall behave is needed while one element is provided within internal-group-ids-Add. NOTE 8 in table 6.4.2.2 should be cited for interGroupIdList. NOTE 2 in table 6.4.2.4 is not applicable for the attribute internalGroupIdsAdd. |
| Summary of change: |  | Clarify the UDR behaviour when searching for only 1 GroupId using internal-group-ids-Add. Correct the wrong NOTE citation for NOTE 8 in Table 6.4.2.2. Correct the wrong NOTE citation for NOTE 2 in Table 6.4.2.4. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The sage of the attributes is not precise and leads to implementation errors. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.5.3.1, 6.4.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR has no impact on OpenAPI |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# *** 1st Change ***

##### 6.2.5.3.1	GET

This method shall support the URI query parameters specified in table 6.2.5.3.1-1.

Table 6.2.5.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| influence-Ids (NOTE 2) | array(string) | O | 1..N | Each element identifies a influenceId, i.e. an Individual Influence Data resource. (NOTE 1) |
| dnns | array(Dnn) | O | 1..N | Each element identifies a DNN. The UDR shall return only Individual Influence Data resource(s) that match one of the provided DNNs. (NOTE 1) |
| snssais | array(Snssai) | O | 1..N | Each element identifies a slice. The UDR shall return only Individual Influence Data resource(s) that match one of the provided S-NSSAIs. (NOTE 1) |
| internal-Group-Ids (NOTE 2) | array(GroupId) | O | 1..N | Each element identifies a group of users. The UDR shall return only Individual Influence Data resource(s) that match one of the provided internal group IDs within the "interGroupId" attribute. (NOTE 1) |
| internal-group-ids-Add (NOTE 2) | array(GroupId) | O | 1..N | Each element identifies an internal group id. The UDR shall return only Individual Influence Data resource(s) that match all the provided internal group IDs within the "interGroupIdList" attribute. (NOTE 1) |
| subscriber-categories | array(string) | O | 1..N | Each element identifies a subscriber category. The UDR shall return only Individual Influence Data resource(s) that match all the provided subscriber category(ies). |
| supis | array(Supi) | O | 1..N | Each element identifies a user. The UDR shall return only Individual Influence Data resource(s) that match one of the provided SUPIs. (NOTE 1) |
| supp-feat | SupportedFeatures | O | 0..1 | Identifies the features supported by the NF service consumer. |
| NOTE 1: At least one of the "influence-Ids", "dnns", "snssais", "internal-Group-Ids", "internal-group-ids-Add" (may be included when the FinerGranUEs feature is supported) or "supis" query parameters shall be provided. NOTE 2: The query parameter does not follow the related naming convention (i.e. "lower-with-hyphen") defined in clause 5.1.3.3 of 3GPP TS 29.501 [7]. This query parameter is however kept as currently defined in this specification for backward compatibility considerations. |  |  |  |  |

NOTE 1:	The combination of a."internal-Goup-Ids" and the "supis", b."internal-group-ids-Add" and the "supis", c."subscriber-categories" and the "supis" and d. "internal-Group-Ids" and the "internal-group-ids-Add" query parameter represent mutually exclusive resource properties, i.e., if both query parameters are simultaneously present in the same request, the search will not match any resource.

NOTE 2:	When the "FinerGranUEs" feature is supported and the "internal-group-ids-Add" query parameter is provided, it is not appropriate for it to contain a single array element as the corresponding "interGroupIdList" attribute within the targeted resource representation is defined to have at least two array elements (see clause 6.4.2.2).

When the request query contains more than one optional query parameters defined as an array, the UDR shall return the Individual Influence Data resources for each matching combination of the values of the elements of the array of the provided query parameters.

EXAMPLE:	If "snssais" query parameter is included with two S-NSSAI entries (SNSSAI_1 and SNSSAI_2) and "dnns" is included with two DNN entries (e.g. DNN_A and DNN_B), and the other optional query parameters are not included, the UDR shall return the Individual Influence Data resources for each matching S-NSSAI and DNN combination (for all the possible values for the SUPI and the Group Id), i.e., the resource(s) matching SNSSAI_1 and DNN_A, the resource(s) matching SNSSAI_1 and DNN_B, the resource(s) matching SNSSAI_2 and DNN_A, and the resource(s) matching SNSSAI_2 and DNN_B.

To query for the Individual Influence Data resource(s) that apply to any UE, an entry of the "internal-Group-Ids" shall be set to "AnyUE".

This method shall support the request data structures specified in table 6.2.5.3.1-2 and the response data structures and response codes specified in table 6.2.5.3.1-3.

Table 6.2.5.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.2.5.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| array(TrafficInfluData) | M | 0..N | 200 OK | The Traffic Influence Data stored in the UDR are returned. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

# *** Next Change ***

#### 6.4.2.2	Type TrafficInfluData

Table 6.4.2.2-1: Definition of type TrafficInfluData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| upPathChgNotifCorreId | string | C | 0..1 | Contains the Notification Correlation Id allocated by the NEF for the UP path change notification. It shall be included when the NEF requests the UP path change notification. |  |
| appReloInd | boolean | O | 0..1 | Identifies whether an application can be relocated once a location of the application has been selected. True: the application cannot be relocated; False (default): the application can be relocated. |  |
| afAppId | string | C | 0..1 | Identifies an application. (NOTE 1) (NOTE 3) |  |
| dnn | Dnn | O | 0..1 | Identifies a DNN |  |
| ethTrafficFilters | array(EthFlowDescription) | C | 1..N | Identifies Ethernet packet filters. (NOTE 1) (NOTE 3) |  |
| snssai | Snssai | O | 0..1 | The identification of slice. |  |
| interGroupId | GroupId | C | 0..1 | Identifies a group of users. (NOTE 2) (NOTE 3)(NOTE 5) |  |
| interGroupIdList | array(GroupId) | C | 2..N | Identifies the list of Internal Groups. (NOTE 2) (NOTE 8) (NOTE 9) (NOTE 10) | FinerGranUEs |
| subscriberCatList | array(string) | O | 1..N | Identifies the list of subscriber category(ies). (NOTE 10) | FinerGranUEs |
| plmnId | PlmnId | O | 0..1 | Identifies the PLMN of the UE. | HR-SBO |
| ipv4Addr | Ipv4Addr | O | 0..1 | Identifies the UE IPv4 address. (NOTE 2) | HR-SBO |
| ipv6Addr | Ipv6Addr | O | 0..1 | Identifies the UE IPv6 address. (NOTE 2) | HR-SBO |
| supi | Supi | C | 0..1 | Identifies a user. (NOTE 2) (NOTE 3) |  |
| trafficDataSets | map(TrafficDataSet) | C | 2..N | Contains multiple sets of traffic filters with the corresponding N6 traffic routing requirements. The key of the map shall be the value of the "setId" attribute of the TrafficDataSet data type. (NOTE 1, NOTE 3, NOTE 12, NOTE 13) | MultiTrafficInflu_Ext1 |
| trafficFilters | array(FlowInfo) | C | 1..N | Identifies IP packet filters. (NOTE 1) (NOTE 3) |  |
| trafficRoutes | array(RouteToLocation) | O | 1..N | Identifies the N6 traffic routing requirement. (NOTE 12) |  |
| sfcIdDl | string | O | 0..1 | Reference to a pre-configured service function chain for downlink traffic. (NOTE 11) | SFC |
| sfcIdUl | string | O | 0..1 | Reference to a pre-configured service function chain for uplink traffic. (NOTE 11) | SFC |
| metadata | Metadata | O | 0..1 | Contains opaque information for the service functions in the N6-LAN that is provided by AF and transparently sent to UPF. It may only be provided when sfcIdDl and/or sfcIdUl are provided. | SFC |
| traffCorreInd | boolean | C | 0..1 | Indication of traffic correlation. May only be included when "interGroupId" attribute is included and not set to "AnyUE". It is used to indicate that for the group of UEs, the targeted PDU sessions should be correlated by a common DNAI. Set to "true" if it should be correlated; otherwise set to "false". Default value is "false" if omitted. (NOTE 7, NOTE 13) |  |
| tfcCorreInfo | TrafficCorrelationInfo | O | 0..1 | Contains the information for traffic correlation. (NOTE 13) | CommonEASDNAI |
| candDnaiInd | boolean | O | 0..1 | Indication of whether the candidate DNAI(s) are requested to be reported or not. - "true": the candidate DNAI(s) for the PDU session are requested to be reported. - "false": the candidate DNAI(s) for the PDU session are not requested to be reported. The default value is "false" if omitted. | EnCommonDnai |
| validStartTime | DateTime | O | 0..1 | Identifies when the traffic routings start to be applicable. (NOTE 4) |  |
| validEndTime | DateTime | O | 0..1 | Identifies when the traffic routings are not applicable. (NOTE 4) |  |
| tempValidities | array(TemporalValidity) | O | 1..N | Indicates the time interval(s) during which the AF request is to be applied. (NOTE 4) | MultiTemporalCondition |
| nwAreaInfo | NetworkAreaInfo | O | 0..1 | Identifies a network area information that the request applies only to the traffic of UE(s) located in this specific zone. |  |
| upPathChgNotifUri | Uri | C | 0..1 | Contains the URI where the NEF receives the UP path change notification. It shall be included when the NEF requests the UP path change notification. |  |
| headers | array(string) | O | 1..N | Headers provisioned by the NEF to be used by other NFs to interact with the NEF. E.g. 3gpp-Sbi-Binding header with the binding indication for the URI where the NEF receives UP path change notification and whose information is used by the SMF. The encoding of the header shall comply with clause 6.3 of IETF RFC 9110 [21]. (NOTE 6) |  |
| subscribedEvents | array(SubscribedEvent) | O | 1..N | Identifies the requirement to be notified of the event(s). (NOTE 14) |  |
| notifUri | Uri | O | 0..1 | Contains the URI via which the NEF desires to receive the event(s) related notifications. | TraffRoutReqOutcome SimConnFailure |
| notifCorreId | string | O | 0..1 | Contains the Notification Correlation Id allocated by the NEF. | TraffRoutReqOutcome SimConnFailure |
| dnaiChgType | DnaiChangeType | O | 0..1 | Identifies a type of notification regarding UP path management event. |  |
| afAckInd | boolean | O | 0..1 | Identifies whether the AF acknowledgement of UP path event notification is expected. Set to "true" if the AF acknowledgement is expected; otherwise set to "false". Default value is "false" if omitted. | URLLC |
| addrPreserInd | boolean | O | 0..1 | Indicates whether UE IP address should be preserved. This attribute shall set to "true" if preserved, otherwise, set to "false". Default value is false if omitted. | URLLC |
| maxAllowedUpLat | Uinteger | O | 0..1 | Indicates the target user plane latency in units of milliseconds. The SMF may use this value to decide whether edge relocation is needed to ensure that the user plane latency does not exceed the value. | AF_latency |
| simConnInd | boolean | O | 0..1 | Indication of simultaneous connectivity temporarily maintained for the source and target PSA. It is used to indicate whether the simultaneous connectivity should be temporarily mantained for the source and target PSA. It is set to "true" if the temporary simultaneous connectivity should be temporarily maintained; otherwise, it is set to "false". Default value is "false" if omitted. (NOTE 14) | SimultConnectivity |
| simConnTerm | DurationSec | C | 0..1 | Indication of the minimum time interval to be considered for inactivity of the traffic routed via the source PSA during the edge re-location procedure before removing the source PSA. It may be included when the "simConnInd" attribute is set to true. | SimultConnectivity |
| n6DelayInd | boolean | O | 0..1 | Indicates whether the AF requests the N6 delay to be considered or not. - "true" indicates that the AF requests the N6 delay to be considered. - "false" indicates that the AF does not request the N6 delay to be considered. - If omitted, “false” is the default value. | N6DelayMeasurement |
| resUri | Uri | C | 0..1 | Represents the URI of Individual Influence Data. It shall only be included in the HTTP GET response. | EnhancedInfluDataNotification |
| resetIds | array(string) | O | 1..N | This IE uniquely identifies a part of temporary data in UDR that contains the created resource. This attribute may be provided in the response of successful resource creation. |  |
| afHdrReq | AfHeaderHandlingControlInfo | O | 0..1 | Contains the AF traffic influence request information for handling of payload headers. | HeaderHandling |
| nscSuppFeats | map(SupportedFeatures) | O | 1..N | A map of Network Function Service Consumer features supported per service. The key used in this map for each entry is the ServiceName value as defined in 3GPP TS 29.510[24] (e.g. for Nsmf_EventExposure API, the key shall be set to nsmf-event-exposure). | NscSupportedFeatures |
| supportedFeatures | SupportedFeatures | C | 0..1 | Indicates the list of negotiated supported features. This attribute shall be supplied by the UDR in the response to the PUT request when it was present in the PUT request and the UDR supports feature negotiation for Influence Data. This attribute shall be supplied by the UDR in the response to the GET request when the GET request includes the "supp-feat" query parameter and the UDR supports feature negotiation for Influence Data. |  |
| NOTE 1: Only one of "afAppId", "trafficFilters", "ethTrafficFilters" or "trafficDataSets" shall be included. NOTE 2: If "HR-SBO" feature is not supported, only one of"supi" or "interGroupId" or "interGroupIdList" shall be included. If "HR-SBO" feature is supported and the AF request to influence traffic routing is working in HR-SBO sessions in the VPLMN, only one of "supi", "interGroupId" set to "AnyUE", "ipv4Addr" or "ipv6Addr" shall be included. NOTE 3: If the EnhancedInfluDataNotification feature is not supported, to indicate the deletion of a Traffic Individual Influence Data resource, only the appropriate combination of "supi" or "interGroupId", and "afAppId", "trafficFilters", "ethTrafficFilters" or "trafficDataSets" that identify the resource shall be included. The rest of attributes shall be omitted. NOTE 4: Properties "validStartTime" and "validEndTime" shall only be included for single temporal validity condition. Property "tempValidities" shall only be included for multiple temporal validity conditions when the feature MultiTemporalCondition is supported. NOTE 5: If the Traffic Influence Data applies to any UE, then the "interGroupId" shall set to "AnyUE". NOTE 6: In this release of the specification, the property "headers" only includes the 3gpp-Sbi-Binding header with the binding indication for the URI included in the property "upPathChgNotifUri". NOTE 7: The indication of traffic correlation shall be provided only when the AF indicated that all the PDU sessions related to the 5G VN group member UEs should be correlated by a common DNAI in the user plane for the traffic as described in 3GPP TS 29.522 [19]. NOTE 8: When the FinerGranUEs feature is supported, the Traffic Influence Data applies to the UE(s) that belong to all the Internal Group Identifiers included within the attribute "interGroupIdList", if present. NOTE 9: When the FinerGranUEs feature is supported, the Traffic Influence Data applies to the UE(s) that belong to all the subscriber categories included within the attribute "subscriberCatList", which is included only if either "interGroupIdList" is included or "interGroupId" is included. NOTE 10: Attributes "interGroupId" and "interGroupIdList" are mutually exclusive attributes. NOTE 11: When the SFC feature is supported, for the purpose of influencing Service Function Chaining, at least one of the “sfcIdDl” and “sfcIdUl” attribute shall be present. NOTE 12: When the "MultiTrafficInflu_Ext1" feature is supported, the "trafficDataSets" and "trafficRoutes" attributes are mutually exclusive. Either one of them may be present. NOTE 13: When the "MultiTrafficInflu_Ext1" feature is supported, if "trafficDataSets" attribute is present, then the "traffCorreInd" and " tfcCorreInfo" attributes shall not be present. NOTE 14: The event "SIM_CONN_FAIL" may be contained in the "subscribedEvents" attribute only when the "SimConnFailure" feature is supported and the "simConnInd" attribute is provided and set to "true". The event "TRAFF_ROUTE_REQ_OUTCOME" may be contained in the "subscribedEvents" attribute only if the "TraffRoutReqOutcome" feature is supported. |  |  |  |  |  |

# *** Next Change ***

#### 6.4.2.4	Type TrafficInfluSub

Table 6.4.2.4-1: Definition of type TrafficInfluSub

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| dnns | array(Dnn) | O | 1..N | Each element identifies a DNN. (NOTE 1) |  |
| snssais | array(Snssai) | O | 1..N | Each element identifies an interna slice. (NOTE 1) |  |
| internalGroupIds | array(GroupId) | O | 1..N | Each element identifies a group of users. (NOTE 1) (NOTE 2) (NOTE 3) (NOTE 5) |  |
| internalGroupIdsAdd | array(GroupId) | O | 1..N | Each element identifies an internal group of users. (NOTE 1) (NOTE 4) (NOTE 5) | FinerGranUEs |
| subscriberCatList | array(string) | O | 1..N | Each element identifies a subscriber category. (NOTE 6) | FinerGranUEs |
| supis | array(Supi) | O | 1..N | Each element identifies the user. (NOTE 1) |  |
| plmnId | PlmnId | O | 0..1 | Identifies the PLMN of the UE. | HR-SBO |
| ipv4Addr | Ipv4Addr | O | 0..1 | Identifies the UE IPv4 address. | HR-SBO |
| ipv6Addr | Ipv6Addr | O | 0..1 | Identifies the UE IPv6 address. | HR-SBO |
| notificationUri | Uri | M | 1 | URI provided by the NF service consumer indicating where to receive the subscribed notifications from the UDR. |  |
| expiry | DateTime | C | 0..1 | This attribute indicates the expiry time of the subscription. In Subscribe request messages this attribute may be included to suggest a time after which the subscription becomes invalid. Absence of this attribute from subscribe request messages indicates that the requested subscription is suggested to be unlimited in time until explicitly terminated by the service consumer. In Subscribe response messages this attribute may be included to confirm a time after which the subscription becomes invalid. The confirmed expiry time should be less than or equal to the suggested expiry time. If the suggested expiry time is present in the request, the confirmed expiry time should not be absent from the response.. |  |
| supportedFeatures | SupportedFeatures | C | 0..1 | Used to negotiate the applicability of the optional features. This attribute shall be provided in the POST request and in the response of successful resource creation. |  |
| resetIds | array(string) | O | 1..N | This IE uniquely identifies a part of temporary data in UDR that contains the created resource. This attribute may be provided in the response of successful resource creation. |  |
| immRep | boolean | O | 0..1 | If provided and set to "true", it indicates that existing entries that match this subscription shall be immediately reported within the "immReports" attribute in the response. The default value is false. | ImmediateReportPcc |
| immReports | array(TrafficInfluDataNotif) | O | 1..N | Contains entries stored in the UDR that match this subscription. It may be included only in the POST (or PUT) response body of a subscription creation (or modification), and only if the request included the "immRep" attribute set to true. | ImmediateReportPcc |
| NOTE 1: If the TrafficInfluSubExt feature is not supported, either the "dnns", "snssais", "internalGroupIds", "internalGroupIdsAdd" or "supis" attribute shall be provided. If the TrafficInfluSubExt feature is supported, any of these attributes may be provided. NOTE 2: For the Traffic Influence Data applies to any UE, the "internalGroupIds" attribute shall have only one entry set to "AnyUE". NOTE 3: The "internalGroupIds" attribute shall be compared only with the "interGroupId" of the TrafficInfluData data type and there is a match only if they both exist and "interGroupId" is contained in "internalGroupIds". NOTE 4: When the FinerGranUEs feature is supported, the "internalGroupIdsAdd" attribute shall be compared only with the "interGroupIdList" of the TrafficInfluData data type and there is a match only if they both exist and are identical. NOTE 5: The attributes "internalGroupIds" and "internalGroupIdsAdd" are mutually exclusive attributes. NOTE 6: When the FinerGranUEs feature is supported, the "subscriberCatList" attribute of the TrafficInfluSub data type shall be compared with the "subscriberCatList" attribute of the TrafficInfluData data type and there is a match only if they both exist and are identical. |  |  |  |  |  |

# *** End of Changes ***