3GPP TSG-CT WG3 Meeting #143	C3-254414

# Sophia-Antipolis, France, 13 - 17 October 2025						(Revision of C3-254106)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.549 | CR | 0455 | rev | 1 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Digital Asset profile management update |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C3 |  |  |  |  |  |  |  |  |  |
| Work item code: | Metaverse_App |  |  |  |  |  | Date: |  |  | 14-10-2025 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Digital Asset profile management API has the following errors: - The wrong clause number reference - Resource name is mentioned as “profile” instead of “profiles” - Response code handling is missing for DELETE service operation description. - The wrong formatting, API name and sentence usage |  |  |  |  |  |  |  |  |
| Summary of change: |  | The above-mentioned Digital Asset profile management issues are fixed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The quality of the Digital Asset profile management API is not up to the mark and having the wrong resource names. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.14.1.2.2.1, 5.14.1.2.2.2, 5.14.1.2.3.1, 5.14.1.2.3.2, 5.14.1.2.4.1, 5.14.1.2.4.2, 5.14.1.2.5.1, 5.14.1.2.5.2, 7.13.1.3.2.3.1, 7.13.1.3.3.2, A.33 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces a backward compatible changes to the following APIs: TS29549_SS_DAProfileManagement.yaml |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First Change * * * *

5.14.1.2.2.1	General

This service operation is used by a service consumer to manage digital asset profile on a DA server.

The following procedures are supported by the "Create" service operation:

-	Create Digital Asset Profile.

# * * * * Next changes * * * *

5.14.1.2.2.2		Create Digital Asset Profile

To create the digital asset profile, the VAL server shall send HTTP POST request message to DA server on the resource URI representing "DA Profiles" resource. The body of the HTTP POST message shall include the DigitalAssetProfile data type, as specified in the clause 7.13.1.3.2.3.1.

Upon receiving the HTTP POST message as described above, the DA server shall:

1.	verify the identity of the VAL server and check if the VAL server is authorized to create DA profile;

2.	if the VAL server is authorized to create DA profile, the DA server shall;

a.	create a new resource for Individual DA profile as specified in clause 7.13.1.3.2.3.1; and

b.	return within the response message body the created resource representation within the DigitalAssetProfile data structure, and within an HTTP Location header the created resource URI in the response message. and

If errors occur when processing the request, the DA Server shall respond to the VAL Server with an appropriate error response as specified in clause 7.13.1.7.

# * * * * Next changes * * * *

5.14.1.2.3.1	General

This service operation is used by VAL server to retrieve an individual digital asset profile information.

This service operation is used by a service consumer to retrieve an individual digital asset profile information on a DA server.

The following procedures are supported by the "Retrieve" service operation:

-	Retrieve Digital Asset Profile.

# * * * * Next changes * * * *

5.14.1.2.3.2	Retrieve Digital Asset Profile

To retrieve the DA profile, the VAL server shall send HTTP GET request message to DA server on the resource URI representing the "Individual DA profile", as specified in clause 7.13.1.3.3.3.1.

Upon receiving the HTTP GET message as described above, the DA server shall:

1.	verify the identity of the VAL server and check if the VAL server is authorized to retrieve the DA profile;

2.	if the VAL server is authorized to retrieve the DA profile, the DA server shall;

a.	return the DA profile in DigitalAssetProfile data type, as specified in clause 7.13.1.3.3.3.1.

3.	if errors occur when processing the request, the DA Server shall respond to the VAL Server with an appropriate error response as specified in clause 7.13.1.7.

# * * * * Next changes * * * *

5.14.1.2.4.1	General

The following procedures are supported by the "Update" service operation:

-	Update Digital Asset Profile.

# * * * * Next changes * * * *

5.14.1.2.4.2	Update Digital Asset Profile

To modify the DA profile, the VAL server shall send HTTP PUT message to the DA server to the Resource URI identifying the "Individual DA profile" resource representation, as specified in the clause 7.13.1.3.3.3.2. The VAL server may send an HTTP PATCH request message to the "Individual DA profile" resource URI as specified in clause 7.13.1.3.3.3.3. The body of the HTTP PATCH request message shall include the requested modifications as specified in clause 7.13.1.3.3.3.3.

When HTTP PUT is used, the "valServerId" attribute within the DigitalAssetProfile data structure shall not be updated.

Upon receiving the HTTP PUT or PATCH request message, the DA server shall:

1.	verify the identity of the VAL server and check if the VAL server is authorized to modify the DA profile;

2.	if the VAL server is authorized to modify the information, then the DA server shall;

a.	if the DA profile in the request is valid, update/modify the resource identified by the Resource URI of the DA profile received in the request; and

b.	return a 200 OK status code with the updated DA profile in the response or a 204 No Content status code.

3.	if errors occur when processing the request, the DA Server shall respond to the VAL Server with an appropriate error response as specified in clause 7.13.1.7.

# * * * * Next changes * * * *

5.14.1.2.5.1	General

The following procedures are supported by the "Delete" service operation:

-	Delete Digital asset profile.

# * * * * Next changes * * * *

5.14.1.2.5.2	Delete Digital Asset profile

To delete the DA profile, the VAL server shall send an HTTP DELETE message to the resource representing "Individual DA profile" as specified in clause 7.13.1.3.3.3.4.

Upon receiving the HTTP DELETE message, the DA server shall:

1.	verify the identity of the VAL server and check if the VAL server is authorized to delete the DA profile; and

2.	if the VAL server is authorized to delete the DA profile, the DA server shall;

a.	delete the resource pointed by the Resource URI for Individual DA profile;

b.	return a 204 No Content status code in the response.

3.	if errors occur when processing the request, the DA Server shall respond to the VAL Server with an appropriate error response as specified in clause 7.13.1.7.

# * * * * Next changes * * * *

7.13.1.3.2.3.1	POST

This method shall support the URI query parameters specified in table 7.13.1.3.2.3.1-1.

Table 7.13.1.3.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.13.1.3.2.3.1-2 and the response data structures and response codes specified in table 7.13.1.3.2.3.1-3.

Table 7.13.1.3.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| DigitalAssetProfile | M | 1 | Create DA profile |

Table 7.13.1.3.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| DigitalAssetProfile | M | 1 | 201 Created | DA profile is created successfully. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] also apply. |  |  |  |  |

Table 7.13.1.3.2.3.1-4: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/ss-da-pm/<apiVersion>/profiles/{daId} |

# * * * * Next changes * * * *

7.13.1.3.3.2	Resource Definition

Resource URI: {apiRoot}/ss-da-pm/<apiVersion>/profiles/{daId}

This resource shall support the resource URI variables defined in the table 7.13.1.3.3.2-1.

Table 7.13.1.3.3.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.5 |
| daId | string | Represents an individual DA Profile resource. |

# * * * * Next changes * * * *

# A.33	SS_DAProfileManagement API

openapi: 3.0.0

info:

title: SS_DAProfileManagement

version: 1.0.0-alpha.1

description: |

SS_DAProfileManagement service to manage digital asset profile on a DA server.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.549 V19.4.0 Service Enabler Architecture Layer for Verticals (SEAL);

Application Programming Interface (API) specification; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.549/

security:

- {}

- oAuth2ClientCredentials: []

servers:

- url: '{apiRoot}/ss-da-pm/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 6.5 of 3GPP TS 29.549

paths:

/profiles:

post:

description: Creates a new Individual DA profile.

operationId: CreateDAProfile

tags:

- DA profile (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/DigitalAssetProfile'

responses:

'201':

description: Digital Asset Profile resource is created sucessfully.

content:

application/json:

schema:

$ref: '#/components/schemas/DigitalAssetProfile'

headers:

Location:

description: Contains the URI of the newly created resource.

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/profiles/{daId}:

get:

description: Retrieves an individual DA profile.

operationId: RetrieveDAProfile

tags:

- Individual DA Profile(Document)

parameters:

- name: daId

in: path

description: String identifying an individual DA profile resource.

required: true

schema:

type: string

responses:

'200':

description: Successful case. The requested "Individual DA profile” is retrieved.

content:

application/json:

schema:

$ref: '#/components/schemas/DigitalAssetProfile'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

description: Updates an individual DA Profile.

operationId: UpdateDAProfile

tags:

- Individual DA Profile(Document)

parameters:

- name: daId

in: path

description: String identifying an individual configuration resource.

required: true

schema:

type: string

requestBody:

description: Configuration information to be updated in DA server.

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/DigitalAssetProfile'

responses:

'200':

description: The configuration is updated successfully.

content:

application/json:

schema:

$ref: '#/components/schemas/DigitalAssetProfile'

'204':

description: No Content

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

patch:

description: Modify an existing DA Profile.

operationId: ModifyDAProfile

tags:

- Individual DA Profile(Document)

parameters:

- name: daId

in: path

description: Identifier of an individual DA Profile.

required: true

schema:

type: string

requestBody:

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/DigitalAssetProfilePatch'

responses:

'200':

description: >

The individual DA Profile is modified successfully and

a representation of the updated DA Profile is returned

in the request body.

content:

application/json:

schema:

$ref: '#/components/schemas/DigitalAssetProfile'

'204':

description: >

No Content. The individual DA Profile is

modified successfully.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

description: Deletes an individual DA Profile.

operationId: DeleteDAProfile

tags:

- Individual DA Profile(Document)

parameters:

- name: daId

in: path

description: String identifying an individual DA profile resource.

required: true

schema:

type: string

responses:

'204':

description: The individual configuration matching daId is deleted.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

DigitalAssetProfile:

description: Represents the DA Profile information.

type: object

properties:

accCtrlLists:

type: array

minItems: 1

items:

$ref: '#/components/schemas/AccessControlList'

description: >

Represents the list of tuples indicating the requestors allowed to perform operations

with this digital asset and the allowed operation types.

accessories:

type: array

minItems: 1

items:

$ref: '#/components/schemas/DaId'

description: >

Represents the list of digital asset identifiers which are accessories (e.g. Hat, watch,

shoes) purchased for this digital asset profile

appIds:

type: array

items:

type: string

minItems: 1

daId:

$ref: '#/components/schemas/DaId'

daOwnerIds:

type: array

minItems: 1

items:

$ref: '#/components/schemas/DigitalAssetOwnerId'

description: >

Represents the list of identifier(s) for the digital asset owner.

daType:

$ref: '#/components/schemas/DigitalAssetType'

daUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

expTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

spatialCondts:

type: array

minItems: 1

items:

$ref: '#/components/schemas/SpatialCondition'

description: >

Represents the list of spatial conditions (e.g. locations) where this digital asset

profile is allowed to be accessed.

srvProviderIds:

type: array

minItems: 1

items:

type: string

description: >

Represents the list of service providers for the digital asset.

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

valServerId:

type: string

DigitalAssetProfilePatch:

description: Represents the DA Profile information patch.

type: object

properties:

accCtrlLists:

type: array

minItems: 1

items:

$ref: '#/components/schemas/AccessControlList'

description: >

Represents the list of tuples indicating the requestors allowed to perform operations

with this digital asset and the allowed operation types.

accessories:

type: array

minItems: 1

items:

$ref: '#/components/schemas/DaId'

description: >

Represents the list of digital asset identifiers which are accessories (e.g. Hat, watch,

shoes) purchased for this digital asset profile

appIds:

type: array

items:

type: string

minItems: 1

daOwnerIds:

type: array

minItems: 1

items:

$ref: '#/components/schemas/DigitalAssetOwnerId'

description: >

Represents the list of identifier(s) for the digital asset owner.

daType:

$ref: '#/components/schemas/DigitalAssetType'

expTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

spatialCondts:

type: array

minItems: 1

items:

$ref: '#/components/schemas/SpatialCondition'

description: >

Represents the list of spatial conditions (e.g. locations) where this digital asset

profile is allowed to be accessed.

srvProviderIds:

type: array

minItems: 1

items:

type: string

description: >

Represents the list of service providers for the digital asset.

AccessControlList:

description: >

Represents the list of tuples indicating the requestors allowed to perform

operations with this digital asset and the allowed operation types

type: object

properties:

allowedUsers:

type: array

items:

$ref: '#/components/schemas/AllowedUser'

description: >

Represents the list of users allowed to access this digital asset.

minItems: 1

AllowedUser:

description: Represents the user allowed to access this digital asset.

type: object

properties:

user:

type: string

allowedOperations:

type: array

items:

$ref: '#/components/schemas/Operations'

description: >

Represents the list of operations allowed by the user.

minItems: 1

predModels:

type: array

items:

type: string

description: >

Represents the list of one or more predictive model for the user.

minItems: 1

required:

- user

SpatialCondition:

description: Represents the location where DA profile is allowed to be accessed.

type: object

properties:

currLoc:

$ref: 'TS29122_MonitoringEvent.yaml#/components/schemas/LocationInfo'

# Simple data types and Enumerations

DigitalAssetOwnerId:

description: Represents the identity of the digital asset owner

type: string

DaId:

description: Represents the identity of the digital asset name

type: string

DigitalAssetType:

anyOf:

- type: string

enum:

- ACCESSORY

- AVATAR

- GIFT_CERTIFICATES

- SOFTWARE_LICENSE

- TOKENS_AND_FILES

- WALLET

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration and is not used to encode

content defined in the present version of this API.

description: |

Represents the different type of digital assets e.g. avatar, wallet

Possible values are:

- ACCESSORY: Represents the accessory.

- AVATAR: Represents the avatar

- GIFT_CERTIFICATES: Represents the gift certificates.

- SOFTWARE_LICENSE: Represents the software license

- TOKENS_AND_FILES: Represents the tokens and files.

- WALLET: Represents the Digital wallet

Operations:

anyOf:

- type: string

enum:

- DISCOVERY

- RETRIEVE

- CREATE

- UDPATE

- DELETE

- UPLOAD

- DOWNLOAD

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration and is not used to encode

content defined in the present version of this API.

description: |

Represents a desired condition of the requested location change.

Possible values are:

- DISCOVERY: Indicates that the operation is discovery.

- RETRIEVE: Indicates that the operation is retrieve.

- CREATE: Indicates that the operation is CREATE.

- UDPATE: Indicates that the operation is UPDATE.

- DELETE: Indicates that the operation is DELETE.

- UPLOAD: Indicates that the Digital asset media operation is UPLOAD.

- DOWNLOAD: Indicates that the Digital asset media operation is DOWNLOAD.

# * * * * End of Changes * * * *