3GPP TSG-CT WG3 Meeting #143	C3-254415

# Sophia-Antipolis, France, 13 - 17 October 2025						(Revision of C3-254107)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.549 | CR | 0456 | rev | 1 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Digital Asset discovery update |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C3 |  |  |  |  |  |  |  |  |  |
| Work item code: | Metaverse_App |  |  |  |  |  | Date: |  |  | 14-10-2025 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Digital Asset discovery API has the following errors: - DaDiscResult update as per stage 2 in the service description and the OpenAPI. - Incorrect URI query parameters names. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The above-mentioned Digital Asset discovery issues are fixed. DaDiscResult attributes are arranged in the alphabetical order. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The digital asset discovery result is not inline with stage 2. The quality of the Digital Asset discover API is not up to the mark in terms of URI query parameters. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.14.2.2.2.2, 7.13.2.3.2.3.1, 7.13.2.6.2.3, A.34 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces a backward compatible feature to the following APIs: TS29549_SS_DAProfileManagement.yaml TS29549_SS_DADiscovery.yaml TS29122_CommonData.yaml |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First Change * * * *

5.14.2.2.2.2	Discover Digital Asset profiles

To discover the digital assets, the Service consumer shall send HTTP GET request message to DA server on the targeting resource URI of the DA profile with the request URI query parameters including the filter criteria to discover the matching digital assets.

Upon receiving the HTTP GET message as described above, the DA server shall:

1.	verify the identity of the Service consumer and check if the Service consumer is authorized to discover the digital asset;

2.	In case the Service consumer is authorized to discover the digital asset, the DA server shall;

a.	Upon success, the DA server shall respond with an HTTP "200 OK" status code with the response body containing the digital asset discovery results within the DaDiscResult data type, as specified in clause 7.13.2.3.2.3.1.

If the error(s) occur when processing the request, the DA Server shall respond to the Service consumer with an appropriate error response as specified in clause 7.13.2.7.

# * * * * Next changes * * * *

7.13.2.3.2.3.1	GET

This operation allows a service consumer to discover digital assets in DA Server. This method shall support the URI query parameters specified in table 7.13.2.3.2.3.1-1.

Table 7.13.2.3.2.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| allowed-users | array(AllowedUser) | C | 1..N | Contains the list of allowed users (NOTE 1) |
| da-desc | string | O | 0..1 | Contains the description of the digital asset. (NOTE 1) |
| da-ids | array(DaId) | C | 1..N | Contains the identifier(s) of the targeted digital asset(s). (NOTE 1) |
| da-name | string | C | 0..1 | Contains the Digital asset name. (NOTE 1) |
| da-owner-id | DigitalAssetOwnerId | C | 0..1 | Contains the Digital asset owner (NOTE 1) |
| da-type | DigitalAssetType | C | 0..1 | Contains the Digital asset type (NOTE 1) |
| spatial-condt | SpatialCondition | C | 0..1 | Contains the Spatial condition including location. (NOTE 1) |
| supp-feats | SupportedFeatures | C | 0..1 | Contains the list of supported feature(s) among the ones defined in clause 6.1.2.8. This attribute shall be present only when feature negotiation is required. |
| val-target-ue | ValTargetUe | C | 0..1 | Contains either VAL User ID or VAL UE ID (NOTE 1) |
| NOTE 1: At least one of these query parameters shall be present. |  |  |  |  |

This method shall support the request data structures specified in table 7.13.2.3.2.3.1-2 and the response data structures and response codes specified in table 7.13.2.3.2.3.1-3.

Table 7.13.2.3.2.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 7.13.2.3.2.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| DaDiscResp | M | 1 | 200 OK | Successful case. DA details matching the query filter criteria shall be returned. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection, during resource retrieval. The response shall include a Location header field containing an alternative URI of the resource located in an alternative DA server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection, during resource retrieval. The response shall include a Location header field containing an alternative URI of the resource located in an alternative DA server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [3]. |
| NOTE: The mandatory HTTP error status codes for the GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [3] also apply. |  |  |  |  |

Table 7.13.2.3.2.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative DA server. |

Table 7.13.2.3.2.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative DA server. |

# * * * * Next changes * * * *

7.13.2.6.2.3	Type: DaDiscResult

Table 7.13.2.6.2.3-1: Definition of type DaDiscResult

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| daDesc | string | O | 0..1 | Contains the description of the digital asset. |  |
| daId | DaId | C | 0..1 | Contains the identifier of the digital asset. (NOTE 1) |  |
| daName | string | C | 0..1 | Contains the name of the digital asset matching the discovery criteria (NOTE 1) |  |
| daStatus | DaStatus | M | 1 | Contains the status of the digital asset. |  |
| uri | Uri | O | 0..1 | A URL of the service provider the digital asset is associated with. |  |
| NOTE 1: At least one of these query parameters shall be present. |  |  |  |  |  |

# * * * * Next changes * * * *

# A.34	SS_DADiscovery API

openapi: 3.0.0

info:

title: DA Server digital asset Discovery Service

version: 1.0.0-alpha.1

description: |

API for Digital Asset Discovery Service.

©2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.549 V19.4.0 Service Enabler Architecture Layer for Verticals (SEAL);

Application Programming Interface (API) specification; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.549/

servers:

- url: "{apiRoot}/ss-da-d/v1"

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 6.5 of 3GPP TS 29.549.

security:

- {}

- oAuth2ClientCredentials: []

paths:

/digitalasset:

get:

summary: Discover Digital Asset.

operationId: DiscDigitalAsset

tags:

- Digital Asset (Collection)

parameters:

- name: allowed-users

in: query

description: Contains the list of allowed users

content:

application/json:

schema:

type: array

items:

$ref: 'TS29549_SS_DAProfileManagement.yaml#/components/schemas/AllowedUser'

minItems: 1

- name: da-desc

in: query

description: Contains the description of the digital asset.

schema:

type: string

- name: da-ids

in: query

description: Contains the identifier(s) of the targeted digital asset(s).

content:

application/json:

schema:

type: array

items:

$ref: 'TS29549_SS_DAProfileManagement.yaml#/components/schemas/DaId'

minItems: 1

- name: da-name

in: query

description: Contains the Digital asset name.

schema:

type: string

- name: da-owner-id

in: query

description: Contains the Digital asset owner Identity.

schema:

$ref: 'TS29549_SS_DAProfileManagement.yaml#/components/schemas/DigitalAssetOwnerId'

- name: da-type

in: query

description: >

Contains the digital asset type.

content:

application/json:

schema:

$ref: 'TS29549_SS_DAProfileManagement.yaml#/components/schemas/DigitalAssetType'

- name: spatial-condt

in: query

description: >

Contains the Spatial condition including location.

content:

application/json:

schema:

$ref: 'TS29549_SS_DAProfileManagement.yaml#/components/schemas/SpatialCondition'

- name: supp-feats

in: query

description: >

Contains the list of supported feature(s) among the ones defined in clause 6.1.2.8.

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

- name: val-target-ue

in: query

required: true

content:

application/json:

schema:

$ref: 'TS29549_SS_UserProfileRetrieval.yaml#/components/schemas/ValTargetUe'

responses:

'200':

description: >

OK. The Digital Asset matching the filter criteria are returned.

content:

application/json:

schema:

$ref: '#/components/schemas/DaDiscResp'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: "{tokenUrl}"

scopes: {}

schemas:

# API specific definitions

#

# STRUCTURED DATA TYPES

#

DaDiscResp:

description: >

Represents the Digital Asset discovery response.

type: object

properties:

results:

type: array

items:

$ref: '#/components/schemas/DaDiscResult'

minItems: 0

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- results

DaDiscResult:

description: >

Represents the Digital Asset discovery result.

type: object

properties:

daDesc:

type: string

daId:

$ref: 'TS29549_SS_DAProfileManagement.yaml#/components/schemas/DaId'

daName:

type: string

daStatus:

$ref: '#/components/schemas/DaStatus'

uri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

required:

- daStatus

anyOf:

- required: [daId]

- required: [daName]

# SIMPLE DATA TYPES

#

#

# ENUMERATIONS

#

DaStatus:

anyOf:

- type: string

enum:

- IN_USE

- ARCHIVED

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration and is not used to encode

content defined in the present version of this API.

description: |

Represents the analytic IDs for location related UE group analytics.

Possible values are:

- INUSE: Indicates that the digital asset is in use.

- ARCHIVED: Indicates that the digital asset is Archived.

# Data types describing alternative data types or combinations of data types:

#

# * * * * End of Changes * * * *