3GPP TSG-CT3 Meeting #143                                                                             C3-254417

# Sophia-Antipolis, France, 13 - 17 October, 2025                                    (was C3-254178)

Source:					Samsung

Title:	Pseudo-CR on Corrections to SS_SAnUsage_Report and SS_SmDiscovery API

Spec:	3GPP TS 29.437 (v1.1.0)

Agenda item:	19.42

Document for:	Approval

1. Introduction

This pCR proposes corrections to section 5.2.3.2.7.2 and Table 6.2.2.6.1-1 in section 6.2.2.6.1 in TS 29.437.

2. Reason for Change

The existing sections are not aligned with the TS 23.437.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 29.437 v1.1.0

* * * First Change * * *

##### 5.2.3.2.7	SS_SAnUsage_Report

5.2.3.2.7.1	General

This service operation is used by a service consumer to report the Spatial Anchors Usage Information at the SAn Server.

The following procedures are supported by the "SS_SAnUsage_Report" service operation:

-	Spatial Anchor Usage Information Report.

5.2.3.2.7.2	Spatial Anchor Usage Information Report

Figure 5.2.3.2.7.2-1 depicts a scenario where a service consumer sends a request to the SAn Server to report spatial anchors usage information (see also SS_SAnUsageReport_Request service operation, clause 8.4.4 of 3GPP TS 23.437 [13]).

Figure 5.2.3.2.7.2-1: Procedure for Spatial Anchor Usage Information Report

1.	In order to report the spatial anchor usage information, the service consumer shall send an HTTP POST request to the SAn Server targeting the URI of the corresponding custom operation (i.e., "Request"), with the request body including the SpatialAnchorUsageReportReq data structure.

2a.	Upon success, the SAn Server shall respond with an HTTP "204 No Content" status code to confirm the fulfilment of spatial anchor usage information report request.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP POST response body, as specified in clause 6.1.3.7.

* * * Second Change * * * *

##### 6.2.2.6.1	General

This clause specifies the application data model supported by the API.

Table 6.2.2.6.1-1 specifies the data types defined for the SS_SmDiscovery API.

Table 6.2.2.6.1-1: SS_SmDiscovery API specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| SpatialMapsDiscFilter | 6.2.2.6.2.2 | Represents the discovery filter information for spatial maps. |  |
| SpatialMapDiscovered | 6.2.2.6.2.4 | Represents the discovered spatial maps. |  |
| SpatialMapsDiscResp | 6.2.2.6.2.3 | Represents the spatial maps discovery response. |  |

Table 6.2.2.6.1-2 specifies data types re-used by the SS_SmDiscovery API from other specifications, including a reference to their respective specifications, and when needed, a short description of their use within the SS_SmDiscovery API.

Table 6.2.2.6.1-2: SS_SmDiscovery API re-used Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| LocationInfo | 3GPP TS 29.122 | Represents the location information and direction. |  |
| ServiceArea | 3GPP TS 29.558 [18] | Represents the service area information. |  |
| SMASProfile | 6.2.5.6.2.4 | Represents the SMAS profile information. |  |
| SpatialMapId | 6.2.1.6.3.2 | Represents the spatial map identifier. |  |
| SpatialMapLayers | 6.2.1.6.2.7 | Represents the spatial map layers information. |  |
| SupportedFeatures | 3GPP TS 29.571 [15] | Represents the list of supported feature(s) and used to negotiate the applicability of the optional features. |  |

* * * End of Changes * * * *