3GPP TSG-CT3 Meeting #143                                                                         C3-254418

Sophia Antipolis, France, 13 - 17 October, 2025                                  (was C3-254179)

Source:	Samsung, Nokia

Title:	Pseudo-CR on SS_SmLocalization API data model

Spec:	3GPP TS 29.437 (v1.0.0)

Agenda item:	19.42

Document for:	Approval

1. Introduction

This pCR proposes the API data model for SS_SmLocalization service API.

Note: updated pose data type, removed TargetLocalizedIdentities datatype, added RelativeCartesianLocation as a datatype, updated the datatype of target-id as ValTargetUe.

2. Reason for Change

The SS_SmLocalization API, as specified in TS 23.437, enables VAL server or SEAL SM Client to get localization information from the spatial maps. The data model of the same needs to be implemented in TS 29.437.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 29.437 v1.0.0

* * * First Change * * * *

### 6.2.3	SS_SmLocalization API

#### 6.2.3.1	Introduction

The SS_SmLocalization service shall use the SS_SmLocalization API.

The API URI of the SS_SmLocalization API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests shall have the Resource URI structure defined in clause 6.5 of 3GPP TS 29.549 [17], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificSuffixes>

with the following components:

-	The {apiRoot} shall be set as described in clause 6.5 of 3GPP TS 29.549 [17].

-	The <apiName> shall be "ssm-loc".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificSuffixes> shall be set as described in clause 6.2.3.4.

NOTE:	When 3GPP TS 29.122 [2] is referenced for the common protocol and interface aspects for API definition in the clauses under clause 5, the SM Server takes the role of the SCEF and the service consumer takes the role of the SCS/AS.

#### 6.2.3.2	Usage of HTTP and common API related aspects

The provisions of clause 6.3 of 3GPP TS 29.549 [17] shall apply for the SS_SmLocalization API.

#### 6.2.3.3	Resources

##### 6.2.X.3.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 6.2.X.3.1-1 depicts the resource URIs structure for the SS_SmDataSourceDiscovery API.

Figure 6.2.X.3.1-1: Resource URI structure of the SS_SmLocalization API

Table 6.2.X.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 6.2.X.3.1-1: Resources and methods overview

| Resource purpose/name | Resource URI (relative path after API URI) | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| Spatial Localization | /localization | GET | Retrieve Spatial Map localization information. |

##### 6.2.X.3.2	Resource: Spatial Localization

6.2.X.3.2.1	Description

This resource represents the collection of SM Data Source Discoveries managed by the SM Server.

6.2.X.3.2.2	Resource Definition

Resource URI: {apiRoot}/ssm-loc/<apiVersion>/localization

This resource shall support the resource URI variables defined in table 6.2.X.3.2.2-1.

Table 6.2.X.3.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.2.X.1. |

6.2.X.3.2.3	Resource Standard Methods

6.2.X.3.2.3.1	GET

The HTTP GET method allows a service consumer to discover SM data source information from the SM Server.

This method shall support the URI query parameters specified in table 6.2.X.3.2.3.1-1.

Table 6.2.X.3.2.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| area-int | ServiceArea | O | 1 | Contains the three-dimensional area or localization information. |  |
| map-id | SpatialMapId | M | 1 | Contains the spatial map identifier. |  |
| target-id | ValTargetUe | O | 0..1 | Contains the list of identity of the users or objects to localize |  |
| supp-feats | SupportedFeatures | O | 0..1 | Contains the list of supported feature(s) among the ones defined in clause 6.2.3.8. This attribute shall be present only when feature negotiation is required. |  |

This method shall support the request data structures specified in Table 6.2.X.3.2.3.1-2 and the response data structures and response codes specified in Table 6.2.X.3.2.3.1-3.

Table 6.2.X.3.2.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.2.X.3.2.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| SpatialMapLocResp | M | 1 | 200 OK | Successful case. The requested SM data source information shall be returned. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative SM Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative SM Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The mandatory HTTP error status codes for the HTTP GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] shall also apply. |  |  |  |  |

Table 6.2.X.3.2.3.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative SM Server. |

Table 6.2.X.3.2.3.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative SM Server. |

6.2.X.3.2.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

#### 6.2.3.4	Custom Operations without associated resources

There are no custom operations without associated resources defined for this API in this release of the specification.

#### 6.2.3.5	Notifications

There are no notifications defined for this API in this release of the specification.

#### 6.2.3.6	Data Model

##### 6.2.3.6.1	General

This clause specifies the application data model supported by the API.

Table 6.2.3.6.1-1 specifies the data types defined for the SS_SmLocalization API.

Table 6.2.3.6.1-1: SS_SmLocalization API specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| SpatialMapLocDetails | 6.2.3.6.2.5 | Represents the localization details of the spatial map. |  |
| SpatialMapLocalizeResp | 6.2.3.6.2.4 | Represents the spatial map localization response. |  |

Table 6.2.3.6.1-2 specifies data types re-used by the SS_SmLocalization API from other specifications, including a reference to their respective specifications, and when needed, a short description of their use within the SS_SmLocalization API.

Table 6.2.3.6.1-2: SS_SmLocalization API re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| LocationInfo | 3GPP TS 29.122 [2] | Represents the location information. |  |
| Pose | 6.1.1.6.2.25 | Represents the Pose details. |  |
| RelativeCartesianLocation | 3GPP TS 29.579 | Represents the relative cartesian location. |  |
| SpatialMapId | 6.2.1.6.3.2 | Represents the spatial map identifier. |  |
| SupportedFeatures | 3GPP TS 29.571 [15] | Represents the list of supported feature(s) and used to negotiate the applicability of the optional features. |  |
| ValTargetUe | 3GPP TS 29.549 [17] | Represents the VAL UE or VAL user identifier. |  |

##### 6.2.3.6.2	Structured data types

6.2.3.6.2.1	Introduction

This clause defines the structures to be used in resource representations.

6.2.3.6.2.2	Type: SpatialMapLocalizeResp

Table 6.2.3.6.2.2-1: Definition of type SpatialMapLocalizeResp

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| results | array(SpatialMapLocDetails) | M | 0..N | Contains the lists of localization information If there are no target identities (for e.g., VAL UEs or VAL Users) matching the provided localization filter criteria, an empty array shall be returned within this attribute. |  |
| suppFeat | SupportedFeatures | C | 0..1 | Contains the list of supported feature(s) among the ones defined in clause 6.2.3.8. This attribute shall be present only when feature negotiation is required. |  |

6.2.3.6.2.3	Type: SpatialMapLocDetails

Table 6.2.3.6.2.3-1: Definition of type SpatialMapLocDetails

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| targetId | ValTargetUe | O | 0..1 | Contains the identity of VAL UE or VAL user. |  |
| position | RelativeCartesianLocation | O | 0..1 | Contains the three dimensional position of the VAL UE or VAL user in spatial map. |  |
| pose | Pose | O | 0..1 | Contains the pose of the VAL UE or VAL user. |  |

##### 6.2.3.6.3	Simple data types and enumerations

6.2.3.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

6.2.3.6.3.2	Simple data types

The simple data types defined in table 6.2.3.6.3.2-1 shall be supported.

Table 6.2.3.6.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

##### 6.2.X.6.4	Data types describing alternative data types or combinations of data types

There are no data types describing alternative data types or combinations of data types defined for this API in this release of the specification.

##### 6.2.X.6.5	Binary data

6.2.X.6.5.1	Binary Data Types

Table 6.2.X.6.5.1-1: Binary Data Types

| Name | Clause defined | Content type |
| --- | --- | --- |
| n/a |  |  |

* * * End of Changes * * * *