3GPP TSG-CT WG3 Meeting #143	C3-254461

# SophiaAntipolis, FR, 13 - 17 October 2025								(Revision of C3-254103)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.561 | CR | 0199 | rev | 1 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Nonce counter handling in MRI packet transform |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C3 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM_Ph2 |  |  |  |  |  | Date: |  |  | 2025-10-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As per TS 33.501 clause 18.2.3, Nonce Counter field: Contains the 16-bit least significant bits of a 64-bit counter used to construct the nonce to the AEAD protection. The 64-bit counter is incremented with one for each payload protected by the transform for the current VCID. The 64-bit counter is initialized to zero (0) for each VCID. For more information about the counter, see clause 18.2.6. …. If there is no Media related information, the transform shall set the length field to zero (0), and no other additional fields (Counter LSB or Media related information) are inserted into those packets. As per TS 33.501 clause 18.2.6, … To maintain synchronization even for large burst losses of XRM transformed packets, the full 64-bit counter is sent every time the 16-bit Least Significant Bit counter wraps. The corresponding update has to be performed in stage 3. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Nonce counter field is added in the transform function. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Nonce counter handling is missing in stage 3 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First Change * * * *

### 22.3.4	Sending Media Related Information using QUIC-aware proxying using HTTP

If the media traffic is transmitted over QUIC between the AS and the UE, the UPF (acting as client) and the AS (acting as UDP proxy) may negotiate to use the dedicated QUIC Connection IDs and packet transform in Forwarded Mode to avoid re-encapsulation and re-encryption as defined in draft-ietf-masque-quic-proxy [67]. In case of multiple UEs using the same QUIC connection between the UPF and the AS, for each UE the UPF shall initiate a new HTTP CONNECT request to associate each UE with a separate Connection ID for mapping to a Virtual Connection ID as described in in draft-ietf-masque-quic-proxy [67].

As described in draft-ietf-masque-quic-proxy [67], the AS may still send some packets using the tunnelled mode even when the UPF and the AS have negotiated to use Forwarded Mode (e.g. during the initial media exchange with media related information from the AS to the UPF until it is possible to start using Forwarded Mode). Packets sent in tunnelled mode shall be encoded in HTTP Datagrams as defined in clause 22.3.2. Packets sent using Forwarded Mode shall be encoded as defined in this clause.

During the HTTP CONNECT method as described in draft-ietf-masque-quic-proxy [67], in addition to what is required according to clause 22.3.2:

a)	the UPF shall insert 3gpp:media-related-info-transform in the accept-transform parameter of the proxy-quic-forwarding header field in the HTTP CONNECT request; and

b)	the AS shall indicate 3gpp:media-related-info-transform in the transform parameter of the proxy-quic-forwarding header field in the HTTP CONNECT successful response.

When the Media Related Information are transformed in the QUIC short header as per IETF RFC 9000 [68], then the 3gpp:media-related-info-transform shall be used to encode the transformed QUIC packet as shown in Figure 22.3.4-1 (using the notational conventions defined in section 1.3 of IETF RFC 9000 [68]).

Transformed QUIC Packet {

Header Form (1) = 0,

Fixed Bit (1) = 1,

Spin Bit (1),

Reserved Bits (2),

Key Phase (1),

Packet Number Length (2),

Destination Connection ID (0..160),

Length of Protected Media Related Information (8),

Length of Nonce counter (6),

Spare Bits (2),

Nonce Counter (0..64),

Protected Media Related Information (..),

Packet Number (8..32),

Packet Payload (8..),

}

Figure 22.3.4-1: Transformed QUIC Packet with Media Related Information Container as per 3gpp:media-related-info-transform

In Figure 22.3.4-1:

a)	Destination Connection ID shall be set to the mapping Virtual Connection ID;

b)	the length of the protected Media Related Information shall be encoded as 8 bits indicating  the number of bytes;

c)	the length of the Nonce Counter shall be encoded as an 6-bit unsigned integer indicating the number of bits of the Nonce Counter. In this release, the possible length value of the Nonce counter shall be 0, 16 or 64. In case of the length of the protected media related information is zero, the length of the Nonce Counter shall be set to 0.

d)	the Spare Bits shall be set to the value of 0. These two bits are added for byte alignment and are spare for possible future use.

e)	Nonce Counter with 16-bit significant bits of a 64-bit counter is sent. The full 64-bit counter is sent every time the 16-bit Least Significant Counter wraps. In case of the length of the Nonce Counter is zero, the Nonce Counter shall not be present. The Nonce Counter details are defined in clause 18.2.3 of 3GPP TS 33.501 [59].

f)	Protected Media Related Information shall be the output of the security algorithm defined in clause 18.2.4 of 3GPP TS 33.501 [59] which takes as an input the Media Related Information container, defined in clause 22.2. In case of the length of the protected media related information is zero, the Protected Media Related Information shall not be present; and

g)	Packet Number and Packet Payload shall be set to the (end-to-end protected) packet number and the packet payload of the end-to-end packet.

Editor's Note: IANA registration of the 3gpp:media-related-info-transform is needed.

If the Media Related Information is missing, the length of Protected Media Related Information shall be set to zero and the Protected Media Related Information field shall be omitted.

# * * * * End of Changes * * * *