3GPP TSG-CT WG3 Meeting #143	C3-254467

# Sophia-Antipolis, France, 13 - 17 October 2025

Source:	Ericsson

Title:	Pseudo-CR on corrections of Data Model in AIMLES_AIMLEClientDiscovery API

Spec:	3GPP TS 29.482 V1.1.0

Agenda item:	19.41

Document for:	Approval

1. Introduction

TS 29.482 AIMLES_AIMLEClientDiscovery API data model is incomplete, has some incorrections, a reference was missing, and ClientVelocity data type do not supported all possible values from stage 2.

2. Reason for Change

The current document has a missing reference and some incorrections and an incomplete data model for AIMLES_AIMLEClientDiscovery API.

In stage 2 ClientVelocity data type has the following description: “Indicates the AIMLE client velocity. It includes mobile (e.g., high, low), static.”, but in TS 29.842 V1.1.0 the static value is missing.

3. Conclusions

This pCR proposes changes to add missing reference, correct and complete the data model of AIMLES_AIMLEClientDiscovery API, and add missing value to ClientVelocity enumeration.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 29.482 V1.1.0.

# *** First Change ***

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 29.122: "T8 reference point for Northbound Application Programming Interfaces (APIs)".

[3]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3".

[4]	OpenAPI: "OpenAPI Specification Version 3.0.0", https://spec.openapis.org/oas/v3.0.0.

[5]	3GPP TR 21.900: "Technical Specification Group working methods".

[6]	3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs; Stage 2".

[7]	3GPP TS 29.222: "Common API Framework for 3GPP Northbound APIs; Stage 3".

[8]	3GPP TS 33.122: "Security aspects of Common API Framework (CAPIF) for 3GPP northbound APIs".

[9]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

[10]	IETF RFC 9113: "HTTP/2".

[11]	IETF RFC 8259: "The JavaScript Object Notation (JSON) Data Interchange Format".

[12]	IETF RFC 9457: "Problem Details for HTTP APIs".

[13]	3GPP TS 23.482: "Functional architecture and information flows for AIML Enablement Service".

[14]	3GPP TS 29.549: "Service Enabler Architecture Layer for Verticals (SEAL); Application Programming Interface (API) specification; Stage 3".

[15]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

[16]	3GPP TS 24.560: "Artificial Intelligence Machine Learning (AIML) Services - Service Enabler Architecture Layer for Verticals (SEAL); Protocol Specification; Stage 3".

[17]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

[18]	3GPP TS 29.572: "5G System; Location Management Services; Stage 3".

[19]	3GPP TS 29.558: "Enabling Edge Applications; Application Programming Interface (API) specification; Stage 3".

[A]	3GPP TS 29.548: "Service Enabler Architecture Layer for Verticals (SEAL); SEAL Data Delivery (SEALDD) Server Services; Stage 3".

# *** Next Change ***

###### 6.1.6.3.2.1	Description

The "AIMLE Clients" resource represents the AIMLE Clients.

# *** Next Change ***

###### 6.1.6.3.2.2	Resource Definition

Resource URI: {apiRoot}/aimles-disc/<apiVersion>/clients

This resource shall support the resource URI variables defined in the table 6.1.6.3.2.2-1.

Table 6.1.6.3.2.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.5 of 3GPP TS 29.549 [14]. |

# *** Next Change ***

###### 6.1.6.3.2.3	Resource Standard Methods

# *** Next Change ***

6.1.6.3.2.3.1	GET

This method shall support the URI query parameters specified in table 6.1.6.3.2.3.1-1.

Table 6.1.6.3.2.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| filt-criteria | ClientDiscCriteria | M | 1 | Represents the AIMLE Client(s) filtering criteria. |
| clNumber | Uinteger | O | 0..1 | Represents the required number of the discovered AIMLE Clients. |
| supported-features | SupportedFeatures | C | 0..1 | Contains supported features information, used to negotiate the applicability of optional features. This query parameter shall be present only if feature negotiation needs to take place. |

This method shall support the request data structures specified in table 6.1.6.3.2.3.1-2 and the response data structures and response codes specified in table 6.1.6.3.2.3.1-3.

Table 6.1.6.3.2.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.1.6.3.2.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ClientDiscResp | M | 1 | 200 OK | Successful case. The response body contains the result of the search over the list of AIMLE Clients. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative AIMLE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative AIMLE Server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The mandatory HTTP error status codes for the HTTP GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [2] shall also apply. |  |  |  |  |

Table 6.1.6.3.2.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AIMLE Server. |

Table 6.1.6.3.2.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AIMLE Server. |

# *** Next Change ***

###### 6.1.6.3.2.4	Resource Custom Operations

There are no resource custom operations defined for this resource in this release of the specification.

# *** Next Change ***

#### 6.1.6.5	Notifications

There are no notifications in the present release of the document.

# *** Next Change ***

#### 6.1.6.6	Data Model

# *** Next Change ***

##### 6.1.6.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.6.6.1-1 specifies the data types defined for the AIMLES_AIMLEClientDiscovery API.

Table 6.1.6.6.1-1: AIMLES_AIMLEClientDiscovery API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| AimlOperationRole | 6.1.6.6.3.5 | Represents the AIMLE operation role. |  |
| ClientAppCapability | 6.1.6.6.2.4 | Represents the AIMLE Client Application layer capabilities. |  |
| ClientDiscCriteria | 6.1.6.6.2.2 | Represents the AIMLE Client discovery criteria. |  |
| ClientDiscResp | 6.1.6.6.2.9 | Represents the AIMLE Client discovery response. |  |
| ClientTaskCapability | 6.1.6.6.2.8 | Represents the AIMLE Client task capability information. |  |
| ClientVelocity | 6.1.6.6.3.10 | Represents the AIMLE Client velocity level. |  |
| ComputeCapability | 6.1.6.6.3.9 | Represents the AIMLE Client compute capability type. |  |
| DataType | 6.1.6.6.3.7 | Represents the type of the collected data. |  |
| DatasetAvailablity | 6.1.6.6.2.6 | Represents the dataset availability information. |  |
| DatasetCapability | 6.1.6.6.2.7 | Represents the dataset capability informatiion. |  |
| DatasetRequirement | 6.1.6.6.2.5 | Represents the dataset requirements. |  |
| MlAppType | 6.1.6.6.3.6 | Represents the ML application type. |  |
| MlModelType | 6.1.6.6.3.4 | Represents the ML model type. |  |
| PerformanceCapability | 6.1.6.6.3.8 | Represents the performance capabilities. |  |
| ServicePermLevel | 6.1.6.6.3.3 | Represents the service permision level. |  |
| ServiceRequirement | 6.1.6.6.2.3 | Represents the AIMLE service requirements. |  |

Table 6.1.6.6.1-2 specifies data types re-used by the AIMLES_AIMLEClientDiscovery API from other specifications, including a reference to their respective specifications, and when needed, a short description of their use within the AIMLES_AIMLEClientDiscovery API.

Table 6.1.6.6.1-2: AIMLES_AIMLEClientDiscovery API Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| DateTime | 3GPP TS 29.122 [2] | Used to indicate a timestamp. |  |
| LocationArea5G | 3GPP TS 29.122 [2] | Represents the location. |  |
| ScheduledCommunicationTime | 3GPP TS 29.122 [2] | Represents an offered scheduled communication time. |  |
| SupportedFeatures | 3GPP TS 29.571 [15] | Represents the supported features, used to negotiate the supported optional features of the API. |  |
| TransQualMeasCriteriaSet | 3GPP TS 29.548 [A] | Represents QoS criteria. |  |
| Uinteger | 3GPP TS 29.571 [15] | Represents an unsigned Integer. |  |

# *** Next Change ***

##### 6.1.6.6.2	Structured data types

# *** Next Change ***

###### 6.1.6.6.2.1	Introduction

This clause defines the structures to be used in resource representations.

# *** Next Change ***

###### 6.1.6.6.2.2	Type: ClientDiscCriteria

Table 6.1.6.6.2.2-1: Definition of type ClientDiscCriteria

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| serviceReq | ServiceRequirement | M | 1 | Represents the service requirements. |  |
| mlModelTypes | array(MlModelType) | O | 1..N | Represents the requested ML model types. |  |
| aimlOpers | array(AimlOperationRole) | M | 1..N | Represents the roles for AI/ML operations. |  |
| clientAppCap | ClientAppCapability | M | 1 | Represents the AIMLE Client application layer capabilities. |  |
| datasetReq | DatasetRequirement | O | 0..1 | Represents the dataset requirements. |  |
| clientTaskCap | ClientTaskCapability | O | 0..1 | Represents the AIMLE Client task capabilities. |  |
| clientVel | ClientVelocity | O | 0..1 | Represents the AIMLE Client velocity. |  |
| location | LocationArea5G | O | 0..1 | Represents the location information for the AIMLE Client discovery. |  |
| clientQosReqs | array(TransQualMeasCriteriaSet) | O | 1..N | Represents the requirements on AIMLE Client QoS for discovery. |  |

# *** Next Change ***

###### 6.1.6.6.2.3	Type: ServiceRequirement

Table 6.1.6.6.2.3-1: Definition of type ServiceRequirement

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| valServId | string | M | 1 | Represents the VAL Service identifier. |  |
| permLevel | ServicePermLevel | O | 0..1 | Represents the service permission level. |  |

# *** Next Change ***

###### 6.1.6.6.2.4	Type: ClientAppCapability

Table 6.1.6.6.2.4-1: Definition of type ClientAppCapability

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| appType | MlAppType | M | 1 | Represents the ML application identifier. |  |
| avail | array(ScheduledCommunicationTime) | O | 1..N | Represents the availablility of AIMLE Client to support opertations. |  |
| dropOfRate | integer | O | 0..1 | Represents the drop off rate in percent. Minumum: 0 Maximum: 100 |  |

# *** Next Change ***

###### 6.1.6.6.2.5	Type: DatasetRequirement

Table 6.1.6.6.2.5-1: Definition of type DatasetRequirement

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| avail | DatasetAvailablity | O | 0..1 | Represents the dataset availablity information. |  |
| cap | DatasetCapability | O | 0..1 | Represents the dataset capability information. |  |

# *** Next Change ***

###### 6.1.6.6.2.6	Type: DatasetAvailablity

Table 6.1.6.6.2.6-1: Definition of type DatasetAvailablity

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| ids | array(string) | O | 1..N | Represents the list dataset identifiers. |  |
| age | DateTime | O | 0..1 | Represents the age of the dataset, i.e., when the data set was created. |  |
| size | Uinteger | O | 0..1 | Represents the dataset size in bytes. |  |
| features | array(string) | O | 1..N | Represents the list of dataset features identifiers. |  |

# *** Next Change ***

###### 6.1.6.6.2.7	Type: DatasetCapability

Table 6.1.6.6.2.7-1: Definition of type DatasetCapability

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| dataType | DataType | O | 0..1 | Represents the type of data. |  |
| functions | array(string) | O | 1..N | Represents the list of the exploratory data analysis functions. |  |

# *** Next Change ***

###### 6.1.6.6.2.8	Type: ClientTaskCapability

Table 6.1.6.6.2.8-1: Definition of type ClientTaskCapability

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| compCap | ComputeCapability | O | 0..1 | Represents the compute capabilities. |  |
| perfCap | PerformanceCapability | O | 0..1 | Represents the performance capabilities. |  |

# *** Next Change ***

###### 6.1.6.6.2.9	Type: ClientDiscResp

Table 6.1.6.6.2.9-1: Definition of type ClientDiscResp

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| clients | array(string) | M | 1..N | Represents the AIMLE client identifiers that matches the discovery criteria. |  |
| suppTasks | array(ClientTaskCapability) | O | 1..N | Represents the supported AIML tasks. |  |
| suppFeats | SupportedFeatures | C | 0..1 | Contains supported features information, used to negotiate the applicability of optional features. This query parameter shall be present only if feature negotiation needs to take place. |  |

# *** Next Change ***

##### 6.1.6.6.3	Simple data types and enumerations

# *** Next Change ***

###### 6.1.6.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

# *** Next Change ***

###### 6.1.6.6.3.2	Simple data types

None.

# *** Next Change ***

6.1.6.6.3.3	Enumeration: ServicePermLevel

The enumeration ServicePermLevel represents the service permission level requested. It shall comply with the provisions defined in table 6.1.6.6.3.3-1.

Table 6.1.6.6.3.3-1: Enumeration ServicePermLevel

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| PREMIUM | Indicates that the service permission level is premium. |  |
| STANDARD | Indicates that the service permission level is standard. |  |
| LIMITED | Indicates that the service permission level is limited. |  |

# *** Next Change ***

6.1.6.6.3.4	Enumeration: MlModelType

The enumeration MlModelType represents the ML model type requested. It shall comply with the provisions defined in table 6.1.6.6.3.4-1.

Table 6.1.6.6.3.4-1: Enumeration MlModelType

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| DECISION_TREES | Indicates that the ML model type is decision trees. |  |
| LINEAR_REGRESSION | Indicates that the ML model type is linear regression. |  |
| NEURAL_NETWORKS | Indicates that the ML model type is neural networks. |  |

# *** Next Change ***

6.1.6.6.3.5	Enumeration: AimlOperationRole

The enumeration AimlOperationRole represents the AIML operation role requested. It shall comply with the provisions defined in table 6.1.6.6.3.5-1.

Table 6.1.6.6.3.5-1: Enumeration AimlOperationRole

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| MODEL_TRAINING | Indicates that the supported AIML operation role is model training. |  |
| MODEL_TRANSFER | Indicates that the supported AIML operation role is model transfer. |  |
| MODEL_INFERENCE | Indicates that the supported AIML operation role is model inference. |  |
| MODEL_OFFLOAD | Indicates that the supported AIML operation role is model offload. |  |
| MODEL_SPLIT | Indicates that the supported AIML operation role is model split. |  |

# *** Next Change ***

6.1.6.6.3.6	Enumeration: MlAppType

The enumeration MlAppType represents the ML application type requested. It shall comply with the provisions defined in table 6.1.6.6.3.6-1.

Table 6.1.6.6.3.6-1: Enumeration MlAppType

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| FL | Indicates that the supported ML application type is Federated Learning. |  |
| TL | Indicates that the supported ML application type is Transfer Learning. |  |
| SL | Indicates that the supported ML application type is Split Learning. |  |

# *** Next Change ***

6.1.6.6.3.7	Enumeration: DataType

The enumeration DataType represents the type of collected data requested. It shall comply with the provisions defined in table 6.1.6.6.3.7-1.

Table 6.1.6.6.3.7-1: Enumeration DataType

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| RAW | Indicates that the type of the collected data is raw. |  |
| PROCESSED | Indicates that the type of the collected data is processed. |  |

# *** Next Change ***

6.1.6.6.3.8	Enumeration: PerformanceCapability

The enumeration PerformanceCapability represents the performance capability requested. It shall comply with the provisions defined in table 6.1.6.6.3.8-1.

Table 6.1.6.6.3.8-1: Enumeration PerformanceCapability

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| GREEN_TASK | Indicates that the performance capability is green task. |  |
| ENERGY_EFFICIENT | Indicates that the performance capability is energy-efficient. |  |
| LOW_COSTS | Indicates that the performance capability is low costs. |  |

# *** Next Change ***

6.1.6.6.3.9	Enumeration: ComputeCapability

The enumeration ComputeCapability represents the compute capability requested. It shall comply with the provisions defined in table 6.1.6.6.3.9-1.

Table 6.1.6.6.3.9-1: Enumeration ComputeCapability

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| LOW | Indicates that the compute capability is low. |  |
| HIGH | Indicates that the compute capability is high. |  |

# *** Next Change ***

6.1.6.6.3.10	Enumeration: ClientVelocity

The enumeration ClientVelocity represents the AIMLE Client velocity requested. It shall comply with the provisions defined in table 6.1.6.6.3.10-1.

Table 6.1.6.6.3.10-1: Enumeration ClientVelocity

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| MOBILE_LOW | Indicates that the AIMLE Client is mobile with low velocity. |  |
| MOBILE_HIGH | Indicates that the AIMLE Client is mobile with high velocity. |  |
| STATIC | Indicates that the AIMLE Client is static. |  |

# *** Next Change ***

##### 6.1.6.6.4	Data types describing alternative data types or combinations of data types

There are no data types describing alternative data types and combinations of data types in this release of the specification.

# *** Next Change ***

##### 6.1.6.6.5	Binary data

There are no binary data defined in this release of the specification.

# *** Next Change ***

#### 6.1.6.7	Error Handling

# *** Next Change ***

##### 6.1.6.7.1	General

For the AIMLES_AIMLEClientDiscovery API, error handling shall be supported as specified in clause 6.7 of 3GPP TS 29.549 [14].

In addition, the requirements in the following clauses are applicable for the AIMLES_AIMLEClientDiscovery API.

# *** Next Change ***

##### 6.1.6.7.2	Protocol Errors

No specific procedures for the AIMLES_AIMLEClientDiscovery API are specified.

# *** Next Change ***

##### 6.1.6.7.3	Application Errors

The application errors defined for AIMLES_AIMLEClientDiscovery API are listed in table 6.1.6.7.3-1.

Table 6.1.6.7.3-1: Application errors

| Application Error | HTTP status code | Description | Applicability |
| --- | --- | --- | --- |

# *** Next Change ***

#### 6.1.6.8	Feature Negotiation

The optional features in table 6.1.6.8-1 are defined for the AIMLES_AIMLEClientDiscovery API. They shall be negotiated using the extensibility mechanism defined in clause 6.8 of 3GPP TS 29.549 [14].

Table 6.1.6.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

# *** Next Change ***

#### 6.1.6.9	Security

The provisions of clause 9 of 3GPP TS 29.549 [14] shall apply for the AIMLES_AIMLEClientDiscovery API.

# *** Next Change ***

##### 6.1.11.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.11.6.1-1 specifies the data types defined for the AIMLES_AssistedMLModelSelection API.

Table 6.1.11.6.1-1: AIMLES_AssistedMLModelSelection API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| AimlProfile | Clause 6.1.11.6.2.4 | Represents the ML model selection operation. |  |
| AssistMLMdlSelNotif | Clause 6.1.11.6.2.3 | Represents the AIMLE Assisted ML Selection notification. |  |
| AssistMLMdlSelSubsc | Clause 6.1.11.6.2.2 | Represents the AIMLE Assisted ML Selection subscription information. |  |
| AssistMLMdlSelSubscPatch | Clause 6.1.11.6.2.7 | Represents the requested modifications to a AIMLE Assisted ML Selection subscription information. |  |
| CandMLMdl | Clause 6.1.11.6.2.9 | Contains the candidate ML model selection information. |  |
| PerformanceMetric | Clause 6.1.11.6.3.3 | Represents the performance metric for training the ML model. |  |
| PerformanceRequirement | Clause 6.1.11.6.2.6 | Represents the performance requirements for ML model selection. |  |
| ReportingInformation | Clause 6.1.11.6.2.8 | Represents the reporting requirements for ML model selection. |  |
| TrainingRequirement | Clause 6.1.11.6.2.5 | Represents the training requirements for ML model selection. |  |

Table 6.1.11.6.1-2 specifies data types re-used by the AIMLES_AssistedMLModelSelection API service.

Table 6.1.11.6.1-2: AIMLES_AssistedMLModelSelection API re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| AimleClientId | Clause 6.1.2.6.3.2 | Represents unique identifier of a AIMLE client. |  |
| ClientDiscCriteria | Clause 6.2.2.6.2.2 | Represents the AIMLE Client selection criteria. |  |
| DataMgmtOp | Clause 6.1.2.6.3.3 | Represents the data management operation type. |  |
| DateTime | 3GPP TS 29.122 [2] | Represents a date and a time. |  |
| DurationSec | 3GPP TS 29.122 [2] | Unsigned integer identifying a period of time in units of seconds. |  |
| MlModelInfo | Clause 6.2.1.6.2.B | Represents the ML model Information. |  |
| MLModelTrainingInfo | Clause 6.2.1.6.2.9 | Represents the ML Model training information. |  |
| Uinteger | 3GPP TS 29.571 [15] | Represents an unsigned Integer. |  |

# *** Next Change ***

# A.5	AIMLES_AIMLEClientDiscovery API

openapi: 3.0.0

info:

title: AIMLES_AIMLEClientDiscovery

description: |

API for AIMLE Client Discovery Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

version: "1.0.0-alpha.1"

externalDocs:

description: >

3GPP TS 29.482 v1.1.0; Artificial Intelligence Machine Learning Enablement

(AIMLE) Services; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.549/

servers:

- url: '{apiRoot}/aimles-disc/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 5.2.4 of 3GPP TS 29.122

security:

- {}

- oAuth2ClientCredentials: []

paths:

/clients:

get:

summary: Retrieve an existing the Individual AIMLE Data Management Assistance Subscription resource.

operationId: GetAimleClients

tags:

- AIMLE Clients (Collection)

parameters:

- name: filt-criteria

in: query

description: >

Represents the AIMLE Client(s) filtering criteria.

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ClientDiscCriteria'

- name: cl-number

in: query

description: >

Represents the required number of the discovered AIMLE Clients.

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

- name: supported-features

in: query

description: >

Contains supported features information, used to negotiate the applicability

of optional features.

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: >

OK. The response body contains the result of the search over the list of AIMLE Clients.

content:

application/json:

schema:

$ref: '#/components/schemas/ClientDiscResp'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

ClientDiscCriteria:

description: Represents the AIMLE Client discovery criteria.

type: object

properties:

serviceReq:

$ref: '#/components/schemas/ServiceRequirement'

mlModelTypes:

type: array

items:

$ref: '#/components/schemas/MlModelType'

minItems: 1

aimlOpers:

type: array

items:

$ref: '#/components/schemas/AimlOperationRole'

minItems: 1

clientAppCap:

$ref: '#/components/schemas/ClientAppCapability'

datasetReq:

$ref: '#/components/schemas/DatasetRequirement'

clientTaskCap:

$ref: '#/components/schemas/ClientTaskCapability'

clientVel:

$ref: '#/components/schemas/ClientVelocity'

location:

$ref: 'TS29122_CommonData.yaml#/components/schemas/LocationArea5G'

clientQosReqs:

type: array

items:

$ref: 'TS29548_SDD_TransmissionQualityMeasurement.yaml#/components/schemas/TransQualMeasCriteriaSet'

minItems: 1

required:

- serviceReq

- aimlOpers

- clientAppCap

ServiceRequirement:

description: Represents the AIMLE service requirements.

type: object

properties:

valServId:

type: string

permLevel:

$ref: '#/components/schemas/ServicePermLevel'

required:

- valServId

ClientAppCapability:

description: Represents the AIMLE Client compute capability type.

type: object

properties:

appType:

$ref: '#/components/schemas/ServicePermLevel'

avail:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ScheduledCommunicationTime'

minItems: 1

dropOfRate:

type: integer

minimum: 0

maximum: 100

required:

- appType

DatasetRequirement:

description: Represents the dataset requirements.

type: object

properties:

avail:

$ref: '#/components/schemas/DatasetCapability'

cap:

$ref: '#/components/schemas/DatasetCapability'

DatasetAvailablity:

description: Represents the dataset availability information.

type: object

properties:

ids:

type: array

items:

type: string

minItems: 1

age:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

size:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

features:

type: array

items:

type: string

minItems: 1

DatasetCapability:

description: Represents the dataset capability information.

type: object

properties:

dataType:

$ref: '#/components/schemas/DataType'

functions:

type: array

items:

type: string

minItems: 1

ClientTaskCapability:

description: Represents the performance capabilities.

type: object

properties:

compCap:

$ref: '#/components/schemas/ComputeCapability'

perfCap:

$ref: '#/components/schemas/PerformanceCapability'

ClientDiscResp:

description: Represents the AIMLE Client discovery response.

type: object

properties:

clients:

type: array

items:

type: string

minItems: 1

suppTasks:

type: array

items:

$ref: '#/components/schemas/ClientTaskCapability'

minItems: 1

suppFeats:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

# SIMPLE DATA TYPES

#

AimleClientId:

description: >

Represents unique identifier of a AIMLE client

type: string

# ENUMERATIONS

ServicePermLevel:

anyOf:

- type: string

enum:

- PREMIUM

- STANDARD

- LIMITED

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration and

is not used to encode content defined in the present version of this API.

description: |

Represents the  service permision level.

Possible values are:

- PREMIUM: Indicates that the service permission level is premium.

- STANDARD: Indicates that the service permission level is standard.

- LIMITED: Indicates that the service permission level is limited.

MlModelType:

anyOf:

- type: string

enum:

- DECISION_TREES

- LINEAR_REGRESSION

- NEURAL_NETWORKS

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration and

is not used to encode content defined in the present version of this API.

description: |

Represents the ML model type.

Possible values are:

- DECISION_TREES: Indicates that the ML model type is decision trees.

- LINEAR_REGRESSION: Indicates that the ML model type is linear regression.

- NEURAL_NETWORKS: Indicates that the ML model type is neural networks.

AimlOperationRole:

anyOf:

- type: string

enum:

- MODEL_TRAINING

- MODEL_TRANSFER

- MODEL_INFERENCE

- MODEL_OFFLOAD

- MODEL_SPLIT

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration and

is not used to encode content defined in the present version of this API.

description: |

Represents the AIMLE operation role.

Possible values are:

- MODEL_TRAINING: Indicates that the supported AIML operation role is model training.

- MODEL_TRANSFER: Indicates that the supported AIML operation role is model transfer.

- MODEL_INFERENCE: Indicates that the supported AIML operation role is model inference.

- MODEL_OFFLOAD: Indicates that the supported AIML operation role is model offload.

- MODEL_SPLIT: Indicates that the supported AIML operation role is model split.

MlAppType:

anyOf:

- type: string

enum:

- FL

- TL

- SL

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration and

is not used to encode content defined in the present version of this API.

description: |

Represents the ML model type.

Possible values are:

- FL: Indicates that the supported ML application type is Federated Learning.

- TL: Indicates that the supported ML application type is Transfer Learning.

- SL: Indicates that the supported ML application type is Split Learning.

DataType:

anyOf:

- type: string

enum:

- RAW

- PROCESSED

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration and

is not used to encode content defined in the present version of this API.

description: |

Represents the type of the collected data.

Possible values are:

- RAW: Indicates that the type of the collected data is raw.

- PROCESSED: Indicates that the type of the collected data is processed.

PerformanceCapability:

anyOf:

- type: string

enum:

- GREEN_TASK

- ENERGY_EFFICIENT

- LOW_COSTS

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration and

is not used to encode content defined in the present version of this API.

description: |

Represents the performance capabilities.

Possible values are:

- GREEN_TASK: Indicates that the performance capability is green task.

- ENERGY_EFFICIENT: Indicates that the performance capability is energy-efficient.

- LOW_COSTS: Indicates that the performance capability is low costs.

ComputeCapability:

anyOf:

- type: string

enum:

- LOW

- HIGH

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration and

is not used to encode content defined in the present version of this API.

description: |

Represents the AIMLE Client compute capability type.

Possible values are:

- LOW: Indicates that the compute capability is low.

- HIGH: Indicates that the compute capability is high.

ClientVelocity:

anyOf:

- type: string

enum:

- MOBILE_LOW

- MOBILE_HIGH

- STATIC

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration and

is not used to encode content defined in the present version of this API.

description: |

Represents the AIMLE Client velocity level.

Possible values are:

- MOBILE_LOW: Indicates that the AIMLE Client is mobile with low velocity.

- MOBILE_HIGH: Indicates that the AIMLE Client is mobile with high velocity.

- STATIC: Indicates that the AIMLE Client is static.

# *** End of Changes ***