3GPP TSG-CT WG3 Meeting #143	C3-254469

Sophia-Antipolis, France, 13 – 17 October 2025	(Revision of C3-254333))

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.517 | CR | 0165 | rev | 1 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support UE altitude information exposure for UAV UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT3 |  |  |  |  |  |  |  |  |  |
| Work item code: | UAS_Ph3 |  |  |  |  |  | Date: |  |  | 2025-09-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Upon SA#109 approved TS 23.288 CR 1496 (S2-2508001) added UE altitude information for UAV UE only in Table 6.21.2-1 with AF as the data source, hence the corresponding changes are needed in this TS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added UE altitude information exposure for UAV UE only. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not support UE altitude information exposure for UAV UE only as required by SA2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1.1, 4.2.2.1, 4.2.2.2, 4.2.4.2, 5.6.1, 5.6.2.5, 5.6.2.6, 5.2.6.31(new), 5.6.3.3, 5.8, A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backwards compatible feature to the OpenAPI file of the following APIs: TS29517_Naf_EventExposure.yaml TS29591_Nnef_EventExposure.yaml |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Additional discussion(if needed):

# Proposed changes:

# *** 1st Change ***

### 4.1.1	Overview

The Application Function Exposure Service, as defined in 3GPP TS 23.502 [3] and 3GPP TS 23.288 [4], is provided by the Application Function (AF). When the UE Application data is collected via the Data Collection AF, the Application Function Exposure Service, as defined in 3GPP TS 26.531 [28], 3GPP TS 26.501 [29], and 3GPP TS 26.512 [30], is provided by the Data Collection AF instantiated in 5GMS AF for the Event Consumer AF instantiated in 5GMS ASP.

This service:

-	allows NF service consumers to subscribe, modify and unsubscribe for application events; and

-	notifies NF service consumers with a corresponding subscription about observed events on the AF.

The types of observed events include:

AF application events exposed by AF:

-	Service Experience information for an application;

-	UE mobility information;

-	UE communication information;

-	Exceptions information;

-	User Data Congestion information;

-	Collective Behaviour information;

-	Dispersion information;

-	Performance Data information;

-	GNSS Assistance Data information;

-	Data volume transfer time information;

-	Application activation information;

-	Signalling information; and

-	UE altitude information.

UE application events exposed via Data Collection AF:

-	Media Streaming QoE metrics;

-	Media Streaming Consumption reports;

-	Media Streaming Network Assistance invocation;

-	Media Streaming Dynamic Policy invocation; and

-	Media Streaming access activity.

When the event to which the NF service consumer has subscribed occurs, the AF reports the requested information to the NF service consumer based on the event reporting information definition requested by the NF service consumer (see 3GPP TS 23.502 [3]).

# *** 2nd Change ***

#### 4.2.2.1	General

This service operation is used by an NF service consumer to subscribe for event notifications on specific event(s), or to modify an existing subscription.

The following are the types of events for which a subscription can be made by the NWDAF, DCCF, MFAF, or NEF as the NF service consumer:

-	Service Experience information for an application;

-	UE mobility information;

-	UE communication information;

-	Exceptions information;

-	User Data Congestion information;

-	Collective Behaviour information;

-	Dispersion information;

-	Performance Data information;

-	End-to-end data volume transfer time information;

-	Activation time information of the application(s);

-	Signalling information;

-	UE altitude information.

The following are the types of events for which a subscription can be made by the NWDAF, DCCF, MFAF, Event Consumer AF, or NEF as the NF service consumer:

-	Media Streaming QoE metrics.

The following are the types of events for which a subscription can be made by the Event Consumer AF or NEF as the NF service consumer:

-	Media Streaming Consumption reports;

-	Media Streaming Network Assistance invocation;

-	Media Streaming Dynamic Policy invocation; and

-	Media Streaming access activity.

The following are the types of events for which a subscription can be made by the LMF or NEF as the NF service consumer:

-	GNSS Assistance Data information

The following procedures using the Naf_EventExposure_Subscribe service operation are supported:

-	creating a new subscription; and

-	modifying an existing subscription.

# *** 3rd Change ***

#### 4.2.2.2	Creating a new subscription

Figure 4.2.2.2-1 illustrates the creation of a subscription.

Figure 4.2.2.2-1: Creation of a subscription

To subscribe to event notifications, the NF service consumer shall send an HTTP POST request to the AF with: "{apiRoot}/naf-eventexposure/<apiVersion>/subscriptions" as request URI as shown in step 1 of figure 4.2.2.2-1, and the AfEventExposureSubsc data structure as request body.

The AfEventExposureSubsc data structure shall include:

-	description of subscribed event information as "eventsSubs" attribute by using one or more EventsSubs data;

-	description of the event reporting information as "eventsRepInfo" attribute;

-	a URI where to receive the requested notifications as "notifUri" attribute;

-	a Notification Correlation Identifier assigned by the NF service consumer for the requested notifications as "notifId" attribute.

The AfEventExposureSubsc data may include:

-	a specific Authorization AS provisioned Data Access Profile Identifier as "dataAccProfId" attribute, if the "DataAccProfileId" feature is supported and the subscribed events including "MS_QOE_METRICS", "MS_CONSUMPTION", "MS_NET_ASSIST_INVOCATION", "MS_DYN_POLICY_INVOCATION", and/or "MS_ACCESS_ACTIVITY".

NOTE 1:	The optional Data Access Profile Identifier provisioned by the Authorization AS procedures are specified in clause 5.8 of 3GPP TS 26.531 [28].

The EventsSubs data shall include:

-	a event to subscribe as a "event" attribute; and

-	event filter information as "eventFilter" attribute associated with the event;

and may include:

-	event-specific reporting information, within the "eventRepInfo" attribute, if the "PerEventRepReq" feature is supported.

The "eventsRepInfo" attribute may include:

-	event notification method (periodic, one time, on event detection) as "notifMethod" attribute;

-	Maximum Number of Reports as "maxReportNbr" attribute;

-	Monitoring Duration as "monDur" attribute;

-	repetition period for periodic reporting as "repPeriod" attribute;

-	immediate reporting indication as "immRep" attribute;

-	sampling ratio as "sampRatio" attribute;

-	partitioning criteria for partitioning the UEs before performing sampling as "partitionCriteria" attribute if the "EneNA" feature is supported;

-	group reporting guard time as "grpRepTime" attribute;

-	a notification flag as "notifFlag" attribute if the "EneNA" feature is supported; and/or

-	notification muting exception instructions within the "notifFlagInstruct" attribute, if the "EnhDataMgmt" feature is supported and the "notifFlag" attribute is provided and set to "DEACTIVATE".

When the "PerEventRepReq" feature is supported, the common events reporting requirements provided within the "eventsRepInfo" attribute shall apply to a subscribed event only when no event-specific reporting requirements are provided within the "eventsSubs" attribute via the "eventRepInfo" attribute of the EventsSubs data structure for this subscribed event, as specified above.

The "eventFilter" shall include:

-	identification of target UE(s) to which the subscription applies via:

1)	identification of individual UE(s) via "gpsis" attribute or "supis" attribute (only applicable to UAV UE if the "UeAltitude" feature is supported and the subscribed event is "UE_ALTITUDE"); or

2)	identification of group(s) of UE(s) via "exterGroupIds" attribute or "interGroupIds" attribute; or

3)	identification of any UE via "anyUeInd" attribute; or

4)	identification of a UE with a specific IP address via the "ueIpAddr" attribute;

NOTE 2:	It is assumed that the AF is provisioned with the list of UE IDs (GPSIs or SUPIs) belonging to an External or Internal Group ID.

Depending on the event type:

-	If the "ServiceExperience" feature is supported and the event is "SVC_EXPERIENCE", the "eventFilter" attribute may provide:

1)	identification of application to which the subscription applies via "appIds" attribute; and

2)	an area of interest via "locArea" attribute.

-	If the "Exceptions" feature is supported and the event is "EXCEPTIONS", the "eventFilter" attribute may provide:

1)	identification of application to which the subscription applies via "appIds" attribute; and

2)	an area of interest via "locArea" attribute;

-	If the "UeCommunication" feature is supported and the event is "UE_COMM", the "eventFilter" attribute may provide:

1)	identification of application to which the subscription applies via "appIds" attribute; and

2)	an area of interest via "locArea" attribute.

-	If the "UeMobility" feature is supported and the event is "UE_MOBILITY", the "eventFilter" attribute may provide:

1)	identification of application to which the subscription applies via "appIds" attribute; and

2)	an area of interest via "locArea" attribute.

-	If the "UserDataCongestion" feature is supported and the event is "USER_DATA_CONGESTION", the "eventFilter" attribute may provide:

1)	identification of application to which the subscription applies via "appIds" attribute; and

2)	an area of interest via "locArea" attribute.

-	If the "PerformanceData" feature is supported and the event is "PERF_DATA", the "eventFilter" attribute may provide:

1)	identification of application to which the subscription applies via "appIds" attribute; and

2)	an area of interest via "locArea" attribute.

-	If the "CollectiveBehaviour" feature is supported and the event is "COLLECTIVE_BEHAVIOUR", the "eventFilter" attribute may provide:

1)	collective attributes information via the "collAttrs" attribute;

2)	an area of interest via "locArea" attribute; and

3)	identification of application to which the subscription applies via "appIds" attribute.

-	If the "Dispersion" feature is supported and the event is "DISPERSION", the "eventFilter" attribute may provide:

1)	identification of application to which the subscription applies via "appIds" attribute; and

2)	an area of interest via "locArea" attribute.

-	If the "MSQoeMetrics" feature is supported and the event is "MS_QOE_METRICS", the "eventFilter" attribute may provide:

1)	identification of application to which the subscription applies via "appIds" attribute; and

2)	an area of interest via "locArea" attribute.

-	If the "MSConsumption" feature is supported and the event is "MS_CONSUMPTION", the "eventFilter" attribute may provide:

1)	identification of application to which the subscription applies via "appIds" attribute; and

2)	an area of interest via "locArea" attribute.

-	If the "MSNetAssInvocation" feature is supported and the event is "MS_NET_ASSIST_INVOCATION", the "eventFilter" attribute may provide:

1)	identification of application to which the subscription applies via "appIds" attribute; and

2)	an area of interest via "locArea" attribute.

-	If the "MSDynPolicyInvocation" feature is supported and the event is "MS_DYN_POLICY_INVOCATION", the "eventFilter" attribute may provide:

1)	identification of application to which the subscription applies via "appIds" attribute; and

2)	an area of interest via "locArea" attribute.

-	If the "MSAccessActivity" feature is supported and the event is "MS_ACCESS_ACTIVITY", the "eventFilter" attribute may provide:

1)	identification of application to which the subscription applies via "appIds" attribute; and

2)	an area of interest via "locArea" attribute.

-	If the "GNSSAssistData" feature is supported and the subscribed event is "GNSS_ASSISTANCE_DATA", the "eventFilter" attribute may include:

1)	an area of interest within the "locArea" attribute.

-	If the "DataVolTransferTime" feature is supported and the event is "DATA_VOLUME_TRANSFER_TIME", the "eventFilter" attribute may provide:

1)	identification of application to which the subscription applies via "appIds" attribute; and

2)	an area of interest via "locArea" attribute.

-	If the feature "AppActiveTime" is supported and the event is "APP_ACTIVE_TIME", the "eventFilter" attribute may provide:

1)	identification of application(s) to which the subscription applies via "appIds" attribute.

-	If the feature "SignallingInfo" is supported and the event is "SIGNALLING_INFO", the "eventFilter" attribute may provide:

1)	identification of time windows for which signalling information shall be reported within the "tws" attribute.

If the AF cannot successfully fulfil the received HTTP POST request due to an internal error or an error in the HTTP POST request, the AF shall send the HTTP error response as specified in clause 5.7.

Upon successful reception of the HTTP POST request with "{apiRoot}/naf-eventexposure/<apiVersion>/subscriptions" as request URI and AfEventExposureSubsc data structure as request body, the AF shall create a new "Individual Application Event Subscription" resource, store the subscription and send an HTTP "201 Created" response as shown in step 2 of figure 4.2.2.2-1, containing:

-	a Location header field; and

-	an AfEventExposureSubsc data type in the content.

The Location header field shall contain the URI of the created individual application session context resource i.e. "{apiRoot}/naf-eventexposure/<apiVersion>/subscriptions/{subscriptionId}".

The AfEventExposureSubsc data type content shall contain the representation of the created "Individual Application Event Subscription".

When the "monDur" attribute is included in the response by the AF, it represents AF selected expiry time that is equal or less than the expiry time received in the request.

When the "immRep" attribute is included and sets to "true" in the subscription and the subscribed events are available, the AF shall include the reports of the events subscribed, if available, in the HTTP POST response.

When the sampling ratio as, "sampRatio" attribute, is included in the subscription without a "partitionCriteria" attribute, the AF shall select a random subset of UEs among the target UEs according to the sampling ratio and only report the event(s) related to the selected subset of UEs. If the "partitionCriteria" attribute is additionally included, then the AF shall first partition the UEs according to the value of the "partitionCriteria" attribute and then select a random subset of UEs from each partition according to the sampling ratio and only report the event(s) related to the selected subsets of UEs.

When the group reporting guard time as the "grpRepTime" attribute is included in the subscription, the AF shall accumulate all the event reports for the target UEs until the group reporting guard time expires. Then the AF shall notify the NF service consumer using the Naf_EventExposure_Notify service operation, as described in clause 4.2.4.2.

When the "notifFlag" attribute is included and set to "DEACTIVATE" in the request, the AF shall mute the event notification and store the available events until the NF service consumer requests to retrieve them by setting the "notifFlag" attribute to "RETRIEVAL" or until a muting exception occurs (e.g. full buffer). When a muting exception occurs, the AF may consider the contents of the "notifFlagInstruct" attribute (if provided) and/or local configuration to determine its actions.

If the "EnhDataMgmt" feature is supported and the AF accepts the muting instructions provided in the "notifFlag" and/or the "notifFlagInstruct" attributes, it may indicate the applied muting notification settings within the "mutingSetting" attribute in the response. If the AF does not accept the muting instructions provided in the "notifFlag" and/or the "notifFlagInstruct" attributes, it shall send an HTTP "403 Forbidden" error response including the "cause" attribute set to "MUTING_INSTR_NOT_ACCEPTED".

# *** 4th Change ***

#### 4.2.4.2	Notification about subscribed events

Figure 4.2.4.2-1 illustrates the notification about subscribed events.

Figure 4.2.4.2-1: Notification about subscribed events

If the AF observes application related event(s) for which an NF service consumer has subscribed, the AF shall send an HTTP POST request as shown in step 1 of figure 4.2.4.2-1, with the "{notifUri}" as request URI containing the value previously provided by the NF service consumer within the corresponding subscription, and the AfEventExposureNotif data structure.

The AfEventExposureNotif data structure shall include:

a)	the notification correlation ID provided by the NF service consumer during the subscription as "notifId" attribute; and

b)	information about the observed event(s) within the "eventNotifs" attribute that shall contain for each observed event an AfEventNotification data structure that shall include:

1)	the application related event as "event" attribute;

2)	the time at which the event was observed encoded as "timeStamp" attribute;

3)	if the "event" attribute is "SVC_EXPERIENCE":

-	service experience information about the application involved in the reported event in the "svcExprcInfos" attribute;

4)	if the "event" attribute is "UE_MOBILITY":

-	UE mobility information associated with the application as "ueMobilityInfos" attribute;

5)	if the "event" attribute is "UE_COMM":

-	application communication information associated with the application as "ueCommInfos" attribute;

6)	if the "event" attribute is "EXCEPTIONS":

-	exceptions information associated with a service flow as "excepInfos" attribute;

7)	if the "event" attribute is "COLLECTIVE_BEHAVIOUR":

-	collective beahviour information associated with the UEs and its applications as "collBhvrInfs" attribute;

8)	if the "event" attribute is "PERF_DATA":

-	performance data information associated with the application as "perfDataInfos" attribute;

9)	if the "event" attribute is "USER_DATA_CONGESTION":

-	user data congestion information collected for an AF application as "congestionInfos" attribute; and

10)	if the "event" attribute is "DISPERSION":

-	UE dispersion information collected for an AF application as "dispersionInfos" attribute.

11)	if the "event" attribute is "MS_QOE_METRICS":

-	Media Streaming QoE metrics information collected for an UE application via the Data Collection AF as "msQoeMetrInfos" attribute. This attribute is deprecated; the attribute "msQoeMetrics" should be used instead.

-	if the "MSEventExposure" feature is supported, the Media Streaming QoE metrics information collected for an UE application via the Data Collection AF as "msQoeMetrics" attribute.

12)	if the "event" attribute is "MS_CONSUMPTION":

-	Media Streaming Consumption reports collected for an UE application via the Data Collection AF as "msConsumpInfos" attribute. This attribute is deprecated; the attribute "msConsumpRpts" should be used instead.

-	if the "MSEventExposure" feature is supported, the Media Streaming Consumption reports collected for an UE application via the Data Collection AF as "msConsumpRpts" attribute.

13)	if the "event" attribute is "MS_NET_ASSIST_INVOCATION":

-	Media Streaming Network Assistance invocation collected for an UE application via the Data Collection AF as "msNetAssInvInfos" attribute. This attribute is deprecated; the attribute "msNetAssistInvs" should be used instead.

-	if the "MSEventExposure" feature is supported, the Media Streaming Network Assistance invocation collected for an UE application via the Data Collection AF as "msNetAssistInvs" attribute.

14)	if the "event" attribute is "MS_DYN_POLICY_INVOCATION":

-	Media Streaming Dynamic Policy invocation collected for an UE application via the Data Collection AF as "msDynPlyInvInfos" attribute. This attribute is deprecated; the attribute "msDynPlyInvs" should be used instead.

-	if the "MSEventExposure" feature is supported, the Media Streaming Dynamic Policy invocation collected for an UE application via the Data Collection AF as "msDynPlyInvs" attribute.

15)	if the "event" attribute is "MS_ACCESS_ACTIVITY":

-	Media Streaming access activity collected for an UE application via the Data Collection AF as "msAccActInfos" attribute. This attribute is deprecated; the attribute "msAccesses" should be used instead.

-	if the "MSEventExposure" feature is supported, the Media Streaming access activity collected for an UE application via the Data Collection AF as "msAccesses" attribute.

16)	if the "event" attribute is "GNSS_ASSISTANCE_DATA":

-	GNSS Assistance Data information within the "gnssAssistDataInfo" attribute;

17)	if the "event" attribute is "DATA_VOLUME_TRANSFER_TIME":

-	data volume transfer information associated with the application as "datVolTransTimeInfos" attribute.

18)	if the "event" attribute is "APP_ACTIVE_TIME":

-	activation time information of the application(s) as "appActiveTimeInfos" attribute.

19)	if the "event" attribute is "SIGNALLING_INFO":

-	signalling information within the "sigInfos" attribute.

20)	if the "event" attribute is "UE_ALTITUDE":

-	UE altitude information within the "ueAltitudeInfos" attribute.

If the NF service consumer cannot successfully fulfil the received HTTP POST request due to an internal error or an error in the HTTP POST request, the NF service consumer shall send an HTTP error response as specified in clause 5.7.

If the feature "ES3XX" is supported, and the NF service consumer determines the received HTTP POST request needs to be redirected, the NF service consumer shall send an HTTP redirect response as specified in clause 6.10.9 of 3GPP TS 29.500 [5].

Upon successful reception of the HTTP POST request with "{notifUri}" as request URI and AfEventExposureNotif data structure as request body, the NF service consumer shall send a "204 No Content" HTTP response, as shown in step 2 of figure 4.2.4.2-1.

# *** 5th Change ***

### 5.6.1	General

This clause specifies the application data model supported by the API.

Table 5.6.1-1 specifies the data types defined for the Naf_EventExposure service based interface protocol.

Table 5.6.1-1: Naf_EventExposure specific Data Types

| Data type |  | Section defined |  | Description |  | Applicability |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ActiveTimeInfo |  | 5.6.2.30 |  | Contains the activation time information. |  | AppActiveTime |  |
| AddrFqdn |  | 5.6.2.18 |  | IP address and/or FQDN. |  | PerformanceData ServiceExperienceExt DataVolTransferTime |  |
| AfEvent |  | 5.6.3.3 |  | Represents Application Events. |  |  |  |
| AfEventExposureSubsc |  | 5.6.2.2 |  | Represents an Individual Application Event Subscription resource. |  |  |  |
| AfEventExposureNotif |  | 5.6.2.3 |  | Describes notifications about application event that occurred in an Individual Application Event Subscription resource. |  |  |  |
| AfEventNotification |  | 5.6.2.6 |  | Represents information related to an event to be reported. |  |  |  |
| AppActiveTimeInfo |  | 5.6.2.29 |  | Contains the activation time information of the application(s). |  | AppActiveTime |  |
| CollectiveBehaviourFilter |  | 5.6.2.19 |  | Contains the parameter type and value pair to express the collective behaviour event filters. |  | CollectiveBehaviour |  |
| CollectiveBehaviourFilterType |  | 5.6.3.4 |  |  |  | CollectiveBehaviour |  |
| CollectiveBehaviourInfo |  | 5.6.2.20 |  | Contains the collective behaviour analytics information. |  | CollectiveBehaviour |  |
| CommunicationCollection |  | 5.6.2.13 |  | Contains communication information. |  | UeCommunication |  |
| DataProcessingType |  | 5.6.3.5 |  | Represents a type of data processing. |  | ExtEventFilters |  |
| DatVolTransTimeCollection |  | 5.6.2.28 |  | Contains data volume transfer time information. |  | DataVolTransferTime |  |
| DispersionCollection |  | 5.6.2.21 |  | Contains Dispersion information collected. |  | Dispersion |  |
| EventFilter |  | 5.6.2.5 |  | Represents event filter information. |  |  |  |
| EventsSubs |  | 5.6.2.4 |  | Represents an event to be subscribed and the related event filter information. |  |  |  |
| ExceptionInfo |  | 5.6.2.14 |  | Describes the exceptions information provided by AF. |  | Exceptions |  |
| MSAccessActivityCollection |  | 5.6.2.27 |  | Represents the Media Streaming access activities of UE Application collected via Data Collection AF. |  | MSAccessActivity |  |
| MsConsumptionCollection |  | 5.6.2.24 |  | Represents the Media Streaming Consumption reports of UE Application collected via Data Collection AF. |  | MSConsumption |  |
| MsDynPolicyInvocationCollection |  | 5.6.2.26 |  | Represents the Media Streaming Dynamic Policy invocation of UE Application collected via Data Collection AF. |  | MSDynPolicyInvocation |  |
| MsQoeMetricsCollection |  | 5.6.2.23 |  | Represents the Media Streaming QoE Metrics of UE Application collected via Data Collection AF. |  | MSQoeMetrics |  |
| MsNetAssInvocationCollection |  | 5.6.2.25 |  | Represents the Media Streaming Network Assistance invocation of UE Application collected via Data Collection AF. |  | MSNetAssInvocation |  |
| PerformanceData |  | 5.6.2.17 |  | Indicates the performance data. |  | PerformanceData |  |
| PerformanceDataCollection |  | 5.6.2.16 |  | Represents the performance data information collected for an AF application. |  | PerformanceData |  |
| PerUeAttribute |  | 5.6.2.22 |  | UE application data collected per UE. |  | CollectiveBehaviour |  |
| RelativeDirection |  | 5.6.3.6 |  | Contains the heading of the UE movement with respect to another UE. |  | RelativeProximity |  |
| ServiceExperienceInfoPerApp |  | 5.6.2.7 |  | Contains service experience associated with the application. |  | ServiceExperience |  |
| ServiceExperienceInfoPerFlow |  | 5.6.2.8 |  | Contains service experience associated with the service flow. |  | ServiceExperience |  |
| SvcExperience |  | 5.6.2.9 |  | Contains a mean opinion score with the customized range. |  | ServiceExperience |  |
| UeAltitudeInfo |  | 5.6.2.31 |  | Contains UE altitude information. |  | UeAltitude |  |
| UeCommunicationCollection |  | 5.6.2.11 |  | Contains UE communication information associated with the application. |  | UeCommunication |  |
| UeMobilityCollection |  | 5.6.2.10 |  | Contains UE mobility information associated with the application. |  | UeMobility |  |
| UeTrajectoryCollection |  | 5.6.2.12 |  | Contains UE trajectory information associated with the application. |  | UeMobility |  |
| UserDataCongestionCollection |  | 5.6.2.15 |  | Contains User Data Congestion Analytics related information collected. |  | UserDataCongestion |  |

Table 5.6.1-2 specifies data types re-used by the Naf_EventExposure service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Naf_EventExposure service based interface.

Table 5.6.1-2: Naf_EventExposure re-used Data Types

| Data type |  | Reference |  | Comments |  | Applicability |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ApplicationId |  | 3GPP TS 29.571 [13] |  | Application Identifier. |  |  |  |
| Altitude |  | 3GPP TS 29.572 [33] |  | Represents an altitude. |  | UeAltitude |  |
| BitRate |  | 3GPP TS 29.571 [13] |  | String representing a bit rate that shall be formatted as follows: pattern: "^\d+(\.\d+)? (bps¦Kbps¦Mbps¦Gbps¦Tbps)$" Examples: "125 Mbps", "0.125 Gbps", "125000 Kbps". |  | UserDataCongestion CollectiveBehaviour |  |
| ConsumptionReportingUnitsCollection |  | 3GPP TS 26.512 [30] |  | Represents the collection of Media Streaming Consumption event records. |  | MSEventExposure |  |
| CpParameterSet |  | 3GPP TS 29.122 [17] |  | The Expected UE Behaviour parameters. |  | UeCommunicationExt_eNA |  |
| DateTime |  | 3GPP TS 29.571 [13] |  | Contains a date and a time. |  |  |  |
| Dnai |  | 3GPP TS 29.571 [13] |  | Identifies a DNAI. |  |  |  |
| Direction |  | 3GPP TS 29.520 [19] |  | Heading directions of the UE flow in the target area. |  | RelativeProximity |  |
| DurationSec |  | 3GPP TS 29.571 [13] |  | Indicates a period of time in units of seconds. |  | Dispersion |  |
| DynamicPolicy |  | 3GPP TS 26.512 [30] |  | Represents the Media Streaming Dynamic Policy. |  | MSDynPolicyInvocation |  |
| DynamicPolicyInvocationsCollection |  | 3GPP TS 26.512 [30] |  | Represents the collection of Media Streaming Dynamic Policy invocation event records. |  | MSEventExposure |  |
| EthFlowDescription |  | 3GPP TS 29.514 [18] |  | Defines a packet filter for an Ethernet flow. |  |  |  |
| Exception |  | 3GPP TS 29.520 [19] |  | Describes the Exception information. |  |  |  |
| ExtGroupId |  | 3GPP TS 29.503 [27] |  | External Group Identifier for a user group. |  |  |  |
| Float |  | 3GPP TS 29.571 [13] |  | Number with format "float" as defined in OpenAPI Specification [8]. |  |  |  |
| FlowDescription |  | 3GPP TS 29.514 [18] |  | Only IP 5-tuple (protocol, source and destination IP address, Source and destination port) is applicable. |  | Dispersion |  |
| FlowInfo |  | 3GPP TS 29.122 [17] |  | Represents flow information. |  |  |  |
| GNSSAssistDataInfo |  | 3GPP TS 29.591 [31] |  | Represents GNSS Assistance Data information. |  | GNSSAssistData |  |
| Gpsi |  | 3GPP TS 29.571 [13] |  | Identifies a GPSI. |  |  |  |
| GroupId |  | 3GPP TS 29.571 [13] |  | Contains a Group identifier. |  |  |  |
| IpAddr |  | 3GPP TS 29.571 [13] |  | Identifies IP address. |  | Dispersion EnPerformanceData |  |
| LocationArea5G |  | 3GPP TS 29.122 [17] |  | Represents a user location area when the UE is attached to 5G. |  |  |  |
| MediaStreamingAccessesCollection |  | 3GPP TS 26.512 [30] |  | Represents the collection of Media Streaming access event records. |  | MSEventExposure |  |
| MediaStreamingAccessRecord |  | 3GPP TS 26.512 [30] |  | Represents the Media Streaming Access activity record. |  | MSAccessActivity |  |
| NetworkAssistanceInvocationsCollection |  | 3GPP TS 26.512 [30] |  | Represents the collection of Media Streaming Network Assistance invocation event records. |  | MSEventExposure |  |
| NetworkAssistanceSession |  | 3GPP TS 26.512 [30] |  | Represents the Media Streaming Network Assistance Session Recommendation. |  | MSNetAssInvocation |  |
| NfSignallingInfo |  | 3GPP TS 29.571 [13] |  | Contains NF signalling information. |  | SignallingInfo |  |
| PacketDelBudget |  | 3GPP TS 29.571 [13] |  | Indicates average Packet Delay. |  | PerformanceData |  |
| PacketLossRate |  | 3GPP TS 29.571 [13] |  | Indicates average Loss Rate. |  | PerformanceData |  |
| QoEMetricsCollection |  | 3GPP TS 26.512 [30] |  | Represents the collection of Media Streaming QoE metrics event records. |  | MSEventExposure |  |
| RedirectResponse |  | 3GPP TS 29.571 [13] |  | Contains redirection related information. |  | ES3XX |  |
| ReportingInformation |  | 3GPP TS 29.523 [12] |  | Represents the requirements of reporting the subscription. |  |  |  |
| Supi |  | 3GPP TS 29.571 [13] |  | Contains a SUPI. |  |  |  |
| SupportedFeatures |  | 3GPP TS 29.571 [13] |  | Indicates the features supported. |  |  |  |
| TimeWindow |  | 3GPP TS 29.122 [17] |  | Represents a time window identified by a start time and a stop time. |  |  |  |
| Uinteger |  | 3GPP TS 29.571 [13] |  | Unsigned integer. |  | ServiceExperienceExt2_eNA RelativeProximity AppActiveTime |  |
| Uri |  | 3GPP TS 29.571 [13] |  | Contains a URI. |  |  |  |
| UsageThreshold |  | 3GPP TS 29.122 [17] |  | data volume during the period |  | Dispersion |  |
| VelocityEstimate |  | 3GPP TS 29.572 [33] |  | Velocity estimate. |  | EnhUeDataExposure |  |
| Volume |  | 3GPP TS 29.122 [17] |  | Unsigned integer identifying a volume in units of bytes. |  |  |  |

# *** 6th Change ***

#### 5.6.2.5	Type EventFilter

Table 5.6.2.5-1: Definition of type EventFilter

| Attribute name |  | Data type |  | P |  | Cardinality |  | Description |  | Applicability (NOTE 4) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| gpsis |  | array(Gpsi) |  | O |  | 1..N |  | Each element represents external UE identifier. (NOTE 1, NOTE 2, NOTE 7, NOTE 8) |  |  |  |
| supis |  | array(Supi) |  | O |  | 1..N |  | Each element represents a SUPI identifying a UE (NOTE 1, NOTE 2, NOTE 7, NOTE 8) |  |  |  |
| exterGroupIds |  | array(ExtGroupId) |  | O |  | 1..N |  | Each element represents a group of UEs identified by an External Group Identifier. (NOTE 1, NOTE 2, NOTE 7) |  |  |  |
| interGroupIds |  | array(GroupId) |  | O |  | 1..N |  | Each element represents a group of UEs identified by an Internal Group Identifier. (NOTE 1, NOTE 2, NOTE 7) |  |  |  |
| anyUeInd |  | boolean |  | O |  | 0..1 |  | Identifies whether the request applies to any UE. This attribute shall set to "true" if applicable for any UE, otherwise, set to "false". May only be present and sets to "true" if "AfEvent" sets to "SVC_EXPERIENCE", "EXCEPTIONS", "GNSS_ASSISTANCE_DATA", "USER_DATA_CONGESTION", or "SIGNALLING_INFO". (NOTE 2, NOTE 7) |  | ServiceExperience Exceptions UserDataCongestion GNSSAssistData SignallingInfo |  |
| ueIpAddr |  | IpAddr |  | O |  | 0..1 |  | Identifies the UE IP address. (NOTE 2) |  | EnPerformanceData |  |
| appIds |  | array(ApplicationId) |  | O |  | 1..N |  | Each element indicates an application identifier. If absent, the EventFilter data applies to any application (i.e. all applications). (NOTE 3) |  | ServiceExperience UeMobility UeCommunication Exceptions UserDataCongestion PerformanceData Dispersion CollectiveBehaviour MSQoeMetrics MSConsumption MSNetAssInvocation MSDynPolicyInvocation MSAccessActivity DataVolTransferTime AppActiveTime |  |
| locArea |  | LocationArea5G |  | O |  | 0..1 |  | Represents area of interest. (NOTE 5) |  | ServiceExperience UeMobility UeCommunication Exceptions UserDataCongestion PerformanceData Dispersion CollectiveBehaviour MSQoeMetrics MSConsumption MSNetAssInvocation MSDynPolicyInvocation MSAccessActivity DataVolTransferTime GNSSAssistData |  |
| collAttrs |  | array(CollectiveBehaviourFilter) |  | O |  | 1..N |  | Each element indicates a collective attribute parameter type and value. This attribute may be included when the subscribed event is "COLLECTIVE_BEHAVIOUR". |  | CollectiveBehaviour |  |
| exceptionReqs |  | array(Exception) |  | O |  | 1..N |  | Each element indicates an Exception Id with associated threshold. This attribute may be included when the subscribed event is "EXCEPTIONS". (NOTE 6) |  | EnPerformanceData |  |
| tws |  | array(TimeWindow) |  | O |  | 1..N |  | Contains the time windows for which the NF service consumer is requesting to receive signalling information. It may only be provided if the "SIGNALLING_INFO" event is subscribed. If omitted, the AF determines the time windows for which the signalling information is reported based on local configuration. |  | SignallingInfo |  |
| NOTE 1: For untrusted AF, only gpsis and exterGroupIds are applicable. For trusted AF, only supis and interGroupIds are applicable. NOTE 2: For an applicable feature, only one attribute identifying the target UE shall be provided. NOTE 3: For event "UE_COMM", "UE_MOBILITY", "EXCEPTIONS" and "PERF_DATA", the "appIds" attribute, if present, shall include only one element. NOTE 4: Properties marked with a feature as defined in clause 5.8 are applicable as described in clause 6.6 of 3GPP TS 29.500 [5]. If no features are indicated, the related property applies for all the features. NOTE 5: The "nwAreaInfo" attribute within the LocationArea5G data type is only applicable for a trusted AF. In addition, for the "SVC_EXPERIENCE" or "GNSS_ASSISTANCE_DATA" event, only the "tais" attribute within the NetworkAreaInfo data type encoding the "nwAreaInfo" attribute is applicable for the trusted AF. NOTE 6: Only "excepId" and "excepLevel" attributes within the Exception data type are applicable to this attribute. NOTE 7: When the "GNSSAssistData" feature is supported, only the "anyUeInd" attribute is applicable (and shall be present and set to "true") for the "GNSS_ASSISTANCE_DATA" event in this release of the specification. NOTE 8: When the "UeAltitude" feature is supported, this attribute is only applicable to the UAV UE(s). |  |  |  |  |  |  |  |  |  |  |  |

# *** 7th Change ***

#### 5.6.2.6	Type AfEventNotification

Table 5.6.2.6-1: Definition of type AfEventNotification

| Attribute name |  | Data type |  |  | P | Cardinality |  | Description |  | Applicability |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| event |  | AfEvent |  |  | M | 1 |  | Represents the reported application related event. |  |  |  |
| timeStamp |  | DateTime |  |  | M | 1 |  | Time at which the event is observed. |  |  |  |
| svcExprcInfos |  | array(ServiceExperienceInfoPerApp) |  |  | C | 1..N |  | Contains the service experience information. Shall be present if the "event" attribute sets to "SVC_EXPERIENCE". |  | ServiceExperience |  |
| ueMobilityInfos |  | array(UeMobilityCollection) |  |  | C | 1..N |  | Contains the UE mobility information. Shall be present if the "event" attribute sets to "UE_MOBILITY". |  | UeMobility |  |
| ueCommInfos |  | array(UeCommunicationCollection) |  |  | C | 1..N |  | Contains the application communication information. Shall be present if the "event" attribute sets to "UE_COMM". |  | UeCommunication |  |
| excepInfos |  | array(ExceptionInfo) |  |  | C | 1..N |  | Each element represents the exception information for a service flow. Shall be present if the "event" attribute sets to "EXCEPTIONS". |  | Exceptions |  |
| congestionInfos |  | array(UserDataCongestionCollection) |  |  | C | 1..N |  | Each element represents the user data congestion information collected for an AF application. Shall be present if the "event" attribute sets to "USER_DATA_CONGESTION". |  | UserDataCongestion |  |
| perfDataInfos |  | array(PerformanceDataCollection) |  |  | C | 1..N |  | Each element represents the performance data information collected for an AF application. Shall be present if the "event" attribute sets to "PERF_DATA". |  | PerformanceData |  |
| collBhvrInfs |  | array(CollectiveBehaviourInfo) |  |  | C | 1..N |  | Each element represents the collective behaviour information related to a set of UEs, applications. Shall be present if the "event" attribute sets to "COLLECTIVE_BEHAVIOUR". |  | CollectiveBehaviour |  |
| dispersionInfos |  | array(DispersionCollection) |  |  | C | 1..N |  | Each element represents the UE dispersion information collected for an AF application. Shall be present if the "event" attribute sets to "DISPERSION". |  | Dispersion |  |
| msQoeMetrInfos |  | array(MsQoeMetricsCollection) |  |  | C | 1..N |  | Each element represents the Media Streaming QoE metrics information collected for an UE application via the Data Collection AF. Shall be present if the "event" attribute sets to "MS_QOE_METRICS". This attribute is deprecated; the attribute "msQoeMetrics" should be used instead. |  | MSQoeMetrics |  |
| msQoeMetrics |  | array(QoEMetricsCollection) |  | C | 1..N |  |  | Each element represents the Media Streaming QoE metrics event record. Shall be present if the "event" attribute sets to "MS_QOE_METRICS". This attribute deprecates "msQoeMetrInfos" attribute. |  | MSEventExposure |  |
| msConsumpInfos |  | array(MsConsumptionCollection) |  |  | C | 1..N |  | Each element represents the Media Streaming Consumption information collected for an UE application via the Data Collection AF. Shall be present if the "event" attribute sets to "MS_CONSUMPTION". This attribute is deprecated; the attribute "msConsumpRpts" should be used instead. |  | MSConsumption |  |
| msConsumpRpts |  | array(ConsumptionReportingUnitsCollection) |  | C | 1..N |  |  | Each element represents the Media Streaming Consumption event record. Shall be present if the "event" attribute sets to "MS_CONSUMPTION". This attribute deprecates "msConsumpInfos" attribute. |  | MSEventExposure |  |
| msNetAssInvInfos |  | array(MsNetAssInvocationCollection) |  |  | C | 1..N |  | Each element represents the Media Streaming Network Assistance invocation information collected for an UE application via the Data Collection AF. Shall be present if the "event" attribute sets to "MS_NET_ASSIST_INVOCATION". This attribute is deprecated; the attribute "msNetAssistInvs" should be used instead. |  | MSNetAssInvocation |  |
| msNetAssistInvs |  | array(NetworkAssistanceInvocationsCollection) |  | C | 1..N |  |  | Each element represents the Media Streaming Network Assistance invocation event record. Shall be present if the "event" attribute sets to "MS_NET_ASSIST_INVOCATION". This attribute deprecates "msNetAssInvInfos" attribute. |  | MSEventExposure |  |
| msDynPlyInvInfos |  | array(MsDynPolicyInvocationCollection) |  |  | C | 1..N |  | Each element represents the Media Streaming Dynamic Policy invocation information collected for an UE application via the Data Collection AF. Shall be present if the "event" attribute sets to "MS_DYN_POLICY_INVOCATION". This attribute is deprecated; the attribute "msDynPlyInvs" should be used instead. |  | MSDynPolicyInvocation |  |
| msDynPlyInvs |  | array(DynamicPolicyInvocationsCollection) |  | C | 1..N |  |  | Each element represents the Media Streaming Dynamic Policy invocation event record. Shall be present if the "event" attribute sets to "MS_DYN_POLICY_INVOCATION". This attribute deprecates "msDynPlyInvInfos" attribute. |  | MSEventExposure |  |
| msAccActInfos |  | array(MSAccessActivityCollection) |  |  | C | 1..N |  | Each element represents the Media Streaming access activity collected for an UE application via the Data Collection AF. Shall be present if the "event" attribute sets to "MS_ACCESS_ACTIVITY". This attribute is deprecated; the attribute "msAccesses" should be used instead. |  | MSAccessActivity |  |
| msAccesses |  | array(MediaStreamingAccessesCollection) |  | C | 1..N |  |  | Each element represents the Media Streaming access event record. Shall be present if the "event" attribute sets to "MS_ACCESS_ACTIVITY". This attribute deprecates "msAccActInfos" attribute. |  | MSEventExposure |  |
| gnssAssistDataInfo |  | GNSSAssistDataInfo |  |  | C | 0..1 |  | Represents the GNSS Assistance data information. This attribute shall be present only if the "event" attribute is set to "GNSS_ASSISTANCE_DATA". |  | GNSSAssistData |  |
| datVolTransTimeInfos |  | array(DatVolTransTimeCollection) |  |  | C | 1..N |  | Each element represents the data volume transfer time information related to a UE. Shall be present if the "event" attribute sets to "DATA_VOLUME_TRANSFER_TIME". |  | DataVolTransferTime |  |
| appActiveTimeInfos |  | array(AppActiveTimeInfo) |  |  | C | 1..N |  | Each element represents activation time information of the application. Shall be present if the "event" attribute sets to "APP_ACTIVE_TIME". |  | AppActiveTime |  |
| sigInfos |  | array(NfSignallingInfo) |  |  | C | 1..N |  | Contains signalling information. Shall be present if the "event" attribute sets to "SIGNALLING_INFO". |  | SignallingInfo |  |
| ueAltitudeInfos |  | array(UeAltitudeInfo) |  |  | C | 1..N |  | Contains UE altitude information. Shall be present if the "event" attribute sets to "UE_ALTITUDE". |  | UeAltitude |  |

# *** 8th Change ***

#### 5.6.2.31	Type UeAltitudeInfo

Table 5.6.2.31-1: Definition of type UeAltitudeInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| altitude | Altitude | M | 1 | Contains the altitude of the UAV UE. |  |
| timeStamp | DateTime | O | 0..1 | Contains the timestamp at which the altitude of the UAV UE provided within the "altitude" attribute was measured. |  |

# *** 9th Change ***

#### 5.6.3.3	Enumeration: AfEvent

The enumeration AfEvent represents the application events that can be subscribed/notified. It shall comply with the provisions defined in table 5.6.3.3-1.

Table 5.6.3.3-1: Enumeration AfEvent

| Enumeration value |  | Description |  | Applicability |  |
| --- | --- | --- | --- | --- | --- |
| SVC_EXPERIENCE |  | Indicates that the event subscribed/notified is service experience information for an application. |  | ServiceExperience |  |
| UE_MOBILITY |  | Indicates that the event subscribed/notified is UE mobility information. |  | UeMobility |  |
| UE_COMM |  | Indicates that the event subscribed/notified is UE communication information. |  | UeCommunication |  |
| EXCEPTIONS |  | Indicates that the event subscribed/notified is exceptions information. |  | Exceptions |  |
| USER_DATA_CONGESTION |  | Indicates that the event subscribed/notified is user data congestion analytics related information. |  | UserDataCongestion |  |
| PERF_DATA |  | Indicates that the event subscribed/notified is performance data information. |  | PerformanceData |  |
| COLLECTIVE_BEHAVIOUR |  | Indicates that the event subscribed/notified is collective behaviour information. If the "RelativeProximity" feature is supported, this event is also applicable for relative proximity data collection. |  | CollectiveBehaviour RelativeProximity |  |
| DISPERSION |  | Indicates that the event subscribed/notified is dispersion information. |  | Dispersion |  |
| MS_QOE_METRICS |  | Indicates that the event subscribed/notified is Media Streaming QoE metrics. |  | MSQoeMetrics |  |
| MS_CONSUMPTION |  | Indicates that the event subscribed/notified is Media Streaming Consumption reports. |  | MSConsumption |  |
| MS_NET_ASSIST_INVOCATION |  | Indicates that the event subscribed/notified is Media Streaming Network Assistance invocation. |  | MSNetAssInvocation |  |
| MS_DYN_POLICY_INVOCATION |  | Indicates that the event subscribed/notified is Media Streaming Dynamic Policy invocation. |  | MSDynPolicyInvocation |  |
| MS_ACCESS_ACTIVITY |  | Indicates that the event subscribed/notified is Media Streaming access activity. |  | MSAccessActivity |  |
| GNSS_ASSISTANCE_DATA |  | Indicates that the subscribed/notified event is GNSS Assistance Data Collection. |  | GNSSAssistData |  |
| DATA_VOLUME_TRANSFER_TIME |  | Indicates that the event subscribed is data volume transfer time information. |  | DataVolTransferTime |  |
| APP_ACTIVE_TIME |  | Indicates that the event subscribed is application activation time information. |  | AppActiveTime |  |
| SIGNALLING_INFO |  | Indicates that the event subscribed/notified is signalling information. |  | SignallingInfo |  |
| UE_ALTITUDE |  | Indicates that the event subscribed/notified is UE altitude information. |  | UeAltitude |  |

# *** 10th Change ***

## 5.8	Feature negotiation

The optional features in table 5.8-1 are defined for the Naf_EventExposure API. They shall be negotiated using the extensibility mechanism defined in clause 6.6 of 3GPP TS 29.500 [5].

Table 5.8-1: Supported Features

| Feature number |  | Feature Name |  | Description |  |
| --- | --- | --- | --- | --- | --- |
| 1 |  | ServiceExperience |  | This feature indicates support for the event related to service experience. |  |
| 2 |  | UeMobility |  | This feature indicates support for the event related to UE mobility. |  |
| 3 |  | UeCommunication |  | This feature indicates support for the event related to UE communication information. |  |
| 4 |  | Exceptions |  | This feature indicates support for the event related to exception information. |  |
| 5 |  | ES3XX |  | Extended Support for 3xx redirections. This feature indicates the support of redirection for any service operation, according to Stateless NF procedures as specified in clauses 6.5.3.2 and 6.5.3.3 of 3GPP TS 29.500 [5] and according to HTTP redirection principles for indirect communication, as specified in clause 6.10.9 of 3GPP TS 29.500 [5]. |  |
| 6 |  | EneNA |  | This feature indicates support for the enhancements of network data analytics requirements. |  |
| 7 |  | UserDataCongestion |  | This feature indicates support for the event related to User Data Congestion Analytics related information. |  |
| 8 |  | PerformanceData |  | This feature indicates support for the event related to performance data information. |  |
| 9 |  | Dispersion |  | This feature indicates support for the event related to Dispersion Analytics related information. |  |
| 10 |  | CollectiveBehaviour |  | This feature indicates support for the event related to collective behaviour information. |  |
| 11 |  | ServiceExperienceExt |  | This feature indicates support for the extensions to the event related to service experience, including reporting Application Server Instance. Supporting this feature also requires the support of feature ServiceExperience. |  |
| 12 |  | MSQoeMetrics |  | This feature indicates support for the event related to Media Streaming QoE metrics for UE Application collected via the Data Collection AF. |  |
| 13 |  | MSConsumption |  | This feature indicates support for the event related to Media Streaming Consumption reports for UE Application collected via the Data Collection AF. |  |
| 14 |  | MSNetAssInvocation |  | This feature indicates support for the event related to Media Streaming Network Assistance invocation for UE Application collected via the Data Collection AF. |  |
| 15 |  | MSDynPolicyInvocation |  | This feature indicates support for the event related to Media Streaming Dynamic Policy invocation for UE Application collected via the Data Collection AF. |  |
| 16 |  | MSAccessActivity |  | This feature indicates support for the event related to Media Streaming access activity for UE Application collected via the Data Collection AF. |  |
| 17 |  | DataAccProfileId |  | This feature indicates support for Data Access Profile Identifier. |  |
| 18 |  | AllApplications |  | This feature indicates applicable to all the applications. |  |
| 19 |  | GNSSAssistData |  | This feature indicates the support of the GNSS Assistance Data Collection functionality as part of the enhancements to the 5G LCS functionality. The following functionalities are supported: - GNSS Assistance Data Collection. |  |
| 20 |  | PerformanceDataExt_AIML |  | This feature indicates the support for the extensions of the analytics related to DN performance supporting AIML, including support of Max/Min UL/DL data collection on packet delay, pack loss and throughput. Supporting this feature also requires the support of feature PerformanceData. |  |
| 21 |  | UeMobilityExt_AIML |  | This feature indicates support for further extensions to the event related to UE mobility supporting AIML including support of list of application service area collection. Supporting this feature also requires the support of feature UeMobility. |  |
| 22 |  | EnPerformanceData |  | This feature indicates support for the enhancements of performance data. This feature requires the support of the PerformanceData feature. |  |
| 23 |  | UeCommunicationExt_eNA |  | This feature indicates support for the enhancements of UE Communication, including support of ordering criterion. Supporting this feature also requires the support of UeCommunication feature. |  |
| 24 |  | ServiceExperienceExt2_eNA |  | This feature indicates support for the extensions to the event related to service experience supporting eNA, including Service Experience Contribution Weights. Supporting this feature also requires the support of feature ServiceExperience. |  |
| 25 |  | EnhDataMgmt |  | Indicates the support of enhanced data management mechanisms. Supporting this feature also requires the support of feature EneNA. |  |
| 26 |  | ExtEventFilters |  | Indicates support of extended AF event filters. |  |
| 27 |  | DataVolTransferTime |  | This feature indicates support for the event related to data volume transfer time. |  |
| 28 |  | MSEventExposure |  | This feature indicates the support for Media Streaming event exposure. This feature is recommended to be implemented to avoid the usage of the deprecated attributes. |  |
| 29 |  | PerEventRepReq |  | This feature indicates the support of the per-event reporting requirements management functionality. The following functionalities are supported: - Provisioning/updating the reporting requirements on a per subscribed event granularity. |  |
| 30 |  | RelativeProximity |  | This feature indicates the support of providing confidence information of the relative proximity data. Supporting this feature requires the support of the CollectiveBehaviour feature. |  |
| 31 |  | EnhUeDataExposure |  | This feature indicates the enhancements for the UE related data exposure. The following functionalities are supported: - providing the expected average speed of the UE over the route planned by a UE application. |  |
| 32 |  | AppActiveTime |  | This feature indicates the support for the application activation time data collection functionality. |  |
| 33 |  | SignallingInfo |  | This feature indicates the support of signalling information events. |  |
| 34 |  | UeAltitude |  | This feature indicates the support for the event related to UE altitude information. |  |

# *** 11th Change ***

# A.2	Naf_EventExposure API

openapi: 3.0.0

info:

version: 1.4.0-alpha.5

title: Naf_EventExposure

description: |

AF Event Exposure Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.517 V19.4.0; 5G System; Application Function Event Exposure Service; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.517/

servers:

- url: '{apiRoot}/naf-eventexposure/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501

security:

- {}

- oAuth2ClientCredentials: []

paths:

/subscriptions:

post:

summary: Creates a new Individual Application Event Exposure Subscription resource

operationId: PostAfEventExposureSubsc

tags:

- Application Event Subscription (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/AfEventExposureSubsc'

responses:

'201':

description: Success

content:

application/json:

schema:

$ref: '#/components/schemas/AfEventExposureSubsc'

headers:

Location:

description: >

Contains the URI of the created individual application event subscription resource

required: true

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

callbacks:

AfEventExposureNotif:

'{$request.body#/notifUri}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/AfEventExposureNotif'

responses:

'204':

description: No Content, Notification was successful

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/subscriptions/{subscriptionId}:

get:

summary: "Reads an existing Individual Application Event Subscription"

operationId: GetAfEventExposureSubsc

tags:

- Individual Application Event Subscription (Document)

parameters:

- name: subscriptionId

in: path

description: Application Event Subscription ID

required: true

schema:

type: string

- name: supp-feat

in: query

description: Features supported by the NF service consumer

required: false

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: OK. Resource representation is returned

content:

application/json:

schema:

$ref: '#/components/schemas/AfEventExposureSubsc'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

put:

summary: "Modifies an existing Individual Application Event Subscription "

operationId: PutAfEventExposureSubsc

tags:

- Individual Application Event Subscription (Document)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/AfEventExposureSubsc'

parameters:

- name: subscriptionId

in: path

description: Application Event Subscription ID

required: true

schema:

type: string

responses:

'200':

description: OK. Resource was successfully modified and representation is returned

content:

application/json:

schema:

$ref: '#/components/schemas/AfEventExposureSubsc'

'204':

description: No Content. Resource was successfully modified

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

delete:

summary: "Cancels an existing Individual Application Event Subscription "

operationId: DeleteAfEventExposureSubsc

tags:

- Individual Application Event Subscription (Document)

parameters:

- name: subscriptionId

in: path

description: Application Event Subscription ID

required: true

schema:

type: string

responses:

'204':

description: No Content. Resource was successfully deleted

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUri}'

scopes: {}

description: >

For trusted AF, the 'naf-eventexposure' shall be used as 'scopes' and

'{nrfApiRoot}/oauth2/token' shall be used as 'tokenUri'.

schemas:

AfEventExposureNotif:

description: >

Represents notifications on application event(s) that occurred for an Individual Application

Event Subscription resource.

type: object

properties:

notifId:

type: string

eventNotifs:

type: array

items:

$ref: '#/components/schemas/AfEventNotification'

minItems: 1

required:

- notifId

- eventNotifs

AfEventExposureSubsc:

description: Represents an Individual Application Event Subscription resource.

type: object

properties:

dataAccProfId:

type: string

eventsSubs:

type: array

items:

$ref: '#/components/schemas/EventsSubs'

minItems: 1

eventsRepInfo:

$ref: 'TS29523_Npcf_EventExposure.yaml#/components/schemas/ReportingInformation'

notifUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

notifId:

type: string

eventNotifs:

type: array

items:

$ref: '#/components/schemas/AfEventNotification'

minItems: 1

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- eventsSubs

- eventsRepInfo

- notifId

- notifUri

AfEventNotification:

description: Represents information related to an event to be reported.

type: object

properties:

event:

$ref: '#/components/schemas/AfEvent'

timeStamp:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

svcExprcInfos:

type: array

items:

$ref: '#/components/schemas/ServiceExperienceInfoPerApp'

minItems: 1

ueMobilityInfos:

type: array

items:

$ref: '#/components/schemas/UeMobilityCollection'

minItems: 1

ueCommInfos:

type: array

items:

$ref: '#/components/schemas/UeCommunicationCollection'

minItems: 1

excepInfos:

type: array

items:

$ref: '#/components/schemas/ExceptionInfo'

minItems: 1

congestionInfos:

type: array

items:

$ref: '#/components/schemas/UserDataCongestionCollection'

minItems: 1

perfDataInfos:

type: array

items:

$ref: '#/components/schemas/PerformanceDataCollection'

minItems: 1

dispersionInfos:

type: array

items:

$ref: '#/components/schemas/DispersionCollection'

minItems: 1

collBhvrInfs:

type: array

items:

$ref: '#/components/schemas/CollectiveBehaviourInfo'

minItems: 1

msQoeMetrInfos:

type: array

items:

$ref: '#/components/schemas/MsQoeMetricsCollection'

minItems: 1

deprecated: true

msQoeMetrics:

type: array

items:

$ref: 'TS26512_EventExposure.yaml#/components/schemas/QoEMetricsCollection'

minItems: 1

description: Represents the Media Streaming QoE metrics event records.

msConsumpInfos:

type: array

items:

$ref: '#/components/schemas/MsConsumptionCollection'

minItems: 1

deprecated: true

msConsumpRpts:

type: array

items:

$ref: 'TS26512_EventExposure.yaml#/components/schemas/ConsumptionReportingUnitsCollection'

minItems: 1

description: Represents the Media Streaming Consumption event records.

msNetAssInvInfos:

type: array

items:

$ref: '#/components/schemas/MsNetAssInvocationCollection'

minItems: 1

deprecated: true

msNetAssistInvs:

type: array

items:

$ref: 'TS26512_EventExposure.yaml#/components/schemas/NetworkAssistanceInvocationsCollection'

minItems: 1

description: >

Represents the Media Streaming Network Assistance Invocations event records.

msDynPlyInvInfos:

type: array

items:

$ref: '#/components/schemas/MsDynPolicyInvocationCollection'

minItems: 1

deprecated: true

msDynPlyInvs:

type: array

items:

$ref: 'TS26512_EventExposure.yaml#/components/schemas/DynamicPolicyInvocationsCollection'

minItems: 1

description: Represents the Media Streaming Dynamic Policy Invocations event records.

msAccActInfos:

type: array

items:

$ref: '#/components/schemas/MSAccessActivityCollection'

minItems: 1

deprecated: true

msAccesses:

type: array

items:

$ref: 'TS26512_EventExposure.yaml#/components/schemas/MediaStreamingAccessesCollection'

minItems: 1

description: Represents the Media Streaming access event records.

gnssAssistDataInfo:

$ref: 'TS29591_Nnef_EventExposure.yaml#/components/schemas/GNSSAssistDataInfo'

datVolTransTimeInfos:

type: array

items:

$ref: '#/components/schemas/DatVolTransTimeCollection'

minItems: 1

appActiveTimeInfos:

type: array

items:

$ref: '#/components/schemas/AppActiveTimeInfo'

minItems: 1

sigInfos:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfSignallingInfo'

minItems: 1

ueAltitudeInfos:

type: array

items:

$ref: '#/components/schemas/UeAltitudeInfo'

minItems: 1

required:

- event

- timeStamp

EventsSubs:

description: Represents an event to be subscribed and the related event filter information.

type: object

properties:

event:

$ref: '#/components/schemas/AfEvent'

eventFilter:

$ref: '#/components/schemas/EventFilter'

eventRepInfo:

$ref: 'TS29523_Npcf_EventExposure.yaml#/components/schemas/ReportingInformation'

required:

- event

- eventFilter

EventFilter:

description: Represents event filter information for an event.

type: object

properties:

gpsis:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

minItems: 1

supis:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

minItems: 1

exterGroupIds:

type: array

items:

$ref: 'TS29503_Nudm_SDM.yaml#/components/schemas/ExtGroupId'

minItems: 1

interGroupIds:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/GroupId'

anyUeInd:

type: boolean

ueIpAddr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/IpAddr'

appIds:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ApplicationId'

minItems: 1

locArea:

$ref: 'TS29122_CommonData.yaml#/components/schemas/LocationArea5G'

collAttrs:

type: array

items:

$ref: '#/components/schemas/CollectiveBehaviourFilter'

minItems: 1

exceptionReqs:

type: array

items:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/Exception'

minItems: 1

tws:

type: array

items:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

minItems: 1

oneOf:

- required: [gpsis]

- required: [supis]

- required: [exterGroupIds]

- required: [interGroupIds]

- required: [anyUeInd]

- required: [ueIpAddr]

ServiceExperienceInfoPerApp:

description: Contains service experience information associated with an application.

type: object

properties:

appId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ApplicationId'

appServerIns:

$ref: '#/components/schemas/AddrFqdn'

svcExpPerFlows:

type: array

items:

$ref: '#/components/schemas/ServiceExperienceInfoPerFlow'

minItems: 1

gpsis:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

minItems: 1

supis:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

minItems: 1

contrWeights:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

minItems: 1

required:

- svcExpPerFlows

ServiceExperienceInfoPerFlow:

description: Contains service experience information associated with a service flow.

type: object

properties:

svcExprc:

$ref: '#/components/schemas/SvcExperience'

timeIntev:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

dnai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnai'

ipTrafficFilter:

$ref: 'TS29122_CommonData.yaml#/components/schemas/FlowInfo'

ethTrafficFilter:

$ref: 'TS29514_Npcf_PolicyAuthorization.yaml#/components/schemas/EthFlowDescription'

SvcExperience:

description: Contains a mean opinion score with the customized range.

type: object

properties:

mos:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Float'

upperRange:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Float'

lowerRange:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Float'

UeMobilityCollection:

description: >

Contains UE mobility information associated with an application. If the allAppInd attribute

is present and set to true, then the value in the appId shall be ignored, which indicates

the collected UE mobility information is applicable to all the applications for the UE.

type: object

properties:

gpsi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

supi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

appId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ApplicationId'

allAppInd:

type: boolean

description: >

Indicates applicable to all applications if set to true, otherwise set to false.

Default value is false if omitted.

ueTrajs:

type: array

items:

$ref: '#/components/schemas/UeTrajectoryCollection'

minItems: 1

areas:

type: array

items:

$ref: 'TS29122_CommonData.yaml#/components/schemas/LocationArea5G'

minItems: 1

required:

- appId

- ueTrajs

UeCommunicationCollection:

description: Contains UE communication information associated with an application.

type: object

properties:

gpsi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

supi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

exterGroupId:

$ref: 'TS29503_Nudm_SDM.yaml#/components/schemas/ExtGroupId'

interGroupId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/GroupId'

appId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ApplicationId'

expectedUeBehavePara:

$ref: 'TS29122_CpProvisioning.yaml#/components/schemas/CpParameterSet'

comms:

type: array

items:

$ref: '#/components/schemas/CommunicationCollection'

minItems: 1

required:

- appId

- comms

UeTrajectoryCollection:

description: Contains UE trajectory information associated with an application.

type: object

properties:

ts:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

locArea:

$ref: 'TS29122_CommonData.yaml#/components/schemas/LocationArea5G'

required:

- ts

- locArea

CommunicationCollection:

description: Contains communication information.

type: object

properties:

startTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

endTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

ulVol:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Volume'

dlVol:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Volume'

required:

- startTime

- endTime

- ulVol

- dlVol

ExceptionInfo:

description: Represents the exceptions information provided by the AF.

type: object

properties:

ipTrafficFilter:

$ref: 'TS29122_CommonData.yaml#/components/schemas/FlowInfo'

ethTrafficFilter:

$ref: 'TS29514_Npcf_PolicyAuthorization.yaml#/components/schemas/EthFlowDescription'

exceps:

type: array

items:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/Exception'

minItems: 1

required:

- exceps

oneOf:

- required: [ipTrafficFilter]

- required: [ethTrafficFilter]

UserDataCongestionCollection:

description: Contains User Data Congestion Analytics related information collection.

type: object

properties:

appId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ApplicationId'

ipTrafficFilter:

$ref: 'TS29122_CommonData.yaml#/components/schemas/FlowInfo'

timeInterv:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

thrputUl:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate'

thrputDl:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate'

thrputPkUl:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate'

thrputPkDl:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate'

oneOf:

- required: [appId]

- required: [ipTrafficFilter]

PerformanceDataCollection:

description: Contains Performance Data Analytics related information collection.

type: object

properties:

appId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ApplicationId'

ueIpAddr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/IpAddr'

ipTrafficFilter:

$ref: 'TS29122_CommonData.yaml#/components/schemas/FlowInfo'

ueLoc:

$ref: 'TS29122_CommonData.yaml#/components/schemas/LocationArea5G'

appLocs:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnai'

minItems: 1

asAddr:

$ref: '#/components/schemas/AddrFqdn'

perfData:

$ref: '#/components/schemas/PerformanceData'

timeStamp:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

required:

- perfData

- timeStamp

PerformanceData:

description: Contains Performance Data.

type: object

properties:

pdb:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PacketDelBudget'

pdbDl:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PacketDelBudget'

maxPdbUl:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PacketDelBudget'

maxPdbDl:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PacketDelBudget'

plr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PacketLossRate'

plrDl:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PacketLossRate'

maxPlrUl:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PacketLossRate'

maxPlrDl:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PacketLossRate'

thrputUl:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate'

maxThrputUl:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate'

minThrputUl:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate'

thrputDl:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate'

maxThrputDl:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate'

minThrputDl:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate'

AddrFqdn:

description: IP address and/or FQDN.

type: object

properties:

ipAddr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/IpAddr'

fqdn:

type: string

description: Indicates an FQDN.

DispersionCollection:

description: Contains the dispersion information collected for an AF.

type: object

properties:

gpsi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

supi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

ueAddr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/IpAddr'

timeStamp:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

dataUsage:

$ref: 'TS29122_CommonData.yaml#/components/schemas/UsageThreshold'

flowDesp:

$ref: 'TS29514_Npcf_PolicyAuthorization.yaml#/components/schemas/FlowDescription'

appId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ApplicationId'

dnais:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnai'

minItems: 1

appDur:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

required:

- dataUsage

oneOf:

- required: [gpsi]

- required: [supi]

- required: [ueAddr]

CollectiveBehaviourFilter:

description: Contains the collective behaviour filter information to be collected from UE.

type: object

properties:

type:

$ref: '#/components/schemas/CollectiveBehaviourFilterType'

value:

type: string

description: Value of the parameter type as in the type attribute.

collBehAttr:

type: array

items:

$ref: '#/components/schemas/PerUeAttribute'

minItems: 1

description: >

Contains the values of collective behaviour attributes at least one of which shall

match for an AF event to be sent.

dataProcType:

$ref: '#/components/schemas/DataProcessingType'

listOfUeInd:

type: boolean

description: >

Indicates whether request list of UE IDs that fulfill a collective behaviour within the

area of interest. This attribute shall set to "true" if request the list of UE IDs,

otherwise, set to "false". May only be present and sets to "true" if "AfEvent" sets to

"COLLECTIVE_BEHAVIOUR".

required:

- type

- value

CollectiveBehaviourInfo:

description: Contains the collective behaviour information to be reported to the subscriber.

type: object

properties:

colAttrib:

type: array

items:

$ref: '#/components/schemas/PerUeAttribute'

minItems: 1

noOfUes:

type: integer

description: Total number of UEs that fulfil a collective within the area of interest.

appIds:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ApplicationId'

minItems: 1

extUeIds:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

minItems: 1

ueIds:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

minItems: 1

collisionDist:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

absDirs:

type: array

items:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/Direction'

minItems: 1

relDirs:

type: array

items:

$ref: '#/components/schemas/RelativeDirection'

minItems: 1

ueTrajectory:

$ref: '#/components/schemas/UeTrajectoryCollection'

confidence:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

oneOf:

- required: [extUeIds]

- required: [ueIds]

PerUeAttribute:

description: UE application data collected per UE.

type: object

properties:

ueDest:

$ref: 'TS29122_CommonData.yaml#/components/schemas/LocationArea5G'

route:

type: string

averageSpeed:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/VelocityEstimate'

timeOfArrival:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

MsQoeMetricsCollection:

description: >

Contains the Media Streaming QoE metrics information collected for an UE Application via AF.

type: object

properties:

msQoeMetrics:

type: array

items:

type: string

minItems: 1

required:

- msQoeMetrics

MsConsumptionCollection:

description: >

Contains the Media Streaming Consumption information collected for an UE Application via AF.

type: object

properties:

msConsumps:

type: array

items:

type: string

description: >

Represents the Media Streaming Consumption reports with formatting as specified in

clause 11.3.3 of 3GPP TS 26.512 [30], if required for Media Streaming UE Application.

minItems: 1

required:

- msConsumps

MsNetAssInvocationCollection:

description: >

Contains the Media Streaming Network Assistance invocation collected for an UE Application

via AF.

type: object

properties:

msNetAssInvocs:

type: array

items:

$ref: 'TS26512_M5_NetworkAssistance.yaml#/components/schemas/NetworkAssistanceSession'

minItems: 1

required:

- msNetAssInvocs

MsDynPolicyInvocationCollection:

description: >

Contains the Media Streaming Dynamic Policy invocation collected for an UE

Application via AF.

type: object

properties:

msDynPlyInvocs:

type: array

items:

$ref: 'TS26512_M5_DynamicPolicies.yaml#/components/schemas/DynamicPolicy'

minItems: 1

required:

- msDynPlyInvocs

MSAccessActivityCollection:

description: Contains Media Streaming access activity collected for an UE Application via AF.

type: object

properties:

msAccActs:

type: array

items:

$ref: 'TS26512_R4_DataReporting.yaml#/components/schemas/MediaStreamingAccessRecord'

minItems: 1

required:

- msAccActs

DatVolTransTimeCollection:

description: Contains the collective data volume transfer time information to be reported to the subscriber.

type: object

properties:

appId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ApplicationId'

appServerInst:

$ref: '#/components/schemas/AddrFqdn'

gpsi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

supi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

ulTransVol:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Volume'

dlTransVol:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Volume'

ulTransTimeDur:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

dlTransTimeDur:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

anyOf:

- anyOf:

- required: [ulTransVol]

- required: [dlTransVol]

- anyOf:

- required: [ulTransTimeDur]

- required: [dlTransTimeDur]

AppActiveTimeInfo:

description: Contains the activation time information of the application.

type: object

properties:

appId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ApplicationId'

activeTimeInfos:

type: array

items:

$ref: '#/components/schemas/ActiveTimeInfo'

minItems: 1

required:

- appId

- activeTimeInfos

ActiveTimeInfo:

description: Contains the activation time information.

type: object

properties:

ueNum:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

gpsis:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

minItems: 1

supis:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

minItems: 1

exterGroupIds:

type: array

items:

$ref: 'TS29503_Nudm_SDM.yaml#/components/schemas/ExtGroupId'

minItems: 1

interGroupIds:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/GroupId'

minItems: 1

activeTime:

type: array

items:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

minItems: 1

inactiveTime:

type: array

items:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

minItems: 1

anyOf:

- required: [activeTime]

- required: [inactiveTime]

UeAltitudeInfo:

description: Contains UE altitude information.

type: object

properties:

altitude:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/Altitude'

timeStamp:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

required:

- altitude

# Simple data types and Enumerations

AfEvent:

anyOf:

- type: string

enum:

- SVC_EXPERIENCE

- UE_MOBILITY

- UE_COMM

- EXCEPTIONS

- USER_DATA_CONGESTION

- PERF_DATA

- DISPERSION

- COLLECTIVE_BEHAVIOUR

- MS_QOE_METRICS

- MS_CONSUMPTION

- MS_NET_ASSIST_INVOCATION

- MS_DYN_POLICY_INVOCATION

- MS_ACCESS_ACTIVITY

- GNSS_ASSISTANCE_DATA

- DATA_VOLUME_TRANSFER_TIME

- APP_ACTIVE_TIME

- SIGNALLING_INFO

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration but

is not used to encode content defined in the present version of this API.

description: |

Represents an application's event.

Possible values are:

- SVC_EXPERIENCE: Indicates that the subscribed/notified event is service experience

information for an application.

- UE_MOBILITY: Indicates that the subscribed/notified event is UE mobility information.

- UE_COMM: Indicates that the subscribed/notified event is UE communication information.

- EXCEPTIONS: Indicates that the subscribed/notified event is exceptions information.

- USER_DATA_CONGESTION: Indicates that the subscribed/notified event is user data congestion

analytics related information.

- PERF_DATA: Indicates that the subscribed/notified event is performance data information.

- DISPERSION: Indicates that the subscribed/notified event is dispersion information.

- COLLECTIVE_BEHAVIOUR: Indicates that the subscribed/notified event is collective behaviour

information.

- MS_QOE_METRICS: Indicates that the subscribed/notified event is Media Streaming QoE

metrics.

- MS_CONSUMPTION: Indicates that the subscribed/notified event is Media Streaming

consumption reports.

- MS_NET_ASSIST_INVOCATION: Indicates that the subscribed/notified event is Media Streaming

network assistance invocation.

- MS_DYN_POLICY_INVOCATION: Indicates that the subscribed/notified event is Media Streaming

dynamic policy invocation.

- MS_ACCESS_ACTIVITY: Indicates that the subscribed/notified event is Media Streaming access

activity.

- GNSS_ASSISTANCE_DATA: Indicates that the subscribed/notified event is GNSS Assistance Data

Collection.

- DATA_VOLUME_TRANSFER_TIME: Indicates that the event subscribed is data volume transfer

time information.

- APP_ACTIVE_TIME: Indicates that the event subscribed is application activation time

information.

- SIGNALLING_INFO: Indicates that the event subscribed/notified is signalling information.

CollectiveBehaviourFilterType:

anyOf:

- type: string

enum:

- COLLECTIVE_ATTRIBUTE

- DATA_PROCESSING

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration but

is not used to encode content defined in the present version of this API.

description: |

Represents the parameter type for collective behaviour information filtering.

Possible values are:

- COLLECTIVE_ATTRIBUTE: Indicates that the parameter type is collective attributes.

- DATA_PROCESSING: Indicates that the parameter type is data processing.

DataProcessingType:

description: Represents a type of data processing.

anyOf:

- type: string

enum:

- AGGREGATION

- NORMALIZATION

- ANONYMIZATION

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration but

is not used to encode content defined in the present version of this API.

RelativeDirection:

anyOf:

- type: string

enum:

- ABOVE

- BELOW

- LEFT

- RIGHT

- BEFORE

- AFTER

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration but

is not used to encode content defined in the present version of this API.

description: |

Represents an application's event.

Possible values are:

- ABOVE: Indicates that UE movement with respect to another UE is above.

- BELOW: Indicates that UE movement with respect to another UE is below.

- LEFT: Indicates that UE movement with respect to another UE is left.

- RIGHT: Indicates that UE movement with respect to another UE is right.

- BEFORE: Indicates that UE movement with respect to another UE is before.

- AFTER: Indicates that UE movement with respect to another UE is after.

*** End of Changes ***