3GPP TSG-CT3 Meeting #143	C3-254480

# Sophia-Antipolis, France, 13th Oct 2025 - 17th Oct 2025				(revision of C3-254287)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.222 | CR | 0440 | rev | 1 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to Open API Discovery Service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT3 |  |  |  |  |  |  |  |  |  |
| Work item code: | CAPIF_Ph3 |  |  |  |  |  | Date: |  |  | 2025-10-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The following error has been identified and need to be corrected: - The GET response body is using the wrong data type (DiscoveryResp). It should instead use OpenDiscoveryResp. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR proposes to correct the above identified errors. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Implementation inconsistencies and incomplete OpenAPI definition. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.11.2.2.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR does not impact any OpenAPI file. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * 1st Change * * * *

###### 8.11.2.2.3.1	GET

The HTTP GET method enables to discover the Service APIs currently registered at the CCF.

Table 8.11.2.2.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| api-names | array(string) | O | 1..N | Contains the name(s) of the target Service API(s). Each Service API name shall be set to the value of the <apiName> placeholder of the Service API URI structure as defined in clause 5.2.4 of 3GPP TS 29.122 [14]. |  |
| api-versions | map(array(string)) | O | 1..N(1..M) | Contains the major version(s) (e.g., v1) of the target Service API(s). Each Service API version shall be set to the value of the <apiVersion> placeholder of the Service API URI structure as defined in clause 5.2.4 of 3GPP TS 29.122 [14]. The key of the map shall be set to the value of the Service API name (i.e., the value of the <apiName> placeholder of the Service API URI structure as defined in clause 5.2.4 of 3GPP TS 29.122 [14]) of the Service API to which the provided list of Service API version(s) provided within the map value applies. |  |
| comm-type | CommunicationType | O | 0..1 | Contains the communication type supported by the target Service API(s). |  |
| protocols | array(Protocol) | O | 1..N | Contains the protocol(s) supported by the target Service API(s). |  |
| data-format | DataFormat | O | 0..1 | Contains data format(s) supported by the target Service API(s). |  |
| api-cats | array(string) | O | 1..N | Contains the category(ies) of the target Service API(s). |  |
| preferred-aef-loc | AefLocation | O | 0..1 | Contains the preferred location information for AEF(s) exposing the target Service API(s). This query parameter is ignored by the CCF if there are no matching records at the CCF. |  |
| api-prov-names | array(string) | O | 1..N | Contains the name(s) of the provider(s) of the target Service API(s). |  |
| api-supported-features | map(SupportedFeatures) | O | 1..N | Contains the list of the feature(s) supported by the target Service API(s) indicated by the "api-names" query parameter. This query parameter may be present only if the "api-name" query parameter is also present. The key of the map shall be set to the value of the Service API name, among the ones provided within the "api-names" query parameter) of the Service API to which the provided list of supported feature(s) provided within the map value applies. |  |
| service-kpis | ServiceKpis | O | 0..1 | Contains information about the service characteristics provided by the target Service API(s). |  |
| api-ids | array(string) | O | 1..N | Contains the identifier(s) of the target Service APIs. |  |
| res-ops | array(ResOperInfo) | O | 1..N | Contains the list of supported API resource(s) and service operation(s). This query parameter may only be present if the "api-names" query parameter is present. |  |
| supported-features | SupportedFeatures | O | 0..1 | Contains the list of supported features among the ones defined in clause 8.11.6. This attribute shall be present only when feature negotiation is required. |  |
| NOTE: In addition to the above standardized query parameters, the service consumer may also provide vendor-specific query parameter(s) as specified in clause 5.2.13.3 of 3GPP TS 29.122 [14]. The CCF shall use any received vendor-specific query parameters in the filtering process of the results to be returned in the response in a similar way and in addition to the standardized query parameters defined in this table. |  |  |  |  |  |

Editor's note:	The access credentials and requirements for this service operation are FFS and pending progress in SA3.

This method shall support the request data structures specified in table 8.11.2.2.3.1-2 and the response data structures and response codes specified in table 8.11.2.2.3.1-3.

Table 8.11.2.2.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 8.11.2.2.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| OpenDiscoveryResp | M | 1 | 200 OK | Successful case. The result of the Service APIs discovery is returned. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative CCF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [14]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI of the resource located in an alternative CCF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [14]. |
| ProblemDetails | O | 0..1 | 414 URI Too Long | Indicates that the server refuses to process the request because the request target is too long. |
| NOTE: The mandatory HTTP error status codes for the HTTP GET method listed in table 5.2.6-1 of 3GPP TS 29.122 [14] shall also apply. |  |  |  |  |

Table 8.11.2.2.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI of the resource located in an alternative CCF. |

Table 8.11.2.2.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative target URI of the resource located in an alternative CCF. |

# * * * * End of Changes * * * *