3GPP TSG CT WG3 Meeting #143	C3-254481

# Sophia-Antipolis, FR, 13th – 17th October, 2025											was C3-254151

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.122 | CR | 0974 | rev | 1 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates and corrections to the CAPIF related Security clause |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |
| Source to TSG: | CT3 |  |  |  |  |  |  |  |  |
| Work item code: | NBI19 |  |  |  |  |  | Date: |  | 2025-10-15 |
| Category: | B |  |  |  |  |  | Release: |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  |  | With the support of resource and operation level access control and RNAA use cases in CAPIF, the provisions of clause 7.2 and the NBI TS Skeleton need to be updated to: - Remove the restriction of only having API level access control. - Remove the restriction on the support of only the "Client Credentials" grant type for OAuth2. |  |  |  |  |  |  |
| Summary of change: |  |  | This CR proposes to: - Address the above-detailed necessary enhancements. |  |  |  |  |  |  |
| Consequences if not approved: |  |  | - The above-detailed necessary enhancements are not addressed and the latest updates (e.g., the definition of resource and operation level access control and RNAA use cases in CAPIF) are not considered in stage 3. |  |  |  |  |  |  |
| Clauses affected: |  | 7.2, Annex B |  |  |  |  |  |  |  |
|  |  | Y |  | N |  |  |  |  |  |
| Other specs |  |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |
| affected: |  |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |
| (show related CRs) |  |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |
| Other comments: |  | This CR does not impact the OpenAPI descriptions of the APIs. |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |

* * * * Start of changes * * * *

## 7.2	Security

When CAPIF is used for managing the exposure of the SCEF T8 APIs, before invoking an API exposed by the SCEF, the SCS/AS, acting as an API Invoker shall negotiate the security method (PKI, TLS-PSK or OAUTH2) with the CAPIF Core Function and ensure that the SCEF has enough credential to authenticate the SCS/AS as defined in clauses 5.6.2.2 and 6.2.2.2 of 3GPP TS 29.222 [48].

If PKI or TLS-PSK is used as the selected security method between the SCS/AS and the SCEF, upon API invocation, the SCEF shall retrieve the authorization information from the CAPIF Core Function as described in clause 5.6.2.4 of 3GPP TS 29.222 [48].

As indicated in 3GPP TS 33.122 [53], the access to the SCEF T8 APIs may be authorized by means of the OAuth2 protocol (see IETF RFC 6749 [51]), where the CAPIF Core Function (see 3GPP TS 29.222 [48]) plays the role of the authorization server.

If OAuth2 is used as the selected security method between the SCS/AS and the SCEF, the SCS/AS, prior to consuming services offered by the SCEF T8 APIs, shall obtain a "token" from the authorization server, by invoking the Obtain_Authorization service, as described in clause 5.6.2.3.2 of 3GPP TS 29.222 [48].

The T8 APIs do not define any scopes for OAuth2 authorization in the present specification. For the definition and handling of scopes for OAuth2 authorization in CAPIF, see 3GPP TS 29.222 [48].

It is the SCEF responsibility to check whether the SCS/AS is authorized to use an API based on the "token". Once the SCEF verifies the "token", it shall check whether the SCEF identifier in the "token" matches its own published identifier, whether the API name in the "token" matches its own published API name and whether the granted scope (see 3GPP TS 29.222 [48]) in the "token" is authorized. If those checks are passed, the SCS/AS has full authority to access any resource(s) and/or operation(s) for the invoked service API and that are within the limits of the granted scope in the "token".

NOTE 1:	The security requirements in the current clause do not apply for the MsisdnLessMoSms API since it is the SCEF initiated interaction with the SCS/AS. How the security scheme works for the MsisdnLessMoSms API is left to configuration.

NOTE 2:	For the aforementioned security requirements, the SCEF needs to apply admission control according to access control policies after performing the authorization checks.

* * * * Next changes * * * *

Annex B (informative):
TS Skeleton Template

A TS Skeleton Template to be used as a starting point of drafting a 3GPP NBI (i.e. northbound and/or application layer interface) Stage 3 specification is available at the following location:

https://www.3gpp.org/ftp/information/All_Templates/29.xxx-NBI-Stage3-Template.zip

* * * * End of changes * * * *