3GPP TSG-CT WG3 Meeting #143	C3-254489

# Sophia-Antipolis, France, 13 - 17 October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.222 | CR | 0436 | rev | 1 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of examples for finer granularity scopes |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C3 |  |  |  |  |  |  |  |  |  |
| Work item code: | CAPIF_Ph3 |  |  |  |  |  | Date: |  |  | 2025-09-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The included examples of the scopes contain white spaces as a part of individual scope that is semantically not correct. Also, the restricted delimiters within the "aefId" field, the "apiName" field, the "scopeLevelType" field and "scopeLevelValue" field do not contain the restriction to provide white spaces. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update of examples to remove white spaces. Addition of the restriction to provide space(s) within the "aefId" field, the "apiName" field, the "scopeLevelType" field and "scopeLevelValue" field. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect examples and potential token scope parsing errors. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.5.4.2.6, 8.5.4.2.7, 8.5.4.2.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR does not affect any OpenAPI file. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First change * * * *

##### 8.5.4.2.6	Type: AccessTokenReq

Table 8.5.4.2.6-1: Definition of type AccessTokenReq

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| grant_type | string | M | 1 | This attribute shall contain the grant type as "client_credentials", or when the "RNAA" feature is supported, either "client_credentials" or "authorization_code". (NOTE 3, NOTE 4) |  |
| client_id | string | M | 1 | This attribute shall contain the API invoker Identifier. (NOTE 3) |  |
| resOwnerId | ResOwnerId | O | 0..1 | Contains the identifier of the resource owner. This attribute shall be present only when the access token request is used for RNAA. | RNAA |
| client_secret | string | O | 0..1 | This attribute when present shall contain the onboarding secret which is got during API invoker onboarding. (NOTE 3) |  |
| scope | string | O | 0..1 | Contains the requested OAuth2 scope.. When neither the "CAPIF_Ext1" feature nor the "CAPIF_Ext2" feature are supported, it takes the following format: 3gpp#aefId1:apiName1,apiName2,…apiNameX;aefId2:apiName1,apiName2,…apiNameY;…aefIdN:apiName1,apiName2,…apiNameZ Using delimiter "#" after the discriminator "3gpp", ":" after AEF identifier, "," between API names and ";" between the last API name of the previous AEF identifier and the next AEF identifier. (NOTE 2) Example: '3gpp#aef-jiangsu-nanjing:3gpp-monitoring-event,3gpp-as-session-with-qos;aef-zhejiang-hangzhou:3gpp-cp-parameter-provisioning,3gpp-pfd-management' When the "CAPIF_Ext1" feature is supported and the "CAPIF_Ext2" feature is not supported, it takes the following format: '3gpp#aefId1:apiName1:scopeLevelType.scopeLevelValue:scopeLevelType.scopeLevelValue:…:scopeLevelType.scopeLevelValue,apiName2:scopeLevelType.scopeLevelValue:scopeLevelType.scopeLevelValue:…:scopeLevelType.scopeLevelValue,…apiNameX:scopeLevelType.scopeLevelValue:scopeLevelType.scopeLevelValue:…:scopeLevelType.scopeLevelValue;…aefIdN:apiName1:scopeLevelType.scopeLevelValue:scopeLevelType.scopeLevelValue:…:scopeLevelType.scopeLevelValue,apiName2:scopeLevelType.scopeLevelValue:scopeLevelType.scopeLevelValue:…:scopeLevelType.scopeLevelValue,…apiNameZ:scopeLevelType.scopeLevelValue:scopeLevelType.scopeLevelValue:…:scopeLevelType.scopeLevelValue' Example 1: '3gpp#aef1:3gpp-monitoring-event:res.subscriptions,3gpp-as-session-with-qos :res.subscriptions:op.create;aef-zhejiang-hangzhou:3gpp-cp-parameter-provisioning,3gpp-pfd-management:res.transactions:op.read' Example 2: '3gpp#aef1:3gpp-time-sync:res.subscriptions:res.configurations:op.update,3gpp-mbs-session:res.mbs-sessions:res.subscriptions:op.create' With the following definitions: - The delimiter "#" shall be used only after the discriminator "3gpp" at the beginning of the scope field. - The delimiter ":" shall be used after the AEF identifier, after the API name and after each scope level. - The delimiter "," shall be used between API names of a certain AEF. - The delimiter ";" shall be used between the last API name of the one AEF section and the next AEF section. - Within the "aefId" field, the "apiName" field, the "scopeLevelType" field and "scopeLevelValue" field, the above defined delimiters and spaces are prohibited. - The "scopeLevelType" field shall be set to either "res" (for resource-level access control) or "op" (for operation-level access control). When the "CAPIF_Ext2" feature is supported and the "CAPIF_Ext1" feature is not supported, the content of this attribute shall be set to a list of space-delimited strings, as defined in clause 3.3 of IETF RFC 6749 [23]. |  |
| authCode | string | C | 0..1 | Contains the authorization code. This attribute shall be included only when the access token request is used for RNAA and the OAuth "authorization code" grant type is used. | RNAA |
| redirect_uri | string | O | 0..1 | Contains the redirection URI that was used to obtain the authorization code provided within the "authCode" attribute. This attribute may be included only when the access token request is used for RNAA and the OAuth "authorization code" grant type is used. (NOTE 3) | RNAA |
| NOTE 1: This data structure shall not be treated as a JSON object. It shall be treated as a key, value pair data structure to be encoded using x-www-urlencoded format as specified in clause 17.13.4.1 of W3C HTML 4.01 Specification [22]. NOTE 2: The scope may contain more space-delimited strings which further add additional access ranges to the scope, the definition of those additional strings is out of the scope of the present document. NOTE 3: The "grant_type", "client_id", "client_secret" and "redirect_uri" attributes do not follow the related naming convention defined in clause 7.2.1. These attributes are however kept as currently defined in this specification in order to keep them aligned with corresponding claims defined in IETF RFC 6749 [23] and for backward compatibility considerations. NOTE 4: The enumeration value "client_credentials" or "authorization_code" of the "grant_type" attribute does not follow the related naming convention defined in clause 7.2.1. This enumeration is however kept as currently defined in this specification for backward compatibility considerations. |  |  |  |  |  |

* * * Next change * * * *

##### 8.5.4.2.7	Type: AccessTokenRsp

Table 8.5.4.2.7-1: Definition of type AccessTokenRsp

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| access_token | string | M | 1 | This IE shall contain JWS Compact Serialized representation of the JWS signed JSON object containing AccessTokenClaims (see clause 8.5.4.2.8). (NOTE 2) |
| token_type | string | M | 1 | This IE shall contain the token type (i.e. "Bearer"). (NOTE 2, NOTE 3) |
| expires_in | DurationSec | M | 1 | This IE when present shall contain the number of seconds after which the access_token is considered to be expired. (NOTE 2) |
| scope | string | O | 0..1 | Contains the granted OAuth2 scope. When neither the "CAPIF_Ext1" feature nor the "CAPIF_Ext2" feature are supported, it takes the following format: 3gpp#aefId1:apiName1,apiName2,…apiNameX;aefId2:apiName1,apiName2,…apiNameY;…aefIdN:apiName1,apiName2,…apiNameZ Using delimeter "#" after the discriminator "3gpp", ":" after AEF identifier, "," between API names and ";" between the last API name of the previous AEF identifier and the next AEF identifier. (NOTE 1) Example: '3gpp#aef-jiangsu-nanjing:3gpp-monitoring-event,3gpp-as-session-with-qos;aef-zhejiang-hangzhou:3gpp-cp-parameter-provisioning,3gpp-pfd-management' When the "CAPIF_Ext1" feature is supported and the "CAPIF_Ext2" feature is not supported, it takes the following format: '3gpp#aefId1:apiName1:scopeLevelType.scopeLevelValue:scopeLevelType.scopeLevelValue:…:scopeLevelType.scopeLevelValue,apiName2:scopeLevelType.scopeLevelValue:scopeLevelType.scopeLevelValue:…:scopeLevelType.scopeLevelValue,…apiNameX:scopeLevelType.scopeLevelValue:scopeLevelType.scopeLevelValue:…:scopeLevelType.scopeLevelValue;…aefIdN:apiName1:scopeLevelType.scopeLevelValue:scopeLevelType.scopeLevelValue:…:scopeLevelType.scopeLevelValue,apiName2:scopeLevelType.scopeLevelValue:scopeLevelType.scopeLevelValue:…:scopeLevelType.scopeLevelValue,…apiNameZ:scopeLevelType.scopeLevelValue:scopeLevelType.scopeLevelValue:…:scopeLevelType.scopeLevelValue' Example 1: '3gpp#aef1:3gpp-monitoring-event:res.subscriptions,3gpp-as-session-with-qos:res.subscriptions:op.create;aef-zhejiang-hangzhou:3gpp-cp-parameter-provisioning,3gpp-pfd-management:res.transactions:op.read' Example 2: '3gpp#aef1:3gpp-time-sync:res.subscriptions:res.configurations:op.update,3gpp-mbs-session:res.mbs-sessions:res.subscriptions:op.create' With the following definitions: - The delimiter "#" shall be used only after the discriminator "3gpp" at the beginning of the scope field. - The delimiter ":" shall be used after the AEF identifier, after the API name and after each scope level. - The delimiter "," shall be used between API names of a certain AEF. - The delimiter ";" shall be used between the last API name of the one AEF section and the next AEF section. - Within the "aefId" field, the "apiName" field, the "scopeLevelType" field and "scopeLevelValue" field, the above defined delimiters and spaces are prohibited. - The "scopeLevelType" field shall be set to either "res" (for resource-level access control) or "op" (for operation-level access control). When the "CAPIF_Ext2" feature is supported and the "CAPIF_Ext1" feature is not supported, the content of this attribute shall be set to a list of space-delimited strings, as defined in clause 3.3 of IETF RFC 6749 [23]. |
| NOTE 1: The scope may contain more space-delimited strings which further add additional access ranges to the scope, the definition of those additional strings is out of the scope of the present document. NOTE 2: The "access_token", "token_type" and "expires_in" attributes do not follow the related naming convention defined in clause 7.2.1. These attributes are however kept as currently defined in this specification for backward compatibility considerations. NOTE 3: The enumeration value "Bearer" of the "token_type" attribute does not follow the related naming convention defined in clause 7.2.1. This enumeration is however kept as currently defined in this specification for backward compatibility considerations. |  |  |  |  |

* * * Next change * * * *

##### 8.5.4.2.8	Type: AccessTokenClaims

Table 8.5.4.2.8-1: Definition of type AccessTokenClaims

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| iss | string | M | 1 | This attribute shall contain the API invoker Identifier. |  |
| scope | string | M | 1 | Contains the OAuth2 scope. When neither the "CAPIF_Ext1" feature nor the "CAPIF_Ext2" feature are supported, it takes the following format: 3gpp#aefId1:apiName1,apiName2,…apiNameX;aefId2:apiName1,apiName2,…apiNameY;…aefIdN:apiName1,apiName2,…apiNameZ Using delimeter "#" after the discriminator "3gpp", ":" after AEF identifier, "," between API names and ";" between the last API name of the previous AEF identifier and the next AEF identifier. (NOTE) Example: '3gpp#aef-jiangsu-nanjing:3gpp-monitoring-event,3gpp-as-session-with-qos;aef-zhejiang-hangzhou:3gpp-cp-parameter-provisioning,3gpp-pfd-management' When the "CAPIF_Ext1" feature is supported and the "CAPIF_Ext2" feature is not supported, it takes the following format: '3gpp#aefId1:apiName1:scopeLevelType.scopeLevelValue:scopeLevelType.scopeLevelValue:…:scopeLevelType.scopeLevelValue,apiName2:scopeLevelType.scopeLevelValue:scopeLevelType.scopeLevelValue:…:scopeLevelType.scopeLevelValue,…apiNameX:scopeLevelType.scopeLevelValue:scopeLevelType.scopeLevelValue:…:scopeLevelType.scopeLevelValue;…aefIdN:apiName1:scopeLevelType.scopeLevelValue:scopeLevelType.scopeLevelValue:…:scopeLevelType.scopeLevelValue,apiName2:scopeLevelType.scopeLevelValue:scopeLevelType.scopeLevelValue:…:scopeLevelType.scopeLevelValue,…apiNameZ:scopeLevelType.scopeLevelValue:scopeLevelType.scopeLevelValue:…:scopeLevelType.scopeLevelValue' Example 1: '3gpp#aef1:3gpp-monitoring-event:res.subscriptions,3gpp-as-session-with-qos:res.subscriptions:op.create;aef-zhejiang-hangzhou:3gpp-cp-parameter-provisioning,3gpp-pfd-management:res.transactions:op.read' Example 2: '3gpp#aef1:3gpp-time-sync:res.subscriptions:res.configurations:op.update,3gpp-mbs-session:res.mbs-sessions:res.subscriptions:op.create' With the following definitions: - The delimiter "#" shall be used only after the discriminator "3gpp" at the beginning of the scope field. - The delimiter ":" shall be used after the AEF identifier, after the API name and after each scope level. - The delimiter "," shall be used between API names of a certain AEF. - The delimiter ";" shall be used between the last API name of the one AEF section and the next AEF section. - Within the "aefId" field, the "apiName" field, the "scopeLevelType" field and "scopeLevelValue" field, the above defined delimiters and spaces are prohibited. - The "scopeLevelType" field shall be set to either "res" (for resource-level access control) or "op" (for operation-level access control). When the "CAPIF_Ext2" feature is supported and the "CAPIF_Ext1" feature is not supported, the content of this attribute shall be set to a list of space-delimited strings, as defined in clause 3.3 of IETF RFC 6749 [23]. |  |
| exp | DurationSec | M | 1 | This attribute shall contain the number of seconds after which the access_token is considered to be expired as defined in clause 4.1.4 of IETF RFC 7519 [24]. |  |
| resOwnerId | ResOwnerId | O | 0..1 | Contains the identifier of the resource owner. This attribute shall be present only when the access token is used for RNAA. | RNAA |
| NOTE: The scope may contain more space-delimited strings which further add additional access ranges to the scope, the definition of those additional strings is out of the scope of the present document. |  |  |  |  |  |

* * * Next change * * * *

* * * End of changes * * * *