3GPP TSG-CT3 Meeting #143	C3-254512

# Sophia-Antipolis,France,13 Oct – 17 Oct 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.222 | CR | 0435 | rev | 1 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of OpenAPI add missing properties and remove incorrect description |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NBI19, CAPIF_Ph3 |  |  |  |  |  | Date: |  |  | 2025-09-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Current version of 29.222 is missing some summaries, tags and OperationIds into the OpenAPI implementation. CAPIF_Logging_API_Invocation_API contains the incorrect description of the resource that should be removed to avoid confusion. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding the missing summaries, tags and OperationsIds in the OpenAPI implementation of the 29.222 specification. Removal of incorrect description of the resource in CAPIF_Logging_API_Invocation_API. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification will remain incomplete missing some common properties of OpenAPI implementation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.7.2.1, A.2, A.3, A.5, A.6, A.7, A.8, A.9, A.10, A.11, A.12, A.13 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR provides backwards compatible corrections for the following APIs: TS29222_CAPIF_Discover_Service_API.yaml TS29222_CAPIF_Publish_Service_API.yaml TS29222_CAPIF_API_Invoker_Management_API.yaml TS29222_CAPIF_Security_API.yaml TS29222_CAPIF_Access_Control_Policy_API.yaml TS29222_CAPIF_Logging_API_Invocation_API.yaml TS29222_CAPIF_Auditing_API.yaml TS29222_CAPIF_API_Provider_Management_API.yaml TS29222_CAPIF_Routing_Info_API.yaml |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Proposed Changes

* * * First Change * * * *

#### 8.7.2.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 8.7.2.1-1 depicts the resource URIs structure for the CAPIF_Logging_API_Invocation_API.

Figure 8.7.2.1-1: Resource URI structure of the CAPIF_Logging_API_Invocation_API

Table 8.7.2.1-1 provides an overview of the resources and applicable HTTP methods.

Table 8.7.2.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| Logs | /{aefId}/logs | POST | Creates a new log entry for service API invocations |
| Individual log | /{aefId}/logs/{logId} | n/a | Individual log entry |

* * * Next Change * * * *

# A.2	CAPIF_Discover_Service_API

openapi: 3.0.0

info:

title: CAPIF_Discover_Service_API

description: |

API for discovering service APIs.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

version: "1.4.0-alpha.4"

externalDocs:

description: 3GPP TS 29.222 V19.4.0 Common API Framework for 3GPP Northbound APIs

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.222/

servers:

- url: '{apiRoot}/service-apis/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 7.5 of 3GPP TS 29.222.

paths:

/allServiceAPIs:

get:

description: >

Discover published service APIs and retrieve a collection of APIs according

to certain filter criteria.

operationId: GetPubServAPIs

tags:

- All published service APIs (Collection)

parameters:

- name: api-invoker-id

in: query

description: >

String identifying the API invoker assigned by the CAPIF core function.

It also represents the CCF identifier in the CAPIF-6/6e interface.

required: true

schema:

type: string

- name: api-name

in: query

description: >

Contains the API name set to the value of the "<apiName>" placeholder of the API URI as

defined in clause 5.2.4 of 3GPP TS 29.122 [14].

schema:

type: string

- name: api-version

in: query

description: API major version the URI (e.g. v1).

schema:

type: string

- name: comm-type

in: query

description: Communication type used by the API (e.g. REQUEST_RESPONSE).

schema:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/CommunicationType'

- name: protocol

in: query

description: Protocol used by the API.

schema:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/Protocol'

- name: aef-id

in: query

description: AEF identifer.

schema:

type: string

- name: data-format

in: query

description: Data formats used by the API (e.g. serialization protocol JSON used).

schema:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/DataFormat'

- name: api-cat

in: query

description: The service API category to which the service API belongs to.

schema:

type: string

- name: preferred-aef-loc

in: query

description: The preferred AEF location.

content:

application/json:

schema:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/AefLocation'

- name: req-api-prov-name

in: query

description: Represents the required API provider name.

schema:

type: string

- name: api-supported-features

in: query

description: >

Features supported by the discovered service API indicated by api-name parameter.

This may only be present if api-name query parameter is present.

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

- name: ue-ip-addr

in: query

description: Represents the UE IP address information.

content:

application/json:

schema:

$ref: '#/components/schemas/IpAddrInfo'

- name: service-kpis

in: query

description: >

Contains iInformation about service characteristics provided by the targeted

service API(s).

content:

application/json:

schema:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/ServiceKpis'

- name: net-slice-info

in: query

description: >

Contains the identifier(s) of the network slice(s) within which the API

shall be available.

content:

application/json:

schema:

type: array

items:

$ref: 'TS29435_NSCE_PolicyManagement.yaml#/components/schemas/NetSliceId'

minItems: 1

- name: grant-types

in: query

description: Contains the OAuth grant types that need to be supported.

required: false

content:

application/json:

schema:

type: array

items:

$ref: 'TS29222_CAPIF_Security_API.yaml#/components/schemas/OAuthGrantType'

minItems: 1

- name: api-ids

in: query

description: >

Contains the identifier(s) of the targeted service APIs.

When this query parameter is present, then all the other query parameters shall be

absent except the supported-features and api-invoker-id query parameters.

required: false

style: form

explode: false

schema:

type: array

items:

type: string

minItems: 1

- name: res-ops

in: query

description: >

Contains the list of supported API resource(s) and service operation(s).

required: false

style: form

explode: false

schema:

type: array

items:

$ref: '#/components/schemas/ResOperInfo'

minItems: 1

- name: supported-features

in: query

description: Features supported by the NF consumer for the CAPIF Discover Service API.

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: >

The response body contains the result of the search over the list of registered APIs.

content:

application/json:

schema:

$ref: '#/components/schemas/DiscoveredAPIs'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'414':

$ref: 'TS29122_CommonData.yaml#/components/responses/414'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

schemas:

DiscoveredAPIs:

type: object

description: >

Represents a list of APIs currently registered in the CAPIF core function

and satisfying a number of filter criteria provided by the API consumer.

properties:

serviceAPIDescriptions:

type: array

items:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/ServiceAPIDescription'

minItems: 1

description: >

Description of the service API as published by the service. Each service

API information shall include AEF profiles matching the filter criteria.

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

IpAddrInfo:

type: object

description: Represents the UE IP address information.

properties:

ipv4Addr:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Ipv4Addr'

ipv6Addr:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Ipv6Addr'

oneOf:

- required: [ipv4Addr]

- required: [ipv6Addr]

ResOperInfo:

type: object

description: >

Represents the resourse and/or service operation.

properties:

resource:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

operations:

type: array

items:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/Operation'

minItems: 1

customServOpers:

type: array

items:

type: string

minItems: 1

* * * Next Change * * * *

# A.3	CAPIF_Publish_Service_API

openapi: 3.0.0

info:

title: CAPIF_Publish_Service_API

description: |

API for publishing service APIs.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

version: "1.4.0-alpha.4"

externalDocs:

description: 3GPP TS 29.222 V19.3.0 Common API Framework for 3GPP Northbound APIs

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.222/

servers:

- url: '{apiRoot}/published-apis/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 7.5 of 3GPP TS 29.222.

paths:

# APF published API

/{apfId}/service-apis:

post:

description: Publish a new API.

operationId: CreateAPFPubAPIs

tags:

- APF published APIs (Collection)

parameters:

- name: apfId

in: path

required: true

schema:

type: string

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ServiceAPIDescription'

responses:

'201':

description: >

Service API published successfully The URI of the created resource

shall be returned in the "Location" HTTP header.

content:

application/json:

schema:

$ref: '#/components/schemas/ServiceAPIDescription'

headers:

Location:

description: >

Contains the URI of the newly created resource, according to the structure

{apiRoot}/published-apis/v1/{apfId}/service-apis/{serviceApiId}

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

get:

description: Retrieve all published APIs.

operationId: GetAPFPubAPIs

tags:

- APF published APIs (Collection)

parameters:

- name: apfId

in: path

required: true

schema:

type: string

responses:

'200':

description: >

Definition of all service API(s) published by the API publishing function.

If an empty array is provided, it indicates that there is no active published API at the

CCF.

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/ServiceAPIDescription'

minItems: 0

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

# Individual APF published API

/{apfId}/service-apis/{serviceApiId}:

get:

description: Retrieve a published service API.

operationId: GetIndAPFPubAPI

tags:

- Individual APF published API (Document)

parameters:

- name: serviceApiId

in: path

required: true

schema:

type: string

- name: apfId

in: path

required: true

schema:

type: string

responses:

'200':

description: Definition of all service API(s) published by the API publishing function.

content:

application/json:

schema:

$ref: '#/components/schemas/ServiceAPIDescription'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

description: Update a published service API.

operationId: UpdateIndAPFPubAPI

tags:

- Individual APF published API (Document)

parameters:

- name: serviceApiId

in: path

required: true

schema:

type: string

- name: apfId

in: path

required: true

schema:

type: string

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ServiceAPIDescription'

responses:

'200':

description: Definition of service API updated successfully.

content:

application/json:

schema:

$ref: '#/components/schemas/ServiceAPIDescription'

'204':

description: No Content

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

patch:

description: Modify an existing published service API.

operationId: ModifyIndAPFPubAPI

tags:

- Individual APF published API (Document)

parameters:

- name: serviceApiId

in: path

required: true

schema:

type: string

- name: apfId

in: path

required: true

schema:

type: string

requestBody:

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/ServiceAPIDescriptionPatch'

responses:

'200':

description: >

The definition of the service API is modified successfully and a

representation of the updated service API is returned in the request body.

content:

application/json:

schema:

$ref: '#/components/schemas/ServiceAPIDescription'

'204':

description: No Content. The definition of the service API is modified successfully.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

description: Unpublish a published service API.

operationId: DeleteIndAPFPubAPI

tags:

- Individual APF published API (Document)

parameters:

- name: serviceApiId

in: path

required: true

schema:

type: string

- name: apfId

in: path

required: true

schema:

type: string

responses:

'204':

description: The individual published service API matching the serviceAPiId is deleted.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

# Components

components:

schemas:

# Data Type for representations

ServiceAPIDescription:

type: object

description: Represents the description of a service API as published by the APF.

properties:

apiName:

type: string

description: >

Contains the API name set to the value of the "<apiName>" placeholder of the API URI as

defined in clause 5.2.4 of 3GPP TS 29.122 [14].

apiId:

type: string

description: >

API identifier assigned by the CAPIF core function to the published service API.

Shall not be present in the HTTP POST request from the API publishing function

to the CAPIF core function. Shall be present in the HTTP POST response from the

CAPIF core function to the API publishing function and in the HTTP GET response

from the CAPIF core function to the API invoker (discovery API).

apiStatus:

$ref: '#/components/schemas/ApiStatus'

aefProfiles:

type: array

items:

$ref: '#/components/schemas/AefProfile'

minItems: 1

description: >

AEF profile information, which includes the exposed API details (e.g. protocol).

description:

type: string

description: Text description of the API

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

shareableInfo:

$ref: '#/components/schemas/ShareableInformation'

serviceAPICategory:

type: string

description: The service API category to which the service API belongs to.

apiSuppFeats:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

pubApiPath:

$ref: '#/components/schemas/PublishedApiPath'

ccfId:

type: string

description: CAPIF core function identifier.

apiProvName:

type: string

description: Represents the API provider name.

netSliceInfo:

type: array

items:

$ref: 'TS29435_NSCE_PolicyManagement.yaml#/components/schemas/NetSliceId'

minItems: 1

description: Represents the applicable network slice identifiers.

required:

- apiName

InterfaceDescription:

type: object

description: Represents the description of an API's interface.

properties:

ipv4Addr:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Ipv4Addr'

ipv6Addr:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Ipv6Addr'

fqdn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Fqdn'

port:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Port'

apiPrefix:

type: string

description: >

A string representing a sequence of path segments that starts with the slash character.

securityMethods:

type: array

items:

$ref: '#/components/schemas/SecurityMethod'

minItems: 1

description: >

Security methods supported by the interface, it take precedence over

the security methods provided in AefProfile, for this specific interface.

grantTypes:

type: array

items:

$ref: 'TS29222_CAPIF_Security_API.yaml#/components/schemas/OAuthGrantType'

minItems: 1

oneOf:

- required: [ipv4Addr]

- required: [ipv6Addr]

- required: [fqdn]

AefProfile:

type: object

description: Represents the AEF profile data.

properties:

aefId:

type: string

description: Identifier of the API exposing function

versions:

type: array

items:

$ref: '#/components/schemas/Version'

minItems: 1

description: API version

protocol:

$ref: '#/components/schemas/Protocol'

dataFormat:

$ref: '#/components/schemas/DataFormat'

securityMethods:

type: array

items:

$ref: '#/components/schemas/SecurityMethod'

minItems: 1

description: Security methods supported by the AEF

grantTypes:

type: array

items:

$ref: 'TS29222_CAPIF_Security_API.yaml#/components/schemas/OAuthGrantType'

minItems: 1

domainName:

type: string

description: Domain to which API belongs to

interfaceDescriptions:

type: array

items:

$ref: '#/components/schemas/InterfaceDescription'

minItems: 1

description: Interface details

aefLocation:

$ref: '#/components/schemas/AefLocation'

serviceKpis:

$ref: '#/components/schemas/ServiceKpis'

ueIpRange:

$ref: '#/components/schemas/IpAddrRange'

required:

- aefId

- versions

oneOf:

- required: [domainName]

- required: [interfaceDescriptions]

Resource:

type: object

description: Represents the API resource data.

properties:

resourceName:

type: string

description: Resource name

commType:

$ref: '#/components/schemas/CommunicationType'

uri:

type: string

description: >

Relative URI of the API resource, it is set as {apiSpecificSuffixes} part

of the URI structure as defined in clause 5.2.4 of 3GPP TS 29.122.

custOpName:

type: string

description: >

it is set as {custOpName} part of the URI structure for a custom operation

associated with a resource as defined in clause 5.2.4 of 3GPP TS 29.122.

custOperations:

type: array

items:

$ref: '#/components/schemas/CustomOperation'

minItems: 1

description: >

Custom operations associated with this resource.

operations:

type: array

items:

$ref: '#/components/schemas/Operation'

minItems: 1

description: >

Supported HTTP methods for the API resource. Only applicable when the

protocol in AefProfile indicates HTTP.

description:

type: string

description: Text description of the API resource

required:

- resourceName

- commType

- uri

CustomOperation:

type: object

description: Represents the description of a custom operation.

properties:

commType:

$ref: '#/components/schemas/CommunicationType'

custOpName:

type: string

description: >

it is set as {custOpName} part of the URI structure for a custom operation

without resource association as defined in clause 5.2.4 of 3GPP TS 29.122.

operations:

type: array

items:

$ref: '#/components/schemas/Operation'

minItems: 1

description: >

Supported HTTP methods for the API resource. Only applicable when the

protocol in AefProfile indicates HTTP.

description:

type: string

description: Text description of the custom operation

required:

- commType

- custOpName

Version:

type: object

description: Represents the API version information.

properties:

apiVersion:

type: string

description: API major version in URI (e.g. v1)

expiry:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

resources:

type: array

items:

$ref: '#/components/schemas/Resource'

minItems: 1

description: Resources supported by the API.

custOperations:

type: array

items:

$ref: '#/components/schemas/CustomOperation'

minItems: 1

description: Custom operations without resource association.

required:

- apiVersion

ShareableInformation:

type: object

description: >

Indicates whether the service API and/or the service API category can be shared

to the list of CAPIF provider domains.

properties:

isShareable:

type: boolean

description: >

Indicates whether the service API and/or the service API category can be shared to the

list of CAPIF provider domain information.

true indicates that the service API and/or the service API category can be shared to the

list of CAPIF provider domain information.

false indicates that the service API and/or the service API category can not be shared

to the list of CAPIF provider domain information.

capifProvDoms:

type: array

items:

type: string

minItems: 1

description: >

List of CAPIF provider domains to which the service API information to be shared.

required:

- isShareable

PublishedApiPath:

type: object

description: Represents the published API path within the same CAPIF provider domain.

properties:

ccfIds:

type: array

items:

type: string

minItems: 1

description: A list of CCF identifiers where the service API is already published.

AefLocation:

description: >

Represents the location information (e.g. civic address, GPS coordinates, data center ID)

where the AEF providing the service API is located.

type: object

properties:

civicAddr:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/CivicAddress'

geoArea:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/GeographicArea'

dcId:

type: string

description: >

Identifies the data center where the AEF providing the service API is located.

ServiceAPIDescriptionPatch:

type: object

description: >

Represents the parameters to request the modification of an APF published API resource.

properties:

apiStatus:

$ref: '#/components/schemas/ApiStatus'

aefProfiles:

type: array

items:

$ref: '#/components/schemas/AefProfile'

description: AEF profile information, which includes the exposed API details.

minItems: 1

description:

type: string

description: Text description of the API

shareableInfo:

$ref: '#/components/schemas/ShareableInformation'

serviceAPICategory:

type: string

description: The service API category to which the service API belongs to.

apiSuppFeats:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

pubApiPath:

$ref: '#/components/schemas/PublishedApiPath'

ccfId:

type: string

description: CAPIF core function identifier.

ApiStatus:

type: object

description: >

Represents the API status.

properties:

aefIds:

type: array

items:

type: string

description: >

Indicates the list of AEF ID(s) where the API is active.

If an empty array is provided, it indicates that the API is inactive in all AEF(s).

required:

- aefIds

ServiceKpis:

type: object

description: >

Represents information about the service characteristics provided by a service API.

properties:

maxReqRate:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

maxRestime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DurationSec'

availability:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

avalComp:

type: string

pattern: '^\d+(\.\d+)? (kFLOPS|MFLOPS|GFLOPS|TFLOPS|PFLOPS|EFLOPS|ZFLOPS)$'

description: >

The maximum compute resource available in FLOPS for the API Invoker.

avalGraComp:

type: string

pattern: '^\d+(\.\d+)? (kFLOPS|MFLOPS|GFLOPS|TFLOPS|PFLOPS|EFLOPS|ZFLOPS)$'

description: >

The maximum graphical compute resource in FLOPS available for the API Invoker.

avalMem:

type: string

pattern: '^\d+(\.\d+)? (KB|MB|GB|TB|PB|EB|ZB|YB)$'

description: >

The maximum memory resource available for the API Invoker.

avalStor:

type: string

pattern: '^\d+(\.\d+)? (KB|MB|GB|TB|PB|EB|ZB|YB)$'

description: >

The maximum storage resource available for the API Invoker.

conBand:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

IpAddrRange:

description: Represents the list of public IP ranges

type: object

properties:

ueIpv4AddrRanges:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv4AddressRange'

description: Represents the IPv4 Address ranges of the UE(s).

minItems: 1

ueIpv6AddrRanges:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6AddressRange'

description: Represents the Ipv6 Address ranges of the UE(s).

minItems: 1

anyOf:

- required: [ueIpv4AddrRanges]

- required: [ueIpv6AddrRanges]

Protocol:

anyOf:

- type: string

enum:

- HTTP_1_1

- HTTP_2

- MQTT

- WEBSOCKET

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: |

Indicates a protocol and protocol version used by the API.

Possible values are:

- HTTP_1_1: Indicates that the protocol is HTTP version 1.1.

- HTTP_2: Indicates that the protocol is HTTP version 2.

- MQTT: Indicates that the protocol is Message Queuing Telemetry Transport.

- WEBSOCKET: Indicates that the protocol is Websocket.

CommunicationType:

anyOf:

- type: string

enum:

- REQUEST_RESPONSE

- SUBSCRIBE_NOTIFY

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: |

Indicates a communication type of the resource or the custom operation.

Possible values are:

- REQUEST_RESPONSE: The communication is of the type request-response.

- SUBSCRIBE_NOTIFY: The communication is of the type subscribe-notify.

DataFormat:

anyOf:

- type: string

enum:

- JSON

- XML

- PROTOBUF3

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: |

Indicates a data format.

Possible values are:

- JSON: Indicates that the data format is JSON.

- XML: Indicates that the data format is Extensible Markup Language.

- PROTOBUF3: Indicates that the data format is Protocol buffers version 3.

SecurityMethod:

anyOf:

- type: string

enum:

- PSK

- PKI

- OAUTH

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: |

Indicates the security method.

Possible values are:

- PSK: Security method 1 (Using TLS-PSK) as described in 3GPP TS 33.122.

- PKI: Security method 2 (Using PKI) as described in 3GPP TS 33.122.

- OAUTH: Security method 3 (TLS with OAuth token) as described in 3GPP TS 33.122.

Operation:

anyOf:

- type: string

enum:

- GET

- POST

- PUT

- PATCH

- DELETE

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: |

Indicates an HTTP method.

Possible values are:

- GET: HTTP GET method.

- POST: HTTP POST method.

- PUT: HTTP PUT method.

- PATCH: HTTP PATCH method.

- DELETE: HTTP DELETE method.

* * * Next Change * * * *

# A.5	CAPIF_API_Invoker_Management_API

openapi: 3.0.0

info:

title: CAPIF_API_Invoker_Management_API

version: 1.4.0-alpha.5

description: |

API for API invoker management.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.222 V19.4.0 Common API Framework for 3GPP Northbound APIs

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.222/

servers:

- url: '{apiRoot}/api-invoker-management/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 7.5 of 3GPP TS 29.222

paths:

/onboardedInvokers:

post:

summary: Request the Creation of a new On-boarded API Invoker.

operationId: CreateOnboardedAPIInvoker

tags:

- On-boarded API Invokers (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/APIInvokerEnrolmentDetails'

responses:

'201':

description: >

Created. The API Invoker is successfully on-boarded.

content:

application/json:

schema:

$ref: '#/components/schemas/APIInvokerEnrolmentDetails'

headers:

Location:

description: >

Contains the URI of the newly created resource.

required: true

schema:

type: string

'202':

description: >

Accepted. The CCF accepted the Onboarding request and is processing it.

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

callbacks:

notificationDestination:

'{$request.body#/notificationDestination}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/OnboardingNotification'

responses:

'204':

description: >

No Content. The onboarding notification is successfully received.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/onboardedInvokers/{onboardingId}:

parameters:

- name: onboardingId

in: path

required: true

schema:

type: string

delete:

description: Deletes an existing Individual On-boarded API Invoker.

summary: Delete an existing Individual On-boarded API Invoker resource.

operationId: DeleteIndOnboardedAPIInvoker

tags:

- Individual On-boarded API Invoker (Document)

responses:

'204':

description: >

No Content. The Individual On-boarded API Invoker resource is successfully deleted.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

summary: Update an existing Individual On-boarded API Invoker resource.

operationId: UpdateIndOnboardedAPIInvoker

tags:

- Individual On-boarded API Invoker (Document)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/APIInvokerEnrolmentDetails'

responses:

'200':

description: >

OK. The Individual On-boarded API Invoker resource is successfully updated and the

representation of the updated resource is returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/APIInvokerEnrolmentDetails'

'202':

description: >

Accepted. The request is accepted and under processing.

'204':

description: >

No Content. The Individual On-boarded API Invoker resource is successfully updated

and no content is returned in the response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

patch:

summary: Modify an existing Individual On-boarded API Invoker resource.

operationId: ModifyIndOnboardedAPIInvoker

tags:

- Individual On-boarded API Invoker (Document)

requestBody:

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/APIInvokerEnrolmentDetailsPatch'

responses:

'200':

description: >

OK. The Individual On-boarded API Invoker resource is successfully modified and the

representation of the updated resource is returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/APIInvokerEnrolmentDetails'

'202':

description: >

Accepted. The request is accepted and under processing.

'204':

description: >

No Content. The Individual On-boarded API Invoker resource is successfully modified

and no content is returned in the response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

#

# STRUCTURED DATA TYPES

#

OnboardingInformation:

type: object

description: Represents the on-boarding information of the API Invoker.

properties:

apiInvokerPublicKey:

type: string

description: The API Invoker's public key

apiInvokerCertificate:

type: string

description: >

The API Invoker's generic client certificate, provided by the CAPIF core function.

onboardingSecret:

type: string

description: >

The API Invoker's onboarding secret, provided by the CAPIF core function.

onboardingCriteria:

type: array

items:

$ref: '#/components/schemas/OnboardingCriteria'

minItems: 1

required:

- apiInvokerPublicKey

APIList:

type: object

description: Represents a list of APIs with the corresponding information.

properties:

serviceAPIDescriptions:

type: array

items:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/ServiceAPIDescription'

minItems: 1

description: >

Represents the list of service APIs that the API Invoker is allowed to invoke.

APIInvokerEnrolmentDetails:

description: Represents the onboarding information of the API Invoker.

type: object

properties:

apiInvokerId:

type: string

description: >

API invoker ID assigned by the CAPIF core function to the API invoker while

on-boarding the API invoker. Shall not be present in the HTTP POST request

from the API invoker to the CAPIF core function, to on-board itself. Shall be

present in all other HTTP requests and responses.

readOnly: true

onboardingInformation:

$ref: '#/components/schemas/OnboardingInformation'

notificationDestination:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

requestTestNotification:

type: boolean

description: >

Set to true to request the CCF to send a

test notification as defined in in clause 7.6.

Set to false or omitted otherwise.

websockNotifConfig:

$ref: 'TS29122_CommonData.yaml#/components/schemas/WebsockNotifConfig'

apiList:

$ref: '#/components/schemas/APIList'

apiInvokerInformation:

type: string

description: >

Generic information related to the API invoker such as details of

the device or the application.

expTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

failureReasons:

type: array

items:

$ref: '#/components/schemas/EnrolFailReason'

minItems: 1

required:

- onboardingInformation

- notificationDestination

OnboardingNotification:

type: object

description: Represents a notification of on-boarding creation or update result.

properties:

result:

type: boolean

description: Set to "true" to indicate successful on-boarding. Otherwise set to "false".

resourceLocation:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

apiInvokerEnrolmentDetails:

$ref: '#/components/schemas/APIInvokerEnrolmentDetails'

apiList:

$ref: '#/components/schemas/APIList'

required:

- result

APIInvokerEnrolmentDetailsPatch:

type: object

description: Represents an API Invoker's enrolment details to be updated.

properties:

onboardingInformation:

$ref: '#/components/schemas/OnboardingInformation'

notificationDestination:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

apiList:

$ref: '#/components/schemas/APIList'

apiInvokerInformation:

type: string

description: >

Generic information related to the API invoker such as details of

the device or the application.

expTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTimeRm'

OnboardingCriteria:

type: object

description: Represents the onboarding criteria information.

properties:

secMethods:

type: array

items:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/SecurityMethod'

minItems: 1

relatedCriteria:

$ref: '#/components/schemas/RelatedCriteria'

required:

- secMethods

RelatedCriteria:

type: object

description: Represents onboarding related criteria.

properties:

aefIds:

type: array

items:

type: string

minItems: 1

apis:

type: array

items:

$ref: '#/components/schemas/ApiInfo'

minItems: 1

serviceAPICategories:

type: array

items:

type: string

minItems: 1

anyOf:

- required: [aefIds]

- required: [serviceAPICategories]

- required: [apis]

ApiInfo:

type: object

description: Represents service API identification related information.

properties:

apiName:

type: string

supportedApiVersions:

type: array

items:

type: string

minItems: 1

required:

- apiName

EnrolFailReason:

type: object

description: Represents the failure reason for not those APIs not enrolled successfully.

properties:

apiName:

items:

type: string

failureCode:

$ref: '#/components/schemas/EnrolFailCause'

required:

- apiName

- failureCode

# SIMPLE DATA TYPES

#

# ENUMERATIONS DATA TYPES

#

EnrolFailCause:

anyOf:

- type: string

enum:

- AUTHORIZATION_ISSUE

- ONBOARDING_CRI_NOT_MET

- UNSPECIFIED

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration but

is not used to encode content defined in the present version of this API.

description: |

Represents API Invoker's per API enrollment failure code.

Possible values are:

- AUTHORIZATION_ISSUE: Indicates that the service API enrollment failed because the service

API is not authorized for the API Invoker.

- ONBOARDING_CRI_NOT_MET: Indicates that the service API entollment failed because the

onboarding criteria not met for this service API for the API Invoker.

- UNSPECIFIED: Indicates that the service API enrollment failed due to an unspecified

reason.

OnboardingFailReason:

anyOf:

- type: string

enum:

- API_INVOKER_NOT_ALLOWED

- ONBOARDING_CRI_NOT_MET

- OTHER

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

and is not used to encode content defined in the present version of this API.

description: |

Represents the API Invoker onboarding failure reason.

Possible values are:

- API_INVOKER_NOT_ALLOWED: Indicates that the onboarding of the API Invoker failed because

the API Invoker is not allowed.

- ONBOARDING_CRI_NOT_MET: Indicates that the onboarding of the API Invoker failed because

the onboarding criteria is not met.

- OTHER: Indicates that the onboarding of the API Invoker failed because of other reasons.

# Data types describing alternative data types or combinations of data types:

#

* * * Next Change * * * *

# A.6	CAPIF_Security_API

openapi: 3.0.0

info:

title: CAPIF_Security_API

description: |

API for CAPIF security management.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

version: "1.4.0-alpha.2"

externalDocs:

description: 3GPP TS 29.222 V19.4.0 Common API Framework for 3GPP Northbound APIs

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.222/

servers:

- url: '{apiRoot}/capif-security/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 7.5 of 3GPP TS 29.222.

paths:

/trustedInvokers/{apiInvokerId}:

get:

summary: Retrieve authentication information of an API invoker.

operationId: GetSecIndTrustedAPIInv

tags:

- Individual trusted API invoker (Store)

parameters:

- name: apiInvokerId

in: path

description: Identifier of an individual API invoker

required: true

schema:

type: string

- name: authenticationInfo

in: query

description: >

When set to 'true', it indicates the CAPIF core function to send the

authentication information of the API invoker. Set to false or omitted otherwise.

schema:

type: boolean

- name: authorizationInfo

in: query

description: >

When set to 'true', it indicates the CAPIF core function to send the

authorization information of the API invoker. Set to false or omitted otherwise.

schema:

type: boolean

responses:

'200':

description: >

The security related information of the API Invoker based on the request

from the API exposing function.

content:

application/json:

schema:

$ref: '#/components/schemas/ServiceSecurity'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'414':

$ref: 'TS29122_CommonData.yaml#/components/responses/414'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

summary: Create a security context for individual API invoker.

operationId: CreateSecIndTrustedAPIInv

tags:

- Individual trusted API invoker (Store)

parameters:

- name: apiInvokerId

in: path

description: Identifier of an individual API invoker

required: true

schema:

type: string

requestBody:

description: create a security context for an API invoker

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ServiceSecurity'

callbacks:

notificationDestination:

'{$request.body#/notificationDestination}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/SecurityNotification'

responses:

'204':

description: No Content (successful notification)

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

responses:

'201':

description: Successful created.

content:

application/json:

schema:

$ref: '#/components/schemas/ServiceSecurity'

headers:

Location:

description: >

Contains the URI of the newly created resource, according to the structure

{apiRoot}/capif-security/v1/trustedInvokers/{apiInvokerId}

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'414':

$ref: 'TS29122_CommonData.yaml#/components/responses/414'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

summary: Delete authorization of the individual trusted API invoker.

operationId: DeleteSecIndTrustedAPIInv

tags:

- Individual trusted API invoker (Store)

parameters:

- name: apiInvokerId

in: path

description: Identifier of an individual API invoker

required: true

schema:

type: string

responses:

'204':

description: No Content (Successful deletion of the existing subscription)

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/trustedInvokers/{apiInvokerId}/update:

post:

summary: Update security context.

operationId: UpdateSecContIndAPIInv

tags:

- Individual trusted API invoker (Store)

parameters:

- name: apiInvokerId

in: path

description: Identifier of an individual API invoker

required: true

schema:

type: string

requestBody:

description: Update the security context (e.g. re-negotiate the security methods).

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ServiceSecurity'

responses:

'200':

description: Successful updated.

content:

application/json:

schema:

$ref: '#/components/schemas/ServiceSecurity'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/trustedInvokers/{apiInvokerId}/delete:

post:

summary: Delete authorization of API invoker for some APIs.

operationId: DeleteAuthIndAPIInvForAPIs

tags:

- Individual trusted API invoker (Store)

parameters:

- name: apiInvokerId

in: path

description: Identifier of an individual API invoker

required: true

schema:

type: string

requestBody:

description: Revoke the authorization of the API invoker for APIs.

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/SecurityNotification'

responses:

'204':

description: Successful revoked.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/securities/{securityId}/token:

post:

summary: Obtain the OAuth 2.0 authorization information.

operationId: GetOAuthIndAPIInv

tags:

- Individual trusted API invoker (Store)

parameters:

- name: securityId

in: path

description: Identifier of an individual API invoker

required: true

schema:

type: string

requestBody:

required: true

content:

application/x-www-form-urlencoded:

schema:

$ref: '#/components/schemas/AccessTokenReq'

responses:

'200':

description: Successful Access Token Request

content:

application/json:

schema:

$ref: '#/components/schemas/AccessTokenRsp'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

description: Error in the Access Token Request

content:

application/json:

schema:

$ref: '#/components/schemas/AccessTokenErr'

'401':

description: Unauthorized

content:

application/json:

schema:

$ref: '#/components/schemas/AccessTokenErr'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

components:

schemas:

ServiceSecurity:

type: object

description: >

Represents the details of the security method for each service API interface.

When included by the API invoker, it indicates the preferred method of security.

When included by the CAPIF core function, it indicates the security method to be

used for the service API interface.

properties:

securityInfo:

type: array

items:

$ref: '#/components/schemas/SecurityInformation'

minimum: 1

notificationDestination:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

requestTestNotification:

type: boolean

description: >

Set to true by API invoker to request the CAPIF core function to send a

test notification as defined in in clause 7.6. Set to false or omitted otherwise.

websockNotifConfig:

$ref: 'TS29122_CommonData.yaml#/components/schemas/WebsockNotifConfig'

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- securityInfo

- notificationDestination

SecurityInformation:

type: object

description: Represents the interface details and the security method.

properties:

interfaceDetails:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/InterfaceDescription'

aefId:

type: string

description: Identifier of the API exposing function

apiId:

type: string

description: API identifier

prefSecurityMethods:

type: array

items:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/SecurityMethod'

minItems: 1

description: Security methods preferred by the API invoker for the API interface.

selSecurityMethod:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/SecurityMethod'

authenticationInfo:

type: string

description: Authentication related information

authorizationInfo:

type: string

description: Authorization related information

grantType:

type: array

items:

$ref: '#/components/schemas/OAuthGrantType'

minItems: 1

required:

- prefSecurityMethods

oneOf:

- required: [interfaceDetails]

- required: [aefId]

SecurityNotification:

type: object

description: Represents the revoked authorization notification details.

properties:

apiInvokerId:

type: string

description: String identifying the API invoker assigned by the CAPIF core function.

aefId:

type: string

description: String identifying the AEF.

apiIds:

type: array

items:

type: string

minItems: 1

description: Identifier of the service API

access_token:

type: string

description: >

JWS Compact Serialized representation of JWS signed JSON object (AccessTokenClaims).

cause:

$ref: '#/components/schemas/Cause'

required:

- apiInvokerId

- apiIds

- cause

AccessTokenReq:

format: x-www-form-urlencoded

description: Represents the access token request information.

properties:

grant_type:

type: string

enum:

- client_credentials

- authorization_code

client_id:

type: string

resOwnerId:

$ref: '#/components/schemas/ResOwnerId'

client_secret:

type: string

scope:

type: string

authCode:

type: string

redirect_uri:

type: string

required:

- grant_type

- client_id

AccessTokenRsp:

type: object

description: Represents the access token response information.

properties:

access_token:

type: string

description: >

JWS Compact Serialized representation of JWS signed JSON object (AccessTokenClaims)

token_type:

type: string

enum:

- Bearer

expires_in:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DurationSec'

scope:

type: string

required:

- access_token

- token_type

- expires_in

AccessTokenClaims:

type: object

description: Represents the claims data structure for the access token.

properties:

iss:

type: string

scope:

type: string

exp:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DurationSec'

resOwnerId:

$ref: '#/components/schemas/ResOwnerId'

required:

- iss

- scope

- exp

ResOwnerId:

type: object

description: >

Represents the identifier of the resource owner.

properties:

gpsi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

anyOf:

- required: [gpsi]

AccessTokenErr:

type: object

description: Represents an error in the access token request.

properties:

error:

type: string

enum:

- invalid_request

- invalid_client

- invalid_grant

- unauthorized_client

- unsupported_grant_type

- invalid_scope

error_description:

type: string

error_uri:

type: string

required:

- error

Cause:

anyOf:

- type: string

enum:

- OVERLIMIT_USAGE

- UNEXPECTED_REASON

- AUTHORIZATION_ISSUE

- OTHER_REASON

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: |

Indicates the cause for revoking the API invoker's authorization to the service API.

Possible values are:

- OVERLIMIT_USAGE:

The revocation of the authorization of the API invoker is due to the overlimit

usage of the service API

- UNEXPECTED_REASON:

The revocation of the authorization of the API invoker is due to unexpected reason.

- AUTHORIZATION_ISSUE:

The revocation of the authorization of the API invoker is due to API Invoker

not being authorized anymore by the API Provider.

- OTHER_REASON:

The revocation of the authorization of the API invoker is due to other reason.

OAuthGrantType:

anyOf:

- type: string

enum:

- CLIENT_CREDENTIALS

- AUTHORIZATION_CODE

- AUTHORIZATION_CODE_WITH_PKCE

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration and

is not used to encode content defined in the present version of this API.

description: |

Indicates the supported authorization flow (e.g. client credentials flow, authorization code         flow, etc.) to the API invoker.

Possible values are:

- CLIENT_CREDENTIALS: Indicate that the grant type is is client credentials flow.

- AUTHORIZATION_CODE: Indicate that the grant type is authorization code.

- AUTHORIZATION_CODE_WITH_PKCE: Indicate that the grant type is authorization code with PKCE.

* * * Next Change * * * *

# A.7	CAPIF_Access_Control_Policy_API

openapi: 3.0.0

info:

title: CAPIF_Access_Control_Policy_API

description: |

API for access control policy.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

version: "1.4.0-alpha.1"

externalDocs:

description: 3GPP TS 29.222 V19.4.0 Common API Framework for 3GPP Northbound APIs

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.222/

servers:

- url: '{apiRoot}/access-control-policy/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 7.5 of 3GPP TS 29.222

paths:

/accessControlPolicyList/{serviceApiId}:

get:

description: Retrieves the access control policy list.

operationId: GetAccContPolList

tags:

- Access Control Policy List (Collection)

parameters:

- name: serviceApiId

in: path

description: Identifier of a published service API

required: true

schema:

type: string

- name: aef-id

in: query

required: true

description: Identifier of the AEF

schema:

type: string

- name: api-invoker-id

in: query

description: Identifier of the API invoker

schema:

type: string

- name: supported-features

in: query

description: To filter irrelevant responses related to unsupported features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: OK.

content:

application/json:

schema:

$ref: '#/components/schemas/AccessControlPolicyList'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'414':

$ref: 'TS29122_CommonData.yaml#/components/responses/414'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

schemas:

AccessControlPolicyList:

type: object

description: Represents the access control policy list for a published service API.

properties:

apiInvokerPolicies:

type: array

items:

$ref: '#/components/schemas/ApiInvokerPolicy'

minItems: 0

description: Policy of each API invoker.

ApiInvokerPolicy:

type: object

description: Represents the policy of an API Invoker.

properties:

apiInvokerId:

type: string

description: API invoker ID assigned by the CAPIF core function

allowedTotalInvocations:

type: integer

description: Total number of invocations allowed on the service API by the API invoker.

allowedInvocationsPerSecond:

type: integer

description: Invocations per second allowed on the service API by the API invoker.

allowedInvocationTimeRangeList:

type: array

items:

$ref: '#/components/schemas/TimeRangeList'

minItems: 0

description: >

The time ranges during which the invocations are allowed on the service API

by the API invoker.

netSliceInfo:

type: array

items:

$ref: 'TS29435_NSCE_PolicyManagement.yaml#/components/schemas/NetSliceId'

minItems: 1

required:

- apiInvokerId

TimeRangeList:

type: object

description: >

Represents the time range during which the invocation of a service API is allowed

by the API invoker.

properties:

startTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

stopTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

* * * Next Change * * * *

# A.8	CAPIF_Logging_API_Invocation_API

openapi: 3.0.0

info:

title: CAPIF_Logging_API_Invocation_API

description: |

API for invocation logs.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

version: "1.4.0-alpha.2"

externalDocs:

description: 3GPP TS 29.222 V19.3.0 Common API Framework for 3GPP Northbound APIs

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.222/

servers:

- url: '{apiRoot}/api-invocation-logs/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 7.5 of 3GPP TS 29.222

paths:

/{aefId}/logs:

post:

description: Creates a new log entry for service API invocations.

operationId: CreateLogEntServAPIInv

tags:

- Logs (Collection)

parameters:

- name: aefId

in: path

description: Identifier of the API exposing function

required: true

schema:

type: string

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/InvocationLog'

responses:

'201':

description: >

Log of service API invocations provided by API exposing function successfully

stored on the CAPIF core function.

content:

application/json:

schema:

$ref: '#/components/schemas/InvocationLog'

headers:

Location:

description: >

Contains the URI of the newly created resource, according to the structure

{apiRoot}/api-invocation-logs/v1/{aefId}/logs/{logId}

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/{aefId}/logs/{logId}:

parameters:

- name: aefId

in: path

description: Identifier of the API exposing function

required: true

schema:

type: string

- name: logId

in: path

description: Identifier of individual log entry

required: true

schema:

type: string

components:

schemas:

InvocationLog:

type: object

description: >

Represents a set of Service API invocation logs to be stored in a CAPIF core function.

properties:

aefId:

type: string

description: >

Identity information of the API exposing function requesting logging of

service API invocations

apiInvokerId:

type: string

description: Identity of the API invoker which invoked the service API

logs:

type: array

items:

$ref: '#/components/schemas/Log'

minItems: 1

description: Service API invocation log

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- aefId

- apiInvokerId

- logs

Log:

type: object

description: Represents an individual service API invocation log entry.

properties:

apiId:

type: string

description: String identifying the API invoked.

apiName:

type: string

description: >

Contains the invoked API name set to the value of the "<apiName>" placeholder of the API

URI as defined in clause 5.2.4 of 3GPP TS 29.122 [14].

apiVersion:

type: string

description: Version of the API which was invoked

resourceName:

type: string

description: Name of the specific resource invoked

uri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

protocol:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/Protocol'

operation:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/Operation'

result:

type: string

description: For HTTP protocol, it contains HTTP status code of the invocation

invocationTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

invocationLatency:

$ref: '#/components/schemas/DurationMs'

inputParameters:

description: >

List of input parameters. Can be any value - string, number, boolean, array or object.

outputParameters:

description: >

List of output parameters. Can be any value - string, number, boolean, array or object.

srcInterface:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/InterfaceDescription'

destInterface:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/InterfaceDescription'

fwdInterface:

type: string

description: >

It includes the node identifier (as defined in IETF RFC 7239 of all forwarding

entities between the API invoker and the AEF, concatenated with comma and space,

e.g. 192.0.2.43:80, unknown:_OBFport, 203.0.113.60

netSliceInfo:

$ref: 'TS29435_NSCE_PolicyManagement.yaml#/components/schemas/NetSliceId'

required:

- apiId

- apiName

- apiVersion

- resourceName

- protocol

- result

DurationMs:

type: integer

description: Represents a period of time in units of milliseconds.

minimum: 0

* * * Next Change * * * *

# A.9	CAPIF_Auditing_API

openapi: 3.0.0

info:

title: CAPIF_Auditing_API

description: |

API for auditing.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

version: "1.4.0-alpha.2"

externalDocs:

description: 3GPP TS 29.222 V19.3.0 Common API Framework for 3GPP Northbound APIs

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.222/

servers:

- url: '{apiRoot}/logs/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 7.5 of 3GPP TS 29.222.

paths:

/apiInvocationLogs:

get:

description: Query and retrieve service API invocation logs stored on the CAPIF core function.

operationId: GetAPIInvLogs

tags:

- All service API invocation logs (Collection)

parameters:

- name: aef-id

in: query

description: String identifying the API exposing function.

schema:

type: string

- name: api-invoker-id

in: query

description: String identifying the API invoker which invoked the service API.

schema:

type: string

- name: time-range-start

in: query

description: Start time of the invocation time range.

schema:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

- name: time-range-end

in: query

description: End time of the invocation time range.

schema:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

- name: api-id

in: query

description: String identifying the API invoked.

schema:

type: string

- name: api-name

in: query

description: >

Contains the API name set to the value of the "<apiName>" placeholder of the API URI as

defined in clause 5.2.4 of 3GPP TS 29.122 [14].

schema:

type: string

- name: api-version

in: query

description: Version of the API which was invoked.

schema:

type: string

- name: protocol

in: query

description: Protocol invoked.

schema:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/Protocol'

- name: operation

in: query

description: Operation that was invoked on the API.

schema:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/Operation'

- name: result

in: query

description: Result or output of the invocation.

schema:

type: string

- name: resource-name

in: query

description: Name of the specific resource invoked.

schema:

type: string

- name: src-interface

in: query

description: Interface description of the API invoker.

content:

application/json:

schema:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/InterfaceDescription'

- name: dest-interface

in: query

description: Interface description of the API invoked.

content:

application/json:

schema:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/InterfaceDescription'

- name: supported-features

in: query

description: To filter irrelevant responses related to unsupported features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

- name: net-slice-info

in: query

description: >

Contains the identifier(s) of the network slice(s) within which

the API shall be available.

content:

application/json:

schema:

type: array

items:

$ref: 'TS29435_NSCE_PolicyManagement.yaml#/components/schemas/NetSliceId'

minItems: 1

responses:

'200':

description: >

Result of the query operation along with fetched service API invocation log data.

content:

application/json:

schema:

$ref: '#/components/schemas/InvocationLogsRetrieveRes'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'414':

$ref: 'TS29122_CommonData.yaml#/components/responses/414'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

schemas:

InvocationLogs:

type: object

description: >

Represents several (more than one) invocation logs.

properties:

multipleInvocationLogs:

type: array

items:

$ref: 'TS29222_CAPIF_Logging_API_Invocation_API.yaml#/components/schemas/InvocationLog'

minItems: 1

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- multipleInvocationLogs

InvocationLogsRetrieveRes:

description: >

Represents the result of an invocation logs retrieval request.

oneOf:

- $ref: 'TS29222_CAPIF_Logging_API_Invocation_API.yaml#/components/schemas/InvocationLog'

- $ref: '#/components/schemas/InvocationLogs'

* * * Next Change * * * *

# A.10	AEF_Security_API

openapi: 3.0.0

info:

title: AEF_Security_API

description: |

API for AEF security management.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

version: "1.4.0-alpha.1"

externalDocs:

description: 3GPP TS 29.222 V19.4.0 Common API Framework for 3GPP Northbound APIs

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.222/

servers:

- url: '{apiRoot}/aef-security/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 7.5 of 3GPP TS 29.222.

paths:

/check-authentication:

post:

summary: Check authentication.

operationId: CheckAuthReq

tags:

- Check Authentication Request

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/CheckAuthenticationReq'

responses:

'200':

description: The request was successful.

content:

application/json:

schema:

$ref: '#/components/schemas/CheckAuthenticationRsp'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/revoke-authorization:

post:

summary: Revoke authorization for service APIs.

operationId: RevokeAuthServAPIs

tags:

- Revoke Authorization Request

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/RevokeAuthorizationReq'

responses:

'200':

description: The request was successful.

content:

application/json:

schema:

$ref: '#/components/schemas/RevokeAuthorizationRsp'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

schemas:

CheckAuthenticationReq:

type: object

description: Represents authentication check request data.

properties:

apiInvokerId:

type: string

description: >

API invoker ID assigned by the CAPIF core function to the API invoker

while on-boarding the API invoker.

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- apiInvokerId

- supportedFeatures

CheckAuthenticationRsp:

type: object

description: Represents authentication check response data.

properties:

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- supportedFeatures

RevokeAuthorizationReq:

type: object

description: Represents authorization revocation request data.

properties:

revokeInfo:

$ref: 'TS29222_CAPIF_Security_API.yaml#/components/schemas/SecurityNotification'

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- revokeInfo

- supportedFeatures

RevokeAuthorizationRsp:

type: object

description: Represents authorization revocation response data.

properties:

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- supportedFeatures

* * * Next Change * * * *

# A.11	CAPIF_API_Provider_Management_API

openapi: 3.0.0

info:

title: CAPIF_API_Provider_Management_API

description: |

API for API provider domain functions management.

© 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

version: "1.2.0"

externalDocs:

description: 3GPP TS 29.222 V18.6.0 Common API Framework for 3GPP Northbound APIs

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.222/

servers:

- url: '{apiRoot}/api-provider-management/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 7.5 of 3GPP TS 29.222

paths:

/registrations:

post:

description: Registers a new API Provider domain with API provider domain functions profiles.

operationId: CreateApiProvRegist

tags:

- All API Provider Domains Registrations (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/APIProviderEnrolmentDetails'

responses:

'201':

description: API provider domain registered successfully

content:

application/json:

schema:

$ref: '#/components/schemas/APIProviderEnrolmentDetails'

headers:

Location:

description: >

Contains the URI of the newly created resource, according to the structure

{apiRoot}/api-provider-management/v1/registrations/{registrationId}

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/registrations/{registrationId}:

delete:

description: Deregisters API provider domain by deleting API provider domain and functions.

operationId: DeleteIndApiProvDomainRegist

tags:

- Individual API Provider Domain Registration (Document)

parameters:

- name: registrationId

in: path

description: String identifying an registered API provider domain resource.

required: true

schema:

type: string

responses:

'204':

description: The API provider domain matching registrationId is deleted.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

description: Updates an API provider domain's registration details.

operationId: UpdateIndApiProvDomainRegist

tags:

- Individual API Provider Domain Registration (Document)

parameters:

- name: registrationId

in: path

description: String identifying an registered API provider domain resource.

required: true

schema:

type: string

requestBody:

description: >

Representation of the API provider domain registration details to be updated

in CAPIF core function.

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/APIProviderEnrolmentDetails'

responses:

'200':

description: API provider domain registration details updated successfully.

content:

application/json:

schema:

$ref: '#/components/schemas/APIProviderEnrolmentDetails'

'204':

description: No Content

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

patch:

description: Modify an individual API provider details.

operationId: ModifyIndApiProvDomainRegist

tags:

- Individual API Provider Domain Registration (Document)

parameters:

- name: registrationId

in: path

required: true

schema:

type: string

requestBody:

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/APIProviderEnrolmentDetailsPatch'

responses:

'200':

description: >

The definition of the service API is modified successfully and a

representation of the updated service API is returned in the request body.

content:

application/json:

schema:

$ref: '#/components/schemas/APIProviderEnrolmentDetails'

'204':

description: No Content. The definition of the service API is modified successfully.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

schemas:

APIProviderEnrolmentDetails:

type: object

description: Represents an API provider domain's enrolment details.

properties:

apiProvDomId:

type: string

description: >

API provider domain ID assigned by the CAPIF core function to the API management

function while registering the API provider domain. Shall not be present in the

HTTP POST request from the API Management function to the CAPIF core function,

to on-board itself. Shall be present in all other HTTP requests and responses.

readOnly: true

regSec:

type: string

description: >

Security information necessary for the CAPIF core function to validate the

registration of the API provider domain. Shall be present in HTTP POST request

from API management function to CAPIF core function for API provider domain

registration.

apiProvFuncs:

type: array

items:

$ref: '#/components/schemas/APIProviderFunctionDetails'

minItems: 1

description: >

A list of individual API provider domain functions details. When included by

the API management function in the HTTP request message, it lists the API

provider domain functions that the API management function intends to

register/update in registration or update registration procedure. When

included by the CAPIF core function in the HTTP response message, it lists

the API domain functions details that are registered or updated successfully.

apiProvDomInfo:

type: string

description: >

Generic information related to the API provider domain such as details

of the API provider applications.

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

failReason:

type: string

description: >

Registration or update specific failure information of failed API provider

domain function registrations.Shall be present in the HTTP response

body if atleast one of the API provider domain function registration or update

registration fails.

apiProvName:

type: string

description: Represents the API provider name.

required:

- regSec

APIProviderFunctionDetails:

type: object

description: Represents an API provider domain function's details.

properties:

apiProvFuncId:

type: string

description: >

API provider domain functionID assigned by the CAPIF core function to the

API provider domain function while registering/updating the API provider domain.

Shall not be present in the HTTP POST request from the API management function to

the CAPIF core function, to register itself. Shall be present in all other HTTP

requests and responses.

regInfo:

$ref: '#/components/schemas/RegistrationInformation'

apiProvFuncRole:

$ref: '#/components/schemas/ApiProviderFuncRole'

apiProvFuncInfo:

type: string

description: >

Generic information related to the API provider domain function such as details

of the API provider applications.

required:

- regInfo

- apiProvFuncRole

RegistrationInformation:

type: object

description: >

Represents registration information of an individual API provider domain function.

properties:

apiProvPubKey:

type: string

description: Public Key of API Provider domain function.

apiProvCert:

type: string

description: API provider domain function's client certificate

required:

- apiProvPubKey

APIProviderEnrolmentDetailsPatch:

type: object

description: >

Represents a list of modifications for the API provider domain's enrolment details.

properties:

apiProvFuncs:

type: array

items:

$ref: '#/components/schemas/APIProviderFunctionDetails'

minItems: 1

description: >

A list of individual API provider domain functions details. When included by

the API management function in the HTTP request message, it lists the API

provider domain functions that the API management function intends to

register/update in registration or update registration procedure.

apiProvDomInfo:

type: string

description: >

Generic information related to the API provider domain such as details

of the API provider applications.

# Simple data types and enumerations

ApiProviderFuncRole:

anyOf:

- type: string

enum:

- AEF

- APF

- AMF

- type: string

description: >

This string provides forward-compatiblity with future extensions to the enumeration

but is not used to encode content defined in the present version of this API.

description: |

Indicates the role (e.g. AEF, APF, etc.) of an API provider domain function.

Possible values are:

- AEF: API provider function is API Exposing Function.

- APF: API provider function is API Publishing Function.

- AMF: API Provider function is API Management Function.

* * * Next Change * * * *

# A.12	CAPIF_Routing_Info_API

openapi: 3.0.0

info:

title: CAPIF_Routing_Info_API

description: |

API for Routing information.

© 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

version: "1.2.0"

externalDocs:

description: 3GPP TS 29.222 V18.6.0 Common API Framework for 3GPP Northbound APIs

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.222/

servers:

- url: '{apiRoot}/capif-routing-info/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 7.5 of 3GPP TS 29.222

paths:

/service-apis/{serviceApiId}:

get:

description: Retrieves the API routing information.

operationId: GetAPIRouteInfo

tags:

- Individual Service API routing Info (Document)

parameters:

- name: serviceApiId

in: path

description: Identifier of a published service API

required: true

schema:

type: string

- name: aef-id

in: query

required: true

description: Identifier of the AEF

schema:

type: string

- name: supp-feat

in: query

required: false

description: To filter irrelevant responses related to unsupported features

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: OK.

content:

application/json:

schema:

$ref: '#/components/schemas/RoutingInfo'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'414':

$ref: 'TS29122_CommonData.yaml#/components/responses/414'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

schemas:

RoutingInfo:

type: object

description: Represents an API routing information.

properties:

routingRules:

type: array

items:

$ref: '#/components/schemas/RoutingRule'

minItems: 1

required:

- routingRules

RoutingRule:

type: object

description: Represents an API routing rule.

properties:

ipv4AddrRanges:

type: array

items:

$ref: 'TS29510_Nnrf_NFManagement.yaml#/components/schemas/Ipv4AddressRange'

minItems: 1

ipv6AddrRanges:

type: array

items:

$ref: '#/components/schemas/Ipv6AddressRange'

minItems: 1

aefProfile:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/AefProfile'

required:

- aefProfile

Ipv6AddressRange:

type: object

description: Represents IPv6 address range.

properties:

start:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Ipv6Addr'

end:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Ipv6Addr'

required:

- start

- end

* * * Next Change * * * *

# A.13	CAPIF_Open_Discover_Service_API

openapi: 3.0.0

info:

title: CAPIF_Open_Discover_Service_API

version: 1.0.0-alpha.1

description: |

API for Routing information.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.222 V19.4.0 Common API Framework for 3GPP Northbound APIs

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.222/

servers:

- url: '{apiRoot}/open-api-disc/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 7.5 of 3GPP TS 29.222.

paths:

/service-apis:

get:

description: >

Enables Open discovery of the currently registered at the CCF and satisfying

a number of filter criteria.

operationId: GetServAPIs

tags:

- Service APIs (Collection)

parameters:

- name: api-names

in: query

description: >

Contains the name(s) of the target Service API(s).

style: form

explode: false

schema:

type: array

items:

type: string

minItems: 1

- name: api-versions

in: query

description: >

Contains the major version(s) (e.g., v1) of the target Service API(s).

schema:

type: object

additionalProperties:

type: array

items:

type: string

minItems: 1

minProperties: 1

- name: comm-type

in: query

description: >

Contains the communication type supported by the target Service API(s).

schema:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/CommunicationType'

- name: protocols

in: query

description: >

Contains the protocol(s) supported by the target Service API(s).

style: form

explode: false

schema:

type: array

items:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/Protocol'

minItems: 1

- name: data-format

in: query

description: >

Contains data format(s) supported by the target Service API(s).

schema:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/DataFormat'

- name: api-cats

in: query

description: >

Contains the category(ies) of the target Service API(s).

style: form

explode: false

schema:

type: array

items:

type: string

minItems: 1

- name: preferred-aef-loc

in: query

description: >

Contains the preferred location information for AEF(s) exposing the target

Service API(s).

This query parameter is ignored by the CCF if there are no matching records at the CCF.

content:

application/json:

schema:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/AefLocation'

- name: api-prov-names

in: query

description: >

Contains the name(s) of the provider(s) of the target Service API(s).

style: form

explode: false

schema:

type: array

items:

type: string

minItems: 1

- name: api-supported-features

in: query

description: >

Contains the features supported by the discovered service API indicated by

api-name parameter. This may only be present if api-name query parameter is

present.

schema:

type: object

additionalProperties:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

minProperties: 1

- name: api-ids

in: query

description: >

Contains the identifier(s) of the targeted service APIs.

required: false

style: form

explode: false

schema:

type: array

items:

type: string

minItems: 1

- name: service-kpis

in: query

description: >

Contains iInformation about service characteristics provided by the targeted

service API(s).

schema:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/ServiceKpis'

- name: res-ops

in: query

description: >

Contains the list of supported API resource(s) and service operation(s).

style: form

explode: false

schema:

type: array

items:

$ref: 'TS29222_CAPIF_Discover_Service_API.yaml#/components/schemas/ResOperInfo'

minItems: 1

- name: supported-features

in: query

description: >

Contains a list of supported features among the ones defined in clause 8.1.6. This

attributed shall be present only when feature negotiation needs to take place.

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: >

The response body contains the result of the search over the list of the registered

APIs.

content:

application/json:

schema:

$ref: '#/components/schemas/OpenDiscoveryResp'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'414':

$ref: 'TS29122_CommonData.yaml#/components/responses/414'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

schemas:

OpenDiscoveryResp:

type: object

description: >

Represents the Open Service API Discovery response..

properties:

discApis:

type: array

items:

$ref: '#/components/schemas/OpenAPIDetails'

minItems: 0

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- discApis

OpenAPIDetails:

type: object

description: >

Represents the Service API details provided within an Open Service API Discovery

response.

properties:

apiName:

type: string

apiId:

type: string

apiStatus:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/ApiStatus'

description:

type: string

serviceAPICategory:

type: string

apiSuppFeats:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

apiProvName:

type: string

aefProfiles:

type: array

items:

$ref: '#/components/schemas/OpenAefProfile'

minItems: 1

required:

- apiName

OpenAefProfile:

type: object

description: Represents the AEF Profile details provided within an Open Service API Discovery response.

properties:

aefId:

type: string

versions:

type: array

items:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/Version'

minItems: 1

protocol:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/Protocol'

dataFormat:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/DataFormat'

aefLocation:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/AefLocation'

serviceKpis:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/ServiceKpis'

* * * End of Changes * * * *