3GPP TSG-CT3 Meeting #143	C3-254533

# Sophia-Antipolis, France, 13th Oct 2025 - 17th Oct 2025				(revision of C3-254286)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.222 | CR | 0439 | rev | 1 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to discoveryCount attribute and related definitions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT3 |  |  |  |  |  |  |  |  |  |
| Work item code: | CAPIF_Ph3 |  |  |  |  |  | Date: |  |  | 2025-10-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The following errors have been identified and need to be corrected: - Typo in the attribute name disoveryCount; it should be corrected to discoveryCount. - The cardinality of the discoveryCount attribute should be updated to 1..N, as it is defined as an array. The description of this attribute should also be corrected. - The DiscoveryCount data type should follow the same approach as ApiInvokerCount data type definition, i.e. to count the discovery request and response of the service API ID. Since the discoveryCount attribute is already defined as an array of DiscoveryCount data type, there is no need to define an array of apiIds within DiscoveryCount. - The description of the SERVICE_API_DISCOVERY_BY_API_INVOKERS_COUNT event should be corrected. - The corresponding OpenAPI file should be updated to align with the corrected DiscoveryCount data model definition. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR proposes to correct the above identified errors. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Implementation inconsistencies and incomplete OpenAPI definition. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.2.4.2, 8.3.4.2.5, 8.3.4.2.10, 8.3.4.3.3, A.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backward compatible correction to the following API: TS29222_CAPIF_Events_API.yaml |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * 1st Change * * * *

##### 5.4.2.4.2	Notifying CAPIF events using Notify_Event service operation

To notify on CAPIF events, the CCF shall send an HTTP POST request message using the Notification Destination URI received during the subscription creation/update request as defined in clauses 5.4.2.2 and 5.4.2.5, with the request body including the EventNotification data structure.

If the "Enhanced_event_report" feature is supported, the CCF may also include within the EventNotification data structure events related detail within the "eventDetail" attribute. The "eventDetail" attribute shall include:

-	if the event is "SERVICE_API_AVAILABLE" or "SERVICE_API_UNAVAILABLE", the API IDs within the "apiIds" attribute and, if the "ApiStatusMonitoring" feature is supported, the service API information within the "serviceAPIDescriptions" attribute;

-	if the event is "SERVICE_API_UPDATE", the API information within the "serviceAPIDescriptions" attribute;

-	if the event is "API_INVOKER_ONBOARDED" or "API_INVOKER_OFFBOARDED" or "API_INVOKER_UPDATED", the API invoker IDs within the "apiInvokerIds" attribute;

-	if the event is "ACCESS_CONTROL_POLICY_UPDATE", the access control policy information within the "accCtrlPolList" attribute;

-	if the event is "SERVICE_API_INVOCATION_SUCCESS" or "SERVICE_API_INVOCATION_FAILURE", the API invocation logs within the "invocationLogs" attribute;

-	if the event is "API_TOPOLOGY_HIDING_CREATED" or "API_TOPOLOGY_HIDING_REVOKED", the API topology hiding information within the "apiTopoHide" attribute; or

-	if the event is API_INVOKER_ONBOARDING_CRITERIA_FAILED, the onboard criteria information within the "onboardingCriteria" attribute;

-	if the "CAPIF_Ext1" feature is supported and the event is "SERVICE_API_ONBOARDED_BY_API_INVOKERS_COUNT", the corresponding count within the "onboardedCount" attribute; or

-	if the "CAPIF_Ext1" feature is supported and the event is "SERVICE_API_DISCOVERY_BY_API_INVOKERS_COUNT", the corresponding count within the "discoveryCount" attribute.

Upon reception of the HTTP POST request message, the Subscriber shall process the Event Notification, and upon success, the Subscriber shall respond with an HTTP "204 No Content" status code to acknowledge the reception of the notification.

If errors occur when processing the request, the Subscriber shall respond to the CCF with an appropriate error status code as defined in clause 8.3.5.

# * * * * 2nd Change * * * *

##### 8.3.4.2.5	Type: CAPIFEventDetail

Table 8.3.4.2.5-1: Definition of type CAPIFEventDetail

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| serviceAPIDescriptions | array(ServiceAPIDescription) | O | 1..N | Contains the description of the service API as published by the APF. |  |
| apiIds | array(string) | O | 1..N | Contains the identifier(s) of the API(s). |  |
| apiInvokerIds | array(string) | O | 1..N | Contains the identifier(s) of the API Invoker(s) that are onboarded/offboarded. |  |
| accCtrlPolList | AccessControlPolicyListExt | O | 0..1 | Contains the access control policy updated list. |  |
| invocationLogs | array(InvocationLog) | O | 1..N | Contains the invocation logs. |  |
| apiTopoHide | TopologyHiding | O | 0..1 | Contains the topology hiding information for a service API. |  |
| onboardingCriteria | array(OnboardingCriteria) | O | 1..N | Contains the onboarding criteria failed to be met during API Invokers onboarding. | CAPIF_Ext1 |
| onboardedCount | array(ApiInvokerCount) | O | 1..N | Contains the count of the number of times the API Invokers requested to onboard the service API(s). | CAPIF_Ext1 |
| discoveryCount | array(DiscoveryCount) | O | 1..N | Contains the count of the number of times the API Invokers requested and/or received the service API(s) during the discovery service. | CAPIF_Ext1 |

# * * * * 3rd Change * * * *

##### 8.3.4.2.10	Type: DiscoveryCount

Table 8.3.4.2.10-1: Definition of type DiscoveryCount

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| apiId | string | M | 1 | Contains the identifier of the service API. |  |
| discoveryReqCount | Uinteger | C | 0..1 | Contains the number of times the API Invokers requested to discover the service API in discovery requests. (NOTE) |  |
| discoveryRspCount | Uinteger | C | 0..1 | Contains the number of times the API Invokers received the service API in discovery responses. (NOTE) |  |
| NOTE: At least one of these attributes shall be present. |  |  |  |  |  |

# * * * * 4th Change * * * *

##### 8.3.4.3.3	Enumeration: CAPIFEvent

The enumeration CAPIFEvent represents a CAPIF event. It shall comply with the provisions defined in table 8.3.4.3.3-1.

Table 8.3.4.3.3-1: Enumeration CAPIFEvent

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| SERVICE_API_AVAILABLE | Events related to the availability of service APIs after the service APIs are published. |  |
| SERVICE_API_UNAVAILABLE | Events related to the unavailability of service APIs after the service APIs are unpublished. |  |
| SERVICE_API_UPDATE | Events related to changes in service API information. |  |
| API_INVOKER_ONBOARDED | Events related to API Invoker onboarded to CAPIF. |  |
| API_INVOKER_OFFBOARDED | Events related to API Invoker offboarded from CAPIF. |  |
| SERVICE_API_INVOCATION_SUCCESS | Events related to the successful invocation of service APIs. |  |
| SERVICE_API_INVOCATION_FAILURE | Events related to the failed invocation of service APIs. |  |
| ACCESS_CONTROL_POLICY_UPDATE | Events related to the update for the access control policy related to the service APIs. |  |
| ACCESS_CONTROL_POLICY_UNAVAILABLE | Events related to the unavailability of the access control policy related to the service APIs. |  |
| API_INVOKER_AUTHORIZATION_REVOKED | Events related to the revocation of the authorization of API Invokers to access the service APIs. |  |
| API_INVOKER_UPDATED | Events related to API Invoker profile updated to CAPIF. |  |
| API_TOPOLOGY_HIDING_CREATED | Events related to the creation or update of the API topology hiding information of the service API after the service APIs are published. |  |
| API_TOPOLOGY_HIDING_REVOKED | Events related to the revocation of the API topology information of the service API after the service APIs are unpublished. |  |
| API_INVOKER_ONBOARDING_CRITERIA_FAILED | Events related to API Invoker onboarding criteria failed to be met. | CAPIF_Ext1 |
| SERVICE_API_ONBOARDED_BY_API_INVOKERS_COUNT | Periodic event related to the number of times the API Invokers requested to onboard the targeted service API(s). | CAPIF_Ext1 |
| SERVICE_API_DISCOVERY_BY_API_INVOKERS_COUNT | Periodic event related to the number of times the API Invokers requested and/or received the targeted service API(s) during the discover service. | CAPIF_Ext1 |

# * * * * 5th Change * * * *

# A.4	CAPIF_Events_API

openapi: 3.0.0

info:

title: CAPIF_Events_API

description: |

API for event subscription management.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

version: "1.4.0-alpha.3"

externalDocs:

description: 3GPP TS 29.222 V19.4.0 Common API Framework for 3GPP Northbound APIs

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.222/

servers:

- url: '{apiRoot}/capif-events/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 7.5 of 3GPP TS 29.222

paths:

/{subscriberId}/subscriptions:

post:

summary: Create a new CAPIFs Event Subscription.

operationId: CreateEventSubsc

tags:

- CAPIFs Events Subscriptions (Collection)

parameters:

- name: subscriberId

in: path

description: Identifier of the Subscriber

required: true

schema:

type: string

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/EventSubscription'

responses:

'201':

description: Created (Successful creation of subscription).

content:

application/json:

schema:

$ref: '#/components/schemas/EventSubscription'

headers:

Location:

description: >

Contains the URI of the newly created resource, according to the structure.

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

callbacks:

notificationDestination:

'{$request.body#/notificationDestination}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/EventNotification'

responses:

'204':

description: No Content (successful notification).

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/{subscriberId}/subscriptions/{subscriptionId}:

delete:

summary: Delete an existing Individual CAPIF Events Subscription resource.

operationId: DeleteIndEventSubsc

tags:

- Individual CAPIFs Events Subscription (Document)

parameters:

- name: subscriberId

in: path

description: Identifier of the Subscriber

required: true

schema:

type: string

- name: subscriptionId

in: path

description: Identifier of an individual Events Subscription

required: true

schema:

type: string

responses:

'204':

description: >

No Content. The Individual CAPIF Events Subscription resource is successfully deleted.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

summary: Update an existing Individual CAPIF Events Subscription resource.

operationId: UpdateIndEventSubsc

tags:

- Individual CAPIFs Events Subscription (Document)

parameters:

- name: subscriberId

in: path

description: Identifier of the Subscriber

required: true

schema:

type: string

- name: subscriptionId

in: path

description: Identifier of the individual Subscriber

required: true

schema:

type: string

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/EventSubscription'

responses:

'200':

description: OK (Successful update of the subscription).

content:

application/json:

schema:

$ref: '#/components/schemas/EventSubscription'

'204':

description: No Content. (Successful update of the subscription).

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

patch:

summary: Modify an existing Individual CAPIF Events Subscription resource.

operationId: ModifyIndEventSubsc

tags:

- Individual CAPIFs Events Subscription (Document)

parameters:

- name: subscriberId

in: path

description: Identifier of the Subscriber

required: true

schema:

type: string

- name: subscriptionId

in: path

description: Identifier of the individual Subscriber

required: true

schema:

type: string

requestBody:

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/EventSubscriptionPatch'

responses:

'200':

description: OK (Successful modification of the subscription).

content:

application/json:

schema:

$ref: '#/components/schemas/EventSubscription'

'204':

description: No Content (Successful modification of the subscription).

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

schemas:

EventSubscription:

type: object

description: Represents a CAPIF Events Subscription.

properties:

events:

type: array

items:

$ref: '#/components/schemas/CAPIFEvent'

minItems: 1

description: Subscribed events.

eventFilters:

type: array

items:

$ref: '#/components/schemas/CAPIFEventFilter'

minItems: 1

description: Subscribed event filters.

eventReq:

$ref: 'TS29523_Npcf_EventExposure.yaml#/components/schemas/ReportingInformation'

notificationDestination:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

requestTestNotification:

type: boolean

description: >

Set to true by Subscriber to request the CAPIF core function to send a

test notification as defined in in clause 7.6. Set to false or omitted otherwise.

websockNotifConfig:

$ref: 'TS29122_CommonData.yaml#/components/schemas/WebsockNotifConfig'

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- events

- notificationDestination

EventNotification:

type: object

description: Represents a CAPIF Events Notification.

properties:

subscriptionId:

type: string

description: >

Identifier of the subscription resource to which the notification

is related.

events:

$ref: '#/components/schemas/CAPIFEvent'

eventDetail:

$ref: '#/components/schemas/CAPIFEventDetail'

required:

- subscriptionId

- events

CAPIFEventFilter:

type: object

description: Represents a CAPIF event filter.

properties:

apiIds:

type: array

items:

type: string

minItems: 1

description: Identifier of the service API.

apiInvokerIds:

type: array

items:

type: string

minItems: 1

description: Identity of the API invoker.

aefIds:

type: array

items:

type: string

minItems: 1

description: Identifier of the API exposing function.

CAPIFEventDetail:

type: object

description: Represents the CAPIF event related details.

properties:

serviceAPIDescriptions:

type: array

items:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/ServiceAPIDescription'

minItems: 1

description: Description of the service API as published by the APF.

apiIds:

type: array

items:

type: string

minItems: 1

description: Identifier of the service API.

apiInvokerIds:

type: array

items:

type: string

minItems: 1

description: Identity of the API invoker.

accCtrlPolList:

$ref: '#/components/schemas/AccessControlPolicyListExt'

invocationLogs:

type: array

items:

$ref: 'TS29222_CAPIF_Logging_API_Invocation_API.yaml#/components/schemas/InvocationLog'

minItems: 1

description: Invocation logs.

apiTopoHide:

$ref: '#/components/schemas/TopologyHiding'

onboardingCriteria:

type: array

items:

$ref: 'TS29222_CAPIF_API_Invoker_Management_API.yaml#/components/schemas/OnboardingCriteria'

minItems: 1

onboardedCount:

type: array

items:

$ref: '#/components/schemas/ApiInvokerCount'

minItems: 1

discoveryCount:

type: array

items:

$ref: '#/components/schemas/DiscoveryCount'

minItems: 1

AccessControlPolicyListExt:

description: Represents the extension for access control policies.

allOf:

- $ref: 'TS29222_CAPIF_Access_Control_Policy_API.yaml#/components/schemas/AccessControlPolicyList'

- type: object

properties:

apiId:

type: string

required:

- apiId

TopologyHiding:

type: object

description: Represents the routing rules information of a service API.

properties:

apiId:

type: string

routingRules:

type: array

items:

$ref: 'TS29222_CAPIF_Routing_Info_API.yaml#/components/schemas/RoutingRule'

minItems: 1

required:

- apiId

- routingRules

EventSubscriptionPatch:

type: object

description: >

Represents the parameters to request the modification of a CAPIF Events Subscription

resource.

properties:

events:

type: array

items:

$ref: '#/components/schemas/CAPIFEvent'

minItems: 1

description: Subscribed events.

eventFilters:

type: array

items:

$ref: '#/components/schemas/CAPIFEventFilter'

minItems: 1

description: Subscribed event filters.

eventReq:

$ref: 'TS29523_Npcf_EventExposure.yaml#/components/schemas/ReportingInformation'

notificationDestination:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

ApiInvokerCount:

type: object

description: Represents the count data for onboarding.

properties:

apiId:

type: string

count:

type: integer

description: >

Contains the number of times the API Invokers requested to onboard the service

API(s) API.

required:

- apiId

- count

DiscoveryCount:

type: object

description: Represents the count data for discovery.

properties:

apiId:

type: string

discoveryReqCount:

type: Uinteger

discoveryRspCount:

type: Uinteger

required:

- apiId

anyOf:

- required: [discoveryReqCount]

- required: [discoveryRspCount]

CAPIFEvent:

anyOf:

- type: string

enum:

- SERVICE_API_AVAILABLE

- SERVICE_API_UNAVAILABLE

- SERVICE_API_UPDATE

- API_INVOKER_ONBOARDED

- API_INVOKER_OFFBOARDED

- SERVICE_API_INVOCATION_SUCCESS

- SERVICE_API_INVOCATION_FAILURE

- ACCESS_CONTROL_POLICY_UPDATE

- ACCESS_CONTROL_POLICY_UNAVAILABLE

- API_INVOKER_AUTHORIZATION_REVOKED

- API_INVOKER_UPDATED

- API_TOPOLOGY_HIDING_CREATED

- API_TOPOLOGY_HIDING_REVOKED

- API_INVOKER_ONBOARDING_CRITERIA_FAILED

- SERVICE_API_ONBOARDED_BY_API_INVOKERS_COUNT

- SERVICE_API_DISCOVERY_BY_API_INVOKERS_COUNT

- type: string

description: >

This string provides forward-compatibility with future

extensions to the enumeration but is not used to encode

content defined in the present version of this API.

description: |

Describes the CAPIF event.

Possible values are:

- SERVICE_API_AVAILABLE:

Events related to the availability of service APIs after the service APIs are

published.

- SERVICE_API_UNAVAILABLE:

Events related to the unavailability of service APIs after the service APIs are

unpublished.

- SERVICE_API_UPDATE: Events related to change in service API information.

- API_INVOKER_ONBOARDED: Events related to API invoker onboarded to CAPIF.

- API_INVOKER_OFFBOARDED: Events related to API invoker offboarded from CAPIF.

- SERVICE_API_INVOCATION_SUCCESS:

Events related to the successful invocation of service APIs.

- SERVICE_API_INVOCATION_FAILURE: Events related to the failed invocation of service APIs.

- ACCESS_CONTROL_POLICY_UPDATE:

Events related to the update for the access control policy related to the service APIs.

- ACCESS_CONTROL_POLICY_UNAVAILABLE:

Events related to the unavailability of the access control policy related to

the service APIs.

- API_INVOKER_AUTHORIZATION_REVOKED: Events related to the revocation of the authorization

of API invokers to access the service APIs.

- API_INVOKER_UPDATED: Events related to API invoker profile updated to CAPIF.

- API_TOPOLOGY_HIDING_CREATED:

Events related to the creation or update of the API topology hiding

information of the service APIs after the service APIs are published.

- API_TOPOLOGY_HIDING_REVOKED:

Events related to the revocation of the API topology hiding information of

the service APIs after the service APIs are unpublished.

- API_INVOKER_ONBOARDING_CRITERIA_FAILED: Events related to API Invoker onboarding criteria

failed to be met.

- SERVICE_API_ONBOARDED_BY_API_INVOKERS_COUNT: Periodic event related to the number of times the

API Invokers requested to onboard the targeted service API(s).

- SERVICE_API_DISCOVERY_BY_API_INVOKERS_COUNT: Periodic event related to the number of times the

API Invokers requested and/or received the targeted service API(s)during the

discovery service.

# * * * * End of Changes * * * *