3GPP TSG-CT WG3 Meeting #143	C3-254534

# Sophia-Antipolis, FR, 13 - 17 October 2025                                          (was C3-2534304)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.222 | CR | 0443 | rev | 1 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update clause 5.1 with CAPIF_Open_Discover_Service_API details |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C3 |  |  |  |  |  |  |  |  |  |
| Work item code: | CAPIF_Ph3 |  |  |  |  |  | Date: |  |  | 06-10-2025 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | CAPIF_Open_Discover_Service_API details have not been added to the API descriptions under clause 5.1. This CR proposes to add the API description. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update “Table 5.1-2: API Descriptions” with CAPIF_Open_Discover_Service_API details. Correct the API name of CAPIF_Open_Discover_Servive_API |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing the complete stage-3 aspects of CAPIF_Open_Discover_Service_API. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1, A.13 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR proposes backward compatible correction to the following OpenAPI: - TS29222_CAPIF_Open_Discover_Service_API.yaml |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * *

## 5.1	Introduction of Services

The table 5.1-1 lists the CCF APIs below the service name. A service description clause for each API gives a general description of the related API.

Table 5.1-1: List of CAPIF Services

| Service Name | Service Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| CAPIF_Discover_Service_API | Discover_Service_API | Request/ Response | API Invoker, CCF |
|  | Event operations (NOTE) | (NOTE) | API Invoker |
| CAPIF_Publish_Service_API | Publish_Service_API | Request/ Response | API Publishing Function, CCF |
|  | Unpublish_Service_API | Request/ Response | API Publishing Function, CCF |
|  | Update_Service_API | Request/ Response | API Publishing Function, CCF |
|  | Get_Service_API | Request/ Response | API Publishing Function, CCF |
| CAPIF_Events_API | Subscribe_Event | Subscribe/Notify | API Invoker, API Publishing Function, API Management Function, API Exposing Function |
|  | Update_Event_Subscription | Subscribe/Notify | API Invoker, API Publishing Function, API Management Function, API Exposing Function |
|  | Notify_Event | Subscribe/Notify | API Invoker, API Publishing Function, API Management Function, API Exposing Function |
|  | Unsubscribe_Event | Subscribe/Notify | API Invoker, API Publishing Function, API Management Function, API Exposing Function |
| CAPIF_API_Invoker_Management_API | Onboard_API_Invoker | Request/ Response | API Invoker |
|  | Offboard_API_Invoker | Request/ Response | API Invoker |
|  | Notify_Onboarding_Completion | Subscribe/Notify | API Invoker |
|  | Update_API_Invoker_Details | Request/Response | API Invoker |
|  | Notify_Update_Completion | Subscribe/Notify | API Invoker |
| CAPIF_Security_API | Obtain_Security_Method | Request/ Response | API Invoker |
|  | Obtain_Authorization | Request/ Response | API Invoker |
|  | Obtain_API_Invoker_Info | Request/ Response | API exposing function |
|  | Revoke_Authorization | Request/ Response | API exposing function |
| CAPIF_Monitoring_API | Event operations (NOTE) | (NOTE) | API Management Function |
| CAPIF_Logging_API_Invocation_API | Log_API_Invocation | Request/ Response | API exposing function |
| CAPIF_Auditing_API | Query_API_Invocation_Log | Request/ Response | API management function |
| CAPIF_Access_Control_Policy_API | Obtain_Access_Control_Policy | Request/Response | API Exposing Function |
| CAPIF_API_Provider_Management_API | Register_API_Provider | Request/Response | API Management Function |
|  | Update_API_Provider | Request/Response | API Management Function |
|  | Deregister_API_Provider | Request/Response | API Management Function |
| CAPIF_Routing_Info_API | Obtain_ Routing_Info | Request/Response | API exposing function |
| NOTE: The service operations of CAPIF Events API are reused by the CAPIF_Discover_Service_API, CAPIF_Publish_Service_API and CAPIF_Monitoring_API for events related services. |  |  |  |

Table 5.1-2 summarizes the corresponding APIs defined in this specification.

Table 5.1-2: API Descriptions

| Service Name | Clause | Description | OpenAPI Specification File | apiName | Annex |
| --- | --- | --- | --- | --- | --- |
| CAPIF_Discover_Service_API | 8.1 | CAPIF API discovery service | TS29222_CAPIF_Discover_Service_API.yaml | service-apis | A.2 |
| CAPIF_Publish_Service_API | 8.2 | CAPIF API Publish Service | TS29222_CAPIF_Publish_Service_API.yaml | published-apis | A.3 |
| CAPIF_Events_API | 8.3 | CAPIF Events Service | TS29222_CAPIF_Events_API.yaml | capif-events | A.4 |
| CAPIF_API_Invoker_Management_API | 8.4 | CAPIF API Invoker Management Service | TS29222_CAPIF_API_Invoker_Management_API.yaml | api-invoker-management | A.5 |
| CAPIF_Security_API | 8.5 | CAPIF Security Service | TS29222_CAPIF_Security_API.yaml | capif-security | A.6 |
| CAPIF_Access_Control_Policy_API | 8.6 | CAPIF Access Control Policy API Service | TS29222_CAPIF_Access_Control_Policy_API.yaml | access-control-policy | A.7 |
| CAPIF_Logging_API_Invocation_API | 8.7 | CAPIF Logging API Invocation Service | TS29222_CAPIF_Logging_API_Invocation_API.yaml | api-invocation-logs | A.8 |
| CAPIF_Auditing_API | 8.8 | CAPIF Auditing API Service | TS29222_CAPIF_Auditing_API.yaml | logs | A.9 |
| CAPIF_API_Provider_Management_API | 8.9 | CAPIF API Provider Management API Service | TS29222_CAPIF_API_Provider_Management_API.yaml | api-provider-management | A.11 |
| CAPIF_Routing_Info_API | 8.10 | CAPIF Routing Information API Service | TS29222_CAPIF_Routing_Info_API.yaml | capif-routing-info | A.12 |
| CAPIF_Open_Discover_Service_API | 8.11 | CAPIF Open API Discovery Service | TS29222_CAPIF_Open_Discover_Service_API.yaml | open-api-disc | A.13 |

* * * Next Change * * *

# A.13	CAPIF_Open_Discover_Service_API

openapi: 3.0.0

info:

title: CAPIF_Open_Discover_Service_API

version: 1.0.0-alpha.1

description: |

API foropen discovery of service APIs.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.222 V19.4.0 Common API Framework for 3GPP Northbound APIs

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.222/

servers:

- url: '{apiRoot}/open-api-disc/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 7.5 of 3GPP TS 29.222.

paths:

/service-apis:

get:

description: >

Enables Open discovery of the currently registered at the CCF and satisfying

a number of filter criteria.

parameters:

- name: api-names

in: query

description: >

Contains the name(s) of the target Service API(s).

style: form

explode: false

schema:

type: array

items:

type: string

minItems: 1

- name: api-versions

in: query

description: >

Contains the major version(s) (e.g., v1) of the target Service API(s).

schema:

type: object

additionalProperties:

type: array

items:

type: string

minItems: 1

minProperties: 1

- name: comm-type

in: query

description: >

Contains the communication type supported by the target Service API(s).

schema:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/CommunicationType'

- name: protocols

in: query

description: >

Contains the protocol(s) supported by the target Service API(s).

style: form

explode: false

schema:

type: array

items:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/Protocol'

minItems: 1

- name: data-format

in: query

description: >

Contains data format(s) supported by the target Service API(s).

schema:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/DataFormat'

- name: api-cats

in: query

description: >

Contains the category(ies) of the target Service API(s).

style: form

explode: false

schema:

type: array

items:

type: string

minItems: 1

- name: preferred-aef-loc

in: query

description: >

Contains the preferred location information for AEF(s) exposing the target

Service API(s).

This query parameter is ignored by the CCF if there are no matching records at the CCF.

content:

application/json:

schema:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/AefLocation'

- name: api-prov-names

in: query

description: >

Contains the name(s) of the provider(s) of the target Service API(s).

style: form

explode: false

schema:

type: array

items:

type: string

minItems: 1

- name: api-supported-features

in: query

description: >

Contains the features supported by the discovered service API indicated by

api-name parameter. This may only be present if api-name query parameter is

present.

schema:

type: object

additionalProperties:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

minProperties: 1

- name: api-ids

in: query

description: >

Contains the identifier(s) of the targeted service APIs.

required: false

style: form

explode: false

schema:

type: array

items:

type: string

minItems: 1

- name: service-kpis

in: query

description: >

Contains iInformation about service characteristics provided by the targeted

service API(s).

schema:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/ServiceKpis'

- name: res-ops

in: query

description: >

Contains the list of supported API resource(s) and service operation(s).

style: form

explode: false

schema:

type: array

items:

$ref: 'TS29222_CAPIF_Discover_Service_API.yaml#/components/schemas/ResOperInfo'

minItems: 1

- name: supported-features

in: query

description: >

Contains a list of supported features among the ones defined in clause 8.1.6. This

attributed shall be present only when feature negotiation needs to take place.

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: >

The response body contains the result of the search over the list of the registered

APIs.

content:

application/json:

schema:

$ref: '#/components/schemas/OpenDiscoveryResp'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'414':

$ref: 'TS29122_CommonData.yaml#/components/responses/414'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

schemas:

OpenDiscoveryResp:

type: object

description: >

Represents the Open Service API Discovery response..

properties:

discApis:

type: array

items:

$ref: '#/components/schemas/OpenAPIDetails'

minItems: 0

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- discApis

OpenAPIDetails:

type: object

description: >

Represents the Service API details provided within an Open Service API Discovery

response.

properties:

apiName:

type: string

apiId:

type: string

apiStatus:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/ApiStatus'

description:

type: string

serviceAPICategory:

type: string

apiSuppFeats:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

apiProvName:

type: string

aefProfiles:

type: array

items:

$ref: '#/components/schemas/OpenAefProfile'

minItems: 1

required:

- apiName

OpenAefProfile:

type: object

description: >

Represents the AEF Profile details provided within an Open Service API Discovery response.

properties:

aefId:

type: string

versions:

type: array

items:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/Version'

minItems: 1

protocol:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/Protocol'

dataFormat:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/DataFormat'

aefLocation:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/AefLocation'

serviceKpis:

$ref: 'TS29222_CAPIF_Publish_Service_API.yaml#/components/schemas/ServiceKpis'

* * * End of Change * * *