3GPP TSG-CT WG3 Meeting #143	C3-254537

# Sophia-Antipolis, France, 13 - 17 October 2025						(Revision of C3-254113)

Source:	Nokia

Title:	Spatial Map Management Pose handling

Spec:	3GPP TS 29.437 and 1.1.0

Agenda item:	19.42

Document for:	Approval

1. Introduction

This pCR proposes to Spatial Map management Pose handling and its corresponding data types.

2. Reason for Change

For Spatial Map management, the pose data type is introduced as per TR 23.700-21 (Metaverse_App study) mentions as follows:

7.1.3.2 Information flows Table 7.1.3.2-1 shows the request sent by a MMEC to a MMES for the discovery of spatial anchor(s). Table 7.1.3.2-1: Spatial anchor discovery request Information element; Status; Description. Requestor identity; M; The identity of the requestor (e.g., MMEC, UE, VAL server). Requestor security credentials; M; The security credentials of the requestor.. Location of interest; O; Location of spatial anchors the requestor is interest in.. Spatial anchor discovery filters; O; Set of characteristics to determine matching spatial anchors (e.g., spatial anchor location, VAL client type, universal filter).. Current UE location; O; Current location of the UE. Current UE pose; O; Current pose of the user (as defined in clause 6.2.2 of 3GPP TS 26.119 [11]). SA Group ID; O; Indicates the identifier of the requested group of SA.

TS 23.437 CR 0072 (S6-254176 – r0, S6-254543 – r1) elaborates the orientation, position and fov

3. Conclusions

Pose Datatype and the corresponding update in the spatial map management update is done.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 29.437 v1.1.0.

# * * * First Change * * * *

##### 6.2.1.6.1	General

This clause specifies the application data model supported by the API.

Table 6.2.1.6.1-1 specifies the data types defined for the SS_SmManagement API.

Table 6.2.1.6.1-1: SS_SmManagement API specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| AugmentLayer | 6.2.1.6.3.3 | Represents the information about layer augmentation. |  |
| AugmentLayerInfo | 6.2.1.6.2.4 | Represents the augmented spatial map layer information. |  |
| AugmentLayerList | 6.2.1.6.2.22 | Represents the list of augment layer values. |  |
| EventFilter | 6.2.1.6.2.16 | Represents the event filters for spatial map events subscription. |  |
| MapEvent | 6.2.1.6.3.4 | Represents the spatial map event. |  |
| MapEventSub | 6.2.1.6.2.14 | Represents the spatial map event subscription details. |  |
| MapLayerInfo | 6.2.1.6.2.9 | Represents the layer specific information of an spatial map layer. |  |
| MapObjectChangeEvent | 6.2.1.6.3.5 | Represents the detected change in spatial map object. |  |
| MapObjectChangeReport | 6.2.1.6.2.20 | Represents the map object change details. |  |
| MapObjectInfo | 6.2.1.6.2.10 | Represents the spatial map object. |  |
| MapsReport | 6.2.1.6.2.19 | Represents the spatial map event report details. |  |
| SpatialMap | 6.2.1.6.2.5 | Represents the spatial map resource representation. |  |
| SpatialMapCreateInfo | 6.2.1.6.2.3 | Represents the spatial map information to create. |  |
| SpatialMapCreateReq | 6.2.1.6.2.2 | Represents the spatial map creation request. |  |
| SpatialMapId | 6.2.1.6.3.2 | Represents the spatial map identifier. |  |
| SpatialMapLayers | 6.2.1.6.2.7 | Represents the spatial map layers information. |  |
| SpatialMapLayerInfo | 6.2.1.6.2.8 | Represents the individual spatial map layer. |  |
| SpatialMapProfile | 6.2.1.6.2.6 | Represents the spatial map profile information. |  |
| SpatialMapPatch | 6.2.1.6.2.11 | Represents the partial spatial map resource representation. |  |
| SpatialMapRetResp | 6.2.1.6.2.12 | Represents the spatial map information returned in response. |  |
| SpatialMapsNotif | 6.2.1.6.2.18 | Represents the spatial maps event notification. |  |
| SpatialMapSub | 6.2.1.6.2.13 | Represents the spatial map events subscription request. |  |
| SpatialMapSubPatch | 6.2.1.6.2.17 | Represents the spatial map event subscription modification. |  |
| SpatialMapTempRes | 6.2.1.6.2.21 | Represents the spatial map temporary response. |  |
| TargetSpatialMaps | 6.2.1.6.2.15 | Represents the target spatial maps for events. |  |

Table 6.2.1.6.1-2 specifies data types re-used by the SS_SmManagement API from other specifications, including a reference to their respective specifications, and when needed, a short description of their use within the SS_SmManagement API.

Table 6.2.1.6.1-2: SS_SmManagement API re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| AllowedRules | 6.1.1.6.2.8 | Represents the allowed entity details. |  |
| DateTime | 3GPP TS 29.122 [2] | Represents a date and a time. |  |
| Local3dPointUncertaintyEllipsoid | 3GPP TS 29.572 [16] | Represents a 3D point. |  |
| LocationInfo | 3GPP TS 29.122 [2] | Represents location information. |  |
| Pose | 6.1.1.6.2.25 | Represents the Pose details. |  |
| ReportingInformation | 3GPP TS 29.523 [19] | Represents the reporting requirements information. |  |
| ServiceArea | 3GPP TS 29.558 [18] | Represents the service area information. |  |
| SupportedFeatures | 3GPP TS 29.571 [15] | Represents the list of supported feature(s) and used to negotiate the applicability of the optional features. |  |
| Uint32 | 3GPP TS 29.571 [15] | Represents an unsigned integer. |  |
| Uri | 3GPP TS 29.122 [2] | Represents an URI. |  |

# * * * * Next changes * * * *

6.2.1.6.2.15	Type: TargetSpatialMaps

Table 6.2.1.6.2.15-1: Definition of type TargetSpatialMaps

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| mapIds | array(SpatialMapId) | C | 1..N | Contains the list of spatial map identifiers. |  |
| area | ServiceArea | C | 0..1 | Contains the three dimensional area of information related to spatial maps. |  |
| locInfo | LocationInfo | C | 0..1 | Contains the current location information related to the spatial maps. |  |
| pose | Pose | C | 0..1 | Contains the current pose information related to the spatial maps. |  |
| NOTE: At least one of these attributes shall be present. |  |  |  |  |  |

# * * * * End of Changes * * * *