# 3GPP TSG CT WG3 Meeting #143	C3-254549

# Sophia-Antipolis, France, 13 – 17 October, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.522 | CR | 1737 | rev | 1 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | IMS Event Exposure Notification corrections |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2025-09-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In previous versions of TS 29.522 CR C3-252509 was approved including some updates in chapter 4.4.47.2 and 4.4.47.3 about NEF may send some notfications to AF related to errors received for non-subscriber and subscriber specific IMS event subscriptions including IMS Failure Cause inside ImsEENotif data structure. However, today, ImsEENotif contains a mandatory attribute (reports) which may not be inserted when IMS Failure Cause is added. On the other hand, today ImsEENotif does not include the possibility of including the target UE Identifier affected by the notification. Without this information, AF will not be able to know to which IMS Public Identity the IMS Event Exposure event notification is related to. This is specially relevant when the IMS Event Exposure Subscription (with its correlation Id) is related to a list of targe UE Identities or to any UE. IMS AS is already including the IMSPublicId as part of the EventNotification data structure sent by IMS AS to NEF, so it is a matter of sending it transparently. Hence the protocol has to be properly updated to reflect the following changes inside ImsEENotif: - Include reports attribute as an optional attribute to allow conveying IMS Failure Cause only - Include the IMS Public Identity affected by the event ocurrence. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update IMS Event Exposure Notification content to update reports attribute as an optional attribute and the IMS Public Identity . |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Sending unnecessary information (reports attribute) in error situations Not letting AF to identify to which target UE Identity the IMS Event Exposure notification was related to |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.43.5.2.3, A.41 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backward compatible feature to the following APIs: TS29522_ImsEventExposure.yaml |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# *** 1st Change ***

##### 5.43.5.2.3	Type: ImsEENotif

Table 5.43.5.2.3-1: Definition of type ImsEENotif

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| subscriptionId | string | M | 1 | Contains the identifier of the subscription to which the notification is related. |  |
| sessionId | SessionId | O | 0..1 | Identifies the session ID. |  |
| reports | array(ImsEventReport) | O | 1..N | Contains the IMS event report(s). |  |
| imsFailureCause | string | O | 0..1 | Contains the failure cause provided from the IMS AS. |  |
| tgtUeId | ImsPublicId | C | 0..1 | Contains the identifier of the UE. |  |

# *** 2nd Change ***

# A.41	ImsEventExposure API

openapi: 3.0.0

info:

title: 3gpp-ims-ee

version: 1.0.0-alpha.4

description: |

API for IMS Event Exposure Services.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.522 V19.4.0; 5G System; Network Exposure Function Northbound APIs.

url: 'https://www.3gpp.org/ftp/Specs/archive/29_series/29.522/'

servers:

- url: '{apiRoot}/3gpp-ims-ee/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 5.2.4 of 3GPP TS 29.122.

security:

- {}

- oAuth2ClientCredentials: []

paths:

/{afId}/subscriptions:

parameters:

- name: afId

in: path

description: Represents the identifier of the AF.

required: true

schema:

type: string

get:

summary: Retrieve all the active IMS EE Subscriptions managed by the NEF.

operationId: GetImsEESubsc

tags:

- IMS EE Subscriptions (Collection)

responses:

'200':

description: >

OK. All the Individual IMS EE Subscription resource(s) managed by the NEF

are returned.

If there are no active Individual IMS EE Subscription resources at the NEF,

an empty array is returned.

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/ImsEESubsc'

minItems: 0

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

post:

summary: Request the creation of a new IMS EE Subscription.

operationId: CreateImsEESubsc

tags:

- IMS EE Subscriptions (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ImsEESubsc'

responses:

'201':

description: >

Created. A representation of the created IMS EE Subscription resource is

returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/ImsEESubsc'

headers:

Location:

description: >

Contains the URI of the newly created resource.

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

callbacks:

IMSEENotif:

'{$request.body#/notifUri}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ImsEENotif'

responses:

'204':

description: >

No Content. The IMS EE Notification is successfully received and acknowledged.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/{afId}/subscriptions/{subscriptionId}:

parameters:

- name: afId

in: path

description: Represents the identifier of the AF.

required: true

schema:

type: string

- name: subscriptionId

in: path

description: >

Represents the identifier of the Individual IMS EE Subscription.

resource.

required: true

schema:

type: string

get:

summary: Retrieve an existing Individual IMS EE Subscription resource.

operationId: GetIndImsEESubsc

tags:

- Individual IMS EE Subscription (Document)

responses:

'200':

description: >

OK. The requested Individual IMS EE Subscription resource is successfully

returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/ImsEESubsc'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

summary: Request the update of an existing Individual IMS EE Subscription resource.

operationId: UpdateIndImsEESubsc

tags:

- Individual IMS EE Subscription (Document)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ImsEESubsc'

responses:

'200':

description: >

OK. The Individual IMS EE Subscription resource is successfully updated and

a representation of the updated resource is returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/ImsEESubsc'

'204':

description: >

No Content. The Individual IMS EE Subscription resource is successfully updated and

no content is returned in the response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

patch:

summary: Request the modification of an existing Individual IMS EE Subscription resource.

operationId: ModifyIndImsEESubsc

tags:

- Individual IMS EE Subscription (Document)

requestBody:

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/ImsEESubscPatch'

responses:

'200':

description: >

OK. The Individual IMS EE Subscription resource is successfully modified and

a representation of the updated resource is returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/ImsEESubsc'

'204':

description: >

No Content. The Individual IMS EE Subscription resource is successfully modified and

no content is returned in the response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

summary: Request the deletion of an existing Individual IMS EE Subscription resource.

operationId: DeleteIndImsEESubsc

tags:

- Individual IMS EE Subscription (Document)

responses:

'204':

description: >

No Content. The Individual IMS EE Subscription resource is successfully deleted.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

#

# STRUCTURED DATA TYPES

#

ImsEESubsc:

description: Represents an IMS Event Exposure Subscription.

type: object

properties:

eventReqs:

type: array

items:

$ref: '#/components/schemas/ImsEventRequirement'

minItems: 1

imsReptReq:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ImsReportingOptions'

immReports:

type: array

items:

$ref: '#/components/schemas/ImsEventReport'

minItems: 1

tgtUeIds:

type: array

items:

$ref: 'TS29562_Nhss_imsSDM.yaml#/components/schemas/ImsPublicId'

minItems: 1

anyUeInd:

type: boolean

enum:

- true

description: >

Identifies whether the subscription is for any UE.

true indicates that the subscription is for any UE.

When present, this attribute shall be set to true. The presence of this attribute set to

the value false is forbidden.

notifUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- eventReqs

- imsReptReq

- notifUri

oneOf:

- required: [tgtUeIds]

- required: [anyUeInd]

ImsEESubscPatch:

description: Represents the requested modifications to an IMS Event Exposure Subscription.

type: object

properties:

eventReqs:

type: array

items:

$ref: '#/components/schemas/ImsEventRequirement'

minItems: 1

notifUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

ImsEENotif:

description: Represents an IMS Event Exposure Notification.

type: object

properties:

subscriptionId:

type: string

sessionId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SessionId'

reports:

type: array

items:

$ref: '#/components/schemas/ImsEventReport'

minItems: 1

imsFailureCause:

type: string

description: Contains the failure cause provided from the IMS AS.

tgtUeId:

$ref: 'TS29562_Nhss_imsSDM.yaml#/components/schemas/ImsPublicId'

required:

- subscriptionId

ImsEventRequirement:

description: Represnets the IMS Event Requirements.

type: object

properties:

imsEventType:

$ref: '#/components/schemas/ImsEventType'

imsEventConfig:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ImsEventConfiguration'

required:

- imsEventType

- imsEventConfig

ImsEventReport:

description: Represents an IMS Event Report.

type: object

properties:

imsEvent:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ImsEvent'

timeStamp:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

eventRptInfo:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ImsEventReportInfo'

required:

- imsEvent

- timeStamp

#

# SIMPLE DATA TYPES

#

#

# ENUMERATIONS DATA TYPES

#

ImsEventType:

anyOf:

- type: string

enum:

- SUBSCRIBER_SPECIFIC

- NON_SUBSCRIBER_SPECIFIC

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration and

is not used to encode content defined in the present version of this API.

description: |

Represents the IMS Event Type.

Possible values are:

- SUBSCRIBER_SPECIFIC: Indicates that the IMS Event type is Subscriber Specific for data

channel.

- NON_SUBSCRIBER_SPECIFIC: Indicates that the IMS Event type is Subscriber Specific for

Data channel.

# *** End of Changes ***