3GPP TSG-CT WG3 Meeting #143	C3-254554

# Sophia-Antipolis, FR, 13th – 17th October, 2025	                         (Revision of C3-254372, 4214)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.508 | CR | 0370 | rev | 2 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of QoS parameters reporting |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT3 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2025-10-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | qosPara attribute, introduced in R19 under the feature control of "QoSAssistance", is applicable for "QFI_ALLOC" event. As this event was introduced in R17, the description of the feature in clause 5.8 should indicate it supports the enhancement on "QFI allocation" event including support of QoS parameter set. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Extend the description of "QoSAssistance" feature. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete feature descripiton may lead misunderstanding. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR does not have any impact in the OpenAPI specification. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Additional discussion(if needed):

# Proposed changes:

# *** 1st Change ***

## 5.8	Feature negotiation

The optional features in table 5.8-1 are defined for the Nsmf_EventExposure API. They shall be negotiated using the extensibility mechanism defined in clause 6.6 of 3GPP TS 29.500 [4].

Table 5.8-1: Supported Features

| Feature number |  | Feature Name |  | Description |  |
| --- | --- | --- | --- | --- | --- |
| 1 |  | DownlinkDataDeliveryStatus |  | This feature indicates support for the "Downlink data delivery status" event. |  |
| 2 |  | CommunicationFailure |  | This feature indicates support for the "communication failure" event. |  |
| 3 |  | PduSessionStatus |  | This feature indicates support for the PDU session establishment event and enhancement (PDU session type, IP address) for the PDU session release event. |  |
| 4 |  | QfiAllocation |  | This feature indicates support for the "QFI allocation" event. |  |
| 5 |  | QoSMonitoring |  | This feature indicates support for the "QoS Monitoring" event. (NOTE 1) (NOTE 3) |  |
| 6 |  | ES3XX |  | Extended Support for 3xx redirections. This feature indicates the support of redirection for any service operation, according to Stateless NF procedures as specified in clauses 6.5.3.2 and 6.5.3.3 of 3GPP TS 29.500 [4] and according to HTTP redirection principles for indirect communication, as specified in clause 6.10.9 of 3GPP TS 29.500 [4]. |  |
| 7 |  | EneNA |  | This feature indicates support for exposing information required the enhancements of network data analytics requirements. |  |
| 8 |  | ULBuffering |  | This feature indicates support for Uplink buffering indication. (See NOTE 2) |  |
| 9 |  | SMCCE |  | This feature indicates support for Session Management Congestion Control Experience for PDU Session. |  |
| 10 |  | Dispersion |  | This feature indicates support for Session Management transactions dispersion. |  |
| 11 |  | ERIR |  | Indicates the support of immediate report of the available subscribed event(s) within the subscription response to the NF service consumer. |  |
| 12 |  | RedundantTransmissionExp |  | This feature indicates support for Redundant Transmission Experience. |  |
| 13 |  | WlanPerformance |  | This feature indicates support for WLAN information on PDU Session for which Access Type is NON_3GPP_ACCESS and RAT Type is TRUSTED_WLAN, to support WLAN performance analytics. |  |
| 14 |  | EASIPreplacement |  | This feature indicates the support of provisioning of EAS IP replacement info. (See NOTE 2) |  |
| 15 |  | BIUMR |  | This feature bit indicates whether the NF Service Consumer (e.g. SMF) and PCF supports Binding Indication Update for multiple resource contexts specified in clauses 6.12.1 and 5.2.3.2.6 of 3GPP TS 29.500 [4]. |  |
| 16 |  | UeCommunication |  | This feature indicates the support exposing information required by UE communication analytics, i.e. User Plane status information. |  |
| 17 |  | ServiceExperience |  | This feature indicates the support for exposing UPF information required e.g. by QoS Sustainability analytics. (NOTE 4) |  |
| 18 |  | DnPerformance |  | This feature indicates the support for exposing UPF information required e.g. by QoS Sustainability analytics. (NOTE 4) |  |
| 19 |  | MultipleFlowDescriptions |  | This feature indicates the support of the report of multiple UL and/or DL flows. |  |
| 20 |  | PacketDelayFailureReport |  | This feature indicates the support of packet delay failure report as part of QoS Monitoring procedures. This feature requires that QosMonitoring feature is supported. (NOTE 1) |  |
| 21 |  | CommonEASDNAI |  | This feature indicates support of enhancements of UP path change event notification. (NOTE 1) |  |
| 22 |  | PduSessionInfo |  | This feature indicates support for PDU Session parameters information. |  |
| 23 |  | EnhDataMgmt |  | Indicates the support of enhanced data management mechanisms. Supporting this feature also requires the support of feature EneNA. |  |
| 24 |  | WlanPerformanceExt_AIML |  | This feature indicates support for the enhancements of WLAN performance supporting AIML, including support of analytics per UE granularity. Supporting this feature also requires the support of feature WlanPerformance. |  |
| 25 |  | EasRelocationEnh |  | This feature indicates enhanced support of EAS relocation procedures via additional information about the AFs that are responsible for certain EAS. |  |
| 26 |  | UPEAS |  | This feature indicates the support of UPF enhancements for exposure. |  |
| 27 |  | EnSatBackhaulCategoryChg |  | This feature indicates the support of notification of a change between different satellite backhaul categories, or dynamic satellite backhaul categories, or between satellite backhaul and non-satellite backhaul. |  |
| 28 |  | Void |  |  |  |
| 29 |  | AreaFilter |  | This feature indicates support for using an area as a subscription filter. |  |
| 30 |  | MultipleAccessTypes |  | This feature indicates the support of providing list of Access Type(s) used for the PDU Session. This is used for MA PDU sessions as well. |  |
| 31 |  | EnQfiAllocation |  | Indicates the enhancement on "QFI allocation" event including support of 5QI. Supporting this feature also requires the support of feature QfiAllocation. |  |
| 32 |  | EnQoSMon |  | This feature indicates the support of enhanced QoS monitoring functionality, i.e. the report of the congestion information, and/or, the data rate information monitoring. (NOTE 1) (NOTE 3) This feature requires that QosMonitoring feature is supported. |  |
| 33 |  | HR-SBO |  | This feature indicates the support of extensions to User Plane Path Change event notifications to support Home Routed sessions with Session Breakout. (NOTE 2) |  |
| 34 |  | EnUPEAS |  | This feature indicates the support of UPF enhancements for exposure during UPF relocation and PDU Session release. The following functionalities are supported: - provision the remaining data reporting indication for the UPF_EVENT event type. This feature requires that UPEAS feature is supported. |  |
| 35 |  | TraffRouteReqOutcome |  | This feature indicates the support for reporting the installation outcome of the requested traffic routing requirements. (NOTE 1) |  |
| 36 |  | UeSatUeComm |  | This feature indicates the support of reporting about serving satellite identity for UE-Satellite-UE communication in IMS. |  |
| 37 |  | SimConnFailure |  | This feature indicates the support of Simultaneous Connectivity failure events. (NOTE 1) |  |
| 38 |  | QoSAssistance |  | This feature indicates the support of QFI deallocation and QoS flow change events, and indicates the enhancement on "QFI allocation" event including support of QoS parameter set. This feature requires the support of the EnQfiAllocation feature. |  |
| 39 |  | Energy |  | This feature indicates the support of provisioning the energy consumption information. |  |
| 40 |  | SignallingInfo |  | This feature indicates the support of signalling information events. |  |
| 41 |  | EnhEventMgmt |  | This feature indicates the support of enhanced event management. The following functionalities are supported: - supporting the reference Id per event. |  |
| NOTE 1: SMF determines the support of this feature by the NF service consumer as part of the implicit subscription information provided by the PCF as described in 3GPP TS 29.512 [14]. NOTE 2: NF service consumers determine the support of this feature as part of the notification of the implicitly subscribed events as described in clause 4.2.2.2. NOTE 3: The negotiation of this feature may be explicit (via Nsmf_EventExposure_Subscribe service operation) or implicit as described in NOTE 1. NOTE 4: The features "ServiceExperience" and "DnPerformance" indicate the support of exactly the same functionality of exposing UPF information, but they are both kept for backwards compatibility purposes. An NF service consumer may use these features for any purpose that requires UPF Information and not only for the calculation of QoS Sustainability analytics. |  |  |  |  |  |

*** End of Changes ***