3GPP TSG-CT WG3 Meeting #143	C3-254560

# Sophia-Antipolis, FR, 13 - 17 October 2025                                            (was C3-254442)

Source:	Samsung, Interdigital, Ericsson

Title:	Pseudo-CR on updates on AIMLES_MLModelTraining API

Spec:	3GPP TS 29.482 v1.1.0

Agenda item:	19.41

Document for:	Decision

1. Introduction

This pseudo CR implements the update to ML Model Training Request API defintion. This CR also implements the service description for ML model training API.

2. Reason for Change

As per the agreed CRs (S6-253761 and S6-253618), there has been modifications to the AIMLES_MLModelTraining API. This pCR implements the agreed changes in stage-2.

3. Conclusions

N/A

4. Proposal

It is proposed to agree the following changes to 3GPP TS 29.482 v1.1.0.

* * * First Change * * *

## 5.1	Introduction

Table 5.1-1 summarizes the corresponding APIs defined for this specification.

Table 5.1-1: API Descriptions

| Service Name | Clause | Description | OpenAPI Specification File | API Name | Annex |
| --- | --- | --- | --- | --- | --- |
| AIMLES_ContextTransfer | 6.1.1 | AIMLE Context Transfer Information Service | TS29482_AIMLES_ContextTransfer.yaml | aimles-ct | A.2 |
| AIMLES_DataManagement | 6.1.2 | AIMLE Data Management assistance Service | TS29482_AIMLES_DataManagement.yaml | aimles-dm | A.3 |
| AIMLES_FLMemberGroupSupport | 6.1.3 | FL Member Grouping Service | TS29482_AIMLES_FLMemberGroupSupport.yaml | aimles-fl | A.X |
| AIMLES_AIMLEServiceOperationsManagement | 6.1.4 | AIMLE Operation Management service | TS29482_AIMLES_AIMLEServiceOperationsManagement API.yaml | aimles-opm | A.8 |
| AIMLES_HierarchicalComputingAssist | 6.1.5 | AIMLE Hierarchical Computing Assist service | TS29482_AIMLES_HierarchicalComputingAssist.yaml | aimles-hca | A.9 |
| AIMLES_AIMLEClientDiscovery | 6.1.6 | AIMLE Client discovery service | TS29482_AIMLES_AIMLEClientDiscovery.yaml | aimles-disc | A.5 |
| AIMLES_AIMLEClientSelection | 6.1.7 | AIMLE Client Selection service | TS29482_AIMLES_AIMLEClientSelection.yaml | aimles-sel | A.7 |
| MLR_MLModelManagement | 6.2.1 | MLR Model Management Service API | TS29482_MLR_MLModelManagement.yaml | mlr-mlmm | A.4 |
| MLR_ModelInformationDiscovery | 6.2.2 | MLR Model Information Discovery service | TS29482_MLR_ModelInformationDiscovery.yaml | mlr-mid | A.6 |
| AIMLES_TLModelSelectionAssistance | 5.2.11 | TL enablement service | TS29482_AIMLES_TLModelSelectionAssistance.yaml | aimles-tlmsa | A.X |
| AIMLES_MLModelTraining | 6.1.8 | AIMLE ML Model Training Service | TS29482_AIMLES_MLModelTraining.yaml | aimles-trn | A.12 |

NOTE:	When 3GPP TS 29.122 [2] is referenced for the common protocol and interface aspects for API definition in the clauses under clause 5, the service producer (e.g., AIMLE Server, AIMLE Repository) takes the role of the SCEF and the service consumer takes the role of the SCS/AS.

* * * Second Change * * *

### 5.2.xx	AIMLES_MLModelTraining Service

#### 5.2.xx.1	Service Description

The AIMLES_MLModelTraining Service, exposed by the AIMLE Server, enables a service consumer to:

-	request the AIMLE server for the ML model training.

#### 5.2.xx.2	Service Operations

##### 5.2.xx.2.1	Introduction

The service operations defined for the AIMLES_MLModelTraining API are shown in the Table 5.2.xx.2.1-1.

Table 5.2.xx.2.1-1: Service operations of the AIMLES_MLModelTraining API

| Service Operation Name | Description | Initiated by |
| --- | --- | --- |
| AIMLES_MLModelTraining_Request | This service operation enables a service consumer to request AIMLE server for ML model training. | e.g., VAL Server |
| AIMLES_MLModelTraining_Notify | This service operation enables a service consumer to receive ML model training notifications. | e.g., VAL server |

##### 5.2.xx.2.2	AIMLES_MLModelTraining_Request

5.2.xx.2.2.1	General

This service operation is used by a service consumer to perform AIMLE ML Model Training Request at the AIMLE Server.

The following procedures are supported by the "AIMLES_MLModelTraining_Request" service operation:

-	AIMLE ML Model Training Request.

5.2.xx.2.2.2	AIMLE ML Model Training Request

Figure 5.2.xx.2.2.2-1 depicts a scenario where a service consumer sends a request to the AIMLE Server to request AIMLE ML Model Training (see also clause 8.3 of 3GPP°TS°23.482°[13]).

Figure 5.2.xx.2.2.2-1: Procedure for AIMLE ML Model Training Request

1.	In order to request to AIMLE ML model training, the service consumer shall send an HTTP POST request to the AIMLE Server targeting the URI of the custom operation "RequestTrain" in "/request-train".

2a.	Upon success, the AIMLE Server shall respond with an HTTP "200 OK" status code with the response body containing the "MlModelTrainResp" data structure.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP GET response body, as specified in clause 6.1.8.7.

##### 5.2.xx.2.3	AIMLES_MLModelTraining_Notify

5.2.xx.2.3.1	General

This service operation is used by a AIMLE Server to notify a previously subscribed service consumer on:

-	AIMLE ML Model Training notification.

The following procedures are supported by the "AIMLE_MLModelTraining_Notify" service operation:

-	AIMLE ML Model Training Notification.

5.2.xx.2.3.2	AIMLE ML Model Training Notification

Figure 5.2.xx.2.3.2-1 depicts a scenario where the AIMLE Server sends a request to notify a previously subscribed service consumer on AIMLE ML Model Training report(s) (see also clause 8.15 of 3GPP°TS°23.482°[13]).

Figure 5.2.xx.2.3.2-1: Procedure for AIMLE ML Model Training Notification

1.	In order to notify a previously subscribed service consumer on AIMLE ML Model Training report(s), the AIMLE Server shall send an HTTP POST request to the service consumer with the request URI set to "{notifUri}", where the "notifUri" variable is set to the value received from the service consumer during the creation of the corresponding AIMLE ML Model Training Request using the procedures defined in clauses 5.2.xx.2.3.2, and the request body including the MlModelTrainNotif data structure.

2a.	Upon success, the service consumer shall respond to the AIMLE Server with an HTTP "204 No Content" status code to acknowledge the reception of the notification.

2b.	On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the HTTP POST response body, as specified in clause 6.1.8.7.

* * * Third Change * * * *

##### 6.1.8.4.2	Operation: RequestTrain

6.1.8.4.2.1	Description

The custom operation enables a service consumer to send AIMLE ML Model training request to the AIMLE server.

6.1.8.4.2.2	Operation Definition

This operation shall support the response data structures and response codes specified in tables 6.1.8.4.2.2-1 and 6.1.8.4.2.2-2.

Table 6.1.8.4.2.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| TrainRequest | M | 1 | Contains the parameters to request AIMLE ML Model training. |

Table 6.1.8.4.2.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MlModelTrainResp | M | 1 | 200 OK | Successful case. The ML Model training request is successfully received and processed. The identifier of the ML model selected by AIMLE server for training and the ML model training identifier shall be included in the response message. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative target URI located in an alternative target AIMLE server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative target URI located in an alternative target AIMLE server. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [2]. |
| NOTE: The manadatory HTTP error status code for the <e.g. HTTP POST> method listed in table 5.2.1.6-1 of 3GPP TS 29.122 [2] also apply. |  |  |  |  |

* * * Fourth Change * * * *

#### 6.1.8.6	Data Model

##### 6.1.8.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.8.6.1-1 specifies the data types defined for the AIMLES_MLModelTraining API.

Table 6.1.8.6.1-1: AIMLES_MLModelTraining API specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| CommonId | 6.1.8.6.2.12 | Represents the common features information. |  |
| EarlyStopCri | 6.1.8.6.2.10 | Represents the early stopping criteria. |  |
| FeatureList | 6.1.8.6.2.13 | Represents the feature list of an AIMLE client. |  |
| MemberInfo | 6.1.8.6.2.6 | Represents the list of AIMLE clients selected or de-selected for the ML model training. |  |
| MemberSelCriteria | 6.1.8.6.2.4 | Represents the criteria to be continuously monitored for selecting the member clients. |  |
| MlModelInfo | 6.1.8.6.2.5 | Represents the ML model information. |  |
| MlModelTrainNotif | 6.1.8.6.2.3 | Represents the ML Model training notification |  |
| MlModelTrainResp | 6.1.8.6.2.8 | Represents the ML model training response. |  |
| PerfParam | 6.1.8.6.2.7 | Represents the output of training, e.g., ML model performance parameters for the training. |  |
| TrainingErr | 6.1.8.6.3.4 | Represents the error encountered during training. |  |
| TrainRequest | 6.1.8.6.2.2 | Represents the ML Model training request |  |
| TrainingObj | 6.1.8.6.2.9 | Represents the termination condition for ML model training. |  |
| TrainingType | 6.1.8.6.3.3 | Represents the training type. |  |
| VFLParam | 6.1.8.6.2.11 | Represents the VFL paramerters. |  |

Table 6.1.8.6.1-2 specifies data types re-used by the AIMLES_MLModelTraining API from other specifications, including a reference to their respective specifications, and when needed, a short description of their use within the AIMLES_MLModelTraining API.

Table 6.1.8.6.1-2: AIMLES_MLModelTraining API re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| ClientCapability | 3GPP TS 24.560 [16] | Represents the client capability information. |  |
| Endpoint | 3GPP TS 29.558 [19] | Represents the endpoint information. |  |
| Float | 3GPP TS 29.571 [15] | Used to represent the fractional part of the proximity range in the reference UE details. |  |
| LocationArea5G | 3GPP TS 29.122 [2] | Used to indicate a location area represented as list of geographic areas, civic addresses and network area. |  |
| MlModel | Clause 6.2.1.6.2.4 | Represents the ML model that has to be distributed to the selected member clients for training. |  |
| PerformanceMetric | Clause 6.1.11.6.3.3 | Represents the performance metric for training the ML model. |  |
| TimeWindow | 3GPP TS 29.122 [2] | Indicates the time window. |  |
| Uri | 3GPP TS 29.122 [2] | Represent an URI, used to indicate the notification URI. |  |

##### 6.1.8.6.2	Structured data types

6.1.8.6.2.1	Introduction

This clause defines the structures to be used in resource representations.

###### 6.1.8.6.2.2	Type: TrainRequest

Table 6.1.8.6.2.2-1: Definition of type TrainRequest

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| trnType | TrainingType | M | 1 | Contains the type of training to be performed. |  |
| members | array(AimleClientId) | C | 1..N | Contains the list of member clients to be utilized for training the ML model. (NOTE 1) |  |
| memSelCrit | MemberSelCriteria | C | 0..1 | Contains the criteria that needs to be continuously monitored for selecting the member clients. (NOTE 1) |  |
| numReqClients | Uinteger | O | 0..1 | Contains the requested number of AIML clients to be selected based on member selection criteria. |  |
| modelInf | MlModelInfo | C | 0..1 | Contains the ML model that has to be distributed to the selected member clients for training. (NOTE 2) |  |
| modelReq | MlModel | C | 0..1 | Contains the requirement for selecting a model to be trained and the filtering criteria for selecting the model. (NOTE 2) |  |
| trainObj | TrainingObj | O | 0..1 | Contains the termination condition for the ML model training. |  |
| datasetId | array(string) | M | 1..N | Contains the identifiers for dataset used for HFL or VFL training. For VFL, multiple dataset identifiers can be specified. |  |
| dataSamples | Uinteger | O | 0..1 | Contains the number of data samples required for one round of HFL or VFL training. |  |
| opSchedule | string | O | 0..1 | Contains the schedule for when training is to occur. |  |
| vflParam | VFLParam | C | 0..1 | Contains the parameters specific to VFL training. This IE shall be present for VFL training. |  |
| memUpdNotif | Boolean | O | 0..1 | Indicates whether the requestor needs to be notified whenever there is update related to new member clients selected or de-selected. Set to "true" to indicate that requestor needs to be notified whenever there is update related to new member clients selected or de-selected. Default value is "false" if omitted. |  |
| notifUri | Uri | C | 0..1 | Contains the notification URI where notifications should be sent. This attribute shall be present when the attribute "memUpdNotif" is set to "true". |  |
| NOTE 1: At least one of these attributes shall be present. NOTE 2: At least one of these attributes shall be present. |  |  |  |  |  |

###### 6.1.8.6.2.3	Type: MlModelTrainNotif

Table 6.1.8.6.2.3-1: Definition of type MlModelTrainNotif

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| trainingID | string | C | 0..1 | Contains an identifier for ML model training request. |  |
| members | array(MemberInfo) | C | 1..N | Contains the list of AIMLE clients selected or de-selected for the ML model training. (NOTE) |  |
| trainOut | PerfParams | C | 0..1 | Contains the output of training, e.g., ML model parameters for the training. (NOTE) |  |
| percentageComp | integer | C | 0..1 | Contains the completion percentage for the training. Minimum: 0, maximum: 100. |  |
| trainErr | TrainingErr | C | 0..1 | Contains the list of errors, if any, encountered during training process. (NOTE) |  |
| NOTE: At least one of these attributes shall be present. |  |  |  |  |  |

* * * Fifth Change * * * *

###### 6.1.8.6.2.7	Type: PerfParam

Table 6.1.8.6.2.7-1: Definition of type PerfParam

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| modelAccuracy | integer | C | 0..1 | Indicates the achieved ML model accuracy, expressed as a percentage. Minimum: 0, maximum: 100. |  |
| modelPrecision | Float | C | 0..1 | Represents the accuracy for the positive predictions made by the model. Minimum: 0, maximum: 1. |  |
| modelRecall | Float | C | 0..1 | Represents model's ability to identify all the actual positive instances within a dataset. Minimum: 0, maximum: 1. |  |
| modelF1Score | Float | C | 0..1 | Represents the combined metric for precision and recall. Minimum: 0, maximum: 1. |  |
| errorMeanSquare | Float | C | 0..1 | Represents the mean squared error between predicted values and actual values. |  |
| errorMeanAbs | Float | C | 0..1 | Represents the mean absolute error between predicted values and actual values. |  |
| NOTE: At least one of these attributes shall be present. |  |  |  |  |  |

###### 6.1.8.6.2.8	Type: MlModelTrainResp

Table 6.1.8.6.2.8-1: Definition of type MlModelTrainResp

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| trainingId | string | M | 1 | Contains an identifier for ML model training request. |  |
| modelId | string | O | 0..1 | Contains the identifier for the ML model selected by AIMLE server. |  |

###### 6.1.8.6.2.9	Type: TrainingObj

Table 6.1.8.6.2.9-1: Definition of type TrainingObj

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| objType | PerformanceMetric | M | 1 | Contains the metric to be optimized by the ML model (e.g., accuracy). |  |
| targetValue | Float | O | 0..1 | Contains the threshold to be reached for the objective type. |  |
| earlyStopCri | EarlyStopCri | O | 0..1 | Contains the metric to be used for early stopping. |  |
| maxEpochs | Uinteger | O | 0..1 | Contains the maximum number of training epochs. |  |
| accTrainingErr | Float | O | 0..1 | Contains the maximum acceptable traning error. |  |
| inferenceLatency | Float | O | 0..1 | Contains the inference latency requirements for the trained model. |  |

###### 6.1.8.6.2.10	Type: EarlyStopCri

Table 6.1.8.6.2.10-1: Definition of type EarlyStopCri

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| minDelta | Float | O | 0..1 | Contains the threshold to consider as improvement. |  |
| patience | Uinteger | O | 0..1 | Contains the number of epochs without improvement to stop the training. |  |

###### 6.1.8.6.2.11	Type: VFLParam

Table 6.1.8.6.2.11-1: Definition of type VFLParam

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| datasetComm | array(CommonId) | O | 1..N | Contains the list of one or more common features required for VFL training. (E.g., UE ID, AIMLE client ID, Group ID, VAL service ID, etc) |  |
| featureList | array(FeatureList) | O | 1..N | Contains the list of features for each data domain of the dataset at the client. |  |
| featureAlign | array(FeatureList) | O | 1..N | Contains the information to align features from dataset of different domains for VFL training. |  |
| dataLabels | array(string) | O | 1..N | Contains the ground truth data for VFL training. |  |

###### 6.1.8.6.2.12	Type: CommonId

Table 6.1.8.6.2.12-1: Definition of type CommonId

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| featureType | string | O | 0..1 | Contains the type of common features that can be UE identifiers, AIMLE client identifiers, group identifier, VAL service identifier, area of interest, and VAL service area. |  |
| featureId | string | O | 0..1 | Contains the common identifier for the given feature type. |  |

###### 6.1.8.6.2.13	Type: FeatureList

Table 6.1.8.6.2.13-1: Definition of type FeatureList

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| clientId | string | O | 0..1 | Contains the identifier for the AIMLE clients participating in the VFL training. |  |
| list | array(string) | O | 1..N | Contains the list of all the features for dataset at the AIMLE clients. |  |

* * * End of Changes * * *