3GPP TSG-CT WG3 Meeting #143	C3-254594

# Sophia-Antipolis, France, 13 – 17 October 2025						(Revision of C3-254348)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.561 | CR | 0202 | rev | 1 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to MRI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C3 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM_Ph2 |  |  |  |  |  | Date: |  |  | 2025-10-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The reference number of TS 26.522 in clause 22.2 need to be corrected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Corrected the reference number of TS 26.522 in clause 22.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Wrong reference number in MRI definition. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * 1st Change * * * *

## 22.2	Media Related Information

Media Related Information supporting information fields as defined in 3GPP TS 23.501 [2] shall be byte-aligned and coded as shown in Figure 22.2.1 and Table 22.2.1.

| 0 |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  |  |  |  |  | 2 |  |  |  |  |  |  |  |  |  | 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 0 | 1 |
| Version |  |  | Bitmask |  |  |  |  |  |  |  |  |  |  |  |  | E | R |  | D | PSI |  |  |  | PSSN |  |  |  |  |  |  |  |
| Cont |  | PSN |  |  |  |  |  | PSSize |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NPDS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | BSize |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Cont |  |  |  |  |  |  |  | TTNB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| I | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Figure 22.2.1: Media Related Information

Table 22.2.1: Media Related Information

| Media Related Information |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Version (octet 0, bits 0 through 2) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  | 0 |  |  |  | 0 |  |  |  |  |  |  | Bits representing MRI version 1 |
| Bitmask (octet 0, bit 3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  | Bits representing PDU Set marking not present |  |  |  |
| 1 |  |  |  |  |  |  |  |  |  |  |  | Bits representing PDU Set marking present |  |  |  |
| Bitmask (octet 0, bit 4) (NOTE 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  | Bits representing PDU Set Size (PSSize) not present |  |  |  |
| 1 |  |  |  |  |  |  |  |  |  |  |  | Bits representing PDU Set Size (PSSize) present |  |  |  |
| Bitmask (octet 0, bit 5) (NOTE 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  | Bits representing Number of PDUs in the PDU Set (NPDS) not present |  |  |  |
| 1 |  |  |  |  |  |  |  |  |  |  |  | Bits representing Number of PDUs in the PDU Set (NPDS) present |  |  |  |
| Bitmask (octet 0, bit 6) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  | Bits representing Burst Size (BSize) not present |  |  |  |
| 1 |  |  |  |  |  |  |  |  |  |  |  | Bits representing Burst Size (BSize) present |  |  |  |
| Bitmask (octet 0, bit 7) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  | Bits representing Time To Next Burst (TTNB) not present |  |  |  |
| 1 |  |  |  |  |  |  |  |  |  |  |  | Bits representing Time To Next Burst (TTNB) present |  |  |  |
| Bitmask (octet 1, bit 0) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  | Bits representing Expedited Transfer Indication (I) not present |  |  |  |
| 1 |  |  |  |  |  |  |  |  |  |  |  | Bits representing Expedited Transfer Indication (I) present |  |  |  |
| Bitmask (octet 1, bits 1 through 6) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| These bits are spared and shall be set to zero. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bitmask (octet 1, bit 7) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Extension for the bitmask by 8 bits, if the existing bits are exhausted and new bits are needed to represent new fields in Media Related Information (NOTE 2). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  |  | Bitmask is not extended by 8 bits |  |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  | Bitmask is extended by 8 bits |  |
| End PDU of the PDU Set (1 bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| If PDU Set marking is included, this bit is encoded as End PDU of the PDU Set (E) field of the PDU Set marking as defined in 3GPP TS 26.522 [70]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reserved (2 bits) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| If PDU Set marking is included, these bits are encoded as Reserved (R) field of the PDU Set marking as defined in 3GPP TS 26.522 [70]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| End of Data Burst (1 bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| If PDU Set marking is included, this bit is encoded as End of Data Burst (D) field of the PDU Set marking as defined in 3GPP TS 26.522 [70]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDU Set Importance (4 bits) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| If PDU Set marking is included, this bit is encoded as PDU Set Importance (PSI) field of the PDU Set marking as defined in 3GPP TS 26.522 [70]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDU Set Sequence Number (10 bits) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| If PDU Set marking is included, these bits are encoded as with PDU Set Sequence Number (PSSN) field of the PDU Set marking as defined in 3GPP TS 26.522 [70]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDU Sequence Number within the PDU Set (6 bits) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| If PDU Set marking is included, these bits are encoded as PDU Sequence Number within the PDU Set (PSN) field of the PDU Set marking as defined in 3GPP TS 26.522 [70]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDU Set Size (24 bits) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| If PDU Set marking is included, these bits are encoded as PDU Set Size (PSSize) field of the PDU Set marking as defined in 3GPP TS 26.522 [70]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Number of PDUs in the PDU Set (16 bits) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| If PDU Set marking is included, these bits are encoded as Number of PDUs in the PDU Set (NPDS) field of the PDU Set marking as defined in 3GPP TS 26.522 [70]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Burst Size (24 bits) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| If Burst Size is included, these bits are encoded as Burst Size (BSize) field of the Dynamically Changing Traffic Characteristics marking as defined in 3GPP TS 26.522 [70]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Time To Next Burst (24 bits) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| If Time To Next Burst is included, these bits are encoded as Time To Next Burst (TTNB) field of the Dynamically Changing Traffic Characteristics marking as defined in 3GPP TS 26.522 [70]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Expedited Transfer Indication (1 bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| If Expedited Transfer Indication is included, this bit corresponds to Expedited Transfer Indication (I) field as defined in 3GPP TS 23.501 [2] and is encoded to the value: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  | false |  |  |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  | true |  |  |
| All other additional zero-padding bits shall be ignored. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: This bit represents an optional field of the PDU Set marking, see 3GPP TS 26.522 [70]. If the PDU Set marking are not included in the Media Related Information, this bit shall be set to zero and the associated bits representing the optional field shall not be included in the Media Related Information. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 2: The extension for the bitmask may be repeated by assigning 1 to the last bit i.e., bit 7 of the previous extension. If the bitmask is extended with additional bits, those unused bits of the bitmask extension are spare and shall be set to zero. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

# * * * * End of Changes * * * *