3GPP TSG CT WG3 Meeting #143	C3-254614

# Sophia-Antipolis, FR, 13th – 17th October, 2025		was C3-254587 was C3-254509 was C3-254147

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.222 | CR | 0431 | rev | 3 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates and corrections to the Initiate_Authentication service operation of the AEF_Security_API |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |
| Source to TSG: | CT3 |  |  |  |  |  |  |  |  |
| Work item code: | NBI19 |  |  |  |  |  | Date: |  | 2025-10-17 |
| Category: | F |  |  |  |  |  | Release: |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  |  | The following issues were identified in the provisions related to the authentication initiation procedure/service operation: - There are some additional editorial issues, misalignments with the drafting rules, missing clauses in the data model section and various text enhancements that are needed. |  |  |  |  |  |  |
| Summary of change: |  |  | This CR proposes to: - Address the above-detailed issues. |  |  |  |  |  |  |
| Consequences if not approved: |  |  | - The above-detailed issues with stage 2 remain in the specification. |  |  |  |  |  |  |
| Clauses affected: |  | 6.1, 6.2.2.1, 6.2.2.2.1, 6.2.2.2.2, 9.1.2A.2.1, 9.1.2A.2.2, 9.1.4.1, A.10 |  |  |  |  |  |  |  |
|  |  | Y |  | N |  |  |  |  |  |
| Other specs |  |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |
| affected: |  |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |
| (show related CRs) |  |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |
| Other comments: |  | This CR does not impact the OpenAPI descriptions of the APIs. |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |

* * * * Start of changes * * * *

## 6.1	Introduction of Services

The table 6.1-1 lists the AEF APIs below the service name. A service description clause for each API gives a general description of the related API.

Table 6.1-1: List of AEF Services

| Service Name | Service Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| AEF_Security_API | Initiate_Authentication | Request/ Response | API Invoker |
|  | Revoke_Authorization | Request/ Response | CCF |

Table 6.1-2 summarizes the corresponding APIs defined in this specification.

Table 6.1-2: API Descriptions

| Service Name | Clause | Description | OpenAPI Specification File | apiName | Annex |
| --- | --- | --- | --- | --- | --- |
| AEF_Security_API | 9.1 | AEF Security API Service | TS29222_AEF_Security_API.yaml | aef-security | A.10 |

* * * * Next changes * * * *

#### 6.2.2.1	Introduction

The service operation defined for AEF_Security_API is shown in table 6.2.2.1-1.

Table 6.2.2.1-1: Operations of the AEF_Security_API

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| Initiate_Authentication | This service operation is used by an API Invoker to request API exposing function to confirm necessary authentication data is available to authenticate the API Invoker | API Invoker |
| Revoke_Authorization | This service operation is used by the CCF to request the AEF to revoke the authorization of service API(s) for an API Invoker. | CCF |

* * * * Next changes * * * *

##### 6.2.2.2.1	General

This service operation is used by an API Invoker to initiate authentication with the AEF. On receiving the Initiate_Authentication, the AEF fetches the authentication information of the API Invoker from the CCF, if required.

* * * * Next changes * * * *

##### 6.2.2.2.2	API invoker initiating authentication using Initiate_Authentication service operation

To initiate authentication with the AEF, the API Invoker shall send an HTTP POST request message to the AEF with the API Invoker ID to the URI "{apiRoot}/aef-security/v1/check-authentication".

Upon receiving the above described HTTP POST request message, the AEF shall check if the credentials of the API invoker for authentication are available with the AEF. If the credentials of the API invoker for authentication are not available, the AEF shall use the service defined in clause 5.6.2.4.2 to fetch the credentials from the CCF.

The AEF shall store the received credentials and respond to the API Invoker with an HTTP "200 OK" status code.

* * * * Next changes * * * *

##### 9.1.2A.2.1	Description

This custom operation allows the API Invoker to confirm from the AEF, that necessary authentication data is available to authenticate the API Invoker on API invocation.

* * * * Next changes * * * *

##### 9.1.2A.2.2	Operation Definition

This method shall support the URI query parameters specified in table 9.1.2A.2.2-1.

Table 9.1.2A.2.2-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This operation shall support the request and response data structures, and response codes specified in tables 9.1.2A.2.2-2 and 9.1.2A.2.2-3.

Table 9.1.2A.2.2-2: Data structures supported by the POST Request Body on this operation

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| CheckAuthenticationReq | M | 1 | Authentication check request data. |

Table 9.1.2A.2.2-3: Data structures supported by the POST Response Body on this operation

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| CheckAuthenticationRsp | M | 1 | 200 OK | The request was successful. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative AEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [14]. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection. The response shall include a Location header field containing an alternative URI of the resource located in an alternative AEF. Redirection handling is described in clause 5.2.10 of 3GPP TS 29.122 [14]. |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [14] shall also apply. |  |  |  |  |

Table 9.1.2A.2.2-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AEF. |

Table 9.1.2A.2.2-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains an alternative URI of the resource located in an alternative AEF. |

* * * * Next changes * * * *

#### 9.1.4.1	General

This clause specifies the application data model supported by the API. Data types listed in clause 7.2 apply to this API.

Table 9.1.4.1-1 specifies the data types defined specifically for the AEF_Security_API service.

Table 9.1.4.1-1: AEF_Security_API specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| CheckAuthenticationReq | Clause 9.1.4.2.2 | Represents authentication check request data. |  |
| CheckAuthenticationRsp | Clause 9.1.4.2.3 | Represents authentication check response data. |  |
| RevokeAuthorizationReq | Clause 9.1.4.2.4 | Represents authorization revocation request data. |  |
| RevokeAuthorizationRsp | Clause 9.1.4.2.5 | Represents authorization revocation response data. |  |

Table 9.1.4.1-2 specifies data types re-used by the AEF_Security_API service.

Table 9.1.4.1-2: Re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| SecurityNotification | Clause 8.5.4.2.5 | Represents information about the revoked APIs. |  |
| SupportedFeatures | 3GPP TS 29.571 [19] | Represents the list of supported feature(s) and used to negotiate the applicability of the optional features defined in Table 9.1.6-1. |  |

* * * * End of changes * * * *