3GPP TSG CT WG3 Meeting #145	C3-260412

# Goa, India, 9 - 13 February, 2026											(Revision of C3-260078)

| CR-Form-v12.5.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.437 | CR | 0003 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Spatial Anchor usage report correction |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C3 |  |  |  |  |  |  |  |  |  |
| Work item code: | Metaverse_App |  |  |  |  |  | Date: |  |  | 2026-02-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) Rel-21 (Release 21) |  |
| Reason for change: |  | In the SS_SAnUsage OpenAPI, the HTTP POST operation’s - 204 “No content” response incorrectly includes a message body - The report customer operation redirection response is missing. In the OpenAPI, the AnchorUsage mandatory parameter is missing |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following updates are introduced: - Correction of the 204 response description and removal of the message body - The report customer operation redirection response handling is added. - In the OpenAPI, the AnchorUsage mandatory parameter namely anchId and access are missing. - The OpenAPI formatting changes - In the OpenAPI, the AnchorUsage mandatory parameter is missing |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The openAPI specification will remain incorrect w.r.t HTTP response handling and the 307, 308 responses & its definitions will continue to be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces a backwards compatible correction to the OpenAPI descriptions of the following APIs: TS29437_SS_SAnUsage.yaml |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First Change * * * *

# A.4	SS_SAnUsage API

openapi: 3.0.0

info:

title: SAn Server Spatial Anchor Usage Service

version: 1.0.0

description: |

API for Spatial Anchor Usage Service.

©2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.437 V19.0.0; Service Enabler Architecture Layer for Verticals (SEAL);

Metaverse Enablement Services; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.437/

servers:

- url: "{apiRoot}/ssan-usg/v1"

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 6.5 of 3GPP TS 29.549.

security:

- {}

- oAuth2ClientCredentials: []

paths:

/subscriptions:

post:

summary: Create a new Spatial Anchors Usage Information Subscription.

operationId: CreateSpatialAnchorsUsageSub

tags:

- Spatial Anchors Usage Information Subscription (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/SpatialAnchorsUsageInfoSub'

responses:

'201':

description: >

Created. The Spatial Anchors Usage Information Subscription is successfully created and

a representation of the created Individual Spatial Anchors Usage Information

Subscription resource is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/SpatialAnchorsUsageInfoSub'

headers:

Location:

description: Contains the URI of the newly created resource.

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/subscriptions/{subscriptionId}:

parameters:

- name: subscriptionId

in: path

description: >

Represents the identifier of the Individual Spatial Anchors Usage Information Subscription

resource.

required: true

schema:

type: string

get:

summary: Retrieve an existing Individual Spatial Anchors Usage Information Subscription resource.

operationId: GetIndSpatialAnchorsUsageInfoSub

tags:

- Individual Spatial Anchors Usage Information Subscription (Document)

responses:

'200':

description: >

OK. The requested Individual Spatial Anchors Usage Information Subscription is returned.

content:

application/json:

schema:

$ref: '#/components/schemas/SpatialAnchorsUsageInfoSub'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

summary: Update an existing Individual Spatial Anchors Usage Information Subscription resource.

operationId: UpdateIndSpatialAnchorsUsageInfoSub

tags:

- Individual Spatial Anchors Usage Information Subscription (Document)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/SpatialAnchorsUsageInfoSub'

responses:

'200':

description: >

OK. The Individual Spatial Anchors Usage Information Subscription resource is

successfully updated and a representation of the updated resource is returned in the

response body.

content:

application/json:

schema:

$ref: '#/components/schemas/SpatialAnchorsUsageInfoSub'

'204':

description: >

No Content. The Individual Spatial Anchors Usage Information Subscription resource is

successfully updated and no content is returned in the response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

patch:

summary: Modify an existing Individual Spatial Anchors Usage Information Subscription resource.

operationId: ModifyIndSpatialAnchorsUsageInfoSub

tags:

- Individual Spatial Anchors Usage Information Subscription (Document)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/SpatialAnchorsUsageInfoSubPatch'

responses:

'200':

description: >

OK. The Individual Spatial Anchors Usage Information Subscription resource is

successfully modified and a representation of the updated resource shall be returned in

the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/SpatialAnchorsUsageInfoSub'

'204':

description: >

No Content. The Individual Spatial Anchors Usage Information Subscription resource is

successfully modified and no content is returned in the response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

summary: Delete an existing Individual Spatial Anchors Usage Information Subscription resource.

operationId: DeleteIndSpatialAnchorsUsageInfoSub

tags:

- Individual Spatial Anchors Usage Information Subscription (Document)

responses:

'204':

description: >

No Content. The Individual Spatial Anchors Usage Information Subscription resource is

successfully deleted.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/report:

post:

summary: Enables a service consumer to report the spatial anchor usage information.

operationId: RequestSpatialAnchorsUsageInfo

tags:

- Report Spatial Anchor Usage Information

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/SpatialAnchorUsageReportReq'

responses:

'204':

description: >

Created. The Spatial Anchor Usage Information is successfully received and

processed.

content:

application/json:

schema:

$ref: '#/components/schemas/SpatialAnchorsUsageInfoSub'

headers:

Location:

description: >

Contains an alternative target URI located in an alternative San Server.

required: true

schema:

type: string

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

callbacks:

SpatialAnchorsUsageInfoNotif:

'{$request.body#/notifUri}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

responses:

'204':

description: >

No Content. The Spatial Anchors Usage Information Notification Successfully

received and acknowledged.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: "{tokenUrl}"

scopes: {}

schemas:

#

# STRUCTURED DATA TYPES

#

SpatialAnchorsUsageInfoSub:

description: >

Represents the spatial anchors usage information subscription.

type: object

properties:

eventSubs:

type: array

items:

$ref: '#/components/schemas/AnchorsUsageSub'

minItems: 1

notifUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

report:

$ref: '#/components/schemas/SpatialAnchorsUsageInfoNotif'

reportReq:

$ref: 'TS29523_Npcf_EventExposure.yaml#/components/schemas/ReportingInformation'

expTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- eventSubs

- notifUri

SpatialAnchorsUsageInfoSubPatch:

description: >

Represents the spatial anchors usage information subscription.

type: object

properties:

eventSubs:

type: array

items:

$ref: '#/components/schemas/AnchorsUsageSub'

minItems: 1

notiUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

reportReq:

$ref: 'TS29523_Npcf_EventExposure.yaml#/components/schemas/ReportingInformation'

expTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTime'

AnchorsUsageSub:

description: >

Represent the subscription details related to spatial anchor usage.

type: object

properties:

event:

$ref: '#/components/schemas/AnchorUsageEvent'

evntFltr:

$ref: '#/components/schemas/AnchorsUsageEventFilter'

required:

- event

- evntFltr

AnchorsUsageEventFilter:

description: >

Represents the filter information for anchors usage event.

type: object

properties:

anchAnlFltr:

$ref: '#/components/schemas/AnchorsUsageFilter'

AnchorsUsageFilter:

description: >

Represents the spatial anchors usage filter information.

type: object

properties:

anchIds:

type: array

items:

$ref: 'TS29437_SS_SAnManagement.yaml#/components/schemas/SpatialAnchorId'

minItems: 1

anchLoc:

$ref: 'TS29122_MonitoringEvent.yaml#/components/schemas/LocationInfo'

area:

$ref: 'TS29558_Eecs_EESRegistration.yaml#/components/schemas/ServiceArea'

appId:

type: string

sampInt:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DurationSec'

anyOf:

- required: [anchIds]

- required: [anchLoc]

- required: [area]

- required: [appId]

- required: [sampInt]

SpatialAnchorsUsageInfoNotif:

description: >

Represents the spatial anchors usage information notification.

type: object

properties:

subId:

type: string

reports:

type: array

items:

$ref: '#/components/schemas/SpatialAnchorsUsageReport'

minItems: 1

required:

- subId

- reports

SpatialAnchorsUsageReport:

description: >

Represents the usage report in the notification.

type: object

properties:

event:

$ref: '#/components/schemas/AnchorUsageEvent'

anchUsgInf:

type: array

items:

$ref: '#/components/schemas/AnchorsUsageInfo'

minItems: 1

required:

- event

AnchorsUsageInfo:

description: >

Represents the anchor usage details.

type: object

properties:

anchIds:

type: array

items:

$ref: 'TS29437_SS_SAnManagement.yaml#/components/schemas/SpatialAnchorId'

minItems: 1

discAcc:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

serAcc:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

anchDensity:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

usrDensity:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

required:

- anchIds

- discAcc

- serAcc

- anchDensity

- usrDensity

SpatialAnchorUsageReportReq:

description: >

Represents the request for spatial anchor usage report.

type: object

properties:

anchUsg:

type: array

items:

$ref: '#/components/schemas/AnchorUsage'

minItems: 1

required:

- anchUsg

AnchorUsage:

description: >

Represents the anchor usage information reported.

type: object

properties:

anchId:

$ref: 'TS29437_SS_SAnManagement.yaml#/components/schemas/SpatialAnchorId'

access:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

repPer:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

required:

- anchId

- access

# SIMPLE DATA TYPES

#

#

# ENUMERATIONS

#

AnchorUsageEvent:

anyOf:

- type: string

enum:

- SPATIAL_ANCHOR_USAGE_INFORMATION

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration but

is not used to encode content defined in the present version of this API.

description: |

Represents the type of entity.

Possible values are:

- SPATIAL_ANCHOR_USAGE_INFORMATION: Indicates that the event is related to spatial

anchors usage.

# Data types describing alternative data types or combinations of data types:

#

# *** End of Changes ***