3GPP TSG-CT WG3 Meeting #143	C3-254311

# Sophia-Antipolis, FR, 13 - 17 October 2025

Source:	Samsung, Interdigital

Title:	Pseudo-CR on OpenAPI for AIMLE_MLModelTraining API

Spec:	3GPP TS 29.482 v1.1.0

Agenda item:	19.41

Document for:	Decision

1. Introduction

This pseudo CR implements the OpenAPI for ML Model Training Request API.

2. Reason for Change

ML Model Training API has been agreed in SA6. The API definition for the API has been agreed in CT3#142 meeting. This pCR implements the OpenAPI for the API.

This pCR also includes the new attributes proposed in the updates to this API in another pCR C3-254310.

3. Conclusions

N/A

4. Proposal

It is proposed to agree the following changes to 3GPP TS 29.482 v1.1.0.

* * * First Change * * *

# A.xx	AIMLES_MLModelTraining API

openapi: 3.0.0

info:

title: AIMLE ML Model Training API

version: 1.0.0-alpha.1

description: |

API for AIMLE ML Model Training Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.482 v1.2.0; Service Enabler Architecture Layer for Verticals (SEAL); Artificial

Intelligence Machine Learning Enablement (AIMLE) Services; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.482/

servers:

- url: '{apiRoot}/aimles-trn/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 6.5 of 3GPP TS 29.549

security:

- {}

- oAuth2ClientCredentials: []

paths:

/request-train:

post:

summary: Request the AIMLE server for AIMLE ML Model Training.

operationId: RequestTrain

tags:

- AIMLE ML Model Training service (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/TrainRequest'

responses:

'200':

description: >

OK. The requested ML model training response containing ML model ID and ML model

training ID shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/MlModelTrainResp'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

callbacks:

myNotification:

'{$request.body#/notifUri}':

post:

summary: Notify on the requested data.

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/MlModelTrainNotif'

responses:

'204':

description: The notification is successfully received.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

TrainRequest:

description: >

Represents the AIMLE ML Model Training service.

type: object

properties:

trnType:

$ref: '#/components/schemas/TrainingType'

members:

type: array

items:

$ref: 'TS29482_AIMLES_ContextTransfer.yaml#/components/schemas/AimleClientId'

minItems: 1

memSelCrit:

$ref: '#/components/schemas/MemberSelCriteria'

numReqClients:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

modelInf:

$ref: '#/components/schemas/MlModelInfo'

modelReq:

$ref: 'TS29482_MLR_MLModelManagement.yaml#/components/schemas/MLModel'

trainObj:

$ref: '#/components/schemas/TrainingObj'

datasetId:

type: array

items:

type: string

minItems: 1

dataSamples:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

opSchedule:

type: string

vflParam:

$ref: '#/components/schemas/VFLParam'

memUpdNotif:

type: boolean

notifUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

required:

- trnType

MlModelTrainNotif:

description: >

Represents the AIMLE ML Model Training Notification.

type: object

properties:

trainingID:

type: string

members:

type: array

items:

$ref: '#/components/schemas/MemberInfo'

minItems: 0

trainOut:

$ref: '#/components/schemas/PerfParams'

percentageComp:

type: integer

minimum: 0

maximum: 100

trainErr:

$ref: '#/components/schemas/TrainingErr'

MlModelTrainResp:

description: >

Represents the AIMLE service status information.

type: object

properties:

trainingId:

type: string

modelId:

type: string

required:

- trainingId

MemberSelCriteria:

description: >

Represents the criteria that needs to be continuously monitored for selecting

the member clients.

type: object

properties:

clientLoc:

$ref: 'TS29122_CommonData.yaml#/components/schemas/LocationArea5G'

clientAvailability:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

clientCapability:

$ref: 'TS24560_Aimles_AIMLEClientRegistration.yaml#/components/schemas/ClientCapability'

MlModelInfo:

description: >

Represents the ML model that has to be distributed to the selected member clients

for training.

type: object

properties:

mlModelId:

type: string

mlModelLoc:

$ref: 'TS29558_Eees_EASRegistration.yaml#/components/schemas/EndPoint'

MemberInfo:

description: >

Represents the list of AIMLE clients selected/de-selected for ML model training.

type: object

properties:

clientSel:

type: boolean

clientUri:

$ref: 'TS29122_CommonData.yaml#/components/schemas/Uri'

PerfParams:

description: >

Represents the output of training, e.g., ML model parameters for the training.

type: object

properties:

modelAccuracy:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Float'

modelPrecision:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Float'

modelRecall:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Float'

modelF1score:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Float'

errorMeanSquare:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Float'

errorMeanAbs:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Float'

TrainingObj:

description: >

Represents the termination condition for the ML model training.

type: object

properties:

objType:

$ref: 'TS29482_AIMLES_AssistedMLModelSelection.yaml#/components/schemas/PerformanceMetric'

targetValue:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Float'

earlyStopCri:

$ref: '#/components/schemas/EarlyStopCri'

maxEpochs:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

accTrainingErr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Float'

inferenceLatency:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Float'

required:

- objType

EarlyStopCri:

description: >

Represents the AIMLE service status information.

type: object

properties:

minDelta:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Float'

patience:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

VFLParam:

description: >

Represents the parameters specific to VFL training.

type: object

properties:

datasetComm:

$ref: '#/components/schemas/CommonId'

featureList:

type: array

items:

$ref: '#/components/schemas/FeatureList'

minItems: 0

featureAlign:

type: array

items:

$ref: '#/components/schemas/FeatureList'

minItems: 0

dataLabels:

type: array

items:

type: string

minItems: 0

CommonId:

description: >

Represents the list of one or more common features required for VFL training.

type: object

properties:

featureType:

type: string

featureId:

type: string

FeatureList:

description: >

Represents the list of features for each data domain of the dataset at the

client.

type: object

properties:

clientId:

type: string

list:

type: array

items:

type: string

minItems: 0

# SIMPLE DATA TYPES

#

# ENUMERATIONS

#

TrainingType:

anyOf:

- type: string

enum:

- TRAIN_VFL

- TRAIN_HFL

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

and is not used to encode content defined in the present version of this API.

description: |

Represents the AIMLE service operation status.

Possible values are:

- TRAIN_VFL: Indicates that the training type is horizontal federated learning

- TRAIN_HFL: Indicates that the training type is vertical federated learning

TrainingErr:

anyOf:

- type: string

enum:

- UNDERFITTING

- OVERFITTING

- PERFORMANCE_ERRORS

- DATA_LEAKAGE

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

and is not used to encode content defined in the present version of this API.

description: |

Represents the training error encountered during ML model training.

Possible values are:

- UNDERFITTING: Indicates that the trained model is underfitting the

training data.

- OVERFITTING: Indicates that the trained mode is overfitting the training

data.

- PERFORMANCE_ERRORS: Indicates that the trained model is unable to meet the

desired performance.

- DATA_LEAKAGE: Indicates that there is data leakage from evaluation data set

to the training data.

* * * End of Changes * * *