3GPP TSG-CT WG3 Meeting #143	C3-254597

# Sophia Antipolis, France, 13-17 October 2025				is revision of C3-254126, 4541

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.569 | CR | 0002 | rev | 2 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduce device cause report |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT3 |  |  |  |  |  |  |  |  |  |
| Work item code: | AmbientIoT-CT |  |  |  |  |  | Date: |  |  | 2025-10-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | C1-256531 and C1-256647 of TS 24.369 introduce some cause value for the AIOT service operation failure. E.g., write command is not supported by the AIoT device, Abnormal cases in the AIoT device. It is needed to report the causes to the AF/NEF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce these failure causes in the AIoT notification request body. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Uncomplete procedure for AIOT. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.6.1, 6.1.6.2.8, 6.1.6.3.3, A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backward compatible feature to the following APIs: TS29569_Naiotf_AIoT.yaml TS29522_AIoT.yaml |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Additional discussion(if needed):

# Proposed changes:

# *** 1st Change ***

6.1.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.6.1-1 specifies the data types defined for the Naiotf_AIoT service-based interface protocol.

Table 6.1.6.1-1: Naiotf_AIoT specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| AIoTNotif | 6.1.6.2.6 | Represents the AIoT Operations Notification. |  |
| AIoTDevFailCause | 6.1.6.3.3 | Represents the per-AIoT Device failure cause of an AIoT service operation. |  |
| CommandReq | 6.1.6.2.4 | Represents the AIoT Command request. |  |
| CommandResp | 6.1.6.2.5 | Represents the AIoT Command response. |  |
| DevicesRepInfo | 6.1.6.2.8 | Represents the AIoT device(s) related reporting information. |  |
| InventoryReq | 6.1.6.2.2 | Represents the AIoT Inventory request. |  |
| InventoryResp | 6.1.6.2.3 | Represents the AIoT Inventory response. |  |
| AIoTDevices | 6.1.6.2.7 | Represents the AIoT device(s) related information. |  |

Table 6.1.6.1-2 specifies data types re-used by the Naiotf_AIoT service-based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Naiotf_AIoT service-based interface.

Table 6.1.6.1-2: Naiotf_AIoT re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| AiotArea | 3GPP TS 29.571 [16] | Represents the AIoT Service Area. |  |
| AiotDevPermId | 3GPP TS 29.571 [16] | Represents the permanent identifier of the AIoT Device. |  |
| AiotFilteringInformation | 3GPP TS 29.571 [16] | Represents the the filtering information used for identifying the target AIoT device(s). |  |
| Bytes | 3GPP TS 29.571 [16] | Represents a sequence of bytes. |  |
| CommandType | 3GPP TS 29.522 [15] | Represents the type of AIoT Command. |  |
| DurationSec | 3GPP TS 29.571 [16] | Represents a time duration in units of seconds. |  |
| ProblemDetails | 3GPP TS 29.571 [16] | Represents error related information. |  |
| RedirectResponse | 3GPP TS 29.571 [16] | Contains redirection related information. |  |
| SupportedFeatures | 3GPP TS 29.571 [16] | Represents the list of supported feature(s) and is used to negotiate the applicability of optional features. |  |
| Uinteger | 3GPP TS 29.571 [16] | Represents an unsigned integer. |  |
| Uri | 3GPP TS 29.571 [16] | Represents a URI. |  |

# *** 2nd Change ***

6.1.6.2.8	Type: DevicesRepInfo

Table 6.1.6.2.8-1: Definition of type DevicesRepInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| deviceId | AiotDevPermId | M | 1 | Contains the identifier of the AIoT device to which the reporting information is related. |  |
| readCmdRep | Bytes | O | 0..1 | Contains the Read command specific report information for the AIoT device identified by the "deviceId" attribute. This attribute may be present only if the reporting information is related to a Read command operation, i.e., the "commandType" attribute is set to "READ" in the corresponding AIoT Command service operation. |  |
| failCause | AIoTDevFailCause | O | 0..1 | Contains the AIoT service operation failure cause for the AIoT Device identified by the "deviceId" attribute. |  |

# *** 3rd Change ***

6.1.6.3.3	Enumeration: AIoTDevFailCause

The enumeration AIoTDevFailCause represents the per-AIoT Device failure cause of an AIoT service operation. It shall comply with the provisions defined in table 6.1.6.3.3-1.

Table 6.1.6.3.3-1: Enumeration AIoTDevFailCause

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| COMMAND_TYPE_SPECIFIC_PARAMETERS_INVALID | Indicate that the failure cause of the AIoT service operation is due to the command type specific parameters being invalid. (NOTE) |  |
| ERROR_UNSPECIFIED | Indicate that the failure cause of the AIoT service operation is because the requested read or write command was not executed successfully. (NOTE) |  |
| LOW_ENERGY | Indicate that the failure cause of the AIoT service operation is because the write command cannot be implemented because the energy will run out. (NOTE) |  |
| INVALID_MANDATORY_INFORMATION | Indicate that the failure cause of the AIoT service operation is because the AIoT Devide sending this cause has received a message with a non-semantical mandatory IE error. (NOTE) |  |
| MESSAGE_TYPE_NON_EXISTENT_OR_NOT_IMPLEMENTED | Indicates that the failure cause of the AIoT service operation is because the AIoT Devide received a message with a message type that it does not recognize either because this is a message that is not defined, or defined but not implemented by the equipment. (NOTE) |  |
| NOTE: This value applies only to the AIoT Command service operation. |  |  |

# *** 3rd Change ***

A.2	Naiotf_AIoT API

openapi: 3.0.0

info:

title: AIOTF Ambient IoT Service

version: 1.0.0-alpha.5

description: |

API for the Naiotf_AIoT Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.569 V19.0.0; Ambient IoT Function (AIOTF) Services.

url: http://www.3gpp.org/ftp/Specs/archive/29_series/29.569/

servers:

- url: '{apiRoot}/naiotf-aiot/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501.

security:

- {}

- oAuth2ClientCredentials:

- naiotf-aiot

paths:

/request-inv:

post:

summary: Request to perform an AIoT Inventory operation.

operationId: InventoryRequest

tags:

- AIoT Inventory Request (custom operation without associated resources)

security:

- {}

- oAuth2ClientCredentials:

- naiotf-aiot

- oAuth2ClientCredentials:

- naiotf-aiot

- naiotf-aiot:inventory

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/InventoryReq'

responses:

'200':

description: >

OK. The AIoT Inventory request is successfully received and processed, and

AIoT Inventory related information is returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/InventoryResp'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

callbacks:

AIoTOperationsNotif:

'{$request.body#/notifUri}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/AIoTNotif'

responses:

'204':

description: >

No Content. The AIoT Operations Notification is successfully received and

acknowledged.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/request-cmd:

post:

summary: Request to perform an AIoT Command operation.

operationId: CommandRequest

tags:

- AIoT Command Request (custom operation without associated resources)

security:

- {}

- oAuth2ClientCredentials:

- naiotf-aiot

- oAuth2ClientCredentials:

- naiotf-aiot

- naiotf-aiot:command

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/CommandReq'

responses:

'200':

description: >

OK. The AIoT Command request is successfully received and processed, and

AIoT Command related information is returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/CommandResp'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

callbacks:

AIoTOperationsNotif:

'{$request.body#/notifUri}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/AIoTNotif'

responses:

'204':

description: >

No Content. The AIoT Operations Notification is successfully received and

acknowledged.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{nrfApiRoot}/oauth2/token'

scopes:

naiotf-aiot: >

Enables to access all the resources and custom operations of the Naiotf_AIoT API.

naiotf-aiot:inventory: >

Enables to access only the InventoryRequest custom operation (Naiotf_AIoT_Inventory

service operation) of the Naiotf_AIoT API.

naiotf-aiot:command: >

Enables to access only the CommandRequest custom operation (Naiotf_AIoT_Command

service operation) of the Naiotf_AIoT API.

schemas:

#

# STRUCTURED DATA TYPES

#

InventoryReq:

description: Represents the AIoT Inventory request.

type: object

properties:

afId:

type: string

targetArea:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AiotArea'

targetDevices:

$ref: '#/components/schemas/AIoTDevices'

numDevices:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

timeInterval:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

notifUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- afId

- notifUri

anyOf:

- required: [targetArea]

- required: [targetDevices]

InventoryResp:

description: Represents the AIoT Inventory response.

type: object

properties:

transId:

type: string

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- transId

CommandReq:

description: Represents the AIoT Command request.

type: object

properties:

afId:

type: string

commandType:

$ref: 'TS29522_AIoT.yaml#/components/schemas/CommandType'

targetArea:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AiotArea'

targetDevices:

$ref: '#/components/schemas/AIoTDevices'

numDevices:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

msgSize:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

offset:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

length:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

data:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Bytes'

notifUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- afId

- commandType

- notifUri

anyOf:

- required: [targetArea]

- required: [targetDevices]

CommandResp:

description: Represents the AIoT Command response.

type: object

properties:

transId:

type: string

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- transId

AIoTNotif:

description: Represents the AIoT Operations Notification.

type: object

properties:

transId:

type: string

devicesRepData:

type: array

items:

$ref: '#/components/schemas/DevicesRepInfo'

minItems: 1

lastRepInd:

type: boolean

enum:

- true

description: >

Indicates that this is the last reporting from the AIOTF for the AIoT service

operation identified by the "transId" attribute.

true indicates that this is the last report.

This attribute shall be present only when this is the last reporting from the NF service

consumer for the AIoT service operation identified by the transId attribute.

When present, this attribute shall be set to true. The presence of this attribute set

to the value false is forbidden.

required:

- transId

anyOf:

- required: [devicesRepData]

- required: [lastRepInd]

AIoTDevices:

description: Represents the AIoT device(s) related information.

type: object

properties:

devices:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AiotDevPermId'

minItems: 1

filteringInfo:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AiotFilteringInformation'

oneOf:

- required: [devices]

- required: [filteringInfo]

DevicesRepInfo:

description: Represents the AIoT device(s) related reporting information.

type: object

properties:

deviceId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AiotDevPermId'

readCmdRep:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Bytes'

failCause:

$ref: '#/components/schemas/AIoTDevFailCause'

required:

- deviceId

#

# SIMPLE DATA TYPES

#

#

# ENUMERATIONS

#

AIoTDevFailCause

anyOf:

- type: string

enum:

- COMMAND_TYPE_SPECIFIC_PARAMETERS_INVALID

- ERROR_UNSPECIFIED

- LOW_ENERGY

- INVALID_MANDATORY_INFORMATION

- MESSAGE_TYPE_NON_EXISTENT_OR_NOT_IMPLEMENTED

- type: string

description: >

This string provides AIoT service operation failure cause for the AIoT Device.

description: |

Possible values are:

- COMMAND_TYPE_SPECIFIC_PARAMETERS_INVALID: Indicate that the failure cause of the AIoT

service operation is due to the command type specific parameters being invalid.

- ERROR_UNSPECIFIED: Indicate that the failure cause of the AIoT service operation is

because the requested read or write command was not executed successfully.

- LOW_ENERGY: Indicate that the failure cause of the AIoT service operation is because the

write command cannot be implemented because the energy will run out.

- INVALID_MANDATORY_INFORMATION: Indicate that the failure cause of the AIoT service

operation is because the AIoT Devide sending this cause has received a message with

a non-semantical mandatory IE error.

- MESSAGE_TYPE_NON_EXISTENT_OR_NOT_IMPLEMENTED: Indicates that the failure cause of the

AIoT service operation is because the AIoT Devide received a message with a message type

that it does not recognize either because this is a message that is not defined, or

defined but not implemented by the equipment.

#

# Data types describing alternative data types or combinations of data types

#

*** End of Changes ***