3GPP TSG CT WG3 Meeting #143

# Sophia-Antipolis, France, 13 – 17 August, 2025                                           C3-254180

Source:	Samsung

Title:	Pseudo-CR on defining the OpenAPI description of the SS_SmLocalization API

Spec:	3GPP TS 29.437 (V1.1.0)

Agenda item:	19.42

Document for:	Approval

1. Introduction

The stage 2 requirements for the new SS_SmLocalization API have been defined in clauses 9.7.4 of TS 23.437. The API data model and the related descriptions of the service operations of the API are specified in TS 29.437.

2. Reason for Change

With stable API data model, the OpenAPI descriptions needs to specified in TS 29.437.

3. Conclusions

N/A

4. Proposal

It is proposed to agree the following changes to 3GPP TS 29.437 V 1.1.0.

* * * First Change * * * *

# A.7	SS_SmLocalization API

openapi: 3.0.0

info:

title: Spatial Map Localization

version: 1.0.0-alpha.1

description: |

Spatial Map Localization.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.437 V1.2.0; Service Enabler Architecture Layer for Verticals (SEAL);

Metaverse Enablement Services; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.437/

servers:

- url: '{apiRoot}/ssm-loc/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 6.5 of 3GPP TS 29.549

security:

- {}

- oAuth2ClientCredentials: []

paths:

/localization:

get:

summary: Retrieve Spatial Map localization information.

operationId: GetSpatialMapLocalization

tags:

- Spatial Localization

parameters:

- name: area-int

in: query

description: Contains the three-dimensional area or localization information.

content:

application/json:

schema:

$ref: 'TS29558_Eecs_EESRegistration.yaml#/components/schemas/ServiceArea'

- name: map-id

in: query

description: >

Contains the spatial map identifier.

content:

application/json:

schema:

$ref: 'TS29437_SS_SmManagement.yaml#/components/schemas/SpatialMapId'

- name: target-id

in: query

description: Contains the list of identity of the users or objects to localize.

content:

application/json:

schema:

$ref: '#/components/schemas/ValTargetUe'

- name: supp-feats

in: query

description: >

Contains the list of supported feature(s) among the ones defined in clause 6.1.2.8.

content:

application/json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: >

OK. The requested SM data source information shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/SpatialMapLocalizeResp'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

#

# STRUCTURED DATA TYPES

#

SpatialMapLocalizeResp:

description: Represents the spatial map localization response.

type: object

properties:

results:

type: array

items:

$ref: '#/components/schemas/SpatialMapLocDetails'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- results

SpatialMapLocDetails:

description: Represents the localization details of the spatial map.

type: object

properties:

targetId:

$ref: 'TS29549_SS_UserProfileRetrieval.yaml#/components/schemas/ValTargetUe'

position:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/RelativeCartesianLocation'

pose:

type: string

# SIMPLE DATA TYPES

#

#

# ENUMERATIONS

#

#

# Data types describing alternative data types or combinations of data types:

#

* * * * End of Changes * * * *