3GPP TSG-CT WG3 Meeting #143	C3-254128

# Sophia Antipolis, France, 13-17 October 2025				is revision of C3-254091,4128

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.165 | CR | 1045 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduce DC-Info to indicate a DC operation request is initiated by the DC AS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2025-10-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | CR 1635 of 3GPP TS 23.228 and CR 6741 of 3GPP TS 24.229 have agreed to add some additional information to indicate the DC operation request initiator is DC AS in existing IMS session update case. The Inter-IMS Network to Network Interface need to be updated accordingly. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add DC-Info to indicate the DC operation request initiator is DC AS |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Uncomplete definition. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.1.3.1, 6.1.3, 33.3(new), A, B.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR does not have any impact in the OpenAPI specification. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Additional discussion(if needed):

# Proposed changes:

# *** 1st Change ***

6.1.1.3.1	Trust and no trust relationship

The IBCF acting as exit point applies the procedures described in clause 5.10.2 of 3GPP TS 24.229 [5] before forwarding the SIP signalling to the IBCF acting as entry point. The IBCF acting as entry point applies the procedures described in clause 5.10.3 of 3GPP TS 24.229 [5].

Additionally, in case there is no trust relationship between the two IM CN subsystems connected by II-NNI, the IBCF applies the procedures described in clause 4.4 of 3GPP TS 24.229 [5], before forwarding the SIP signalling.

These procedures may be utilized on a per header field basis to realize overall trust as well as per service level screening of header fields. Trust relationships and trust domains may be defined by inter-operator agreements for individual services and/or individual SIP header fields.

The management of the SIP header fields (if present) over II-NNI in case of a presence or not of a trust relationship between the two interconnected IM CN subsystems is wrapped up in the following table.

Table 6.2: Management of SIP header fields and parameters over II-NNI in presence or not of a trust relationship

| Item | Header field or parameter | Reference | Trust relationship | Not trust relationship |
| --- | --- | --- | --- | --- |
| 1 | P-Asserted-Identity | IETF RFC 3325 [44] | As specified in 3GPP TS 24.229 [5], clause 4.4 | As specified in 3GPP TS 24.229 [5], clause 4.4 |
| 2 | P-Access-Network-Info | IETF RFC 7315 [24] | As specified in 3GPP TS 24.229 [5], clause 4.4 | As specified in 3GPP TS 24.229 [5], clause 4.4 |
| 3 | Resource-Priority | IETF RFC 4412 [78] | As specified in 3GPP TS 24.229 [5], clause 4.4 | As specified in 3GPP TS 24.229 [5], clause 4.4 |
| 4 | History-Info | IETF RFC 7044 [25] | As specified in 3GPP TS 24.229 [5], clause 4.4 | As specified in clause 7 of IETF RFC 7044 [25] and in 3GPP TS 24.229 [5], clause 4.4 |
| 5 | P-Asserted-Service | IETF RFC 6050 [26] | As specified in 3GPP TS 24.229 [5], clause 4.4 (NOTE 3) | As specified in 3GPP TS 24.229 [5], clause 4.4 (NOTE 3) |
| 6 | P-Charging-Vector | IETF RFC 7315 [24] | As specified in 3GPP TS 24.229 [5], clause 5.10 | As specified in 3GPP TS 24.229 [5], clause 5.10 |
| 7 | P-Charging-Function-Addresses (NOTE 4) | IETF RFC 7315 [24] | As specified in 3GPP TS 24.229 [5], clause 5.10 | As specified in 3GPP TS 24.229 [5], clause 5.10 |
| 8 | P-Profile-Key (NOTE 2) | IETF RFC 5002 [64] | As specified in 3GPP TS 24.229 [5], clause 4.4 | As specified in 3GPP TS 24.229 [5], clause 4.4 |
| 9 | P-Private-Network-Indication | IETF RFC 7316 [84] | As specified in 3GPP TS 24.229 [5], clause 4.4 | As specified in 3GPP TS 24.229 [5], clause 4.4 |
| 10 | P-Served-User (NOTE 1, NOTE 2) | IETF RFC 5502 [85] | As specified in 3GPP TS 24.229 [5], clause 4.4 | As specified in 3GPP TS 24.229 [5], clause 4.4 |
| 11 | Reason (in a response) | IETF RFC 6432 [49] | As specified in 3GPP TS 24.229 [5], clause 4.4 | As specified in 3GPP TS 24.229 [5], clause 4.4 |
| 12 | P-Early-Media | IETF RFC 5009 [74] | As specified in 3GPP TS 24.229 [5], clause 4.4 | As specified in 3GPP TS 24.229 [5], clause 4.4 |
| 13 | Feature-Caps | IETF RFC 6809 [143] | As specified in 3GPP TS 24.229 [5], clause 4.4 | As specified in 3GPP TS 24.229 [5], clause 4.4 |
| 14 | Priority (NOTE 6) | IETF RFC 7090 [184] | As specified in 3GPP TS 24.229 [5], clause 4.4 | As specified in 3GPP TS 24.229 [5], clause 4.4 |
| 15 | "iotl" SIP URI parameter (NOTE 7) | IETF RFC 7549 [188] | As specified in 3GPP TS 24.229 [5], clause 4.4 | As specified in 3GPP TS 24.229 [5], clause 4.4 |
| 16 | "cpc" tel URI parameter (NOTE 5) | 3GPP TS 24.229 [5] clause 7.2A.12 | As specified in 3GPP TS 24.229 [5], clause 4.4 | As specified in 3GPP TS 24.229 [5], clause 4.4 |
| 17 | "oli" tel URI parameter (NOTE 5) | 3GPP TS 24.229 [5] clause 7.2A.12 | As specified in 3GPP TS 24.229 [5], clause 4.4 | As specified in 3GPP TS 24.229 [5], clause 4.4 |
| 18 | Restoration-Info (NOTE 2) | 3GPP TS 24.229 [5] clause 7.2.11 | As specified in 3GPP TS 24.229 [5], clause 4.4 | As specified in 3GPP TS 24.229 [5], clause 4.4 |
| 19 | Relayed-Charge (NOTE 4) | 3GPP TS 24.229 [5] clause 7.2.12 | As specified in 3GPP TS 24.229 [5], clause 4.4 | As specified in 3GPP TS 24.229 [5], clause 4.4 |
| 20 | Service-Interact-Info | 3GPP TS 24.229 [5] clause 7.2.14 | As specified in 3GPP TS 24.229 [5], clause 4.4 | As specified in 3GPP TS 24.229 [5], clause 4.4 |
| 21 | Cellular-Network-Info | 3GPP TS 24.229 [5] clause 7.2.15 | As specified in 3GPP TS 24.229 [5], clause 4.4 | As specified in 3GPP TS 24.229 [5], clause 4.4 |
| 22 | Response-Source | 3GPP TS 24.229 [5] clause 7.2.17 | As specified in 3GPP TS 24.229 [5], clause 4.4 | As specified in 3GPP TS 24.229 [5], clause 4.4 |
| 23 | Attestation-Info (NOTE 8) | 3GPP TS 24.229 [5] clause 7.2.18 | As specified in 3GPP TS 24.229 [5], clause 4.4 | As specified in 3GPP TS 24.229 [5], clause 4.4 |
| 24 | Origination-Id (NOTE 8) | 3GPP TS 24.229 [5] clause 7.2.19 | As specified in 3GPP TS 24.229 [5], clause 4.4 | As specified in 3GPP TS 24.229 [5], clause 4.4 |
| 25 | Additional-Identity | 3GPP TS 24.229 [5] clause 7.2.20 | As specified in 3GPP TS 24.229 [5], clause 4.4 | As specified in 3GPP TS 24.229 [5], clause 4.4 |
| 26 | Priority-Verstat (NOTE 8) | 3GPP TS 24.229 [5] clause 7.2.21 | As specified in 3GPP TS 24.229 [5], clause 4.4 | As specified in 3GPP TS 24.229 [5], clause 4.4 |
| 27 | DC-Info | 3GPP TS 24.229 [5] clause 7.2.23 | As specified in 3GPP TS 24.229 [5], clause 4.4 | As specified in 3GPP TS 24.229 [5], clause 4.4 |
| NOTE 1: For a roaming II-NNI, a trust relationship with respect to this header field is required. NOTE 2: This header field is only applicable on a roaming II-NNI. NOTE 3: In addition, value-dependent operator policies may be applied. NOTE 4: This header field is not applicable at II-NNI. NOTE 5: The tel URI parameters "cpc" and "oli" can be included in the URI in the P-Asserted-Identity header field. NOTE 6: Only the "psap-callback" value is part of the trust domain. NOTE 7: The "iotl" SIP URI parameter can be transported in the Request-URI, Route header field, Path header field, Service-Route header field, "+g.3gpp.trf" header field parameter, "+g.3gpp.atcf-mgmt-uri" header field parameter and in the "ATU-STI" parameter in the "application/vnd.3gpp.srvcc-info+xml" MIME body. NOTE 8: This header field is only applicable on non-roaming II-NNI. |  |  |  |  |

# *** Next Change ***

6.1.3	Major capabilities

This clause contains the major capabilities to be supported over the II-NNI.

The table 6.1.3.1 specifies which capabilities are applicable for II-NNI. The profile status codes within table 6.1.3.1 are defined in table 6.1.3.2.

For the "Basic SIP" capabilities part of table 6.1.3.1, the last column "Profile status over II-NNI" specifies the general status of applicability of the IETF RFC 3261 [13] main mechanisms described in the 2nd column "Capability over the Ici".

For the "Extensions to basic SIP" capabilities part, the last column "Profile status over II-NNI" specifies the general status of applicability of the RFC referenced in the 2nd column "Capability over the Ici".

If necessary, the applicability of RFCs at the II-NNI level is further detailed in the present Technical Specification.

The columns "Reference item in 3GPP TS 24.229 [5] for the profile status" provide informative references for comparison purposes into the UA and Proxy role major capabilities tables in 3GPP TS 24.229 [5], where the capabilities are defined via additional references.

Table 6.1.3.1: Major capabilities over II-NNI

| Item |  | Capability over the Ici | Reference item in 3GPP TS 24.229 [5] for the profile status |  | Profile status over II-NNI |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | UA Role (NOTE 1) | Proxy role (NOTE 2) |  |  |
|  | Basic SIP (IETF RFC 3261 [13]) |  |  |  |  |
| 1 | registrations | 1, 2, 2A | - |  | c2 |
| 2 | initiating a session | 2B, 3, 4 | - |  | m |
| 3 | terminating a session | 5 | 3 |  | m |
| 4 | General proxy behaviour | - | 4, 5, 14, 15 |  | n/a |
| 5 | Managing several responses due to forking | 9,10 | 6 |  | m |
| 6 | support of indication of TLS connections in the Record-Route header | - | 7, 8 |  | n/a |
| 7 | Support of authentication | 7, 8, 8A | 8A |  | c2 |
| 8 | Timestamped requests (Timestamp header field) | 6 | - |  | m |
| 9 | Presence of date in requests and responses (Date header field) | 11 | 9 |  | m |
| 10 | Presence of alerting information data (Alert-info header field) | 12 | 10 |  | o |
| 11 | Support and handling of the Require header field for REGISTER and other requests or responses for methods other than REGISTER | - | 11, 12, 13 |  | m |
| 12 | Support and reading of the Supported and Unsupported header fields | - | 16, 17, 18 |  | m |
| 13 | Support of the Error-Info header field in 3xx - 6xx responses | - | 19 |  | o |
| 14 | Support and handling of the Organization header field | - | 19A, 19B |  | m |
| 15 | Support and handling of the Call-Info header field | - | 19C, 19D |  | m |
| 16 | Support of the Contact header field in 3xx response | - | 19E |  | m |
| 16A | Proxy reading the contents of a body or including a body in a request or response | - | 19F |  | n/a |
|  | Extensions to basic SIP |  |  |  |  |
| 16B | 3GPP TS 24.237 [131]: proxy modifying the content of a body | - | 19G |  | n/a |
| 17 | IETF RFC 6086 [39]: SIP INFO method and package framework | 13 | 20 |  | o |
| 17A | IETF RFC 6086 [39]: legacy INFO usage | 13A | 20A |  | o |
| 18 | IETF RFC 3262 [18]: reliability of provisional responses in SIP (PRACK method) | 14 | 21 |  | m |
| 19 | IETF RFC 3515 [22]: the SIP REFER method | 15 | 22 |  | o |
| 19A | IETF RFC 7647 [194]: Clarifications for the Use of REFER with RFC6665 | 15A | 22A |  | n/a |
| 19B | IETF RFC 7614 [195]: Explicit Subscriptions for the REFER Method | 15B | 22B |  | o |
| 20 | IETF RFC 3312 [40] and IETF RFC 4032 [41]: integration of resource management and SIP (Preconditions framework) | 2C, 16 | 23 |  | o |
| 21 | IETF RFC 3311 [23]: the SIP UPDATE method | 17 | 24 |  | m |
| 22 | IETF RFC 3313 [42]: SIP extensions for media authorization (P-Media-Authorization header field) | 19 | 26 |  | n/a |
| 23 | IETF RFC 6665 [20]: SIP specific event notification (SUBSCRIBE/NOTIFY methods) | 20, 22, 23 | 27 |  | c1 |
| 23A | IETF RFC 7621 [196]: A Clarification on the Use of Globally Routable User Agent URIs (GRUUs) in the Session Initiation Protocol SIP Event Notification Framework | 22A | 28 |  | n/a |
| 24 | IETF RFC 3327 [43]: session initiation protocol extension header field for registering non-adjacent contacts (Path header field) | 24 | 29 |  | c2 |
| 25 | IETF RFC 3325 [44]: private extensions to the Session Initiation Protocol (SIP) for network asserted identity within trusted networks | 25 | 30 |  | c4 |
| 26 | IETF RFC 3325 [44]: the P-Preferred-Identity header field extension | - | - |  | n/a |
| 27 | IETF RFC 3325 [44]: the P-Asserted-Identity header field extension | - | - |  | c4 |
| 28 | IETF RFC 3323 [34], IETF RFC 3325 [44] and IETF RFC 7044 [25]: a privacy mechanism for the Session Initiation Protocol (SIP) (Privacy header field) | 26, 26A, 26B, 26C, 26D, 26E, 26F, 26G, 26H | 31, 31A, 31B, 31C, 31D, 31E, 31F, 31G, 31H |  | m |
| 29 | IETF RFC 3428 [19]: a messaging mechanism for the Session Initiation Protocol (SIP) (MESSAGE method) | 27 | 33 |  | o |
| 30 | IETF RFC 3608 [45]: session initiation protocol extension header field for service route discovery during registration (Service-Route header field) | 28 | 32 |  | c2 |
| 31 | IETF RFC 3486 [46]: compressing the session initiation protocol | 29 | 34 |  | n/a |
| 32 | IETF RFC 7315 [24]: private header extensions to the session initiation protocol for the 3rd-Generation Partnership Project (3GPP) | 30 | 35 |  | o |
| 32A | IETF RFC 3325 [44]: act as first entity within the trust domain for asserted identity | 30A | 30A |  | n/a |
| 32B | IETF RFC 3325 [44]: act as entity within trust network that can route outside the trust network | 30B | 30B |  | n/a |
| 32C | IETF RFC 3325 [44]: act as entity passing on identity transparently independent of trust domain | 30C | 30C |  | n/a |
| 33 | IETF RFC 7315 [24] and IETF RFC 7976 [24A]: the P-Associated-URI header field extension | 31 | 36 |  | c2 |
| 34 | IETF RFC 7315 [24] and IETF RFC 7976 [24A]: the P-Called-Party-ID header field extension | 32 | 37 |  | c2 |
| 35 | IETF RFC 7315 [24] and IETF RFC 7976 [24A]: the P-Visited-Network-ID header field extension | 33 | 38, 39 |  | c2 |
| 36 | IETF RFC 7315 [24], IETF RFC 7976 [24A] and IETF RFC 7913 [24B]: the P-Access-Network-Info header field extension | 34 | 41, 42, 43 |  | c4 |
| 37 | IETF RFC 7315 [24] and IETF RFC 7976 [24A]: the P-Charging-Function-Addresses header field extension | 35 | 44, 44A |  | n/a |
| 38 | IETF RFC 7315 [24] and IETF RFC 7976 [24A]: the P-Charging-Vector header field extension | 36 | 45, 46 |  | c1 |
| 39 | IETF RFC 3329 [47]: security mechanism agreement for the session initiation protocol | 37 | 47 |  | n/a |
| 39A | 3GPP TS 24.229 [5] clause 7.2A.7: Capability Exchange for Media Plane Security | 37A | 47A |  | n/a |
| 40 | IETF RFC 3326 [48]: the Reason header field for the session initiation protocol | 38 | 48 |  | o |
| 41 | IETF RFC 6432 [49]: carrying Q.850 codes in reason header fields in SIP (Session Initiation Protocol) responses | 38A | 48A |  | c4 |
| 41A | IETF RFC 8606 [214]: the Location Parameter for the SIP Reason Header Field | 38B | 48B |  | o |
| 41B | IETF RFC 9410 [220]: Handling of Identity Header Errors for Secure Telephone Identity Revisited (STIR) (carrying STIR codes in Reason header fields in SIP responses) | 38C | 48C |  | c5 |
| 41C | IETF RFC 9366 [221]: Multiple SIP Reason Header Field Values | 38D | 48D |  | c5 |
| 42 | IETF RFC 3581 [50]: an extension to the session initiation protocol for symmetric response routeing | 39 | 49 |  | o |
| 43 | IETF RFC 3841 [51]: caller preferences for the session initiation protocol (Accept-Contact, Reject-Contact and Request-Disposition header fields) | 40, 40A, 40B, 40C, 40D, 40E, 40F | 50, 50A, 50B, 50C, 50D, 50E, 50F |  | m |
| 44 | IETF RFC 3903 [21]: an event state publication extension to the session initiation protocol (PUBLISH method) | 41 | 51 |  | c1 |
| 45 | IETF RFC 4028 [52]: SIP session timer (Session-Expires and Min-SE headers) | 42 | 52 |  | m |
| 46 | IETF RFC 3892 [53]: the SIP Referred-By mechanism | 43 | 53 |  | m |
| 47 | IETF RFC 3891 [54]: the Session Initiation Protocol (SIP) "Replaces" header | 44 | 54 |  | o |
| 48 | IETF RFC 3911 [55]: the Session Initiation Protocol (SIP) "Join" header | 45 | 55 |  | o |
| 49 | IETF RFC 3840 [56]: the callee capabilities | 46 | 56 |  | o |
| 50 | IETF RFC 7044 [25]: an extension to the session initiation protocol for request history information (History-Info header field) | 47 | 57 |  | o |
| 50A | IETF RFC 7044 [25]: the "mp" header field parameter | 47A | 57A |  | o |
| 50B | IETF RFC 7044 [25]: the "rc" header field parameter | 47B | 57B |  | o |
| 50C | IETF RFC 7044 [25]: the "np" header field parameter | 47C | 57C |  | o |
| 51 | IETF RFC 5079 [57]: Rejecting anonymous requests in the session initiation protocol | 48 | 58 |  | o |
| 52 | IETF RFC 4458 [58]: session initiation protocol URIs for applications such as voicemail and interactive voice response (NOTE 3) | 49 | 59 |  | o |
| 52A | IETF RFC 8119 [193]: Session Initiation Protocol (SIP) Cause URI parameter for Service Number translation | 49A | 59A |  | o |
| 53 | IETF RFC 4320 [59]: Session Initiation Protocol's (SIP) non-INVITE transactions | 50 | 61 |  | m |
| 54 | IETF RFC 4457 [60]: the P-User-Database private header field extension | 51 | 60 |  | n/a |
| 55 | IETF RFC 5031 [61]: A Uniform Resource Name (URN) for Emergency and Other Well-Known Services | 52 | 62 |  | c7 |
| 56 | IETF RFC 5627 [62]: obtaining and using GRUUs in the Session Initiation Protocol (SIP) | 53 | 63 |  | c1 |
| 57 | Void |  |  |  |  |
| 58 | IETF RFC 4168 [27]: the Stream Control Transmission Protocol (SCTP) as a Transport for the Session Initiation Protocol (SIP) | 55 | 65 |  | o |
| 59 | IETF RFC 5002 [64]: the SIP P-Profile-Key private header field extension | 56 | 66, 66A, 66B |  | c3 |
| 60 | IETF RFC 5626 [65]: managing client initiated connections in SIP | 57 | 67 |  | c1 |
| 61 | IETF RFC 5768 [66]: indicating support for interactive connectivity establishment in SIP | 58 | 68 |  | n/a |
| 62 | IETF RFC 5365 [67]: multiple-recipient MESSAGE requests in the session initiation protocol | 59 | 69 |  | o if 29, else n/a |
| 63 | IETF RFC 6442 [68]: Location conveyance for the Session Initiation Protocol | 60 | 70, 70A, 70B |  | m |
| 64 | IETF RFC 5368 [69]: referring to multiple resources in the session initiation protocol | 61 | 71 |  | o if 19, else n/a |
| 65 | IETF RFC 5366 [70]: conference establishment using request-contained lists in the session initiation protocol | 62 | 72 |  | o |
| 66 | IETF RFC 5367 [71]: subscriptions to request-contained resource lists in the session initiation protocol | 63 | 73 |  | o if 23, else n/a |
| 67 | IETF RFC 4967 [72]: dialstring parameter for the session initiation protocol uniform resource identifier | 64 | 74 |  | c2 |
| 68 | IETF RFC 4964 [73]: the P-Answer-State header extension to the session initiation protocol for the open mobile alliance push to talk over cellular | 65 | 75 |  | o |
| 69 | IETF RFC 5009 [74]: the SIP P-Early-Media private header field extension for authorization of early media | 66 | 76 |  | c4 |
| 70 | IETF RFC 4694 [75]: number portability parameters for the 'tel' URI | 67, 67A, 67B | 77, 77A, 77B |  | o |
| 71 | Void |  |  |  |  |
| 72 | IETF RFC 4411 [77]: extending the session initiation protocol Reason header for preemption events | 69 | 79 |  | o |
| 73 | IETF RFC 4412 [78]: communications resource priority for the session initiation protocol (Resource-Priority header field) | 70, 70A, 70B | 80, 80A, 80B |  | o |
| 74 | IETF RFC 5393 [79]: addressing an amplification vulnerability in session initiation protocol forking proxies | 71 | 81 |  | m |
| 75 | IETF RFC 5049 [80]: the remote application identification of applying signalling compression to SIP | 72 | 82 |  | n/a |
| 76 | IETF RFC 5688 [81]: a session initiation protocol media feature tag for MIME application sub-types | 73 | 83 |  | c1 |
| 77 | IETF RFC 6050 [26]: Identification of communication services in the session initiation protocol | 74 | 84, 84A |  | o |
| 78 | IETF RFC 5360 [82]: a framework for consent-based communications in SIP | 75, 75A, 75B | 85 |  | o |
| 79 | IETF RFC 7433 [83]: a mechanism for transporting user-to-user call control information in SIP | 76 | 86 |  | c1 |
| 79A | IETF RFC 7434 [83A]: interworking ISDN call control user information with SIP | 76A | - |  | c1 |
| 80 | IETF RFC 7316 [84]: The SIP P-Private-Network-Indication private header (P-Header) | 77 | 87 |  | c1 |
| 81 | IETF RFC 5502 [85]: the SIP P-Served-User private header | 78 | 88 |  | c2 |
| 82 | IETF 8498 [203]: the SIP P-Served-User header extension for Originating CDIV session case | 79 | 89 |  | n/a |
| 83 | IETF RFC 8497 [87]: marking SIP messages to be logged | 80 | 90 |  | o |
| 84 | IETF RFC 6228 [88]: the 199 (Early Dialog Terminated) response code | 81 | 91 |  | m |
| 85 | IETF RFC 5621 [89]: message body handling in SIP | 82 | 92 |  | m |
| 86 | IETF RFC 6223 [90]: indication of support for keep-alive | 83 | 93 |  | o |
| 87 | IETF RFC 5552 [91]: SIP Interface to VoiceXML Media Services | 84 | 94 |  | n/a |
| 88 | IETF RFC 3862 [92]: common presence and instant messaging (CPIM): message format | 85 | 95 |  | o |
| 89 | IETF RFC 5438 [93]: instant message disposition notification | 86 | 96 |  | o |
| 90 | IETF RFC 5373 [94]: requesting answering modes for SIP (Answer-Mode and Priv-Answer-Mode header fields) | 87 | 97, 97A |  | o |
| 91 | Void |  |  |  |  |
| 92 | IETF RFC 3959 [96]: the early session disposition type for SIP | 89 | 99 |  | o |
| 93 | Void |  |  |  |  |
| 94 | IETF RFC 7989 [124]: End-to-End Session Identification in IP-Based Multimedia Communication Networks | 91 | 101 |  | o |
| 95 | IETF RFC 6026 [125]: correct transaction handling for 200 responses to Session Initiation Protocol INVITE requests | 92 | 102 |  | m |
| 96 | IETF RFC 5658 [126]: addressing Record-Route issues in the Session Initiation Protocol (SIP) | 93 | 103 |  | o |
| 97 | IETF RFC 5954 [127]: essential correction for IPv6 ABNF and URI comparison in IETF RFC 3261 [13] | 94 | 104 |  | m |
| 98 | IETF RFC 4488 [135]: suppression of session initiation protocol REFER method implicit subscription | 95 | 105 |  | m if 19, else n/a |
| 99 | IETF RFC 7462 [136]: Alert-Info URNs for the Session Initiation Protocol | 96 | 106 |  | o |
| 100 | 3GPP TS 24.229 [5] clause 3.1: multiple registrations | 97 | 107 |  | c2 |
| 101 | IETF RFC 5318 [141]: the SIP P-Refused-URI-List private-header | 98 | 108 |  | c5 |
| 102 | IETF RFC 4538 [140]: request authorization through dialog Identification in the session initiation protocol (Target-Dialog header field) | 99 | 109 |  | o |
| 103 | IETF RFC 6809 [143]: Mechanism to indicate support of features and capabilities in the Session Initiation Protocol (SIP) | 100 | 110 |  | o |
| 104 | IETF RFC 6140 [160]: registration of bulk number contacts | 101 | 111 |  | c3 |
| 105 | IETF RFC 6230 [161]: media control channel framework | 102 | 112 |  | o |
| 105A | 3GPP TS 24.229 [5] clause 4.14: S-CSCF restoration procedures | 103 | 113 |  | c3 |
| 106 | IETF RFC 6357 [164]: SIP overload control | 104 | 114 |  | o |
| 107 | IETF RFC 7339 [165]: feedback control | 104A | 114A |  | o |
| 108 | IETF RFC 7200 [167]: distribution of load filters | 104B | 114B |  | o |
| 109 | 3GPP TS 24.229 [5] clauses 5.1.2A.1.1, 5.1.3.1, 5.1.6.8, and 5.2.10: Handling of a 380 (Alternative service) response | 105 | 115 |  | n/a |
| 110 | IETF RFC 7090 [184]: Public Safety Answering Point (PSAP) Callback | 107 | 117 |  | o |
| 111 | IETF RFC 8055 [185]: Via header field parameter to indicate received realm | 106 | 116 |  | n/a |
| 112 | IETF RFC 7549 [188]: SIP URI parameter to indicate traffic leg | 108 | 118 |  | o (NOTE 4) |
| 113 | 3GPP TS 24.229 [5] clause 4.14: PCF or PCRF based P-CSCF restoration | 109 | 119 |  | c3 |
| 114 | 3GPP TS 24.229 [5] clause 4.14: UDM/HSS or HSS based P-CSCF restoration | 110 | 120 |  | c3 |
| 115 | 3GPP TS 24.229 [5] clause 7.2.12: the Relayed-Charge header extension | 111 | 121 |  | n/a |
| 116 | 3GPP TS 24.229 [5]: resource sharing | 112 | 122 |  | c3 |
| 117 | 3GPP TS 24.229 [5] clause 7.2.15: the Cellular-Network-Info header extension | 113 | 123 |  | c4 |
| 118 | 3GPP TS 24.229 [5] clause 7.2.16: the Priority-Share header field | 114 | 124 |  | c3 |
| 119 | IETF RFC 8224 [206]: Authenticated Identity Management in the Session Initiation Protocol (SIP) | 116 | 126 |  | c5 |
| 120 | IETF RFC 8197 [207]: A SIP Response Code for Unwanted Calls | 117 | 127 |  | o |
| 121 | 3GPP TS 24.229 [5] clause 7.2.17: the Response-Source header extension | 115 | 125 |  | c6 |
| 121A | 3GPP TS 24.229 [5]: the 3GPP PS data off extension | 118 | - |  | c3 |
| 121B | 3GPP TS 24.229 [5]: Next-Generation Pan-European eCall emergency service | 120 | - |  | c8 |
| 122 | IETF RFC 8262 [216]: Content-ID Header Field in the Session Initiation Protocol (SIP) | 119 | - |  | o |
| 123 | 3GPP TS 24.229 [5] clause 7.2.18: the Attestation-Info header field | 121 | 128 |  | c5 |
| 124 | 3GPP TS 24.229 [5] clause 7.2.19: the Origination-Id | 122 | 129 |  | c5 |
| 125 | 3GPP TS 24.229 [5] clause 4.18: Dynamic services interactions | 123 | 130 |  | c6 |
| 126 | 3GPP TS 24.229 [5] clause 7.2.20: the Additional-Identity | 124 | 131 |  | c6 |
| 127 | 3GPP TS 24.229 [5] clause 4.19: RLOS | 125 | 132 |  | c3 |
| 128 | 3GPP TS 24.229 [5] clause 7.2.21: the Priority-Verstat header field | 126 | 133 |  | c6 |
| 129 | IETF RFC 9796 [223]: SIP Call-Info Parameters for Rich Call Data | 128 | 134 |  | c5 |
| 130 | 3GPP TS 24.229 [5] clause 7.2.23: the DC-Info header field | 129 | 135 |  | c6 |
| c1: m in case of roaming II-NNI, else o c2: m in case of roaming II-NNI, else n/a c3: o in case of roaming II-NNI, else n/a c4: m in case of trust relationship between the interconnected networks, else n/a c5: o in case of non-roaming II-NNI and loopback traversal scenario, else n/a c6: o in case of trust relationship between the interconnected networks, else n/a c7: m in case of IMS emergency session traversal scenario on non-roaming II-NNI, else n/a c8: o in case of IMS emergency session traversal scenario on non-roaming II-NNI, else n/a |  |  |  |  |  |
| NOTE 1: The item numbering corresponds to the one provided in table A.4 in 3GPP TS 24.229 [5]. NOTE 2: The item numbering corresponds to the one provided in table A.162 in 3GPP TS 24.229 [5]. NOTE 3: A common URI namespace is required to apply this feature on the II-NNI. NOTE 4: For the roaming II-NNI the support of this major capability is recommended. |  |  |  |  |  |

Table 6.1.3.2: Key to notation codes for major capabilities

| Notation code | Notation name | Explanation |
| --- | --- | --- |
| m | mandatory | The capability shall be supported at II-NNI. SIP message relating to this capability shall be sent over the II-NNI if received from the serving network, unless they also make use of other unsupported capabilities. SIP headers or other information elements relating to this capability shall be passed over the II-NNI if received from the sending side. This does not imply that network elements inside the serving network or served network or user equipment connected to these networks shall support this capability. |
| o | optional | The capability may or may not be supported at II-NNI. The support of the capability is provided based on bilateral agreement between the operators. |
| n/a | not applicable | It is impossible to use/support the capability at the II-NNI. |
| c <integer> | conditional | The support of the capability ("m", "o" or "n/a") depends on the support of other optional or conditional items. <integer> is the identifier of the conditional expression. |

# *** Next Change ***

33.3	Support of data channel info

If the data channel info is supported, the procedures specified in 3GPP TS 24.186 [222] clause 9.3.2.2.1A, 9.3.2.2.2.4 and 9.3.2.2.2.6 shall be applied.

The initial INVITE request and re-INVITE including the DC-Info header field (defined in 3GPP TS 24.229 [5]) shall be supported at the II-NNI.

Editor's Note:	The initial BYE request including the DC-Info header field is FFS due to uncomplete definition of 3GPP TS 24.229.

# *** Next Change ***

Annex A (informative):
Summary of SIP header fields

A summary of the SIP header fields to be used in case of interconnection by using II-NNI is proposed in table A.1.

The starting point is the sending behaviour described for proxy and UA roles in annex A of 3GPP TS 24.229 [5]:

-	In case of misalignment between table A.1 and the behaviour described in 3GPP TS 24.229 [5], the behaviour in 3GPP TS 24.229 [5] has the precedence.

-	In case a header field is not described in table A.1 and it is described in 3GPP TS 24.229 [5], the description in 3GPP TS 24.229 [5] is applicable over II-NNI.

-	If a header field is not described in 3GPP TS 24.229 [5], the description in table A.1 is applicable over II-NNI.

The definition of the notation codes used in table A.1 is provided in table A.2.

Table A.1: Supported header fields

| Item |  | Header field |  | Ref. |  | II-NNI |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  | Accept |  | [5] |  | m |  |
| 2 |  | Accept-Contact |  | [5] |  | m |  |
| 3 |  | Accept-Encoding |  | [5] |  | m |  |
| 4 |  | Accept-Language |  | [5] |  | m |  |
| 4a |  | Accept-Resource-Priority |  | [5] |  | o |  |
| 4b |  | Additional-Identity |  | [5], clause 6.1.1.3.1 (table 6.2, item 25) and clause 12.26.2 |  | o in case of a trust relationship between the interconnected networks, else n/a |  |
| 5 |  | Alert-Info |  | [5] |  | o |  |
| 6 |  | Allow |  | [5] |  | m |  |
| 7 |  | Allow-Events |  | [5] |  | m on roaming II-NNI, else o |  |
| 7a |  | Attestation-Info |  | [5], clause 6.1.1.3.1 (table 6.2, item 23) and clause 29 |  | o on non-roaming II-NNI, else n/a |  |
| 8 |  | Authentication-Info |  | [5] |  | m on roaming II-NNI, else n/a |  |
| 9 |  | Authorization |  | [5] |  | m on roaming II-NNI, else n/a |  |
| 9a |  | Answer-Mode |  | [5] |  | o |  |
| 10 |  | Call-ID |  | [5] |  | m |  |
| 11 |  | Call-Info |  | [5] |  | m |  |
| 11a |  | Cellular-Network-Info |  | clause 6.1.1.3.1 (table 6.2, item 21) |  | o |  |
| 12 |  | Contact |  | [5] |  | m |  |
| 13 |  | Content-Disposition |  | [5] |  | m |  |
| 14 |  | Content-Encoding |  | [5] |  | m |  |
| 14a |  | Content-ID |  | [5] |  | o |  |
| 15 |  | Content-Language |  | [5] |  | m |  |
| 16 |  | Content-Length |  | [5] |  | m |  |
| 17 |  | Content-Type |  | [5] |  | m |  |
| 18 |  | CSeq |  | [5] |  | m |  |
| 19 |  | Date |  | [5] |  | m |  |
| 19a |  | DC-Info |  | [5] |  | o in case of a trust relationship between the interconnected networks, else n/a |  |
| 20 |  | Error-Info |  | [5] |  | o |  |
| 21 |  | Expires |  | [5] |  | m |  |
| 21a |  | Flow-Timer |  | [5] |  | m on roaming II-NNI, else o |  |
| 21b |  | Feature-Caps |  | clause 6.1.1.3.1 (table 6.2, item 13) |  | o |  |
| 22 |  | Event |  | [5] |  | m |  |
| 23 |  | From |  | [5] |  | m |  |
| 24 |  | Geolocation |  | [5] |  | m |  |
| 24a |  | Geolocation-Error |  | [5] |  | m |  |
| 24b |  | Geolocation-Routing |  | [5] |  | m |  |
| 25 |  | History-Info |  | clause 6.1.1.3.1 (table 6.2, item 4) |  | o |  |
| 25b |  | Identity |  | [206], clause 29 and clause 34 |  | o on non-roaming II-NNI, else n/a |  |
| 25a |  | Info-Package |  | [5] |  | o |  |
| 26 |  | In-Reply-To |  | [5] |  | o |  |
| 27 |  | Join |  | [5] |  | o |  |
| 27a |  | Max-Breadth |  | [5] |  | m |  |
| 28 |  | Max-Forwards |  | [5] |  | m |  |
| 29 |  | Min-Expires |  | [5] |  | m |  |
| 30 |  | MIME-Version |  | [5] |  | m |  |
| 31 |  | Min-SE |  | [5] |  | m |  |
| 32 |  | Organization |  | [5] |  | m |  |
| 32a |  | Origination-Id |  | [5], clause 6.1.1.3.1 (table 6.2, item 24) and clause 29 |  | o on non-roaming II-NNI, else n/a |  |
| 33 |  | P-Access-Network-Info |  | clause 6.1.1.3.1 (table 6.2, item 2) |  | m in case of a trust relationship between the interconnected networks, else n/a |  |
| 33a |  | P-Answer-state |  | [5] |  | o |  |
| 34 |  | P-Asserted-Identity |  | clause 6.1.1.3.1 (table 6.2, item 1) |  | m in case of a trust relationship between the interconnected networks, else n/a |  |
| 35 |  | P-Asserted-Service |  | clause 6.1.1.3.1 (table 6.2, item 5) |  | o |  |
| 35a |  | P-Associated-URI |  | [5] |  | m on roaming II-NNI, else n/a |  |
| 36 |  | P-Called-Party-ID |  | [5] |  | m on roaming II-NNI, else n/a |  |
| 37 |  | P-Charging-Function-Addresses |  | clause 6.1.1.3.1 (table 6.2, item 7) |  | n/a |  |
| 38 |  | P-Charging-Vector |  | clause 6.1.1.3.1 (table 6.2, item 6) |  | m on roaming II-NNI, else o |  |
| 39 |  | P-Early-Media |  | clause 6.1.1.3.1 (table 6.2, item 12) |  | m in case of a trust relationship between the interconnected networks, else n/a |  |
| 40 |  | P-Media-Authorization |  | [5] |  | n/a |  |
| 41 |  | P-Preferred-Identity |  | [5] |  | n/a |  |
| 42 |  | P-Preferred-Service |  | [5] |  | m on roaming II-NNI, else n/a |  |
| 43 |  | P-Private-Network-Indication |  | clause 6.1.1.3.1 (table 6.2, item 9) |  | m on roaming II-NNI, else o |  |
| 44 |  | P-Profile-Key |  | clause 6.1.1.3.1 (table 6.2, item 8) |  | o on roaming II-NNI, else n/a |  |
| 44a |  | P-Refused-URI-List |  | [5] |  | o on non-roaming II-NNI, else n/a |  |
| 45 |  | P-Served-User |  | clause 6.1.1.3.1 (table 6.2, item 10) |  | m on roaming II-NNI, else n/a |  |
| 46 |  | P-User-Database |  | [5] |  | n/a |  |
| 47 |  | P-Visited-Network-ID |  | [5] |  | m on roaming II-NNI, else n/a |  |
| 47a |  | Path |  | [5] |  | m on roaming II-NNI, else n/a |  |
| 47b |  | Permission-Missing |  | [5] |  | o |  |
| 47c |  | Policy-Contact |  | [133] and clause 15.6.2 |  | o |  |
| 48 |  | Priority |  | clause 6.1.1.3.1 (table 6.2, item 14) |  | o |  |
| 48b |  | Priority-Share |  | [5] clause 7.2.16 |  | o on roaming II-NNI, else n/a |  |
| 48c |  | Priority-Verstat |  | [5], clause 6.1.1.3.1 (table 6.2, item 26) and clause 34 |  | o on non-roaming II-NNI, else n/a |  |
| 48a |  | Priv-Answer-Mode |  | [5] |  | o |  |
| 49 |  | Privacy |  | [5] |  | m |  |
| 50 |  | Proxy-Authenticate |  | [5] |  | m on roaming II-NNI, else n/a |  |
| 51 |  | Proxy-Authorization |  | [5] |  | m on roaming II-NNI, else n/a |  |
| 52 |  | Proxy-Require |  | [5] |  | m |  |
| 52a |  | RAck |  | [5] |  | m |  |
| 53 |  | Reason |  | [5] and clause 6.1.1.3.1 (table 6.2, item 11) |  | o when in a request. When in a response, m in case of a trust relationship between the interconnected networks, else n/a |  |
| 54 |  | Record-Route |  | [5] |  | m |  |
| 54a |  | Recv-Info |  | [5] |  | o |  |
| 55 |  | Referred-By |  | [5] |  | m |  |
| 55a |  | Refer-Sub |  | [5] |  | m in the case the REFER request is supported, else n/a |  |
| 55b |  | Refer-To |  | [5] |  | m in the case the REFER request is supported, else n/a |  |
| 56 |  | Reject-Contact |  | [5] |  | m |  |
| 56a |  | Relayed-Charge |  | clause 6.1.1.3.1 (table 6.2, item 19) |  | n/a |  |
| 57 |  | Replaces |  | [5] |  | o |  |
| 58 |  | Reply-To |  | [5] |  | o |  |
| 59 |  | Request-Disposition |  | [5] |  | m |  |
| 60 |  | Require |  | [5] |  | m |  |
| 61 |  | Resource-Priority |  | clause 6.1.1.3.1 (table 6.2, item 3) |  | o |  |
| 61c |  | Resource-Share |  | [5] clause 7.2.13 |  | o on roaming II-NNI, else n/a |  |
| 61d |  | Response-Source |  | [5] |  | o in case of a trust relationship between the interconnected networks, else n/a |  |
| 61b |  | Restoration-Info |  | clause 6.1.1.3.1 (table 6.2, item 18) |  | o on roaming II-NNI, else n/a |  |
| 61a |  | Retry-After |  | [5] |  | o |  |
| 62 |  | Route |  | [5] |  | m |  |
| 62a |  | RSeq |  | [5] |  | m |  |
| 63 |  | Security-Client |  | [5] |  | n/a |  |
| 63a |  | Security-Server |  | [5] |  | n/a |  |
| 64 |  | Security-Verify |  | [5] |  | n/a |  |
| 65 |  | Server |  | [5] |  | o |  |
| 65c |  | Service-Interact-Info |  | [5] and clause 6.1.1.3.1 (table 6.2, item 20) |  | o in case of a trust relationship between the interconnected networks, else n/a |  |
| 65a |  | Service-Route |  | [5] |  | m on roaming II-NNI, else n/a |  |
| 65b |  | Session-ID |  | [5] |  | o |  |
| 66 |  | Session-Expires |  | [5] |  | m |  |
| 66a |  | SIP-ETag |  | [5] |  | m in the case the PUBLISH request is supported, else n/a |  |
| 66b |  | SIP-If-Match |  | [5] |  | m in the case the PUBLISH request is supported, else n/a |  |
| 67 |  | Subject |  | [5] |  | o |  |
| 67a |  | Subscription-State |  | [5] |  | m in the case the NOTIFY request is supported, else n/a |  |
| 67b |  | Suppress-If-Match |  | [144] and clause 15.6.4 |  | o |  |
| 68 |  | Supported |  | [5] |  | m |  |
| 68a |  | Target-Dialog |  | [5] |  | o |  |
| 69 |  | Timestamp |  | [5] |  | m |  |
| 70 |  | To |  | [5] |  | m |  |
| 71 |  | Trigger-Consent |  | [5] |  | m |  |
| 71a |  | Unsupported |  | [5] |  | m |  |
| 72 |  | User-Agent |  | [5] |  | m |  |
| 73 |  | User-to-User |  | [5] |  | o |  |
| 74 |  | Via |  | [5] |  | m |  |
| 75 |  | Warning |  | [5] |  | o |  |
| 76 |  | WWW-Authenticate |  | [5] |  | m on roaming II-NNI, else n/a |  |

Table A.2: Key to notation codes for SIP header fields

| Notation code | Meaning |
| --- | --- |
| m | The SIP header field is applicable at II-NNI. Supporting a SIP header field at the II-NNI means that this header field is passed through the IBCF. It does not imply that network elements inside the serving and served networks or user equipment connected to these networks shall support this header field, where 3GPP TS 24.229 [5] is applied. If specified in 3GPP TS 24.229 [5], the IBCF modifies the SIP header field. |
| o | The applicability of SIP header field at II-NNI depends on bilateral agreement between the operators. |
| n/a | It is impossible to use the SIP header field at the II-NNI. This header field could be discarded by the IBCF. |

# *** Next Change ***

B.7	INVITE method

The table B.7.1 lists the supported header fields within the INVITE request.

Table B.7.1: Supported header fields within the INVITE request

| Item | Header field | Ref. | RFC status | II-NNI condition |
| --- | --- | --- | --- | --- |
| 1 | Accept | [13] | o | IF dc1 (AOC: clause 12.22) THEN dm ELSE do |
| 2 | Accept-Contact | [51] | o | IF dc2 (PNM: clause 12.17) THEN dm ELSE do |
| 3 | Accept-Encoding | [13] | o | do |
| 4 | Accept-Language | [13] | o | do |
| 5 | Additional-Identity | [5] | n/a | IF table 6.1.3.1/126 THEN do (NOTE 3) |
| 6 | Alert-Info | [13] | o | IF dc3 (CRS: clause 12.15) THEN dm ELSE IF table 6.1.3.1/10 THEN do (NOTE 3) |
| 7 | Allow | [13] | o | do |
| 8 | Allow-Events | [20] | o | IF table 6.1.3.1/23 THEN do (NOTE 3) |
| 9 | Answer-Mode | [94] | o | IF table 6.1.3.1/90 THEN do (NOTE 3) |
| 10 | Authorization | [13] | o | IF table 6.1.3.1/7 THEN do (NOTE 3) |
| 11 | Attestation-Info | [5] | n/a | IF (non-roaming II-NNI OR loopback traversal scenario) AND table 6.1.3.1/123 THEN do (NOTE 3) |
| 12 | Call-ID | [13] | m | dm |
| 13 | Call-Info | [13], [223] | o | IF dc4 (CCBS/CCNR/CCNL: clause 12.11/12.12/12.23) OR dc5 (announcements: clause 12.21) OR dc13 (eCNAM: clause 12.25) THEN dm ELSE do |
| 14 | Cellular-Network-Info | [5] | n/a | IF table 6.1.3.1/117 THEN do (NOTE 3) |
| 15 | Contact | [13] | m | dm |
| 16 | Content-Disposition | [13] | o | IF dc6 (AOC: clause 12.22) THEN dm ELSE do |
| 17 | Content-Encoding | [13] | o | do |
| 18 | Content-ID | [216] | o | IF table 6.1.3.1/122 THEN do |
| 19 | Content-Language | [13] | o | do |
| 20 | Content-Length | [13] | t | dt |
| 21 | Content-Type | [13] | * | d* |
| 22 | CSeq | [13] | m | dm |
| 23 | Date | [13] | o | do |
| 23a | DC-Info | [5] | n/a | IF table 6.1.3.1/128 THEN do |
| 24 | Expires | [13] | o | do |
| 25 | Feature-Caps | [143] | o | IF table 6.1.3.1/103 THEN do (NOTE 3) |
| 26 | From | [13] | m | dm |
| 27 | Geolocation | [68] | o | do |
| 28 | Geolocation-Routing | [68] | o | do |
| 29 | History-Info | [25] | o | IF dc7 (CDIV: clause 12.6) OR dc2 (PNM: clause 12.17) THEN dm ELSE IF table 6.1.3.1/50 AND initial request THEN do (NOTE 1, NOTE 3) |
| 30 | Identity | [206] | o | IF (non-roaming II-NNI OR loopback traversal scenario) AND table 6.1.3.1/119 THEN do (NOTE 3) |
| 31 | In-Reply-To | [13] | o | do |
| 32 | Join | [55] | o | IF table 6.1.3.1/48 THEN do (NOTE 3) |
| 33 | Max-Breadth | [79] | o | do |
| 34 | Max-Forwards | [13] | m | dm |
| 35 | MIME-Version | [13] | o | do |
| 36 | Min-SE | [52] | o | do |
| 37 | Organization | [13] | o | do |
| 38 | Origination-Id | [5] | n/a | IF (non-roaming II-NNI OR loopback traversal scenario) AND table 6.1.3.1/124 THEN do (NOTE 3) |
| 39 | P-Access-Network-Info | [24], [24B] | o | IF table 6.1.3.1/36 THEN do (NOTE 3) |
| 40 | P-Asserted-Identity | [44] | o | IF table 6.1.3.1/27 AND initial request THEN dm (NOTE 2, NOTE 3) |
| 41 | P-Asserted-Service | [26] | o | IF (non-roaming II-NNI OR loopback traversal scenario OR home-to-visited request on roaming II-NNI) AND table 6.1.3.1/77 AND initial request THEN do (NOTE 3) |
| 42 | P-Called-Party-ID | [24] | o | IF home-to-visited request on roaming II-NNI AND table 6.1.3.1/34 THEN do (NOTE 3) |
| 43 | P-Charging-Function-Addresses | [24] | o | dn/a |
| 44 | P-Charging-Vector | [24] | o | IF table 6.1.3.1/38 AND initial request THEN dm (NOTE 3) |
| 45 | P-Early-Media | [74] | o | IF table 6.1.3.1/69 THEN do (NOTE 3) |
| 46 | P-Media-Authorization | [42] | o | dn/a |
| 47 | P-Preferred-Identity | [44] | o | dn/a |
| 48 | P-Preferred-Service | [26] | o | IF visited-to-home request on roaming II-NNI AND table 6.1.3.1/77 AND initial request THEN do (NOTE 3) |
| 49 | P-Private-Network-Indication | [84] | o | IF table 6.1.3.1/80 AND initial request THEN do (NOTE 3) |
| 50 | P-Profile-Key | [64] | o | IF table 6.1.3.1/59 AND initial request THEN do (NOTE 3) |
| 51 | P-Served-User | [85] | o | IF visited-to-home request on roaming II-NNI AND initial request being sent from "privileged sender" THEN dm (NOTE 3) |
| 52 | P-User-Database | [60] | o | dn/a |
| 53 | P-Visited-Network-ID | [24] | o | dn/a |
| 54 | Priority | [13] | o | do |
| 55 | Priority-Share | [5] | n/a | IF home-to-visited request on roaming II-NNI AND table 6.1.3.1/118 THEN do (NOTE 3) |
| 56 | Priority-Verstat | [5] | n/a | IF (non-roaming II-NNI OR loopback traversal scenario) AND table 6.1.3.1/128 THEN do (NOTE 3) |
| 57 | Privacy | [34] | o | IF dc8 (OIP/OIR: clause 12.3) THEN dm ELSE do |
| 58 | Priv-Answer-Mode | [94] | o | IF table 6.1.3.1/90 THEN do (NOTE 3) |
| 59 | Proxy-Authorization | [13] | o | IF table 6.1.3.1/7 THEN do (NOTE 3) |
| 60 | Proxy-Require | [13] | o | do |
| 61 | Reason | [48] | o | IF table 6.1.3.1/40 AND subsequent request THEN do (NOTE 3) |
| 62 | Record-Route | [13] | o | do |
| 63 | Recv-Info | [39] | m | IF table 6.1.3.1/17 THEN dm (NOTE 3) |
| 64 | Referred-By | [53] | o | IF dc9 (ECT: clause 12.13) OR dc10 (CONF: clause 12.19) THEN dm ELSE do |
| 65 | Reject-Contact | [51] | o | do |
| 66 | Relayed-Charge | [5] | n/a | dn/a |
| 67 | Replaces | [54] | o | IF dc11 (ECT: clause 12.13) OR dc12 (CONF: clause 12.19) THEN dm ELSE IF table 6.1.3.1/47 THEN do (NOTE 3) |
| 68 | Reply-To | [13] | o | do |
| 69 | Request-Disposition | [51] | o | do |
| 70 | Require | [13] | c | IF dc11 (ECT: clause 12.13) THEN dm ELSE dc |
| 71 | Resource-Priority | [78] | o | IF table 6.1.3.1/73 THEN do (NOTE 3) |
| 72 | Resource-Share | [5] | n/a | IF (home-to-visited request on roaming II-NNI OR visited-to-home request on roaming II-NNI) AND table 6.1.3.1/116 THEN do (NOTE 3) |
| 73 | Restoration-Info | [5] | n/a | IF home-to-visited request on roaming II-NNI AND initial request AND table 6.1.3.1/113 THEN do (NOTE 3) |
| 74 | Route | [13] | c | dc |
| 75 | Security-Client | [47] | o | dn/a |
| 76 | Security-Verify | [47] | o | dn/a |
| 77 | Service-Interact-Info | [5] | n/a | IF table 6.1.3.1/125 AND initial request THEN do (NOTE 3) |
| 78 | Session-Expires | [52] | o | do |
| 79 | Session-ID | [124] | m | IF table 6.1.3.1/94 THEN dm (NOTE 3) |
| 80 | Subject | [13] | o | do |
| 81 | Supported | [13] | m* | IF dc2 (PNM: clause 12.17) THEN dm ELSE dm* |
| 82 | Target-Dialog | [140] | o | IF table 6.1.3.1/102 THEN do (NOTE 3) |
| 83 | Timestamp | [13] | o | do |
| 84 | To | [13] | m | dm |
| 85 | Trigger-Consent | [82] | o | IF table 6.1.3.1/78 THEN do (NOTE 3) |
| 86 | User-Agent | [13] | o | do |
| 87 | User-to-User | [83] | o | IF table 6.1.3.1/79 THEN do (NOTE 3) |
| 88 | Via | [13] | m | dm |
| dc1: request invoked due to AOC AND visited-to-home request on roaming II-NNI dc2: initial request invoked due to PNM in case of ("PN UE redirection" OR "PN access control") AND (non-roaming II-NNI OR loopback traversal scenario OR home-to-visited request on roaming II-NNI) dc3: initial request invoked due to CRS AND (non-roaming II-NNI OR loopback traversal scenario OR home-to-visited request on roaming II-NNI) dc4: initial request invoked due to CCBS/CCNR/CCNL AND "CC call" request AND (non-roaming II-NNI OR loopback traversal scenario OR home-to-visited request on roaming II-NNI) dc5: subsequent request invoked due to announcements using the Call-info header field AND (non-roaming II-NNI OR loopback traversal scenario OR home-to-visited request on roaming II-NNI) dc6: request invoked due to AOC AND home-to-visited request on roaming II-NNI dc7: initial request towards the network serving "the diverted-to user" AND CFU/CFB/CFNR/CFNRc/CD/CFNL AND (non-roaming II-NNI OR loopback traversal scenario OR home-to-visited request on roaming II-NNI) dc8: (initial request invoked due to OIP/OIR AND "override the default setting" in temporary mode AND visited-to-home request on roaming II-NNI) OR (initial request invoked due to OIP/OIR AND "presentation of the public user identity is restricted" AND (non-roaming II-NNI OR loopback traversal scenario OR home-to-visited request on roaming II-NNI)) dc9: initial request invoked due to ECT AND (non-roaming II-NNI OR loopback traversal scenario OR home-to-visited request on roaming II-NNI) dc10: (initial request from "conference participant" receiving REFER request with Referred-By header field AND (visited-to-home request on roaming II-NNI OR non-roaming II-NNI OR loopback traversal scenario)) OR (initial request from "conference focus" receiving REFER request AND (non-roaming II-NNI OR loopback traversal scenario OR home-to-visited request on roaming II-NNI)) dc11: "Consultative transfer" due to ECT AND (non-roaming II-NNI OR loopback traversal scenario OR home-to-visited request on roaming II-NNI) dc12: initial request from "conference focus" receiving REFER request with Replaces header field escaped in Refer-To header field AND (non-roaming II-NNI OR loopback traversal scenario OR home-to-visited request on roaming II-NNI) dc13: initial request AND home-to-visited request on roaming II-NNI |  |  |  |  |
| NOTE 1: The Privacy header field can be escaped in the header field for CDIV. NOTE 2: The header field is not included in some exceptional cases. (e.g. when an incoming call from a circuit-switched network has not included the required information to be mapped into the header field) NOTE 3: If the capability specified in table 6.1.3.1 is not supported over the II-NNI, the IBCF in the receiving network can omit or modify the SIP header fields in the received SIP message prior to forwarding the SIP message as specified in 3GPP TS 24.229 [5] clause 5.10.6.2. |  |  |  |  |

The table B.7.2 lists the supported header fields within the INVITE response.

Table B.7.2: Supported header fields within the INVITE response

| Item | Header field | SIP status code | Ref. | RFC status | II-NNI condition |
| --- | --- | --- | --- | --- | --- |
| 1 | Accept | 2xx | [13] | o | do |
|  |  | 415 |  | c | dc |
| 2 | Accept-Encoding | 2xx | [13] | o | do |
|  |  | 415 |  | c | dc |
| 3 | Accept-Language | 2xx | [13] | o | do |
|  |  | 415 |  | c | dc |
| 4 | Accept-Resource-Priority | 2xx 417 | [78] | o | IF table 6.1.3.1/73 THEN do (NOTE 3) |
| 5 | Alert-Info | 180 | [13] | o | IF dc1 (CW: clause 12.7) OR dc2 (announcements: clause 12.21) THEN dm ELSE IF table 6.1.3.1/10 THEN do (NOTE 3) |
| 6 | Allow | 2xx | [13] | m* | dm* |
|  |  | 405 |  | m | dm |
|  |  | others |  | o | do |
| 7 | Allow-Events | 2xx | [20] | o | IF table 6.1.3.1/23 THEN do (NOTE 3) |
| 8 | Answer-Mode | 2xx | [94] | o | IF table 6.1.3.1/90 THEN do (NOTE 3) |
| 9 | Authentication-Info | 2xx | [13] | o | IF table 6.1.3.1/7 THEN do (NOTE 3) |
| 10 | Call-ID | 100 others | [13] | m | dm |
| 11 | Call-Info | r | [13] | o | IF dc3 (CCBS: clause 12.11) OR dc4 (CCNR: clause 12.12) OR dc5 (CCNL: clause 12.23) THEN dm ELSE do |
| 12 | Cellular-Network-Info | r | [5] | n/a | IF table 6.1.3.1/117 THEN do (NOTE 3) |
| 13 | Contact | 18x 199 | [13] [5] | o | dm |
|  |  | 2xx |  | m | dm |
|  |  | 3xx 485 |  | o | do |
| 14 | Content-Disposition | r | [13] | o | IF dc6 (AOC: clause 12.22) THEN dm ELSE do |
| 15 | Content-Encoding | r | [13] | o | do |
| 16 | Content-ID | r | [216] | o | IF table 6.1.3.1/122 THEN do |
| 17 | Content-Language | r | [13] | o | do |
| 18 | Content-Length | 100 others | [13] | t | dt |
| 19 | Content-Type | r | [13] | * | IF dc7 (CCBS: clause 12.11) OR dc8 (CCNR/CCNL: clause 12.12/12.23) THEN dm ELSE d* |
| 20 | CSeq | 100 others | [13] | m | dm |
| 21 | Date | 100 others | [13] | o | IF dc7 (CCBS: clause 12.11) OR dc8 (CCNR/CCNL: clause 12.12/12.23) THEN dm ELSE do |
| 22 | Error-Info | 3xx-6xx | [13] | o | IF dc9 (announcements: clause 12.21) THEN dm ELSE IF table 6.1.3.1/13 THEN do (NOTE 3) |
| 23 | Expires | r | [13] | o | do |
| 24 | Feature-Caps | 18x 2xx | [143] | o | IF table 6.1.3.1/103 THEN do (NOTE 3) |
| 25 | From | 100 others | [13] | m | dm |
| 26 | Geolocation-Error | 424 | [68] | m | dm |
|  |  | others |  | o | do |
| 27 | History-Info | r | [25] | o | IF table 6.1.3.1/50 AND response to initial request THEN do (NOTE 2, NOTE 3, NOTE 4) |
| 28 | MIME-version | r | [13] | o | do |
| 29 | Min-SE | 422 | [52] | m | dm |
| 30 | Organization | r | [13] | o | do |
| 31 | P-Access-Network-Info | r | [24], [24A], [24B] | o | IF table 6.1.3.1/36 THEN do (NOTE 3) |
| 32 | P-Answer-State | 18x 2xx | [73] | o | IF table 6.1.3.1/68 THEN do (NOTE 3) |
| 33 | P-Asserted-Identity | r | [44] | o | IF dc11 (TIP/TIR: clause 12.4) THEN dm ELSE IF table 6.1.3.1/27 AND response to initial request THEN do (NOTE 3) |
| 34 | P-Charging-Function-Addresses | r | [24], [24A] | o | dn/a |
| 35 | P-Charging-Vector | 100 | [24], [24A] | o | dn/a |
|  |  | 18x, 2xx |  | o | IF (table 6.1.3.1/38 AND response to initial request) OR dc 12(CONF: clause 12.19) THEN dm (NOTE 3) |
|  |  | 3xx-6xx |  | o | do (NOTE 3) |
| 36 | P-Early-Media | 18x | [74] | o | IF dc13 (CAT: clause 12.14) OR dc14 (announcements: clause 12.21) THEN dm ELSE IF table 6.1.3.1/69 THEN do (NOTE 3) |
| 37 | P-Media-Authorization | 18x 2xx | [42] | o | dn/a |
| 38 | P-Preferred-Identity | r | [44] | o | dn/a |
| 39 | P-Refused-URI-List | 403 | [141] | o | IF (non-roaming II-NNI OR loopback traversal scenario) AND table 6.1.3.1/101 THEN do (NOTE 3) |
| 40 | Permission-Missing | 470 | [82] | o | IF table 6.1.3.1/78 THEN do (NOTE 3) |
| 41 | Priority-Share | 18x 2xx | [5] | n/a | IF home-to-visited response on roaming II-NNI AND table 6.1.3.1/118 THEN do (NOTE 3) |
| 42 | Privacy | r | [34] | o | IF dc15 (TIP/TIR: clause 12.4) THEN dm ELSE do |
| 43 | Priv-Answer-Mode | 2xx | [94] | o | IF table 6.1.3.1/90 THEN do (NOTE 3) |
| 44 | Proxy-Authenticate | 401 (NOTE 1) | [13] | o | do |
|  |  | 407 (NOTE 1) |  | m | dm |
| 45 | Reason | 18x 199 3xx-6xx | [48] | o | IF dc16 (CW: clause 12.7) OR dc17 (ICB: clause 12.10.1) OR dc18 (announcements: clause 12.21) THEN dm ELSE IF table 6.1.3.1/40 OR ((non-roaming II-NNI OR loopback traversal scenario) AND table 6.1.3.1/41B) THEN do (NOTE 3) |
|  |  | 2xx | [48] | o | IF (non-roaming II-NNI OR loopback traversal scenario) AND table 6.1.3.1/41B THEN do (NOTE 3) |
| 46 | Record-Route | 18x 199 2xx | [13] | o | do |
| 47 | Recv-Info | 18x 199 2xx | [39] | c | IF table 6.1.3.1/17 THEN dc (NOTE 3) |
| 48 | Relayed-Charge | r | [5] | n/a | dn/a |
| 49 | Reply-To | r | [13] | o | do |
| 50 | Require | r | [13] | c | IF dc19 (CAT: clause 12.14) THEN dm ELSE dc |
| 51 | Resource-Share | 18x 2xx | [5] | n/a | IF (home-to-visited response on roaming II-NNI OR visited-to-home response on roaming II-NNI) AND table 6.1.3.1/116 THEN do (NOTE 3) |
| 52 | Response-Source | 3xx-6xx | [5] | n/a | IF table 6.1.3.1/121 THEN do (NOTE 3) |
| 53 | Restoration-Info | 408 | [5] | n/a | IF table 6.1.3.1/114 AND response to initial request AND visited-to-home response on roaming II-NNI THEN do (NOTE 3) |
| 54 | Retry-After | 404 413 480 486 500 503 600 603 | [13] | o | do |
| 55 | RSeq | 18x 199 | [18] | o | do |
| 56 | Security-Server | 421 494 | [47] | o | dn/a |
| 57 | Server | r | [13] | o | do |
| 58 | Service-Interact-Info | 18x 2xx | [5] | n/a | IF table 6.1.3.1/125 AND response to initial request THEN do (NOTE 3) |
| 59 | Session-Expires | 2xx | [52] | o | do |
| 60 | Session-ID | r | [124] | m | IF table 6.1.3.1/94 THEN dm (NOTE 3) |
| 61 | Supported | 2xx | [13] | m* | dm* |
| 62 | Timestamp | r | [13] | o | do |
| 63 | To | 100 others | [13] | m | dm |
| 64 | Unsupported | 420 | [13] | m | dm |
| 65 | User-Agent | r | [13] | o | do |
| 66 | User-to-User | r | [83] | o | IF table 6.1.3.1/79 THEN do (NOTE 3) |
| 67 | Via | 100 others | [13] | m | dm |
| 68 | Warning | r | [13] | o | IF dc20 (CDIV: clause 12.6) THEN dm ELSE do |
| 69 | WWW-Authenticate | 401 (NOTE 1) | [13] | m | dm |
|  |  | 407 (NOTE 1) |  | o | do |
| dc1: response invoked due to CW AND (non-roaming II-NNI OR loopback traversal scenario OR home-to-visited response on roaming II-NNI) dc2: response to initial request invoked due to announcements using the Alert-Info header field AND (non-roaming II-NNI OR loopback traversal scenario OR home-to-visited response on roaming II-NNI) dc3: 486 (Busy Here) response invoked due to CCBS AND (non-roaming II-NNI OR loopback traversal scenario OR home-to-visited response on roaming II-NNI) dc4: 180 (Ringing) response invoked due to CCNR AND (non-roaming II-NNI OR loopback traversal scenario) dc5: 480 (Temporarily Unavailable) response invoked due to CCNL AND (non-roaming II-NNI OR loopback traversal scenario OR home-to-visited response on roaming II-NNI) dc6: non-100 provisional response /200 (OK) response invoked due to AOC AND home-to-visited response on roaming II-NNI dc7: 486 (Busy Here) response invoked due to CCBS AND home-to-visited response on roaming II-NNI dc8: 480 (Temporarily Unavailable) response invoked due to CCNR/CCNL AND home-to-visited response on roaming II-NNI dc9: response to initial request invoked due to announcements using the Error-Info header field AND (non-roaming II-NNI OR loopback traversal scenario OR home-to-visited response on roaming II-NNI) dc11: (TIP/TIR AND 2xx response to initial request AND (visited-to-home response on roaming II-NNI OR non-roaming II-NNI OR loopback traversal scenario)) OR (TIP/TIR AND response to initial request AND "presentation is allowed" AND home-to-visited response on roaming II-NNI) dc12: first response from "conference focus" to initial request AND (non-roaming II-NNI OR loopback traversal scenario OR home-to-visited response on roaming II-NNI) dc13: reliable 18x response to initial request invoked due to CAT using ("forking model" OR "gateway model") AND (non-roaming II-NNI OR loopback traversal scenario OR home-to-visited response on roaming II-NNI) dc14: 18x response to initial request invoked due to announcements authorizing early media AND (non-roaming II-NNI OR loopback traversal scenario OR home-to-visited response on roaming II-NNI) dc15: (TIP/TIR AND non-100 response to initial request AND "override the default setting" in temporary mode AND visited-to-home response on roaming II-NNI) OR (TIP/TIR AND 2xx response to initial request AND "presentation of the network asserted identity is restricted" AND (non-roaming II-NNI OR loopback traversal scenario OR home-to-visited response on roaming II-NNI)) dc16: 480 (Temporarily Unavailable) response invoked due to CW at the expiry of the "CW timer" dc17: 603 (Decline) response invoked due to "dynamic ICB" on an early dialog dc18: 183 (Session Progress) response to initial request invoked due to announcements authorizing early media when rejecting a communication request AND (non-roaming II-NNI OR loopback traversal scenario OR home-to-visited response on roaming II-NNI) dc19: first 18x response to initial request invoked due to CAT using "early session model" AND (non-roaming II-NNI OR loopback traversal scenario OR home-to-visited response on roaming II-NNI) dc20: 480 (Temporarily Unavailable)/486 (Busy Here) response to initial request AND CFU/CFB/CFNR/CFNRc/CD/CFNL in case "maximum number of diversions" exceeds AND (non-roaming II-NNI OR loopback traversal scenario OR home-to-visited response on roaming II-NNI) |  |  |  |  |  |
| NOTE 1: The SIP status code is only applicable over the roaming II-NNI. NOTE 2: The Privacy header field can be escaped in the header field for CDIV. NOTE 3: If the capability specified in table 6.1.3.1 is not supported over the II-NNI, the IBCF in the receiving network can omit or modify the SIP header fields in the received SIP message prior to forwarding the SIP message as specified in 3GPP TS 24.229 [5] clause 5.10.6.2. NOTE 4: History-Info header field is always included, if the AS supporting CDIV requirements is applied to the corresponding initial INVITE request applying CDIV at the home IMS network of the terminating user. |  |  |  |  |  |

*** End of Changes ***