3GPP TSG-CT WG3 Meeting # 145	C3-260368

# Goa, India, 09 – 13 February 2026											(Revision of C3-260090)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.522 | CR | 1768 | rev | 1 | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | ImsParamProvision PATCH update |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2026-02-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Nnef_ImsParamProvision PATCH update is missing in the service description, resource and in the method overview table. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Nnef_ImsParamProvision PATCH update for the service description - PATCH operation is added in the resource and method overview table |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | PATCH operation details are not available for the service description and in the resource and method overview table. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4.48.2, 5.44.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR does not impact the OpenAPI descriptions defined in this specification. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First Change * * * *

#### 4.4.48.2	Procedures for IMS Parameters Provisioning

This procedure is used by the AF to request the creation/update/deletion of an IMS parameters provisioning.

In order to request the creation of an IMS Parameters Provisioning:

-	the AF shall trigger the ImsParamProvisioning API by sending an HTTP POST request to the NEF targeting the "IMS Parameters Provisionings" collection resource, with the request body including the ImsPpData data structure;

-	the NEF shall then check whether the AF is authorized to perform this operation or not;

-	if the AF is authorized:

-	the NEF shall then determine the target IMS AS instance either based on local configuration or through discovery from the NRF via the Nnrf_NFDiscovery service API, as defined in 3GPP TS 29.510 [57], and provision the received IMS parameters provisioning data to the determined IMS AS instance using the Nimsas_ImsPP_Create service operation as specified in 3GPP TS 29.175 [78];

and

-	upon reception of a successful response from the IMS AS as defined in 3GPP TS 29.175 [78] and successful processing of the request, the NEF shall respond to the AF with an HTTP "201 Created" status code including an HTTP "Location" header field containing the URI of the created resource, and the response body including a representation of the created "Individual IMS Parameters Provisioning" resource within the ImsPpData data structure.

In order to request the update of an existing "Individual IMS Parameters Provisioning" resource:

-	the AF shall trigger the ImsParamProvisioning API by sending an HTTP PUT/PATCH request to the NEF targeting the corresponding "Individual IMS Parameters Provisioning" resource with the request body including either:

-	the ImsPpData data structure, in case the HTTP PUT method is used; or

-	the requested modifications to the resource, in case the HTTP PATCH method is used.

-	after authorizing the request, the NEF shall interact with the IMS AS to request the provisioning of the received updated IMS parameters provisioning data using the Nimsas_ImsPP_Update service operation as defined in 3GPP TS 29.175 [78]; and

-	upon reception of a successful response from the IMS AS as defined in 3GPP TS 29.175 [78] and successful processing of the request, the NEF shall respond to the AF with either:

-	an HTTP "200 OK" status code with the response body containing a representation of the updated "Individual IMS Parameters Provisioning" resource within the ImsPpData data structure; or

-	an HTTP "204 No Content" status code.

In order to request the deletion of an existing "Individual IMS Parameters Provisioning" resource:

-	the AF shall trigger the ImsParamProvisioning API by sending an HTTP DELETE request targeting the corresponding "Individual IMS Parameters Provisioning" resource to the NEF;

-	after authorizing the request, the NEF shall interact with the IMS AS using the Nims_ImsPP_Delete service operation to request the deletion of the correponding IMS parameters provisioning data as defined in 3GPP TS 29.175 [78]; and

-	upon reception of a successful response from the IMS AS as defined in 3GPP TS 29.175 [78] and successful processing of the request, the NEF shall respond to the AF with an HTTP "204 No Content" status code.

On failure or if the NEF receives an error code from the IMS AS, the NEF shall take proper error handling actions, as specified in clause 5.44.7, and respond to the AF with an appropriate error status code. If the NEF received within an error response a "ProblemDetails" data structure with the "cause" attribute indicating an application error, the NEF shall relay this error response to the AF with a corresponding application error, when applicable.

# * * * *2nd change * * * *

#### 5.44.2.1	Overview

This clause describes the structure for the Resource URIs as shown in figure 5.44.2.1-1 and the resources and HTTP methods used for the ImsParamProvision API.

Figure 5.44.2.1-1: Resource URI structure of the ImsParamProvision API

Table 5.44.2.1-1 provides an overview of the resources and applicable HTTP methods.

Table 5.44.2.1-1: Resources and methods overview

| Resource name |  | Resource URI (relative path under API URI) |  | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- | --- | --- |
| IMS Parameters Provisionings |  | /pp |  | GET | Retrieve all the active IMS Parameters Provisioning(s) managed by the NEF. |
|  |  |  |  | POST | Request the creation of a new IMS Parameters Provisioning at the NEF. |
| Individual IMS Parameters Provisioning |  | /pp/{ppId} |  | GET | Retrieve an existing "Individual IMS Parameters Provisioning" resource. |
|  |  |  |  | PUT | Update an existing "Individual IMS Parameters Provisioning" resource. |
|  |  |  | PATCH |  | Modify an existing "Individual IMS Parameters Provisioning" resource. |
|  |  |  |  | DELETE | Delete an existing "Individual IMS Parameters Provisioning" resource. |

# *** End of Changes ***