3GPP TSG-CT WG3 Meeting #143	C3-254594

# Sophia-Antipolis, France, 13 – 17 October 2025						(Revision of C3-254348)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.561 | CR | 0202 | rev | 1 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | IANA registration for MRI packet transforms |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C3 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM_Ph2 |  |  |  |  |  | Date: |  |  | 2025-10-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Upon TS 33.501 clause 18.2.3 defined packet transform name 3GPP_XRM_AESCCM_8 which has editor’s note needs to be registered in IANA: The defined packet transform below is named 3GPP_XRM_AESCCM_8 in the extended connect HTTP request negotiating the Forward Mode. Editor’s note: The transformation label 3GPP_XRM_AESCCM_8 needs to be registered in IANA. As per IETF draft-ietf-masque-quic-proxy, it is required to register the 3GPP specified packet transform details in the IANA. TS 29.561 clause 22.3.4 specifies the respective packet transform details. As draft-ietf-masque-quic-proxy is in the IETF draft state, IANA early allocation along with RFC 7120 - Early IANA Allocation of Standards Track Code Points has to be followed. The related editor’s note has to be removed: - Editor's Note: IANA registration of the 3gpp:media-related-info transform is needed. The reference number of TS 26.522 in clause 22.2 also need to be corrected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New informative annex with details to register packet transfer name as defined in TS 33.501 clause 18.2.3. The editor’s note is removed. Corrected the reference number of TS 26.522 in clause 22.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | IANA registration is not available for 3GPP specified Media related information transform in the public as defined in TS 33.501 clause 18.2.3. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.3.4, Annex B <new> |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR is based on the revision CR C3-253087 r2 discussed on CT3#142 and now added discussion paper C3-254349 with related analysis reasonings on transform name for QUIC-aware proxying using HTTP, also added the correction to the reference number of TS 26.522 in clause 22.2. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * 1st Change * * * *

## 22.2	Media Related Information

Media Related Information supporting information fields as defined in 3GPP TS 23.501 [2] shall be byte-aligned and coded as shown in Figure 22.2.1 and Table 22.2.1.

| 0 |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  |  |  |  |  | 2 |  |  |  |  |  |  |  |  |  | 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 0 | 1 |
| Version |  |  | Bitmask |  |  |  |  |  |  |  |  |  |  |  |  | E | R |  | D | PSI |  |  |  | PSSN |  |  |  |  |  |  |  |
| Cont |  | PSN |  |  |  |  |  | PSSize |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NPDS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | BSize |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Cont |  |  |  |  |  |  |  | TTNB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| I | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Figure 22.2.1: Media Related Information

Table 22.2.1: Media Related Information

| Media Related Information |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Version (octet 0, bits 0 through 2) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  | 0 |  |  |  | 0 |  |  |  |  |  |  | Bits representing MRI version 1 |
| Bitmask (octet 0, bit 3) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  | Bits representing PDU Set marking not present |  |  |  |
| 1 |  |  |  |  |  |  |  |  |  |  |  | Bits representing PDU Set marking present |  |  |  |
| Bitmask (octet 0, bit 4) (NOTE 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  | Bits representing PDU Set Size (PSSize) not present |  |  |  |
| 1 |  |  |  |  |  |  |  |  |  |  |  | Bits representing PDU Set Size (PSSize) present |  |  |  |
| Bitmask (octet 0, bit 5) (NOTE 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  | Bits representing Number of PDUs in the PDU Set (NPDS) not present |  |  |  |
| 1 |  |  |  |  |  |  |  |  |  |  |  | Bits representing Number of PDUs in the PDU Set (NPDS) present |  |  |  |
| Bitmask (octet 0, bit 6) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  | Bits representing Burst Size (BSize) not present |  |  |  |
| 1 |  |  |  |  |  |  |  |  |  |  |  | Bits representing Burst Size (BSize) present |  |  |  |
| Bitmask (octet 0, bit 7) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  | Bits representing Time To Next Burst (TTNB) not present |  |  |  |
| 1 |  |  |  |  |  |  |  |  |  |  |  | Bits representing Time To Next Burst (TTNB) present |  |  |  |
| Bitmask (octet 1, bit 0) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  | Bits representing Expedited Transfer Indication (I) not present |  |  |  |
| 1 |  |  |  |  |  |  |  |  |  |  |  | Bits representing Expedited Transfer Indication (I) present |  |  |  |
| Bitmask (octet 1, bits 1 through 6) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| These bits are spared and shall be set to zero. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bitmask (octet 1, bit 7) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Extension for the bitmask by 8 bits, if the existing bits are exhausted and new bits are needed to represent new fields in Media Related Information (NOTE 2). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  |  | Bitmask is not extended by 8 bits |  |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  | Bitmask is extended by 8 bits |  |
| End PDU of the PDU Set (1 bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| If PDU Set marking is included, this bit is encoded as End PDU of the PDU Set (E) field of the PDU Set marking as defined in 3GPP TS 26.522 [70]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reserved (2 bits) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| If PDU Set marking is included, these bits are encoded as Reserved (R) field of the PDU Set marking as defined in 3GPP TS 26.522 [70]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| End of Data Burst (1 bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| If PDU Set marking is included, this bit is encoded as End of Data Burst (D) field of the PDU Set marking as defined in 3GPP TS 26.522 [70]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDU Set Importance (4 bits) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| If PDU Set marking is included, this bit is encoded as PDU Set Importance (PSI) field of the PDU Set marking as defined in 3GPP TS 26.522 [70]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDU Set Sequence Number (10 bits) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| If PDU Set marking is included, these bits are encoded as with PDU Set Sequence Number (PSSN) field of the PDU Set marking as defined in 3GPP TS 26.522 [70]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDU Sequence Number within the PDU Set (6 bits) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| If PDU Set marking is included, these bits are encoded as PDU Sequence Number within the PDU Set (PSN) field of the PDU Set marking as defined in 3GPP TS 26.522 [70]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDU Set Size (24 bits) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| If PDU Set marking is included, these bits are encoded as PDU Set Size (PSSize) field of the PDU Set marking as defined in 3GPP TS 26.522 [70]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Number of PDUs in the PDU Set (16 bits) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| If PDU Set marking is included, these bits are encoded as Number of PDUs in the PDU Set (NPDS) field of the PDU Set marking as defined in 3GPP TS 26.522 [70]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Burst Size (24 bits) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| If Burst Size is included, these bits are encoded as Burst Size (BSize) field of the Dynamically Changing Traffic Characteristics marking as defined in 3GPP TS 26.522 [70]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Time To Next Burst (24 bits) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| If Time To Next Burst is included, these bits are encoded as Time To Next Burst (TTNB) field of the Dynamically Changing Traffic Characteristics marking as defined in 3GPP TS 26.522 [70]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Expedited Transfer Indication (1 bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| If Expedited Transfer Indication is included, this bit corresponds to Expedited Transfer Indication (I) field as defined in 3GPP TS 23.501 [2] and is encoded to the value: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  | false |  |  |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  | true |  |  |
| All other additional zero-padding bits shall be ignored. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: This bit represents an optional field of the PDU Set marking, see 3GPP TS 26.522 [70]. If the PDU Set marking are not included in the Media Related Information, this bit shall be set to zero and the associated bits representing the optional field shall not be included in the Media Related Information. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 2: The extension for the bitmask may be repeated by assigning 1 to the last bit i.e., bit 7 of the previous extension. If the bitmask is extended with additional bits, those unused bits of the bitmask extension are spare and shall be set to zero. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

# * * * * 2nd Change * * * *

### 22.3.4	Sending Media Related Information using QUIC-aware proxying using HTTP

If the media traffic is transmitted over QUIC between the AS and the UE, the UPF (acting as client) and the AS (acting as UDP proxy) may negotiate to use the dedicated QUIC Connection IDs and packet transform in Forwarded Mode to avoid re-encapsulation and re-encryption as defined in draft-ietf-masque-quic-proxy [67]. In case of multiple UEs using the same QUIC connection between the UPF and the AS, for each UE the UPF shall initiate a new HTTP CONNECT request to associate each UE with a separate Connection ID for mapping to a Virtual Connection ID as described in in draft-ietf-masque-quic-proxy [67].

As described in draft-ietf-masque-quic-proxy [67], the AS may still send some packets using the tunnelled mode even when the UPF and the AS have negotiated to use Forwarded Mode (e.g. during the initial media exchange with media related information from the AS to the UPF until it is possible to start using Forwarded Mode). Packets sent in tunnelled mode shall be encoded in HTTP Datagrams as defined in clause 22.3.2. Packets sent using Forwarded Mode shall be encoded as defined in this clause.

During the HTTP CONNECT method as described in draft-ietf-masque-quic-proxy [67], in addition to what is required according to clause 22.3.2:

a)	the UPF shall insert 3GPP_MRI_AESCCM_8 in the accept-transform parameter of the proxy-quic-forwarding header field in the HTTP CONNECT request; and

b)	the AS shall indicate 3GPP_MRI_AESCCM_8 in the transform parameter of the proxy-quic-forwarding header field in the HTTP CONNECT successful response.

When the Media Related Information are transformed in the QUIC short header as per IETF RFC 9000 [68], then the 3GPP_MRI_AESCCM_8 shall be used to encode the transformed QUIC packet as shown in Figure 22.3.4-1 (using the notational conventions defined in section 1.3 of IETF RFC 9000 [68]).

Transformed QUIC Packet {

Header Form (1) = 0,

Fixed Bit (1) = 1,

Spin Bit (1),

Reserved Bits (2),

Key Phase (1),

Packet Number Length (2),

Destination Connection ID (0..160),

Length of Protected Media Related Information(8)

Protected Media Related Information (..),

Packet Number (8..32),

Packet Payload (8..),

}

Figure 22.3.4-1: Transformed QUIC Packet with Media Related Information Container as per 3GPP_MRI_AESCCM_8

In Figure 22.3.4-1:

a)	Destination Connection ID shall be set to the mapping Virtual Connection ID;

b)	the length of the protected Media Related Information shall be encoded in the number of bytes;

c)	Protected Media Related Information shall be the output of the security algorithm defined in clause 18.2.4 of 3GPP TS 33.501 [59] which takes as an input the Media Related Information container, defined in clause 22.2. In case of the length of the protected media related information is zero, the Protected Media Related Information shall not be present; and

d)	Packet Number and Packet Payload shall be set to the (end-to-end protected) packet number and the packet payload of the end-to-end packet.

If the Media Related Information is missing, the length of Protected Media Related Information shall be set to zero and the Protected Media Related Information field shall be omitted.

Editor’s Note:	The transform naming is FFS.

# * * * * 3rd changes * * * *

Annex B (informative):
IANA registration of 3GPP defined Media Related Information packet transform Name

# B.1	Introduction

This annex contains the 3GPP defined Media Related Information transform defined in this specification that are registered to IANA as "Packet Transform Names". It follows the Registration Template for Packet Transform Names registry defined in clause 10.3 of IETF draft-ietf-masque-quic-proxy [67].

Editor's Note: The IANA registration is to be completed after RFC is finalized by IETF.

# B.2	3GPP Media Related Information Packet transform name

| Transform Name | Description | Specification | Notes |
| --- | --- | --- | --- |
| 3GPP_MRI_AESCCM_8 | 3GPP defined Media Related Information transform with 3GPP_MRI_AESCCM_8 in the extended connect HTTP request negotiating the Forward Mode. | 3GPP TS 29.561 | Clause 22.3.4 |

Frame Type Name: "3GPP_MRI_AESCCM_8"

Status: "permanent".

Specification: Clause 22.3.4 of 3GPP TS 29.561.

Change Controller: 3GPP Specifications Manager

Contact:

3gppContact@etsi.org

+33 (0)492944200

Notes:

None.

Editor’s Note:	The transform naming is FFS.

# * * * * End of Changes * * * *