3GPP TSG-CT WG4 Meeting #131	C4-254107

# Sophia Antipolis, France; 13th – 17th October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.510 | CR | 1246 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to Key Retrieval from NRF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19, SBA_KDATV-SEC |  |  |  |  |  | Date: |  |  | 2025-09-26 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | CR 29.510 #1231 (approved at CT#109) has extended the Nnrf_AccessToken service to enable an NFp to retrieve the key (raw public key or the X.509 certificate) required to validate the signature of an Access Token (C4-253557) The following aspects should be corrected: 1) Since the URIs used to request an access token and to retrieve keys to validate the signature of an access token are different, the bootstrapping APIs should be able to report both URIs. This is not possible currently. 2) No feature has been defined for the new retrieve-key service operation. As a result, the features supported by the NRF that are returned in the Bootstrapping response cannot indicate whether the NRF supports the new service operation. 3) The description of the service and service operations in clauses 5.4.1 and 5.4.2 should be updated to refer to the new retrieve-key service operation. 4) The title of Figure 5.4.2.3.1-1 is incorrect. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1) A new relation name is defined for the Bootstrapping API to allow returning the URI to be used for retrieving keys from the NRF. 2) A new feature is defined for the Access Token Service to allow the bootstrapping API to report whether the NRF supports the retrieve-key service operation. 3) The description of the service and service operations in clauses 5.4.1 and 5.4.2 is updated to refer to the new retrieve-key service operation. 4) The title of Figure 5.4.2.3.1-1 is corrected. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The NRF consumer (i.e. NF service producer here) cannot determine whether the NRF supports the new Retrieve Key service operation, and the URI to use for retrieving keys from the NRF. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.1, 5.4.2.1, 5.4.2.3.1, 5.5.2.2.1, 6.3.x (new), 6.4.6.3.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The CR does not cause any OpenAPI change. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * 1st Change * * *

## 5.4	Nnrf_AccessToken Service

### 5.4.1	Service Description

The NRF offers an Nnrf_AccessToken service (used for OAuth2 authorization, see IETF RFC 6749 [16]), following the "Client Credentials" authorization grant, as specified in 3GPP TS 33.501 [15]. It exposes a "Token Endpoint" where the Access Token Request service operation can be requested by NF Service Consumers. It may also expose a URI where the Retrieve Request Request service operation can be requested.

### 5.4.2	Service Operations

#### 5.4.2.1	Introduction

The services operations defined for the Nnrf_AccessToken service are as follows:

-	Access Token Request (i.e. Nnrf_AccessToken_Get)

-	Retrieve Key Request (i.e. Nnrf_AccessToken_RetrieveKey)

* * * Next Change * * *

#### 5.4.2.3	Retrieve Key Request

##### 5.4.2.3.1	General

This service operation is used by an NF Service Consumer of the NRF (i.e. the NF Service Producer verifying the Access Token) to request the NRF to provide the key (raw public key, X.509 certificate or certificate chain) that is required to validate the signature of the OAuth 2.0 Access Token.

Figure 5.4.2.3.1-1: Retrieve Key Request

1.	The NF Service Consumer shall send a POST request to resource URI of the "Retrieve Key Request" custom operation, i.e.	"{nrfApiRoot}/oauth2/retrieve-key".

The content of the POST request shall include the following:

-	the NF Instance Id of the token issuer;

-	the header parameter(s) identifying the key used to validate the signature of the access token.

The content of the POST request may include the following:

-	the NF Instance Id of NF Service Producer verifying the Access Token

2a.	On success, "200 OK" shall be returned, the content of the POST response shall contain the raw public key, the X.509 certificate or a certificate chain required to validate the signature of the access token.

2b.	On failure or redirection, 3xx/4xx/5xx response code shall be returned. The content of the POST response may contain a ProblemDetails object with the cause IE set to the value listed table 6.3.4.3.2-2.

* * * Next Change * * *

#### 5.5.2.2	Get

##### 5.5.2.2.1	General

This service operation is used by an NF Service Consumer to request bootstrapping information from the NRF.

Figure 5.5.2.2.1-1: Bootstrapping Request

1.	The NF Service Consumer shall send a GET request to the "Bootstrapping Endpoint".

The "Bootstrapping Endpoint" URI shall be constructed as:

{nrfApiRoot}/bootstrapping

where {nrfApiRoot} represents the concatenation of the "scheme" and "authority" components of the NRF, as defined in IETF RFC 3986 [17]; see also the definition of NRF FQDN and NRF URI in 3GPP TS 23.003 [12], clause 28.3.2.3.

2a.	On success, "200 OK" shall be returned, the content of the GET response shall contain the requested bootstrapping information.

EXAMPLE:

GET https://nrf.example.com/bootstrapping

Accept: application/3gppHal+json

HTTP/2 200 OK

Content-Type: application/3gppHal+json

{

"status": "OPERATIVE",

"_links": {

"self": {

"href": "https://nrf.example.com/bootstrapping"

},

"manage": {

"href": "https://nrf.example.com/nnrf-nfm/v1/nf-instances"

},

"subscribe": {

"href": "https://nrf.example.com/nnrf-nfm/v1/subscriptions"

},

"discover": {

"href": "https://nrf.example.com/nnrf-disc/v1/nf-instances"

},

"authorize": {

"href": "https://nrf.example.com/oauth2/token"

},

"retrieve-key": {

"href": "https://nrf.example.com/oauth2/retrieve-key"

}

},

"nrfFeatures": {

"nnrf-nfm": "1",

"nnrf-disc": "D",

"nnrf-oauth2": "1"

},

"oauth2Required": {

"nnrf-nfm": true,

"nnrf-disc": false

},

"nrfSetId": "set12.nrfset.5gc.mnc012.mcc345",

"nrfInstanceId": "4947a69a-f61b-4bc1-b9da-47c9c5d14b67"

}

2b. On failure or redirection:

-	Upon failure, the NRF shall return "400 Bad Request" status code, including in the response content a JSON object that provides details about the specific error(s) that occurred.

-	In the case of redirection, the NRF shall return 3xx status code, which shall contain a Location header with an URI pointing to the endpoint of another NRF service instance.

* * * Next Change * * *

### 6.3.X	Features supported by the AccessToken service

The following features are defined for the Nnrf_AccessToken service.

Table 6.3.X-1: Features supported by Nnrf_AccessToken service

| Feature Number | Feature | M/O | Description |
| --- | --- | --- | --- |
| 1 | Retrieve-Key | O | Support of the Retrieve Key service operation (see clause 6.3.4.3) |
| Feature number: The order number of the feature within the supportedFeatures attribute (starting with 1). Feature: A short name that can be used to refer to the bit and to the feature. M/O: Defines if the implementation of the feature is mandatory ("M") or optional ("O"). Description: A clear textual description of the feature. |  |  |  |

* * * Next Change * * *

6.4.6.3.3.1	General

This clause describes the possible relation types defined within NRF API. See clause 4.7.5.2 of 3GPP TS 29.501 [5] for the description of the relation types.

Table 6.4.6.3.3.1-1: supported registered relation types

| Relation Name | Description |
| --- | --- |
| self | The "href" attribute of the object associated to this relation type contains the URI of the same resource returned in the response body (i.e. the "bootstrapping" resource). |
| manage | The "href" attribute of the object associated to this relation type contains the URI of the resource used in the Nnrf_NFManagement API to register/deregister/update NF Instances profiles in the NRF (i.e. the "nf-instances" store resource). (NOTE 1) |
| subscribe | The "href" attribute of the object associated to this relation type contains the URI of the resource used in the Nnrf_NFManagement API to manage subscriptions to the NRF (i.e. the "subscriptions" collection resource). (NOTE 1) |
| discover | The "href" attribute of the object associated to this relation type contains the URI of the resource used in the Nnrf_NFDiscovery API (i.e. the "nf-instances" collection resource). |
| authorize | The "href" attribute of the object associated to this relation type contains the URI of the OAuth2 Access Token Request endpoint, used to request authorization to other APIs in the 5G Core Network. (NOTE 2) |
| retrieve-key | The "href" attribute of the object associated to this relation type contains the URI of the OAuth2 Retrieve Key Request endpoint, used to retrieve keys to validate the signature of an OAuth2 Access Token. (NOTE 2) |
| NOTE 1: The URIs of the "manage" and "subscribe" "href" attributes shall have the same apiRoot (i.e. authority and prefix) since these service operations belong to the same service. NOTE 2: The URIs of the "authorize" and "retrieve-key" "href" attributes shall have the same apiRoot (i.e. authority and prefix) since these service operations belong to the same service. |  |

* * * End of Changes * * *