3GPP TSG-CT WG4 Meeting #130	C4-254127

Sophia Antipolis, France, 13th – 17th October 2025

Source:	Qualcomm Incorporated

Title:	Pseudo-CR on update of Solution #3

Spec:	3GPP TR 29.867 v0.2.0

Agenda item:	20.4

Document for:	Approval

1. Introduction

<Introduction part (optional)>

2. Reason for Change

Evaluation of the Technical Report is missing.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 29.867 v0.2.0.

# * * * 1st Change * * * *

## 6.3	Solution #3: SMF-initiated PDU session release

### 6.3.1	General

This solution is proposed to address Key Issue #1: "Preventing UE SIP Registration attempts during P-CSCF Failures." In KI#1, it was requested to identify how to prevent the UE from repeatedly attempting the SIP registration procedure with the P-CSCF in the case of P-CSCF failure.

This proposal suggests utilizing the network-requested PDU session release mechanism defined in 3GPP TS 24.501 [6] to release the PDU session used by the UE to send SIP signalling (e.g. PDU session to the IMS DNN) and thus prevent repeated UE SIP registration attempts. In this solution, P-CSCF failure is detected by the SMF according to the features defined in the existing specifications..

### 6.3.2	Description

#### 6.3.2.1		P-CSCF failure detection in SMF

According to the current specification and/or implementation, the SMF is capable of detecting P-CSCF failure. This clause introduces how to detect the P-CSCF failure.

6.3.2.1.1		P-CSCF failure detection by using P-CSCF Restoration Enhancement Support

In P-CSCF restoration enhancements procedure, the P-CSCF includes the following in the "PcscfRestorationRequestData" data type of the content of the HTTP POST request:

-	the IP address (IPv4 or IPv6) of the UE in the "ueIpv4" or "ueIpv6" attribute, and if the IP address is not unique (e.g. private IPv4 case), the "ipDomain" attribute or the "sliceInfo" attribute if available; or

-	if the IP address is not available or if the IP address is not unique and the "ipDomain" attribute and the "sliceInfo" attribute are not available, the SUPI in the "supi" attribute and the DNN in the "dnn" attribute.

The PCF shall identify the PDU session for which the HTTP POST request applies. If the PCF fails in identifying the PDU session, the PCF shall reject the "P-CSCF restoration" custom operation with an HTTP "500 Internal Server Error" status code with the response body including the ProblemDetails data structure with the "cause" attribute set to "PDU_SESSION_NOT_AVAILABLE".

The PCF sends a request for P-CSCF restoration to the SMF for the corresponding PDU session as described in the clause 4.2.3.18 of 3GPP TS 29.512 [12].

This detection assumes that the P-CSCF is able to report its status to the SMF before it completely fails to handle connections from the UE.

6.3.2.1.2		P-CSCF failure detection via PCF or NRF

As introduced in clause 6.2.6.2.1 (P-CSCF failure/recovery detection via PCF) and clause 6.2.6.2.2 (P-CSCF failure/recovery detection via NRF), SMF can be notified of a P-CSCF failure by PCF or NRF.

#### 6.3.2.2	SMF-initiated PDU session release

The purpose of the network-requested PDU session release procedure defined in 3GPP TS 24.501 [6] is to enable the network to release a PDU session. The SMF initiates this procedure by creating a PDU SESSION RELEASE COMMAND message and setting the 5GSM cause IE to indicate the reason for releasing the session.

In this proposal, the SMF releases the PDU session used by the UE to send SIP signalling, sets the 5GSM cause IE in the PDU SESSION RELEASE COMMAND to #67: "Insufficient resources for specific slice and DNN" to prevent UE SIP registration attempts during P-CSCF failures and includes a Back-off timer IE in the PDU SESSION RELEASE COMMAND message indicating for how long the UE shall not re-try establishing a PDU session to send SIP signalling .

When the UE receives a PDU SESSION RELEASE COMMAND with cause #67 and the Back-off timer IE, it interprets this as a network-side rejection, indicating that the requested S-NSSAI and DNN combination cannot be supported due to insufficient resources. Consequently, the UE will terminate the affected PDU session as instructed by the network, will start the back-off timer and will not re-try establishing the PDU session to send SIP signalling until the back-off timer expires. Once the back-off timer expires, the UE may send another PDU SESSION ESTABLISHMENT REQUEST message or PDU SESSION MODIFICATION REQUEST message for the same DNN. This ensures that SIP signaling does not resume prematurely during network recovery.

Given that releasing the PDU session to send SIP signalling with a back-off timer will cause voice-centric UEs to disable N1 mode and possibly fall back to EPS to obtain voice services, existing mechanisms such as Access Control and NAS-level mobility management congestion control should be proactively applied to prevent network congestion.

### 6.3.3	Impacts on services, entities, and interfaces

SMF:

- Explicit description for detecting the P-CSCF failure

### 6.3.4	Pros

The pros of the solution:

- The solution avoids any impact on AMF and IMS network.

- The solution leverages existing features for P-CSCF failure detection.

- The solution is compatible with legacy UEs, thus offering a practical and backward-compatible alternative.

### 6.3.5	Cons

None

# * * * End of Changes * * * *