3GPP TSG-CT WG4 Meeting #131	C4-254129

# Sophia Antipolis, France; 13th – 17th October 2025

Source:	OPPO

Title:	Discussion paper on Fallback to CP-based LCS

Spec:	3GPP TS 29.572

Agenda item:	19.4 (TEI19, 5G_eLCS_Ph3)

Document for:	Discussion

1. Introduction and reasoning

Clause 6.18.3 of 3GPP TS 23.273 specifies the LMF relocation procedure for sunny day scenarios.

If, however, the target LMF is temporarily unable to take over the provision of the LCS positioning services over the user plane connection, the target LMF will have to switch to legacy control plane provisioning option. If we look into the below pasted quote from TS 23.273, then the following open issues can be identified:

a.	With the Nlmf_Location_UPConfig Response message, the source LMF needs to inform the AMF that the service cannot be continued over the UP. Such application error code, however, is missing in 3GPP TS 29.572, Table 6.1.7.3-1. In addition to this shortcoming, the source LMF needs to receive such information from the target LMF, as explained in the below point (b).

b.	With the Nlmf_Location_LocationContextTransfer Response service operation, the target LMF needs to inform the source LMF that the service cannot be continued over the UP. Such application error code, however, is missing in 3GPP TS 29.572, Table 6.1.7.3-1.

### 6.18.3	Modification of User Plane Connection between UE and LMF

The figure 6.18.3-1 shows how a secure user plane connection between UE and LMF is modified. The flow describes change of LMF but applies also when source and target LMF is the same. The procedure can also be used to terminate the user plane connection to Source LMF not selecting any Target LMF.

Figure 6.18.3-1: Connection modification between UE and LMFs

2. Conclusions

Currently, the same application error codes that are used by both Nlmf_Location_LocationContextTransfer and Nlmf_Location_UPConfig service operations cannot indicate the actual and correct result of these operations. The target LMF cannot inform the source LMF that during the ongoing LMF relocation procedure the target LMF falls back to the LCS provision over CP. Consequently, the source LMF also cannot inform the AMF about that. In both cases the delivered application error code will be misleading. The problem can be solved by adding an appropriate new application error code, which will enable both the target LMF and the source LMF to accurately report the status of the ongoing operations.

3. Proposal

It is proposed to discuss the matter and agree on the way forward. The respective CR could be discussed at CT4#132.