3GPP TSG-CT WG4 Meeting #131	C4-254135

Sophia Antipolis, France; 13th – 17th October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.570 | CR | 0002 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support event notification muting support for Nscp_EventExposure service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19, eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2025-10-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | TS 23.288 specifies requirements for event notification muting (see clause 6.2.7.2), including mechanisms for muting, unmuting, buffering, and exception handling. Other service exposure APIs (e.g. Namf_EventExposure in TS 29.518) already support muting via NotificationFlag, MutingExceptionInstructions, and MutingNotificationsSettings. TS 29.570 (Nscp_EventExposure) currently does not support muting of event notifications during subscription creation, modification, or reporting. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Extend ScpEventExposureSubscription (Table 6.1.6.2.2-1) by adding the following IEs for muting support , notifFlag, and mutingExcInstructions. Extend ScpEventExposureSubsResp by adding mutingNotSettings to provide the SCP’s applied notification buffering settings to the NF Service Consumer. Add new Application Error, when SCP does not accept the received muting exception instructions. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Nscp_EventExposure remains inconsistent with SA2 requirements in TS 23.288. NF Service Consumers will not be able to control event muting, buffering, or retrieval behavior, resulting in potential overload or loss of notifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.2.2, 5.2.2.2.3, 6.1.3.2.3.1, 6.1.6.1, 6.1.6.2.2, 6.1.6.2.4, 6.1.7.3, A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backward compatible correction to the following APIs: TS29570_Nscp_EventExposure.yaml |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * *

##### 5.2.2.2.2	Creation of a subscription

This procedure allows NF Service Consumer to subscribe by using HTTP POST method with the URI of subscriptions collection.

Figure 5.2.2.2.2-1: Subscription of NF service consumer to SCP Event Exposure

1.	The NF Service Consumer shall send an HTTP POST request to the resource URI associated with the subscription collection.

2a.	On success, "201 Created" shall be returned. The response body shall include a HTTP Location header including the subscription ID together with the status code 201 indicating the requested resource is created in the response message.

If the NF Service Consumer includes the notifFlag IE set to "DEACTIVATE", the SCP shall mute the event notification and buffer the event notifications. Additionally, if the NF service consumer includes event muting instructions in mutingExcInstructions, the SCP evaluates the received event muting instructions against local actions (if configured) and, if the subscription creation request is accepted, the SCP may indicate the following information to the NF service consumer in the response:

-	the maximum number of notifications that the SCP can buffer;

-	the maximum duration that the SCP can buffer the notifications.

2b.	On failure or redirection, one of the HTTP status codes listed in Table 6.1.3.2.3.1-3 shall be returned. For a 4xx/5xx response, the message body should contain a ProblemDetails structure indicating appropriate additional error information.

If the SCP cannot accept the received instructions, it shall reject the request with a 403 Forbidden response and the application error "MUTING_EXC_INSTR_NOT_ACCEPTED".

* * * Next Change * * *

##### 5.2.2.2.3	Modification of a subscription

This procedure allows NF Service Consumer to modify an existing subscription by using HTTP PATCH method with the URI of the individual subscription resource.

Figure 5.2.2.2.3-1: Modification of subscription for SCP Event Exposure

1.	The NF Service Consumer shall send a PATCH request to modify a subscription resource in the SCP.

2a.	On success, either "204 No Content" or "200 OK" shall be returned.

If the NF service consumer modifies the notifFlag IE at the SCP :

-	If the notifFlag attribute is set to "DEACTIVATE" in the request and the event notification is currently not muted, the SCP shall mute the event notification and buffer the available events, or

-	If the notifFlag attribute is set to "RETRIEVAL" in the request and the event notification is currently muted, the SCP shall send the buffered events to the NF Service Consumer, mute the event notification again and buffer subsequent events, or

-	If the notifFlag attribute is set to "ACTIVATE" in the request and the event notification is currently muted, the SCP shall send the buffered events to the NF Service Consumer and stop muting the event notification.

In addition, if the PATCH request modifies the notifFlag attribute to "DEACTIVATE" and contains muting exception instructions, the SCP should evaluate the received muting exception instructions against local policies (if configured). If the subscription modification request is accepted, the SCP may include in the response:

-	the maximum number of notifications that the LMF can buffer;

-		the maximum duration that the LMF can buffer the notifications.

2b.	On failure or redirection, one of the HTTP status codes listed in Table 6.1.3.3.3.1-3 shall be returned. For a 4xx/5xx response, the message body contains a ProblemDetails structure indicating appropriate additional error information.

If the SCP cannot accept the received instructions, it shall reject the request with a 403 Forbidden response and the application error "MUTING_EXC_INSTR_NOT_ACCEPTED".

* * * Next Change * * *

###### 6.1.3.2.3.1	POST

This method shall support the URI query parameters specified in table 6.1.3.2.3.1-1.

Table 6.1.3.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.2.3.1-2 and the response data structures and response codes specified in table 6.1.3.2.3.1-3.

Table 6.1.3.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ScpEventExposureSubscription | M | 1 | SCP Event Exposure Subscription to be created |

Table 6.1.3.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ScpEventExposureSubscription | M | 1 | 201 Created | Represents successful creation of an SCP Subscription |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. |
| ProblemDetails | O | 0..1 | 403 Forbidden | Indicates the subscription or the modification of subscription has failed due to application error. The "cause" attribute may be used to indicate one of the following application errors: - MUTING_EXC_INSTR_NOT_ACCEPTED |
| NOTE: The manadatory HTTP error status code for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply, with response body containing an object of ProblemDetails data type (see clause 5.2.7 of 3GPP TS 29.500 [4]). |  |  |  |  |

Table 6.1.3.2.3.1-4: Headers supported by the 201 response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/nscp-ee/<apiVersion>/subscriptions/{subscriptionId} |

Table 6.1.3.2.3.1-5: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same SCP. |

Table 6.3.3.2.3.1-6: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same SCP. |

* * * Next Change * * *

###### 6.1.3.3.3.1	PATCH

This method shall support the URI query parameters specified in table 6.1.3.3.3.1-1.

Table 6.1.3.3.3.1-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 6.1.3.3.3.1-2 and the response data structure and response codes specified in table 6.1.3.3.3.1-3.

Table 6.1.3.3.3.1-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| array(PatchItem) | M | 1..N | It contains the list of changes to be made to the subscription, according to the JSON PATCH format specified in IETF RFC 6902 [15]. |

Table 6.1.3.3.3.1-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Represents a successful update on SCP Event Exposure Subscription |
| ScpEventExposureSubscription | M | 1 | 200 OK | Represents a successful update on SCP Event Exposure Subscription. |
| RedirectResponse | O | 0..1 | 307 Temporary Redirect | Temporary redirection. |
| RedirectResponse | O | 0..1 | 308 Permanent Redirect | Permanent redirection. |
| ProblemDetails | O | 0..1 | 403 Forbidden | Indicates the subscription or the modification of subscription has failed due to application error. The "cause" attribute may be used to indicate one of the following application errors: - MUTING_EXC_INSTR_NOT_ACCEPTED |
| NOTE: The mandatory HTTP error status code for the PATCH method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply, with response body containing an object of ProblemDetails data type (see clause 5.2.7 of 3GPP TS 29.500 [4]). |  |  |  |  |

Table 6.1.3.3.3.1-4: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same SCP. |

Table 6.1.3.3.3.1-5: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the same SCP. |

* * * Next Change * * *

#### 6.1.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.6.1-1 specifies the data types defined for the Nscp_EventExposure service based interface protocol.

Table 6.1.6.1-1: Nscp_EventExposure specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| ScpEventExposureSubscription | 6.1.6.2.2 | Represents an event subscription resource to SCP |  |
| ScpEventExposureNotification | 6.1.6.2.3 | Represents an event notification. |  |
| ScpEventExposureSubsResp | 6.1.6.2.4 | Represent the SCP Event Exposure Response |  |
| ScpEventFilter | 6.1.6.2.5 |  |  |
| ScpEventReport | 6.1.6.2.6 |  |  |
| FailureCauseOccurrence | 6.1.6.2.7 |  |  |
| ScpSignallingInfo | 6.1.6.2.8 | SCP signalling information |  |
| ScpEventFilterConfig | 6.1.6.2.9 | Event filter |  |
| ReselectionStat | 6.1.6.2.10 | Reselection statistics |  |
| FailureCause | 6.1.6.3.3 |  |  |
| ScpEventType | 6.1.6.3.4 |  |  |
| ReselectionReason | 6.1.6.3.5 | Reselection reason |  |
| ConnectionStatus | 6.1.6.3.6 | Connection status |  |

Table 6.1.6.1-2 specifies data types re-used by Nscp_EventExposure service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a brief description of their use within the Nscp_EventExposure service based interface.

Table 6.1.6.1-2: Nscp_EventExposure re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| ServiceName | 3GPP TS 29.510 [10] | Service Name |  |
| NFType | 3GPP TS 29.510 [10] | NF type |  |
| DateTime | 3GPP TS 29.571 [16] | Date time |  |
| DurationSec | 3GPP TS 29.571 [16] | Duration in seconds |  |
| NfInstanceId | 3GPP TS 29.571 [16] | Identifier (UUID) of the NF Instance. The hexadecimal letters of the UUID should be formatted by the sender as lower-case characters and shall be handled as case-insensitive by the receiver. |  |
| NfSetId | 3GPP TS 29.571 [16] | NF Set ID |  |
| Uinteger | 3GPP TS 29.571 [16] | Unsigned Integer |  |
| Uri | 3GPP TS 29.571 [16] | URI |  |
| OverloadControlInfo | 3GPP TS 29.571 [16] | Overload Control information |  |
| NotificationFlag | 3GPP TS 29.571 [16] | Notification flag |  |
| MutingExceptionInstructions | 3GPP TS 29.571 [16] | Muting exception instructions |  |
| MutingNotificationsSettings | 3GPP TS 29.571 [16] | Muting notifications settings |  |
| TimeWindow | 3GPP TS 29.122 [17] | Time window |  |
| RecurTime | 3GPP TS 29.503 [18] | Recurring time |  |

* * * Next Change * * *

##### 6.1.6.2.2	Type: ScpEventExposureSubscription

Table 6.1.6.2.2-1: Definition of type ScpEventExposureSubscription

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| eventList | array(ScpEventFilter) | M | 1..N | Describes the events to be subscribed in subscription request or the events successfully subscribed for this subscription in subscription response. |  |
| eventNotifyUri | Uri | M | 1 | Identifies the recipient of notifications sent by SCP for this subscription. |  |
| notifyCorrelationId | string | M | 1 | Identifies the notification correlation ID. The SCP shall include this ID in the notifications. The value of this IE shall be unique per subscription for a given NF service consumer. |  |
| expiry | DateTime | O | 0..1 | When present, this IE shall represent the UTC time after which the subscribed event(s) shall stop generating report and the subscription becomes invalid. |  |
| notifFlag | NotificationFlag | O | 0..1 | This IE shall Indicate the notification flag. It shall be Used to mute/unmute notifications and to retrieve buffered events during a muted period. See clause 6.2.7 of 3GPP TS 23.288[14]. |  |
| mutingExcInstructions | MutingExceptionInstructions | C | 0..1 | This IE may be included if notifFlag = "DEACTIVATE". It specifies instructions for handling buffered notifications and the subscription if an exception occurs (e.g. buffer full, exceeded number of buffered events). See clause 6.2.7 of 3GPP TS 23.288[14]. |  |

* * * Next Change * * *

##### 6.1.6.2.4	Type: ScpEventExposureSubsResp

Table 6.1.6.2.4-1: Definition of type ScpEventExposureSubsResp

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| expiryTime | DateTime | O | 0..1 | When present, it shall indicate the expiration time of the subscription, as determined by the operator's policy. The NF consumer shall renew or re-subscribe before expiration if continued event notifications are required. |  |
| mutingNotSettings | MutingNotificationsSettings | C | 0..1 | This IE shall Indicate the SCP’s applied muting notification settings (e.g. maximum number of buffered notifications, duration) if event notification muting is activated. See clause 6.2.7 of 3GPP TS 23.288[14]. |  |

* * * Next Change * * *

#### 6.1.7.3	Application Errors

The application errors defined for the Nscp_EventExposure service are listed in Table 6.1.7.3-1.

Table 6.1.7.3-1: Application errors

| Application Error | HTTP status code | Description |
| --- | --- | --- |
| MUTING_EXC_INSTR_NOT_ACCEPTED | 403 Forbidden | The SCP does not accept the received muting exception instructions. |

* * * Next Change * * *

# A.2	Nscp_EventExposure API

openapi: 3.0.0

info:

title: Nscp_EventExposure

version: 1.0.0-alpha.5

description: |

SCP Event Exposure Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.570 V2.0.0; Service Communication Proxy Services.

url: http://www.3gpp.org/ftp/Specs/archive/29_series/29.570/

[Skipped for clarity]

/subscriptions/{subscriptionId}:

patch:

summary: Nscp_EventExposure Subscribe Modify service Operation

tags:

- Individual subscription (Document)

operationId: ModifySubscription

parameters:

- name: subscriptionId

in: path

required: true

description: Unique ID of the subscription to be modified

schema:

type: string

requestBody:

content:

application/json-patch+json:

schema:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PatchItem'

minItems: 1

required: true

responses:

'204':

description: Expected response to a successful subscription modification

'200':

description: Subscription modified successfully

content:

application/json:

schema:

$ref: '#/components/schemas/ScpEventExposureSubsResp'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'501':

$ref: 'TS29571_CommonData.yaml#/components/responses/501'

'502':

$ref: 'TS29571_CommonData.yaml#/components/responses/502'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

[Skipped for clarity]

#

# STRUCTURED TYPES

#

ScpEventExposureSubscription:

description: Describes an event to be subscribed

type: object

properties:

eventList:

type: array

items:

$ref: '#/components/schemas/ScpEventFilter'

minItems: 1

eventNotifyUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

notifyCorrelationId:

type: string

expiry:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

notifFlag:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NotificationFlag'

mutingExcInstructions:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MutingExceptionInstructions'

required:

- eventList

- eventNotifyUri

- notifyCorrelationId

[Skipped for clarity]

ScpEventExposureSubsResp:

description: Represents SCP Event Exposure Subscription Response

type: object

properties:

expiryTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

mutingNotSettings:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MutingNotificationsSettings'

[Skipped for clarity]

* * * End of Changes * * *