3GPP TSG-CT WG4 Meeting #131	C4-254177

Sophia-Antipolis; 13th – 17th October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.510 | CR | 1250 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Default Notification Subscription for UP Notify |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | SBIProtoc19 |  |  |  |  |  | Date: |  |  | 2025-09-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For LCS-UPP connection, the AMF maintains the context for each LCS-UPP connection in the UE context, and the AMF needs to subscribe for UP Notify on the serving LMF for each LCS-UPP connection. When the UE moves from one AMF to another during mobility, the new AMF then needs to explicitly subscribe to UP Notify for all the connections and the old AMF needs to unsubscribe. As an improvement, the AMF may register the default notification subscription for UP Notify and the LMF can find the default notification callback URI to deliver the UP Notify to avoid all these explicit subscribe / unsubscribe service operations. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1/ Define new notification type for UP Notify 2/ Update OpenAPI accordingly. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | When LCS-UPP connections exist in UE Context, the AMF needs to explicitly subscribe to serving LMF for all LCS-UPP connections and the old AMF needs to explicitly unsubscribe to all the serving LMFs during UE mobility with AMF change. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.6.3.4, A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduce backward compatible feature to following APIs: TS29510_Nnrf_NFDiscovery.yaml TS29510_Nnrf_NFManagement.yaml TS29520_Nnwdaf_AnalyticsInfo.yaml TS29520_Nnwdaf_DataManagement.yaml TS29520_Nnwdaf_RoamingData.yaml TS29574_Ndccf_DataManagement.yaml TS29575_Nadrf_DataManagement.yaml TS29576_Nmfaf_3caDataManagement.yaml TS29576_Nmfaf_ContextManagement.yaml |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 6.1.6.3.4	Enumeration: NotificationType

Table 6.1.6.3.4-1: Enumeration NotificationType

| Enumeration value | Description |
| --- | --- |
| "N1_MESSAGES" | Notification of N1 messages. This notification type may be registered by the NF Instance in a default notification subscription at NFProfile level (see clause 6.1.6.2.2) or as part of a specific service instance. If the AMF registers a default notification subscription with this notification type in an NF Service Instance, it may be associated with the service "namf-comm" or with a custom service. If the LMF registers a default notification subscription with this notification type in an NF Service Instance, it may be associated with the service "nlmf-loc" or with a custom service. |
| "N2_INFORMATION" | Notification of N2 information. This notification type may be registered by the NF Instance in a default notification subscription at NFProfile level (see clause 6.1.6.2.2) or as part of a specific service instance. If the AMF registers a default notification subscription with this notification type in an NF Service Instance, it may be associated with the service "namf-comm" or with a custom service. If the LMF registers a default notification subscription with this notification type in an NF Service Instance, it may be associated with the service "nlmf-loc" or with a custom service. |
| "LOCATION_NOTIFICATION" | Notification of Location Information sent by AMF/LMF towards NF Service Consumers (e.g GMLC). This notification type may be registered by the NF Instance in a default notification subscription at NFProfile level (see clause 6.1.6.2.2) or as part of a specific service instance. If the the GMLC registers a default notification subscription with this notification type in an NF Service Instance, it may be associated with the service "ngmlc-loc" or with a custom service. |
| "DATA_REMOVAL_NOTIFICATION" | Notification of Data Removal sent by UDR (e.g., removal of UE registration data upon subscription withdrawal). This notification type shall be registered by the NF Instance in a default notification subscription at NFProfile level (see clause 6.1.6.2.2). |
| "DATA_CHANGE_NOTIFICATION" | Notification of Data Changes sent by UDR. This notification type shall be registered by the NF Instance in a default notification subscription at NFProfile level (see clause 6.1.6.2.2). |
| "LOCATION_UPDATE_NOTIFICATION" | Notification of UE Location Information Update sent by GMLC towards NF Service Consumers (e.g. H-GMLC, NEF), during MO_LR procedure. This notification type may be registered by the NF Instance in a default notification subscription at NFProfile level (see clause 6.1.6.2.2) or as part of a specific service instance. If the the GMLC registers a default notification subscription with this notification type in an NF Service Instance, it may be associated with the service "ngmlc-loc" or with a custom service. If the the NEF registers a default notification subscription with this notification type in an NF Service Instance, it may be associated with the service "nnef-eventexposure" or with a custom service. |
| "NSSAA_REAUTH_NOTIFICATION" | Re-authentication notification for slice-specific authentication and authorization sent by NSSAAF towards NF Service Consumers (e.g. AMF). This notification type should be registered by the NF Instance in a default notification subscription at NFProfile level (see clause 6.1.6.2.2); otherwise, it may be registered in a custom service instance. |
| "NSSAA_REVOC_NOTIFICATION" | Revocation notification for slice-specific authentication and authorization sent by NSSAAF towards NF Service Consumers (e.g. AMF). This notification type should be registered by the NF Instance in a default notification subscription at NFProfile level (see clause 6.1.6.2.2); otherwise, it may be registered in a custom service instance. |
| "MATCH_INFO_NOTIFICATION" | Notification of a matching result, and the information that can be used for charging purpose by 5G DDNMF towards NF Service Consumers (e.g. 5G DDNMF), during Discovery Reporting procedures. This notification type should be registered by the NF Instance in a default notification subscription at NFProfile level (see clause 6.1.6.2.2); otherwise, it may be registered in a custom service instance. |
| "DATA_RESTORATION_NOTIFICATION" | Notification by UDR to its NF Service Consumers (e.g. UDM, PCF, NEF…) or by UDM to its NF Service Consumers (e.g. AMF, SMF, SMSF…) of a potential data-loss event originated at UDR or due to a Subscriber Data Migration procedure (see 3GPP TS 23.527 [27], clause 6.7.3). The content of the notification shall be as described in 3GPP TS 29.503 [36], clause 5.3.2.12.2 and 6.2.5.4. This notification type should be registered by the NF Instance in a default notification subscription at NFProfile level (see clause 6.1.6.2.2); otherwise, it may be registered in a custom service instance. |
| "TSCTS_NOTIFICATION" | Notification sent by PCF to TSCTSF of TSC user-plane node information. The content of the notification is described in 3GPP TS 29.514 [47], clause 4.2.5.16. This notification type should be registered by the NF Instance in a default notification subscription at NFProfile level (see clause 6.1.6.2.2); otherwise, it may be registered in a custom service instance. |
| "LCS_KEY_DELIVERY_NOTIFICATION" | Notification sent by LMF to AMF to deliver cipering key information. This notification type should be registered by the NF Instance in a default notification subscription at NFProfile level (see clause 6.1.6.2.2); otherwise, it may be registered in a custom service instance. |
| "UUAA_MM_AUTH_NOTIFICATION" | Authentication notification sent by UAS-NF towards NF Service Consumers (i.e. AMF), during USS Initiated reauthorization, update authorization data or revoke authorization with UUAA-MM procedures. This notification type should be registered by the NF Instance in a default notification subscription at NFProfile level (see clause 6.1.6.2.2); otherwise, it may be registered in a custom service instance. |
| "DC_SESSION_EVENT_NOTIFICATION" | Notification sent by IMS AS towards NF Service Consumers (i.e. DCSF) for IMS AS session event control. This notification type should be registered by the NF Instance in a default notification subscription at NFProfile level (see clause 6.1.6.2.2); otherwise, it may be registered in a custom service instance. |
| "LCS_UP_NOTIFY" | Notification sent by LMF to AMF to deliver LCS-UPP connection related information. This notification type should be registered by the NF Instance in a default notification subscription at NFProfile level (see clause 6.1.6.2.2); otherwise, it may be registered in a custom service instance. |

* * * Next Change * * * *

# A.2	Nnrf_NFManagement API

openapi: 3.0.0

************************ Text Sipped for Clarity *****************

NotificationType:

description: >

Types of notifications used in Default Notification URIs in the NF Profile of an NF Instance

anyOf:

- type: string

enum:

- N1_MESSAGES

- N2_INFORMATION

- LOCATION_NOTIFICATION

- DATA_REMOVAL_NOTIFICATION

- DATA_CHANGE_NOTIFICATION

- LOCATION_UPDATE_NOTIFICATION

- NSSAA_REAUTH_NOTIFICATION

- NSSAA_REVOC_NOTIFICATION

- MATCH_INFO_NOTIFICATION

- DATA_RESTORATION_NOTIFICATION

- TSCTS_NOTIFICATION

- LCS_KEY_DELIVERY_NOTIFICATION

- UUAA_MM_AUTH_NOTIFICATION

- DC_SESSION_EVENT_NOTIFICATION

- LCS_UP_NOTIFY

- type: string

************************ Text Sipped for Clarity *****************

* * * End of Changes * * * *