3GPP TSG-CT WG4 Meeting #131	C4-254179

Sophia-Antipolis; 13th – 17th October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.502 | CR | 0912 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on DNN Failure Handling with I-SMF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19, 5G_CIoT |  |  |  |  |  | Date: |  |  | 2025-09-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | When AMF received subscription for UE availability after DDN failure, the AMF subscribes to SMF for DNN failure report with Create/Update SM Context including the traffic descriptors. The SMF shall ask the PCF (if PCC is used and DDN Failure Report PCRT is supported) to create PDRs accordingly to monitor the DDN failure matching the traffic descriptors. The motivation to involve PCF for the DDN failure report PCRT is to provide the full flexibility on controlling the PCC rule precedence at PCF, as specified in solution B (agreed and implemented by SA2) of DP Coexistence of PCC control and detection of "DDD status" event and the "Availability after DDN Failure" events with buffering at the UPF (S2-2001809). For PDU session with I-SMF, the AMF provides the DDN failure subscription including the traffic descriptors to the I-SMF, the I-SMF may require the SMF/PCF for the PDRs (if DDN Failure Report PCRT is supported). There are two issues in current specification: 1. The I-SMF isn't aware whether DDN Failure Report PCRT is supported for the PCF for SM Policy Association; and 2. There is no way for I-SMF to provide the Traffic Descriptors for DDN failure subscription to the SMF/PCF. This CR proposes the following solution: 1, introduce a new indication from SMF to I-SMF to indicate that PCRT for DDN failure report is supported, and the indication will be stored in SM Context for I-SMF change. 2. introduce a new IE to allow the I-SMF to pass the traffic descriptors to the SMF/PCF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1/ Define new data type for DDN Failure Subscription with Traffic Descriptors. 2/ Introduces new IEs over N16a to allow the I-SMF and Anchor SMF to exchange the PCRT information and the Traffic Descriptors. 3/ Update OpenAPI accordingly. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | DDN Failure Notification Report for PDU session with I-SMF does not work properly according to the definition of stage 2 requirement. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.8.2.xx(New), 6.1.6.1, 6.1.6.2.9, 6.1.6.2.10, 6.1.6.2.11, 6.1.6.2.12, 6.1.6.2.15, 6.1.6.2.39, 6.1.6.2.xx(New), A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces backward compatible correction to the following APIs: TS29502_Nsmf_PDUSession.yaml |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

###### 5.2.2.8.2.xx	Update for DDN Failure Subscription with Traffic Descriptors

The service operation shall be invoked by I-SMF to provide the updated DDN Failure Notification Subscription information with Traffic Descriptors, when the anchor SMF and the PCF of for SM Policy Association support DDN Failure Notification PCRT.

The requirements specified in clause 5.2.2.8.2.1 shall apply with the following modifications.

1.	Same as step 1 of Figure 5.2.2.8.2-1, with the following modifications.

The POST request shall contain the ddnFailureSubInfo IE containing the DDN Failure Notification Subscription information.

* * * Next Change * * * *

#### 6.1.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.6.1-1 specifies the data types defined for the Nsmf service-based interface protocol.

Table 6.1.6.1-1: Nsmf specific Data Types

| Data type | Clause defined | Description |
| --- | --- | --- |
| SmContextCreateData | 6.1.6.2.2 | Data within Create SM Context Request |
| SmContextCreatedData | 6.1.6.2.3 | Data within Create SM Context Response |
| SmContextUpdateData | 6.1.6.2.4 | Data within Update SM Context Request |
| SmContextUpdatedData | 6.1.6.2.5 | Data within Update SM Context Response |
| SmContextReleaseData | 6.1.6.2.6 | Data within Release SM Context Request |
| SmContextRetrieveData | 6.1.6.2.7 | Data within Retrieve SM Context Request |
| SmContextStatusNotification | 6.1.6.2.8 | Data within Notify SM Context Status Request |
| PduSessionCreateData | 6.1.6.2.9 | Data within Create Request |
| PduSessionCreatedData | 6.1.6.2.10 | Data within Create Response |
| HsmfUpdateData | 6.1.6.2.11 | Data within Update Request towards H-SMF, or from I-SMF to SMF |
| HsmfUpdatedData | 6.1.6.2.12 | Data within Update Response from H-SMF to V-SMF, or from SMF to I-SMF |
| ReleaseData | 6.1.6.2.13 | Data within Release Request |
| HsmfUpdateError | 6.1.6.2.14 | Error within Update Response from H-SMF to V-SMF, or from SMF to I-SMF |
| VsmfUpdateData | 6.1.6.2.15 | Data within Update Request towards V-SMF, or from SMF to I-SMF |
| VsmfUpdatedData | 6.1.6.2.16 | Data within Update Response from V-SMF, or from I-SMF to SMF |
| StatusNotification | 6.1.6.2.17 | Data within Notify Status Request |
| QosFlowItem | 6.1.6.2.18 | Individual QoS flow |
| QosFlowSetupItem | 6.1.6.2.19 | Individual QoS flow to setup |
| QosFlowAddModifyRequestItem | 6.1.6.2.20 | Individual QoS flow requested to be created or modified |
| QosFlowReleaseRequestItem | 6.1.6.2.21 | Individual QoS flow requested to be released |
| QosFlowProfile | 6.1.6.2.22 | QoS flow profile |
| GbrQosFlowInformation | 6.1.6.2.23 | GBR QoS flow information |
| QosFlowNotifyItem | 6.1.6.2.24 | Notification related to a QoS flow |
| SmContextRetrievedData | 6.1.6.2.27 | Data within Retrieve SM Context Response |
| TunnelInfo | 6.1.6.2.28 | Tunnel Information |
| StatusInfo | 6.1.6.2.29 | Status of SM context or of PDU session |
| VsmfUpdateError | 6.1.6.2.30 | Error within Update Response from V-SMF, or from I-SMF to SMF |
| EpsPdnCnxInfo | 6.1.6.2.31 | EPS PDN Connection Information from H-SMF to V-SMF, or from SMF to I-SMF |
| EpsBearerInfo | 6.1.6.2.32 | EPS Bearer Information from H-SMF to V-SMF, or from SMF to I-SMF |
| PduSessionNotifyItem | 6.1.6.2.33 | Notification related to a PDU session |
| EbiArpMapping | 6.1.6.2.34 | EBI to ARP mapping |
| SmContextCreateError | 6.1.6.2.35 | Error within Create SM Context Response |
| SmContextUpdateError | 6.1.6.2.36 | Error within Update SM Context Response |
| PduSessionCreateError | 6.1.6.2.37 | Error within Create Response |
| MmeCapabilities | 6.1.6.2.38 | MME capabilities |
| SmContext | 6.1.6.2.39 | Complete SM Context |
| ExemptionInd | 6.1.6.2.40 | Exemption Indication |
| PsaInformation | 6.1.6.2.41 | PSA Information |
| DnaiInformation | 6.1.6.2.42 | DNAI Information |
| N4Information | 6.1.6.2.43 | N4 Information |
| IndirectDataForwardingTunnelInfo | 6.1.6.2.44 | Indirect Data Forwarding Tunnel Information |
| SmContextReleasedData | 6.1.6.2.45 | Data within Release SM Context Response |
| ReleasedData | 6.1.6.2.46 | Data within Release Response |
| SendMoDataReqData | 6.1.6.2.47 | Data within Send MO Data Request |
| CnAssistedRanPara | 6.1.6.2.48 | SMF derived CN assisted RAN parameters tuning |
| UlclBpInformation | 6.1.6.2.49 | UL CL or BP Information |
| TransferMoDataReqData | 6.1.6.2.50 | Data within Transfer MO Data Request |
| TransferMtDataReqData | 6.1.6.2.51 | Data within Transfer MT Data Request |
| TransferMtDataError | 6.1.6.2.52 | Transfer MT Data Error Response |
| TransferMtDataAddInfo | 6.1.6.2.53 | Transfer MT Data Error Response Additional Information |
| VplmnQos | 6.1.6.2.54 | VPLMN QoS |
| DdnFailureSubs | 6.1.6.2.55 | DDN Failure Subscription |
| RetrieveData | 6.1.6.2.56 | Data within Retrieve Request |
| RetrievedData | 6.1.6.2.57 | Data within Retrieve Response |
| SecurityResult | 6.1.6.2.58 | Security Result |
| UpSecurityInfo | 6.1.6.2.59 | User Plane Security Information |
| DdnFailureSubInfo | 6.1.6.2.60 | DDN Failure Subscription Information |
| AlternativeQosProfile | 6.1.6.2.61 | Alternative QoS Profile |
| ProblemDetailsAddInfo | 6.1.6.2.62 | Problem Details Additional Information |
| ExtProblemDetails | 6.1.6.2.63 | Extended Problem Details |
| QosMonitoringInfo | 6.1.6.2.64 | QoS Monitoring Information |
| IpAddress | 6.1.6.2.65 | IP Address |
| RedundantPduSessionInformation | 6.1.6.2.66 | Redundant PDU Session Information |
| QosFlowTunnel | 6.1.6.2.67 | Tunnel Information per QoS Flow |
| TargetDnaiInfo | 6.1.6.2.68 | Target DNAI Information |
| AfCoordinationInfo | 6.1.6.2.69 | AF Coordination Information |
| NotificationInfo | 6.1.6.2.70 | Notification Correlation ID and Notification URI provided by the NF service consumer |
| AnchorSmfFeatures | 6.1.6.2.71 | Anchor SMF supported features |
| HrsboInfoFromVplmn | 6.1.6.2.72 | HR-SBO Information signaled from VPLMN |
| HrsboInfoFromHplmn | 6.1.6.2.73 | HR-SBO Information signaled from HPLMN |
| EasInfoToRefresh | 6.1.6.2.74 | EAS information to be refreshed for EAS re-discovery |
| EcnMarkingCongestionInfoReq | 6.1.6.2.75 | ECN Marking or Congestion Information Request |
| EcnMarkingCongestionInfoStatus | 6.1.6.2.76 | ECN Marking or Congestion Information Status |
| TscAssistanceInformation | 6.1.6.2.77 | TSC Assistance Information |
| N6JitterInformation | 6.1.6.2.78 | N6 Jitter Information |
| TrafficInfluenceInfo | 6.1.6.2.79 | Traffic influence information applicable at the VPLMN for an HR-SBO PDU session. |
| TrafficInfluenceData | 6.1.6.2.80 | Traffic influence data comprising the Service Data Flow description and the Application Function influence on traffic routing Enforcement Control parameters of a PCC rule. |
| LocalOffloadingMgtInfoFromIsmf | 6.1.6.2.81 | Information signaled by the I-SMF to the SMF for I-SMF based Local Offloading Management |
| LocalOffloadingMgtInfoToIsmf | 6.1.6.2.82 | Information signaled by the SMF to the I-SMF for I-SMF based Local Offloading Management |
| AvailableBitrateMonitoringRequest | 6.1.6.2.83 | Available Bitrate Monitoring Request information |
| DdnFailureSubInfoTrafficDesc | 6.1.6.2.xx | DDN Failure Notification Subscription Information with Traffic Descriptors |
| Teid | 6.1.6.3.2 | GTP Tunnel Endpoint Identifier |
| ProcedureTransactionId | 6.1.6.3.2 | Procedure Transaction Identifier |
| EpsPdnCnxContainer | 6.1.6.3.2 | UE EPS PDN Connection container from SMF to AMF |
| EpsBearerId | 6.1.6.3.2 | EPS Bearer Id |
| EpsBearerContainer | 6.1.6.3.2 | EPS Bearer container from SMF to AMF |
| EpsBearerContextStatus | 6.1.6.3.2 | EPS Bearer context status |
| DrbId | 6.1.6.3.2 | Data Radio Bearer Identifier |
| AdditionalTnlNb | 6.1.6.3.2 | Indication of additional indirect data forwarding tunnel for multi-connectivity |
| ForwardingBearerContainer | 6.1.6.3.2 | Forwarding bearer container |
| SecondaryRatUsageDataReportContainer | 6.1.6.3.2 | Secondary RAT usage data report container |
| DnsServerSecurityInformation | 6.1.6.3.2 | DNS server security information |
| PduSessionPriority | 6.1.6.3.2 | PDU Session Priority |
| UpCnxState | 6.1.6.3.3 | User Plane Connection State |
| HoState | 6.1.6.3.4 | Handover State |
| RequestType | 6.1.6.3.5 | Request Type in Create (SM context) service operation. |
| RequestIndication | 6.1.6.3.6 | Request Indication in Update (SM context) service operation. |
| NotificationCause | 6.1.6.3.7 | Cause for generating a notification |
| Cause | 6.1.6.3.8 | Cause information |
| ResourceStatus | 6.1.6.3.9 | Status of SM context or PDU session resource |
| DnnSelectionMode | 6.1.6.3.10 | DNN Selection Mode |
| EpsInterworkingIndication | 6.1.6.3.11 | EPS Interworking Indication |
| N2SmInfoType | 6.1.6.3.12 | N2 SM Information Type |
| MaxIntegrityProtectedDataRate | 6.1.6.3.13 | Maximum Integrity Protected Data Rate |
| MaReleaseIndication | 6.1.6.3.14 | Multi-Access PDU session release Indication |
| SmContextType | 6.1.6.3.15 | Type of SM Context information |
| PsaIndication | 6.1.6.3.16 | Indication of whether a PSA is inserted or removed |
| N4MessageType | 6.1.6.3.17 | N4 Message Type |
| QosFlowAccessType | 6.1.6.3.18 | Access type associated with the QoS Flow |
| UnavailableAccessIndication | 6.1.6.3.19 | Indicates the access type of a MA PDU session that is unavailable |
| ProtectionResult | 6.1.6.3.20 | Protection Result of the security policy indicated as "preferred" |
| QosMonitoringReq | 6.1.6.3.21 | Indicates to measure UL, or DL, or both UL/DL delays, or to stop on-going measurements. |
| Rsn | 6.1.6.3.22 | Redundancy Sequence Number |
| SmfSelectionType | 6.1.6.3.23 | SMF Selection Type |
| PduSessionContextType | 6.1.6.3.24 | PDU Session Context Type |
| PendingUpdateInfo | 6.1.6.3.25 | Pending Update Information |
| EstablishmentRejectionCause | 6.1.6.3.26 | PDU Session Establishment Rejection Cause |
| EcnMarkingReq | 6.1.6.3.27 | ECN Marking Request |
| CongestionInfoReq | 6.1.6.3.28 | Congestion Information Request |
| ActivationStatus | 6.1.6.3.29 | Activation Status |
| QosMonitoringPdSupported | 6.1.6.3.30 | QoS Monitoring for packet delay supported information |
| QosMonitoringCongestionSupported | 6.1.6.3.31 | QoS Monitoring for congestion supported information |
| AvailableBitrateRequest | 6.1.6.3.32 | Available Bitrate Request |
| AvailBitRateMonSupported | 6.1.6.3.33 | Available Bitrate Monitoring Support Indication |

Table 6.1.6.1-2 specifies data types re-used by the Nsmf service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nsmf service based interface.

Table 6.1.6.1-2: Nsmf re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| Uint32 | 3GPP TS 29.571 [13] | Unsigned 32-bit integers |
| Ipv4Addr | 3GPP TS 29.571 [13] | IPv4 Address |
| Ipv6Prefix | 3GPP TS 29.571 [13] | IPv6 Prefix |
| Uri | 3GPP TS 29.571 [13] | Uniform Resource Identifier |
| Supi | 3GPP TS 29.571 [13] | Subscription Permanent Identifier |
| Pei | 3GPP TS 29.571 [13] | Permanent Equipment Identifier |
| Gpsi | 3GPP TS 29.571 [13] | General Public Subscription Identifier |
| AccessType | 3GPP TS 29.571 [13] | Access Type (3GPP or non-3GPP access) |
| SupportedFeatures | 3GPP TS 29.571 [13] | Supported features |
| Qfi | 3GPP TS 29.571 [13] | QoS Flow Identifier |
| PduSessionId | 3GPP TS 29.571 [13] | PDU Session Identifier |
| PduSessionType | 3GPP TS 29.571 [13] | PDU Session Type |
| Ambr | 3GPP TS 29.571 [13] | PDU Session Aggregate Maximum Bit Rate |
| 5Qi | 3GPP TS 29.571 [13] | 5G QoS Identifier |
| Arp | 3GPP TS 29.571 [13] | Allocation and Retention Priority |
| ReflectiveQoSAttribute | 3GPP TS 29.571 [13] | Reflective QoS Attribute |
| Dynamic5Qi | 3GPP TS 29.571 [13] | QoS characteristics for a 5QI that is neither standardized nor pre-configured. |
| NonDynamic5Qi | 3GPP TS 29.571 [13] | QoS characteristics that replace the default QoS characteristics for a standardized or pre-configured 5QI. |
| PacketLossRate | 3GPP TS 29.571 [13] | Packet Loss Rate |
| NotificationControl | 3GPP TS 29.571 [13] | Notification Control |
| Dnn | 3GPP TS 29.571 [13] | Data Network Name |
| Snssai | 3GPP TS 29.571 [13] | Single Network Slice Selection Assistance Information |
| NfInstanceId | 3GPP TS 29.571 [13] | NF Instance Identifier |
| UserLocation | 3GPP TS 29.571 [13] | User Location |
| TimeZone | 3GPP TS 29.571 [13] | Time Zone |
| ProblemDetails | 3GPP TS 29.571 [13] | Error description |
| UpSecurity | 3GPP TS 29.571 [13] | User Plane Security Policy Enforcement information |
| RefToBinaryData | 3GPP TS 29.571 [13] | Cross-Reference to binary data encoded within a binary body part in an HTTP multipart message. |
| Guami | 3GPP TS 29.571 [13] | Globally Unique AMF ID |
| BackupAmfInfo | 3GPP TS 29.571 [13] | Backup AMF Information |
| PresenceState | 3GPP TS 29.571 [13] | Indicates the UE presence in or out of a LADN service area |
| TraceData | 3GPP TS 29.571 [13] | Trace control and configuration parameters |
| PlmnId | 3GPP TS 29.571 [13] | PLMN Identity |
| RatType | 3GPP TS 29.571 [13] | RAT Type |
| NgApCause | 3GPP TS 29.571 [13] | NGAP Cause |
| 5GMmCause | 3GPP TS 29.571 [13] | 5G MM Cause |
| DurationSec | 3GPP TS 29.571 [13] | Duration in units of seconds |
| AdditionalQosFlowInfo | 3GPP TS 29.571 [13] | Additional QoS Flow Information |
| NfGroupId | 3GPP TS 29.571 [13] | Network Function Group Id |
| SecondaryRatUsageReport | 3GPP TS 29.571 [13] | Secondary RAT Usage Report |
| SecondaryRatUsageInfo | 3GPP TS 29.571 [13] | Secondary RAT Usage Information |
| Dnai | 3GPP TS 29.571 [13] | Data Network Access Identifier |
| PlmnIdNid | 3GPP TS 29.571 [13] | PLMN Identity and, for SNPN, Network Identity |
| SmallDataRateStatus | 3GPP TS 29.571 [13] | Small Data Rate Control Status |
| ApnRateStatus | 3GPP TS 29.571 [13] | APN Rate Control Status |
| StationaryIndication | 3GPP TS 29.571 [13] | Stationary Indication |
| ScheduledCommunicationTime | 3GPP TS 29.571 [13] | Scheduled Communication Time |
| ScheduledCommunicationType | 3GPP TS 29.571 [13] | Scheduled Communication Type |
| TrafficProfile | 3GPP TS 29.571 [13] | Traffic Profile |
| BatteryIndication | 3GPP TS 29.571 [13] | Battery Indication |
| NfSetId | 3GPP TS 29.571 [13] | NF Set Identifier |
| MoExpDataCounter | 3GPP TS 29.571 [13] | MO Exception Data Counter |
| DddTrafficDescriptor | 3GPP TS 29.571 [13] | Traffic Descriptor |
| NfServiceSetId | 3GPP TS 29.571 [13] | NF Service Set ID |
| RedirectResponse | 3GPP TS 29.571 [13] | Response body of the redirect response message |
| ServerAddressingInfo | 3GPP TS 29.571 [13] | Information of a Provisioning Server |
| PcfUeCallbackInfo | 3GPP TS 29.571 [13] | The callback information of the PCF for the UE to allow the PCF for the PDU session to send SM Policy Association Establishment and Termination events notification |
| SatelliteBackhaulCategory | 3GPP TS 29.571 [13] | Satellite backhaul category |
| Uint16 | 3GPP TS 29.571 [13] | Unsigned 16-bit integer |
| GeoSatelliteId | 3GPP TS 29.571 [13] | GEO satellite ID (string) |
| Uinteger | 3GPP TS 29.571 [13] | Unsigned Integer common data type |
| VplmnOffloadingInfo | 3GPP TS 29.571 [13] | VPLMN Specific Offloading Information |
| GroupId | 3GPP TS 29.571 [13] | Internal Group Id |
| OffloadIdentifier | 3GPP TS 29.571 [13] | Offload identifier uniquely identifying a VPLMN offloading policy information instance from the HPLMN (for HR-SBO) or a Local Offloading Management information instance of the PLMN (for I-SMF based Local Offloading Management). |
| Ipv4AddressRange | 3GPP TS 29.571 [13] | IPv4 address range |
| Ipv6AddressRange | 3GPP TS 29.571 [13] | IPv6 address range |
| Fqdn | 3GPP TS 29.571 [13] | FQDN |
| PduSetQosPara | 3GPP TS 29.571 [13] | PDU Set QoS Parameters |
| SmfChargingId | 3GPP TS 29.571 [13] | SMF Charging Identifier |
| Int32 | 3GPP TS 29.571 [13] | Signed 32-bit integer |
| VplmnDlAmbr | 3GPP TS 29.571 [13] | Authorized DL Session AMBR for Offloading for the V-PLMN |
| UeLevelMeasurementsConfiguration | 3GPP TS 29.571 [13] | 5GC UE level measurements configuration |
| LocalOffloadingManagementInfo | 3GPP TS 29.571 [13] | Local Offloading Management Information |
| SatelliteId | 3GPP TS 29.571 [13] | Unique identifier of a satellite |
| Bitrate | 3GPP TS 29.571 [13] | Bit rate |
| EcsAddrConfigInfo | 3GPP TS 29.503 [46] | ECS Address Configuration Information |
| ServiceName | 3GPP TS 29.510 [19] | Service Name |
| WAgfInfo | 3GPP TS 29.510 [19] | Information about N3 terminations at the W-AGF |
| TngfInfo | 3GPP TS 29.510 [19] | Information about N3 terminations at the TNGF |
| TwifInfo | 3GPP TS 29.510 [19] | Information about N3 terminations at the TWIF |
| ChargingInformation | 3GPP TS 29.512 [30] | CHF addresses |
| TrafficControlData | 3GPP TS 29.512 [30] | Contains parameters influencing the routing of traffic for service data flows. |
| FlowInformation | 3GPP TS 29.512 [30] | Ethernet or IP flow packet filter information |
| MultiModalId | 3GPP TS 29.514 [49] | Multi-Modal Service Identifier |
| NgRanTargetId | 3GPP TS 29.518 [20] | NG-RAN Target Id |
| SbiBindingLevel | 3GPP TS 29.518 [20] | SBI Binding Level |
| IpIndex | 3GPP TS 29.519 [38] | Information that identifies which IP pool or external server is used to allocate the IP address. |
| EventType | 3GPP TS 29.564 [48] | Event type supported by the UPF Event Exposure service |
| RoamingChargingProfile | 3GPP TS 32.291 [26] | Roaming Charging Profile |

* * * Next Change * * * *

##### 6.1.6.2.9	Type: PduSessionCreateData

Table 6.1.6.2.9-1: Definition of type PduSessionCreateData

| Attribute name | Data type |  | P |  | Cardinality | Description |  |  | Applicability |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ********************** Table Rows Skipped for Clarity ********************** |  |  |  |  |  |  |  |  |  |
| pendingUpdateInfoList |  | array(PendingUpdateInfo) | O | 1..N |  |  | When present, this IE shall indicate the list of information that are not required to be updated in real-time to the (H-)SMF, i.e. the change of the listed information (e.g. UE location or Timezone) may be piggybacked in a subsequent essential update (e.g. to exchange the N1 message from the UE) to the (H-)SMF. The NF service consumer (i.e. I-SMF/V-SMF) should not trigger an Update to the (H-)SMF including only the change(s) of the listed information. |  |  |
| ddnFailureSubInfo |  | DdnFailureSubInfoTrafficDesc | O | 0..1 |  |  | When present, this IE shall indicate the DDN failure notification Subscription information. |  |  |
| NOTE 1: This IE contains information that the V-SMF or I-SMF only needs to transfer to the UE (without interpretation). It is sent as a separate IE rather than within the n1SmInfoToUE binary data because the Selected SSC mode IE is defined as a "V" IE (i.e. without a Type field) in the NAS PDU Session Establishment Accept message. NOTE 2: In scenarios with a V-SMF/I-SMF insertion, the V-SMF/I-SMF may receive in the Create Response some IEs it has already received during the earlier SM context retrieval from the SMF (e.g. due to the condition of presence of IEs in the Create Response). In such a case, the V-SMF/I-SMF shall overwrite the IEs earlier received with the new IEs received in the Create Response. NOTE 3: The V-SMF/I-SMF shall ignore any QoS Rule(s) associated to a QoS flow received in PduSessionCreatedData during V-SMF/I-SMF insertion scenarios where no QoS Rule(s) can be sent to the UE, i.e. during Registration, Inter NG-RAN node N2 based handover, and EPS to 5GS Idle mode mobility/handover using N26 interface procedures with V-SMF/I-SMF insertion, or during Service Request and Xn based handover procedures with I-SMF insertion. In such scenarios, the (H-)SMF shall initiate a subsequent PDU session modification procedure if it needs to change the QoS Rules associated to the QoS flows. NOTE 4: The I-SMF may use IPv6 index to assist in selecting how the IPv6 prefix is to be allocated for local PSA when IPv6 multi-homing is applied for the PDU session. If the IPv6 index indicates UE IP address allocation should be performed towards DN-AAA server, the DN-AAA server address may be included from the SMF to the I-SMF. NOTE 5: The chargingId IE in SmContext (see clause 6.1.6.2.39) shall be set to the value received in the homeProvidedChargingId IE during an EPS to 5GS Idle mode mobility or Handover of a HR PDU session. NOTE 6: During inter-system mobility from EPS to 5GS, the UE Integrity Protection Maximum Data Rate is not available at the SMF during PDU Session Creation. The UE will provide UE Integrity Protection Maximum Data Rate to the network within a subsequent UE triggered PDU session modification procedure, as specified in clause 4.3.3.2 of 3GPP TS 23.502 [3]. NOTE 7: During an inter-PLMN mobility, after retrieving the SM context from the old V-SMF, I-SMF or anchor SMF, the target V-SMF or I-SMF shall replace the apiRoot of the pduSessionRef with the interPlmnApiRoot (if available) if the anchor SMF is not in the target PLMN, or with the intraPlmnApiRoot (if available) otherwise. The Operator Identifier in the DNN indicates the PLMN ID of the anchor SMF. NOTE 8: Usage of Charging ID with Uint32 value for roaming scenarios may lead to Charging ID collision between SMFs. NOTE 9: The smfChargingId IE in SmContext (see clause 6.1.6.2.39) shall be set to the value in the homeProvidedSmfChargingId IE if received from the H-SMF. |  |  |  |  |  |  |  |  |  |

* * * Next Change * * * *

##### 6.1.6.2.10	Type: PduSessionCreatedData

Table 6.1.6.2.10-1: Definition of type PduSessionCreatedData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| ********************** Table Rows Skipped for Clarity ********************** |  |  |  |  |  |
| pendingUpdateInfoList | array(PendingUpdateInfo) | O | 1..N | When present, this IE shall indicate the list of information that are not required to be updated in real-time to the (H-)SMF, i.e. the change of the listed information (e.g. UE location or Timezone) may be piggybacked in a subsequent essential update (e.g. to exchange the N1 message from the UE) to the (H-)SMF. The NF service consumer (i.e. I-SMF/V-SMF) should not trigger an Update to the (H-)SMF including only the change(s) of the listed information. |  |
| ddnFailurePcrtInd | boolean | O | 0..1 | When present with the value true, this IE indicates that the DDN Failure Notification PCRT is supported by SMF and the PCF for SM Policy Association. Presence of this IE with the value false shall be prohibited. Absence of this IE means the support of DDN Failure Notification PCRT is unknown. |  |
| NOTE 1: This IE contains information that the V-SMF or I-SMF only needs to transfer to the UE (without interpretation). It is sent as a separate IE rather than within the n1SmInfoToUE binary data because the Selected SSC mode IE is defined as a "V" IE (i.e. without a Type field) in the NAS PDU Session Establishment Accept message. NOTE 2: In scenarios with a V-SMF/I-SMF insertion, the V-SMF/I-SMF may receive in the Create Response some IEs it has already received during the earlier SM context retrieval from the SMF (e.g. due to the condition of presence of IEs in the Create Response). In such a case, the V-SMF/I-SMF shall overwrite the IEs earlier received with the new IEs received in the Create Response. NOTE 3: The V-SMF/I-SMF shall ignore any QoS Rule(s) associated to a QoS flow received in PduSessionCreatedData during V-SMF/I-SMF insertion scenarios where no QoS Rule(s) can be sent to the UE, i.e. during Registration, Inter NG-RAN node N2 based handover, and EPS to 5GS Idle mode mobility/handover using N26 interface procedures with V-SMF/I-SMF insertion, or during Service Request and Xn based handover procedures with I-SMF insertion. In such scenarios, the (H-)SMF shall initiate a subsequent PDU session modification procedure if it needs to change the QoS Rules associated to the QoS flows. NOTE 4: The I-SMF may use IPv6 index to assist in selecting how the IPv6 prefix is to be allocated for local PSA when IPv6 multi-homing is applied for the PDU session. If the IPv6 index indicates UE IP address allocation should be performed towards DN-AAA server, the DN-AAA server address may be included from the SMF to the I-SMF. NOTE 5: The chargingId IE in SmContext (see clause 6.1.6.2.39) shall be set to the value received in the homeProvidedChargingId IE during an EPS to 5GS Idle mode mobility or Handover of a HR PDU session. NOTE 6: During inter-system mobility from EPS to 5GS, the UE Integrity Protection Maximum Data Rate is not available at the SMF during PDU Session Creation. The UE will provide UE Integrity Protection Maximum Data Rate to the network within a subsequent UE triggered PDU session modification procedure, as specified in clause 4.3.3.2 of 3GPP TS 23.502 [3]. NOTE 7: During an inter-PLMN mobility, after retrieving the SM context from the old V-SMF, I-SMF or anchor SMF, the target V-SMF or I-SMF shall replace the apiRoot of the pduSessionRef with the interPlmnApiRoot (if available) if the anchor SMF is not in the target PLMN, or with the intraPlmnApiRoot (if available) otherwise. The Operator Identifier in the DNN indicates the PLMN ID of the anchor SMF. NOTE 8: Usage of Charging ID with Uint32 value for roaming scenarios may lead to Charging ID collision between SMFs. NOTE 9: The smfChargingId IE in SmContext (see clause 6.1.6.2.39) shall be set to the value in the homeProvidedSmfChargingId IE if received from the H-SMF. |  |  |  |  |  |

* * * Next Change * * * *

##### 6.1.6.2.11	Type: HsmfUpdateData

Table 6.1.6.2.11-1: Definition of type HsmfUpdateData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| ********************** Table Rows Skipped for Clarity ********************** |  |  |  |  |  |
| amfResynchedInd | boolean | C | 0..1 | This IE shall be present if the AMF has initiated data resynchronization for the corresponding UE, as a result of the processing of the Data Restoration Notification received from the UDM, as specified in clause 6.7.4 of 3GPP TS 23.527 [24]. Presence of this IE with the value false shall be prohibited. | AMF-RESYNCHED |
| ddnFailureSubInfo | DdnFailureSubInfoTrafficDesc | O | 0..1 | When present, this IE shall indicate the DDN failure notification Subscription information with the traffic descriptors. |  |
| NOTE 1: In shared networks, the PLMN ID that is communicated in this IE shall be that of the selected Core Network Operator. NOTE 2: An SMF complying with this version of the specification should report secondary RAT usage using the secondaryRatUsageInfo attribute that replaces the secondaryRatUsageReport attribute. NOTE 3: If the smPolicyNotifyInd IE is received with the value "true", the SMF shall forward the callback information of the PCF for the UE to the PCF for SM Policy if exists via SM Policy Association Modification. See clause 4.3.3.2 of 3GPP TS 23.502 [3]. NOTE 4: The attribute name does not follow the naming conventions specified in 3GPP TS 29.501 [5]. The attribute name is kept though as defined in the current specification for backward compatibility reason. NOTE 5: The maxIntegrityProtectedDataRateUl and maxIntegrityProtectedDataRateDl IEs shall be ignored by the H-SMF or SMF if the Integrity protection maximum data rate in n1SmInfoFromUe IE is received. NOTE 6: The H-SMF shall determine that the UE is not registered for Disaster Roaming service if the disasterRoamingInd is not present during a V-SMF change procedure. NOTE 7: The (H-)SMF shall consider the PDU Set based handling is not supported if the pduSetSupportInd IE is absent during a V-SMF change. |  |  |  |  |  |

* * * Next Change * * * *

##### 6.1.6.2.12	Type: HsmfUpdatedData

Table 6.1.6.2.12-1: Definition of type HsmfUpdatedData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| ********************** Table Rows Skipped for Clarity ********************** |  |  |  |  |  |
| localOffloadMgtInfo | LocalOffloadingMgtInfoToIsmf | C | 0..1 | This IE shall be present if I-SMF based Local Offloading Management applies and any information defined in clause 6.1.6.2.82 needs to be signaled to the I-SMF. | ISMF-LOM |
| ddnFailurePcrtInd | boolean | O | 0..1 | When present with the value true, this IE indicates that the DDN Failure Notification PCRT is supported by the anchor SMF and the PCF for SM Policy Association. Presence of this IE with the value false shall be prohibited. Absence of this IE means the support of DDN Failure Notification PCRT is unknown. |  |
| NOTE 1: During a handover between 3GPP and non-3GPP accesses, the V-SMF or I-SMF shall delete any corresponding information received earlier for the source access type and use the new information received for the target access type. NOTE 2: During inter-system mobility from EPS to 5GS, the UE Integrity Protection Maximum Data Rate is not available at the SMF during PDU Session Creation. The UE will provide UE Integrity Protection Maximum Data Rate to the network within a subsequent UE triggered PDU session modification procedure, as specified in clause 4.3.3.2 of 3GPP TS 23.502 [3]. NOTE 3: The chargingId IE in SmContext (see clause 6.1.6.2.39) shall be set to the value received in the homeProvidedChargingId IE during a HPLMN to VPLMN mobility of a PDU session with I-SMF in HPLMN. NOTE 4: Usage of Charging ID with Uint32 value for roaming scenarios may lead to Charging ID collision between SMFs. NOTE 5: The smfChargingId IE in SmContext (see clause 6.1.6.2.39) shall be set to the value in the homeProvidedSmfChargingId IE if received from the H-SMF. |  |  |  |  |  |

* * * Next Change * * * *

##### 6.1.6.2.15	Type: VsmfUpdateData

Table 6.1.6.2.15-1: Definition of type VsmfUpdateData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| ********************** Table Rows Skipped for Clarity ********************** |  |  |  |  |  |
| netLocInfoReqInd | boolean | C | 0..1 | This IE shall be present with the value true when the (H-)SMF requires the V-SMF or I-SMF to provide the current NetLoc information (UE Location and Time Zone). The presence of this IE with the value "false" shall be prohibited. | NLIRN16 |
| ddnFailurePcrtInd | boolean | O | 0..1 | When present with the value true, this IE indicates that the DDN Failure Notification PCRT is supported by the anchor SMF and the PCF for SM Policy Association. Presence of this IE with the value false shall be prohibited. Absence of this IE means the support of DDN Failure Notification PCRT is unknown. |  |
| NOTE: This IE contains information that the V-SMF shall transfer to the UE without interpretation. It is sent as a separate IE rather than within the n1SmInfoToUE binary data because the 5GSM cause IE is defined as a "V" IE (i.e. without a Type field) in the NAS PDU Session Release Command message. |  |  |  |  |  |

* * * Next Change * * * *

##### 6.1.6.2.39	Type: SmContext

Table 6.1.6.2.39-1: Definition of type SmContext

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| ********************** Table Rows Skipped for Clarity ********************** |  |  |  |  |  |
| localOffloadingInfoList | array(LocalOffloadingManagementInfo) | C | 1..N | This IE shall be present for a PDU session with an I-SMF, during an I-SMF change, if available, the localOffloadingMgtAllowedInd IE is set to true and the request indicates that the new I-SMF supports I-SMF based local offloading management. When present, it shall contain the Local Offloading Information list applicable to the PDU session and whose offload identifiers are not yet known by the target I-SMF. (NOTE 5) | ISMF-LOM |
| ddnFailurePcrtInd | boolean | C | 0..1 | When present with the value true, this IE indicates that the DDN Failure Notification PCRT is supported by the anchor SMF and the PCF for SM Policy Association. Presence of this IE with the value false shall be prohibited. Absence of this IE means the support of DDN Failure Notification PCRT is unknown. |  |
| NOTE 1: If present, this attribute shall be used together with routingIndicator. This attribute is only used by the HPLMN in roaming scenarios. NOTE 2: See NOTE 7 of Table 6.1.6.2.10-1. NOTE 3: If the anchorSmfOauth2Required IE was received in SmContextCreateData from the AMF, this IE shall be ignored by the new I-SMF or V-SMF. NOTE 4: Usage of Charging ID with Uint32 value for roaming scenarios may lead to Charging ID collision between SMFs. NOTE 5: The same offloadId should not appear in both the vplmnOffloadingInfoList/localOffloadingInfoList and offloadIds attributes. Both vplmnOffloadingInfoList/localOffloadingInfoList and offloadIds attributes may be present when multiple offload identifers apply to the PDU session and some of them are already known by the target V-SMF/I-SMF while others are not yet known by the target V-SMF/I-SMF. |  |  |  |  |  |

* * * Next Change * * * *

##### 6.1.6.2.xx	Type: DdnFailureSubInfoTrafficDesc

Table 6.1.6.2.xx-1: Definition of type DdnFailureSubInfoTrafficDesc

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| ddnFailureSubsInd | boolean | M | 1 | Indicates whether the DDN Failure Notification is subscribed or unsubscribed: - true: Notification of DDN failure is subscribed. - false: Notification of DDN failure is unsubscribed. |
| ddnFailureTrafficDescriptorList | array(DddTrafficDescriptor) | C | 1..N | This IE shall be present if ddnFailureSubsInd is set to true. When present, this IE shall contain the TrafficDescriptors requesting for subscribed DDN Failure Notification. |

* * * Next Change * * * *

# A.2	Nsmf_PDUSession API

openapi: 3.0.0

info:

version: '1.4.0-alpha.4'

title: 'Nsmf_PDUSession'

description: |

SMF PDU Session Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

****************** Text skipped for Clarity *******************

PduSessionCreateData:

description: Data within Create Request

type: object

properties:

supi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

unauthenticatedSupi:

type: boolean

default: false

pei:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Pei'

pduSessionId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PduSessionId'

dnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

selectedDnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

sNssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

altSnssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

hplmnSnssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

vsmfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

ismfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

servingNetwork:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnIdNid'

requestType:

$ref: '#/components/schemas/RequestType'

epsBearerId:

type: array

items:

$ref: '#/components/schemas/EpsBearerId'

minItems: 1

pgwS8cFteid:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Bytes'

vsmfPduSessionUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

ismfPduSessionUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

vcnTunnelInfo:

$ref: '#/components/schemas/TunnelInfo'

icnTunnelInfo:

$ref: '#/components/schemas/TunnelInfo'

n9ForwardingTunnelInfo:

$ref: '#/components/schemas/TunnelInfo'

additionalCnTunnelInfo:

$ref: '#/components/schemas/TunnelInfo'

anType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AccessType'

additionalAnType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AccessType'

ratType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RatType'

ueLocation:

$ref: 'TS29571_CommonData.yaml#/components/schemas/UserLocation'

ueTimeZone:

$ref: 'TS29571_CommonData.yaml#/components/schemas/TimeZone'

addUeLocation:

$ref: 'TS29571_CommonData.yaml#/components/schemas/UserLocation'

gpsi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

n1SmInfoFromUe:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RefToBinaryData'

unknownN1SmInfo:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RefToBinaryData'

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

hPcfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

pcfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

pcfGroupId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfGroupId'

pcfSetId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfSetId'

hoPreparationIndication:

type: boolean

selMode:

$ref: '#/components/schemas/DnnSelectionMode'

alwaysOnRequested:

type: boolean

default: false

udmGroupId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfGroupId'

routingIndicator:

type: string

hNwPubKeyId:

type: integer

epsInterworkingInd:

$ref: '#/components/schemas/EpsInterworkingIndication'

vSmfServiceInstanceId:

type: string

iSmfServiceInstanceId:

type: string

recoveryTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

roamingChargingProfile:

$ref: 'TS32291_Nchf_ConvergedCharging.yaml#/components/schemas/RoamingChargingProfile'

chargingId:

type: string

pattern: '^(0|([1-9]{1}[0-9]{0,9}))$'

smfChargingId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SmfChargingId'

oldPduSessionId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PduSessionId'

epsBearerCtxStatus:

$ref: '#/components/schemas/EpsBearerContextStatus'

amfNfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

guami:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Guami'

maxIntegrityProtectedDataRateUl:

$ref: '#/components/schemas/MaxIntegrityProtectedDataRate'

maxIntegrityProtectedDataRateDl:

$ref: '#/components/schemas/MaxIntegrityProtectedDataRate'

cpCiotEnabled:

type: boolean

default: false

cpOnlyInd:

type: boolean

default: false

invokeNef:

type: boolean

default: false

maRequestInd:

type: boolean

default: false

maNwUpgradeInd:

type: boolean

default: false

dnaiList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnai'

minItems: 1

presenceInLadn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PresenceState'

secondaryRatUsageInfo:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SecondaryRatUsageInfo'

minItems: 1

smallDataRateStatus:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SmallDataRateStatus'

apnRateStatus:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ApnRateStatus'

dlServingPlmnRateCtl:

type: integer

minimum: 10

upSecurityInfo:

$ref: '#/components/schemas/UpSecurityInfo'

vplmnQos:

$ref: '#/components/schemas/VplmnQos'

oldSmContextRef:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

redundantPduSessionInfo:

$ref: '#/components/schemas/RedundantPduSessionInformation'

oldPduSessionRef:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

smPolicyNotifyInd:

type: boolean

default: false

pcfUeCallbackInfo:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PcfUeCallbackInfo'

satelliteBackhaulCat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SatelliteBackhaulCategory'

upipSupported:

type: boolean

default: false

upCnxState:

$ref: '#/components/schemas/UpCnxState'

disasterRoamingInd:

type: boolean

default: false

hrsboInfo:

$ref: '#/components/schemas/HrsboInfoFromVplmn'

ecsAddrConfigInfos:

type: array

items:

$ref: 'TS29503_Nudm_PP.yaml#/components/schemas/EcsAddrConfigInfo'

minItems: 1

localOffloadMgtInfo:

$ref: '#/components/schemas/LocalOffloadingMgtInfoFromIsmf'

pduSetSupportInd:

type: boolean

default: false

ecnMarkingCongestionInfoStatus:

type: array

items:

$ref: '#/components/schemas/EcnMarkingCongestionInfoStatus'

minItems: 1

qosMonitoringPdSupported:

$ref: '#/components/schemas/QosMonitoringPdSupported'

qosMonitoringCongestionSupported:

$ref: '#/components/schemas/QosMonitoringCongestionSupported'

availBitRateMonSupported:

$ref: '#/components/schemas/AvailBitRateMonSupported'

pgwChangeInd:

type: boolean

enum:

- true

ddnFailureSubInfo:

$ref: '#/components/schemas/DdnFailureSubInfoTrafficDesc'

required:

- dnn

- servingNetwork

- anType

oneOf:

- required: [ vsmfId, vsmfPduSessionUri ]

- required: [ ismfId, ismfPduSessionUri ]

PduSessionCreatedData:

description: Data within Create Response

type: object

properties:

pduSessionType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PduSessionType'

sscMode:

type: string

pattern: '^[0-7]$'

hcnTunnelInfo:

$ref: '#/components/schemas/TunnelInfo'

cnTunnelInfo:

$ref: '#/components/schemas/TunnelInfo'

additionalCnTunnelInfo:

$ref: '#/components/schemas/TunnelInfo'

sessionAmbr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ambr'

qosFlowsSetupList:

type: array

items:

$ref: '#/components/schemas/QosFlowSetupItem'

minItems: 1

hSmfInstanceId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

smfInstanceId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

pduSessionId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PduSessionId'

sNssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

additionalSnssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

enablePauseCharging:

type: boolean

default: false

ueIpv4Address:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv4Addr'

ueIpv6Prefix:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Prefix'

n1SmInfoToUe:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RefToBinaryData'

epsPdnCnxInfo:

$ref: '#/components/schemas/EpsPdnCnxInfo'

epsBearerInfo:

type: array

items:

$ref: '#/components/schemas/EpsBearerInfo'

minItems: 1

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

maxIntegrityProtectedDataRate:

$ref: '#/components/schemas/MaxIntegrityProtectedDataRate'

maxIntegrityProtectedDataRateDl:

$ref: '#/components/schemas/MaxIntegrityProtectedDataRate'

alwaysOnGranted:

type: boolean

default: false

gpsi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

upSecurity:

$ref: 'TS29571_CommonData.yaml#/components/schemas/UpSecurity'

roamingChargingProfile:

$ref: 'TS32291_Nchf_ConvergedCharging.yaml#/components/schemas/RoamingChargingProfile'

hSmfServiceInstanceId:

type: string

smfServiceInstanceId:

type: string

recoveryTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

dnaiList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnai'

minItems: 1

ipv6MultiHomingInd:

type: boolean

default: false

maAcceptedInd:

type: boolean

default: false

homeProvidedChargingId:

type: string

pattern: '^(0|([1-9]{1}[0-9]{0,9}))$'

homeProvidedSmfChargingId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SmfChargingId'

nefExtBufSupportInd:

type: boolean

default: false

smallDataRateControlEnabled:

type: boolean

default: false

ueIpv6InterfaceId:

type: string

pattern: '^[A-Fa-f0-9]{16}$'

ipv6Index:

$ref: 'TS29519_Policy_Data.yaml#/components/schemas/IpIndex'

dnAaaAddress:

$ref: '#/components/schemas/IpAddress'

redundantPduSessionInfo:

$ref: '#/components/schemas/RedundantPduSessionInformation'

nspuSupportInd:

type: boolean

interPlmnApiRoot:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

intraPlmnApiRoot:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

udmGroupId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfGroupId'

pcfGroupId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfGroupId'

hrsboInfo:

$ref: '#/components/schemas/HrsboInfoFromHplmn'

localOffloadMgtInfo:

$ref: '#/components/schemas/LocalOffloadingMgtInfoToIsmf'

pendingUpdateInfoList:

type: array

items:

$ref: '#/components/schemas/PendingUpdateInfo'

minItems: 1

ddnFailurePcrtInd:

type: boolean

enum:

- true

required:

- pduSessionType

- sscMode

oneOf:

- required: [ hSmfInstanceId ]

- required: [ smfInstanceId ]

HsmfUpdateData:

description: Data within Update Request towards H-SMF, or from I-SMF to SMF

type: object

properties:

requestIndication:

$ref: '#/components/schemas/RequestIndication'

pei:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Pei'

vcnTunnelInfo:

$ref: '#/components/schemas/TunnelInfo'

icnTunnelInfo:

$ref: '#/components/schemas/TunnelInfo'

additionalCnTunnelInfo:

$ref: '#/components/schemas/TunnelInfo'

servingNetwork:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnIdNid'

anType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AccessType'

additionalAnType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AccessType'

ratType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RatType'

ueLocation:

$ref: 'TS29571_CommonData.yaml#/components/schemas/UserLocation'

ueTimeZone:

$ref: 'TS29571_CommonData.yaml#/components/schemas/TimeZone'

addUeLocation:

$ref: 'TS29571_CommonData.yaml#/components/schemas/UserLocation'

pauseCharging:

type: boolean

pti:

$ref: '#/components/schemas/ProcedureTransactionId'

n1SmInfoFromUe:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RefToBinaryData'

unknownN1SmInfo:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RefToBinaryData'

qosFlowsRelNotifyList:

type: array

items:

$ref: '#/components/schemas/QosFlowItem'

minItems: 1

qosFlowsVsmfRejectedList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Qfi'

minItems: 1

qosFlowsNotifyList:

type: array

items:

$ref: '#/components/schemas/QosFlowNotifyItem'

minItems: 1

NotifyList:

type: array

items:

$ref: '#/components/schemas/PduSessionNotifyItem'

minItems: 1

# The attribute name does not follow the naming conventions specified in 3GPP TS 29.501. The attribute name is kept though as defined in the current specification for backward compatibility reason.

epsBearerId:

type: array

items:

$ref: '#/components/schemas/EpsBearerId'

minItems: 0

hoPreparationIndication:

type: boolean

revokeEbiList:

type: array

items:

$ref: '#/components/schemas/EpsBearerId'

minItems: 1

cause:

$ref: '#/components/schemas/Cause'

ngApCause:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NgApCause'

5gMmCauseValue:

$ref: 'TS29571_CommonData.yaml#/components/schemas/5GMmCause'

alwaysOnRequested:

type: boolean

default: false

epsInterworkingInd:

$ref: '#/components/schemas/EpsInterworkingIndication'

secondaryRatUsageReport:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SecondaryRatUsageReport'

minItems: 1

secondaryRatUsageInfo:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SecondaryRatUsageInfo'

minItems: 1

anTypeCanBeChanged:

type: boolean

default: false

maReleaseInd:

$ref: '#/components/schemas/MaReleaseIndication'

maNwUpgradeInd:

type: boolean

default: false

maRequestInd:

type: boolean

default: false

unavailableAccessInd:

$ref: '#/components/schemas/UnavailableAccessIndication'

psaInfo:

type: array

items:

$ref: '#/components/schemas/PsaInformation'

minItems: 1

ulclBpInfo:

$ref: '#/components/schemas/UlclBpInformation'

n4Info:

$ref: '#/components/schemas/N4Information'

n4InfoExt1:

$ref: '#/components/schemas/N4Information'

n4InfoExt2:

$ref: '#/components/schemas/N4Information'

presenceInLadn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PresenceState'

vsmfPduSessionUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

vsmfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

vSmfServiceInstanceId:

type: string

ismfPduSessionUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

ismfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

iSmfServiceInstanceId:

type: string

dlServingPlmnRateCtl:

type: integer

minimum: 10

nullable: true

dnaiList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnai'

minItems: 1

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

roamingChargingProfile:

$ref: 'TS32291_Nchf_ConvergedCharging.yaml#/components/schemas/RoamingChargingProfile'

moExpDataCounter:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MoExpDataCounter'

vplmnQos:

$ref: '#/components/schemas/VplmnQos'

securityResult:

$ref: '#/components/schemas/SecurityResult'

upSecurityInfo:

$ref: '#/components/schemas/UpSecurityInfo'

amfNfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

guami:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Guami'

secondaryRatUsageDataReportContainer:

type: array

items:

$ref: '#/components/schemas/SecondaryRatUsageDataReportContainer'

minItems: 1

smPolicyNotifyInd:

type: boolean

enum:

- true

pcfUeCallbackInfo:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PcfUeCallbackInfo'

satelliteBackhaulCat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SatelliteBackhaulCategory'

maxIntegrityProtectedDataRateUl:

$ref: '#/components/schemas/MaxIntegrityProtectedDataRate'

maxIntegrityProtectedDataRateDl:

$ref: '#/components/schemas/MaxIntegrityProtectedDataRate'

upCnxState:

$ref: '#/components/schemas/UpCnxState'

ecsAddrConfigInfos:

type: array

items:

$ref: 'TS29503_Nudm_PP.yaml#/components/schemas/EcsAddrConfigInfo'

minItems: 1

hrsboInfo:

$ref: '#/components/schemas/HrsboInfoFromVplmn'

localOffloadMgtInfo:

$ref: '#/components/schemas/LocalOffloadingMgtInfoFromIsmf'

altSnssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

nsReplTerminInd:

type: boolean

enum:

- true

disasterRoamingInd:

type: boolean

enum:

- true

pduSetSupportInd:

type: boolean

ecnMarkingCongestionInfoStatus:

type: array

items:

$ref: '#/components/schemas/EcnMarkingCongestionInfoStatus'

minItems: 1

qosMonitoringPdSupported:

$ref: '#/components/schemas/QosMonitoringPdSupported'

qosMonitoringCongestionSupported:

$ref: '#/components/schemas/QosMonitoringCongestionSupported'

availBitRateMonSupported:

$ref: '#/components/schemas/AvailBitRateMonSupported'

udmGroupId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfGroupId'

amfResynchedInd:

type: boolean

enum:

- true

ddnFailureSubInfo:

$ref: '#/components/schemas/DdnFailureSubInfoTrafficDesc'

required:

- requestIndication

HsmfUpdatedData:

description: Data within Update Response from H-SMF, or from SMF to I-SMF

type: object

properties:

n1SmInfoToUe:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RefToBinaryData'

n4Info:

$ref: '#/components/schemas/N4Information'

n4InfoExt1:

$ref: '#/components/schemas/N4Information'

n4InfoExt2:

$ref: '#/components/schemas/N4Information'

dnaiList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnai'

minItems: 1

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

roamingChargingProfile:

$ref: 'TS32291_Nchf_ConvergedCharging.yaml#/components/schemas/RoamingChargingProfile'

homeProvidedChargingId:

type: string

pattern: '^(0|([1-9]{1}[0-9]{0,9}))$'

homeProvidedSmfChargingId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SmfChargingId'

upSecurity:

$ref: 'TS29571_CommonData.yaml#/components/schemas/UpSecurity'

maxIntegrityProtectedDataRateUl:

$ref: '#/components/schemas/MaxIntegrityProtectedDataRate'

maxIntegrityProtectedDataRateDl:

$ref: '#/components/schemas/MaxIntegrityProtectedDataRate'

ipv6MultiHomingInd:

type: boolean

default: false

qosFlowsSetupList:

type: array

items:

$ref: '#/components/schemas/QosFlowSetupItem'

minItems: 1

sessionAmbr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ambr'

epsPdnCnxInfo:

$ref: '#/components/schemas/EpsPdnCnxInfo'

epsBearerInfo:

type: array

items:

$ref: '#/components/schemas/EpsBearerInfo'

minItems: 1

pti:

$ref: '#/components/schemas/ProcedureTransactionId'

interPlmnApiRoot:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

intraPlmnApiRoot:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

hrsboInfo:

$ref: '#/components/schemas/HrsboInfoFromHplmn'

localOffloadMgtInfo:

$ref: '#/components/schemas/LocalOffloadingMgtInfoToIsmf'

ddnFailurePcrtInd:

type: boolean

enum:

- true

****************** Text skipped for Clarity *******************

VsmfUpdateData:

description: Data within Update Request towards V-SMF, or from SMF to I-SMF

type: object

properties:

requestIndication:

$ref: '#/components/schemas/RequestIndication'

sessionAmbr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ambr'

qosFlowsAddModRequestList:

type: array

items:

$ref: '#/components/schemas/QosFlowAddModifyRequestItem'

minItems: 1

qosFlowsRelRequestList:

type: array

items:

$ref: '#/components/schemas/QosFlowReleaseRequestItem'

minItems: 1

epsBearerInfo:

type: array

items:

$ref: '#/components/schemas/EpsBearerInfo'

minItems: 1

assignEbiList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Arp'

minItems: 1

revokeEbiList:

type: array

items:

$ref: '#/components/schemas/EpsBearerId'

minItems: 1

modifiedEbiList:

type: array

items:

$ref: '#/components/schemas/EbiArpMapping'

minItems: 1

pti:

$ref: '#/components/schemas/ProcedureTransactionId'

n1SmInfoToUe:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RefToBinaryData'

alwaysOnGranted:

type: boolean

default: false

hsmfPduSessionUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

newSmfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

newSmfPduSessionUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

cause:

$ref: '#/components/schemas/Cause'

n1smCause:

type: string

backOffTimer:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

maReleaseInd:

$ref: '#/components/schemas/MaReleaseIndication'

maAcceptedInd:

type: boolean

default: false

additionalCnTunnelInfo:

$ref: '#/components/schemas/TunnelInfo'

dnaiList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnai'

n4Info:

$ref: '#/components/schemas/N4Information'

n4InfoExt1:

$ref: '#/components/schemas/N4Information'

n4InfoExt2:

$ref: '#/components/schemas/N4Information'

n4InfoExt3:

$ref: '#/components/schemas/N4Information'

smallDataRateControlEnabled:

type: boolean

qosMonitoringInfo:

$ref: '#/components/schemas/QosMonitoringInfo'

epsPdnCnxInfo:

$ref: '#/components/schemas/EpsPdnCnxInfo'

n9DataForwardingInd:

type: boolean

default: false

n9InactivityTimer:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

hrsboInfo:

$ref: '#/components/schemas/HrsboInfoFromHplmn'

localOffloadMgtInfo:

$ref: '#/components/schemas/LocalOffloadingMgtInfoToIsmf'

altHplmnSnssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

pduSessionRetainInd:

type: boolean

enum:

- true

pendingUpdateInfoList:

type: array

items:

$ref: '#/components/schemas/PendingUpdateInfo'

netLocInfoReqInd:

type: boolean

enum:

- true

ddnFailurePcrtInd:

type: boolean

enum:

- true

required:

- requestIndication

****************** Text skipped for Clarity *******************

SmContext:

description: Complete SM Context

type: object

properties:

pduSessionId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PduSessionId'

dnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

selectedDnn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

sNssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

altSnssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

hplmnSnssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

altHplmnSnssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

pduSessionType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PduSessionType'

gpsi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

hSmfUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

smfUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

pduSessionRef:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

interPlmnApiRoot:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

intraPlmnApiRoot:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

pcfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

pcfGroupId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfGroupId'

pcfSetId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfSetId'

selMode:

$ref: '#/components/schemas/DnnSelectionMode'

udmGroupId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfGroupId'

routingIndicator:

type: string

hNwPubKeyId:

type: integer

sessionAmbr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ambr'

qosFlowsList:

type: array

items:

$ref: '#/components/schemas/QosFlowSetupItem'

minItems: 1

hSmfInstanceId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

smfInstanceId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

pduSessionSmfSetId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfSetId'

pduSessionSmfServiceSetId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfServiceSetId'

pduSessionSmfBinding:

$ref: 'TS29518_Namf_Communication.yaml#/components/schemas/SbiBindingLevel'

enablePauseCharging:

type: boolean

default: false

ueIpv4Address:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv4Addr'

ueIpv6Prefix:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Prefix'

epsPdnCnxInfo:

$ref: '#/components/schemas/EpsPdnCnxInfo'

epsBearerInfo:

type: array

items:

$ref: '#/components/schemas/EpsBearerInfo'

minItems: 1

maxIntegrityProtectedDataRate:

$ref: '#/components/schemas/MaxIntegrityProtectedDataRate'

maxIntegrityProtectedDataRateDl:

$ref: '#/components/schemas/MaxIntegrityProtectedDataRate'

alwaysOnGranted:

type: boolean

default: false

upSecurity:

$ref: 'TS29571_CommonData.yaml#/components/schemas/UpSecurity'

hSmfServiceInstanceId:

type: string

smfServiceInstanceId:

type: string

recoveryTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

forwardingInd:

type: boolean

default: false

psaTunnelInfo:

$ref: '#/components/schemas/TunnelInfo'

chargingId:

type: string

pattern: '^(0|([1-9]{1}[0-9]{0,9}))$'

smfChargingId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SmfChargingId'

chargingInfo:

$ref: 'TS29512_Npcf_SMPolicyControl.yaml#/components/schemas/ChargingInformation'

roamingChargingProfile:

$ref: 'TS32291_Nchf_ConvergedCharging.yaml#/components/schemas/RoamingChargingProfile'

nefExtBufSupportInd:

type: boolean

default: false

ipv6Index:

$ref: 'TS29519_Policy_Data.yaml#/components/schemas/IpIndex'

dnAaaAddress:

$ref: '#/components/schemas/IpAddress'

redundantPduSessionInfo:

$ref: '#/components/schemas/RedundantPduSessionInformation'

ranTunnelInfo:

$ref: '#/components/schemas/QosFlowTunnel'

addRanTunnelInfo:

type: array

items:

$ref: '#/components/schemas/QosFlowTunnel'

minItems: 1

redRanTunnelInfo:

$ref: '#/components/schemas/QosFlowTunnel'

addRedRanTunnelInfo:

type: array

items:

$ref: '#/components/schemas/QosFlowTunnel'

minItems: 1

nspuSupportInd:

type: boolean

smfBindingInfo:

type: string

satelliteBackhaulCat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SatelliteBackhaulCategory'

sscMode:

type: string

pattern: '^[0-7]$'

dlsetSupportInd:

type: boolean

n9fscSupportInd:

type: boolean

anchorSmfOauth2Required:

type: boolean

fullDnaiList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnai'

minItems: 1

hrsboAuthReqInd:

type: boolean

hDnsAddr:

$ref: '#/components/schemas/IpAddress'

hPlmnAddr:

$ref: '#/components/schemas/IpAddress'

vplmnOffloadingInfoList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VplmnOffloadingInfo'

minItems: 1

vplmnDlAmbr:

$ref: 'TS29571_CommonData.yaml#/components/schemas/VplmnDlAmbr'

offloadIds:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/OffloadIdentifier'

minItems: 1

targetDnai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnai'

trafficInfluInfo:

$ref: '#/components/schemas/TrafficInfluenceInfo'

pendingUpdateInfoList:

type: array

items:

$ref: '#/components/schemas/PendingUpdateInfo'

minItems: 1

easInfoToRefresh:

$ref: '#/components/schemas/EasInfoToRefresh'

localOffloadingMgtAllowedInd:

type: boolean

dnsAddr:

$ref: '#/components/schemas/IpAddress'

psaUpfAddr:

$ref: '#/components/schemas/IpAddress'

localOffloadingInfoList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/LocalOffloadingManagementInfo'

minItems: 1

ddnFailurePcrtInd:

type: boolean

enum:

- true

required:

- pduSessionId

- dnn

- sNssai

- pduSessionType

- sessionAmbr

- qosFlowsList

****************** Text skipped for Clarity *******************

AvailableBitrateMonitoringRequest:

description: Available Bitrate Monitoring Request

type: object

properties:

availBitrateReq:

$ref: '#/components/schemas/AvailableBitrateRequest'

availBitrateUlThrs:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate'

minItems: 1

availBitrateDlThrs:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate'

minItems: 1

required:

- availBitrateReq

DdnFailureSubInfoTrafficDesc:

description: DDN Failure Subscription Information with Traffic Descriptors.

type: object

properties:

ddnFailureSubsInd:

type: boolean

ddnFailureTrafficDescriptorList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DddTrafficDescriptor'

minItems: 1

#

# SIMPLE DATA TYPES

#

****************** Text skipped for Clarity *******************

* * * End of Changes * * * *