3GPP TSG-CT WG4 Meeting #131	C4-254213

# Sophia Antipolis, France; 13th – 17th October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.380 | CR | 0134 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | EPC/5GC health check and recovery upon IMS Terminating Call |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | IMS_RES-CT |  |  |  |  |  | Date: |  |  | 2025-09-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | TR 29.866 concludes the following for Key Issue #3: - For the failure check, the health check triggered by the P-CSCF described in Solution #6 and #9 are proposed to be specified in normative work. - For the failure restoration, Solution #9 is proposed to be specified in normative work for it is an enhancement to Solution #6. On one hand, this change request introduces Solution #9, which is able to detect upon IMS Terminating message failure that there is an ongoing PGW-U/UPF restoration procedure as described in TS 23.507 for EPC and TS 23.527 for 5GC, prioritizing the recovery of the affected session, so that next IMS terminating messages can benefit from this recovery. On the other hand, TR 29.866 was requesting to introduce this Solution#9 as an enhancement of solution 6. However, unfortunately, the introduction of Solution#6 in 23.380 is not considering current PCF/PCRF behavior in other similar procedures defined in TS 23.380 (clause 5.8.5.2) and CT3 Technical Specifications like TS 29.513 (e.g. clauses 5.5.3, 5.5.9 or B.9) and TS 29.213 (e.g. clauses 4.3, B.1, B.2, B.5, B.7 or L.2). In all these procedures the Rx AAA/ Npcf_PolicyAuthorization message is answered before any Gx RAA/ Npcf_SmPolicyControl message is executed. The solution currently included in chapter 7.2.2 of TS 23.380 requires PCRF/PCF to wait for the reception of a Gx RAA/ Npcf_SmPolicyControl message to send an Rx AAA/ Npcf_PolicyAuthorization message to P-CSCF, which is not following current PCF/PCRF behavior when it comes to Rx/ Npcf_PolicyAuthorization and Gx/Npcf_SmPolicyControl interaction. Finally, solution 9 in the Technical Report is following the behaviour already used in TS 29.513 and TS 29.213 where Rx AAA/ Npcf_PolicyAuthorization messages are answered before Gx RAA/ Npcf_SmPolicyControl message is executed. Hence, solution 9 cannot be defined as an extension of solution 6 since they follow different Rx/Gx interaction approaches in PCF/PCRF. This contradiction was already pointed out by LS response C3-250657 to C4-244485. For those reasons, this change request is proposing to change chapter 7.2.2 of TS 23.380 to follow current CT3 approach so that both SMF/PGW-C restart (solution 6 in TR 29.866) and UPF/PGW-U (solution 9 in TR 29.866) are handled by PCF/PCRF in the same way, so that the second is extending the first one. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update chapter 7.2.2 of TS 23.380 to follow current CT3 approach when it comes to managing Rx AAA/ Npcf_PolicyAuthorization and Gx/Npcf_SmPolicyControl procedures independently. Include both SMF and UPF restart restoration procedures following these approach so that UPF restoration handling is an extension of SMF restoration handling. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Impossible to have a solution where SMF restart (solution 6 in TR 29.866) and UPF restart procedures (solution 9 in TR 29.866) handling in PCF/PCRF can coexist. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.1, 7.2.2, 7.2.2.1, 7.2.2.2, 7.2.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.007: "Restoration procedures".

[3]	3GPP TS 29.228: "IP Multimedia (IM) Subsystem Cx and Dx interfaces; Signalling flows and message contents".

[4]	3GPP TS 24.008: "Mobile radio interface Layer 3 specification".

[5]	3GPP TS 29.213: "Policy and charging control signalling flows and Quality of Service (QoS) parameter mapping".

[6]	3GPP TS 29.212: "Policy and Charging Control (PCC); Reference points".

[7]	3GPP TS 29.214: "Policy and Charging Control over Rx reference point".

[8]	3GPP TS 29.060: "General Packet Radio Service (GPRS); GPRS Tunnelling Protocol (GTP) across the Gn and Gp interface".

[9]	3GPP TS 29.061: "Interworking between the Public Land Mobile Network (PLMN) supporting packet based services and Packet Data Networks (PDN)".

[10]	3GPP TS 29.274: "3GPP Evolved Packet System. Evolved GPRS Tunnelling Protocol for EPS (GTPv2)".

[11]	IETF RFC 3361: "Dynamic Host Configuration Protocol (DHCP-for-IPv4) Option for Session Initiation Protocol (SIP) Servers".

[12]	IETF RFC 1034: "Session Initiation Protocol (SIP): Locating SIP Servers".

[13]	IETF RFC 3319: "Dynamic Host Configuration Protocol (DHCPv6) Options for Session Initiation Protocol (SIP) Servers".

[14]	IETF RFC 6223: "Indication of Support for Keep-Alive".

[15]	3GPP TS 29.275: "Proxy Mobile IPv6 (PMIPv6) based Mobility and Tunnelling protocols; Stage 3".

[16]	IETF RFC 7077: "Update Notifications for Proxy Mobile IPv6".

[17]	3GPP TS 23.401: "GPRS Enhancements for E-UTRAN Access".

[18]	IETF RFC 3261: "SIP: Session Initiation Protocol".

[19]	3GPP TS 24.229: "IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

[20]	3GPP TS 23.060: "General Packet Radio Service (GPRS); Service description; Stage 2".

[21]	GSMA PRD IR.65: "IMS Roaming and Interworking Guidelines, version 14.0".

[22]	3GPP TS 24.244: "Wireless LAN control plane protocol for trusted WLAN access to EPC; Stage 3".

[23]	IETF RFC 7296: "Internet Key Exchange Protocol Version 2 (IKEv2)".

[24]	3GPP TS 29.273: "Evolved Packet System (EPS); 3GPP EPS AAA interfaces".

[25]	Void

[26]	IETF RFC 5996 (September 2010): "Internet Key Exchange Protocol Version 2 (IKEv2)".

[27]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[28]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[29]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[30]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System; Stage 2".

[31]	3GPP TS 29.503: "5G System; Unified Data Management Services; Stage 3".

[32]	3GPP TS 29.512: "5G System; Session Management Policy Control Service; Stage 3".

[33]	3GPP TS 23.632: "User Data Interworking, Coexistence and Migration; Stage 2".

[34]	3GPP TS 24.237: "IP Multimedia Subsystem (IMS) Service Continuity; Stage 3".

[35]	3GPP TS 29.513: "5G System; Policy and Charging Control signalling flows and QoS parameter mapping; Stage 3".

[36]	3GPP TS 23.527: "5G System; Restoration Procedures".

# * * * Next Change * * * *

### 7.2.1	Introduction

When the failure of EPC/5GC NF, e.g. PGW-C, PGW-U SMF or UPF is detected during the IMS terminating procedure, the following clauses specify the procedures to speed up the restoration of the IMS terminating session.

Failure may happen to EPC/5GC NF during the IMS terminating procedure. In such case, the P-CSCF may detect such EPC/5GC NF failure as follows:

-	P-CSCF requires PCRF/PCF to perform EPC/5GC NF (e.g. PGW-U, PGW-C, UPF or SMF) failure detection and recovery;

-	P-CSCF receives EPC/5GC NF failure notification from PCRF/PCF via Rx Abort-Session-Request (ASR) or Npcf_PolicyAuthorization_Notify message indicating service unavailability;

-	P-CSCF detects PGW-U/UPF failure by detecting transmission failure of IMS message to terminating UE.

NOTE:	When the P-CSCF forwards IMS message to the terminating UE while the user plane nodes (e.g. PGW-U or UPF) of EPC/5GC fail, the P-CSCF can receive transport failure for this message, e.g. an ICMP Destination Unreachable message.

# * * * Next Change * * * *

### 7.2.2	P-CSCF triggering EPC/5GC NF failure checking and recovery

#### 7.2.2.1	Introduction

The P-CSCF activation of EPC/5GC health monitoring and failure recovery is an optional mechanism. The EPC/5GC health monitoring shall be executed when the terminating UE does not respond to the initial SIP message. If the health check monitoring detects a failure in an EPC/5GC NF (e.g. PGW-U, PGW-C, UPF or SMF), the mechanism shall trigger existing restoration procedures to speed up the recovery of these EPC/5GC NFs.

In 5GC scenarios, the following requirements shall be supported by 5GC Network Functions for the procedure specified in this clause:

-	P-CSCF executes a monitoring mechanism to check 5GC health upon initial SIP message timeout by invoking a specific Npcf_PolicyAuthorization Create/Update service operation in PCF.

-	P-CSCF triggers existing "HSS-based P-CSCF restoration for 3GPP access" mechanism described in clause 5.4 by sending a 503 Service Unavailable response with the P-CSCF Restoration indication to the I-/S-CSCF in the following situations:

-	After receiving a Npcf_PolicyAuthorization Create/Update answer stating there is no valid session context in PCF.

-	Upon reception of a specific immediate Npcf_PolicyAuthorization Notify stating there is no valid session in SMF and PDU Session restoration procedures were not triggered.

-	P-CSCF supports sending a 500 Internal Server Error response with the PDU Session restoration triggered indication to the I-/S-CSCF, after receiving a specific Npcf_PolicyAuthorization Notify from PCF stating PDU Session restoration procedures were already triggered.

-	PCF supports the reception of a specific Npcf_PolicyAuthorization Create/Update service operation to invoke monitoring mechanism to check 5GC health.

-	PCF supports the following requirements when requested to execute monitoring mechanism to check 5GC health:

-	It returns a Npcf_PolicyAuthorization Create/Update answer to P-CSCF stating there is no valid session when corresponding session context is not found in PCF.

-	If a valid session context was found, it sends a specific Npcf_SmPolicyControl Update Notification operation to trigger 5GC health monitoring in SMF.

-	When receiving a Npcf_SmPolicyControl Update Notification answer from SMF stating no valid IP-CAN/PDU, it forwards its content to P-CSCF in a Npcf_PolicyAuthorization Notify service operation.

-	When it is requested to execute 5GC health monitoring, SMF supports to return a Npcf_SmPolicyControl Update Notification answer stating if there is no valid IP-CAN/PDU session and whether PDU Session restoration procedures were already triggered or not. In case PDU Session restoration procedures were already triggered in SMF, as it is specified in clause 17.1b in 3GPP TS 23.007 [2] for EPC scenario and in clause 4.3.2 in 3GPP TS 23.527 [36] for 5GC scenario, the recovery of the affected sessions is prioritized.

-	S-CSCF supports to start the HSS based P-CSCF Restoration procedure specified in clause 5.4.2.1 in EPC network or the UDM/HSS based P-CSCF Restoration procedure via AMF specified in clause 5.8.4.3, after receiving a 503 Service Unavailable response with the P-CSCF Restoration indication.

For EPC scenario P-CSCF and PCRF shall support specific Rx Authorization-Authentication (AAR/AAA) and Rx Re-Auth (RAR/RAA) Diameter messages instead of Npcf_PolicyAuthorization service operations, and PCRF and PGW-C shall support specific Gx Re-Auth (RAR/RAA) Diameter messages instead of Npcf_SmPolicyControl service operations.

#### 7.2.2.2	EPC/5GC NF failure checking and recovery after SMF/PGW-C restart

Figure 7.2.2.2-1 illustrates the execution of 5GC health monitoring and failure recovery upon IMS terminating session not answered after SMF restart.

Figure 7.2.4.2-1: 5GC health monitoring and failure recovery upon IMS terminating session not answered after SMF restart

0	Upon SMF restart, the UPF shall delete all session contexts affected by the restart as it is specified in clause 4.4.2 in 3GPP TS 23.527 [38]. Session context will be restored upon UE activity; hence UE-B might not have restored the session upon reception of IMS terminating session. For EPC scenario, PGW-U shall delete all session contexts affected by the PGW-C restart as it is specified in clause 17.1a in 3GPP TS 23.007 [2]. On the other hand, PCF/PCRF may also detect PGW-C/SMF restart and invalidate all related session contexts.

1-3.	The I-/S-CSCF in the terminating network receives an incoming INVITE request and routes it to the P-CSCF serving UE B. Then the P-CSCF routes the INVITE request to UE B.

4-5.	The P-CSCF does not receive any response from UE B after some time out, so the P-CSCF determines that the SIP signalling has already attempted the corresponding re-transmissions or retrials. Therefore, the P-CSCF invokes a specific Npcf_PolicyAuthorization_Create service operation on the PCF. This action initiates the 5GC health monitoring and failure recovery in PCF.

6-7.	Upon receiving the Npcf_PolicyAuthorization_Create request, if the PCF has identified that the session exists for UE-B, it successfully answers to P-CSCF.

7a.	If there is no session, an error is returned stating no session was found in PCF to let P-CSCF trigger P-CSCF Restoration procedures in step 13.

8.	The PCF sends a specific Npcf_SMPolicyControl_UpdateNotify service request to the SMF. This action initiates the 5GC health monitoring and failure recovery in SMF.

9-10.	If the SMF finds that there is no PDU session context for UE B, it returns an Npcf_SMPolicyControl_UpdateNotify answer to the PCF stating no session was found in SMF.

11-12.	The PCF sends a Npcf_PolicyAuthorization_Notify to the P-CSCF to inform about no session was found in SMF.

13-14.	The P-CSCF sends a 503 Service Unavailable response with the P-CSCF Restoration indication to the I-/S-CSCF. Then the I-/S-CSCF starts the HSS based P-CSCF Restoration procedure in EPC network specified in clause 5.4.2.1, or the UDM/HSS based P-CSCF Restoration procedure via AMF specified in clause 5.8.4.3 by sending a Cx SAR to the HSS including the P-CSCF Restoration indication.

UE B will re-attach/re-registered to the EPC/5GC and register into the IMS quickly as specified in the HSS based or UDM/HSS based P-CSCF restoration procedure. Then UE B will receive the next incoming session for it has re-connected to the EPC/5GC network successfully. After the EPS bearer/QoS flow for IMS signalling is established, UE A initiates the initial registration procedure which is specified in 3GPP TS 23.228 [27].

In EPC scenario P-CSCF and PCRF shall send specific Rx Authorization-Authentication (AAR/AAA) and Rx Re-Auth (RAR/RAA) Diameter messages instead of Npcf_PolicyAuthorization service operations, and PCRF and PGW-C shall send specific Gx Re-Auth (RAR/RAA) Diameter messages instead of Npcf_SmPolicyControl service operations.

#### 7.2.2.3	EPC/5GC NF failure checking and recovery after UPF/PGW-U restart

Figure 7.2.4.3-1 illustrates the execution of 5GC health monitoring and failure recovery upon IMS terminating session not answered after UPF restart.

Figure 7.2.2.3-1: 5GC health monitoring and failure recovery upon IMS terminating session not answered after UPF restart

0	Upon UPF restart, the SMF shall restore the session contexts affected by the restart as it is specified in clause 4.3.2 in 3GPP TS 23.527 [38]. For EPC scenario, PGW-C shall restore the session contexts affected by the PGW-C restart as it is specified in clause 17.1b in 3GPP TS 23.007 [2].

0b.	UE-B sessions might not have been restored yet when receiving IMS terminating session.

1-3.	The I-/S-CSCF in the terminating network receives an incoming INVITE request and routes it to the P-CSCF serving UE B. Then the P-CSCF routes the INVITE request to UE B.

4-5.	The P-CSCF does not receive any response from UE B after some time out, so the P-CSCF determines that the SIP signalling has already attempted the corresponding re-transmissions or retrials. Therefore, the P-CSCF invokes a specific Npcf_PolicyAuthorization_Create service operation on the PCF. This action initiates the 5GC health monitoring and failure recovery in PCF.

6-7.	Upon receiving the Npcf_PolicyAuthorization_Create request, if the PCF has identified that the PDU session exists, it successfully answers to P-CSCF.

8.	The PCF sends a specific Npcf_SMPolicyControl_UpdateNotify service request to the SMF. This action initiates the 5GC health monitoring and failure recovery in SMF.

9-10.	If the SMF finds that there is no PDU session context for UE B, it returns a Npcf_SMPolicyControl_UpdateNotify answer to the PCF stating no session was found in SMF. As there is an ongoing UPF restoration procedure, in the answer, SMF includes an indication stating that PDU Session restoration is already triggered.

10a.	SMF prioritizes the triggering of the PDU session restoration procedure for UE-B (see clause 5.8.2.4 "PDU Session Release with Reactivation Procedure").

10b.	After the PDU Session reactivation is completed, the UE-B may need to re-register on the IMS network.

11-12.	The PCF sends a Npcf_PolicyAuthorization_Notify to the P-CSCF to inform about no session was found in SMF including PDU Session restoration triggered indication.

13.	The P-CSCF sends a 500 Internal Server Error response with the PDU Session restoration triggered indication to the I-/S-CSCF. Then the I-/S-CSCF forwards the error to the originating party.

Note: There is no need to trigger P-CSCF Restoration procedure since IMS Session will be eventually restored.

In EPC scenario P-CSCF and PCRF shall send specific Rx Authorization-Authentication (AAR/AAA) and Rx Re-Auth (RAR/RAA) Diameter messages instead of Npcf_PolicyAuthorization service operations, and PCRF and PGW-C shall send specific Gx Re-Auth (RAR/RAA) Diameter messages instead of Npcf_SmPolicyControl service operations.

# * * * End of Changes * * * *