3GPP TSG-CT WG4 Meeting #131	C4-254220

# Sophia Antipolis, France; 13th – 17th October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.003 | CR | 0727 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | AIoT Filtering information corrections |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AmbientIoT-CT |  |  |  |  |  | Date: |  |  | 2025-09-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | TS 23.369 clause 5.8 specifies that there may be multiple filtering elements corresponding to the component Identification Information of the AIoT Device Permanent Identifier. ## “5.8 Filtering Information [..] When the Component Type Information of the filtering element indicates an Identification Information component, the Offset is always included, i.e. the component from the start location indicated by the corresponding Offset is compared with the bitstring. There may be multiple filtering elements corresponding to the component Identification Information of the AIoT Device Permanent Identifier. [..]” The current definition in clause 31.3 does not clearly allow multiple filtering elements of component type Identification Information. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Propose to specify that multiple filtering elements of component type Identification Information is supported. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambient IoT stage 3 specification not aligned with stage 2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 31.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# *** First Change ***

## 31.3	Structure of Filtering Information

The Filtering Information is used to identify or filter multiple AIoT Devices, it includes a list of filtering elements as specified in clause 5.8 of 3GPP TS 23.369 [152], with a maximum length of 256 bits, the structure is shown in Figure 31.3.1.

Figure 31.3.1: Structure of Filtering Information

The Filtering Information includes at most one filtering element within the Filtering Information for the following component of AIoT Device Permanent Identifier (i.e. ID Type, PLMN ID, NID or Third Party Identifier). The indicated filtering element within a Filtering Information should be matched and shall be coded as the binary representation of their value, it is composed of the following parts:

1)	Count of Filtering Element shall have a fixed length of 4 bits to indicate the number of Flitering Element which included in a Filtering Information

2)	The Filtering Element is composed of the following parts, the structure is shown in Figure 31.3.2:

Figure 31.3.2: Structure of Filtering Element

1)	Filtering Type indicates the AIoT Device Permanent Identifier is used to match the bitstring, with the following mapping:

-	000	indicate the Filtering Type for the filtering element is ID Type;

-	001	indicate the Filtering Type for the filtering element is PLMN ID;

-	010	indicate the Filtering Type for the filtering element is NID;

-	011	indicate the Filtering Type for the filtering element is Third Party Identifier;

-	100	indicate the Filtering Type for the filtering element is Identification Information;

-	101	reserved;

-	110	reserved;

-	111	reserved.

2)	ID Type Filter, this field is present only if the Filtering Type is set to 000. The structure is show in Figure 31.3.3.

Figure 31.3.3: Structure of ID Type Filter

-	Offset consisting of 3 bits. It represents the position of the ID Type's bitstring to be compared. Value 0 means the starting position is 1, and value 7 means the starting position is 8.

-	Length consisting of 3 bits. It represents the length of value of the bitstring to be compared.

-	Component Value consisting of 1 - 8 bits. It represents the component value of the bitstring to be compared.

NOTE:	It is suggested that a Filtering Information does not only include ID Type Filter Element as Filter Information.

3)	Filtering information for PLMN ID shall have a fixed length of 24 bits, this field is present only if the Filtering Type is set to 001.

4)	Filtering information for NID shall have a fixed length of 44 bits, this field is present only if the Filtering Type is set to 010.

5)	Filtering information for Third Party Identifier shall have a fixed length of 24 bits, this field is present only if the Filtering Type is set to 011.

6)	Identification Information Filter, there should be one Identification Information Filter within a Filtering Element, this field is present only if the Filtering Type is set to 100.

It is used to filter the target devices with bitstring of Identification Information equal to a specific value.

Multiple filtering elements with Filtering Type set to 100 (Identification Information) shall be supported within the Filtering Information. Each filtering element may contain a distinct Offset, Length, and Component Value. All such filtering elements shall match the Identification Information for the Filtering Information to be considered a match.

The structure is shown in Figure 31.3.2.

Figure 31.3.2: Structure of Identification Information Filter

-	Offset consisting of 7 bits. It represents the position of the Identification Information's bitstring to be compared. Value 0 means the starting position is 1, and value 127 means the starting position is 128.

-	Length consisting of 7 bits. It represents the length of value of the bitstring to be compared.

-	Component Value consisting of 1 - 128 bits. It represents the component value of the bitstring to be compared.

# *** End of Changes ***