3GPP TSG-CT WG4 Meeting #154	C4-254367

Sophia Antipolis, France; 13th – 17th October 2025	revision of C4-254104

Source:	NTT DOCOMO, Qualcomm Incorporated

Title:	Pseudo-CR on P-CSCF Failure recovery with UAC Access Category 9

Spec:	3GPP TR 29.867 v0.2.0

Agenda item:	21.4

Document for:	Approval

1. Introduction

<Introduction part (optional)>

2. Reason for Change

Solution of the Technical Report is missing.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 29.867 v0.2.0.

* * * First Change * * * *

6.X	Solution #X: Access restriction with UAC Access Category 9

6.X.1	Description

This solution is proposed to address Key Issue #1: "Preventing UE SIP Registration attempts during P-CSCF Failures." In KI#1, UEs continue to initiate the SIP registration request when the IMS is unable to respond, resulting in sustained traffic congestion. Therefore, it is required that the solution prevents repeated registration attempts by UEs under total P-CSCF failure conditions.

To address the issue, this proposal solution proposes delegating by using Unified Access Control (UAC, defined in 3GPP TS 24.501 [6]. UAC has the access categories and access identifiers to control the restriction of UE accessing more flexible.

For this KI#1, this solution uses the access category 9 which is reserved for MO IMS registration related signaling and defined as “Includes IMS registration related signaling, e.g. IMS initial registration, re-registration, and subscription refresh.”. Access control may be applied to communications from UEs using access category 9, and the barring rate may be gradually relaxed over time to avoid concentrated load.

Since KI#1 indicates the situation of a severe IMS failure, i.e. all P-CSCF instance fail and it triggers the total IMS service down, affecting for whole PLMN is allowed. In addition, since access category 9 targets only IMS registration-related signalling, barring can be applied without impacting other types of communications.

6.X.1.1	Manual recovery

The manual recovery flow of P-CSCF failure is based on the regular operation flow of UAC barring.

- Upon detection of a P-CSCF failure, the operator initiates UAC barring for UEs of access category 9.

- UAC barring information is immediately broadcast to all UEs via SIB1.

- Based on the configured barring rate, IMS initial registration, re-registration, and subscription refresh requests from affected UEs are suppressed prior to transmission. Affected UEs cannot retry until the barring timer is ended.

- When the situation changes, e.g. P-CSCF recovery, the operator updates the barring rate to gradually increase the number of UEs permitted to access IMS services.

Since UAC restricts on SIB1, ongoing transactions	are not affected and continue / retry to connect with established NAS session.

6.X.1.2	Automatic recovery

The operator may automate UAC recovery procedures based on operator’s policy. The detection of P-CSCF failure may be performed based on KPI data, for example, alarms, increased failure rates of IMS traffic or status monitor on surrounding node e.g. PCRF/S-CSCF/PGW/UPF.

6.X.2	Impacts on services, entities, and interfaces

No impact.

6.X.3	Pros

The pros of the solution:

- This solution only consists of the existing features.

- IMS registration is immediately restricted once UAC barring is configured to avoid burst load, based on operator configuration.

- The solution is compatible with legacy UEs.

6.X.4	Cons

The cons of the solution:

- Restoration or retry requests from already connected UEs cannot be prevented, resulting in remaining load.

- In case of roaming and network sharing including MVNO scenario, SLA or OAM level agreement may be needed.

* * * End of Changes * * * *