3GPP TSG RAN Meeting #110	RP-253074

Baltimore, USA, December 8-11, 2025

Agenda Item:	8.2.1.4

Source:	Futurewei

Title:	Massive communication needs a minimum peak data rate of at least 10Mbps

Document for:	Discussion and Decision

# Introduction

In RAN#109 the following was agreed [1] for Massive Communication in TR38.914:

### 5.4.3	Massive Communication (IoT)

The 6GR and 6G RAN architecture shall support the following minimum requirements for Massive Communication (IoT):

- 6G Massive Communication (IoT) shall be supported for FR1.

  - 6GR should have a common/scalable design that supports the above usage scenario in addition to eMBB

    - Prioritize 6GR design for eMBB

  - The above usage scenario should not overlap with Ambient IoT and NB-IoT

- [PHY or MAC] [minimum] peak data rate is [TBD] Mbps in DL and [TBD] Mbps in UL for lowest-tier device.

This contribution discusses the data rate TBD and will show that the minimum peak data rate needs to be at least 10Mbps.

# Discussion

In RAN#109, the peak data rate for massive communication (IoT) was discussed without agreement. Most proposals were on the order of 10Mbps but some wanted lower (such as 1Mbps) and some mentioned higher. The argument for 10Mbps was the support of applications that use LTE cat 1bis or NR Rel-18 eRedCap, as well as eMTC. The argument for 1Mbps was to have a device type only for the optimization of eMTC applications. The argument for more than 10Mbps was either to (a) also support applications that use Rel-17 RedCap (e.g., industrial IoT, wearables, video surveillance) or (b) to make 6G more advanced than previous generations for IoT support as well as eMBB.

In a sense, no data rate requirement is needed for massive communication. The ITU requirement is only about connection density, which was previously shown to be satisfied by NB-IoT, eMTC, and even normal NR. However if a data rate requirement is included for massive communication IoT services, it should at least cover the typical 10Mbps data rate that is widely (and successfully) deployed with cat 1bis, as well as eMTC. That was a reason why 10Mbps was set as the data rate for NR Rel-18 eRedCap. Setting the target at 1Mbps is far too low in our view. In addition to not being an ambitious target for 6G, developing eMTC-like and cat 1bis-like device types would fragment the IoT market for little in the way of cost benefits. The 6GR low-tier device type is going to achieve cost-reduction from economies of scale, not from a custom (non-common/non-scalable) design, and a device type with a bandwidth of 5, 10, or 20MHz can support 10Mbps easily.

An argument can also be made to consider a peak data rate of around 50+Mbps, similar to Rel-17 RedCap. In RAN#109 we agreed [2]:

Proposal 3: To investigate further:

  - Motivations/justifications behind the proposed diverse device types, which should be a limited set

  - Whether/how to have one or more device types for eMBB or 6G IoT

  - Whether/how to have other device types for, e.g., XR/immersive experiences, FWA, VUE, wearables/RedCap, sensing, NTN-specific, AI agents, collaborative robots, etc.

  - Whether/how to explicitly standardize device types

  - Ensuring forward compatibility

  - Minimizing/avoiding potential market fragmentation

Note: the terminology “device type” is subject to further discussion and possible refinement.

If we are truly to have a “limited set” of device types for “minimizing/avoiding potential market fragmentation”, a data rate of around 50Mbps could cover many of the Rel-17 RedCap applications, as well as eMTC/cat 1bis/Rel-18 eRedCap.  A 20MHz bandwidth, as used by RedCap, can support 50+Mbps. The discussion of selecting 3, 5, 10, or 20MHz bandwidth for the low-tier device type is still ongoing in RAN1, but there are compelling arguments presented that 20MHz would offer more applications with more economies of scale / less market fragmentation.

Observation 1: The 6GR lower-tier device type is going to achieve cost-reduction from economies of scale, not from a custom (non-common/non-scalable) design.

Observation 2: A lowest-tier device with a larger peak data rate will be able to meet the data rate requirements of more services/applications, without the need to introduce more device types.

Proposal 1: Massive communication needs a minimum peak data rate of at least 10Mbps.

Proposal 2: Around 50Mbps peak data rate can be considered to further limit the number of device types.

# Conclusion

The following observations and proposals were discussed in this contribution.

Observation 1: The 6GR lower-tier device type is going to achieve cost-reduction from economies of scale, not from a custom (non-common/non-scalable) design.

Observation 2: A lowest-tier device with a larger peak data rate will be able to meet the data rate requirements of more services/applications, without the need to introduce more device types.

Proposal 1: Massive communication needs a minimum peak data rate of at least 10Mbps.

Proposal 2: Around 50Mbps peak data rate can be considered to further limit the number of device types.

# References

RP-252960, “pCR for 38.914 on Requirements of new and existing services for 6G.”

RP-252873, “Summary of 6G Diverse Device Types (including Minimum UE Bandwidth).”