3GPP TSG RAN Meeting #110								   								  RP-253199

Baltimore, USA, December 8-11, 2025

Title: 	Discussion on diverse device types

Source: 	ZTE Corporation, Sanechips

Agenda item:	8.2.1.3.2

Document for:	Discussion and decision

- Introduction

In RAN#109, the RAN-level discussion on the “Device Type” has been conducted, resulting in initial agreements [1]. In this contribution, further insights on how to define the diverse device types along with initial parameter sets, smallest maximum supported RF and BB UE BW are provided.

- Discussion on the Diverse device types

General categorization on the device types

In RAN#109 meeting, whether and how to introduce the device types have been conducted, achieving the following progress.

Proposal 3: To investigate further: - Motivations/justifications behind the proposed diverse device types, which should be a limited set - Whether/how to have one or more device types for eMBB or 6G IoT - Whether/how to have other device types for, e.g., XR/immersive experiences, FWA, VUE, wearables/RedCap, sensing, NTN-specific, AI agents, collaborative robots, etc. - Whether/how to explicitly standardize device types - Ensuring forward compatibility - Minimizing/avoiding potential market fragmentation Note: the terminology “device type” is subject to further discussion and possible refinement.

Regarding the necessity, in our view, as shown in Figure 2-1, various devices (e.g., Redcap, XR) are considered with different use cases and hardware capabilities. For each type of device, the flexible UE capability framework has been introduced in 5G. However, in real commercialization, due to complexity and challenges of device development, network planning and various roadmaps from different stakeholders, a large number of candidate capabilities were not implemented.

Figure 2-1. Device types and use cases in deployed in 5G and suggested for 6G

For 6G, since it aims to support the agreed existing and new services, e.g., immersive communication, HRLLC and massive connection, along with the consideration on scalable and forward-compatible design. Then, as shown above, introduction of device types with well-balanced design across more factors, e.g., hardware restriction, market scale/fragmentation and impacts on technical design (e.g., initial access and common channel) are still needed. Meanwhile, we prefer to focus on some typical categories since it would be challenging and inefficient to define many different types with fine granularity . For example,

- Since eMBB service is still with a high priority, normal UEs, e.g. smartphone UEs, should be definitely considered as one direction.

- Additionally, in robotics applications such as industrial automation, autonomous systems and collaborative robotics, the requirements for data rate, latency and reliability become increasingly higher than normal UEs. Furthermore, CPE and FWA are essential components in communication networks, primarily used to provide wireless broadband connectivity. In these cases, an advanced UE type (other than normal UE) should be considered to meet more advanced requirements e.g. joint requirements of high data rate and low latency. For this advanced device type, the form factor is not a limiting factor, allowing for inclusion of advanced features such higher modulation orders, more Tx/Rx chains to enhance performance. The motivation of defining an advanced UE type is to setup a common set of advanced features to create a good size of economy scale.

- For other cases with lower targets, e.g., IoT, Redcap-like application, the one with scalable design is considered, e.g. scalable UE, aiming to lower complexity and cost.

In addition, for basic capability sets for different UE types, e.g., scalable UEs and normal UEs, it should be clearly differentiated to avoid market fragmentation.

Proposal 1: For 6GR, at least the following three type of devices are recommended as the target.

- Scalable UE: Designed for the reduced capability scenarios, offering lower complexity and cost.

- Normal UE: Providing improved performance compared to 5G eMBB.

- Advanced UE: Designed for the demand of high data rate, low latency scenario, with relaxed restriction of form factor and higher capability than normal UE.

Views on configuration of each device type

Views on the smallest maximum UE bandwidth

Based on the device types mentioned above, additional consideration on the corresponding requirements including impacts on the system design should be further discussed based on the agreement achieved in RAN#109 as follows.

Proposal 1: Endorse the following two RAN1 agreements (with the clarification that the 2nd agreement is applicable to FR1). Companies are invited to bring contributions regarding the minimum spectrum allocation in RAN#110, while RAN1 is requested to continue the study on both the minimum spectrum allocation and the smallest maximum UE bandwidth from the 6GR design perspective. Revisit in RAN#110. Agreement - For the study of RAN1 6GR design, consider the minimum spectrum allocation in which 6G can operate, subject to further discussion and confirmation in RAN. - Note: RAN4 involvement is necessary. Agreement - Study the following smallest maximum supported RF and BB UE BW without spectrum aggregation for at least one low-tier device type supported by 6GR framework from physical layer perspective, subject to further discussion and confirmation in RAN - Opt1: 3MHz - Opt2: 5MHz - Opt3: 10MHz - Opt4: 20MHz - FFS: the UL bandwidth may be different to the DL bandwidth - FFS: the bandwidth value may be different for different SCS, duplex modes, and bands. - FFS: whether RF and BB UE BW are same or different

Furthermore, in RAN1#123, the following agreement about the smallest maximum bandwidth has been achieved [2].

Agreement - For the smallest maximum supported RF and BB UE BW without spectrum aggregation for at least one low-tier device type supported by 6GR framework, from physical layer perspective, RAN1 to consider at least - Overall device complexity - Overall system performance impact - Energy efficiency for both BS and UE - Aim at a single common signals/channels design in idle mode and initial access for diverse device types, as well as meeting mobile broadband service requirements as high priority

Table 2-2. Comparison among different smallest maximum bandwidth

|  | Option 1: 3MHz | Option 2: 5MHz | Option 3: 10MHz | Option 4:20MHz |
| --- | --- | --- | --- | --- |
| System performance | SSB: redesign SSB with more symbols SIB1 PDCCH needs to be redesigned within 3MHz SIB1 PDSCH: adapted to 3MHz | Common SSB design based on 5MHz grid SIB1 PDCCH: 24RB CORESET can be applied within 5MHz SIB1 PDSCH: adapted to 5MHz | Common SSB design based on 5MHz grid SIB1 PDCCH: 24 or 48RB CORESET can be applied within 10MHz SIB1 PDSCH: adapted to 10MHz | Common signal design can be applied within 20MHz |
| Cost/complexity Note 1 | 83%~87% | 85%~89% | 90%~94% | 100% |
| L2 buffer size | Low | Med-low | Med-high | High |
| UE power consumption Note 2 | 68% | 72% | 81% | 100% |
| Note 1: Based on complexity/cost calculation in TR 38.865 compared to 20MHz Note 2: Based on formula X MHz = 0.4 + 0.6 * (X - 20) / 80 in TR 38.840 |  |  |  |  |

The comparison among Option 1~4 are listed in Table 2-2. It can be found that option 1 has significant system performance impact while offering minimal additional benefits, which should be deprioritized. Option 2 and option 3 have similar impact on common signal design and impose fewer restrictions on normal UE. Furthermore, reduction of the bandwidth from 20MHz to 10MHz and 5MHz achieves more than 5% and 10% complexity reduction, respectively. Therefore, 5MHz can be considered as starting point for a tradeoff among complexity, system performance and UE power consumption.

Proposal 2: For 6GR design, as a starting point, the smallest UE RF/BB bandwidth of diverse device types is recommended as 5MHz with 15KHz.

Views on other parameters/factors for each device type

Regarding other potential factor/parameters for consideration, based on the list agreed in RAN#109, to ensure sufficient performance gap comparing with 5G and among device types (e.g., Advanced UE vs. Normal UE vs. Scalable UE), the basic capability set, including bandwidth, Rx/Tx, modulation order, can be further considered as shown in Table 2-3.

Proposal 4: In terms of diverse device types, study further: - Possible parameters/factors, e.g.: - Number of Tx antennas/chains - Number of Rx antennas/chains - Power classes - Maximum UE bandwidth (DL/UL) - Peak data rate (DL/UL) - Maximum MIMO layers (DL/UL) - Duplex mode - Max modulation order (DL/UL) - CA/spectrum aggregation (DL/UL) - UE processing capabilities - Coverage - Energy efficiency - Mobility/speed - Sensing - AI Note: some of the above parameters/factors may be related with form factor Note: aim to have a focused/limited set of parameters/factors for a device type The value(s) for the identified parameters for a device type

For example, for a low-end RedCap UE targeting 10Mbps peak data rate, a 1-Rx configuration with up to 16QAM is sufficient. However, an advanced UE (e.g., FWA, CPE or vehicle) may equip 8Rx/8Tx to achieve higher data rate and stringent latency requirement.

Table 2-3. Basic capabilities for different device types

| Parameter list | Advanced UE (e.g., CPE, FWA) | Normal UE (Smartphone) | Scalable UE (IoT) |
| --- | --- | --- | --- |
| RF/BB BW | 400MHz | 100MHz for 15KHz SCS, 200MHz for 30KHz SCS for sub 6G 400MHz for 30KHz SCS around 7GHz | As starting point, 5MHz for 15KHz SCS, 10MHz for 30KHzSCS |
| Rx/Tx antenna | 8Rx/8Tx | Up to 8Rx/4Tx | 1Rx/1Tx |
| MIMO layer（DL/UL） | 8/8 | 8/4 | 1/1 |
| Max modulation order（DL/UL） | 1024QAM/1024QAM | 1024QAM/256QAM | 64QAM (or 16QAM)/16QAM |
| Duplex Mode | FDD/TDD | FDD/TDD | Half-duplex |

Additionally, a forward-compatible mechanism should be designed to make sure a certain lower capability is only allowed in devices with form factor limitation, to prevent the abuse of using a scalable UE and avoid the implementation of mandatory features of a normal UE.

Proposal 3: For 6GR design, the impacts of BW, Antenna Configuration should be at least considered to achieve the scalable design.

- The assumption on basic capabilities listed in Table 2-3 for different device types (e.g., Advanced UE, Normal UE and Scalable UE (IoT)) can be considered as the baseline.

Then, given the analysis above, the corresponding TP is also recommended to be captured in TR 38.914.

Proposal 4: It’s recommended to capture the following TP in TR 38.914.

< Unchanged text omitted > - 5.3.2 Diverse device types - 6G RAT shall support a limited set of device types, e.g., Scalable UE, normal UE and advanced UE. - 6G UE shall support 5 MHz as the minimum supported bandwidth with SCS = 15kHz. - A forward-compatible mechanism should be designed to distinguish among different device types. < Unchanged text omitted >

Views on minimum spectrum allocation

In RAN4 #117, the support of 3MHz spectrum allocation is agreed subject to commercial need for particular bands as shown below.

Agreement: - Define minimum CBW based on SCS (e.g., 5 MHz for 15 kHz, 10 MHz for 30 kHz, 50 MHz for 120 kHz). - Support 3 MHz with 15kHz SCS for particular bands subject to operator requests, commercial need and spectrum regulations, - Work with RAN1 for the chosen minimum CBW in initial access design to accommodate majority spectrum scenarios.

Then, from RAN perspective, if such interests are still held, and confirmation of minimum spectrum allocation as 3 MHz can be considered.

Regarding the impact on initial access, in RAN1#123, the following two solutions for 3MHz spectrum allocation is considered:

Agreement If the minimum spectrum allocation is 3MHz with 15kHz SCS for 6GR, - Opt1: Design of the common signals/channels (at least for SSB) for initial access by assuming bandwidth larger than 3MHz, which is applicable to any spectrum allocations with adjustment, if applicable - Opt2: A single design of the common signals/channels (at least for SSB) for initial access by assuming minimum spectrum allocation as target bandwidth 3MHz, which is applicable to any spectrum allocations

However, as highlighted above, Opt1 should be considered for common signals/channels design, which is aligned with the priortization on eMBB service. Additionally, for some specific deployment, it can still be adjusted to 3MHz bandwidth to mitigate performance impact, if needed. For option 2, if all the signals/channels are designed based on 3MHz, system performance and eMBB service would be severely impacted.

Proposal 5: If the necessity is justified, RAN can confirm that the minimum spectrum allocation is 3MHz.

- For the design of common signals/channels (at least for SSB) for initial access, bandwidth larger than 3MHz is assumed.

- Summary

In this contribution, our views on diverse device types are provided along with the following observations and proposals:

Proposal 1: For 6GR, at least the following three type of devices are recommended as the target.

- Scalable UE: Designed for the reduced capability scenarios, offering lower complexity and cost.

- Normal UE: Providing improved performance compared to 5G eMBB.

- Advanced UE: Designed for the demand of high data rate, low latency scenario, with relaxed restriction of form factor and higher capability than normal UE.

Proposal 2: For 6GR design, as a starting point, the smallest UE RF/BB bandwidth of diverse device types is recommended as 5MHz with 15KHz.

Proposal 3: For 6GR design, the impacts of BW, Antenna Configuration should be at least considered to achieve the scalable design.

- The assumption on basic capabilities listed in Table 2-2 for different device types (e.g., Advanced UE, Normal UE and Scalable UE (IoT)) can be considered as the baseline.

Proposal 4: It’s recommended to capture the following TP in TR 38.914.

< Unchanged text omitted > - 5.3.2 Diverse device types - 6G RAT shall support a limited set of device type, e.g., Scalable UE, normal UE and advanced UE. - 6G UE shall support 5 MHz as the minimum supported bandwidth with SCS = 15kHz. - A forward-compatible mechanism should be designed to distinguish among different device types. < Unchanged text omitted >

Proposal 5: If the necessity is justified, RAN can confirm that the minimum spectrum allocation is 3MHz.

- For the design of common signals/channels (at least for SSB) for initial access, bandwidth larger than 3MHz is assumed.

- Reference

- Chair notes RAN#109.

- Chair notes RAN1#123.