3GPP TSG RAN Meeting #109   											   RP-253303

Baltimore, US, December 8-11, 2025

Agenda Item:	8.2.1.0

Source:	CableLabs, Charter Communications, Rogers Communications, Comcast

Title:	Discussion on single layer urban micro scenario in 6GR deployment for 38.914

Document for:	Approval

# Introduction

In RAN#107, the Revised Study Item Description (SID): “Study on 6G Scenarios and requirements” was approved [1]. Subsequently, indoor hotspot, dense urban, rural, urban macro, and sub-urban macro scenarios were discussed in RAN#109 [2].

Building upon the progress made in RAN#109 [2], as well as the references in TR 38.913 [3] and the current version of TR 38.914 [4], this contribution propose the inclusion of a single-layer urban micro (UMi) outdoor hotspot scenario for 6GR.

# UMi deployment scenarios for 6G

In RAN#109 [2], double-layer layouts were discussed for dense urban and urban macro scenarios, in which a micro layer was overlaid on a macro layer. However, in many markets, mid-tier operators operate under MVNO agreements with larger mobile operators, leveraging the large operators’ macro or low-band networks to provide seamless coverage. These mid-tier operators typically deploy their own micro-layer network to offload data traffic, managing network planning, deployment, and operation independently. This business model differs from the conventional double-layer architecture.

Existing 5G NR bands around 4 GHz may be repurposed for 6G, and new bands are under consideration by various regional regulators. The physics of signal propagation in these mid-band frequencies warrants the consideration of high-density low power base station deployments. In addition, certain service provider and enterprise business models are optimally satisfied with a very dense deployment of low-power base stations which do not reply on the existence of a macro overlay. Therefore, we propose that RAN consider adding an single-layer UMi outdoor hotspot scenario for 6GR around 4 GHz and around 7 GHz.

The UMi deployment scenario focuses on urban hotspot data traffic offloading. The key characteristics of this scenario include relatively low base station (BS) height and low transmit power, which result in reducing intra-network interference and enable dense BS deployment (compared to the urban macro scenario) with efficient frequency reuse. This UMi scenario typically leverages pole-mounted and strand-mounted infrastructure for BS deployment. In this scenario, seamless coverage is not the primary objective, instead, the focus is on capacity enhancement and localized data traffic handing with hotspot areas. Some of the attributes are listed in Table 1.

Table 1: Attributes for urban micro

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | Around 4 GHz (single layer) Around 7 GHz (single layer) |
| Aggregated system bandwidth | Around 4 GHz: Up to 300 MHz (DL+UL) Around 7 GHz: Up to 400 MHz (DL+UL) |
| Layout | Single layer: - Hex. Grid |
| ISD | 289 m (1/3 of UMa coverage area) All micro TRxPs are outdoors |
| BS antenna elements | Around 4 GHz: Up to 16 Tx and Rx antenna elements Around 7 GHz: Up to 64 Tx and Rx antenna elements |
| UE antenna elements | TBD |
| User distribution and UE speed | TBD |
| Service profile | NOTE: Whether to use full buffer traffic or non-full-buffer traffic depends on the evaluation methodology adopted for each KPI. For certain KPIs, full buffer traffic is desirable to enable comparison with IMT-Advanced values. |

# Proposal

We propose that RAN consider adding an single-layer UMi outdoor hotspot scenario for 6GR around 4 GHz and around 7 GHz, and the UMi scenario to be captured in the new TR 38.914.

# References

- RP-250810, “Revised SID: Study on 6G Scenarios and requirements”, CMCC, Verizon, NTT DOCOMO, Deutsche Telekom, Incheon, Korea, March 2025.

- RP-252888, “pCR on Deployment scenarios for TR 38.914 (after 2. Offline),” Deutsche Telekom (Moderator), RAN#109, Beijing China, Sept. 2025.

- 3GPP TR 38.913, Study on Scenarios and Requirements for Next Generation Access Technologies.

- 3GPP TR 38.914, Study on 6G Scenarios and Requirements.