3GPP TSG-RAN #110	RP-253333

Baltimore, USA, Dec. 8th – 11th, 2025

Agenda Item:	8.2.2

Source:	InterDigital, Inc.

Title:	AI/ML for 6G Air Interface

Document for:	Discussion

# Studies on AI/ML for 6GR

During RANP #108, a new study item was approved for 6G Radio (6GR) [1]. For AI/ML, the objectives of the study item include identification of AI/ML use cases and AI/ML framework.

Following from the experience from the introduction of AI/ML in 5GA, we observe the following.

Observation 1: Study/work on AI/ML use cases implies workload across multiple RAN WGs.

Observation 2: In previous releases, RANP approved use cases and scope of study for AIML use cases and managed cross-WG impacts.

With this background, we consider the agreed objectives related to studies on AI/ML as part of the 6GR SI [1].

- AI/ML for 6GR and Radio Access Network, leveraging 5G AI/ML framework, as appropriate [See TR38.843] [RAN1, RAN2, RAN3, RAN4] - Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity, etc… Coordinated discussion needs to be ensured with related design areas, where needed (e.g., MIMO, Mobility, etc…) NOTE: lead WG depends on the use case. - AI/ML framework: Extensible AI/ML enablers based on the identified Use Case(s), including - LCM procedures [RAN2, RAN1, RAN3, RAN4] - Data collection and data management, in coordination with SA WGs [RAN2, RAN3, RAN1] Note: NW for AI is assumed to be covered by new services 6GR and RAN design shall ensure that the 6G System can also operate without AI/ML

As can be observed from the 6GR SI description, study on AI/ML is one of multiple objectives with primary focus on studies on developing a baseline design for 6GR that does not rely on use of AI/ML for network/UE operation to satisfy all the 3GPP and ITU requirements. In this light, the study objective for AI/ML is to facilitate enablers for AI/ML applications, including development of framework for LCM procedures, e.g., training data collection, applicability reporting/alignment between UE and network, performance monitoring, functionality/model management, etc.

Observation 3: Per the 6GR SID, the 6GR design should satisfy all 3GPP and ITU requirements for 6G without relying on AI/ML-based operation.

Observation 4: Efforts on AI/ML for 6GR SI should focus on performance evaluations, enablers, and signaling framework for LCM procedures for a reasonable number of selected use cases, considering the workload, that can be deployed with tangible benefits.

Following the study objective, RAN1 agreed on the procedure for the first phase of the study to perform initial categorization of submitted proposals on AI/ML applications from contributing sources. While there has been good progress with respect to the categorization of the submitted proposals, as can be noted from the cited RAN1 agreements in Appendix A, there has been no decision on any focus/down-selection of use cases/sub-cases yet, leading to a general lack of clarity on the effective workload and the decision-making process, not only for RAN1 but for other RAN WGs as well. Furthermore, it should be noted that use case identification also needs to consider decisions in RAN2/RAN4 for use cases that may not have been considered by/within the scope of RAN1.

Based on the above observations and the rationale detailed in the following section, we propose the following.

Proposal 1: Given the workload and impact to other WGs, confirm that RAN will downselect and provide guidance on scope of AI/ML use cases (as for Rel-18/Rel-19 for 5GA) no later than RAN #111.

# Discussion

## Unique Opportunities and Challenges for AI/ML studies in 6GR SI

Consideration of AI/ML-based methods and use of AI/ML as a tool for specification/solution development offer significant new opportunities that can be uniquely leveraged with the advent of a new non-backward compatible RAT for 6G, and such is evident from the level of interest and the number of use cases being proposed in RAN1.

With the opportunity to shape the fundamental design elements across layers, e.g., at PHY layer, signaling framework, UE capability characterization, etc., 6GR can take into consideration and also benefit from AI/ML methods. In this regard, it would be imperative for 3GPP to evaluate different options to maximally leverage Machine Learning techniques that include:

- application of both supervised and unsupervised learning methods to support RAN functionalities (the latter being attractive from the perspective of being able to simplify complex LCM aspects like training data collection/ground truth acquisition), and

- offline utilization of AI/ML techniques as tools to develop solutions for specifying 3GPP features/components, i.e., without involving any post-deployment inferencing operation at the UE or NW sides.

Multiple use cases have been proposed that belong to the two categories above. Notably, these include use cases employing AIML as a tool for developing codebooks (CBs), design sequences for physical signals with desirable auto/cross-correlation properties or PAPR characteristics, etc. Most of these use cases can be studied as part of the non-AI/ML solution development, e.g., codebook design for CSI feedback can consider CBs designed using AI/ML tools; once designed, they can be specified for use without resorting to use of AI/ML models during actual operation.

From another perspective, the 6GR SI also offers the opportunity to develop a signaling framework best suited for LCM procedures for different AI/ML methods. The core 6GR components, including signaling framework as well as PHY signals/channels and their configuration, can be developed to enable low OH and low complexity LCM, including training data collection, performance monitoring, etc.

Observation 5: 6GR SI offers unique and rare opportunities to leverage AI/ML techniques for 6GR:

- to support RAN functionalities (identification of use cases with most tangible gains),

- with offline use of AI/ML as tools for the development of core components of specifications, e.g., codebooks, sequences, etc., without necessitating use of AI/ML models during network/UE operation (studies can be performed as part of non-AI/ML solution development),

- to enable efficient design of the PHY signals/channels and signaling frameworks with LCM-awareness (“AI/ML enablers”).

However, these require in-depth studies of different solution approaches for a given use case. Thus, a careful and focused study of a small set of use cases can be key to the success of identifying the most lucrative opportunities in designing solutions for 6GR. On selection/prioritization of use cases, different views have been expressed in RAN1 on suitable criteria. Proposed considerations include sufficient performance gains (subject to tradeoffs against complexity/OH/power consumption/etc.); suitability of studying across working groups (cross WG interactions); wide applicability (across frequency bands, MIMO config, UE distributions); location of the model (NW-sided vs UE-sided vs two-sided); standardization efforts, etc.

While all of these are valid considerations and should be used to guide prioritization of study of use cases, those use cases for which the benefits would be best realized if available as day 1 features for 6GR should ultimately be prioritized to ensure that 6GR can offer a real differentiation against previous technologies in terms of use of AI/ML.

Figure 3.1-1 Unique opportunities and challenges for AI/ML studies in 6GR SI

In addition to the approximately five-fold larger number of use cases being proposed for study as part of the 6GR SI, there are some challenges unique to the study of AI/ML use cases for “Day 1” 6GR. Some of these are elaborated next.

- Simultaneous study of non-AI/ML and AI/ML solutions for 6GR features

  - Non-AI/ML solutions serve not only as default options for operation (“6GR and RAN design shall ensure that the 6G System can also operate without AI/ML”) but also as benchmarks against which gains from AI/ML-based solutions are to be assessed

    - Upon first glance, such an approach, that aims to consolidate discussions on non-AI/ML and AI/ML-based solutions, may appear attractive as non-AI/ML and AI/ML solutions may be evaluated adjacently. However, AI/ML solutions, especially those using supervised learning, involve various other LCM procedures beyond inferencing that rely on basic PHY design/assumptions and demand study of additional aspects (see below on new aspects for study for 6GR AI/ML) that make comparative evaluations less straightforward in practice.

    - The dependency on non-AI/ML baseline(s) and related design choices for 6GR fundamental PHY channels/signals/procedures that are also being studied poses a unique challenge for studies on AI/ML solutions and their evaluations, implying need for increased study efforts and discussion time in RAN WGs.

    - As an unintended consequence of spreading the efforts thin across too many AI/ML use cases/sub-cases, 3GPP runs the risk of compromising the quality of study of the core non-AI/ML features.

- New aspects for study for 6GR AI/ML use cases relative to the studies during 5GA

  - These include considerations on aspects relating to dimensioning of UE (and network, as applicable) to support AI/ML functionalities and on LCM aspects beyond inferencing, e.g.,

    - characterization of processing complexity not only associated with inference but also other LCM procedures like performance monitoring, data collection including obtaining labels (ground truth information),

    - characterization of processing complexity in view of simultaneous support of multiple AI/ML-based functionalities,

    - system overhead (OH) associated not only with inferencing-related configurations, but also configurations, OTA signal/channel Tx-Rx, and measurement reporting,

    - impact on storage/memory requirements,

    - impact on power consumption,

    - aspects related to online training/fine-tuning (if feasible and applicable), etc.

  - Most of the above were not studied as part of the 5GA studies on AI/ML for air interface and thus can be expected to require additional time and efforts relative to 5GA studies. For instance, assessment of impact on system OH associated with ground truth acquisition or performance monitoring are critical details necessary for proper evaluation of the demands on radio resources to support AI/ML-based solutions for a target functionality –a “key learning from 5G”.

- Real-world performance of two-sided models unclear

  - While for the AI/ML-based solutions employing one-sided models (at UE or network (NW) sides respectively) there is a decent level of understanding of the achievable performance in practice, at least for some of the use cases, e.g., AI/ML-based beam prediction or CSI prediction, the same cannot be claimed for two-sided models, e.g., in the context of AI/ML-based CSI compression. AI/ML-based CSI compression, using two-sided models, was studied in RAN1 and other RAN WGs across two 3GPP releases (Rel-18 and Rel-19) with normative work underway for Rel-20. At the same time, there remains lack of clarity on achievable performance in real deployments for the different options identified for inter-vendor training collaboration that go beyond vendor-specific IODT efforts. In fact, for inter-vendor training collaboration per Options 3-1a (model parameter exchange for known model structure) and 4-1 (dataset exchange), there remains opens related to feasibility, pending feedback from SA WGs (cf. “check-point in RAN#110 upon SA WG feedback” in the Rel-20 WID for AI/ML based CSI compression [5]).

- Dependency on progress of the studies across multiple WGs

  - Examples include those potentially impacting AI/ML LCM procedures in RRC states beyond CONNECTED (e.g., sub-case D: “Beam prediction for initial access”), inter-cell/M-TRP DL Tx beam prediction and management overlapping with mobility aspects (sub-case A), both with significant involvement of RAN2 expected. Such cases pose an additional challenge as the dimensioning of workload is unlikely to be always aligned across WGs during the remainder of the 6GR SI. Beyond these examples, in general, there exists considerable cross-WG dependence making it impractical for RAN1 to able to determine the scope of the studied use cases unilaterally w/o considering impact and workload in other WGs.

Observation 6: Some unique challenges for studies on AI/ML in 6GR SI include:

- Simultaneous study of non-AI/ML and AI/ML solutions for 6GR features

- New aspects for study for 6GR AI/ML use cases relative to the studies during 5GA

- Real-world performance of two-sided models unclear

- Dependency on progress of the studies across multiple WGs.

## AI/ML use cases for 6GR studies

There has been great interest in studies on AI/ML use cases for the 6GR air interface with numerous use case families and their sub-cases being proposed to RAN1 since the inception of the RAN1-led 6GR SI.

Figure 2.2-1 Figure 3.2-1presents a schematic for the identified use cases and sub-cases based on the AI/ML use cases reported to RAN1 by individual sources. Counting across the use cases and sub-cases, 41 use cases/sub-cases are observed. Note that the figure does not include the potential study objective of RAN enhancements for more efficient support of AI/ML service (e.g., token traffic).

Figure 2.2-1 AI/ML use cases and sub-cases at RAN1 #123

For reference, Figure 2.2-2 presents a similar schematic for the list of RAN1-led AI/ML use cases pursued as part of 5GA. The number of use cases for 5GA is about 20% of that in the categorized list of use cases for 6GR SI, and the study of these spanned more than two full 3GPP releases (Rel-18 and Rel-19) with still some pending open aspects on inter-vendor training coordination for two-sided models for AI/ML-based CSI compression. While it is acknowledged that a significant amount of exploratory efforts expended during studies on AI/ML for 5GA air interface can be leveraged for the studies as part of 6GR SI, it is rather unlikely to be able to offset an increase in workload corresponding to a five-fold increase in the number of use cases when subject to a much shorter timeframe (remainder of the 6GR SI) than close to two full releases (Rel-18, Rel-19).

Figure 2.2-2 RAN1-led AI/ML use cases in 5GA

Observation 7: Studies on AI/ML use cases in 6GR currently include more than 40 use cases/sub-cases for a time frame spanning about 9 RAN1 meetings. In contrast, during 5GA studies on AI/ML for air interface, study of about 8 use cases/sub-cases spanned about 20 RAN1 meetings over two 3GPP Releases.

It is worth noting that a majority of the categorized “sub-cases” are effectively separate use cases, each requiring attention in adequate measure. Considering the example of CSI-RS OH reduction and CSI prediction, we notice that multiple variants of CSI prediction for the first sub-case (sub-case A: “Frequency and/or spatial domain CSI prediction with sparse/low overhead CSI-RS with AI/ML”) have been identified. These correspond to rather disparate “sub-cases” in terms of the characterization of AI/ML model(s) including model input/output/label acquisition/location of model, and consequently, different impact regarding UE and NW behavior and solution applicability, etc., and for a proper assessment of the sub-case, RAN1 needs to evaluate/analyze most of these variants and avoid hasty down-selection of approaches to address a use case/sub-case.

As another example, consider the DMRS OH reduction use case. Although three sub-cases have been captured as part of the categorization exercise, a variety of receiver options also need to be considered as evident from the related discussion point from RAN1 #123 meeting provided in the Appendix B [6]. While many of these receiver options may be effectively combined and exact implementation choices abstracted for the purpose of normative specification, RAN1 would still need to study and evaluate most of these options to assess potential gains and identify any nuanced impact to LCM aspects across the receiver choices.

Observation 8: In terms of study workload, many of the reported sub-cases require studying of multiple combinations of different options, effectively expanding the actual use cases to evaluate.

## Review of the Objectives on AI/ML studies in 6GR SI

As quoted in the Section 1, the objectives on AI/ML studies in 6GR SI require RAN WGs to fulfil the following:

- Identification of use case(s) of interest with compelling trade-off between e.g., performance, complexity, etc.

- Study on development of an AI/ML framework for 6GR with focus on extensible AI/ML enablers for the identified use case(s)

With a large number of use cases to be studied for identification of use cases “…with compelling trade-off between, e.g., performance, complexity ...”, there is a risk for RAN1 and other involved/leading RAN WGs of not being able to complete the necessary studies and provide recommendations to RAN as per the workplan for the 6GR SI, and especially so, in view of the objective of studying AI/ML and extensible AI/ML enablers for the identified use case(s). There runs the additional risk of not accommodating sufficient time for the objective if the use case(s) of interest are identified at a rather late stage in the 6GR SI.

Observation 9: With a large number of use cases to be studied, there is a risk for RAN WGs of not being able to complete the necessary studies and provide recommendations to RAN as per the workplan for the 6GR SI.

## Work dimensioning for studies on AI/ML use cases within 6GR SI and RAN guidance

While the challenges outlined in the previous section may not be insurmountable, they certainly demand considerably more efforts and discussion time in WGs for adequate studies compared to current workplan for the study item, in RAN1 and other RAN WGs. Avoiding a dilution of efforts due to lack of focus and spreading the study across many use cases/sub-cases (40+ cases already) would be strongly desirable.

In this regard, it is worth noting that studies on all the use cases/sub-cases may not commence at once. Clearly, there are multiple use cases (e.g., PDCCH-related ones, Digital Twin construction-related ones, etc.) for which studies may only start once some of the basic design aspects for 6G PHY are in place. Thus, it may be reasonable to expect that RAN1 may not initiate studies across all 40+ use cases/sub-cases simultaneously. On the other hand, this also poses a different challenge – that of dimensioning sufficient time and efforts across the AI/ML use cases with many of them starting studies towards the latter part of the 6GR SI – a time that one can expect to be rather overloaded and critical in ensuring that the fundamental design elements for 6GR are studied with sufficient rigor.

Based on the discussion presented above, it should be appreciated that the 6GR SI would benefit immensely from focusing on a small set of AI/ML use cases for 6G Day 1 that are studied and evaluated across a multitude of different solutions/approaches that can achieve the optimal performance-complexity-OH-specification efforts trade-offs. Accordingly, we propose the following (repeated from Section 1).

Proposal 1: Given the workload and impact to other WGs, confirm that RAN will downselect and provide guidance on scope of AI/ML use cases (as for Rel-18/Rel-19 for 5GA) no later than RAN #111.

On two-sided models, in view of the complexity involved, it would be prudent to initiate new studies on two-sided models only once the normative work on AI/ML-based CSI compression is completed with learnings from 5GA available.

Proposal 2: Due to the significant complexity for inter-vendor training collaboration, lack of clarity on suitable baseline(s) for 6GR, and the critical open items as part of the ongoing work towards specifying the 5GA use case of CSI compression, study of two-sided models for 6GR are deferred to after successful completion of the Rel-20 WI for 5GA.

# Conclusion

In this contribution, we discussed the current scope of the studies on AI/ML use cases for 6GR and presented our views on the need for further focus on down-selection of AI/ML use cases for a successful outcome of the 6GR SI, for both AI/ML and non-AI/ML components. As part of the presented discussion, the following observations and proposals are made.

Observation 1: Study/work on AI/ML use cases implies workload across multiple RAN WGs. Observation 2: In previous releases, RANP approved use cases and scope of study for AIML use cases and managed cross-WG impacts. Observation 3: Per the 6GR SID, the 6GR design should satisfy all 3GPP and ITU requirements for 6G without relying on AI/ML-based operation. Observation 4: Efforts on AI/ML for 6GR SI should focus on performance evaluations, enablers, and signaling framework for LCM procedures for a reasonable number of selected use cases, considering the workload, that can be deployed with tangible benefits. Observation 5: 6GR SI offers unique and rare opportunities to leverage AI/ML techniques for 6GR: - to support RAN functionalities (identification of use cases with most tangible gains), - with offline use of AI/ML as tools for the development of core components of specifications, e.g., codebooks, sequences, etc., without necessitating use of AI/ML models during network/UE operation (studies can be performed as part of non-AI/ML solution development), - to enable efficient design of the PHY signals/channels and signaling frameworks with LCM-awareness (“AI/ML enablers”). Observation 6: Some unique challenges for studies on AI/ML in 6GR SI include: - Simultaneous study of non-AI/ML and AI/ML solutions for 6GR features - New aspects for study for 6GR AI/ML use cases relative to the studies during 5GA - Real-world performance of two-sided models unclear - Dependency on progress of the studies across multiple WGs. Observation 7: Studies on AI/ML use cases in 6GR currently include more than 40 use cases/sub-cases for a time frame spanning about 9 RAN1 meetings. In contrast, during 5GA studies on AI/ML for air interface, study of about 8 use cases/sub-cases spanned about 20 RAN1 meetings over two 3GPP Releases. Observation 8: In terms of study workload, many of the reported sub-cases require studying of multiple combinations of different options, effectively expanding the actual use cases to evaluate. Observation 9: With a large number of use cases to be studied, there is a risk for RAN WGs of not being able to complete the necessary studies and provide recommendations to RAN as per the workplan for the 6GR SI.

Proposal 1: Given the workload and impact to other WGs, confirm that RAN will downselect and provide guidance on scope of AI/ML use cases (as for Rel-18/Rel-19 for 5GA) no later than RAN #111. Proposal 2: Due to the significant complexity for inter-vendor training collaboration, lack of clarity on suitable baseline(s) for 6GR, and the critical open items as part of the ongoing work towards specifying the 5GA use case of CSI compression, study of two-sided models for 6GR are deferred to after successful completion of the Rel-20 WI for 5GA.

# References

- RP-251881, “New SID: Study on 6G Radio”, NTT DOCOMO (Moderator), Prague, Czech Republic, June 9-13, 2025.

- 3GPP RAN1 Chair’s notes, RAN1#122, Bengaluru, India, Aug. 2025.

- 3GPP RAN1 Chair’s notes, RAN1 #122bis, Prague, Czech Republic, Oct. 2025.

- 3GPP RAN1 Chair’s notes, RAN1 #123, Dallas, USA, Nov. 2025.

- RP-252445, “Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface Phase 2,” Qualcomm, Beijing, China, September 2025.

- R1-2509598, “Moderator summary #5 on AI/ML for 6GR,” Moderator (Samsung), RAN1 #123, Dallas, USA, Nov. 2025.

# Appendix A

The following agreement was made in RAN1#122 [2].

Agreement For 6GR AI/ML use cases identification/categorization, for each (sub-)use case proposed, proponent companies are encouraged to study and report the following: - Definition of each (sub-)use case, including at least AI/ML model input/output - The evaluation assumption, methodology, KPIs, benchmark, and preliminary simulation results - Assumption on training types, e.g., - offline training, online training/finetuning - Label construction (if applicable), including whether/how to obtain label data for model training - Assumption on model location for inference, e.g., UE-sided model, NW-sided model, and two-sided model - Collaboration/interaction between UE and NW, e.g., - no collaboration/interaction - UE/Network collaboration targeting at separate or joint ML operation - High level potential specification impact

Following the above guidance, preliminary observations related to use cases (and sub-cases) that were reported by individual sources to RAN1 were made during RAN1#122b [3] and were updated during RAN1 #123 [4].

For continued studies on AI/ML use cases during the remainder of the 6GR SI starting from RAN1 #124 (February 2026), at RAN1 #123 meeting, RAN1 agreed the following mapping of use cases to different agenda items for potential submission of contributions [4].

Agreement From RAN 1 perspective, the following use cases can be matched to the identified primary agendas of RAN1 Use cases; Primary agendas. Low overhead CSI-RS or CSI prediction with AI/ML; CSI-RS and CSI acquisition. Low overhead DMRS with AI/ML receiver; UL & DL DMRS associated with PUSCH/PDSCH Note: Sub-Case C: DMRS-free may be related to modulation. CSI compression and feedback; CSI acquisition Note: this may be related to uplink control. AI/ML for beam management and extension; Initial access for Sub-case D Beam management for other sub-cases Note: sub-case A/B/D maybe related to mobility. AI/ML for SRS; Low overhead SRS with AI/ML; SRS. Low PAPR SRS sequence design. AI/ML based SRS power imbalance compensation; Not RAN1-led. AI-enabled UL precoder indication; UL MIMO. AI-based non-linearity handling at transmitter or receiver; Not RAN1-led Note: this may be related to DMRS, SRS, Power control in RAN 1. AI/ML for (de)modulation; modulation Note: Sub-case B may be related to MIMO Note: assuming no change to DMRS. AI/ML based waveform for PAPR reduction; Waveform. AI/ML based HARQ-ACK feedback; Channel coding Note: this may be related to uplink control. PDCCH related; Prior-Information-Aided DCI Decoding; DL control. Lossless DCI Compression. Power control related; UL closed-loop power control; Power control. Pathloss prediction. RACH related design; Early contention resolution in RACH; Random access/PRACH. Low PAPR PRACH sequence design. Site Specific Learning for AI/ML using RAN Digital Twin; Depending on corresponding use case where site specific learning is applicable, e.g., DMRS. Digital twin construction related use cases; AI/ML-enabled RAN digital twin with distributed model; ISAC. Sensing based RAN digital twin construction with NW-side AI/ML model; ISAC. AI for positioning; Positioning related agenda, if any. To accommodate AI/ML service, e.g., token traffic; if any impact, most likely scheduling/HARQ.

# Appendix B

#### Proposal 2.2-1B: (receiver assumptions for each sub-use cases) [from R1-2509598] For 6GR AI/ML use cases identification/categorization, for the use case of low overhead DMRS with AI/ML receiver, the following receiver assumptions are reported by companies: For Sub-case A: Sparse orthogonal DMRS in frequency and/or time domain Option 1, Option 2, Option 3, Option 4, Option 5 For Sub-case B: Superimposed pilot Option 2, Option 4, Option 5 For Sub-case C: DMRS free Option 2, Option 4, Option 5

Receiver options: - Option 1: AI-based denoise on DMRS REs only - Note: channel interpolation, equalization, demodulation rely on non-AI - Input: estimated channel on DMRS REs (e.g., LS) - Output: denoised channel on DMRS REs only - Label: - Option M: Self-supervise (received signal on adjacent RE as label) - Option N: supervise (Known sequence/data (end to end training) - Option 2: AI-based channel estimation for data REs (with or without denoise on DMRS REs) - Note: equalization, demodulation rely on non-AI - Input: - Option A: Received signal on DMRS REs, [and DMRS sequence/pattern or for a given DMRS sequence/pattern] - Option B: Estimated channel on DMRS REs (e.g., LS or MMSE) - Option C: Received signal on DMRS REs and data REs, [and DMRS sequence/pattern or for a given DMRS sequence/pattern] (3) - Output: - Option M: Estimated channel on target REs (data REs with/without DMRS REs) - Option N: Estimated filtering coefficients for channel estimation - Label: - Option X: Ideal channel information on target REs (all) - Option Y: Estimated channel on target REs (in any SNR or high SNR range) - Option Z: Known sequence/data (end to end training) - Option 3: AI based channel prediction for target REs - Note: interpolation, equalization, demodulation rely on non-AI - Input: estimated channel on DMRS REs (e.g., MMSE) - Output: estimated channel on target REs (partial data REs) - Label: - Option X: estimated channel on target REs (e.g., MMSE) - Option Y: Known sequence/data (end to end training) - Option 4: AI-based joint {channel estimation + equalization + demodulation/LLR calculation} - Input: - Option A: Received signal on DMRS REs and data REs (all) - Additionally: noise variance - Option B: Estimated channel on DMRS REs (e.g., MMSE) - Output: - Option M: LLRs - Note: decoding is based on non-AI - Option N: Estimated constellation points - Note: LLR calculation and decoding is based on non-AI - Option O: equalizer coefficients - Label: - Option X: Known sequence/data - Option Y: label free - Option 5: AI-based {channel estimation} + AI-based {equalization + demodulation/LLR calculation} For Model 1: Channel estimation - Input: Received signal on superimposed signal - Output: Estimated channel on target REs (data REs) - Label: - Option X: Ideal channel information on target REs - Option Y: Estimated channel on target REs (in any SNR or high SNR range) - Option Z: Known sequence/data (end to end training) For Model 2: equalization + demodulation/LLS calculation - Input: Received signal on DMRS REs and data REs - Output: LLRs - Note: decoding is based on non-AI - Label: Known sequence/data

(a) AI Receiver Option 1 (b) AI Receiver Option 2 (d) AI Receiver Option 3 (e) AI Receiver Option 4 (f) AI Receiver Option 5