3GPP TSG RAN meeting #110										RP-253393

Baltimore, USA, December 8-11, 2025

Status Report to TSG

Agenda item:			9.2.3

| WI / SI Name | Study on enhancements for solutions for Ambient IoT (Internet of Things) in NR outdoor for active devices |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: Yes | Core part: No | Performance part: No |  | Testing part: No |
| Acronym | FS_Ambient_IoT_Outdoor_Active |  |  |  |  |
| Unique ID | 1080073 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-252964 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: 03/2026 | Core part: | Performance part: | Testing part: |  |
| Overall Completion level | Study Item: Overall: 75 % | Core part: | Performance Part: | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN1 |
| --- | --- | --- |
| Rapporteur | Name | Jay KIM |
|  | Company | LG Electronics |
|  | Email | jaehyung.kim@lge.com |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

2.1	RAN1

2.1.1	Agreements

2.1.1.1	RAN1#122bis

10.3.1	Evaluations

Agreement

For Rel-20 study, values of “W” and “h” used for RMa pathloss model are clarified as follows (as per TR38.901):

- W (avg. street width) = 20m

- h (avg. building height) = 5m

Agreement

For the bullet of “FFS: 40dB as additional assumption for Device 2b with IF-ED” under Device 2b noise figure assumption in link budget for evaluation purpose:

FFS: 40dB as additional assumption for Device 2b with IF-ED

Agreement

For Rel-20 study, the value of X dBm/Hz for “Receiver interference density (dBm/Hz)” in the link budget template is from the followings:

- For R2D, X = -169.3 dBm/Hz

- For D2R, X = -165.7 dBm/Hz

- Companies can also report other higher value(s) for R2D or -999dBm/Hz along with their assumption

Agreement

For reference data rate in LLS assumption for evaluation purpose, RAN1 select one option from the followings:

- Option 1: Remove the FFS bullet, i.e.

  - FFS whether to add additional value(s)

Agreement

For message size in LLS assumption for evaluation purpose, update the previous agreement as follows:

- FFS whether to add value of 800 bits.

Agreement

For the SFO and CFO in LLS assumption of Device C, update the previous RAN1 agreement (in RAN1#122) as follows,

Agreement

For Rel-20 study, SFO and CFO in LLS assumption for evaluation purpose is from the followings:

- Device C is smaller than Device 2b

|  | Device 2b | Device C |
| --- | --- | --- |
| Initial SFO | Randomly select a value from the range of 10^3~10^4 ppm | [50] ppm |
| SFO after clock sync/calibration | 10^2 (O), 10^3 (M) ppm | [10] ppm |
| Initial CFO | Randomly select a value from the range of 10^3~10^4 ppm | [50] ppm |
| CFO after clock sync/calibration | 100 (M), 200 (O), 50 (O) ppm | [10] ppm |

Agreement

For Rel-20 study, following template is used for companies to report energy harvesting/storage assumptions to obtain the reported data rates,

| Source | Energy harvesting/storage assumptions |
| --- | --- |
| Source [x] | Energy source: FL notes: Company to report energy source e.g. RF energy source and/or solar etc. Energy storage capacity: FL notes: Company to report energy storage capacity values in Joules or in farad with a voltage value. Reported data rate: FL notes: Company to report data rate for R2D and/or D2R respectively. Other information (if any): FL notes: In addition, companies can also report any related information they would like to indicate e.g. message size, reception/transmission duration, device power consumption, voltage, etc. Companies can also report how they derive the reported energy storage capacity. |

Agreement

For Rel-20 study, “Link budget template” is updated as followings,

Link budget template for Rel-20 study

| No. | Item | Reader-to-Device | Device-to-Reader |
| --- | --- | --- | --- |
| (0) System configuration |  |  |  |
| [0A] | Scenarios | Deployment scenario 4 with topology 1 with no external carrier wave | Deployment scenario 4 with topology 1 with no external carrier wave |
| [0B] | Device 2b/C | Device 2b/C | Device 2b/C |
| [0C] | Center frequency (MHz) | 900MHz (M), 2GHz (O) | 900MHz (M), 2GHz (O) |
| [0D] | Pathloss model | UMa NLOS defined in TR38.901 RMa NLOS defined in TR38.901 Note: companies can also report to use RMa LOS defined in TR38.901 UMi NLOS defined in TR38.901 | UMa NLOS defined in TR38.901 RMa NLOS defined in TR38.901 Note: companies can also report to use RMa LOS defined in TR38.901 UMi NLOS defined in TR38.901 |
| (1) Transmitter |  |  |  |
| [1D] | Number of Tx antenna elements / TxRU/ Tx chains modelled in LLS | Refer to LLS table [0h1] and [0h2] | 1 |
| [1E] | Total Tx Power (dBm) | For standalone: 43 dBm For in-band: 38 or 33 dBm over R2D transmission BW Note: companies should report any PSD limitation they assume in their RAN1 evaluations | Device 2b: {-10, -20} dBm Device C: {-3, 0, 5} dBm |
| [1F] | Transmission Bandwidth used for the evaluated channel (Hz) | Refer to LLS table [1a] | Refer to LLS table [2a1] |
| [1G] | Tx antenna gain (dBi) | 17dBi | For A-IoT device, 0dBi |
| [1J] | Ambient IoT on-object antenna penalty | N/A | 0dB or 0.9dB or 4.7dB |
| [1N] | Cable, connector, combiner, body losses, etc. (dB) | 3dB | N/A |
| [1M] | EIRP (dBm) | Calculated (see Note 1) | Calculated (see Note 1) |
| (2) Receiver |  |  |  |
| [2A] | Number of receive antenna elements / TxRU / chains modelled in LLS | Same as [1D]-D2R | Same as [1D]-R2D |
| [2B] | Bandwidth used for the evaluated channel (Hz) | Refer to LLS table [1b] ED bandwidth | Refer to LLS table [2a3] |
| [2C] | Receiver antenna gain (dBi) | same as [1G]-D2R | Same as [1G]-R2D |
| [2X] | Cable, connector, combiner, body losses, etc. (dB) | N/A | Same as [1N]-R2D |
| [2D] | Receiver Noise Figure (dB) | Device 2b: 15 dB FFS: 40dB as additional assumption for Device 2b with IF-ED Device C: 13 dB | For BS as reader: 5dB |
| [2E] | Thermal Noise power spectrum density (dBm/Hz) | -174 | -174 |
| [2E1] | Receiver interference density (dBm/Hz) | -169.3 dBm/Hz Companies can also report other higher value(s) for R2D or -999 dBm/Hz along with their assumption | -165.7 dBm/Hz Companies can also report -999 dBm/Hz along with their assumption |
| [2F] | Effective Noise Power (dBm) | Calculated (see Note 1) | Calculated (see Note 1) |
| [2G] | Required SNR/CNR | Reported by companies for Budget-Alt2 | Reported by companies for Budget-Alt2 |
| [2H] | Ambient IoT on-object antenna penalty | 0dB or 0.9 dB or 4.7 dB | Not applicable |
| [2J] | Budget-Alt1/ Budget-Alt2 | Budget-Alt2 | Budget-Alt2 |
| [2L] | Receiver Sensitivity (dBm) | Calculated (see Note 1) | Calculated (see Note 1) |
| (3) System margins |  |  |  |
| [3A] | Shadow fading margin (dB) | For UMa NLOS: 6dB For RMa NLOS: 8dB For UMi NLOS: 7.82dB | For UMa NLOS: 6dB For RMa NLOS: 8dB For UMi NLOS: 7.82dB |
| [3B] | polarization mismatching loss (dB) | 3 dB | 3 dB |
| [3C] | BS selection/macro-diversity gain (dB) | 0 dB | 0 dB |
| [3D] | Other gains (dB) (if any please specify) | Reported by companies with justification | Reported by companies with justification |
| [3E] | Penetration margin (dB) | {0, 20, 80} dB | {0, 20, 80} dB |
| (4) MPL / distance |  |  |  |
| [4A] | MPL (dB) | Calculated (see Note 1) | Calculated (see Note 1) |
| [4B] | Distance (m) | Calculated (see Note 1) | Calculated (see Note 1) |
| （5）Other |  |  |  |
| [5A] | Other notes | Companies to report | Companies to report |

Note 1: Calculated values are derived according to the following.

[1M]:

-	For R2D,

-	[1M] = [1E] + [1G] - [1N]

-	For D2R

-	[1M] = [1E] + [1G] - [1J]

[2F]:

-	[2F] = lin2dB(dB2lin([2D] + [2E]) + dB2lin([2E1])) +lin2dB([2B])

[2G]

-	For the R2D LLS for ED, CINR/CNR is reported, where CINR/CNR is defined as the ratio of signal power spectral density in the transmission bandwidth to the noise and interference (if any) power spectral density in the device ED channel bandwidth.

-	For R2D ZIF receiver, report the same metrics (i.e., CNR/CINR, signal transmission bandwidth, ED bandwidth) as agreed for RF-ED/IF receiver.

-	For the D2R LLS, the SINR/SNR is reported and it is defined as the ratio of signal power to noise and interference (if any) power in the receiver bandwidth. Receiver bandwidth is the bandwidth used at the reader side to filter the D2R signals for calculating noise and interference (if any) power.

[2L]:

-	For R2D,

-	[2L] = [2G] - lin2dB([2B] / [1F]) + [2F]

Note E: The term ‘lin2dB([2B] / [1F])’ is applied due to scaling from CNR/CINR to SNR/SINR.

-	For D2R,

-	[2L] = [2G] + [2F]

[4A]

-	For R2D

-	 [4A] = [1M] + [2C] - [2H] - [2L] - [3A] - [3B] + [3C] + [3D] - [3E]

-	For D2R

-	 [4A] = [1M] + [2C] - [2X] - [2L] - [3A] - [3B] + [3C] + [3D] - [3E]

[4B]

-	 [4B] is derived by assuming pathloss is [4A] and using the pathloss formula as agreed.

Agreement

For Rel-20 study, “Coverage evaluation assumptions for link-level simulation” is updated as followings,

Coverage evaluation assumptions for link-level simulation for Rel-20 study

| No. | Parameters |  | Assumptions |
| --- | --- | --- | --- |
| R2D/D2R common parameters |  |  |  |
| [0a] | Carrier frequency |  | Refer to link budget template |
| [0b] | SCS |  | 15 kHz as baseline |
| [0c] | Block structure |  | Blocks as agreed in Rel-19 with update reported by companies |
| [0d] | Channel model |  | TDL-C |
| [0e] | Delay spread |  | an RMS delay spread of 300 ns |
| [0f] | Device velocity |  | 3 km/h Companies can also report 10 km/h |
| [0g] | Number of Tx/Rx chains for Ambient IoT device |  | 1 |
| [0h1] | BS | Number of antenna elements | 2 or 4 |
| [0h2] |  | Number of TXRUs | 2 or 4 |
| [0m] | Reference data rate |  | 1 kbps (M) 5 - 7 kbps (M) 48 - 60 kbps (O) 0.1 kbps for message size of 20 bits or 96 bits (O) Other data rates can be reported by companies Note 1: Companies to report the exact data rate. Note 2: The exact data rate is close to the values listed above. Note 3: The exact data rate is calculated by dividing the total message size (excluding CRC) by the total transmission time including applicable overheads(e.g., CRC, pre/mid/post-ambles if present). Note 4: The exact data rate may be related to coding scheme, repetition and etc. Note 5: All data rates considered are for evaluation purpose only FFS whether to add additional value(s) |
| [0n] | Message size |  | {20 bits, 96 bits, 400 bits} are considered for message size. Note 1: companies to report the M value and chip length used for each message size Note 2: CRC is not included for the message size FFS whether to add value of 800 bits. |
| [0p] | BLER target |  | 1%, 10% |
| [0q] | Sampling frequency |  | Companies to report the sampling frequency (e.g., 1.92Msps or other feasible values if any) Initial SFO (Sampling Frequency Offset) (Fe): For Device 2b: Randomly select a value from the range of 10^3~10^4 ppm For Device C: 50 ppm SFO after clock sync/calibration: For Device 2b: 10^3 (M) ppm, 10^2 (O) ppm For Device C: 10 ppm Note: For random selection, the value is randomly selected per simulation drop, according to a uniform distribution Note: Above values are only for sampling purpose. Note: Above assumptions are only for LLS evaluation purpose only for R2D and D2R. The timing drift ΔT over a time T is modelled as ΔT = ±Fe * T. Note: SFO after clock calibration can be applied to Fe. Initial CFO: For Device 2b: Randomly select a value from the range of 10^3~10^4 ppm For Device C: 50 ppm CFO after clock sync/calibration: For Device 2b: 100 (M) ppm, 200 (O) ppm, 50 (O) ppm For Device C: 10 ppm Drift rate of 0.1 or 1 ppm/s. Companies to report which value they used. Note: Above assumptions are for LLS evaluation purpose only |
| [0r] | Device 2b/C |  | ZIF/IF-ED |
| R2D specific parameters |  |  |  |
| [1a] | Transmission bandwidth |  | 180kHz Other values e.g. 360kHz, 540kHz or other multiples of 180kHz are not precluded and up to company report |
| [1b] | ED bandwidth |  | Up to company report with considering: R2D transmission bandwidth Device Rx LO uncertainty (e.g. CFO ppm assumption) |
| [1c] | BB LPF |  | [X]-order Butterworth/RC filter with cutoff frequency at half of ED bandwidth. Companies to report X = {3, 5}. |
| [1d] | Waveform |  | OOK waveform generated by OFDM modulator |
| [1e] | Modulation |  | OOK-4 Companies to report M chips per OFDM symbol |
| [1f] | Line code |  | Manchester |
| [1g] | FEC |  | Companies who report evaluations using FEC should also report evaluations without FEC |
| [1h] | ADC bit width |  | 4-bit |
| [1j] | Detection/decoding method for Line code |  | Companies to report |
| [1k] | Repetition |  | Companies who report R2D evaluations using repetition should also report R2D evaluations without repetition |
| D2R specific parameters |  |  |  |

10.3.2	Study of air interface for Device 2b/C

Agreement

Study L1 R2D control information with separate CRC for Device 2b and for Device C considering the following aspects

- Necessary information to be included in the L1 R2D control

- Details on how the L1 R2D control information is transmitted

Agreement

Study necessity and feasibility of R2D FDM from the same reader for Device 2b and for Device C considering at least the following aspects:

- complexity of device to support FDM

- how a device determines the frequency resource for receiving one of the FDMed R2D transmissions

Agreement

Study necessity and feasibility of D2R CDM(A) for Device 2b and for Device C with sequence-based Msg1 transmission and potential impact to random access procedures.

- FFS: other cases, except for data transmission

Agreement

For the functionality of indicating the start of the R2D transmission that contains PRDCH, study at least the following options if the functionality relies on SIP:

- Option 1:  binary sequence-based SIP (e.g M-sequence or Golay sequence).

- Option 2: Reuse the Rel-19 SIP.

Agreement

For the functionality of indicating the end of the PRDCH, study the following option:

- Option 1: Use the L1 R2D control information to indicate the TBS.

Agreement

For the functionality of chip duration determination of the data payload in the PRDCH, study the following options:

- Option 1: same as the chip duration of the corresponding L1 R2D control information

- Option 2: indicated by the corresponding L1 R2D control information

Agreement

For the functionality of chip duration determination of the L1 R2D control information, study at least the following options:

- Option 1: Fixed chip duration. No use of CAP for chip duration determination.

- Option 2: Blind detection from a predefined set of the chip durations of L1 R2D control information. No use of CAP for chip duration determination.

- Option 3: Use CAP

  - Alt.3-1 Rel-19 CAP pattern

  - Alt.3-2 Enhanced CAP

    - Alt.3-2a: Rel-19 CAP pattern with repetition

    - Alt.3-2b: CAP pattern and the association between chip durations for CAP and chip durations of L1 R2D control information

- Option 4: Use a set of different binary sequences with fixed or variable length

  - Alt.4-1: using binary sequences for SIP. No use of CAP for chip duration determination.

  - Alt.4-2: using binary sequences for CAP

- Option 5: Use broadcast information (if any) to indicate the chip duration

Agreement

For the functionality of CFO estimation/LO calibration at the device side, study CFO calibration signal with the following aspects:

- Whether the signal is

  - Option a: unmodulated sinusoid single tone

  - Option b: unmodulated multiple sinusoid single tones

  - Option c: modulated multiple tones

    - Whether the CFO calibration signal is the same or different from the synchronization signal (if supported)

- Whether the CFO calibration signal is transmitted periodically and/or as needed (aperiodically)

- The time location(s) of CFO calibration signal.

  - Option 1: in relation to the L1 R2D control information

  - Option 2: in relation to the data payload of the PRDCH

  - Option 3: in relation to the PDRCH

  - Option 4: in relation to the synchronization signal (if supported)

  - Option 5: in relation to broadcast information (if supported)

  - Combination of Options is not precluded

- Other details (e.g., time duration, frequency location(s) of the CFO calibration signal)

Agreement

Study R2D synchronization signal at least for the functionality of frequency synchronization (including frequency acquisition) and for the functionality of timing synchronization/tracking.

- Alt1: periodic synchronization signal

- Alt2: aperiodic synchronization signal

Agreement

For resource allocation method for the first D2R transmission for DO-A for Device 2b and for Device C, study following options considering feasibility:

- Option 1: Periodic D2R resource allocation

- Option 2: Aperiodic D2R resource allocation

Agreement

For providing necessary information for the first D2R transmission for DO-A for Device 2b and for Device C, study at least following options considering feasibility:

- Option 1: necessary information provided by broadcast information

- Option 2: necessary information provided by a paging-like R2D message

- Option 3: necessary information derived from a synchronization signal and provided by broadcast information

Agreement

Study whether necessary information for the first D2R transmission for DO-A for Device 2b and for Device C is provided periodically and/or aperiodically by Reader.

Agreement

For the content of the first D2R transmission for DO-A for Device 2b and for Device C, study the following options:

- Option 1: Msg1-like content

- Option 2: DO-A data payload

  - FFS: with or without device ID

- Option 3: DO-A scheduling request

Agreement

The study assumes single-carrier waveform for D2R.

- FFS whether to use 2SB or 1SB for D2R

Agreement

Regarding D2R preamble for PDRCH, study the following aspects of the D2R preamble.

- Preamble length (e.g., longer than Rel-19 D2R preamble)

- Sequence type

- Number of sequences using the same length (e.g., for reader/device differentiation)

- Considering other factors affecting preamble design (e.g., D2R modulation and detection schemes)

Agreement

Regarding D2R midamble for PDRCH, study the following aspects of the D2R midamble.

- Midamble pattern(s) including the length(s) and interval(s) within (and possibly at the end of) a PDRCH

- Sequence type, and whether the same or different sequence type is used for D2R preamble and D2R midamble

- Considering other factors affecting midamble design (e.g., D2R modulation and detection schemes)

Agreement

Study the necessity of D2R Tx power control for Device 2b and for Device C for outdoor scenarios

- Target to agree on observations on pros and cons of D2R Tx power control vs. no Tx power control at RAN1#123

Continue studying feasibility of the following open loop power control methods considering device complexity:

- Methods of open loop power control

  - Option 1: Device calculates Tx power based on measured pathloss, a predefined formula, and some configured parameters from reader

    - Required information at the device side at least includes reader Tx power, received target power at the reader, etc.

  - Option 2: Device measures RSRP, and uses a corresponding Tx power when the RSRP falls in an RSRP range.

    - The RSRP range is based on RSRP threshold(s) configured by reader or predefined

    - Required information at the device side at least includes RSRP threshold(s), corresponding D2R Tx power related information per each RSRP range defined by the RSRP threshold(s)

  - FFS details (e.g., required measurements, required information)

Continue studying the necessity and feasibility of closed loop power control for D2R.

Agreement

Study interleaving for PDRCH with FEC, including the necessity and feasibility of the following:

- Alt 1: reuse both LTE sub-block interleaver and LTE bit collection scheme

- Alt 2: reuse LTE bit collection scheme without LTE sub-block interleaver

- Alt 3: based on LTE bit collection scheme with some update (e.g., for memory reduction) without LTE sub-block interleaver

- Combination of alternatives can be studied

Agreement

Study interleaving for PRDCH with FEC, including the necessity and feasibility of the following:

- Alt 1: reuse both LTE sub-block interleaver and LTE bit collection scheme

- Alt 2: reuse LTE bit collection scheme without LTE sub-block interleaver

- Alt 3: based on LTE bit collection scheme with some update (e.g., for memory reduction) without LTE sub-block interleaver

- Combination of alternatives can be studied

Agreement

For PDRCH, further study the necessity of scrambling with following details:

- Scrambling applied after block-level repetition (if applied) and channel coding (if applied, including interleaving if supported)

- Scrambling sequence type and generation method

Agreement

Regarding D2R modulation for device 2b and device C, further study the following:

- Option 1: BPSK

  - FFS the corresponding preamble and midamble design for coherent detection at reader side

- Option 2: OOK

  - FFS the corresponding preamble and midamble design for coherent or non-coherent detection at reader side

- Option 3: DBPSK

  - FFS the corresponding preamble and midamble design for non-coherent detection at reader side

- Option 4: MSK

  - FFS the corresponding preamble and midamble design for coherent or non-coherent detection at reader side

Agreement

The minimum R2D transmission bandwidth of 1 PRB is the baseline for Rel-20.

Agreement

For R2D signals, study the necessity for the functionality of SFO calibration, and study the feasibility of the following options:

- Option 1: Reuse CAP and/or Manchester encoding

- Option 2: sequence-based SIP with or without R2D midamble(s)

- Option 3: using the CFO calibration signal

Agreement

For the study of necessity and feasibility of FEC for R2D for Device 2b and for Device C, further study the following aspects:

- Coding schemes

  - LTE TBCC

  - Tailed convolutional code with the same polynomial as LTE TBCC

  - RM code

- FEC code rates: 1/2, 1/3, 1/4

Agreement

For R2D and D2R, study the order of channel coding (including interleaver if supported) and block-level repetition (if supported) for complexity reduction, considering necessity and feasibility.

Agreement

For the study of R2D binary sequence-based SIP, study the following aspects:

- Sequence type

  - Option 1: m-sequence

  - Option 2: Golay sequence

- Length of sequence

- Number of sequence(s)

- FFS: whether to apply Manchester coding to the sequence

- Note: the same aspects can be studied for R2D midamble if applied with a sequence-based SIP

Agreement

For the study of binary sequence-based Msg1 for the study of necessity and feasibility of D2R CDM(A) for Device 2b and for Device C, study at least the following aspects:

- Sequence type (e.g. Gold sequence, m-sequence)

- Length of the sequence

- Number of sequences

Agreement

For R2D, study at least the following aspects for the study of broadcast information:

- Information included in the broadcast information

- Whether to use PRDCH to carry the broadcast information

- Transmission of broadcast information:

  - Option 1: periodic

  - Option 2: aperiodic

- Time and frequency resources for transmission of the broadcast information, e.g. in relation to the synchronization signal.

Agreement

For the L1 R2D control information preceding and scheduling the data payload of PRDCH, study the following aspects:

- Time location of L1 R2D control information:

  - Option 1: L1 R2D control information and data payload of PRDCH are contiguous in time.

  - Option 2: L1 R2D control information and data payload of PRDCH are not contiguous in time.

- Study frequency resource of L1 R2D control information

- Whether L1 R2D control information is included in the PRDCH that carries that data payload, or in another PRDCH, or in another channel

2.1.1.2	RAN1#123

10.3	Study of Enhancements for solutions for Ambient IoT (Internet of Things) in NR outdoor for active devices

R1-2509565 (Skeleton of update to TR 38.769) is endorsed

10.3.1	Evaluations

Agreement

For Rel-20 study, adopt the TP in Moderator’s evaluation summary attached in R1-2509500 into TR38.769

Note: companies can further update evaluation results (if any) into spreadsheet until RAN1#124, TP for summary of evaluation results is expected to be updated accordingly.

Note: based on the results in the summary, observation based on company results on achievable cell edge data rate at least 50m – 500m will also be discussed/captured.

Agreement

For Rel-20 study, values between -3 dBm and 5 dBm are feasible maximum transmit power for Device C (from device power consumption perspective).

Agreement

The following energy harvesting/storage assumptions used to obtain the reported data rates from companies are provided during the Rel-20 study

- Note: Companies can further add their reports until RAN1#124

| Source | Energy harvesting/storage assumptions |
| --- | --- |
| Source [Huawei] | Energy source: Solar power/light Energy storage capacity: - ~300 mF capacitor for low data rates - ~60 mF capacitor for middle data rates - ~40 mF capacitor for high data rates - High voltage 2.7 V, low voltage 1.1 V Reported data rate: - Low data rate: ~ 0.1 kbps for D2R and ~ 1 kbps for R2D - Middle data rate: ~ 1 kbps for D2R and ~ 1 kbps for R2D - High data rate: ~ 5 kbps for D2R and ~ 5 kbps for R2D Other information (if any): - For D2R transmission, the consumed energy per hour is - For 0.1 kbps data rate: ~50 mJ - For 1 kbps data rate: ~6.25 mJ - For 5-7 kbps data rate: ~1.6 mJ - For R2D transmission, the consumed energy per hour is - For 1 kbps data rate: 4 mJ - For 5-7 kbps data rate: 0.8 mJ |
| Source [vivo] | Energy source: Energy sources with high energy conversion efficiency, e.g., solar, vibration, thermal, etc., can be considered. Energy storage capacity: >=1mJ for 1mW peak power consumption and >=10mJ for 10mW peak power consumption. Reported data rate: For D2R, data rate is 1kbps. Other information (if any): When considering energy storage capacity, device at least can finish one D2R transmission. For 1000bits and 1kbps data rate, the D2R transmission time is 1000ms. For device C, the peak power consumption is ≤ 1 mW to ≤ 10 mW as agreed. Thus, for 10mW peak power consumption the energy is at least larger than 10mW*1000ms, i.e., 10mJ. Assuming the rated maximum voltage is 3V and the minimum available voltage is 2V, it can be derived that the capacitance of capacitor should be larger than 4mF. And for 1mW peak power consumption. The energy is at least larger than 1mW*1000ms, i.e., 1mJ corresponding to 400uF capacitance of capacitor. |
| Source [ZTE] | Energy source: - Solar or other energy source are used. - RF energy harvesting is deprioritized Energy storage capacity: - > 15 mJ for device 2b - > 75 mJ for device C Reported data rate: - ~ 4 kbps for R2D transmission - ~ 0.1 kbps for D2R transmission Other information (if any): The energy storage of > 15 mJ (device 2c) and > 75 mJ (device C) is considered to support an entire CBRA process for multiple devices. For a single PDRCH transmission, assuming a D2R data rate of 0.1 kbps and a TBS of 400 bits, the transmission takes approximately 4 seconds. This requires Device 2b (with a Tx power consumption of 800 uW) to have an energy storage capacity greater than 3.2 mJ and Device C (with a Tx power consumption of 2 mW) greater than 8 mJ to support a single D2R transmission. |
| Source [Tejas] | Energy source: Solar/light energy harvesting Energy storage capacity: 180-300 mF for Device 2b with voltage = 3.3 V. 180-1700 mF for Device C with voltage = 5V Reported data rate: R2D data rate is 12-15 kbps with M=2 D2R data rate is 5-7 kbps Other information (if any): energy conversion efficiency of energy harvester is considered as 80-90%. Device 2b power consumption is considered as 480 µW to 770 µW. Device 2b power consumption is considered as 1 mW to 10 mW. Solar panel size for device 2b is 20x20 mm2 and Device C is 30x25 mm2. |
| Source [NEC] | Energy source: Solar energy Energy storage capacity: 10~100µF with voltage of 3V Reported data rate: 1 kbps or 5 – 7 kbps Other information (if any): Device 2b and Device C harvest energy at similar rate. |
| Source [QC] | For device 2b Energy source: RF signal/Solar Energy storage capacity: 10s ~ 100s uF < 1mF Reported data rate: 1.7/2.5/5kbps Other information (if any): Device 2b, message size= 96 bits, tx duration = 2.28ms, device power consumption = 500uW, voltage = 1V, assumed only 50% of energy can be used. For device C Energy source: Solar Energy storage capacity: 1m ~ 10mF Reported data rate: 1.7/2.5/5kbps Other information (if any): Device C, message size= 96 bits, tx duration = 2.28ms, device power consumption = 5mW, voltage = 1V, assumed only 50% of energy can be used. FL notes: In addition, companies can also report any related information they would like to indicate e.g. message size, reception/transmission duration, device power consumption, voltage, etc. Companies can also report how they derive the reported energy storage capacity. |
| Source [DCM] | Energy source: Solar, thermal, etc. Energy storage capacity: For device 2b with 500uW peak power consumption: - 0.1kbps: ~5mJ/~20mF - 1kbps: ~500uJ/~2mF - 7kbps: ~71.4uJ~285uF For device C with 10mW peak power consumption: - 0.1kbps: ~100mJ/~400mF - 1kbps: ~10mJ/~40mF - 7kbps: ~1.4mJ/~5.7mF Reported data rate: 0.1 kbps, 1kbps, 7kbps Other information (if any): For the maximum message size, 1000 bits is assumed for single PRDCH/PDRCH transmission. For voltage, max voltage as 2V and min voltage as 1V are assumed. For power consumption of each device type, 500uW is assumed for device 2b and 10mW is assumed for device C. It should be noted that the reported capacity is for single PRDCH/PDRCH transmission. To sustain whole procedure of DT, DO-DTT, DO-A, larger capacity is required. |
| Source [FUTUREWEI] | Energy sources: solar, thermal, vibration etc. Energy storage capacity: For device 2b with 500uW peak power consumption: 0.5 milli-joules or 500 μF using 2V power supply For device C with 10mW peak power consumption: 10 milli-joules or 10000 μF using 2V power supply Reported data rate: 1kbps Other information (if any): It is observed that using RF energy harvesting is challenging as the energy source for Device 2b based on the following description. Typical leakage current for 500 μF capacitors is at least higher than 3 μA. With a supply voltage of 2 volts, the leakage current translates to ~ -22.2 dBm leakage power. Using RF harvesting is challenging in this case. |
| Source [Samsung] | Energy source: Solar Energy storage capacity: 5μJ for device 2b, 30μJ for device C Reported data rate: 5.3 kbps for DL 20 bits, 6.7 kbps for UL 20 bits, 7.2 kbps for DL 96 bits, 8.4 kbps for UL 96 bits; with an average of 40% charging time |

10.3.2	Study of air interface for Device 2b/C

10.3.2.1	R2D signals, channels, waveform and procedures

Agreement

For R2D Repetition, block-level repetition is considered for R20 AIoT.

- bit-level and chip level repetition are not considered for further study.

Agreement

At least for the functionality of initial frequency synchronization (including frequency acquisition) and initial timing synchronization, the periodic synchronization signal transmission is necessary.

- Default transmission periodicity is predefined

- FFS (in SI or WI): whether synchronization signal transmission is strictly periodic at reader

Conclusion:

There is no need to remove Manchester coding for Rel-20 AIoT PRDCH.

Agreement

Regarding removal of M=24 for R2D: Capture following observations in TR 38.769

Sources [Huawei], [Ericsson], [Panasonic], [Xiaomi] and [vivo] state that it is not necessary to keep M value of 24 for R2D.

- Source [Huawei], [Ericsson] and [Panasonic] state and provide evaluation results show that, M=24 cannot work well for outdoor scenarios as its chip length is too short to resist large multipath delay and channel fading.

  - Source [Huawei] shows that M=24 cannot work for outdoor scenarios with BLER close to 1 under different SNRs

  - Source [Ericsson] shows that 10% BLER cannot be achieved with M=24 (even with repetitions)

  - Source [Panasonic] shows that, when M=24 (and M≥12), both Device 2b and Device C cannot achieve 1% BLER and have an error floor around CNR=10 dB or larger (even without SFO/CFO)

- Source [vivo] states and provides evaluation results show that, R2D transmission with M=24 cannot achieve 1% BLER even with 2 block-level repetitions, and under similar data rates, the performance of M=24 (with 2 repetitions) is worse than that of M=12 (without repetition)

Sources [Docomo], [Futurewei], [OPPO], [Samsung], [ZTE], [Qualcomm], [CATT] and [Nokia] state that it is necessary to keep M value of 24 for R2D.

- Source [Docomo] states that M=24 can be supported for a device which is close to reader and beneficial for device power saving to reduce PRDCH reception duration with higher peak data rate

- Source [Futurewei] provides evaluation results show that M=24 can still achieve 10% BLER with a reasonable SNR

- Sources [Futurewei], [OPPO] and [Samsung] state that M=24 is needed to achieve data rates similar as R19.

- Source [Nokia] states that, M=24 should not be removed, since it is still useful for indoor scenario, and the air interface for Device 2b/C studied for outdoor scenarios will be reused in indoor scenarios.

Note: tdoc number is referred as source for the observation, and it can be updated in next meeting.

Agreement

Regarding addition of M=32 for R2D: Capture following observations in TR 38.769

Sources [Futurewei], [Qualcomm], [LGE], [Spreadtrum], [Xiaomi], [Transsion] state it is beneficial to add M value of 32 for R2D.

- Source [Futurewei] states and provide evaluation results shows that, for the TDL-C 300 ns environment, the BLER of M=32 is below 10% when CP handling method is used with 8RB resource allocation;

- Source [Futurewei], [Qualcomm], [LGE], [Xiaomi], [Transsion] and [Spreadtrum] state that, the addition of M=32 could be considered for the purpose of achieving higher data rate. Qualcomm also states that it can increase peak data rate and reduce latency, and active devices can support it for TDL-A fading or LOS channel conditions, with the decision up to reader scheduling.

- Source [Xiaomi] states that, from the energy consumption perspective, M = 32 can enable short transmission duration to avoid interruption.

Sources [vivo], [Huawei], [OPPO], [CATT], [Samsung], [ZTE], [Panasonic], [Docomo] and [Ericsson] state that it is not necessary to add M value of 32 for R2D.

- Sources [vivo], [Huawei], [Samsung], [CATT], [ZTE], [Docomo], [Panasonic], and [OPPO] state concern on feasibility of M=32 due to too short chip duration.

  - Source [Huawei] provide evaluation results show that, M=32 cannot work for outdoor scenarios with BLER close to 1 under different SNRs, as its chip length is too short to resist large multi-path delay and channel fading, so addition of M=32 is not supported.

  - Source [CATT] provide evaluation results show that, for Device 2b, M=32 cannot satisfy the 500m coverage requirement at 10% BLER, and for Device C, though it can reach 10% BLER, it has an error floor above 1% BLER.

  - Source [vivo] provide evaluation results show that R2D transmission with M=32 cannot achieve 1% BLER even with 3 block-level repetitions, and under similar data rates, the performance of M=32 (with 3 repetitions) is worse than that of M=12 (without repetition)

  - Source [Panasonic] provide evaluation results show that, when M =32, both Device 2b and Device C cannot achieve 1% BLER and have an error floor around CNR=10 dB and larger due to smaller chip duration.

  - Source [Ericsson] provide evaluation results show that, for Device 2b and Device C, a 10% BLER cannot be achieved with M=32.

- Sources [OPPO] and [Samsung] state that, higher data rate with M=32 lacks motivation.

- [Docomo] states that, it would increase device complexity as devices need to blindly detect and differentiate M=24 and M=32 with certain FDR.

Note: tdoc number is referred as source for the observation, and it can be updated in next meeting.

Agreement

Regarding the order of channel coding (with interleaver, if supported) and block-level repetition: Capture following observations in TR 38.769

- Sources [vivo], [Huawei], [NEC], [CATT], [ZTE], [Futurewei], [Apple] and [Qualcomm] report that channel coding before block-level repetition should be supported, with following justifications

  - Sources [vivo], [NEC], [CATT] and [Qualcomm] state that less memory is required by the channel coding before block-level repetition, compared with channel coding after block-level repetition.

  - Source [Huawei] states that the order of the channel coding first, followed by block-level repetition should be supported because of the diversity gain achieved due to the increased distance of a given parity bit in the output stream when compared to the reverse order.

  - Source [CATT] states that block-level repetition before FEC scheme requires Rblock times more interleaver memory as well as higher implementation complexity and processing delay for both devices and reader.

- Sources [OPPO] and [LGE] state that channel coding after block-level repetition should be supported.

Agreement

Regarding Addition M=1 for R2D: Capture following observations in TR 38.769

- Sources [Futurewei], [Spreadtrum], [Nokia], [CMCC], Huawei], [Ericsson], [Xiaomi], [CATT], [Transsion], [LGE], [Panasonic], [Apple], [TCL], [IIT Kanpur], and [Sequans] states that addition of M=1 is beneficial to improve R2D coverage.

  - Source [Huawei] shows that, M = 1 provides ~3 dB performance gain compared with M = 2 using correlation detection with TBS 20 bits and 96 bits, and ~2.5dB performance gain with TBS 400 bits, when repetition and FEC is not applied, where the CINR/CNR for OOK4 signal, is calculated according to section 4.3.2 of TR 38.769.

  - Source [Panasonic] shows that, M = 1 shows performance gain, when repetition and FEC is not applied, where the CINR/CNR for OOK4 signal, is calculated according to section 4.3.2 of TR 38.769.

  - Source [CATT] states that, For M=1, the larger chip duration could reduce the DS impact and improve the detection performance for Device 2b/C in outdoor deployment scenario.

- Sources [vivo], [ZTE], [OPPO], [Qualcomm], [DOCOMO] states that addition of M=1 is not necessary.

  - Sources [vivo], [ZTE], [OPPO], [DOCOMO] states that, OOK with M=2 using together with repetition achieve the same data rates as M=1.

  - Source [ZTE] further observed that OOK with M=2 using together with repetition can achieve better time-domain diversity gain and power boosting gain than OOK with M=1, where the CINR/CNR for OFDM symbol with non-zero power is calculated according to section 4.3.2 of TR 38.769, OFDM symbol with zero power is not counted.

  - Source [ZTE] states that, OOK4 with M =2 and M=4 have better detection performance than OOK4 with M =1, where the CINR/CNR for OFDM symbol with non-zero power is calculated according to Section 4.3.2 of TR 38.769, OFDM symbol with zero power is not counted.

  - Source [Qualcomm] shows that, M=1 cannot provide better performance than M=2, if the CINR/CNR for OFDM symbol with non-zero power is calculated according to Section 4.3.2 of TR 38.769, and OFDM symbol with zero power is not counted. Source [Qualcomm] also shows that M=1 provide less than 2dB performance gain than M=2, if the CINR/CNR, for OOK4 signal across all OFDM symbols, is calculated according to Section 4.3.2 of TR 38.769.

Agreement

For the functionality of indicating the start of the R2D transmission that contains PRDCH,

- At least, it is feasible to use binary sequence-based SIP (i.e., m-sequence or Golay sequence), and the binary sequence-based SIP can be used for timing synchronization/tracking.

- Regarding Rel-19 SIP,

  - Source [CATT] provide evaluation results show that R-TAS SIP defined in Rel-19 A-IoT can satisfy the detection requirements of target MDR of 10% for the FAR up to 1% in outdoor scenarios

  - Source [vivo, Huawei, Qualcomm, ZTE] provide evaluation results to show R19 SIP cannot be reliably detected.

    - Source [vivo] show that R19 SIP does not outperform sequence-based SIP in MDR, even if R19 is using sequence correlation detection instead of edge detection in R19.

    - Source [Qualcomm] shows that, for Rel19 SIP using energy detection (Option 2), the required CNR at BLER=10% is about 8dB higher than PRDCH with 20bit+6bit CRC, M=2, TBCC 1/3 (e.g., for R2D L1 control), indicating that Rel19 SIP cannot be reused and enhancement is needed.

    - Source [Huawei] show that if the same R-TAS design is used for a device C using correlation detection, the required SNR is 17.0 dB to achieve MDR ≤ 1% and FAR ≤ 1%, which is also not sufficient for the coverage enhancement for outdoor scenarios

    - Source [ZTE] provide evaluation results shows that the Release 19 SIP sequence performance at BLER=1% cannot satisfy the coverage requirements, while the performance at BLER=10% can satisfy the coverage requirements.

Agreement

R2D periodic synchronization signal is a binary sequence in time domain

- The binary sequence reuses Rel-19 R2D waveform

- The binary sequence has a pre-defined chip length

- Device assumes that the binary sequence has a predefined length

- The number of binary sequence(s) is less than or equal to 4

  - If more than 1 sequence is supported, it is for potential reader identification/differentiation

- Study whether Manchester coding is applied to the binary sequence

- Study necessity of other aspects, including overlaid sequence

Agreement

Regarding R2D FEC schemes, Capture following observations in TR 38.769

If FFC is supported for R2D,

- Sources [vivo], [CMCC], [Huawei], [NEC], [CATT], [CTC], [Transsion], [LGE], [Apple], [TCL], [Sequans] [OPPO], [Panasonic], [Qualcomm] report LTE TBCC should be supported.

- Sources [Futurewei], [Transsion] report tailed CC with same polynomial as LTE TBCC can be supported.

- Source [Qualcomm] reports Reed-Muller coding can be supported. Source [CATT] reports Reed-Muller coding can be supported for L1 control, if L1 control is no larger than 11 bits, otherwise, TBCC is supported.

The following are observations regarding comparison of LTE TBCC, Tailed CC with same polynomial as LTE TBCC and Reed-Muller coding

For comparison between LTE TBCC and Tailed CC with same polynomial as LTE TBCC.

- Source [Futurewei] states, for the same number of information bits N, a tailed convolutional code encodes L-1 additional bits (tail bits) to ensure the final state returns to 0. With tail bits, a typical decoder can operate on N+L-1 bits. With TBCC, no additional bits are needed but a typical decoder operates on at least 2N bits. For moderate and larger number of information bits, tailed convolutional codes and TBCC have the same performance. For smaller number of information bits, with additional decoding iterations, the performance of TBCC can approach the performance of tailed convolutional codes.

- Source [vivo] states, compared with LTE TBCC, tailed convolutional code can reduce decoding complexity while increase the overhead.

- Source [Huawei] states tailed convolutional codes with the same polynomial as LTE TBCC has a performance similar to LTE TBCC but suffers rate loss with zero bits padded at the end of the input bits.

- Source [TCL] states, the decoding for Tailed convolutional code maybe simpler, but the tail bits need to be sent, and the actual effective code rate slightly decreases

For comparison between LTE TBCC and RM

- Source [vivo] states RM code suffers performance degradation and larger complexity at least for larger transport block size, without power reduction benefit

- Source [CMCC] states, RM code is deprioritized due to the restricted operational range

- Source [Huawei] states RM codes have a 4-5 times higher power consumption than convolutional codes due to the complicated fast Hadamard transform calculation for the FEC decoder, and has a 3-11 bit TBS restriction in the NR baseline design.

- Source [TCL] states the combinations of code length and code rate for RM code are limited, which is less flexible than convolutional codes. And the evaluation for the performance of the RM code is needed under low signal-to-noise ratio conditions.

- Source [Qualcomm] states RM codes performs worse than TBCC. RM with k=8 or 10, n=32 has lower decoding complexity, memory cost, hardware area cost and lower power consumption than Viterbi decoding for TBCC with 1/2 or 1/3 coding rate.

- Source [Docomo] states decoding complexity of RM code is expected to be lower than Viterbi decoding for TBCC. RM is not supported for payload size larger than 11.

Note: tdoc number is referred as source for the observation, and it can be updated in next meeting.

Agreement

For Option a: unmodulated sinusoid single tone

Sources [Futurewei], [Spreadtrum], [vivo], [CMCC], [Huawei], [Ericsson], [NEC], [CTC], [Xiaomi], [Samsung], [ZTE], [LGE], [Lenovo], [Apple], [MTK], [Sharp], [Quectel], [Qualcomm], [Docomo], [TCL], [IIT Kanpur], [CEWiT], [Sequans], [interDigital], and [Sony] state that, option-a unmodulated sinusoid single tone is feasible, and can be considered for CFO calibration signal.

[Working principle]

- Sources [vivo] and [Huawei] and [Docomo], report with references, and state that, the CFO calibration is based on injection lock mechanism, and the working principle of injection lock is to leverage the phase interaction between an external single-tone signal and the internal oscillation signal of an oscillator to lead the oscillator’s frequency lock to that of the unmodulated sinusoid single-tone signal.

- Source [CMCC] report with simulation result, and states that, CFO calibration using unmodulated sinusoid is performed at intermediate frequency, where the reader transmits the CFO calibration signal with a pre-defined duration, the device mixes the CFO calibration signal with its internally generated wave, can then count the number of periodicities of the residual frequency within the duration to estimate and calibrate the CFO.

[Positive views]

- Source [CMCC] report that with a duration of 1 OFDM symbol, the CFO after calibration can be reduced to less than 10 ppm.

- Sources [Futurewei], [Ericsson], [CTC], [Samsung], [ZTE], [LGE], [Qualcomm], [Lenovo] and [IIT Kanpur] state that unmodulated sinusoid single tone can achieve CFO calibration with low complexity.

- Source [vivo] states that single tone signal is already supported in NB-IoT, (e.g., NPRACH), there is no co-existence issue.

[Negative views]

- Source [CATT] states that it will create the inter-channel interference to neighboring NR sub-bands.

- Source [Qualcomm] states that this method may be vulnerable to deep fading, reducing reliability.

For option b, unmodulated multiple sinusoid single tones

Sources [LGE], [Qualcomm] and [TCL] report that option b con be considered for CFO calibration

[Positive views]

- Source [LGE] states that, option b provides frequency diversity gain while maintaining low complexity; considered as optional feature. It is preferable that the tones are sufficiently separated in frequency, e.g., by at least the Rx filter bandwidth.

- Source [Qualcomm] states that, using multiple sinusoidal tones can improve robustness against fading by providing frequency diversity.

- Source [TCL] states that, although option b is slightly more complex than a single tone, it is still a very simple waveform.

[Negative views:]

- Sources [Futurewei], [vivo], [Spreadtrum], [Sony], [TCL] and [Quectel] state that option b increases in detection complexity, and it may also complicate frequency resource allocation.

- Sources [vivo] and [Huawei] raised feasibility concerns on option b

  - Source [vivo] states that even with increased complexity, it is not clear how does it work, since frequency of each tone cannot be accurately determined, due to only frequency range of each can be determined if captured by Rx filter, and the target frequency for LO calibration cannot be accurately determined.

  - Sources [Huawei] and [NEC] state that multiple frequency components would occur when mixed with the local RF frequency signal and it is difficult to lock the LO frequency.

- Source [Sequans] state that multiple unmodulated tones increase PAPR and require FFT-based tone separation

For Option c, modulated multiple tones, 3 different designs are reported by companies

[Working principle]

- Source [Ericsson] state that the unmodulated multiple tones are transmitted in different frequency locations in different time resources

  - The reader transmits the synchronization signal using a centered bandwidth sweeping method, where each centered bandwidth covers a portion of the overall synchronization signal bandwidth

  - The device detects the synchronization signal power at different strengths on each of the subsets and based on the subset with the strongest synchronization signal power, the device applies a frequency correction

- Sources [CATT] and [MTK] report that, CFO calibration is done in baseband domain, and CFO Calibration is OOK sequence (Not R19 SIP or CAP). The CFO pre-compensation method is given as follows:

  - Step 1: The received signals with pre-compensated CFO ±Δf is added in the received R2D signals to limited the frequency offset within ±1/2 Δf.

  - Step 2: The correlation processing of the CFO calibration signal with the received signals, the precompensated received signals with additional Δf, and the pre-compensated received signals with additional-Δf.

  - Step 3: The peak of the correlation values from the three correlation outputs would be selected as the received signals for the R2D signal processing.

- Sources [Docomo] states that, the CFO calibration can be achieved by measuring chip length

  - Source [Docomo] states that one possible implementation is devices calibrate CFO based on SFO calibration by counting the number of samples of 0s and 1s and comparing with the defined duration of a pattern.

[Positive views]

- Source [Ericsson] state that using the synchronization signal to perform the CFO calibration of device 2b is foreseen to be suitable (residual CFO of 100 ppm)

- Source [CATT] state that the OFDM waveform-based CFO calibration signals would be orthogonal to neighboring NR channels

- Sources [CATT] and [MTK] state that R2D synchronization signal can be used for CFO calibration signal

- Source [Docomo] state that may be feasible if same clock (LO) is used for both carrier frequency generation and sampling.

[Negative views]

- Source [Ericsson] state that the CFO calibration of device C may require using unmodulated sinusoid single tone to meet a residual CFO of 10 ppm.

- Sources [Futurewei], [vivo], [CMCC] and [CEWiT] state that the CFO hypothesis at AIoT receiver cannot be constructed in baseband due to phase information for received signal is not kept in envelop detector, and no I/Q path in receiver baseband.

- Source [ZTE] report that, in multi-tone LP-SS discussion, the LP-WUR doesn’t have the capability to acquire good frequency calibration, similarly this method is not reliable.

- Sources [Huawei] and [NEC] state that the LO frequency calibration cannot rely on the baseband clock to fulfil the calibration procedure and the calibration signal should be utilized for the RF-end clock.

- Source [vivo] state that using ON chip duration to calibration Clock it not feasible, since according to RAN4 requirements on pulse width, Table 6.4.2-1 in section 6.4.2 of Ts 39.194, the pulse of a chip, the allowed chip length can be <=1.3 Tc when the nominal chip length is Tc, in this case, it is not feasible to use chip length/pulse width for CFO calibration.

- Source [Lenovo] states that pulse width distortion analysis is possible to be used for A-IoT device due to simplicity, but timer resolution at device is required to be much less than the difference between estimated pulse width and the transmitted pulse width.

Note: whether/how to capture blue parts can be up to editor

Agreement

Regarding whether the CFO calibration signal is transmitted periodically and/or as needed (aperiodically), capture following in TR38.769

[Periodic transmission CFO calibration signal]

Sources [Futurewei], [vivo], [CMCC], [Huawei], [CATT], [CTC], [Xiaomi], [ZTE], [Apple], [interdigital], [MTK], [Quectel], [Sequans], [Apple], [Qualcomm], [TCL], [Lenovo] and [CEWiT] state that the CFO calibration signal is periodically transmitted.

[Positive observations]

- Sources [vivo], [CMCC], [CTC] and [Sony] state that, CFO calibration signal with hundreds ms or 1 second periodicity is already sufficient. Aperiodic CFO calibration signal, e.g., on demand transmission with PRDCH is not needed due to slow frequency drift. Source [Ericsson] also states that the CFO calibration signal can be transmitted in very sparse manner.

- Sources [CMCC], [Huawei], [CTC], [Lenovo] and [Quectel] states that, to support DO-A traffic, periodic CFO calibration signal is needed.

  - Source [CMCC] states that, periodic CFO calibration is needed to avoid a device from keep monitoring the CFO calibration signal before transmitting the very first D2R transmission for DOA traffic.

- Source [MTK] and [Spreadtrum] states that it is beneficial to provide regular opportunities for devices to perform frequency calibration and maintain accurate synchronization with the reader.

[Aperiodic transmission CFO calibration signal]

Sources [Huawei], [CATT], [Xiaomi], [ZTE], [LGE], [MTK], [Quectel], [Spreadtrum], [Lenovo], [Apple], [Sharp], [Qualcomm], [IIT Kanpur], [Samsung] and [CEWiT] state that the CFO calibration signal can be transmitted aperiodically. In which, Sources [Huawei], [CTC], and [Quectel] state that Aperiodic CFO calibration signal can be considered with periodic CFO calibration signal as baseline.

[Positive observations]

- Source [Huawei] state that with the DRX configuration, the device will miss several instances of the periodic CFO calibration signal. In this case, the CFO calibration signal can be considered to be transmitted as needed, e.g., transmitted after the PRDCH carrying the paging message.

- Source [CATT] states that the R2D calibration signal should be transmitted before R2D reception and D2R transmission. Since R2D reception and D2R transmission may be either periodic or aperiodic, both periodic and aperiodic CFO calibration signals should be supported.

- Sources [ZTE], [LGE], [Lenovo], and [MTK] states that, at least for D2R triggered by PRDCH, the on-demand transmission of CFO calibration signals is beneficial to keep the interval between the CFO calibration signal and the subsequent D2R transmission short for better CFO calibration accuracy.

[Negative views]

- Source [vivo] states that if CFO calibration totally relying on aperiodic signal, the CFO calibration accuracy cannot be guaranteed, which depends on traffic.

Note: whether/how to capture blue parts can be up to editor

Agreement

Capture following in TR38.769

Company reports that whether a time gap between L1 R2D control information and data payload in PRDCH, depends on following factors

- Whether device buffers data part before finishing L1 control decoding, as reported by sources [vivo], [Huawei], [Samsung], [Spreadtrum], [Docomo], [CMCC], [OPPO], [Ericsson], [Qualcomm], [NEC], [Nokia], [Xiaomi], [ZTE] and [Apple].

  - Sources [vivo], [Huawei], [Samsung], [Spreadtrum], [Docomo], [NEC] and [ZTE] state that gap between L1 R2D control and data is not needed, assuming device can buffer data part before finishing L1 R2D control decoding. Source [Huawei] further state that the time gap would increase the power consumption of the device.

  - Sources [CMCC][OPPO] and [Xiaomi] state that time gap is needed to reduce/avoid buffer at AIoT device.

- Whether frequency resource is different for L1 R2D control information and data payload, as reported by sources [OPPO] and [NOKIA]

- Whether chip duration (M value) is different for L1 R2D control information and data payload, which is reported by sources [Futurewei], [Huawei] and [LGE]

- Whether L1 R2D control information and data payload are separate channel, as reported by source [CMCC]

- Whether CFO calibration signal is placed between L1 R2D control information and data payload, as reported by [Lenovo].

For Option 1, L1 R2D control information and data payload of PRDCH are contiguous in time,

- Sources [Samsung] and [Apple] state that, transmit L1 R2D control information and data payload in the same PRDCH with contiguous time resources and same frequency resource is beneficial for low device complexity. [Apple] further report that this simplifies the timeline design.

- Sources [vivo] and [NEC] report that the time gap is not long enough to introduce gap between L1 R2D control information and data payload. Sources [Huawei], [NEC] further states that decoding of L1 R2D control information the device can parallelly store the data payload in buffer, and receive L1 R2D control information, hence time gap is not needed.

- Sources [vivo] and [ZTE] report that L1 R2D control transmitted in the same frequency as data payload in PRDCH, and time gap is not needed.

- Source [LGE] report that no time gap needed if the chip length for L1 R2D control information is the same as that for data payload of PRDCH.

For Option 2, L1 R2D control information and data payload of PRDCH are NOT contiguous in time,

- Sources [FUTUREWEI], [Ericsson] and [Xiaomi] report that a short time gap is needed if the device is not capable of decoding L1 R2D control and buffering data payload of PRDCH.

- Source [CMCC] state that, L1 R2D control and data payload are transmitted in separated physical channel, hence they are not contiguous.

- Source [OPPO] state that, time gap is needed to switch the frequency if L1 R2D control is transmitted in different frequency from data payload.

- Source [FUTUREWEI], [LGE] report that time gap maybe needed if the chip length for L1 R2D control information is different from that for data payload of PRDCH.

- Source [Qualcomm] report that, if there is a gap, midamble can be inserted between L1 R2D control information and data payload.

Agreement

Regarding frequency resource of L1 R2D control information

- Sources [Futurewei], [Spreadtrum], [vivo], [Huawei], [Samsung], [NEC], [ZTE], [Apple] and [Qualcomm] report that same frequency resource for R2D preamble, L1 control and data part.

  - Source [Futurewei] state that different frequency resource will cause additional latency as the transmission carrying control information must be first received followed by receiving the transmission carrying the payload. And unclear of the necessity of this option if FDM is not supported

  - Sources [vivo] and [Huawei] state that frequency retuning can be avoided if L1 control information and the corresponding data payload are located with same frequency locations. Otherwise, it will lead to additional adjustment delay and power consumption.

  - Sources [Spreadtrum] and [vivo] state that the motivation of transmitting L1 control and data payload in different frequency is not clear.

- [OPPO] report that frequency resource for R2D preamble, L1 control and data part can be different such as to reduce the load in the frequency resource used for L1 control.

Agreement

Capture following in TR 38.769

For the functionality of chip duration determination of the L1 R2D control information, the following options are studied

For option 1, Fixed chip duration. No use of CAP for chip duration determination.

[positive observations]

- Sources [Ericsson], [CATT], [LGE] and [CTC] states that fixed chip duration for L1 R2D control information lowers the complexity and power consumption of chip duration detection.

- Source [CATT] states that fixed chip duration for L1 R2D control information minimize the ambiguity of the chip duration detection, and thus enhance the success rate of L1 R2D control information reception.

- Source [Ericsson] states that using fixed chip duration (using the smallest M value for optimal coverage) can achieve higher reliability for L1 R2D control information.

[Negative observations]

- Source [Qualcomm] and [Samsung] states that to achieve the largest coverage, L1 control would need to use the lowest M, which increases overhead for each R2D transmission.

- Sources [FUTUREWEI], [Samsung], [ZTE], [Huawei], [vivo], [Docomo] and [CMCC] states that, if chip duration for L1 control information is fixed, the flexibility or transmission efficiency is restricted.

For option 2, Blind detection from a predefined set of the chip durations of L1 R2D control information. No use of CAP for chip duration determination.

[positive observations]

- [CMCC] and [Panasonic] state that blind detection within configured set provide better flexibility for chip length for L1 R2D control information.

- [CMCC] states that blind detection is viable if the number of predefined chip duration candidates is restricted. Additionally, [CMCC] states that Option 2 can be combined with device-specific configuration signaling to enhance flexibility without excessive complexity.

[Negative observations]

- Sources [FUTUREWEI], [Samsung], [vivo], [DOCOMO], [Ericsson] and [ZTE] state that Option 2 increases detection complexity.

- Source [Qualcomm] states that it may be impractical due to constraints on power consumption and memory needed for buffering the incoming R2D L1 control information samples.

- Sources [Huawei] and [LGE] state that option 2 would require additional time gap between the L1 control information and the data payload of the PRDCH to allow for decoding delay.

- Sources [vivo] and [Ericsson] state that reliability is doubtful due to blind detection.

- Source [vivo] states that repetition and FEC parameters should be indicated anyway to achieve flexibilities in coverage and resource overhead, no need to leave chip duration for blind detection.

For option 3, Use CAP

[positive observations]

- For Alt.3-1 Rel-19 CAP pattern, Sources [Samsung] and [Sequans] state that Rel-19 CAP pattern to determine chip length of L1 R2D control part offer a balance between detection complexity and performance.

- For Alt.3-2b, CAP pattern and the association between chip durations for CAP and chip durations of L1 R2D control information, Sources [LGE], [Apple] and [Qualcomm] report that this option can achieve better detection reliability compared with R19 CAP. Apple further states that this option maintains consistency with Release-19 design principles. Source [Qualcomm] report that R19 CAP with repetition using correlation-based detection improves reliability, with low detection flexibility.

[Negative observations]

- Source [Qualcomm] states option 3 requires blind detection; and MD/FA rates and CAP overhead needs to be considered.

- Source [ZTE] states option 3 results in additional overhead due to transmission of CAP.

- Sources [CMCC], [vivo] and [Huawei] state that Option 3 is based on edge detection to determine the chip duration by using CAP, so the performance is limited or unfeasible due to the use of edge detection to detect the CAP pattern. Sources [ZTE] and [Ericsson] also report the performance of this option should be further evaluated.

- Source [Ericsson] states that option 3 would make the decoding of the L1 R2D control information more complicated for the device.

- Source [vivo] states that repetition and FEC parameters should be indicated anyway, no need to leave chip duration for blind detection using CAP.

For Option 4, Use a set of different binary sequences with fixed or variable length

[positive observations]

- For Alt 4-1, using binary sequences for SIP. No use of CAP for chip duration determination

  - Sources [Nokia], [Huawei], [NEC], [LGE], [Docomo] and [Panasonic] state that this method can provide flexibility for R2D chip duration.

  - Source [Huawei] state that a restricted set of binary sequences of differing lengths are mapped to different chip durations and coverage distances, which can be detected with limited blind detection complexity. Parallel processing for the blind detection of different sequence lengths avoids and processing delay. Besides, R-TAS design is simplified, which consist only sequence-based SIP.Alt.4-2: using binary sequences for CAP.

  - Sources [Nokia] and [Panasonic] report that flexibility on chip length of L1 R2D control can be achieved.

[Negative observations]

- Sources [vivo], [Ericsson] and [Qualcomm] states that option 4 increases detection complexity. And Ericsson states that option 4 would make the decoding of the L1 R2D control information more complicated for the device.

- Sources [ZTE] and [Ericsson] report that the performance of this option should be further evaluated.

- Source [vivo] states that repetition and FEC parameters should be indicated anyway, no need to leave chip duration for blind detection; vivo further report that it leader more challenges on preamble design, requiring larger number of sequences to indicate four M values in addition to reader/cell ID differentiation;

For option 5, Use broadcast information (if any) to indicate the chip duration

[positive observations]

- Sources [vivo], [ZTE] and [Qualcomm] state that better flexibility can be achieved without blind detection, and sources [vivo] and [ZTE] further state that the chip duration for L1 can be indicated together with other parameters for control information, e.g., repetition and FEC related parameters. Source [CMCC] state that this method can be used for L1 R2D control information common for devices, for example, the L1 R2D control information for scheduling paging message.

- Source [OPPO] state that the chip length should be pre-known by device to minimize complexity and power consumption at AIoT device.

[Negative observations]

- Source [Qualcomm] states that it requires system info acquisition and memorization.

- Source [Huawei] and [Samsung] states that only to indicate chip duration in broadcast information limits flexible time resource allocation from system perspective.

- Source [Ericsson] states that this option is a simple solution but not applicable for paging and broadcast PRDCH.

- Source [Docomo] states that state that option 5 has less flexibility on chip duration of L1 R2D control compared to option 2/3/4.

Note: whether/how to capture blue parts can be up to editor

Agreement

Regarding interleaving and LTE bit collection schemes for R2D, capture following observations in TR 38.769

Sources [vivo], [Huawei], [Qualcomm], [Docomo] and [ZTE] state that it is beneficial to support interleaving to improve coverage by obtaining time diversity gain.

Sources [vivo], [Huawei] and [Docomo] state it is feasible to support LTE subblock interleaving and LTE bit collection.

- Source [vivo] report with references to show that the power consumption for LTE de-interleaving for turbo decoding is tens of uW, even with multiple parallel (de)interleaving operation, and each operation with max number of bits of 6144. It can be expected that the power consumption for TBCC with LTE bit collection and with or without LTE sub-block interleaving can be even simpler. Hence, both LTE TBCC with bit collection with or without LTE sub-block interleaving are feasible.

- Source [Huawei] and [Docomo] report that the additional power consumption can be supported by active devices due to higher peak power consumption, allowing them to support larger memory. Source [Huawei] further report that larger memory would enable these devices to support the use of a LTE de-interleaver, and can follow the same hardware as used for the LTE sub-block interleaver with LTE bit collection scheme in D2R.

The following are observations regarding comparison of three alternatives.

For Alt 1: reuse both LTE sub-block interleaver and LTE bit collection scheme

- Sources [Futurewei], [Huawei], [OPPO], [CATT], [LGE] and [Panasonic] report LTE sub-block interleaver and LTE bit collection scheme can be supported, if FEC is supported.

  - Source [Futurewei] states that, reusing both LTE sub-block interleaver and LTE bit collection scheme enables a device to eliminate the repetition block and associated memory as the bit collection scheme can use the memory for the sub-block interleaver for repeated bits. Another benefit is the various coding rates are achievable without modifying the encoder specification.

  - Source [Huawei] states that, reusing the LTE sub-block interleaver and LTE bit collection scheme should be supported due to its performance gain especially for larger TBS transmissions, providing 1dB and 2.8 dB performance gain compared with Alt 2 for BLER 10% and BLER 1% respectively, for a TBS of 400 bits.

  - Source [LGE] states that, Alt 1 and Alt 2 are beneficial as they reuse the LTE bit collection scheme and/or the LTE sub-block interleaver, enabling R2D coverage enhancement with minimal specification effort

  - Source [Panasonic] states that, LTE sub-block interleaver could be beneficial for R2D to reduce the burst error caused by fading, especially for the longer message size with smaller values of M

  - Source [Qualcomm] states that, Alt 1 and Alt 2 can achieve similar BLER performance, and both require buffering full-length coded bits.

For Alt 2: reuse LTE bit collection scheme without LTE sub-block interleaver

- Sources [vivo], [NEC], [ZTE], [LGE], [Apple] and [Qualcomm] report LTE bit collection scheme without LTE sub-block interleaver can be supported, if FEC is supported.

  - Source [vivo] states that, Alt 1 and Alt 2 presents similar performance; thus Alt 2 is sufficient.

  - Source [ZTE] states that, since bit collection does not require interleaving, the complexity of Alt 2 is lower than interleaving. FEC with bit collection and FEC with sub-block interleaver has similar performance compared with FEC with bit collection.

  - Source [LGE] states that, Alt 1 and Alt 2 are beneficial as they reuse the LTE bit collection scheme and/or the LTE sub-block interleaver, enabling R2D coverage enhancement with minimal specification effort

  - Source [Apple] states that, Alt 2 provides a good balance between performance and complexity for device 2b/C, reusing existing 3GPP designs while minimizing memory requirements.

  - Source [Qualcomm] states that, for small number of coded bits, Alt2 can be used to improve interleaving gain

For Alt 3: based on LTE bit collection scheme with some update (e.g., for memory reduction) without LTE sub-block interleaver

- Source [Qualcomm] and [NEC] report that, LTE bit collection scheme with segment can be supported, if FEC is supported.

  - Source [NEC] states that, Alt 3 can be adopted if memory size limitation exists in devices

  - Source [Qualcomm] states that, Alt3 with two segmentations requires half of the buffering size versus Alt2 at price of 2.5dB performance loss, but still outperforms the no-FEC approach using three block-level repetitions. For large number of coded bits, Alt3 can be applied to reduce buffering requirements

  - Source [vivo] and [Huawei] state that, Alt 3 degrades performance due to lower time diversity gain compared with Alt 1/2, and [vivo] observes that, Alt 3 with two segments for 1000 bits TBS also suffers loss compared with block-level repetition.

  - Sources [vivo] and [Huawei] state that, since memory/power consumption by Alt 1 and Alt 2 is feasible for AIoT device, it is unnecessary to further reduce memory/power consumption by Alt 3.

  - Source [LGE] states that, Alt 3 is potentially advantageous in terms of memory reduction for A-IoT devices, but is considered less favourable due to the extensive discussion required to evaluate the performance of various bit collection schemes

Agreement

Capture following observations for sequence type for R2D SIP in the TR 38.769, that

For binary sequence based R2D SIP, m-sequence and Golay sequence are studied.

- Sources [Qualcomm] and [interdigital] provide evaluation results show that m-sequence and Golay sequence shows similar MDR/FAR and correlation property. [Docomo] states that both m-sequence and Golay sequence have good auto-correlation property.

- Sources [vivo], [OPPO], [Apple], [interdigital], [Qualcomm] and [LGE] report that m-sequence can be considered as binary sequence for R2D SIP, with the following observations:

  - Source [vivo] states that m-sequences with same length but different cyclic shifts/initial states can be considered to achieve good cross-correlation between different sequences.

  - Sources [interdigital], [LGE] and [Qualcomm] state that m-sequence design can be expected to be implemented with lower complexity than a Golay sequence. Source [Qualcomm] further state that m-sequences can be generated by storing the initialization state of the polynomial, reducing memory requirements.

- Sources [Huawei], [Qualcomm], [Docomo], [LGE] report that Golay sequence can be considered as binary sequence for R2D SIP, with the following observations:

  - Source [Huawei] states that Golay sequence achieves lower operational and correlation complexity compared with m-sequence.

  - Source [Docomo] states that Golay sequence with flexible sequence length may be beneficial for alignment with the OFDM symbol boundary.

  - Sources [Qualcomm] and [LGE] states that Golay-sequence offers slightly better cross-correlation than m-sequence, and source [Qualcomm] further states cross-correlation depends on the choice of Golay pairs, and Golay sequences may be programmed into devices.

Agreement

Capture following observations for number of sequences for binary sequence based R2D SIP in the TR 38.769, that

For number of sequences for binary sequence based R2D SIP

- Sources [Futurewei], [vivo], [Nokia], [Huawei], [Xiaomi], [Panasonic], [Lenovo] and [Qualcomm] report that multiple sequences can be used for a given length for R2D SIP.

  - Sources [vivo], [Panasonic], [Lenovo] report the purpose of multiple sequences is for reader differentiation, Sources [vivo] and [Panasonic] suggest to consider 3 sequences.

  - Source [Huawei] report that different length sequences are mapped to different coverage distances, in order to maximize the performance for each of the distances for outdoor scenarios while maintaining MDR and FAR to be ≤ 1%.

  - Sources [Xiaomi], [Lenovo] report the purpose of multiple sequences is to indicate parameters of upcoming R2D transmission. [Xiaomi] [Lenovo]report that presence of CFO calibration signal can be indicated by SIP sequences, and [Lenovo] also report that inclusion or exclusion of L1 control information, synchronization signal, or PRDCH, can be indicated by SIP sequences.

- Source [NEC] report that single length and single sequence is beneficial for lower complexity and power consumption.

- Source [Docomo] report that device complexity on correlation-based detection for multiple sequences should be considered.

Agreement

Regarding whether to apply Manchester coding to the binary sequence-based SIP, Capture following in TR38.769.

Sources [vivo], [CTC], [Qualcomm] and [Sony] report that Manchester coding can be applied to binary sequence for R2D SIP

- Source [vivo] and [Qualcomm] state that sequence Manchester coding achieves similar MDR and timing performance.

  - Source [vivo] provide evaluation results show that for 31-bit m-sequence with Manchester coding, 31-bit m- sequence without Manchester coding, and 63-bit without Manchester coding, these three cases have comparable durations (~15 OFDM symbols), and shows similar MDR performance.

  - Source [Qualcomm] provide evaluation results show that the 15-bit m-sequence with Manchester coding and the 31-bit m sequence without Manchester coding have comparable durations (~7.5 OFDM symbol), and both show similar MD performance.

- Source [Sony] states that applying Manchester coding to the sequence is important in OOK modulation with low data rate where a long number of zeros and ones may result in the clock drift.

- Source [vivo] also states that timing performance for sequence with Manchester coding is slightly better compared with longer sequence w/o Manchester coding (same overall duration), due to narrower main lobe of auto-correlation can be achieved with Manchester coding.

Source [Futurewei] states that the disadvantages of applying Manchester coding include increasing complexity and decreasing the number of sequences.

Agreement

Regarding whether to apply same or different chip duration for L1 R2D control and data payload, Capture following in TR38.769.

Sources [Spreadtrum], [Huawei], [Ericsson], [Samsung], [LGE], [Apple] and [Sequans] report that, the use of the same chip duration for L1 R2D control information and data payload in the PRDCH, have the following benefits:

- Sources [Spreadtrum], [Huawei], [Samsung], [Sequans] and [Apple] state that the use of the same chip duration simplifies device implementation, and [Huawei] further states that the L1 R2D control information can avoid any decoding delay and avoid the need for a time gap between the L1 R2D control information and the data payload of the PRDCH.

- Sources [Ericsson] states that, for broadcast PRDCH and paging PRDCH, the R2D parameters, including chip length for data payload of PRDCH, can be fixed for cater to the cell-edge scenarios.

- Sources [Samsung], [Sequans] and [LGE] state that using the same M value for the L1 R2D control part and the data payload avoids additional signaling overhead, and [Samsung] [Sequans] further states that this ensures consistent coverage for both control and data, and no clear benefit to use different chip length for L1 control part and data part.

Sources [Futurewei], [vivo], [CMCC], [Ericsson], [NEC], [CTC], [Xiaomi], [ZTE], [LGE], [Panasonic],[Docomo], [Apple], [Qualcomm] and [CEWiT] report that, chip duration of the data payload in the PRDCH indicated by the corresponding L1 R2D control information, have the following benefits:

- Sources [Futurewei], [vivo], [CMCC], [NEC], [CTC], [Xiaomi], [LGE], [ZTE], [Panasonic], [Docomo] and [CEWiT] state that chip duration indication through L1 control information provides better flexibility to meet diverse reliability and data rate needs. [ZTE] further states that it better coordinates with TB size, repetition rate, and code rate for improved link adaptation; [Futurewei] states that it allows tailoring data rates for specific devices and can improve efficiency;

- Source [vivo] state that with better coverage for L1 control part using longer chip length, unnecessary data reception can be avoided due to more robust L1 control;

- Sources [Apple] and [Ericsson] state that this option provides flexibility to adapt to different coverage conditions or message types, with [Ericsson] states this option provides benefits for PRDCH like Msg2 and Msg4.

Agreement

Following update is applied to the agreement made at RAN1#122bis.

Agreement For R2D signals, study the necessity for the functionality of SFO calibration, and study the feasibility of the following options: - Option 1: Reuse CAP and/or Manchester encoding - Option 2: sequence-based SIP with or without R2D midamble(s) - Option 3: using the CFO calibration signal

Agreement

Do not introduce a new R2D physical channel for L1 R2D control information, and update previous agreements as follows

Agreement (in RAN1#122bis) For the L1 R2D control information preceding and scheduling the data payload of PRDCH, study the following aspects: - Time location of L1 R2D control information: - Option 1: L1 R2D control information and data payload of PRDCH are contiguous in time. - Option 2: L1 R2D control information and data payload of PRDCH are not contiguous in time. - Study frequency resource of L1 R2D control information - Whether L1 R2D control information is included in the PRDCH that carries that data payload, or in another PRDCH, or in another channel

Agreement

Capture following observations in TR 38.769

For R2D FDM from the same reader for Device 2b and for Device C, feasibility and necessity are studied

[Feasibility]

- Sources [FUTUREWEI], [Spreadtrum], [Huawei] and [DOCOMO] states that R2D FDM is feasible for Device 2b and for Device C, considering device equipped with IF/ZIF filters at the receiver.

- Source [Ericsson] states that the total number of PRBs required to multiplex devices using FDM needs to be defined.

[Necessity]

- Source [vivo] reports that the transmission resource efficiency by using R2D FDM cannot be improved compared with R2D non-FDMA, when AIoT R2D bandwidth is no larger than 4 RBs.

- Source [LGE] states that:

  - If allows all devices to receive the initial paging message without FDM, while enabling efficient sub-band utilization using FDM for following messages, it will introduce complexity in managing mode transitions between FDM and non-FDM, and coordinating sub-band usage.

  - If enables device-specific optimization and parallel reception across sub-bands, it will increase transmission resource usage due to duplication of paging messages.

- Sources [vivo], [Ericsson], [NEC], [Samsung] and [TCL] state that guard band between FDMed transmissions need to be allocated to compensate the impact of CFO.

- Source [Docomo] states that for R2D, FDM can be beneficial to extend multiplexing capacity. Sources [ZTE] and [Qualcomm] state that in A-IoT systems, the capacity bottleneck lies on the R2D link. And to enhance system capacity, FDM is a potential solution.

[Device complexity]

Regarding device complexity to support R2D FDM from the same reader for Device 2b and for Device C

- Sources [CMCC], [CATT], [Xiaomi] and [Apple] state that R2D FDM will increase the device complexity since the adjustment of filter bandwidth and/or IF center frequency is required.

- Source [Samsung] raises the related issue that whether different FDMed frequency resources can be supported by the same device, and if so, how the Rx filter architecture of IF/ZIF receivers can support such flexible determination.

- Sources [ZTE], [Huawei] and [NEC] state that additional complexity is limited since device with IF/ZIF receiver can already support to distinguish different frequency resources.

[Determines the frequency resource]

Regarding how a device determines the frequency resource for receiving one of the FDMed R2D transmissions:

- Sources [FUTUREWEI], [Huawei] [Spreadtrum] and [Qualcomm] state that the frequency resource of synchronization signal, CFO calibration signal or broadcast information need to be fixed or predefined.

- Sources [Huawei], [Spreadtrum], [Qualcomm], [DOCOMO], [NEC], [ZTE] and [OPPO] state that some of R2D transmissions e.g., L1 control information or a R2D transmission other than sync signal or broadcast information can determine their frequency resources based on the indication.

- Source [OPPO] state that the frequency resource for data payload is indicated by corresponding L1 R2D control.

- Source [NEC] and [DOCOMO] state that the frequency resource of a R2D transmission can be associated with the previous corresponding D2R transmission.

- Source [NEC] state that the exact frequency resource/channel for a R2D transmission can be associated with a specific coverage level from a set of candidates R2D frequency resources preconfigured.

Note: whether/how to capture blue parts can be up to editor

Agreement

Regarding R2D Repetition, Capture following observations in TR 38.769

Regarding R2D repetition, it is reported by sources [Futurewei], [vivo], [CMCC], [Huawei], [NEC], [CATT], [CTC],  [OPPO], [Transsion], [LGE], [Apple], [Qualcomm], [CEWiT], [Sequans], [Panasonic], [Quectel], [NTT DOCOMO] and [IIT Kanpur] state that R2D repetition is feasible and can be considered. Sources [vivo], [Huawei], [CMCC], [OPPO], [CATT], [Transsion], [Apple], and [IIT Kanpur] report that repetition schemes including block-level repetition are feasible.

Agreement

Regarding how the L1 R2D control information is transmitted, capture following in TR 38.769.

[FEC and repetition for L1 R2D control information]

Regarding FEC and repetition for L1 R2D control part, sources [vivo], [OPPO], [CMCC], [Futurewei], [NEC], [CATT], [CTC], [ZTE] and [Qualcomm] report that FEC can also be applied L1 R2D control information. Source [Huawei] state that repetition is needed for L1 R2D control information.

[Parameters for transmission of L1 R2D control information]

Sources [vivo], [CMCC], [NEC], [OPPO], [CTC], [ZTE] and [Qualcomm] report methods to determine FEC, repetition related parameters for L1 R2D control information, if FEC/repetition is supported for L1 control.

- Sources [vivo], [CMCC], [NEC], [ZTE] and [Qualcomm] report that the parameters for repetition and FEC for L1 R2D control information can be pre-configured, e.g., by broadcast information. [OPPO] report that these parameters should be pre-known by device.

- Sources [NEC] and [CTC] states that parameters for repetition and FEC for L1 R2D control information can be fixed and pre-defined. Source [NEC] also state that these parameters can be indicated by a set of different binary sequences with fixed or variable length for indication. Source [NEC] report that a separate interleaving is needed for L1 R2D control.

[information size]

Regarding bit size for L1 control information

- Sources [vivo], [OPPO], [Ericsson] and [Apple] report that device only detects one TBS at a given time. And [vivo] also report that up to two TBS can be considered and detected by preamble.

[CRC Attachment]

Regarding separate CRC for L1 control information

- Source [Futurewei] state that, for Separate CRC on the L1 R2D control information, when the size of the L1 R2D control information is 24 bits or less, the existing CRC6 specification can be reused, if the size of the L1 R2D control information is between 25 and 57 bits, the existing CRC6 polynomial can be reused without impact to the undetected error rate.

- Source [Xiaomi] and [Apple] state that, how to attach the separate CRC for L1 R2D control information needs to be considered, and a fixed length (equal to or larger than 16) of CRC is required to ensure the reliability and avoid unnecessary blind detection for L1 R2D control information.

Note: whether/how to capture blue parts can be up to editor

Agreement

Capture following observations in TR 38.769:

For LTE TBCC Sources [vivo], [Huawei], [Qualcomm], [NOKIA], [CATT], [ZTE] and [Apple] report that the power consumption for TBCC decoding, as follows

- Source [vivo] states power consumption is tens of uW to few hundreds of uW

- Sources [Huawei] and [ZTE] state power consumption is less than 200uW.

- Source [NOKIA] states power consumption can be less than 100uW.

- Source [CATT] states power consumption is about 100uW

- Source [Qualcomm] states power consumption is tens of percent higher than a few mWs or less, dependent on the clock frequency, with constraint length K=7.

- Source [Apple] states power consumption is tens of uW.

Sources [Futurewei] and [Qualcomm] also provide complexity and memory cost analysis for LTE TBCC

- Source [Futurewei] states that, the decoding complexity depends on number of bits, constraint length K and number of iterations.

- Source [Qualcomm] states that, the decoding complexity and memory depend on constraint length K and trackback depth D.

For Tailed convolutional code with the same polynomial as LTE TBCC.

- Source [Futurewei] states the complexity is no larger than LTE TBCC.

- Source [Huawei] states that its performance is similar to LTE TBCC but suffers rate loss with zero bits padded at the end of the input bits.

For RM

- Sources [Qualcomm], [Docomo] and [Xiaomi] state memory and power consumption for RM is lower than TBCC, and it is feasible.

  - Source [Qualcomm] further state that, the decoding complexity, power consumption, memory cost and hardware area cost of RM code is lower than that of LTE TBCC, and suitable for ultra-low power A-IoT devices.

- Sources [Huawei] and [Transsion] state that, RM decoding based on fast Hadamard Transform incurs higher power consumption, compared with the short-constraint length convolutional codes. Source [Transsion] states that, the power consumption for RM decoding may not meet power consumption limit of device 2b/C.

Agreement

Regarding the code rate for FEC: Capture following observations in TR 38.769

For R2D FEC, code rate 1/2, 1/3 and 1/4 are studied

For Code rate R=1/3

- Sources [vivo], [NOKIA], [CMCC], [Huawei], [NEC], [LGE], [Panasonic], [Apple], [ZTE], [Qualcomm] and [TCL] state R=1/3 can be supported.

  - Source [CMCC] states that, 1/3 code rate is supported when interleaving is not applied.

  - Source [Huawei] states that, 1/3 code rate is necessary and feasible, and should be supported due to its performance gain and an increased power consumption for active devices.

  - Source [LGE] states that, it is beneficial to reuse the LTE TBCC with a preferred code rate of 1/3 to ensure reliable transmission and reduce specification complexity

  - Source [Panasonic] states that, for Device C, LTE TBCC with a coding rate of at least 1/3 could be beneficial to support for better coverage.

  - Source [Qualcomm] states that, LTE TBCC with a coding rate of 1/3 could achieve better performance than 3 repetitions.

For Code rate R=1/2, achieved by puncturing 3rd branch

[Positive views]

- For Code rate R=1/2, sources [vivo], [NOKIA], [NEC], [CATT] and [Apple] report that R=1/2 can be supported.

  - Source [vivo] reports, 1/2 code rate can improve spectrum efficiency and reduce power consumption for decoding.

  - Source [CATT] states that ,1/2 coding rate is preferable for TBCC, as it reduces device power consumption while achieving the same spectral efficiency as the Manchester coding in Rel-19 R2D.

  - Sources [Apple] and [NEC] state that, 1/2 and 1/3 coding rate as baseline.

  - Sources [Futurewei] state that, the performance difference between R=1/2 and R=1/3 is about 2 dB.

[Negative views]

- Source [Huawei] states that, the necessity of supporting other code rates apart from 1/3 for TBCC, which was supported in D2R for Rel-19, is not clear, because a 1/2 code rate performs worse than a 1/3 code rate

For Code rate R=1/4, source [Docomo] reports that Code rate of R=1/4 can be achieved by repetition and puncturing based on LTE TBCC with 1/3 code rate.

For Code rate R=1/4, obtained by adding a new polynomial for the fourth branch

[Positive views]

- Sources [Qualcomm], [Docomo] and [Apple] report that, Code rate R=1/4 obtained by adding a new polynomial for the fourth branch, can be supported.

  - Source [Apple] states that, 1/4 coding rate can be considered for extreme coverage scenarios.

  - Source [Qualcomm] states that, 1/4 coding rate can further improve the performance than that of 1/3 coding rate. The extra polynomial [133 171 165 137], nested to the existing LTE CC (K=7 and R=1/3), is used

  - Source [Futurewei] states that, the performance difference between R=1/3 and R=1/4 is about 1.25 dB.

[Negative views]

- Source [vivo] states that, 1/4 mother code rate increases power consumption and larger buffer for decoding, and it requires large standard effort to determine new polynomial for the 4th branch. The coverage enhancement can also be achieved by 1/3 mother code rate with block-level repetition.

- Source [NOKIA] states that, 1/4 code rate should be deprioritized since it increases complexity.

[Coding scheme assumption]

Source [Futurewei] provide views for above code rates using tailed convolutional code with the same polynomial as LTE TBCC; while other sources provide views for above code rates using LTE TBCC.

Note: whether/how to capture blue parts can be up to editor

Agreement

Capture following observations for sequence length for binary sequence based R2D SIP in the TR 38.769, that

For sequence length for binary sequence based R2D SIP

- Source [vivo] report that

  - At least 31-length m-sequence with M=2 and with Manchester coding (i.e., 31 OFDM symbols), is needed for R2D SIP, assuming 38 dBm Tx power and 20 dB penetration.

  - At least 127-length m-sequence with M=2 and with Manchester coding (i.e., 127 OFDM symbols), is needed for R2D SIP, assuming 33 dBm Tx power and 20 dB penetration.

- Source [Huawei] report that

  - 256, 64 and 16-length Golay sequence-based SIP can achieve an SNR performance of 4.0 dB, 8.2 dB and 12.0 dB, respectively, with the MDR ≤ 1%, FAR ≤ 1% and M=1 without Manchester coding.

- Source [Apple] report that

  - Consider m-sequence based SIP with length 15 or 31 to improve detection performance while maintaining reasonable overhead

- Source [Qualcomm] report that SIP duration with at least 7 OFDM symbols by using 15-length and M=2 without Manchester coding or 31-length and M=2 with Manchester coding may be needed to achieve required SINR for PRDCH.

- Source [NEC] report that single length and single sequence is beneficial for lower complexity and power consumption.

- Source [ZTE] report that the Release 19 SIP sequence with a length of 8 can achieve a BLER of less than 10% at an SNR of 4 dB, and BLER of less than 1% at an SNR of 10 dB.

Agreement

Capture following in TR38.769

Regarding whether L1 R2D control information is included in the PRDCH that carries that data payload, or in another PRDCH

- Sources [vivo], [Qualcomm], [Huawei], [Samsung], [Futurewei], [Docomo], [NEC], [LGE], [Apple], and [Interdigital] state that L1 R2D control is transmitted in the same PRDCH carries data payload for simplicity for standard and device.

  - [Huawei] states if L1 control and data are in different PRDCH, the time relationship design is complicated and the device needs to buffer the L1 control information for a long time which is hard for the device to implement.

- Sources [ZTE], [CMCC], [Transsion], [Panasonic] and [MTK] state that L1 R2D control information is transmitted in a separate PRDCH

  - Source [ZTE] report that, L1 R2D control information is encoded separately, it is more appropriate to transmit L1 R2D control information on a separate PRDCH

  - Sources [CMCC] and [Transsion] state that, defining a new PRDCH format to carry L1 R2D control information only is more efficient for scheduling and resource utilization.

  - Source [Panasonic] state that, L1 control information is transmitted in separated PRDCH since control part would require higher reliability, and one control information could be used for multiple PRDCH transmissions.

Agreement

Regarding necessary information to be included in the L1 R2D control: capture following observations in TR 38.769

In addition to TBS for PRDCH data payload, the following L1 R2D control information, which potentially can be indicated to the device, is studied:

- M value for PRDCH data payload chip duration, as reported by [vivo], [Qualcomm], [CMCC], [OPPO], [Futurewei], [Ericsson], [Docomo], [NOKIA], [NEC], [CATT], [CTC], [Xiaomi], [ZTE], [Transsion], [LGE], [Panasonic], [Apple], [Interdigital], [TCL] and [CEWiT].

  - On the other hand, sources [Spreadtrum], [Huawei], [Ericsson] and [Sequans] suggest to use same chip duration between L1 R2D control information and data payload, hence M value indication is not needed in L1 R2D control.

- Repetition/FEC related parameters for PRDCH data payload, as reported by sources [vivo], [Qualcomm], [Huawei], [CMCC], [OPPO], [Futurewei], [Spreadtrum], [ERICSSON], [Docomo], [NEC], [CATT], [CTC], [ZTE], [Transsion], [LGE], [Apple], [Interdigital], [TCL] and [CEWiT], if Repetition/FEC is supported for R2D.

- Timing related information for PRDCH data payload, as reported by sources [Qualcomm], [CMCC], [OPPO], [NOKIA], [Transsion], [LGE], [Interdigital] and [CEWiT], considering potential need of gap between L1 R2D control and corresponding data payload.

  - On the other hand, sources [vivo], [Huawei], [Samsung], [Spreadtrum], [Docomo], [NEC] and [ZTE] state that, this information is not included in L1 R2D control, because of no need of gap between L1 R2D control and corresponding data payload.

- Timing gap between R2D and the corresponding D2R transmission, as reported by [Qualcomm]

- Frequency resource for PRDCH data payload, as reported by sources [Ericsson], [Quectel] and [CEWiT], if FDM for R2D is supported.

- Message types/Functional indication, as reported by Sources [Qualcomm], [vivo], [OPPO], [Samsung], [Futurewei], [NOKIA], [Xiaomi], [ZTE] and [CMCC], for the purpose of early termination of R2D detection.

- ID related information, which potentially include at least one of reader ID, device ID or Transaction ID related information

  - ID related information associated with device(s)

    - Sources [Futurewei], [vivo], [Qualcomm], [OPPO], [Samsung] and [TCL] state that, ID related information associated with device(s), indicating for which device or group of devices the upcoming data is intended, can be included in L1 R2D control for early termination.

  - Reader ID related information, if needed, as reported by Source [Qualcomm].

- Indication of next R2D transmission, as reported by Source [Futurewei], used to indicate when the next R2D transmission, can help non-intended devices have the opportunity to harvest energy.

Agreement

Regarding the functionality of SFO calibration at device: capture following in TR 38.769

- It is reported by sources [Futurewei], [Spreadtrum], [vivo], [Huawei], [CTC], [CATT], [Samsung], [ZTE], [TCL], [Qualcomm], [Docomo] and [Quectel] that, SFO calibration is necessary for device 2b. And it is reported by sources [Futurewei], [Spreadtrum], [Huawei], [CATT], [TCL], [Qualcomm], [Docomo] and [Quectel] that, SFO calibration is necessary for device C.

- Sources [Spreadtrum], [Huawei], [OPPO], [Samsung], [ZTE], [Apple], and [Quectel] state that the SFO calibration is feasible and can be achieved by Manchester coding for PRDCH. Additionally, source [Huawei] state that this method applies to device 2b.

- Sources [CATT], [Samsung], [Apple], [Quectel] and [TCL] report that SFO is feasible and can be achieved by reusing CAP.

- Sources [vivo], [NEC], [CTC] and [ZTE] report that SFO calibration is feasible and can be achieved by receiving sequence-based SIP.

- Sources [Futurewei], [Spreadtrum], [Huawei], [CATT], [OPPO], [Apple], [Qualcomm], [Quectel], [Sony] and [Docomo] report that, SFO calibration is feasible and can be achieved by CFO calibration signal. Additionally, source [Huawei] state that this method applies to device C.

10.3.2.2	D2R signals, channels, waveform and procedures

Agreement

Rel-19 SFS is not necessary in Rel-20 A-IoT D2R.

Agreement

Regarding D2R modulation schemes, capture following observations based on the table as a starting point.

- Companies are encouraged to report their analysis/observation with assumptions on each modulation/aspect in the table.

- RAN1 does not need to reach consensus on particular value for the observation.

|  | BPSK | OOK | DBPSK | MSK |
| --- | --- | --- | --- | --- |
| Sensitivity to CFO | [Qualitative observation based on analysis on other aspects] | [Qualitative observation based on analysis on other aspects] | [Qualitative observation based on analysis on other aspects] | [Qualitative observation based on analysis on other aspects] |
| Required SNR Performance | Required SNR = [X-Y]dB @ 10% BLER Required SNR = [X]dB @ 1% BLER | Required SNR = [X-Y]dB @ 10% BLER Required SNR = [X]dB @ 1% BLER Theoretical BER performance is 6 dB worse than BPSK | Required SNR = [X-Y]dB @ 10% BLER Required SNR = [X]dB @ 1% BLER Theoretical BER performance is 3 dB worse than BPSK | Required SNR = [X-Y]dB @ 10% BLER Required SNR = [X]dB @ 1% BLER |
| Receiver detection type | Coherent only | Non-coherent | Non-coherent | Coherent/non-coherent |
| Overhead | [X-Y]% (Note 1) | [X-Y]% (Note 1) | [X-Y]% (Note 1) | [X-Y]% (Note 1) |
| Spectrum | [Observation in terms of pulse shaping and/or guard band] | [Observation in terms of pulse shaping and/or guard band] | [Observation in terms of pulse shaping and/or guard band] | [Observation in terms of pulse shaping and/or guard band] |
| Transmitter complexity | Medium complexity. [Differential LO.] | Lower complexity | Medium complexity [Differential encoding. Differential LO] | Higher transmitter complexity (modulation index, rapid frequency shift or IQ branches needed) |
| Receiver complexity | Higher CFO estimation / compensation complexity (e.g., large # of hypothesis needed). Supporting coherent detection is not so complex for reader. | Lower complexity for non-coherent detection. | Medium complexity for CFO estimation / non-coherent detection. Phase estimation is still required for differential detection. | Accurate CFO estimation / compensation required for coherent detection increasing receiver complexity e.g., large # of hypothesis needed |
| Note 1: X-amble overhead is computed for (96+16)bits payload |  |  |  |  |

Agreement

For the study of 1SB/2SB without small frequency shift (SFS), consider two options and capture following observations.

- Option 1) 1SB

  - [HW]1SB signal has only one side band, i.e., lower side band or upper side band, on either side of center frequency.

  - [Xiaomi] [HW][vivo] described 1SB signal generation methods based on phase shift, i.e., Hilbert transform of 2SB baseband signal and filtering of one side band signal from 2SB signal.

  - [vivo] [HW] [ZTE] reported that filtering method could be challenging since two sides are adjacent to each other and it is hard to realize filter with steep roll off.

  - [ZTE] [HW] reported that phase shift method could be challenging because it requires a precise 90-degree phase-shift network, which is non-ideal and highly sensitive to circuit matching and stability.

  - [QC] [Samsung] reported that Tx LO re-tuning to different frequency points may require additional LO retuning/stabilization time and/or power, especially when frequent change is needed.

  - [ZTE] reported that BLER performance is more sensitive to phase errors and frequency offset.

- Option 2) 2SB

  - [HW] 2SB signal has two sidebands, i.e., lower sideband and upper sideband, on either side of center frequency.

  - [QC] [ZTE] [vivo] [CATT] [DCM] [Xiaomi] pointed out that the transmission signal bandwidth of 2SB signal is two times that for 1SB signal in transmitting the same number of bits; 2SB has lower spectral efficiency than 1SB.

  - [ZTE] reported that 2SB modulation is more tolerant to phase error and CFO, offering relatively more robust demodulation performance, due to its symmetric spectrum structure.

  - [QC] [Samsung] reported that Tx LO re-tuning to different frequency points may require additional LO retuning/stabilization time and/or power, especially when frequent change is needed.

Following figure is provided to help the understanding of the two options.

Agreement

Regarding D2R preamble for PDRCH, preamble supports at least following functionality.

- D2R timing acquisition

- FFS: D2R SFO estimation

- D2R CFO estimation considering modulation schemes

  - Companies can report assumed reader CFO estimation/compensation mechanism(s) and its feasibility especially in relation to detection method of modulation schemes.

Agreement

Regarding D2R preamble for PDRCH, study at least one part structure.

- Note: other structure is not precluded.

Agreement

Regarding D2R preamble for PDRCH, at least following preamble sequence types are considered.

- M-sequence

- Gold sequence

Agreement

Regarding D2R preamble for PDRCH, following aspects are considered for preamble sequence length.

- Additional longer preamble length than Rel-19

- Multiple length values considering dependency of required length on modulation schemes, coverage level, etc

Agreement

Regarding D2R preamble for PDRCH, sequence differentiation is considered considering

- Differentiation across e.g., multiple readers, devices, etc.

- 3 or more sequences

Agreement

Regarding D2R midamble for PDRCH, midamble supports at least one or more of following functionalities.

- Channel estimation

- Time tracking

- FFS: SFO estimation

- CFO estimation

  - Companies can report assumed reader CFO estimation/compensation mechanism(s) and its feasibility especially in relation to detection method of modulation schemes.

- FFS: Interference estimation

Agreement

Regarding D2R midamble for PDRCH, study following midamble sequence types.

- m-sequence

- All 1 sequence

- Hadamard sequence

Agreement

Regarding D2R midamble for PDRCH, study following sequence length and interval considering different functionality.

- Additional lengths shorter/longer than that for Rel-19

  - The length for preamble and midamble within a PDRCH can be different.

- Additional intervals shorter/longer than that for Rel-19

Agreement

D2R scrambling is necessary for randomizing interference and preventing long runs of identical bits when Manchester code is not applied. For D2R scrambling, take following factors into consideration.

- Sequence types: Gold sequence, m-sequence

- Complexity

- Scrambling is applied after FEC and repetition, if any.

- Applicable channel/signal: PDRCH, FFS midamble

Agreement

For D2R interleaving and bit-collection, companies can provide evaluation results with observations on performance gain and required memory according to following form and study the feasibility of supporting additional memory for Rel-20 active device.

- LTE subblock interleaving and/or LTE bit collection can provide additional performance gain with increased time diversity gain.

  - [Source] reports that Alt1 provides performance gain of [A~B] dB over Rel-19 scheme

  - [Source] reports that Alt2 provides performance gain of [C~D] dB over Rel-19 scheme.

  - [Source] reports that Alt3 provides performance gain of [E~F] dB over Rel-19 scheme.

- LTE sub-block interleaving and/or LTE bit collection requires additional memory for sub-block interleaving and buffering (i.e., virtual circular buffer)

  - [Source] reports that Alt1 requires memory size [P~Q] times of max TBS.

  - [Source] reports that Alt2 requires memory size [R~S] times of max TBS.

  - [Source] reports that Alt3 requires memory size [T~U] times of max TBS.

- Companies report A, B, C, D, E, F, P, Q, R, S, T, U

- Companies can use following assumption for their evaluation and report other assumptions.

  - TBS = 96 bits, 400 bits

Agreement

Revise the agreement from #122bis as follows.

Agreement Study interleaving for PDRCH with FEC, including the necessity and feasibility of the following: - Alt 1: reuse both LTE sub-block interleaver and LTE bit collection scheme - Alt 2: reuse LTE bit collection scheme without LTE sub-block interleaver - Alt 3: based on LTE bit collection scheme with the following update (e.g., for memory reduction) without LTE sub-block interleaver. - Each sub-block is divided into one or two segments. For one segment case, it is the same as Alt 2. For two segments case, the bit collection is performed such that 1st segment bits from each sub-block are collected first and then 2nd segment bits from each sub-block are collected. - Combination of alternatives can be studied

Agreement

Further study D2R sequence based Msg1, considering following aspects.

- Msg1 sequence type

  - m-sequence

  - Gold sequence

  - others

- Length of sequences

  - 31, 63, others

- Number of sequences

  - <64, 64, 128

- Performance aspect compared Rel-19, e.g.,

  - Impact of CFO and SFO on sequence-based Msg1 detection performance

  - Impact of power control

  - MDR/FAR

  - Modulation

  - Impact to random access procedure

  - Access efficiency gain (e.g., inventory procedure, Msg1)

Agreement

For D2R FEC scheme, study the necessity of a coding rate of 1/2 based on puncturing.

- Puncturing pattern: The third polynomial bits are punctured.

Agreement

For D2R FEC scheme, study the necessity of introducing a lower final coding rate e.g.,

- Option 1) mother code rate 1/4 based on an additional polynomial

- Option 2) repetition of bits from virtual circular buffer based on mother code rate 1/3

Agreement

For D2R repetition, consider introducing additional number of D2R repetitions larger than 2.

- additional numbers of repetitions can be further discussed

- maximum repetition number can be further discussed

Agreement

Further study following modulation schemes.

- BPSK: The input bit “1” is mapped to the real-valued modulation symbol “1” and input bit “0” is mapped to the real-valued modulation symbol “-1” in the baseband.

- OOK: The input bit “1” is mapped to the real-valued modulation symbol “1” and input bit “0” is mapped to the real-valued modulation symbol “0” in the baseband.

- DBPSK: The n-th input bit bn is differentially encoded as dn=bn⊕dn-1, where dn and dn-1 are the n-th and n-1-th differentially encoded input bit and ⊕ is XOR operation. Then, the differentially encoded bits dn are modulated by BPSK.

- MSK: The input bit “1” is mapped to frequency f1 and input bit “0” is mapped to frequency f2, with f2-f1=12T, where T is a symbol duration.

Conclusion

The number of TDM resources for Msg1 is to be discussed in 10.3.2.3.

Timing offset is to be discussed in 10.3.2.3.

10.3.2.3	Other procedures

Agreement

Initial frequency acquisition procedure is necessary for Device 2b and for Device C, at least for flexible deployment of A-IoT frequency location(s) for outdoor scenario.

Agreement

Reader identification/differentiation is necessary for Device 2b and for Device C, at least in outdoor scenario to differentiate R2D among different readers.

Agreement

For initial frequency acquisition for Device 2b and for Device C, a device is expected to determine a frequency point from predefined sync raster points for reception of the periodic synchronization signal.

- Details on the sync raster design is handled by RAN4.

Agreement

- The functionality of broadcast information is to provide the device with at least necessary information for subsequent transmission and reception

- On how A-IoT devices acquire the broadcast information, study the following two cases:

  - Case B1: Single-step acquisition of the broadcast information

  - Case B2: Two-step acquisition of the broadcast information (e.g., MIB-like and then SIB1-like broadcast information)

- On whether to use PRDCH to carry the broadcast information,

  - For Case B1, PRDCH is used to carry the broadcast information.

  - For Case B2

    - PRDCH is used to carry the SIB1-like broadcast information.

    - A separate PRDCH or a new physical channel dedicated to MIB-like is used to carry the MIB-like broadcast information

Agreement

Periodic transmission of broadcast information to provide at least necessary information for subsequent transmission and reception is necessary for Device 2b and for Device C.

- FFS (in SI or WI): whether the transmission of broadcast information is strictly periodic at reader

- FFS (in SI or WI): periodicity of the transmission of broadcast information

Agreement

For the 3 options for the content of the first D2R transmission for DO-A for Device 2b and for Device C, study the following aspects:

- Recommended scenarios (event-triggered or periodic)

- Subsequent transmission(s)/reception(s)/procedure(s) if any after the first D2R transmission for DO-A

- Other aspects

Agreement

On D2R Tx power control for Device 2b and for Device C, capture the following in TR 38.769.

The necessity of D2R Tx power control for Device 2b and Device C for outdoor scenarios were studied and the following observations were provided:

- 22 sources (Futurewei, LGE, Spreadtrum, vivo, Nokia, Semtech, CMCC, CEWiT, BOSCH, NEC, InterDigital, TCL, Apple, China Telecom, ZTE, OPPO, Huawei, Docomo, Ericsson, Xiaomi, Samsung, Qualcomm) reported that the D2R Tx power control for Device 2b and Device C is necessary for outdoor scenarios.

  - 20 sources (Futurewei, LGE, Spreadtrum, vivo, Nokia, Semtech, CMCC, CEWiT, BOSCH, Quectel, InterDigital, TCL, Apple, OPPO, Huawei, Docomo, Ericsson, Xiaomi, Samsung, ZTE) state that it can reduce the interference between different devices (i.e., the Near-Far problem).

  - 14 sources (Futurewei, LGE, Spreadtrum, vivo, Semtech, CMCC, NEC, InterDigital, TCL, Apple, OPPO, Xiaomi, Samsung, ZTE) reported that it helps devices remain available longer.

  - 4 sources (Futurewei, Semtech, Apple, Docomo) reported that the interference due to a transmission from a device to a reader can be reduced at an unintended reader

  - 1 source (Qualcomm) added that depending on device’s peak power consumption and PA support, the D2R Tx power control may not be feasible.

- 1 source (CATT) reported that the D2R Tx power control for Device 2b and Device C is not necessary mentioning in its contribution that the inter-channel interference from different devices is negligible, D2R coverage is limited, and the complexity and the power consumption required at the device side is high.

Agreement

For reader identification/differentiation at least in outdoor scenario to differentiate R2D among different readers, a device acquires the reader ID-related information by at least one of the following options:

- Option 1: Reader ID-related information is contained in the broadcast information.

- Option 2: Reader ID-related information is derived from the periodic sync signal sequence(s).

- Option 3: Reader ID-related information is carried by L1 R2D control information.

- Option 4: Reader ID-related information is contained in the higher-layer information in the paging-like message.

Agreement

For providing necessary information for the first D2R transmission for DO-A for Device 2b and for Device C, following options are considered feasible.

- Option 1: necessary information provided by broadcast information

- Option 2: necessary information provided by a paging-like R2D message

- Option 3: necessary information derived from a synchronization signal and provided by broadcast information

For providing necessary information for the first D2R transmission for DO-A for Device 2b and for Device C, study following option considering feasibility and applicability

- Option 4: necessary information provided by device-specific R2D transmission

2.1.2	Remaining Open issues

RAN1-led

- Study necessary and feasible changes to the Rel-19 air interface to support the above scenarios

  - Air interface for device 2b/C studied for outdoor will be reused for supporting device 2b/C in indoor scenarios

- Study applicability and necessity of Device 2b and Device C to the above scenarios, assuming similar device architectures for Device 2b and Device C.

- While providing clear differentiation with existing 3GPP LPWA IoT technology and the features of 6G relevant to IoT, study outcomes should include achievable cell edge data rate assuming

  - Maximum distance between Reader and Device of 50-500 m

  - Maximum transmit power between -3 dBm and +5 dBm for device C

    - RAN1 to recommend feasible values within this range

  - Maximum transmit power of -20 dBm and -10 dBm for device 2b

Study outcome should include assumptions on the energy harvesting/storage used to obtain the reported data rates and this includes the possibility of capturing results using energy harvesting with energy provided by the reader or by other sources.

- Identify D2R RSRP-like measurement, and the involved A-IoT signal(s)/channel(s), which are feasible for Device 2b/Device C for more accurate outdoor Device localization than based on Reader-ID [RAN1].

  - This objective will start at RAN1#124

2.2	RAN2

2.2.1	Agreements

2.2.2	Remaining Open issues

2.3	RAN3

2.3.1	Agreements

2.3.2	Remaining Open issues

2.4	RAN4

2.4.1	Agreements

2.4.1.1	RAN4#116bis

Topic #1: Workplan and reply LS

Issue 1-1: Workplan

Agreement: Approved with R4-2515151

Issue 1-2: Reply LS

Agreement: No need to send reply LS

Topic #2: Evaluation cases and assumptions for co-existence study

Issue 2-1-1: General

Agreement:

- The research in R20 focus on the coexistence between AIOT and NR

Issue 2-2-1: Deployment scenario

Agreement:

  - The following assumptions are used for Rel-20 SI co-existence simulation calibration

    - Deployment scenario is considered

      - UMa scenario

    - Operating mode is considered with both in-band and standalone

      - In-band: only consider co-located

        - There is no grid shift between A-IoT BS and NR BS

      - Standalone: only consider non-co-located

        - 100% grid shift

        - Note: A-IoT BS and NR BS belonging to different operators are not deployed at the same site. The device may be far from its own cell and closer to the cell of a different operator, resulting in a near-far effect.

    - NR UE only outdoor is considered

    - A-IoT device location is considered as follow

      - Only outdoor

        - 100% outdoor 0 % in cargo trucks

        - Not preclude to provide simulation result in X% in cargo trucks

          - For example, X=20

    - RAN4 consider as follow

      - both Device C and Device 2b

Note. AIoT device 2b and C are simulated separately in the RAN4 co-existence simulation.

Issue 2-2-2: Evaluation cases

Agreement:

  - Evaluation cases for calibration purpose are captured in the follow table.

Table. Evaluation cases

| Operation mode | Device type | Evaluation case number | Aggressor | Victim | Direction |
| --- | --- | --- | --- | --- | --- |
| Standalone | Device 2b | 1 | Device | NR UL | D2R |
|  |  | 2 | NR UL | reader | D2R |
|  |  | 3 | Reader | NR DL | R2D |
|  |  | 4 | NR DL | device | R2D |
|  | Device C | 5 | Device | NR UL | D2R |
|  |  | 6 | NR UL | reader | D2R |
|  |  | 7 | Reader | NR DL | R2D |
|  |  | 8 | NR DL | device | R2D |

  - Evaluation cases for formal simulation are captured in the follow table.

Table. Evaluation cases

| Operation mode | Device type | Evaluation case number | Aggressor | Victim | Direction |
| --- | --- | --- | --- | --- | --- |
| Standalone | Device 2b | 1 | Device | NR UL | D2R |
|  |  | 2 | NR UL | reader | D2R |
|  |  | 3 | Reader | NR DL | R2D |
|  |  | 4 | NR DL | device | R2D |
|  | Device C | 5 | Device | NR UL | D2R |
|  |  | 6 | NR UL | reader | D2R |
|  |  | 7 | Reader | NR DL | R2D |
|  |  | 8 | NR DL | device | R2D |
| In-band | Device 2b | 9 | Device | NR UL | D2R |
|  |  | 10 | NR UL | reader | D2R |
|  |  | 11 | Reader | NR DL | R2D |
|  |  | 12 | NR DL | device | R2D |
|  | Device C | 13 | Device | NR UL | D2R |
|  |  | 14 | NR UL | reader | D2R |
|  |  | 15 | Reader | NR DL | R2D |
|  |  | 16 | NR DL | device | R2D |

Issue 2-3-1-1: Deployment parameters assumptions

Agreement:

Table. Deployment parameters

| Parameter | Values for calibration purpose |
| --- | --- |
| Carrier frequency | 900MHz (Band 8) |
| BW for NR | 10MHz with 15KHz SCS |
| R2D BW | 180kHz, for 15kHz SCS |
| D2R BW | 15kHz |
| Waveform | R2D: OFDM-based OOK D2R: BPSK |
| A-IoT DL power control | No |
| A-IoT UL power control | Use fixed maximum output power |
| Traffic model | Ful buffer |
| NR BS/A-IoT reader deployment (outdoor) | Hexagonal grid, 19 macro sites, 3 sectors per site with wrap around Inter-NR BS/A-IoT reader distance: [X]m - X= 750m (high priority) or 500m NR BS height: 25m the minimum 2D distance between macro NR BS/A-IoT reader and device 2b/C as 35 m. |
| Frequency reuse | option1) 1:3 option2) 1 (High priority) |
| Device distribution | Device Height= 1.5 m A-IoT devices drop uniformly distributed over the horizontal area - 100% outdoor 0 % in cargo trucks - Not preclude to provide simulation result in X% in cargo trucks - For example, X=20 Number of A-IoTs = Total area × activated density (1.5 A-IOT devices/m²) 1 active A-IoT device under one reader at one drop Minimum distance between reader and device is 35m |
| NR UE dropping | For NR UE only outdoor, uniformly distributed outdoor. UE number: - DL active UE: 1 UE per cell - UL active UE: 3 UE per cell |
| MCL for NR BS to NR UE | MCL of 70 dB |
| Pathloss model | Use UMa pathloss model defined in TR38.901 |
| penetration loss | OdB |
| Shadow fading margin | 6dB for UMa NLOS |

Issue 2-3-2-1: A-IoT Reader RF parameter assumptions

Agreement

| A-IoT reader parameters |  | Values for calibration purpose |
| --- | --- | --- |
| A-IoT reader total Tx power | Standalone | 43dBm |
| A-IoT reader receiver Noise Figure (dB) |  | 5dB |
| A-IoT reader antenna gain (dBi) |  | 15dBi including feeder loss |
| ACLR |  | 45dB |
| ACS |  | 46dB |
| Antenna configuration |  | Antenna Array Geometry：BS point at fixed beam direction: vertical: θtilt + 90°, horizontal: 0, 120, 240 ° Antenna pattern (horizontal) (For 3-sector cell sites with fixed antenna patterns): AHϕ=-min12ϕϕ3dB2,Am φ3dB = 65 degrees, Am = 25 dB Antenna pattern (vertical) (For 3-sector cell sites with fixed antenna patterns): AVθ=-min12θ-90°-θetiltθ3dB2,SLAv θ3dB = 10 degrees, SLAv = 25 dB, θetilt= 9 degrees Combining method in 3D antenna pattern: Aφ,θ=-min⁡{-AHφ+AVθ, Am} |

Issue 2-3-3-1: A-IoT Device RF parameters assumptions

Agreement:

| A-IoT device parameters | Values for calibration purpose |  |
| --- | --- | --- |
|  | Device 2b | Device C |
| A-IoT device effective antenna gain per Tx or Rx branch (dBi) | 0 | 0 |
| A-IoT device output power（dBm） | -10 | 5 |
| A-IoT device noise figure (dB) | 15 | 13 |
| ACLR(dB) | 20 | 25 |
| ACS(dB) | 30 | 35 |
| Guard band | 0PRB | 0PRB |
| Antenna configuration | Omni direction antenna | Omni direction antenna |

Issue 2-3-4-1: NR BS RF parameters assumptions

Agreement:

| NR BS Parameter | Values for calibration and formal simulation |
| --- | --- |
| Macro-BS Tx power (dBm) | 46dBm |
| BS antenna pattern | Antenna Array Geometry：BS point at fixed beam direction: vertical: θtilt + 90°, horizontal: 0, 120, 240 ° Antenna pattern (horizontal) (For 3-sector cell sites with fixed antenna patterns): AHϕ=-min12ϕϕ3dB2,Am φ3dB = 65 degrees, Am = 25 dB Antenna pattern (vertical) (For 3-sector cell sites with fixed antenna patterns): AVθ=-min12θ-90°-θetiltθ3dB2,SLAv θ3dB = 10 degrees, SLAv = 25 dB, θetilt= 9 degrees Combining method in 3D antenna pattern: Aφ,θ=-min⁡{-AHφ+AVθ, Am} |
| BS antenna gain (dBi) | 15dBi including feeder loss |
| Height of macro NR BS (m) | 25 |
| NR Macro-BS Noise Figure(dB) | 5 |
| Network location | Outdoor |
| ACLR (in-band) | Reuse R19 AIoT assumption: 30 dBc |
| ACLR (standalone) | Same with legacy NR BS |
| ACS (in-band) | Same with legacy NR BS |
| ACS(standalone) | Same with legacy NR BS |

Issue 2-3-5-1: NR UE RF parameters assumptions

Agreement:

| NR UE Parameter | Values for calibration and formal simulation |
| --- | --- |
| UE TX power in dBm | -40 to 23 dBm |
| NR UE Antenna gain (dBi) | 0dBi |
| Height of UE antenna (m) | 1.5m |
| NR UE Tx emission（dB） (standalone) | Same with NR UE ACLR |
| NR UE Tx emission (dB) (inband) | Same with NR UE IBE |
| NR UE ACS（dB） | 33dB |
| NR UE Noise Figure（dB） | 9dB |
| Antenna configuration | Omni direction antenna |

Topic #3: Performance metric

Issue 3-1: performance metric

Agreement:

- For calibration purpose, consider coupling loss, SINR with ACI, and SINR without ACI

- For formal coexistence simulation,

  - Option 1: Use SINR degradation of [1dB]

  - Option 2: Use SNR from LLS

2.4.1.2	RAN4#117

Topic #1: Evaluation cases and assumptions for formal simulation

Issue 1-2-1: Main issues

Issue 1-2-1-1: Output power for A-IoT reader for in-band operation

- WayForward: Use 38dBm

Issue 1-2-1-2: Mitigation scheme for baseline

- WayForward: Depending on companies as mitigation schemes can be following

  - Frequency reuse

    - Option 1: 3(Baseline)

    - other options is not precluded

  - Other mitigation scheme is not precluded

  - If RAN4 use migration solution, the coordination with other working group may be needed.

Issue 1-2-1-3: ACIR assumption

•	WayForward:

  - Standalone

| Aggressor | Victim | ACLR | ACS |
| --- | --- | --- | --- |
| Device D2R | NR UL | Depending on companies with following option Opton1: 2-step modeling For UE3, XdB For UE1, 2, XdB-10dB Option2: Flat model (Baseline) ACLRe=XdB for all UE | ACSe=46dBc for all UEs. |
| NR UL | Reader D2R | Bandwidth scale factor(F) = 10*log10(17RB /1RE) UE1 -> Reader ACLRe :43dBc+F UE2 -> Reader ACLRe:43dBc+F UE3 -> Reader ACLRe:30dBc+F | ACSe for UE3=X No interference from UE1 and UE2 |
| Reader R2D | NR DL | Depending on companies with following option Option 1: 2-step model ACLR @1st adjacent channel=XdB/180KHz ACLR @other channel=X+13dB Option2: Flat model(Baseline) ACLRe=X | ACS= 33dB |
| NR DL | Device R2D | Bandwidth scale factor = 10*log10(52RB/1RB) = 17.4dB ACLR_e=45+17.4=62.4dB/1RB | ACSe= X |

  - In-band

| Aggressor | Victim | ACLR | ACS |
| --- | --- | --- | --- |
| Device D2R | NR UL | Depending on companies with following options Opton1: 2-step modeling For UE3: XdB For UE1, 2: XdB-10dB Option2: Flat model(Baseline) ACLR_e=XdB for all UE | ACSe=46dBc for all UEs. |
| NR UL | Reader D2R | Bandwidth scale factor = 10*log10(1RB/1RE) = 10.8dB UE1 -> D2R ACLR: IBE+10.8dB UE2 -> D2R ACLR: IBE+10.8dB UE3 -> D2R ACLR: IBE+10.8dB | ACSe for UE2=X ACSe for UE3=X Ignore UE1 |
| Reader R2D | NR DL | Depending on companies with following options Option 1: 2-step model ACLR @1st adjacent channel=XdB/180KHz ACLR @other channel=X+13dB Option2: Flat model(Baseline) ACLRe=X | ACS= 33dB |
| NR DL | Device R2D | Bandwidth scale factor = 10*log10(52RB/1RB) = 17.4dB ACLR=45+17.4=62.4dB/1RB | ACSe =X |

Issue 1-2-1-4: Performance metric

- WayForward:

  - For NR, 5% throughput loss

  - For A-IoT, Companies are encouraged to provide the absolute SINR at 5% and 50% CDF for comparison.

    - The outage can be provided based on the SINR decoding threshold based on company’s report

    - Companies can choose suitable ACIR range and SINR degradation range

Issue 1-2-1-5: Power control

- WayForward: Both options can be provided based on companies’ interests for formal simulation.

  - Option 1(Baseline): Use fixed Tx power without power control

    - Device 2b :-10dBm

    - Device C: 5dBm(Baseline) or 0dBm

  - Option 2: Use power control as following equation

    - Company report CLx-ile, PMax and Rmin

    - Target SNR:15dB(starting point) or other values to be reported

Issue 1-2-2: Deployment scenario

- WayForward: Use following table for formal simulation

- Deployment scenario is considered - UMa scenario - Operating mode is considered with both in-band and standalone - In-band: only consider co-located - There is no grid shift between A-IoT BS and NR BS - Standalone: only consider non-co-located - 100% grid shift - Note: A-IoT BS and NR BS belonging to different operators are not deployed at the same site. The device may be far from its own cell and closer to the cell of a different operator, resulting in a near-far effect. - NR UE only outdoor is considered - A-IoT device location is considered as follow - Only outdoor - 100% outdoor 0 % in cargo trucks - Not preclude to provide simulation result in X% in cargo trucks - For example, X=80 - RAN4 consider as follow - both Device C and Device 2b Note. AIoT device 2b and C are simulated separately in the RAN4 co-existence simulation.

Issue 1-2-3: Evaluation cases

- WayForward:

  - Use following table for formal simulation

| Operation mode | Device type | Evaluation case | Aggressor | Victim | Direction |
| --- | --- | --- | --- | --- | --- |
| Standalone | Device 2b | 1 | Device | NR UL | D2R |
|  |  | 2 | NR UL | reader | D2R |
|  |  | 3 | Reader | NR DL | R2D |
|  |  | 4 | NR DL | device | R2D |
|  | Device C | 5 | Device | NR UL | D2R |
|  |  | 6 | NR UL | reader | D2R |
|  |  | 7 | NR DL | device | R2D |
| In-band | Device 2b | 8 | NR UL | reader | D2R |
|  |  | 9 | NR DL | device | R2D |
|  | Device C | 10 | NR UL | reader | D2R |
|  |  | 11 | NR DL | device | R2D |

Issue 1-2-4: Deployment scenario parameters for D4T1

- WayForward:

  - Use following table for formal simulation

Table. Deployment parameters

| Parameter | Value |
| --- | --- |
| Carrier frequency | 900MHz (Band 8) |
| BW for NR | 10MHz with 15KHz SCS |
| R2D BW | 180kHz, for 15kHz SCS |
| D2R BW | 15kHz |
| Waveform | R2D: OFDM-based OOK D2R: BPSK |
| A-IoT DL power control | No |
| A-IoT UL power control | Follow Issue 1-2-1-5 conclusion |
| Traffic model | Ful buffer |
| NR BS/A-IoT reader deployment (outdoor) | Hexagonal grid, 19 macro sites, 3 sectors per site with wrap around Inter-NR BS/A-IoT reader distance: [X]m - X= 750m (high priority) or 500m NR BS height: 25m the maximum distance between macro NR BS/A-IoT reader and device 2b/C as 50-500 m |
| Frequency reuse | Follow issue 1-2-1-2 conclusion |
| Device distribution | Device Height= 1.5 m A-IoT devices drop uniformly distributed over the horizontal area - 100% outdoor 0 % in cargo trucks - Not preclude to provide simulation result in X% in cargo trucks - For example, X=80 Number of A-IoTs = Total area × activated density (1.5 A-IOT devices/m²) 1 active A-IoT device under one reader at one drop Minimum distance between reader and device is 35m |
| NR UE dropping | For NR UE only outdoor, uniformly distributed outdoor. UE number: - DL active UE: 1 UE per cell - UL active UE: 3 UE per cell |
| MCL for NR BS to NR UE | MCL of 70 dB |
| Pathloss model | Use UMa pathloss model defined in TR38.901 |
| penetration loss | 20dB |
| Shadow fading margin | 6dB for UMa NLOS |

Issue 1-2-5: A-IoT Reader RF parameters

- WayForward

  - Use following table for formal simulation

| A-IoT reader parameters |  | Value |
| --- | --- | --- |
| A-IoT reader total Tx power | Standalone | 43dBm |
|  | In-band | Follow issue 1-2-1-1 conclusion |
| A-IoT reader receiver Noise Figure (dB) |  | 5dB |
| A-IoT reader antenna gain (dBi) |  | 15dBi including feeder loss |
| ACLR |  | Follow issue 1-2-1-3 conclusion |
| ACS |  | Follow issue 1-2-1-3 conclusion |
| Antenna configuration |  | Antenna Array Geometry：BS point at fixed beam direction: vertical: θtilt + 90°, horizontal: 0, 120, 240 ° Antenna pattern (horizontal) (For 3-sector cell sites with fixed antenna patterns): AHϕ=-min12ϕϕ3dB2,Am φ3dB = 65 degrees, Am = 25 dB Antenna pattern (vertical) (For 3-sector cell sites with fixed antenna patterns): AVθ=-min12θ-90°-θetiltθ3dB2,SLAv θ3dB = 10 degrees, SLAv = 25 dB, θetilt= 9 degrees Combining method in 3D antenna pattern: Aφ,θ=-min⁡{-AHφ+AVθ, Am} |

Issue 1-2-6: A-IoT Device RF parameters assumptions

- WayForward:

  - Use following table for formal simulation

| A-IoT device parameters | Value |  |
| --- | --- | --- |
|  | Device 2b | Device C |
| A-IoT device effective antenna gain per Tx or Rx branch (dBi) | 0 | 0 |
| A-IoT device max output power（dBm） | -10 | 5 |
| A-IoT device noise figure (dB) | 15 | 13 |
| ACLR(dB) | Follow issue 1-2-1-3 conclusion | Follow issue 1-2-1-3 Conclusion |
| ACS(dB) | Follow issue 1-2-1-3 conclusion | Follow issue 1-2-1-3 conclusion |
| Guard band | 0PRB | 0PRB |
| Antenna configuration | Omni direction antenna | Omni direction antenna |

Issue 1-2-7: NR BS RF parameters

- WayForward:

  - Use following table for formal simulation

| NR BS Parameter | Value |
| --- | --- |
| Macro-BS Tx power (dBm) | 46dBm |
| BS antenna pattern | Antenna Array Geometry：BS point at fixed beam direction: vertical: θtilt + 90°, horizontal: 0, 120, 240 ° Antenna pattern (horizontal) (For 3-sector cell sites with fixed antenna patterns): AHϕ=-min12ϕϕ3dB2,Am φ3dB = 65 degrees, Am = 25 dB Antenna pattern (vertical) (For 3-sector cell sites with fixed antenna patterns): AVθ=-min12θ-90°-θetiltθ3dB2,SLAv θ3dB = 10 degrees, SLAv = 25 dB, θetilt= 9 degrees Combining method in 3D antenna pattern: Aφ,θ=-min⁡{-AHφ+AVθ, Am} |
| BS antenna gain (dBi) | 15dBi including feeder loss |
| Height of macro NR BS (m) | 25 |
| NR Macro-BS Noise Figure(dB) | 5 |
| Network location | Outdoor |
| ACLR (in-band) | Reuse R19 AIoT assumption: 30 dBc |
| ACLR (standalone) | Same with legacy NR BS |
| ACS (in-band) | No interference caused by ACS impact |
| ACS(standalone) | Same with legacy NR BS |

Issue 1-2-8: NR UE RF parameters

- WayForward:

  - Use following table for formal simulation

| NR UE Parameter | Value |
| --- | --- |
| UE TX power in dBm | -40 to 23 dBm |
| NR UE Antenna gain (dBi) | 0dBi |
| Height of UE antenna (m) | 1.5m |
| NR UE Tx emission（dB） (standalone) | Same with NR UE ACLR |
| NR UE Tx emission (dB) (inband) | Same with NR UE IBE |
| NR UE ACS（dB） (Standalone) | 33dB |
| NR UE ACS（dB） (In-band) | No interference caused by ACS impact |
| NR UE Noise Figure（dB） | 9dB |
| Antenna configuration | Omni direction antenna |

2.4.2	Remaining Open issues

RAN4-led

- Study coexistence between Device 2b/Device C and NR/LTE in the above outdoor scenarios.

- Use band n8 as an example band.

NOTE: Strive to minimize evaluation cases in RAN1 and RAN4.

2.5	RAN5

2.5.1	Agreements

2.5.2	Remaining Open issues

2.5.3	Remaining Open issues with cross-WG dependencies

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

3.1	SAx/CTs

3.1.1	Agreements with cross-TSG impacts

3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN1#122bis

- R1-2507357	Updated work plan for Rel-20 Ambient IoT SI	LG Electronics

10.3.1	Evaluations

- R1-2506747	Discussion on Evaluation Methodology for R20 A-IoT	FUTUREWEI

- R1-2506788	Evaluation for Rel-20 AIoT	Nokia

- R1-2506808	Discussion on evaluation assumptions and results for Ambient IoT	Spreadtrum, UNISOC

- R1-2506827	Discussion on evaluation of active Ambient IoT device	ZTE Corporation, Sanechips

- R1-2506892	Evaluation methodologies, assumptions and results for R20 AIoT	vivo

- R1-2506944	Evaluation for active A-IoT device in outdoor scenario	Huawei, HiSilicon

- R1-2506983	Discussion on evaluation methodology for Ambient IoT in NR outdoor for active devices	Xiaomi

- R1-2507009	Discussion on evaluation assumptions	CMCC

- R1-2507024	Evaluation assumptions for outdoor Ambient IoT	Tejas Network Limited

- R1-2507066	Discussion on Rel-20 A-IoT evaluation assumptions and results	Ericsson

- R1-2507113	Evaluation methodology for A-IoT outdoor deployment scenarios	CATT

- R1-2507170	Discussion on EVM for R20 A-IoT	OPPO

- R1-2507209	Evaluation results for Device 2b&C for Ambient IoT	HONOR

- R1-2507247	Evaluation methodology and assumptions for Rel-20 Ambient IoT	Samsung

- R1-2507299	Evaluations for Ambient IoT	NEC

- R1-2507326	Discussion on evaluations for Ambient IoT	China Telecom

- R1-2507358	Evaluations for Rel-20 Ambient IoT SI	LG Electronics

- R1-2507591	Evaluation of Ambient IoT for outdoor active device	Sony

- R1-2507619	Ambient IoT evaluations	MediaTek Inc.

- R1-2507671	On Rel-20 Ambient IoT evaluations	Apple

- R1-2507715	Evaluations	Qualcomm Incorporated

- R1-2507755	Evaluations for Device 2b/C in Ambient IoT	InterDigital, Inc.

- R1-2507807	Study on evaluations for Ambient IoT outdoor for active device	NTT DOCOMO, INC.

- R1-2507839	Evaluations for outdoor AIoT devices	Indian Institute of Tech (M)

- R1-2507944	Evaluations for Rel-20 AIoT	IIT Kanpur

- R1-2507958	FL summary #1 for Ambient IoT: “10.3.1 Evaluations”	Moderator (Huawei)

- R1-2507959	FL summary #2 for Ambient IoT: “10.3.1 Evaluations”	Moderator (Huawei)

- R1-2507960	FL summary #3 for Ambient IoT: “10.3.1 Evaluations”	Moderator (Huawei)

10.3.2	Study of air interface for Device 2b/C

- R1-2506748	Discussion on Air Interface for Device 2b/C	FUTUREWEI

- R1-2506789	Air interface for Device 2b/C for Rel-20 AIoT	Nokia

- R1-2506809	Discussion on study of air interface for Device 2b/C	Spreadtrum, UNISOC

- R1-2506828	Discussion on air interface of active Ambient IoT device	ZTE Corporation, Sanechips

- R1-2506893	Discussion on air interface for Device 2b/C for R20 AIoT	vivo

- R1-2506945	Study of air interface for A-IoT 2b/C	Huawei, HiSilicon

- R1-2506984	Discussion on the air interface for Device 2b/C	Xiaomi

- R1-2507010	Discussion on air interface for Device 2b/c	CMCC

- R1-2507025	Study the air interface for Device 2b and C for outdoor coverage 	Tejas Network Limited

- R1-2507067	Study of air interface for Device 2b/C	Ericsson

- R1-2507114	Study of A-IoT enhancement for device 2b/C	CATT

- R1-2507171	Discussion on air interface enhancement for device 2b/C	OPPO

- R1-2507190	Discussion on air interface for Device 2b and Device C	Transsion Holdings

- R1-2507210	Views on air interface for Device 2b&C for Ambient IoT	HONOR

- R1-2507248	Study on air interface design for active devices	Samsung

- R1-2507300	Study of air interface for active device	NEC

- R1-2507323	Discussion on air interface for Device 2b/C	Lenovo

- R1-2507327	Discussion on air interface for Device 2b/C for Ambient IoT	China Telecom

- R1-2507359	Air interface for Device 2b/C	LG Electronics

- R1-2507382	Discussion on A-IoT Air Interface for Device 2b/C	Panasonic

- R1-2507463	Views on AIoT outdoor for active devices	Ofinno

- R1-2507501	Discussion on air interface for device 2b and device C	ETRI

- R1-2507568	Discussion on air interface for active devices	Sharp

- R1-2507570	Discussion on Air Interface Enhancements for R20 A-IoT	Fraunhofer HHI, Fraunhofer IIS

- R1-2507592	Air interface for Ambient IoT device type 2b / C	Sony

- R1-2507620	Study of air interface for Device 2b/C	MediaTek Inc.

- R1-2507631	Study of air interface and power control for outdoor active ambient IoT devices	ROBERT BOSCH GmbH

- R1-2507672	On Rel-20 Ambient IoT air interface for device 2b/C	Apple

- R1-2507716	Study of air interface for Device 2b/C	Qualcomm Incorporated

- R1-2507754	Air interface for Device 2b/C in Ambient IoT	InterDigital, Inc.

- R1-2507787	Discussion on A-IoT active device	ASUSTeK

- R1-2507808	Study on air interface for Ambient IoT device 2b/C	NTT DOCOMO, INC.

- R1-2507833	Discussion on air interface for Device 2b/C	Quectel

- R1-2507855	Discussion on air interface for active devices for Ambient IoT	TCL

- R1-2507856	On AIoT air interface enhancement for Rel.20 Device 2b/C	Sequans Communications

- R1-2507887	Study of A-IoT air interface for device 2b/C 	Wiliot Ltd.

- R1-2507899	Discussion on air interface for Device 2b/C	CEWiT

- R1-2507943	Discussion on air interface for active device types	IIT Kanpur

- R1-2508046	FL summary #1 for 10.3.2 “Study of air interface for Device 2b/C	Moderator (LG Electronics)

- R1-2508047	FL summary #2 for 10.3.2 “Study of air interface for Device 2b/C”	Moderator (LG Electronics)

- R1-2508048	FL summary #3 for 10.3.2 “Study of air interface for Device 2b/C”	Moderator (LG Electronics)

- R1-2508049	FL summary #4 for 10.3.2 “Study of air interface for Device 2b/C”	Moderator (LG Electronics)

- R1-2508103	FL summary #5 for 10.3.2 “Study of air interface for Device 2b/C”	Moderator (LG Electronics)

- R1-2508104	FL summary #6 for 10.3.2 “Study of air interface for Device 2b/C”	Moderator (LG Electronics)

- R1-2508129	FL summary #7 for 10.3.2 “Study of air interface for Device 2b/C”	Moderator (LG Electronics)

- R1-2508130	FL summary #8 for 10.3.2 “Study of air interface for Device 2b/C”	Moderator (LG Electronics)

- R1-2508175	FL summary #9 for 10.3.2 “Study of air interface for Device 2b/C” Moderator (LG Electronics)

RAN1#123

- R1-2508475	Skeleton of update to TR 38.769	Huawei (editor)

- R1-2508475	Skeleton of update to TR 38.769	Huawei (editor)

- R1-2509565	Skeleton of update to TR 38.769	Huawei (editor)

10.3.1	Evaluations

- R1-2509499	Ambient IoT evaluation results spreadsheet for TR38.769	Moderator (Huawei)

- R1-2509500	Summary of Ambient IoT evaluation results for TR38.769	Moderator (Huawei)

- R1-2508742	FL summary #1 for Ambient IoT: “10.3.1 Evaluations”	Moderator (Huawei)

- R1-2508743	FL summary #2 for Ambient IoT: “10.3.1 Evaluations”	Moderator (Huawei)

- R1-2508744	FL summary #3 for Ambient IoT: “10.3.1 Evaluations”	Moderator (Huawei)

- R1-2508329	Evaluations for R20 A-IoT	FUTUREWEI

- R1-2508379	Evaluations for outdoor Ambient IoT	Spreadtrum, UNISOC

- R1-2508423	Evaluation on Coverage for R20 AIoT	vivo

- R1-2508438	Evaluation for Rel-20 AIoT	Nokia

- R1-2508447	Discussion on evaluation results	CMCC

- R1-2508499	Evaluation for active A-IoT device in outdoor scenario	Huawei, HiSilicon

- R1-2508511	Discussion on Rel-20 A-IoT evaluation assumptions and results	Ericsson

- R1-2508542	Evaluations for Ambient IoT	NEC

- R1-2508588	Evaluation methodology for A-IoT outdoor deployment scenarios	CATT

- R1-2508675	Discussion on evaluation methodology for Ambient IoT in NR outdoor for active devices				Xiaomi

- R1-2509453	Discussion on evaluation methodology for Ambient IoT in NR outdoor for active devices				Xiaomi	(Revision of R1-2508675)

- R1-2508718	Discussion on EVM for R20 A-IoT	OPPO

- R1-2508758	Evaluation assumptions for outdoor Ambient IoT	Tejas Network Limited

- R1-2508793	Evaluations for Rel-20 Ambient IoT	Samsung

- R1-2508817	Discussion on evaluation for  active Ambient IoT device	ZTE Corporation, Sanechips

- R1-2508899	Evaluations for Rel-20 Ambient IoT SI	LG Electronics

- R1-2509101	On Rel-20 Ambient IoT evaluations	Apple

- R1-2509119	Evaluations for Active Devices in Ambient IoT	InterDigital, Inc.

- R1-2509153	Evaluation aspects for Ambient IoT	MediaTek Inc.

- R1-2509222	Evaluations for Ambient IoT in NR outdoor for active devices	Qualcomm Incorporated

- R1-2509271	Study on evaluations for Ambient IoT outdoor for active device	NTT DOCOMO, INC.

- R1-2509327	Evaluation for Rel-20 AIoT	IIT Kanpur 		(Late submission)

- R1-2509421	Initial evaluations of Ambient IoT for outdoor devices	Sony

10.3.2	Study of air interface for Device 2b/C

- R1-2508967	Discussion on air interface for device 2b and device C	ETRI

- R1-2508980	Discussion on Air Interface Enhancements for R20 A-IoT	Fraunhofer HHI, Fraunhofer IIS

- R1-2508991	Views on air interface for Device 2b&C for Ambient IoT	HONOR

10.3.2.1	R2D signals, channels, waveform and procedures

- R1-2509510	FL summary #1 on AI 10.3.2.1 R2D Aspects for R20 AIoT	Moderator (vivo)

- R1-2509511	FL summary #2 on AI 10.3.2.1 R2D Aspects for R20 AIoT	Moderator (vivo)

- R1-2509512	FL summary #3 on AI 10.3.2.1 R2D Aspects for R20 AIoT	Moderator (vivo)

- R1-2509513	FL summary #4 on AI 10.3.2.1 R2D Aspects for R20 AIoT	Moderator (vivo)

- R1-2509514	FL summary #5 on AI 10.3.2.1 R2D Aspects for R20 AIoT	Moderator (vivo)

- R1-2509613	FL summary #6 on AI 10.3.2.1 R2D Aspects for R20 AIoT	Moderator (vivo)

- R1-2509614	FL summary #7 on AI 10.3.2.1 R2D Aspects for R20 AIoT	Moderator (vivo)

- R1-2508330	Discussion on R2D Air Interface for Device 2b/C	FUTUREWEI

- R1-2508380	Discussion on R2D signals, channels, waveform and procedures for Ambient IoT	Spreadtrum, UNISOC

- R1-2508424	Discussion on R2D Aspects for R20 AIoT	vivo

- R1-2508439	AIoT R2D signals, channels, waveform and procedures	Nokia

- R1-2508448	Discussion on R2D signals, channels, waveform and procedures	CMCC

- R1-2508500	Study on R2D signals, channels, waveform and procedures	Huawei, HiSilicon

- R1-2508512	R2D signals, channels, waveform, and procedures	Ericsson

- R1-2508543	Study on R2D signals, channels, waveform and procedures	NEC

- R1-2508589	Study of R2D signals, channels, and waveform and procedure of A-IoT enhancement for device 2b/C		CATT

- R1-2508604	Discussion on R2D procedures for Ambient IoT	China Telecom

- R1-2508676	Discussion on R2D signals, channels, waveform and procedures for Device 2b/C		Xiaomi

- R1-2508719	Discussion on necessary and feasible change to R2D for Rel-20 A-IoT	OPPO

- R1-2508794	Study on R2D aspects of air interface for Device 2b/C	Samsung

- R1-2508818	Discussion on R2D design for active Ambient IoT device	ZTE Corporation, Sanechips

- R1-2508834	Discussion on R2D  transmission	Transsion Holdings

- R1-2508900	R2D air interface for Device 2b/C	LG Electronics

- R1-2508914	Discussion on A-IoT Air Interface for R2D	Panasonic

- R1-2508957	Discussion on R2D for R20 Ambient IoT	Lenovo

- R1-2509102	On R2D design details for device 2b/C	Apple

- R1-2509120	R2D Design for Active Devices in Ambient IoT	InterDigital, Inc.

- R1-2509154	R2D aspects for Ambient IoT	MediaTek Inc.

- R1-2509174	Discussion on R2D signals, channels, waveform and procedures	Sharp

- R1-2509194	 Discussion on R2D signals, channels, waveform and procedures 	Quectel

- R1-2509223	Study of R2D designs for Device 2b/C	Qualcomm Incorporated

- R1-2509272	Study on R2D design and procedures for Ambient IoT outdoor for active device	NTT DOCOMO, INC.

- R1-2509323	Discussion on R2D signals, channels, waveform and procedures for Ambient IoT	TCL

- R1-2509329	Discussion on R2D signals, channels, waveform and procedures	IIT Kanpur

- R1-2509343	Discussion on R2D signals, channels, waveform and procedures for Ambient IoT	CEWiT

- R1-2509380	On AIoT air interface R2D design for Rel.20 Device 2b/C	Sequans Communications

- R1-2509422	Consideration on R2D signals and procedures	Sony

10.3.2.2	D2R signals, channels, waveform and procedures

- R1-2509463	RAN1#123 FL Summary #1 for 10.3.2.2. D2R signals, channels, waveform and procedures	Moderator (Qualcomm)

- R1-2509464	RAN1#123 FL Summary #2 for 10.3.2.2. D2R Signals, channels, waveform and procedures	Moderator (Qualcomm)

- R1-2509465	RAN1#123 FL Summary #3 for 10.3.2.2. D2R Signals, channels, waveform and procedures	Moderator (Qualcomm)

- R1-2509466	RAN1#123 FL Summary #4 for 10.3.2.2. D2R Signals, channels, waveform and procedures	Moderator (Qualcomm)

- R1-2509467	RAN1#123 FL Summary Final for 10.3.2.2. D2R Signals, channels, waveform and procedures	Moderator (Qualcomm)

- R1-2508331	Discussion on D2R Air Interface for Device 2b/C	FUTUREWEI

- R1-2508349	Discussion on D2R transmissions for active ambient IoT	TCL

- R1-2508381	Discussion on D2R signals, channels, waveform and procedures for Ambient IoT	Spreadtrum, UNISOC

- R1-2508425	Discussion on D2R Aspects for R20 AIoT	vivo

- R1-2508440	AIoT D2R signals, channels, waveform and procedures	Nokia

- R1-2508449	Discussion on D2R signals, channels, waveform and procedures	CMCC

- R1-2508501	Study on D2R signals, channels, waveform and procedures	Huawei, HiSilicon

- R1-2508513	D2R signals, channels, waveform, and procedures	Ericsson

- R1-2508544	Study on D2R signals, channels, waveform and procedures	NEC

- R1-2508590	Study of D2R signals, channels, and waveform and procedure of A-IoT enhancement for device 2b/C		CATT

- R1-2508605	Discussion on D2R procedures for Ambient IoT	China Telecom

- R1-2508677	Discussion on D2R signals, channels, waveform and procedures for Device 2b/C	Xiaomi

- R1-2508720	Discussion on Rel-20 A-IoT D2R for Device 2b/C	OPPO

- R1-2508795	Study on D2R aspects of air interface for Device 2b/C	Samsung

- R1-2508819	Discussion on D2R design for active Ambient IoT device	ZTE Corporation, Sanechips

- R1-2508835	Discussion on D2R transmission	Transsion Holdings

- R1-2508901	D2R air interface for Device 2b/C	LG Electronics

- R1-2508915	Discussion on A-IoT Air Interface for D2R	Panasonic

- R1-2508958	Discussion on D2R for R20 Ambient IoT	Lenovo

- R1-2509103	On D2R design details for device 2b/C	Apple

- R1-2509121	D2R Design for Active Devices in Ambient IoT	InterDigital, Inc.

- R1-2509175	Discussion on D2R signals, channels, waveform and procedures	Sharp

- R1-2509189	Discussion on D2R multiplexing/multiple access for A-IoT active device	ASUSTeK

- R1-2509224	Study of D2R designs for Device 2b/C	Qualcomm Incorporated

- R1-2509273	Study on D2R design and procedures for Ambient IoT outdoor for active device	NTT DOCOMO, INC.

- R1-2509330	Discussion on D2R signals, channels, waveform and procedures	IIT Kanpur

- R1-2509344	Discussion on D2R signals, channels, waveform and procedures for Ambient IoT	CEWiT

- R1-2509423	D2R signals, waveforms and procedures for Ambient IoT outdoor devices	Sony

10.3.2.3	Other procedures

- R1-2508903	FL summary #1 for 10.3.2.3 “Other procedures for Device 2b/C”	Moderator (LG Electronics)

- R1-2508904	FL summary #2 for 10.3.2.3 “Other procedures for Device 2b/C”	Moderator (LG Electronics)

- R1-2508905	FL summary #3 for 10.3.2.3 “Other procedures for Device 2b/C”	Moderator (LG Electronics)

- R1-2508906	FL summary #4 for 10.3.2.3 “Other procedures for Device 2b/C”	Moderator (LG Electronics)

- R1-2508907	FL summary #5 for 10.3.2.3 “Other procedures for Device 2b/C”	Moderator (LG Electronics)

- R1-2509587	FL summary #6 for 10.3.2.3 “Other procedures for Device 2b/C”	Moderator (LG Electronics)

- R1-2509588	FL summary #7 for 10.3.2.3 “Other procedures for Device 2b/C”	Moderator (LG Electronics)

- R1-2508332	Discussion on Other Air Interface Procedures	FUTUREWEI

- R1-2508382	Discussion on other procedure for Ambient IoT	Spreadtrum, UNISOC

- R1-2508426	Discussion on Other Procedures for R20 AIoT	vivo

- R1-2508441	Other procedures for AIoT	Nokia

- R1-2508450	Discussion on other procedures	CMCC

- R1-2508502	Study on other procedures	Huawei, HiSilicon

- R1-2508514	Other procedures of A-IoT	Ericsson

- R1-2508545	Study on other procedures for Device 2b and C	NEC

- R1-2508591	Study of general procedures of A-IoT enhancement for device 2b/C	CATT

- R1-2508606	Discussion on other procedures for Ambient IoT	China Telecom

- R1-2508678	Discussion on other procedures for Device 2b/C	Xiaomi

- R1-2508721	Discussion on other procedures for Rel-20 A-IoT	OPPO

- R1-2508759	Study AIoT air interface for DO-A traffic	Tejas Network Limited

- R1-2508796	Study on other procedures of air interface for Device 2b/C	Samsung

- R1-2508820	Discussion on other procedures for active Ambient IoT device	ZTE Corporation, Sanechips

- R1-2508836	Discussion on other aspects of Device 2b_C	Transsion Holdings

- R1-2508860	Discussion on A-IoT other procedures	Panasonic

- R1-2508902	Other procedures for Device 2b/C	LG Electronics

- R1-2508959	Discussion on other procedures for R20 Ambient IoT	Lenovo

- R1-2509041	Procedures for Air Interface of Devices 2b/C	ROBERT BOSCH GmbH

- R1-2509104	On other procedures for device 2b/C	Apple

- R1-2509122	Other Procedures for Active Devices in Ambient IoT	InterDigital, Inc.

- R1-2509123	Considerations for A-IoT other procedures	Semtech Neuchatel SA

- R1-2509155	Other procedures for Ambient IoT	MediaTek Inc.

- R1-2509176	Discussion on other procedures	Sharp

- R1-2509190	Discussion on procedures for A-IoT active device	ASUSTeK

- R1-2509195	 Discussion on other procedures for Device 2b/C 	Quectel

- R1-2509225	Study of other procedures for Device 2b/C	Qualcomm Incorporated

- R1-2509274	Study on other procedures for Ambient IoT outdoor for active device	NTT DOCOMO, INC.

- R1-2509324	Discussion on other procedures for Ambient IoT	TCL

- R1-2509332	Discussion on other aspects of Rel-20 AIoT	IIT Kanpur

- R1-2509345	Discussion on other procedures for Ambient IoT	CEWiT

- R1-2509424	System information and DO-A aspects for Ambient IoT outdoor devices	Sony

- R1-2509432	Discussion on other procedures for Device 2b/C	KT Corp.

RAN4#116bis

7.11.1 Moderator summary and conclusions

- R4-2514444	Topic summary for [116bis][321] Study of A-IoT for outdoor	Moderator (LGE)

- R4-2515070	Ad-hoc meeting minutes for [116bis][321] Study of A-IoT for outdoor	LGE

- R4-2515150	Offline meeting minutes for [116bis][321] Study of A-IoT for outdoo	LGE

- R4-2515152	Way Forward for [116bis][321] Study of A-IoT for outdoor	LGE

7.11.2 General aspects and workplan

- R4-2513451	Draft reply LS on BS Tx power assumption for coverage evaluation in A-IoT	Xiaomi

- R4-2513542	Cross-Frequency Channel Correlation Modelling for Inter-Frequency RSRP Prediction	NTU

- R4-2514154	Discussion on RF requirements for A-IoT device	Ericsson Korea Partners Co Ltd

- R4-2514255	Ambient IoT Study Item work plan	LG Electronics UK

- R4-2514312	Discussion on PSD limitation for A-IOT in band operation	Apple

- R4-2514313	LS reply on BS Tx power assumption for coverage evaluation in A-IoT	Apple

- R4-2514368	LS reply on the inband PSD power	Ericsson

- R4-2515151	Ambient IoT Study Item work plan	LG Electronics UK

7.11.3 Coexistence between Device 2b/Device C and NR/LTE in the above outdoor scenarios

- R4-2513180	Considerations on coexistence between AIoT and NR/LTE in outdoor scenarios	CATT

- R4-2513384	Discussion on R20 A-IoT SI coexistence scenarios and simulation parameters	Huawei, HiSilicon

- R4-2513452	Discussion on R20 AIoT co-existence simulation assumptions	Xiaomi

- R4-2513561	Discussion on the co-existence assumption for outdoor device	vivo

- R4-2513618	On R20 Ambient IOT co-existence	Apple

- R4-2513672	Discussion on co-existence evaluation for Rel-20 Ambient IoT outdoor SID	Spreadtrum,UNISOC

- R4-2513694	Discussion on Ambient IoT Coexistence in outdoor scenario	Qualcomm Incorporated

- R4-2513743	General consideration on R20 A-IoT Coexistence	CMCC

- R4-2513937	Coexistence assumptions for outdoor AIoT system	Sony

- R4-2514256	Discussion on co-existence evaluation cases and assumption for A-IoT outdoor scenario	LG Electronics UK

- R4-2514277	Discussion on Rel-20 A-IoT coexistence study	ZTE Corporation, Sanechips

- R4-2514369	A-IoT coeixsting simulation assumptions	Ericsson

RAN4#117

7.11.1 Moderator summary and conclusions

- R4-2522233	Topic summary for [117][321] Study of A-IoT for outdoor	Moderator (LGE)

- R4-2523029	Ad-hoc meeting minutes for [117][321] Study of A-IoT for outdoor	LGE

- R4-2523061	Way Forward for [117][321] Study of A-IoT for outdoor	LGE

7.11.3 Coexistence between Device 2b/Device C and NR/LTE in the outdoor scenarios

- R4-2520099	Further Considerations on coexistence between AIoT and NR/LTE in outdoor scenarios	CATT

- R4-2520414	Discussion on R20 A-IoT SI coexistence simulation parameters	Huawei, HiSilicon

- R4-2520722	Further views on the co-existence assumption for outdoor device	vivo

- R4-2520759	Consideration on R20 A-IoT Coexistence	CMCC

- R4-2520882	The remaining issues for R20 AIoT co-existence simulation assumption	Xiaomi

- R4-2520883	R20 AIoT co-existence simulation calibration results	Xiaomi

- R4-2521255	Discussion on Ambient IoT Coexistence Assumptions in Outdoor	Qualcomm Incorporated

- R4-2521417	Collection of calibration simulation results for A-IoT	LG Electronics UK

- R4-2521518	Discussion on co-existence evaluation for A-IoT outdoor scenario	LG Electronics UK

- R4-2521754	Discussion on Rel-20 A-IoT coexistence study	ZTE Corporation, Sanechips

- R4-2522001	A-IoT coeixsting simulation initial result	Ericsson

- R4-2522002	A-IoT coeixsting simulation assumptions	Ericsson

- R4-2522042	Further analysis on coexistence of outdoor AIoT system	Sony

- R4-2522137	Initial coexistence simulatoion results for outdoor AIoT	Huawei, HiSilicon

17.10.2025		minor adaptations for RAN #110

10.08.2025		minor adaptations for RAN #109

06.04.2025		minor adaptations for RAN #108

04.02.2025		minor adaptations for RAN #107

17.10.2024		minor adaptations for RAN #106 and 2.6 for RAN6 removed

12.08.2024		minor adaptations for RAN #105

30.04.2024		minor adaptations for RAN #104

16.02.2024		minor adaptations for RAN #103

10.11.2023		minor adaptations for RAN #102

02.08.2023		minor adaptations for RAN #101

26.04.2023		minor adaptations for RAN #100

01.02.2023		minor adaptations for RAN #99

27.10.2022		minor adaptations for RAN #98e

01.08.2022		minor adaptations for RAN #97e

21.05.2022		minor adaptations for RAN #96

10.01.2022		minor adaptations for RAN #95e

04.10.2021		minor adaptations for RAN #94e

08.08.2021		minor adaptations for RAN #93e

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template