3GPP TSG-RAN4 Meeting #117	R4-2520473

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 6059 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for LTM event-triggered L1 reporting |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_Ph4-Core |  |  |  |  |  | Date: |  |  | 2025-11-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Clarification of earlier change and addition of spec reference |  |  |  |  |  |  |  |  |
| Summary of change: |  | This is a resubmission of RAN4#116bis endorsed draft CR R4-2514052. Changes: - Clarify the agreed applicability rule related to event-triggered reporting and on leave reporting: - Event-triggered L1 report for LTM candidate cell may be SSB or CSI-RS based, so change “SSB” to “RS” to cover both. - Clarify that the on leave report needs to be for the same RS as the event-triggered report, because without this clarification any on leave report even for a different candidate cell would be covered, which does not make sense. - Add MAC spec reference for the event-triggered L1 reporting MAC-CE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inaccurate requirements |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.1.1, 8.20.1, 8.25.1, 9.14.3.4, 9.14.3.5, 9.15.3.4, 9.15.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### Change 1

### 6.3.1	LTM PCell Cell Switch

#### 6.3.1.1	Introduction

The purpose of LTM cell switch is to switch the PCell or PSCell to a target cell indicated in LTM cell switch command. The requirements in this section are applicable to LTM PCell switch.

The requirements in this clause are applicable to SA and NR-DC, and to both intra-frequency and inter-frequency LTM cell switch.

The requirements for inter-frequency cell switch in this clause are only applicable, when

-	network has configured UE to perform SSB based L3 measurement with beam measurement reporting or L1 measurement for the target cell before the cell switch command, or

-	the SFN of the serving cell from which cell switch command is received and the SFN of the target cell are the same.

The requirements in this clause are applicable to SA for the following scenarios:

PCell switch to a neighbouring LTM candidate cell

-	FR1 cell to FR1 cell

-	FR1 cell to FR2 cell

-	FR2 cell to FR2 cell

-	FR2 cell to FR1 cell

PCell switch to an LTM candidate cell that is a serving SCell in MCG

-	FR1 cell to FR1 cell

-	FR2 cell to FR2 cell

The requirements in this clause are applicable to NR-DC for the following scenarios:

PCell switch to a neighbouring LTM candidate cell

-	FR1 cell to FR1 cell

PCell switch to an LTM candidate cell that is a serving SCell in MCG

-	FR1 cell to FR1 cell

The requirements in this section apply when

-	the target cell is known, and

-	the target joint UL/DL TCI state or separate UL and DL TCI states in the MAC-CE LTM cell switch command are known, or

-	the target cell is an FR1 cell, and the UE has reported L3-RSRP measurement result with SSB index associated to the target TCI state within 5 seconds before receiving the LTM cell switch command, and SNR of the SSB associated to the target TCI state ≥ -3dB, or

-	the target cell is an FR1 cell, and the UE is configured with event-triggered L1 measurement reporting and report on leave, and the UE has sent an event-triggered L1 measurement report with RS associated to the target TCI state and has not sent report on leave for the same RS before receiving the cell switch command.

-	PL-RS is QCLed with RS of the TCI state indicated by LTM cell switch command, and

-	For CFRA cell switch, RS of the TCI state indicated by LTM cell switch command is QCLed with the SSB indicated for CFRA.

The target cell in the LTM cell switch command is known if the following conditions are met:

-	During the last 5 seconds before the reception of the cell switch command:

-	The UE has sent a valid L1 or L3 measurement report for the target cell, and

-	One of the SSBs measured from the NR target cell configured for measurement remains detectable according to the cell identification conditions specified in clause 9.2 for intra-frequency cell and in clause 9.3 for inter-frequency cell,

-	One of the SSBs measured from the NR target cell configured for measurement remains detectable according to the cell identification conditions specified in clause 9.2 for intra-frequency cell and in clause 9.3 for inter-frequency cell,

Otherwise, the cell is unknown.

The target joint DL/UL TCI state or separate DL and UL TCI states in the LTM cell switch command are known if the following conditions are met:

-	During the period from the last transmission of the RS resource used for the L1-RSRP measurement reporting for the target DL/UL TCI state to the completion of LTM cell switch, where the RS resource for L1-RSRP measurement is the RS in target DL/UL TCI state or QCLed to the target DL/UL TCI state

-	LTM cell switch command is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement

-	The UE has sent at least 1 L1-RSRP report for the target DL/UL TCI state before the LTM cell switch command

-	The target DL/UL TCI state remains detectable during the LTM cell switching period

-	The SSB associated with the target DL/UL TCI state remain detectable during the cell switching period

-	SNR of the TCI state ≥ -3dB

Otherwise, the target joint DL/UL TCI state or separate DL and UL TCI state is unknown.

### Change 2

## 8.20	LTM PSCell Cell Switch

### 8.20.1	Introduction

The purpose of LTM cell switch is to switch the PCell or PSCell to another cell. The requirements in this clause are applicable to LTM PSCell switch.

The requirements in this clause are applicable to both intra-frequency and inter-frequency LTM cell switch.

The requirements for inter-frequency cell switch in this clause are only applicable, when

-	network has configured UE to perform SSB based L3 measurement with beam measurement reporting or L1 measurement for the target cell before cell switch command, or

-	the SFN of the serving cell from which cell switch command is received and the SFN of the target cell are the same.

The requirements in this clause are applicable to NR-DC for the following scenarios:

PSCell switch to a neighbouring LTM candidate cell

-	FR1 cell to FR1 cell

-	FR1 cell to FR2 cell

-	FR2 cell to FR2 cell

-	FR2 cell to FR1 cell

PSCell switch to an LTM candidate cell that is a serving SCell in SCG

-	FR1 cell to FR1 cell

-	FR2 cell to FR2 cell

The requirements in this section apply when

-	the target cell is known, and

-	the target joint UL/DL TCI state or separate UL and DL TCI states in the MAC-CE LTM cell switch command are known, or

-	the target cell is an FR1 cell, and the UE has reported L3-RSRP measurement result with SSB index associated to the target TCI state within 5 seconds before receiving the LTM TCI state activation command, and SNR of the SSB associated to the target TCI state ≥ -3dB, or

-	the target cell is an FR1 cell, and the UE is configured with event-triggered L1 measurement reporting and report on leave, and the UE has sent an event-triggered L1 measurement report with RS associated to the target TCI state and has not sent report on leave for the same RS before receiving the cell switch command.

-	PL-RS is QCLed with RS of the TCI state indicated by LTM cell switch command, and

-	For CFRA cell switch, RS of the TCI state indicated by LTM cell switch command is QCLed with the SSB indicated for CFRA.

The definition of known LTM target cell and the definition of known target joint DL/UL TCI state or separate DL and UL TCI states are as stated in clause 6.3.1.2.

### Change 3

## 8.25	TCI state activation for LTM candidate cell

### 8.25.1	Introduction

The requirements in this clause apply for a UE configured with one or more joint or separate uplink/downlink TCI state configurations on LTM candidate cell. UE shall complete the activation of the target joint or separate uplink/downlink TCI states within the delay defined in this clause after receiving a MAC CE indicating LTM candidate cell TCI state activation before cell switch command as specified in clause 6.1.3.75 of TS 38.321 [7].

The requirements in this clause apply for

-	Known downlink TCI state in FR1 and FR2 specified in clause 8.25.2, and

-	Unknown downlink TCI state in FR1 specified in clause 8.25.2, provided the following conditions are met:

-	the UE is configured with event-triggered L1 measurement reporting and report on leave, and the UE has sent an event-triggered L1 measurement report with RS associated to the target TCI state and has not sent report on leave for the same RS before receiving the TCI state activation command, or

-	UE has reported L3 measurement result with the associated SSB index of the downlink TCI state within 5 seconds before the LTM TCI state activation command, and

-	SNR of the associated SSB is above -3 dB.

### Change 4

#### 9.14.3.4	Event Triggered Reporting

Reported L1-RSRP measurements contained in an event triggered L1-RSRP measurement report shall meet the requirements in clauses 10.1.19/10.1.19D for FR1 and 10.1.20/10.1.20A for FR2, respectively. This requirement applies for event triggered L1-RSRP reports sent on PUSCH using the MAC CE specified in clause 6.1.3.84 of TS 38.321 [7].

The UE shall not send any event triggered measurement reports if no reporting criteria is fulfilled.

The event triggered measurement reporting delay is defined as the time between an event that will trigger an event triggered measurement report and the point when the UE starts to transmit the first UL transmission to report the measurement results over the air interface. This measurement reporting delay excludes a delay which is caused by no UL resources being available for UE to send the measurement report on.

The event triggered measurement reporting delay shall be no larger than the maximum L1-RSRP measurement period of the cells corresponding to the event.

If serving cell is involved in event evaluation, L1-RSRP measurement period of serving cell refers to TL1-RSRP_Measurement_Period_SSB as defined in 9.5, assuming TReport = 0 and TSSB is the periodicity of the serving cell SSB-Index configured for event triggered L1-RSRP measurement report, which is indicated by the ssb-periodicityServingCell

If neighbor cell is involved in event evaluation, L1-RSRP measurement period of neighbor cell refers to TL1-RSRP_Measurement_Period_SSB_intra for SSB based intra-frequency measurement, as defined in 9.14, assuming TReport = 0 and TSSB_NBC is the periodicity of the neighbor cell SSB-Index configured for intra-frequency event-triggered L1-RSRP measurement report.

#### 9.14.3.5	Event-triggered Periodic Reporting

Reported L1-RSRP measurements contained in an event triggered L1-RSRP measurement report shall meet the requirements in clauses 10.1.19/10.1.19D for FR1 and 10.1.20/10.1.20A for FR2, respectively. This requirement applies for event triggered L1-RSRP reports sent on PUSCH using the MAC CE specified in clause 6.1.3.84 of TS 38.321 [7].

The first report in event triggered periodic measurement reporting shall meet the requirements specified in clause 9.14.3.4. For the subsequent reports in the event triggered periodic report, UE follows the reportInterval-r19 and reportAmount-r19 as specified in TS 38.331.

### Change 5

#### 9.15.3.4	Event Triggered Reporting

Reported L1-RSRP measurements contained in an event triggered L1-RSRP measurement report shall meet the requirements in clauses 10.1.19/10.1.19E for FR1 and 10.1.20/10.1.20B for FR2, respectively. This requirement applies for event triggered L1-RSRP reports sent on PUSCH using the MAC CE specified in clause 6.1.3.84 of TS 38.321 [7].

The UE shall not send any event triggered measurement reports if no reporting criteria is fulfilled.

The event triggered measurement reporting delay is defined as the time between an event that will trigger an event triggered measurement report and the point when the UE starts to transmit the first UL transmission to report measurement result over the air interfac. This measurement reporting delay excludes a delay which is caused by no UL resources being available for UE to send the measurement report on.

The event triggered measurement reporting delay shall be no larger than the maximum L1-RSRP measurement period of the cells corresponding to the event.

If serving cell is involved in event evaluation, L1-RSRP measurement period of the serving cell refers to TL1-RSRP_Measurement_Period_SSB as defined in clause 9.5, where TReport = 0 and TSSB is the periodicity of the serving cell SSB-Index configured for event triggered L1-RSRP measurement report, which is indicated by the ssb-periodicityServingCell.

If an inter-frequency neighbor cell is involved in event evaluation, L1-RSRP measurement period of the neighbor cell refers to TL1-RSRP_Measurement_Period_SSB_inter as defined in clause 9.15, where TReport = 0 and TSSB_NBC is the periodicity of the neighbor cell SSB-Index configured for inter-frequency event-triggered L1-RSRP measurement report.

#### 9.15.3.5	Event Triggered Periodic Reporting

Reported L1-RSRP measurements contained in an event triggered L1-RSRP measurement report shall meet the requirements in clauses 10.1.19/10.1.19E for FR1 and 10.1.20/10.1.20B for FR2, respectively. This requirement applies for event triggered L1-RSRP reports sent on PUSCH using the MAC CE specified in clause 6.1.3.84 of TS 38.321 [7].

The first report in event triggered periodic measurement reporting shall meet the requirements specified in clause 9.15.3.4. For the subsequent reports in the event triggered periodic report, UE follows the reportInterval-r19 and reportAmount-r19 as specified in TS 38.331.

### End of changes