3GPP TSG RAN Meeting #110															RP-253768

Baltimore, USA, December 8-11, 2025 	(was RP-253747, R2-2508816)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 25.304 | CR | 0395 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Restriction on RAT utilization |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Apple, OPPO, InterDigital, Huawei, HiSilicon, Nokia, Samsung, Ericsson |  |  |  |  |  |  |  |  |  |
| Work item code: | ECRATU |  |  |  |  |  | Date: |  |  | 2025-11-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | With the LS "UE usage of the RAT restrictions" from CT1 (C1-247193), R2 is informed that network may indicate the access technology utilization restriction information to UE with NAS EMM and 5GMM procedures. And UE will maintain a list of "PLMNs with associated access technology restrictions" which is applicable for the PLMN and optionally for EPLMN: - not consider any PLMN/access technology combination for which an access technology restriction is configured in the list of "PLMNs with associated access technology restrictions" as a candidate PLMN/access technology for PLMN selection purpose. - If an access technology is restricted in a PLMN, not select any cell of the PLMN deployed in the restricted access technology during cell selection and re-selection for normal service. The restrictions are not applicable if the UE is selecting a cell for limited service. In summary, this feature requires UE to avoid selecting or re-selecting a cell which is deployed in an access technology for which an access technology restriction is applicable for the PLMN or, if applicable EPLMN. In RAN2#130 meeting, RAN2 confirms that AS layer support of ECRATU feature (i.e., cell selection and reselection procedure) by GERAN/UTRAN is optional. And according to CT1 Reply LS (C1-255319), CT1 sees no need to introduce separate NAS capabilities for 2G/3G RAT. Hence, from AS layer perspective, we need considedr the case that the UE may not support RAT restriction related to UTRAN when specifying the corresponding UE behevior when dealing with RAT restriction, |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce a description associated with access technology utilization restriction information including the following aspects: - The usage of the list of "PLMNs with associated access technology restrictions" during cell selection and cell reselection. - The impact on the suitable cell definition. (NOTE: The restrictions are not applicable if the UE is selecting a cell for limited service.) - Fixed a few editorial issues in the change in section 4.3 (inserting “the” between “if” and “UE” & replacing curly quotes) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Cell selection and reselection taking RAT utilization restriction information as received over NAS is not possible. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2, 4.3, 5.2.1, 5.2.6.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Was discussed and endorsed in draft CR R2-2508816 at RAN2 #132. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 1: fixed editorial issue in “cell selection” row of Table 1 in section 4.2 (missing quote mark) and fixed editorial issues in the change in section 4.3 (inserting “the” between “if” and “UE” & replacing curly quotes) |  |  |  |  |  |  |  |  |

First change

## 4.2	Functional division between AS and NAS in Idle mode

Table 1 presents the functional division between UE non-access stratum (NAS) and UE access stratum (AS) in idle mode. The NAS part is specified in [5] and the AS part in the present document. Examples of different idle mode procedures are presented in Clause 10.

Table 1: Functional division between AS and NAS in idle mode

| Idle Mode Process | UE Non-Access Stratum | UE Access Stratum |
| --- | --- | --- |
| PLMN Selection | Maintain the list of allowed PLMN types. It can be GSM-MAP only, ANSI-41 only or both. Maintain a list of PLMNs in priority order according to [5]. Select a PLMN using automatic or manual mode as specified in [5] and request AS to select a cell belonging to this PLMN. For each PLMN, associated RAT(s) may be set. Evaluate reports of available PLMNs from AS for PLMN selection. Maintain a list of equivalent PLMN identities. | Search for available PLMNs. If associated RAT(s) is (are) set for the PLMN, search in this (these) RAT(s) and other RAT(s) for that PLMN as specified in [5]. Perform measurements to support PLMN selection. Synchronise to a broadcast channel to identify found PLMNs. Report available PLMNs with associated PLMN type and RAT to NAS on request from NAS or autonomously. It shall respect allowed PLMN types indications from NAS. |
| Cell Selection | Control cell selection for example by indicating RAT(s) associated with the selected PLMN to be used initially in the search of a cell in the cell selection. NAS is also maintaining lists of forbidden registration areas, a list of "PLMNs with associated access technology restrictions" and a Permitted CSG list. NAS provides these lists to AS. Maintain a list of equivalent PLMN identities and provide the list to AS. Maintain a list of "PLMNs with associated access technology restrictions" and provide the list to AS. | Perform measurements needed to support cell selection. Detect and synchronise to a broadcast channel. Receive and handle broadcast information. Forward NAS system information to NAS. Search for a suitable cell. The cells broadcast their 'PLMN identity' (GSM-MAP) or 'SID' and may broadcast a CSG ID and CSG indicator in the system information. Respond to NAS whether such cell is found or not. If associated RATs is (are) set for the PLMN, perform the search in this (these) RAT(s) and other RATs for that PLMN as specified in [5]. If such a cell is found, the cell is selected to camp on. |
| Cell Reselection | Control cell reselection by for example, maintaining lists of forbidden registration areas. Maintain a list of equivalent PLMN identities and provide the list to AS. Maintain a list of "PLMNs with associated access technology restrictions" and provide the list to AS. Maintain a list of forbidden LAs and provide the list to AS. Maintain a Permitted CSG list and provide the list to AS. | Perform measurements needed to support cell reselection. Detect and synchronise to a broadcast channel. Receive and handle broadcast information. Forward NAS system information to NAS. Change cell if a more suitable cell is found. |
| Location registration | Register the UE as active after power on. Register the UE's presence in a registration area, for instance regularly or when entering a new registration area. Maintain lists of forbidden LAs. Deregister UE when shutting down. | Report registration area information to NAS. |
| MBMS | Maintain priority information for various services and participate in service selection. Indicate to AS if an offered session has previously been received. | Maintain a list of TMGIs of activated services. Notify NAS of service availability and request NAS to prioritise in case of service conflicts. Report to NAS when an activated service requires action. Receive and handle the MCCH and MTCH, and, if applicable, the MSCH and MICH. |
| Support for manual CSG selection | Provide request to search for available CSG(s). Evaluate reports of available CSG(s) from AS for CSG selection. Select a CSG and request AS to select a suitable cell belonging to this CSG. | Search for cells with a CSG ID. Read the HNB name from system information if a cell with a CSG ID is found. Report CSG ID(s) of the found cells broadcasting a CSG ID together with the HNB name and PLMN(s) to NAS. On selection of a CSG by NAS, select any cell belonging to the selected CSG fulfilling the cell selection criteria and not barred or reserved for operator use for UEs not belonging to AC 11 or 15 and give an indication to NAS that access is possible (for the registration procedure). |

Next change

## 4.3	Service type in Idle and Connected Mode

This clause defines the level of service that may be provided by the network to a UE in Idle mode and Connected Mode.

The action of camping on a cell is necessary to get access to some services. Three levels of services are defined for UE:

-	Limited service (emergency calls, ETWS if supported, and CMAS if supported on an acceptable cell)

-	Normal service (for public use on a suitable cell)

-	Operator service (for operators only on a reserved cell)

Furthermore, the cells are categorised according to which services they offer:

acceptable cell:

An "acceptable cell" is a cell on which the UE may camp to obtain limited service (originate emergency calls, receive ETWS if supported, and receive CMAS if supported). Such a cell shall fulfil the following requirements, which is the minimum set of requirements to initiate an emergency call and to receive ETWS primary notification and ETWS secondary notification and to receive CMAS notification in a UTRAN network:

-	The cell is not barred, see subclause 5.3.1.1;

-	The cell selection criteria are fulfilled, see subclause 5.2.3.1.2;

suitable cell:

A "suitable cell" is a cell on which the UE may camp on to obtain normal service. Such a cell shall fulfil all the following requirements.

-	The cell shall be part of either:

-	the selected PLMN, or:

-	the registered PLMN, or:

-	a PLMN of the Equivalent PLMN list

-	For a CSG cell, the cell is a CSG member cell for the UE;

According to the latest information provided by the NAS.

-	The cell is not barred, see subclause 5.3.1.1;

-	The cell is part of at least one LA that is not part of the list of "forbidden LAs for roaming" [9], which belongs to a PLMN that fulfills the first bullet above;

-	If the UE supports "access technology restriction", the access technology of the cell is not marked as restricted for the PLMN that fulfils the first bullet above in the list of "PLMNs with associated access technology restrictions" (TS 23.122 [5]);

-	The cell selection criteria are fulfilled, see subclause 5.2.3.1.2.

If the IE "Multiple PLMN List" [4] is broadcast in the cell, the cell is considered to be part of all LAs with LAIs constructed from the PLMN identities in the "Multiple PLMN List" and the LAC broadcast in the cell.

barred cell:

A cell is barred if it is so indicated in the system information [4].

reserved cell:

A cell is reserved if it is so indicated in system information [4].

Following exceptions to these definitions are applicable for UEs:

-	camped on a cell that belongs to a LA that is forbidden for regional provision of service, and during emergency calls.

-	as an outcome of the manual CSG selection procedure the UE is allowed to access an acceptable cell which fulfils the cell selection criteria and is not barred or reserved for operator use for UEs not belonging to AC 11 or 15 and inform NAS that access is possible (for location registration procedure).

NOTE:	UE is not required to support manual search and selection of CSG(s) while in RRC CONNECTED state.

A cell that belongs to a LA that is forbidden for regional provision service ([5], [16]) is suitable but provides only limited service.

If a UE has an ongoing emergency call, all acceptable cells of that PLMN are treated as suitable for cell reselection for the duration of the emergency call.

suitable MBSFN cluster:

A cell is part of an MBSFN cluster if it is indicated in system information [4]. An MBSFN cluster provides exclusively MBMS services. A suitable MBSFN cluster shall fulfil all the following requirements:

-	For FDD, 3.84 Mcps TDD IMB and 3.84/7.68 Mcps TDD the MBSFN cluster shall be part of either:

-	the registered PLMN, or:

-	a PLMN of the Equivalent PLMN list

The MBSFN cluster shall be indicated as:

-	providing MBSFN only service

For FDD, 3.84 Mcps TDD IMB and 3.84/7.68 Mcps TDD MBSFN cluster selection/reselection process for MBSFN clusters operating on the same frequency shall be based on reception quality. Details of the process are not specified.

Next change

### 5.2.1	Introduction

As stated in clause 1, the present document applies to UEs that support at least UTRA.

Different types of measurements are used in different RATs and modes for the cell selection and reselection. The performance requirements for the measurements are specified in [10] and [11].

The NAS can control the RAT(s) in which the cell selection should be performed, for instance by indicating RAT(s) associated with the selected PLMN, by maintaining a list of forbidden registration area(s), and by maintaining a list of "PLMNs with associated access technology restrictions" and a list of equivalent PLMNs. The UE shall select a suitable cell and the radio access mode based on idle mode measurements and cell selection criteria.

In order to speed up the cell selection process, stored information for several RATs may be available in the UE.

When camped on a cell, the UE shall regularly search for a better cell according to the cell reselection criteria. If a better cell is found, that cell is selected. The change of cell may imply a change of RAT. Details on performance requirements for cell reselection can be found in [10] and [11].

The NAS is informed if the cell selection and reselection results in changes in the received system information.

For normal service, the UE has to camp on a suitable cell, tune to that cell's control channel(s) so that the UE can:

-	Receive system information from the PLMN;

-	Receive registration area information from the PLMN, e.g., location area and routing area; and

-	Receive other AS and NAS Information;

-	If registered:

-	receive paging and notification messages from the PLMN; and

-	initiate call setup for outgoing calls or other actions from the UE.

Next change

##### 5.2.6.1.3	Highest ranked cells with cell reservations, access restrictions or unsuitable for normal camping

For the highest ranked cell (including serving cell) according to cell reselection criteria specified in subclause 5.2.6.1.4, or for the best cell according to absolute priority reselection criteria specified in subclause 5.2.6.1.4a, the UE shall check if the access is restricted according to the rules in subclause 5.3.1.1.

If that cell and other cells have to be excluded from the candidate list, as stated in subclause 5.3.1.1, the UE shall not consider these as candidates for cell reselection. This limitation is removed when the highest ranked cell changes.

If the highest ranked cell or best cell according to absolute priority reselection rules is an intra-frequency or inter-frequency cell which is not suitable due to being part of the "list of forbidden LAs for roaming" or belonging to a PLMN which is not indicated as being equivalent to the registered PLMN, the UE shall not consider this cell and other cells on the same frequency, as candidates for reselection for a maximum of 300s. If the highest ranked cell or best cell according to absolute priority reselection rules is an intra-frequency or inter-frequency cell which is not suitable due to its access technology being restricted based on the list of "PLMNs with associated access technology restrictions" (TS 23.122 [5]), the UE should not consider this cell and other cells of the restricted access technology on the same frequency, as candidates for reselection for a maximum of 300s. If the UE has to perform an any cell selection procedure any limitation shall be removed. If the UE is redirected under UTRAN control to a frequency for which the timer is running, any limitation on that frequency shall be removed.

If the highest ranked cell or best cell according to absolute priority reselection rules is an inter-RAT cell which is not suitable due to being part of the "list of forbidden LAs for roaming" or belonging to a PLMN which is not indicated as being equivalent to the registered PLMN, the UE shall not consider this cell and other cells on the same frequency as candidates for reselection for a maximum of 300s. If the highest ranked cell or best cell according to absolute priority reselection rules is an inter-RAT cell which is not suitable due to its access technology being restricted based on the list of "PLMNs with associated access technology restrictions" (TS 23.122 [5]), the UE should not consider this cell and other cells of the restricted access technology on the same frequency as candidates for reselection for a maximum of 300s. If the UE has to perform an any cell selection procedure any limitation shall be removed. If the UE is redirected under UTRAN control to a frequency for which the timer is running, any limitation on that frequency shall be removed.

End of change