3GPP TSG RAN WG1 #123			R1-2509451

Dallas, USA, Nov 17th – 21st, 2025

Agenda Item:	10.4

Source:	Ad-Hoc Chair (NTT DOCOMO, INC.)

Title:	Session Notes of AI 10.4

Document for:	Endorsement

  - Coverage Enhancement Phase 3

Please refer to RP-252824 for detailed scope of the WI.

[123-R20-CE] Email discussion on Rel-20 CE– Hang (China Telecom)

- To be used for sharing updates on online/offline schedule, details on what is to be discussed in online/offline sessions, tdoc number of the moderator summary for online session, etc

R1-2509451	Session Notes of AI 10.4	Ad-Hoc Chair (NTT DOCOMO, INC.)

    - Coverage enhancement

Including PRACH coverage enhancements, PUSCH repetition scheduled by DCI 0_0 with C-RNTI and Extending pi/2-BPSK to more MCS entries.

R1-2508608	FL's summary #1 on NR coverage enhancements Phase 3	China Telecom

Possible Agreement:

Support indicating the UL beam information via one or a combination of multiple options from following options, for down-selection:

- Option 1: implicitly indicated by RA-RNTI

- Option 3: explicitly indicated by repurposing field(s) in UL Grant in RAR

- Option 4: explicitly indicated by introducing new field in MAC RAR

  - gNB complexity due to different payload size for different UEs

  - whether RAN2 can accept the introduction of new field

Possible agreement

For the determination of total number of PRACH transmissions per RACH attempt, RSRP-based criteria is supported.

- FFS: details

Agreement:

Support [request or capability report] of PUSCH repetition scheduled by DCI 0_0 with C-RNTI before receiving RRCReconfiguration in Msg3.

Possible agreement:

Pi/2-BPSK is extended to all MCS entries in MCS tables with spectrum efficiency smaller than 0.8770.

- FFS: when spectrum efficiency equals to 0.8770

R1-2508609	FL's summary #2 on NR coverage enhancements Phase 3	China Telecom

R1-2508610	FL's summary #3 on NR coverage enhancements Phase 3	China Telecom

R1-2508611	FL's summary #4 on NR coverage enhancements Phase 3	China Telecom

R1-2508612	Final FL's summary on NR coverage enhancements Phase 3	China Telecom

R1-2508383	Discussion on NR Coverage enhancement	Spreadtrum, UNISOC

R1-2508393	Discussion on coverage enhancement	LG Electronics

R1-2508427	Further discussions on NR phase 3 coverage enhancements	vivo

R1-2508498	Coverage enhancements for NR Phase 3	Huawei, HiSilicon

R1-2508592	Discussion on coverage enhancement	CATT

R1-2508607	Discussion on NR coverage enhancements Phase 3	China Telecom

R1-2508679	Discussion on coverage enhancement	Xiaomi

R1-2508722	Discussion on further enhancement for coverage enhancement	OPPO

R1-2508755	NR coverage enhancement Phase 3	Tejas Network Limited

R1-2508797	Discussion on Coverage Enhancement	Samsung

R1-2508837	Discussion on coverage enhancement	Transsion Holdings

R1-2508853	Discussion on coverage enhancement	ZTE Corporation, Sanechips

R1-2508913	MCS entries for pi/2-BPSK	Wisig, IITH

R1-2508968	Discussion on coverage enhancements	ETRI

R1-2508998	Discussion on 5G NR coverage enhancement phase 3	Panasonic

R1-2509005	Coverage enhancements for NR Phase 3	Nokia

R1-2509015	Coverage enhancement for NR Phase 3	InterDigital, Inc.

R1-2509038	Views on Coverage Enhancement Phase 3	Ofinno

R1-2509105	Discussion on coverage enhancements for NR Phase 3	Apple

R1-2509138	Discussions on NR coverage enhancements Phase 3	KT Corp.

R1-2509179	Coverage enhancements	Lenovo

R1-2509181	Discussions on further coverage enhancement for NR	Sharp

R1-2509226	Coverage enhancement Phase 3	Qualcomm Incorporated

R1-2509275	Discussions on coverage enhancement	NTT DOCOMO, INC.

R1-2509308	Discussion on NR Coverage enhancement Phase 3	ITRI, Acer Incorporated

R1-2509334	Discussion on Rel-20 Coverage Enhancement	Ericsson (China)

(Withdrawn)

R1-2509346	Discussion on Coverage Enhancements Phase 3	CEWiT

R1-2509365	Discussion on coverage enhancement	ASUSTeK

R1-2509379	Discussion on coverage enhancement	DENSO CORPORATION

R1-2509420	Discussion on Rel-20 Coverage Enhancement	Ericsson