3GPP TSG RAN WG1 #123			R1-2509451

Dallas, USA, Nov 17th – 21st, 2025

Agenda Item:	10.4

Source:	Ad-Hoc Chair (NTT DOCOMO, INC.)

Title:	Session Notes of AI 10.4

Document for:	Endorsement

  - Coverage Enhancement Phase 3

Please refer to RP-252824 for detailed scope of the WI.

[123-R20-CE] Email discussion on Rel-20 CE– Hang (China Telecom)

- To be used for sharing updates on online/offline schedule, details on what is to be discussed in online/offline sessions, tdoc number of the moderator summary for online session, etc

R1-2509451	Session Notes of AI 10.4	Ad-Hoc Chair (NTT DOCOMO, INC.)

    - Coverage enhancement

Including PRACH coverage enhancements, PUSCH repetition scheduled by DCI 0_0 with C-RNTI and Extending pi/2-BPSK to more MCS entries.

R1-2508608	FL's summary #1 on NR coverage enhancements Phase 3	China Telecom

Agreement:

Support [request or capability report] of PUSCH repetition scheduled by DCI 0_0 with C-RNTI before receiving RRCReconfiguration in Msg3.

R1-2508609	FL's summary #2 on NR coverage enhancements Phase 3	China Telecom

Agreement

Confirm the following working assumption in RAN1 #122bis,

Working assumption The multiple PRACH transmission with different Tx beams is supported for CBRA, ReconfigurationWithSync case in CFRA and SI request. - RAN1 assumes that all the cases are supported with a common solution

Agreement

At least for initial Msg3 transmission, support indicating the UL beam information via one of multiple options from following options, for down-selection:

- Option 1: based on implicit indication by RA-RNTI

- Option 3: based on explicit indication by repurposing field(s) in UL Grant in RAR

- Option 4: based on explicit indication by introducing new field in MAC RAR

Agreement

For multiple PRACH transmissions with different Tx beams, candidate values of the total number of PRACH transmissions are {2,4,8} in one RACH attempt.

Agreement

For the power ramping between different RACH attempt for multiple PRACH transmission with different Tx beams, down-select from the following options:

- Option 1: when any of the Tx beams used in the next attempt is changed, Layer 1 (may) notifies higher layer to suspend the power ramping counter.

- Option 2: when all the Tx beams used in the next attempt are changed, Layer 1 notifies higher layer to suspend the power ramping counter

- Option 3: when M>1 UL Tx beams used in the next attempt are changed, Layer 1 (may) notifies higher layer to suspend the power ramping counter, FFS the value of M.

- Option 4: multiple power ramping counters are used for each PRACH transmission in the RO group.

R1-2508610	FL's summary #3 on NR coverage enhancements Phase 3	China Telecom

Agreement

Support to configure separate ROs or separate preambles on shared RO to differentiate different number of multiple PRACH transmissions with different Tx beams via RRC parameters.

- FFS: Other details

Agreement

At least for initial RACH attempt, the determination of total number of PRACH transmissions per RACH attempt is down-selected based on the following options

- Option 1: A single RSRP threshold is introduced to determine whether multiple PRACH transmissions with different Tx beams can be applied

- Option 2: each of the values of multiple RPSP thresholds corresponding to each of the values of the total number of multiple PRACH transmissions

- Option 3: each of the values of multiple RSRP thresholds corresponding to each of the values of the number of repetitions of a RACH transmission per Tx beam.

- Option 4: UE determines total number of RACH transmission based on the number of beams it supports.

- Other options are not precluded.

- Combination of options not precluded.

R1-2508611	FL's summary #4 on NR coverage enhancements Phase 3	China Telecom

Agreement

Support PUSCH repetitions scheduled by DCI format 0_0 with C-RNTI both before and after receiving RRCReconfiguration.

Note: Strive for a single mechanism for PUSCH repetition scheduled by DCI 0_0 with C-RNTI

Agreement

The first RV id of PUSCH repetition scheduled by DCI format 0_0 with C-RNTI should follows the indication in DCI format 0_0.

Note: this agreement intends following update for the agreement made at RAN1#122bis

Reuse the following rules of Msg3 repetition for PUSCH repetition scheduled by DCI 0_0 with C-RNTI.

- RV determination rule, including first RV id determination, RV sequence selection, RV cycling, etc.

- Available slot determination rule

- Only applicable to PUSCH Repetition Type A

Agreement

For PUSCH repetition scheduled by DCI 0_0 with C-RNTI, inter-slot frequency hopping is supported

- Note: intra-slot frequency hopping is not supported unless the repetition time is 1

Agreement

Down-select one of the following options to indicate the number of repetitions of PUSCH scheduled by DCI 0_0 with C-RNTI both before and after receiving RRCReconfiguration

- Option 1: Using at most 2 MSB of MCS field in DCI format 0_0 with CRC scrambled by C-RNTI

  - FFS: details

- Option 2: Based on TDRA

  - FFS: details

Agreement

For extension of pi/2-BPSK to more MCS entries, down-select one of the following Options:

- Option 1：Pi/2-BPSK is extended to all MCS entries in MCS table(s) with spectrum efficiency equals to or smaller than N = 0.8770.

  - gNB can configure the maximum MCS up to which pi/2 BPSK is applicable.

    - Details on configuration to be discussed further

  - FFS: UE signaling to the gNB for this configuration

    - FFS: whether/how to provide assistance information and what is the assistance information

- Option 2：Support extending Pi/2-BPSK to all MCS entries in MCS table(s) with spectrum efficiency equals to or smaller than X.

  - Where X the fixed value select from:

a)	X = 0.8770

b)	X = 0.6016

  - Extension is captured in the modified MCS table.

  - gNB enables/disables this feature via RRC signalling.

R1-2508612	Final FL's summary on NR coverage enhancements Phase 3	China Telecom

R1-2508383	Discussion on NR Coverage enhancement	Spreadtrum, UNISOC

R1-2508393	Discussion on coverage enhancement	LG Electronics

R1-2508427	Further discussions on NR phase 3 coverage enhancements	vivo

R1-2508498	Coverage enhancements for NR Phase 3	Huawei, HiSilicon

R1-2508592	Discussion on coverage enhancement	CATT

R1-2508607	Discussion on NR coverage enhancements Phase 3	China Telecom

R1-2508679	Discussion on coverage enhancement	Xiaomi

R1-2508722	Discussion on further enhancement for coverage enhancement	OPPO

R1-2508755	NR coverage enhancement Phase 3	Tejas Network Limited

R1-2508797	Discussion on Coverage Enhancement	Samsung

R1-2508837	Discussion on coverage enhancement	Transsion Holdings

R1-2508853	Discussion on coverage enhancement	ZTE Corporation, Sanechips

R1-2508913	MCS entries for pi/2-BPSK	Wisig, IITH

R1-2508968	Discussion on coverage enhancements	ETRI

R1-2508998	Discussion on 5G NR coverage enhancement phase 3	Panasonic

R1-2509005	Coverage enhancements for NR Phase 3	Nokia

R1-2509015	Coverage enhancement for NR Phase 3	InterDigital, Inc.

R1-2509038	Views on Coverage Enhancement Phase 3	Ofinno

R1-2509105	Discussion on coverage enhancements for NR Phase 3	Apple

R1-2509138	Discussions on NR coverage enhancements Phase 3	KT Corp.

R1-2509179	Coverage enhancements	Lenovo

R1-2509181	Discussions on further coverage enhancement for NR	Sharp

R1-2509226	Coverage enhancement Phase 3	Qualcomm Incorporated

R1-2509275	Discussions on coverage enhancement	NTT DOCOMO, INC.

R1-2509308	Discussion on NR Coverage enhancement Phase 3	ITRI, Acer Incorporated

R1-2509334	Discussion on Rel-20 Coverage Enhancement	Ericsson (China)

(Withdrawn)

R1-2509346	Discussion on Coverage Enhancements Phase 3	CEWiT

R1-2509365	Discussion on coverage enhancement	ASUSTeK

R1-2509379	Discussion on coverage enhancement	DENSO CORPORATION

R1-2509420	Discussion on Rel-20 Coverage Enhancement	Ericsson