3GPP TSG RAN WG1 #123			R1-2509452

Dallas, USA, Nov 17th – 21st, 2025

Agenda Item:	10.5

Source:	Ad-Hoc Chair (NTT DOCOMO, INC.)

Title:	Session Notes of AI 10.5

Document for:	Endorsement

  - Study on Integrated Sensing And Communication (ISAC) for NR

Please refer to RP-252819 for detailed scope of the SI.

[123-R20-ISAC] Email discussion on Rel-20 ISAC – Yingyang (Xiaomi)

- To be used for sharing updates on online/offline schedule, details on what is to be discussed in online/offline sessions, tdoc number of the moderator summary for online session, etc

R1-2509452	Session Notes of AI 10.5	Ad-Hoc Chair (NTT DOCOMO, INC.)

R1-2509237	Draft TR 38.765 v010: Study on Integrated Sensing And Communication (ISAC) for NR	Xiaomi, China Telecom

To be updated in R1-250xxxx

    - Evaluation assumptions and performance evaluation

Including study of metrics, measurements, and relevant measurement quantization for UAV use case.

R1-2509238	Summary #1 on evaluations for NR ISAC	Moderator (Xiaomi)

Agreement:

- In section 6.2 Performance evaluation results in TR 38.765,

- Multiple configurations of the following parameters are defined to respectively analyze the evaluation results

  - At least Two baseline configurations 1 & 2 are defined for evaluation purpose

|  | Parameters | Configuration 1 Baseline | Configuration 2 Baseline |
| --- | --- | --- | --- |
| 1 | Scenario | (UMa-AV, 500m) | (UMa-AV, 500m) |
| 2 | Sensing Tx/Rx operating simultaneously or not | Sensing Tx/Rx operating simultaneously | Sensing Tx/Rx operating simultaneously |
| 3 | Carrier frequency | 4 or 4.9GHz | 4 or 4.9GHz |
| 4 | Max BS Tx power | 52 dBm | 37 dBm |
| 5 | BS antenna configuration | - (M, N, P, Mg, Ng, Mp, Np) - Tx: (8,8,2,1,1;4,8) - Rx: (8,8,2,1,1;4,8) dH,dV = (0.5, 0.8)λ, +45°/-45° polarization | - (M, N, P, Mg, Ng, Mp, Np) - Tx: (8,8,2,1,1;4,8) - Rx: (8,8,2,1,1;4,8) dH,dV = (0.5, 0.8)λ, +45°/-45° polarization |
| 6 | Number of targets per sector in center site | N=5 | N=5 |
| 7 | Target vertical distribution | 25-300m | 25-300m |

  - Additional configurations can be defined for other assumptions of the parameters, based on reported evaluation results in RAN1 #124.

Agreement:

- Companies should report whether a same target is modeled in the ISAC channel of single, multiple or all STXs/SRXs

  - Company should report how to determine the single or multiple STXs/SRXs for a target

  - The details of STX/SRX determination will be captured along with the corresponding evaluation results in the TR for NR ISAC

  - If the evaluation results are derived by measurement reports from multiple/all STXs/SRXs, companies should report how measurement reports from multiple/all STXs/SRXs are used

R1-2509239	Summary #2 on evaluations for NR ISAC	Moderator (Xiaomi)

Proposed agreement

From RAN1 perspective, the following measurements that may be reported from RAN are identified in the study of NR ISAC. Note: Down selection of Level(s)/Option(s) for recommendation can be discussed separately.

- Level A: Raw data per Tx antenna port per OFDM symbol per RX antenna port per TRP for a given time stamp

  - Option A1: Amplitude and phase samples in time/delay domain of the estimated channel, i.e., Amplitude and phase values of channel impulse response

  - Option A2: Amplitude and phase per subcarrier in frequency domain of the estimated channel

- Level B: Amplitude and phase profile of delay, and/or Doppler, and/or angle per TRP for a given time stamp by using window(s) of the [consecutive] samples in delay, Doppler and/or angle domain

  - Option B1: Delay-Doppler profile per Tx antenna port per Rx antenna port, which includes the amplitude and phase samples distributed across different delays and Doppler shifts.

  - Option B2: Delay-Angle profile per Tx antenna port per OFDM symbol, which includes the amplitude and phase samples distributed across different delays and spatial angles (e.g., Angle of Arrival).

  - Option B3: Delay-Doppler-Angle profile per Tx antenna port, which includes the amplitude and phase samples distributed across delay, Doppler, and angle domains.

  - Option B4: Delay profile per Tx antenna port per OFDM symbol per Rx antenna port, which includes the amplitude and phase samples distributed across different delays.

  - Note: Level B is applicable for either LCS or GCS

- Level C: per detected path/point measurements per Tx antenna port [per TRP or per gNB] which may be reflected/scattered from scattering point(s).

  - Option C1: Delay/range, Doppler/velocity, 3D angles, and [power/confidence metric] per detected path for a given time stamp.

    - A path is associated with one couple {delay/range, Doppler/velocity, power}, and one or multiple 3D angles.

  - Option C2: Doppler/velocity, position, and [power/confidence metric] per detected path for a given time stamp.

    - A path is associated with one velocity (radial velocity), power}, one or multiple positions

  - Option C3: Delay/range, Doppler/velocity, 3D angles, and [power/confidence metric] per detected path for a given time stamp.

    - A path is associated with one delay, and one or multiple triple {Doppler, 3D angles, power}

  - Option C4: Delay/range, Doppler/velocity, 3D angles, and [power/confidence metric] per detected point for a given time stamp

    - A point is associated with one range/delay, one Doppler, one pair of 3D angles [and one power]

    - FFS: Velocity can be

      - Opt 1: radial velocity

      - Opt 2: true velocity

  - Option C5: Position, velocity, and [power/confidence metric] per detected point for a given time stamp

    - A point is associated with one location, one velocity [and one power]

    - FFS: Velocity can be

      - Opt 1: radial velocity

      - Opt 2: true velocity

  - Note:

    - Position can be defined in either LCS or GCS

    - Angle can be defined in either LCS or GCS

    - Ambiguity issue will be further discussed

    - Up to RAN3 to discuss on proper confidence metric of a path/point

    - Up to RAN3 to discuss each parameter reported per detected path/point in the measurement report is mandatory or optional.

- Level D: Object/target level measurement [per xxx]

  - One or more value pair(s) {position, velocity} in GCS for a given time stamp is reported for a detected object/target,

    - Option D1: Only one value pair {position, velocity} in GCS for a given time stamp is reported for a detected object/target. The association of multiple measurements across different time stamps for the same detected object/target is not reported.

    - Option D2: Only one value pair {position, velocity} in GCS for a given time stamp is reported for a detected object/target. The association of multiple measurements across different time stamps for the same detected object/target is reported.

    - Option D3: One or more value pairs {position, velocity} in GCS for a given time stamp are reported for a detected object/target. The association of multiple measurements across different time stamps for the same detected object/target is reported.

    - Option D4: One or more value pairs {position, velocity} in GCS for a given time stamp are reported for a detected object/target. The association of multiple measurements across different time stamps for the same detected object/target is not reported.

  - Note: confidence metric will be further discussed for options of Level D as well

  - Note: Ambiguity issue can be further discussed if any

  - Note: velocity is [radial or true]

R1-2509240	Summary #3 on evaluations for NR ISAC	Moderator (Xiaomi)

R1-2509241	Summary #4 on evaluations for NR ISAC	Moderator (Xiaomi)

R1-2509242	Summary #5 on evaluations for NR ISAC	Moderator (Xiaomi)

R1-2509243	Summary #6 on evaluations for NR ISAC	Moderator (Xiaomi)

R1-2508372	Views on evaluation assumptions and performance evaluation for ISAC	Sharp

R1-2508384	Discussion on evaluation assumption and performance evaluation for ISAC	Spreadtrum, UNISOC

R1-2508428	Evaluation methodology and performance evaluation  for 5G-A ISAC	vivo

R1-2508451	Discussion on evaluations and measurements for NR ISAC	CMCC

R1-2508464	Evaluation assumptions and performance evaluation of ISAC for NR	InterDigital, Inc.

R1-2508473	Discussion on Evaluation Assumptions and Performance Evaluation for ISAC	Tiami Networks

R1-2508485	Performance metric, methodologies, and initial evaluation results for ISAC	Huawei, HiSilicon

R1-2508593	Discussion on evaluation assumptions and performance evaluation for R20 ISAC for NR	CATT, CICTCI

R1-2508613	Discussion on ISAC evaluation assumptions and performance evaluation	China Telecom

R1-2508618	Evaluation assumptions and performance evaluation for ISAC	EURECOM

R1-2509458	On ISAC performance evaluations and assumptions	Lenovo	(Revision of R1-2508634)

R1-2508680	Discussion on performance evaluation for ISAC	Xiaomi

R1-2508723	Discussion of ISAC evaluation in 5GA	OPPO

R1-2508798	Discussion on ISAC evaluation assumptions and performance evaluation	Samsung

R1-2508859	Evaluation assumptions and performance evaluations of ISAC for NR	Nokia, Nokia Shanghai Bell

R1-2508877	Considerations on Evaluations for NR ISAC	CAICT

R1-2508883	Discussion on ISAC Performance Evaluation for 5G-A   	NIST

R1-2508938	Discussion on evaluation assumptions and performance evaluation for NR ISAC	Panasonic

R1-2508969	Discussion on evaluation assumptions and performance evaluation for NR ISAC	ETRI

R1-2509002	Discussion on ISAC for NR	Ericsson

R1-2509040	Evaluation assumptions and performance evaluation for NR ISAC	Hanbat National University

R1-2509054	Discussion on Performance Evaluations for Rel-20 ISAC	Google

R1-2509070	Discussion on Evaluation Assumptions and Performance Evaluation of NR ISAC	Sony

R1-2509106	Rel-20 Evaluation assumptions and performance evaluation for 5G-A ISAC	Apple

R1-2509124	ISAC Performance metrics and overhead	T-Mobile USA

R1-2509157	Discussion on ISAC evaluation assumptions and performance evaluation	MediaTek Inc.

R1-2509227	Evaluation Assumptions and performance evaluation for UAV gNB-monostatic sensing	Qualcomm Incorporated

R1-2509503	Discussion on 5G-A ISAC evaluation	ZTE Corporation, Sanechips,VIAVI				(Revision of R1-2509246)

R1-2509276	Evaluation assumptions and performance evaluation for ISAC	NTT DOCOMO, INC.

R1-2509293	Discussion on Evaluation assumptions and of ISAC  for NR	SK Telecom

R1-2509457	Discussion on evaluation assumptions and performance evaluation for NR ISAC	LG Electronics

(Revision of R1-2509305)

R1-2509497	Evaluation assumptions and Performance Evaluation for Self-Interference Modeling for ISAC			Tejas Network Limited	(Revision of R1-2509326)