3GPP TSG RAN WG1 #124 			R1-2600550

Gothenburg, SE, Feb. 9th ~ 13th, 2026

Source: 	Moderator (LG Electronics)

Title:	FL summary #3 for 9.3.2.3 “Other procedures”

Document for:	Discussion and decision

Introduction

This feature lead summary (FLS) concerns the sub-agenda 9.3.2.3 of Rel-20 Ambient IoT SI [1].

Based on the contributions submitted to the sub-agenda 9.3.2.3 ([3] – [33]), draft proposals/questions are prepared for offline and online discussions, and the proposals/questions will be updated during the meeting based on follow-on discussions.

In this document, the following tags will be used:

- [H]/[L] to mark high/low-priority issues. The issue tagged [H] will be interpreted as an essential issue that needs to be resolved to finish this SI, and the issue tagged [L] or without any tag may be a non-essential issue but can also be handled offline and online after all or most of the high priority issues are resolved or if they are considered very stable.

- [Open]/[Closed] to mark open/closed issues

With that understanding, please provide your views on the proposals tagged [H] first as they can be discussed first.

Please add (if absent) your e-mail address below for the points of contact for this sub-agenda.

| Company | Point(s) of contact | Email address(es) |
| --- | --- | --- |
| LGE | Jay KIM (FL) | jaehyung.kim@lge.com |
| LGE | Seokmin SHIN | seokmin.shin@lge.com |
| Panasonic | Hamidreza SHARIATMADARI | hamidreza.shariatmadari@eu.panasonic.com |
| Panasonic | HORIIKE Naoto | naoto.horiike@eu.panasonic.com |
| Panasonic | KANG Yang | yang.kang@sg.panasonic.com |
| Panasonic | SUZUKI Hidetoshi | suzuki.hidetoshi@jp.panasonic.com |
| TCL | Wenwen Huang | wenwen5.huang@tcl.com |
| TCL | Rongling Jian | rongling.jian@tcl.com |
| TCL | Yuanqing Yang | yuanqing4.yang@tcl.com |
| Samsung | Hyemin choe | hams.choe@samsung.com |
| Samsung | Miao Zhou | miao.zhou@samsung.com |
| Samsung | JJ | jeongho.j@samsung.com |
| Nokia | Torsten Wildschek | torsten.wildschek@nokia.com |
| Nokia | Hyun-Su Cha | Hyun-su.cha@nokia.com |
| Xiaomi | Teng Ma | mateng5@xiaomi.com |
| Xiaomi | Huan Sun | sunhuan7@xiaomi.com |
| Huawei, HiSilicon | Sarun Selvanesan | sarun.selvanesan@huawei.com |
| Huawei, HiSilicon | Mixiang | shawn.mixiang@huawei.com |
| Huawei, HiSilicon | Jin Zhe | jinzhe.jinzhe@huawei.com |
| Apple | Ankit Bhamri | a.bhamri@apple.com |
| OPPO | Kevin Lin | kevin.lin@oppo.com |
| OPPO | Shichang Zhang | shichangzhang@oppo.com |
| FUTUREWEI | Vip Desai | vdesai@futurewei.com |
| FUTUREWEI | Ruikang Yang | ryang@futurewei.com |
| NTT DOCOMO | Mayuko Okano | mayuko.okano.ca@nttdocomo.com |
| NTT DOCOMO | Sun Weiqi | sunwq@docomolabs-beijing.com.cn |
| NTT DOCOMO | Hiroki Harada | hiroki.harada.sv@nttdocomo.com |
| CMCC | Jingwen Zhang | zhangjingwen@chinamobile.com |
| CMCC | Xiaodong Shen | shenxiaodong@chinamobile.com |
| NEC | Pengyu Ji | ji_pengyu@nec.cn |
| NEC | Tingting Fan | Tingting.Fan@EMEA.NEC.COM |
| NEC | Yun Liu | liu_yun@nec.cn |
| Lenovo | Xin GUO | guoxin9@lenovo.com |
| HONOR | JIAN ZHANG | zhangjian23@honor.com |
| Qualcomm | Yuchul Kim | yuchulk@qti.qualcomm.com |
| Qualcomm | Le Liu | leliu@qti.qualcomm.com |
| IDCC | Erdem Bala | Erdem.bala@interdigital.com |
| IDCC | Kevin Wanuga | Kevin.wanuga@interdigital.com |
| Spreadtrum | Yu Ding | Yu.Ding@unisoc.com |
| China Telecom | Yi Gu | guy6@chinatelecom.cn |
| China Telecom | Jing Guo | guojing6@chinatelecom.cn |

Online/offline proposals

This section intends to provide online/offline proposals for each specific online session (e.g. proposals for Tuesday morning online) with a list of ordered proposals, which saves time for everyone especially online where the Chairman just needs to look at this section to find all the proposals for the session.

Proposals for Tuesday morning online session

[D2R power control]

Feasibility study of the two open-loop Tx power control methods for D2R

Previous agreements

Agreement (RAN1#122bis)

Study the necessity of D2R Tx power control for Device 2b and for Device C for outdoor scenarios

- Target to agree on observations on pros and cons of D2R Tx power control vs. no Tx power control at RAN1#123

Continue studying feasibility of the following open loop power control methods considering device complexity:

- Methods of open loop power control

  - Option 1: Device calculates Tx power based on measured pathloss, a predefined formula, and some configured parameters from reader

    - Required information at the device side at least includes reader Tx power, received target power at the reader, etc.

  - Option 2: Device measures RSRP, and uses a corresponding Tx power when the RSRP falls in an RSRP range.

    - The RSRP range is based on RSRP threshold(s) configured by reader or predefined

    - Required information at the device side at least includes RSRP threshold(s), corresponding D2R Tx power related information per each RSRP range defined by the RSRP threshold(s)

  - FFS details (e.g., required measurements, required information)

Continue studying the necessity and feasibility of closed loop power control for D2R.

Work plan for RAN1#124 meeting (R1-2509614)

- D2R Tx power control - Necessity of D2R Tx power control (Pros and cons of Tx power control vs. no Tx power control for D2R) - Feasibility study of the two open-loop Tx power control methods for D2R - Necessity and feasibility of closed-loop Tx power control for D2R - Necessary information for D2R Tx power control and its provisioning

Summary

Views on feasibility of each option

- Option 1 is

  - feasible: FUTUREWEI, Xiaomi, LGE, DOCOMO, TCL, vivo

  - not feasible:

- Option 2 is

  - feasible: FUTUREWEI, Xiaomi, LGE, DOCOMO, TCL, vivo

  - not feasible: Samsung

Preference for Option 1 and Option 2 for methods of open loop power control

- Prefer Option 1: Xiaomi, ZTE, LGE, ETRI, ASUSTeK, Quectel, TCL, OPPO,

  - Not prefer Option 2: Samsung, vivo, OPPO, NEC

- Prefer Option 2: Panasonic, Apple, Qualcomm, KT, Fraunhofer

- [H] Proposal 3.3.2a-1

On the two open‑loop Tx power control methods for D2R, capture the following in TR 38.769.

For Option 1, sources (FUTUREWEI, Xiaomi, LGE, DOCOMO, TCL, vivo) report that Option 1 is feasible.

- [FUTUREWEI] mentions that Option 1 is feasible, indicating that a device can measure the received R2D power at RF and derive its D2R transmit power based on a pathloss estimate obtained from reader‑provided transmit power and target power level.

- [Xiaomi] considers Option 1 feasible and prefers to support it due to its higher adjustment accuracy based on actual pathloss.

- [LGE] notes that Option 1 provides higher‑accuracy and more adaptive open‑loop power control based on pathloss, and is therefore the preferred approach.

- [DOCOMO] states that Option 1 offers finer‑granularity and more accurate D2R transmit‑power control, giving it an advantage for open‑loop operation.

- [TCL] states that Option 1 provides precise pathloss‑based D2R transmit‑power control that improves reliability, reduces interference, and enables energy‑efficient operation.

For Option 2, sources (FUTUREWEI, Xiaomi, LGE, DOCOMO, TCL, vivo) report that Option 2 is feasible, while source (Samsung) reports that Option 2 is not feasible.

- [FUTUREWEI] mentions that Option 2 is feasible, indicating that a device can perform an RSRP‑like baseband measurement that includes AGC effects and determine its D2R transmit power using a lookup table provided by the reader.

- [Xiaomi] notes that Option 2 is feasible due to its low complexity.

- [LGE] notes that Option 2 offers lower complexity through RSRP‑based power selection but provides coarser and less adaptive power control.

- [DOCOMO] states that Option 2 avoids device‑side formula computation, though the impact of such complexity remains unclear.

- [TCL] states that Option 2 offers simple RSRP‑range‑based selection but may cause under‑ or over‑powering due to coarse power levels.

Between Option 1 and Option 2, sources (Xiaomi, ZTE, LGE, ETRI, ASUSTeK, Quectel, TCL, vivo, [OPPO]) report that they prefer Option 1 over Option 2, while sources (Panasonic, Apple, Qualcomm, KT, Fraunhofer) report that they prefer Option 2 over Option 1. For Option 1, following observations were also provided during the study.

- [OPPO] mentions that in Option 1 the device needs to estimate the pathloss using the reader‑indicated Tx power and an R2D measurement, and that limited measurement accuracy and power‑adjustment capability may cause the received power at the reader to deviate from the configured P0, which can affect CDM‑based Msg1 transmission.

- [Ericsson] mentions that under an open‑loop power‑control mechanism, a simplified NB‑IoT–like approach can be used as a baseline for Rel‑20 A‑IoT.

- [Nokia] notes that Option 1 should cover cases where the device cannot use the required or indicated transmit power.

- [ZTE] mentions that Option 1 provides smooth and adaptive power control based on real‑time pathloss and a predefined formula, is therefore the preferred approach.

- [vivo] notes that Option 1 achieves better power‑control accuracy because each device derives its own transmit power using pathloss calculated from RSRP.

- [NEC] notes that Option 1 for open‑loop power control follows a traditional pathloss‑based approach and can be studied for Rel‑20.

- [Panasonic] notes that Option 1 can provide more precise transmit‑power control but increases the computational complexity of Device 2b/C.

- [Samsung] notes that Option 1 enables open‑loop D2R power control using pathloss estimation based on comparing the average ON‑state power of known R2D signals with the configured ON‑state power.

- [Apple] notes that Option 1 enables continuous Tx power adjustment and can be applied for device C.

- [ETRI] states that Option 1 provides higher‑accuracy D2R transmit power adjustment based on pathloss and is therefore more suitable for A‑IoT operation.

- [Qualcomm] states that Option 1 provides more precise pathloss‑based power control but may not be essential for ultra‑low‑power devices given their large transmit‑power tolerance and higher complexity.

- [Quectel] states that Option 1 provides higher‑precision D2R transmit‑power control through pathloss‑based calculation, and the additional complexity is acceptable for device 2b/C.

- [ASUSTeK] states that Option 1 allows pathloss‑based D2R transmit‑power calculation using reader‑configured parameters and is slightly preferred due to lower specification complexity.

- [KT] states that Option 1 provides finer‑grained D2R transmit‑power control through pathloss‑based calculation.

- [Fraunhofer] states that Option 1 performs pathloss‑based open‑loop power control but requires explicit pathloss calculation and higher device complexity.

For Option 2, following observations were also provided during the study.

- [OPPO] mentions that in Option 2 the device maps a measured RSRP to a D2R transmit power, but frequent updates of the mapping between RSRP ranges and D2R transmit powers are required when the reader’s Tx power changes, resulting in increased R2D signaling overhead.

- [Ericsson] mentions that within the open‑loop power‑control framework, Option 2 can be considered alongside Option 1.

- [Nokia] notes that Option 2 assumes that the corresponding D2R transmit power includes zero power.

- [ZTE] notes that Option 2 uses an RSRP‑based mapping with complexity comparable to Option 1.

- [vivo] notes that Option 2 provides worse power‑control accuracy, as devices rely only on RSRP ranges and may select identical transmit power within the same range, by which the power control is unachievable within each RSRP range. And suitable RSRP thresholds are difficult for the reader to configure. [For example, the reader cannot be aware of the certain range of RSRP for devices. To reach all the devices, reader has to consider a very wide total RSRP range, leading to the inventory or service latency.]

- [NEC] notes that Option 2 can also be studied, but it relies on the underlying assumption that the reader’s transmit power remains constant.

- [Panasonic] notes that Option 2 offers a simpler RSRP‑based implementation and is therefore the preferred approach for Device 2b/C.

- [Samsung] notes that Option 2 relies on an RSRP‑to‑power lookup table with coarse or dense quantization and is therefore not considered feasible for low‑complexity device operation.

- [Apple] notes that Option 2 offers a simpler implementation suitable for device 2b, making it a more practical choice.

- [ETRI] states that Option 2 reduces complexity but offers only coarse power control due to its RSRP‑range‑based design.

- [Qualcomm] states that Option 2 offers a simpler RSRP‑range‑based approach that aligns well with the practicality of ultra‑low‑power devices and is generally more suitable for such deployments.

- [Quectel] states that Option 2 avoids pathloss calculation but offers only lower‑precision power control despite having similar measurement requirements.

- [ASUSTeK] states that Option 2 uses RSRP‑range‑based power control with lower calculation complexity but involves a trade‑off between required information overhead and power‑control performance.

- [KT] states that Option 2 offers a simpler RSRP‑range‑based method that aligns well with low‑complexity A‑IoT devices and can be enhanced through refined RSRP range design.

- [Fraunhofer] states that Option 2 provides a simpler RSRP‑range‑based open‑loop power control method with configurable power‑ramping behavior, making it suitable for A‑IoT devices with intermittent availability.

Necessity and feasibility of closed-loop Tx power control for D2R

Previous agreements

Agreement (RAN1#122bis)

Study the necessity of D2R Tx power control for Device 2b and for Device C for outdoor scenarios

- Target to agree on observations on pros and cons of D2R Tx power control vs. no Tx power control at RAN1#123

Continue studying feasibility of the following open loop power control methods considering device complexity:

- Methods of open loop power control

  - Option 1: Device calculates Tx power based on measured pathloss, a predefined formula, and some configured parameters from reader

    - Required information at the device side at least includes reader Tx power, received target power at the reader, etc.

  - Option 2: Device measures RSRP, and uses a corresponding Tx power when the RSRP falls in an RSRP range.

    - The RSRP range is based on RSRP threshold(s) configured by reader or predefined

    - Required information at the device side at least includes RSRP threshold(s), corresponding D2R Tx power related information per each RSRP range defined by the RSRP threshold(s)

  - FFS details (e.g., required measurements, required information)

Continue studying the necessity and feasibility of closed loop power control for D2R.

Work plan for RAN1#124 meeting (R1-2509614)

- D2R Tx power control - Necessity of D2R Tx power control (Pros and cons of Tx power control vs. no Tx power control for D2R) - Feasibility study of the two open-loop Tx power control methods for D2R - Necessity and feasibility of closed-loop Tx power control for D2R - Necessary information for D2R Tx power control and its provisioning

Summary

Views on necessity and feasibility of closed-loop Tx power control for D2R

- Both feasible and necessary: DOCOMO

- Necessary: Spreadtrum, OPPO, Nokia, ZTE, Samsung, KT

- [Feasible and] can be beneficial: Qualcomm

- Feasible but not necessary: LGE

- Not feasible: FUTUREWEI, Xiaomi

- Not necessary: vivo, ETRI

- [H] Proposal 3.3.3a-1

On the necessity and feasibility of closed‑loop Tx power control for D2R, capture the following in TR 38.769.

Necessity and feasibility of closed‑loop Tx power control for D2R are studied. Sources (DOCOMO, Spreadtrum, OPPO, Nokia, ZTE, Samsung, KT) indicate that closed‑loop Tx power control is necessary and source (DOCOMO) further states that it is feasible and necessary.

- [DOCOMO] states that closed‑loop power control can effectively mitigate interference in multi‑reader scenarios, with only minimal additional R2D overhead.

- [Spreadtrum] mentions that because device‑side measurement is difficult, at least closed‑loop D2R power control should be supported, with the reader measuring the D2R signal, determining the appropriate transmit power, and indicating it to the device for power control.

- [OPPO] mentions that closed‑loop power control is suitable for Msg 3 and other D2R data transmissions, where the reader measures the preceding D2R transmission and indicates a power‑control command to the device through the D2R scheduling message.

- [Nokia] observes that in a stationary environment, closed‑loop power control can achieve significantly higher accuracy than open‑loop power control due to device measurement errors, device power‑setting tolerance, and FDD uplink/downlink pathloss differences.

- [ZTE] notes that closed‑loop power control enables devices to transmit at a configured power level through R2D control signaling.

- [Samsung] notes that closed‑loop D2R power control should be studied, as reader feedback can refine device transmit power beyond OLPC accuracy.

- [KT] states that closed‑loop power control should be studied for D2R transmission, as it enables real‑time power adjustment based on reader feedback.

Source (LGE) states that closed‑loop Tx power control is feasible but not necessary.

- [LGE] notes that closed‑loop power control is feasible but not essential compared to open‑loop power control.

Source (Qualcomm) states that closed‑loop Tx power control [is feasible and] can be beneficial.

- [Qualcomm] states that the closed-loop power control [is feasible and] can be beneficial for interference management of device-specific D2R transmission at the price of higher overhead and complexity.

Sources (FUTUREWEI, Xiaomi) state that closed‑loop Tx power control is not feasible.

- [FUTUREWEI] mentions that closed‑loop power control is not feasible, noting that it introduces additional signaling overhead for power adjustment, is limited by device availability, and still requires support for open‑loop power control for the initial D2R transmission.

- [Xiaomi] notes that closed‑loop power control increases device power consumption due to continuous monitoring and is therefore not feasible for A‑IoT devices with limited power and low complexity.

Sources (vivo, ETRI) state that closed‑loop Tx power control is not necessary.

- [vivo] notes that closed‑loop power control is unnecessary for R20 A‑IoT devices if open‑loop power control is introduced, given the added design complexity and unclear HARQ‑ACK feedback mechanism.

- [ETRI] states that closed‑loop power control is not considered for Rel‑20 A‑IoT due to its signaling and energy overhead.

Source (Panasonic) states that the closed-loop power control would increase the signaling overhead and the device complexity, but a device/group-specific correction parameter could be considered along with the open-loop power control for better power control flexibility.

Necessary information for D2R Tx power control and its provisioning

Previous agreements

Agreement (RAN1#122)

Study necessity and feasibility of D2R Tx power control for Device 2b and for Device C, including the following aspects

- Open loop and/or closed loop power control

- Required measurements (e.g., signal strength)

- Required information at the device

- Required information at reader side

- Signaling overhead and power consumption

Work plan for RAN1#124 meeting (R1-2509614)

- D2R Tx power control - Necessity of D2R Tx power control (Pros and cons of Tx power control vs. no Tx power control for D2R) - Feasibility study of the two open-loop Tx power control methods for D2R - Necessity and feasibility of closed-loop Tx power control for D2R - Necessary information for D2R Tx power control and its provisioning

Summary

Views on necessary information for D2R Tx power control and its provisioning

- Required information at the device

  - Tx power of the reader: OPPO, Samsung, Qualcomm (Option 1)

  - Coefficient of pathloss: DOCOMO (Option 1)

  - Target received power at reader: Qualcomm (Option 1)

  - RSRP thresholds: Qualcomm (Option 2)

  - Tx power mapping per RSRP range: Qualcomm (Option 2)

- Required information at reader side

  - Available power of A-IoT device: Nokia

  - Device energy level: InterDigital

- [H] Proposal 3.3.4a-1

On the necessary information for D2R Tx power control and its provisioning, capture the following in TR 38.769.

On the required information for D2R Tx power control at the reader and device, sources (OPPO, Samsung, Qualcomm, DOCOMO) state that Tx‑power–related parameters are necessary.

- Sources (OPPO, Samsung, Qualcomm) state that the Tx power of the reader can be set by the reader.

- Source (DOCOMO) states that coefficient of pathloss can be beneficial for balancing between inter-cell interference and performance at cell edge for Option 1.

- Source (Qualcomm, Panasonic) states that the target received power at the reader can be configured by the reader for Option 1.

- Source (Qualcomm, Panasonic) states that the RSRP thresholds and the Tx‑power mapping per RSRP range can be configured by the reader for Option 2.

Sources (Nokia, InterDigital) state that device power/energy‑related information is needed [for closed-loop Tx power control for D2R].

- Source (Nokia) states that at least the available power of an A‑IoT device can be reported by the device.

- Source (InterDigital) states that D2R power control depends on the device energy level.

[Random access]

Necessity and feasibility of X1>2 for Msg1 and X3>1 for Msg3

Previous agreements

Agreement (RAN1#122)

At least for CBRA for DT and DO-DTT, study necessity and feasibility of enhancements of D2R TDM(A) for Device 2b and for Device C based on the following:

- X1 > 2 for Msg1 (X1: number of time domain resources for Msg1 transmission indicated by the corresponding R2D message)

- X3 > 1 for Msg3 (X3: number of time domain resources for Msg3 transmission indicated by the corresponding R2D message)

Work plan for RAN1#124 meeting (R1-2509614)

- Enhancements on random access from procedural aspects - Necessity and feasibility of X1>2 for Msg1 and X3>1 for Msg3 - Potential impact of D2R CDM(A) on random access procedures - Potential impact of R2D FDM on random access procedures

Summary

Views on necessity and feasibility of X1>2 for Msg1 and X3>1 for Msg3

- X1>2 for Msg1 is

  - both feasible and necessary: FUTUREWEI, ETRI, LGE, DOCOMO, ASUSTeK

  - feasible: InterDigital, Qualcomm, Samsung

  - feasible but not necessary: vivo, ZTE

  - not necessary: Huawei, Xiaomi

- X3>1 for Msg3 is

  - both feasible and necessary: FUTUREWEI, ETRI, LGE, DOCOMO, ASUSTeK

  - feasible: Samsung, InterDigital, Qualcomm

  - feasible but not necessary: vivo

  - not necessary: Huawei

  - not feasible: Xiaomi

  - neither feasible nor necessary: ZTE

- [H] Proposal 3.4.1a-1 [Further grouping needed]

On enhancements of D2R TDM(A) for Device 2b and for Device C for CBRA for DT and DO-DTT, capture the following in TR 38.769.

On enhancements to increase the number of time domain resources indicated by the corresponding R2D message for the Access Random ID message (Msg1) to greater than 2, sources (FUTUREWEI, LGE, ETRI, DOCOMO, ASUSTeK, InterDigital, Qualcomm, Samsung, vivo, ZTE, CMCC, , Panasonic, NEC, [Sharp]) state that it is feasible. Sources (FUTUREWEI, ETRI, LGE, DOCOMO, ASUSTeK, [Sharp]) further state that it is feasible and necessary, and sources (vivo, ZTE) state that it is feasible but not necessary. Sources (Huawei, Xiaomi) report that it is not necessary. Following observations were provided during the study.

- [FUTUREWEI] mentions that increasing X1 for Msg1 transmission is necessary, as the larger PRDCH overhead can be mitigated by allocating more time‑domain resources.

- [LGE] notes that the smaller residual SFO of Device 2b/C may enable the use of more than two TDM resources for Msg1. (e.g., X1 > 2).

- [ETRI] states that supporting more than two Msg1 time‑domain access occasions (X1 > 2) is feasible and necessary for device 2b and device C.

- [DOCOMO] states that supporting X1 > 2 for Msg1 can enhance multiplexing capacity and shorten the overall random‑access procedure.

- [ASUSTeK] states that supporting X1 = {1, 2, 4, 8} for Msg1 is reasonable given the larger coverage and improved SFO capability of active devices.

- [Samsung] notes that with the improved timing accuracy of Device 2b/C, X1 for Msg1 transmission can be increased (e.g., 4) though it must remain limited due to A‑IoT device constraints.

- [InterDigital] states that supporting X1 > 2 for Msg1 is feasible in Rel‑20 because active devices have higher capability and more accurate clocks.

- [Qualcomm] states that supporting X1 > 2 for Msg1 is feasible because device 2b/C can use more accurate clocks and calibration to reduce the required time gap between TDMed D2Rs.

- [ZTE] notes that increasing X1 is feasible due to smaller SFO and reduced guard time for Device 2b/C, but its necessity remains unclear because a larger X1 introduces multiple drawbacks and therefore requires further evaluation.

- [vivo] notes that although X1 > 2 for Msg1 is feasible due to the reduced SFO in Rel‑20, its necessity is low because the resulting latency‑reduction gain is very limited.

- [Huawei] mentions that larger values of X1 for Msg1 are unnecessary because the performance gain is limited and further decreases as X1 increase.

- [Xiaomi] notes that it is not necessary to support X1 > 2 for Msg1, as the 16 Msg1 transmission resources enabled by 8 FDM + 2 TDM D2R in Rel‑19 are already sufficient.

- [CATT] notes that the maximum number of time domain resources for Msg1 transmission should be limited to 11 for Device 2b/C, as larger values provide limited performance gain.

- [CMCC] notes that supporting X1 > 2 for Msg1 can be studied.

- [Panasonic] notes that supporting X1 > 2 for Msg1 transmissions could be considered due to the lower SFO of Device 2b/C.

- [Apple] states that configurable X1 values such as 2, 4, and 8 help reduce Msg1 collision probability in high‑density environments.

- [Ericsson] mentions that it should be determined whether X1 = 2 is sufficient for outdoor capacity and latency requirements, and that any need for enhancement beyond X1 = 2 for Msg1 should be justified with supporting evaluations.

- [Panasonic] states that the Device types 2b and C have more accurate oscillators with lower SFO. To reduce the signaling overhead for Msg1 trigger signal and Msg2, larger transmission occasions could be considered, i.e., X1>2 and X3>1.

- [NEC] states that X1 values such as 4 and 8 to provide more efficient access.

On enhancements to increase the number of time domain resources indicated by the corresponding R2D message for the D2R Upper Layer Data Transfer message (Msg3) to greater than 1, sources (FUTUREWEI, LGE, ETRI, DOCOMO, ASUSTeK, Samsung, InterDigital, Qualcomm, vivo, CMCC, Panasonic, NEC) state that it is feasible. Sources (FUTUREWEI, LGE, ETRI, DOCOMO, ASUSTeK) further state that it is feasible and necessary, and source (vivo) states that it is feasible but not necessary. Source (Huawei) reports that it is not necessary, source (Xiaomi) states that it is not feasible, and source (ZTE) states that is neither feasible nor necessary. Following observations were provided during the study.

- [FUTUREWEI] mentions that supporting X3 > 1 is feasible due to the higher clock accuracy of Device 2b/C, although increasing X3 beyond 2 is unlikely because of Msg3 length and clock drift.

- [LGE] notes that the smaller residual SFO of Device 2b/C may enable the use of multiple TDM resources for Msg3 (e.g., X3 > 1).

- [ETRI] states that supporting more than one Msg3 time‑domain access occasion (X3 > 1) is feasible and necessary for device 2b and device C.

- [DOCOMO] states that supporting X3 > 1 for Msg3 can improve random‑access efficiency and reduce total access time.

- [ASUSTeK] states that enhancements on X3 > 1 should be limited to X3 = 2, since supporting higher X3 adds design and signaling complexity without clear benefit.

- [Samsung] notes that the X3 capacity for Msg3 transmission should match that of Msg1 and can likewise be increased (e.g., 4) rather than being restricted to half as in Rel‑19.

- [InterDigital] states that supporting X3 > 1 for Msg3 is feasible in Rel‑20 with shorter gaps between time resources compared to Rel‑19.

- [Qualcomm] states that supporting X3 > 1 for Msg3 is feasible since improved clock accuracy of device 2b/C makes the time‑gap overhead marginal even for longer Msg3 durations.

- [vivo] notes that although X3 > 1 for Msg3 is feasible, its necessity is low given the similarly limited latency‑reduction benefit.

- [Huawei] mentions that larger values of X3 for Msg3 are unnecessary because the performance gain is limited and further decreases as X3 increase.

- [Xiaomi] notes that it is not feasible to support X3 > 1 for Msg3, as the device ID can be several hundred bits long.

- [ZTE] notes that supporting X3 > 1 is technically feasible but brings significant performance and scheduling penalties, so its necessity is weak and requires further study.

- [CATT] notes that TDMA support for Msg3 can reduce transmission latency, though the latency‑reduction benefit diminishes as the TDMA size increases, and at least X3 = 2 should be supported for Device 2b/C.

- [CMCC] notes that supporting X3 > 1 for Msg3 can be studied.

- [Panasonic] notes that supporting X3 > 1 for Msg3 transmissions could also be considered given the improved oscillator accuracy of Device 2b/C.

- [Apple] states that configurable X3 values greater than 1 allow parallel Msg3 processing and improve random‑access performance.

- [Ericsson] mentions that from the device perspective, X3 should be 1.

- [Panasonic] states that the Device types 2b and C have more accurate oscillators with lower SFO. To reduce the signaling overhead for Msg1 trigger signal and Msg2, larger transmission occasions could be considered, i.e., X1>2 and X3>1.

- [NEC] states that X3 values such as 2 and 4 to provide more efficient access.

Potential impact of D2R CDM(A) on random access procedures

Previous agreements

Agreement (RAN1#122bis)

Study necessity and feasibility of D2R CDM(A) for Device 2b and for Device C with sequence-based Msg1 transmission and potential impact to random access procedures.

- FFS: other cases, except for data transmission

Work plan for RAN1#124 meeting (R1-2509614)

- Enhancements on random access from procedural aspects - Necessity and feasibility of X1>2 for Msg1 and X3>1 for Msg3 - Potential impact of D2R CDM(A) on random access procedures - Potential impact of R2D FDM on random access procedures

Necessity and feasibility of D2R CDM(A) for Device 2b and for Device C with sequence-based Msg1 transmission will be discussed in sub-agenda 10.3.2.2.

Summary

Views on the potential impact of D2R CDM(A) on random access procedures

- If supported, random access procedure would need updates: FUTUREWEI, Huawei, LGE, DOCOMO, Qualcomm

- [H] Proposal 3.4.2a-1

On potential impact of D2R CDM(A) on random access procedure for Device 2b and for Device C, capture the following in TR 38.769.

Potential impact of D2R CDM(A) on random access procedure for Device 2b and for Device C is studied and the following observations were provided during the study.

- [FUTUREWEI] mentions that although CDMA can increase the number of supported Msg1 transmissions in the contention‑based access procedure, it would modify the handling of Msg2 and the scheduling of Msg3, while the dominant bottleneck remains the much larger Msg3 transmission.

- [Huawei] mentions that if sequence‑based Msg1 is introduced, the RA procedure becomes very similar to the legacy NR 4‑step RACH procedure and can be handled by RAN2 during the WI phase.

- [LGE] notes that the necessity of Msg1 CDMA remains unclear given existing Msg1 TDMA/FDMA designs, but if required, binary sequence–based Msg1 and related procedural enhancements—including sequence indication via Msg2 and predefined D2R modulation—should be considered.

- [NEC] notes that D2R CDM(A) for sequence‑based Msg1 is challenging for Device 2b due to its residual SFO/CFO and may also impact the random‑access procedure, requiring consideration of a four‑step RA process.

- [DOCOMO] states that collision handling for sequence‑based Msg1 should be studied and can be resolved with minimal enhancements in the RAN2 random‑access procedure.

- [Qualcomm] states that explicit or implicit contention resolution using Msg3 and Msg4 should be considered when sequence‑based CDMA is used for Msg1 in the CBRA procedure.

- [vivo] states that if msg1 is sequence-based, instead of RN16, contention resolution i.e., msg4 is needed for random access procedure, aiming to deal with the msg1 collision.

- [CMCC] states that contention resolution using Msg3 and Msg4 should be considered when sequence‑based CDMA is used for Msg1 in the CBRA procedure.

- [[CATT] states that sequence-based Msg1 could be supported. The length of sequences should not be shorter than 31. The NR preamble number could be as starting point for Device 2b/C in outdoor scenario.]

Proposals for Tuesday afternoon offline session

[Initial frequency/broadcast information acquisition]

Broadcast information

Previous agreements

Agreement (RAN#122bis)

For R2D, study at least the following aspects for the study of broadcast information:

- Information included in the broadcast information

- Whether to use PRDCH to carry the broadcast information

- Transmission of broadcast information:

  - Option 1: periodic

  - Option 2: aperiodic

- Time and frequency resources for transmission of the broadcast information, e.g. in relation to the synchronization signal.

Agreement (RAN#123)

- The functionality of broadcast information is to provide the device with at least necessary information for subsequent transmission and reception

- On how A-IoT devices acquire the broadcast information, study the following two cases:

  - Case B1: Single-step acquisition of the broadcast information

  - Case B2: Two-step acquisition of the broadcast information (e.g., MIB-like and then SIB1-like broadcast information)

- On whether to use PRDCH to carry the broadcast information,

  - For Case B1, PRDCH is used to carry the broadcast information.

  - For Case B2

    - PRDCH is used to carry the SIB1-like broadcast information.

    - A separate PRDCH or a new physical channel dedicated to MIB-like is used to carry the MIB-like broadcast information

Agreement (RAN#123)

Periodic transmission of broadcast information to provide at least necessary information for subsequent transmission and reception is necessary for Device 2b and for Device C.

- FFS (in SI or WI): whether the transmission of broadcast information is strictly periodic at reader

- FFS (in SI or WI): periodicity of the transmission of broadcast information

Work plan for RAN1#124 meeting (R1-2509614)

- Broadcast information - Purposes of the broadcast information and the necessary information per each purpose - Views on Case B1 and Case B2 for how A-IoT devices acquire the broadcast information - For Case B2, views on whether to use a separate PRDCH or a new physical channel to carry the MIB-like broadcast information - Time and freq. resource for the broadcast information, e.g. in relation to the synchronization signal To be discussed in AI 10.3.2.1

Summary

Information included in the broadcast information

- Reader ID related information: Ericsson, CMCC, Xiaomi, ZTE, vivo, LGE, NEC, ETRI, DCM, Qualcomm, Spreadtrum

- D2R power control related information: Ericsson, Nokia, vivo, LGE, Samsung, ETRI, DCM, Qualcomm

- Information related to resource allocation for the first D2R transmission for DO-A: OPPO, Ericsson, CMCC, Xiaomi, ZTE, vivo, LGE, NEC, China Telecom, Samsung, ETRI, DCM, Qualcomm, Spreadtrum

- Parameters for subsequent R2D reception (e.g., for L1 R2D control information, paging monitoring/reception): OPPO, Ericsson, Nokia, CMCC, Xiaomi, ZTE, vivo, Samsung, DCM, Qualcomm

- Timing information: CMCC, Xiaomi, ZTE, vivo, LGE, China Telecom, ETRI

- Reader selection/reselection related information: Ericsson, vivo

- SIB1-like scheduling information (for Case B2): vivo

- Other information deemed necessary by RAN2: Ericsson, vivo

- [H] Proposal 3.1.2a-1

On the potential necessary information to be included in the broadcast information, capture the following observations in TR 38.769:

On the potential information included in the broadcast information, at least the following is studied:

- Reader ID related information for reader identification/differentiation (Ericsson, CMCC, Xiaomi, ZTE, vivo, LGE, NEC, ETRI, DCM, Qualcomm, Spreadtrum, Lenovo, Huawei, Lenovo, [China Telecom])

- D2R power control related information (Ericsson, Nokia, vivo, LGE, Samsung, ETRI, DCM, Qualcomm, Huawei)

- The information related to resource allocation for the first D2R transmission for DO-A and sources (OPPO, Ericsson, CMCC, Xiaomi, ZTE, vivo, LGE, NEC, China Telecom, Samsung, ETRI, DCM, Qualcomm, Spreadtrum, Huawei)

- Timing information (CMCC, Xiaomi, ZTE, vivo, LGE, China Telecom, ETRI, Huawei)

- parameters for subsequence R2D reception (OPPO, Ericsson, Nokia, CMCC, Xiaomi, ZTE, vivo, Samsung, DCM, Qualcomm)

- Reader selection/reselection related information (Ericsson, vivo)

- SIB1-like scheduling information (for Case B2) (vivo, Huawei)

- A list of available frequencies for device access (Lenovo)

- Time–frequency resources for device transmission/reception (Lenovo)

- Other information deemed necessary by RAN2 (Ericsson, vivo)

Following observations were provided during the study.

- [OPPO] Broadcast information can contain time/frequency resource for L1 R2D control monitoring, time and frequency resources and related configurations for the initial DO-A transmission, etc.

- [Ericsson] Time offset (and possibly frequency resource) of the random-access occasions (RO) for the first D2R transmission (Msg1) for all traffic types (DO-A, DO-DTT, and DT). Cell-specific power control parameters. Time offset (and possibly frequency resource) of the paging occasions for Msg0 transmission of DT and DO-DTT traffic. Reader/cell selection information including certain RSRP level for reader selection/re-selection. D2R transmission configurations/parameters, e.g., size of preambles/midambles. Reader/network related information. Other information deemed necessary by RAN2.

- [CATT] It’s not necessary to include reader identification information, SFN, DO-A resource configuration, power control related parameters, device (un)availability related parameters and transmission parameters for L1 control information in broadcast information.

- [Nokia] Broadcast information, if supported, includes power control parameters such as target received power, allowed increment of transmission power, one or more chip duration values for a PRDCH for L1 control

- [CMCC] Study the following information to be included in broadcast information. Timing information, e.g. SFN-like info. Periodic random access resource configuration, e.g. periodicity, offset, available resource in each periodicity, and frequency resources for FDM transmission. DRX configuration for paging monitoring. Configurations of cell/reader-common L1 R2D control information. Cell ID related information.

- [Xiaomi] The number of Cell/Reader ID can be larger than 4, e.g., 1008, which may consider acquisition method based on synchronization signal sequence(s) in addition to MIB-like and/or SIB-like information. Timing information can be contained in broadcast information. Whether to use SFN or timing offset indication can be study by avoiding over-designing. Broadcast information containing L1 R2D control related information can provide configuration/updating flexibility.

- [ZTE] The following information can be considered included in the broadcast information: Paging-related information, timing information, e.g., SFN, frequency information for DL FDM, UL reference frequency position, DO-A resource related information, i.e., time and frequency resource information, parameters of R2D L1 control, e.g., chip duration, repetition number, code rate, cell/Reader/Transaction ID, SIP length information.

- [vivo] Study Case B2: Two-step acquisition of the broadcast information including both MIB-like and SIB1-like: The information can be carried by MIB-like: SFN information, AIoT SIB scheduling information. The information can be carried by SIB1-like: DOA resource configuration, L1 control related information configuration(s), R2D preamble configuration, The frequency information of AIOT carrier, DRX configuration for device power saving, Criteria for cell-(re)selection, Reader ID related information, Power control related information, Periodicity of sync signal (and the payload i.e., MIB). Note: other information is subject to RAN2 study.

- [LGE] Regarding the purpose and necessary information of broadcast information, it is observed that following information is needed: Cell/reader ID‑related information to enable reader identification. Resource‑allocation information for DO‑A transmissions. Timing information indicating the allocation timing of the D2R resources. D2R power‑control parameters for device transmit‑power control.

- [NEC] At least the following information can be carried in the A-IoT broadcast information. Scheduling Info for the first DO-A D2R transmission; Reader ID-related information; FFS: other information to be carried in A-IoT broadcast information.

- [China Telecom] In Rel-20, study the necessity and feasibility of the following scheduling information for device 2b/C: T/F resource configuration for SIB-like message and/or MSG1, SFN value, repetition index or period index of synchronization signal, time unit index (e.g., half-frame index), frequency offset between the synchronization signal and the SIB-like message or MSG1.

- [Samsung] Broadcast information is transmitted for the following purposes: Purpose #1: Triggering the inventory procedure (i.e., paging-like message for device discovery/presence checking), Purpose #2: Providing DO-A resources so that a device can initiate RA by itself, and Purpose #3: Providing reference information for open-loop power control. Broadcast information can carry at least one of the following information sets, depending on the purpose: Information for the inventory procedure trigger, DO-A resource information (e.g., periodicity, access occasion configuration, resource pool information, etc.) and Reference information related to R2D Tx power for power control.

- [ETRI] The broadcast information should include at least the following information elements: Cell/reader ID-related information, power control-related information, periodic D2R resource configuration for DO-A, and timing information (e.g., SFN-like information).

- [DCM] For broadcast information, following information should be considered. Information for reader differentiation/identification such as reader ID. Parameters for first D2R transmission for DO-A such as D2R resource, physical layer design related parameters such as chip duration, FEC, repetition, etc., and power control information. Parameters for subsequent R2D reception such as potential R2D reception resources of R2D sync signal, L1 R2D control, A-IoT paging, R2D command and corresponding physical layer design related parameters such as M value, FEC, repetition, etc.

- [Qualcomm] Periodic system information (SIB) can provide necessary information including at least: Reader ID-related information (e.g., higher layer reader ID), R2D-related configuration (frequency location(s) for R2D carriers, configuration of periodic R2D monitoring occasions for SS and paging, L1 control information per coverage level), D2R-related configuration (frequency location(s) for D2R carriers, configuration of periodic DO-A D2R resources, power control per coverage level).

- [Lenovo] states a two-step method for acquiring broadcast information. In the first step, the broadcast provides a list of available frequencies for device access. In the second step, further time–frequency resources are supplied for device transmission/reception, along with possible reader ID information.

- [Huawei] For the broadcast information, the MIB-like transmission contains the reader identifier, SFN and the time and frequency resource allocation information for the SIB-like transmission. The SIB-like broadcast transmission contains the time and frequency resource allocation of DO-A, power control parameters, ARFCN-like frequency location information, RSRP threshold/level for the R2D measurement and other necessary parameters for DO-A and the active device.

Preference for Case B1 and Case B2 for how A-IoT devices acquire the broadcast information

- Case B1: Spreadtrum, Ericsson, CATT, Xiaomi, ZTE, Panasonic, Samsung, Qualcomm, Quectel, Fraunhofer

- Case B2: FUTUREWEI, Huawei, CMCC, vivo, Lenovo

- Other views: NEC, Sharp, ETRI, DCM

- [H] Proposal 3.1.2b-1

On Case B1 and Case B2 for how A-IoT devices acquire the broadcast information, capture the following in TR 38.769:

On how A-IoT devices acquire the broadcast information, sources (Spreadtrum, Ericsson, CATTOPPO, Xiaomi, ZTE, Panasonic, Samsung, Qualcomm, Quectel, Fraunhofer) state that Case 1 (Single-step acquisition of the broadcast information) should be supported. Following observations on the Case 1 were provided during the study.

- [FUTUREWEI] Potential for high system overhead with frequent broadcast information transmission. Possible increase of device unavailability as a device may need to perform blind decoding of the L1 R2D control information before obtaining broadcast information.

- [Spreadtrum] For broadcast information, single-step acquisition of the broadcast information is supported. PRDCH is used to carry the broadcast information.

- [Ericsson] It is simpler and more efficient to transmit all the necessary information in a single step using a single PRDCH. Case B1 may also be better from reader and A-IoT device energy consumption perspectives.

- [CATTOPPO] Due to the limited battery capacity of A-IoT devices, it is desirable to provide all necessary system information to A-IoT devices at one time rather than introducing extra dedicated message for system information delivering (e.g., SIB1 like message), unless the payload size of the broadcast information identified at the end may result in an unacceptable A-IoT SSB length.

- [Xiaomi] If the payload size of broadcast information is small and fixed, there is no need to split it into two parts.

- [ZTE] For Case B1-1, the broadcast information includes L1 control, broadcast information scheduling is more flexible. For Case B1-2, the broadcast information does not include L1 control, the design is simpler and makes it easier for the device to detect the broadcast information.

- [vivo] If the periodicity of single broadcast information is same as that of periodic sync signal, larger resource overhead will be caused since some of the information is only used after initial reader search and don’t need to be transmission as frequent as periodic sync signal. If the periodicity of single broadcast information is larger than that of periodic sync signal, during initial search, a device needs to do more blind detection to find the potential broadcast information to obtain the essential information for completing the initial search e.g., SFN for timing alignment. This will increase the initial search complexity and latency. If the single broadcast information is SIB1-like information, the starting time location of SIB1-like broadcast information has to be fixed, which is less flexible for reader resource allocation.

- [LGE] The single‑step acquisition allows simple and fast acquisition at the device side but incurs signaling overhead and may introduce reader‑identification timing delay.

- [Panasonic] A single-step acquisition of broadcast signal using PRDCH should be considered.

- [Samsung] A single-step approach (i.e., Case B1) should be used for broadcast information, and introducing a two-step structure (i.e., Case B2) is not strongly motivated from an information-size perspective.

- [Sharp] For Case B1, the broadcast PRDCH should always be preceded by a synchronizing signal to facilitate detection and acquisition.

- [Qualcomm] For Case B1, devices may perform SIB monitoring with a predefined periodicity and detect the SIB directly. This approach is more straightforward and potentially more efficient.

- [Quectel] For the acquisition method of broadcast information, study single-step acquisition, i.e., Case B1 as baseline. Single-step acquisition of broadcast is simpler and costs less standardization efforts than two-step acquisition method. MIB-like in Ambient IoT is not necessary since ambient IoT devices do not require precise synchronization like SFN information and carrier offset. Another reason is the message size of broadcast information may be fixed and is not as large as SIB information.

- [Fraunhofer] For broadcast information acquisition prioritize Case B1. Single‑step acquisition minimizes the number of decoding tasks, and therefore, reducing the device activation time and avoiding the need to coordinate multiple acquisition stages. On the contrary, Case B2 introduces additional operational steps, requiring devices to decode two separate layers of system information, potentially from different physical channels. While this structure allows for modularity and independent periodicities for MIB‑like and SIB1‑like information, it also increases decoding complexity and extends the time during which devices must remain active. Therefore, Case B1 should be prioritized given the energy constrains of A-IoT devices.

Sources (FUTUREWEI, Huawei, CMCC, vivo, Lenovo) state that Case 2 (Two-step acquisition of the broadcast information) should be supported. Following observations were provided during the study.

- [FUTUREWEI] Allows the PRDCH containing broadcast information to be similar to other PRDCH by providing parameters to decode L1 R2D control information. Allows a device to know whether to process PRDCH containing broadcast information and whether the PRDCH containing broadcast information is scheduled. Allows a reader to coordinate device availability of all devices it serves.

- [Huawei] Advantages of two-step acquisition of the broadcast information (Case B2): The two-step split acquisition of the broadcast information is to facilitate an efficient method to provide the required information to the device at the required time - the system information carried by the MIB-like transmission is required by the device more frequently than the DO-A resource allocation information carried by the SIB-like broadcast information since it does not need to change frequently. The overhead of the resource allocation information for DO-A is larger than the other system information, and transmitting this in the SIB-like transmission with a longer periodic interval as compared to the MIB-like broadcast information can avoid a degradation of transmission efficiency. The impact of overhead is reduced when compared with Case B1 where a single-step acquisition of the broadcast information is used. The MIB-like transmission contains the reader identifier, SFN and the time and frequency resource allocation information for the SIB-like transmission. The SIB-like broadcast transmission contains the time and frequency resource allocation of DO-A, power control parameters, ARFCN-like frequency location information, RSRP threshold/level for the R2D measurement and other necessary parameters for DO-A and the active device.

- [ZTE] For Case B2, the design is more complex compared with Case B1, and incurs higher signaling overhead, which increases downlink blockage probability and consequently degrades inventory efficiency.

- [vivo] MIB-like is regarded as the payload of periodic sync signal to carry the minimum required information for device to complete the initial reader search, which has the same periodicity as that of periodic sync signal. Thus, it is less complex for a device to finish the initial reader search within the periodic sync signal cycle. With two-step acquisition of the broadcast information, the periodicity of SIB1-like can be more flexible e.g., larger than that of MIB-like to avoid unnecessary resource overhead. MIB-like can flexibly indicate the scheduling information of SIB1 e.g., the start time location of SIB1, which is benefit for reader resource allocation.

- [LGE] The two‑step acquisition reduces signaling overhead and allows early reader identification but may cause acquisition delay and may require additional specification work for a new physical channel.

- [Sharp] For Case B2, the MIB-like channel should be designed to be as simple as possible, in order to minimize device complexity.

- [Lenovo] Support two-step acquisition of the broadcast information (i.e., Case B2). In the first step, broadcast information transmitted on initial frequency provides a list of available frequencies for devices to access. In the second step, broadcast information transmitted on the available frequencies provides further resource occasions for devices to access.

- [Qualcomm] For Case B2, devices may perform MIB monitoring with a predefined periodicity. Devices first detect the MIB to obtain the SIB monitoring configuration and then acquire the SIB.

- [Quectel] For case B2, i.e., two step acquisition of the broadcast information results in more complexity comparing with single step acquisition.

Sources (NEC, Sharp, ETRI, DCM) state that the decision can be made after the content/size of the broadcast information becomes clear. Following observations were provided during the study.

- [NEC] RAN1 can further decide whether single-step or two-step acquisition is required for broadcast information in Rel-20 A-IoT after all the information carried by broadcast information and corresponding size have been clearly identified.

- [Sharp] The down-selection between Case B1 and Case B2 may depend on how much broadcast information needs to be carried, as well as the resulting impact on device complexity, coverage, and power consumption.

- [ETRI] The introduction of an MIB/SIB-based structure for broadcast information should be decided after the contents of the broadcast information are agreed.

- [DCM] Whether the broadcast information acquisition is single-step or two-step depends on the payload size of whole broadcast information, i.e., two-step is necessary if it would be expected to be large, otherwise, single-step is sufficient.

Views on whether to use a separate PRDCH or a new physical channel to carry the MIB-like broadcast information for Case B2

- PRDCH: Huawei, Spreadtrum, OPPO, Ericsson, CATT, CMCC, Xiaomi, vivo, LGE, China Telecom, Fraunhofer

- New PHY channel: CMCC

  - Definition of the “new” physical channel: For MIB-like part, the R2D parameters such as TBS, payload size, FEC or repetition number can be predefined.

- [H] Proposal 3.1.2c-1

On whether to use a separate PRDCH or a new physical channel to carry the MIB-like broadcast information for Case B2, capture the following in TR 38.769:

For Case 2, sources (Huawei, Spreadtrum, OPPO, Ericsson, CATT, CMCC, Xiaomi, vivo, LGE, China Telecom, Fraunhofer) state that PRDCH should be used to carry the MIB-like broadcast information, while sources (CMCC, FUTUREWEI) states that are also open to use a new physical channel dedicated to MIB-like information should be used. During the study, following observations were provided.

- [Huawei] Firstly, we do not see the PRDCH is used to carry the MIB-like broadcast information as there is no additional motivation to define a separate or new channel like NR SSB, in which 8 information bits are carried in the physical layer payload. Secondly, carrying the MIB-like broadcast information on an additional channel would increase the workload in terms of specification work for the WI.

- [SRPD] For broadcast information, single-step acquisition of the broadcast information is supported and PRDCH is used to carry the broadcast information.

- [China Telecom] If case B2 is selected for the broadcast information transmission scheme, support to reuse a separate PRDCH for the MIB-like broadcast information. While using PRDCH may require a new R2D message format for MIB-like information, defining an entirely new broadcast channel would be significantly a larger workload. Given the limited TU before the SID is completed, reusing the PRDCH to carry MIB-like information would be more feasible and simpler to achieve.

- [vivo] Use PRDCH as the transmission channel for both MIB-like and SIB1-like broadcast information. Do not introduce a new channel to carry MIB-like broadcast information.

while sSources (CMCC, FUTUREWEI) states that are also open to use a new physical channel dedicated to MIB-like information should be used. During the study, following observations were provided.

- [CMCC] MIB-like broadcast information can be carried by a separate PRDCH or a new physical channel dedicated to MIB-like information. For MIB-like part, the R2D parameters such as TBS, payload size, FEC or repetition number can be predefined.

- [FUTUREWEI] Whether the MIB-like information is carried in its own channel can follow the concept of the PBCH, which is a separate channel for the MIB. This separate channel can simplify the specifications.

Not limited to Case 2, following sources state that a new physical channel other than PRDCH should not be used to carry broadcast information.

- [OPPO] It is not necessary to introduce a new physical channel other than PRDCH to convey the broadcast information. There is no need to introduce a new physical channel other than PRDCH because the PRDCH was already used to transmit A-IoT paging message in Rel-19 A-IoT and the A-IoT paging message is also a kind of broadcast information especially when Paging ID Presence Indication field indicates Paging ID field is absent.

- [Ericsson] Using PRDCH to carry broadcast information could reduce the design complexity and specification impact. The broadcast information should be transmitted periodically in PRDCH with fixed parameters and the periodicity could be different from the periodic sync signal.

- [CATT] If the transmission parameters for L1 control information are indicated by broadcast information, a new physical channel other than PRDCH must be defined to carry the broadcast information including the configurations for L1 control information. Otherwise, if the broadcast information carried by PRDCH schedules a L1 control information, there will be a chicken-and-egg problem in the scheduling relationship between L1 control information and PRDCH. The introduction of a new channel will increase the complexity of R2D reception and relative standardization work.

- [Xiaomi] PRDCH in case B1 is capable to contain the single part of broadcast information.

- [LGE] If the introduction of a new physical channel for the partial broadcast information is considered, the specification work may significantly increase.

- [NEC] PRDCH is used to carry the broadcast information, no need to introduce new channel(s) for R2D. For the channel to carry this broadcast information, we prefer to reuse PRDCH since the coding/repetition schemes for broadcast information may not be quite different, and no specific RS will be designed for the broadcast information. Furthermore, keep only one channel for each of R2D or D2R would maintain the simplest design for Rel-20 A-IoT.

- [Fraunhofer] PRDCH is used to carry the broadcast information. From a device perspective, supporting multiple channels undermines the simplicity benefits targeted for A-IoT. Furthermore, the separation of MIB-like and SIB1-like information does not provide a clear functional advantage for the devices, given the limited system information needs.

- [CMCC] MIB-like broadcast information can be carried by a separate PRDCH or a new physical channel dedicated to MIB-like information. For MIB-like part, the R2D parameters such as TBS, payload size, FEC or repetition number can be predefined.

Proposals for Tuesday afternoon online session

[A-IoT positioning]

Work plan for RAN1#124 meeting (R1-2509614)

- Positioning - Which D2R signal(s)/channel(s) applicable to positioning - Measurement aspects (e.g., metric) for positioning

According to Vice Chair’s guidance, the aspects related to A-IoT positioning will also be handled in this sub-agenda.

The related SID objective on A-IoT positioning is shown below.

Objectives … - Identify D2R RSRP-like measurement, and the involved A-IoT signal(s)/channel(s), which are feasible for Device 2b/Device C for more accurate outdoor Device localization than based on Reader-ID [RAN1]. - RAN1 is not expected to conclude on the positioning accuracy as part of the identification of feasible D2R RSRP-like measurement - Note: Device localization based on Reader ID as specified in Rel-19 can be supported for active device(s) in Rel-20 with no additional work. - The design of D2R signal(s)/channel(s) for active devices is not impacted by the identification of the D2R RSRP-like measurement - This objective does not include the study of dedicated Device procedures for Device localization - The objective assumes no dedicated positioning architecture will be specified in Rel-20 - The objective assumes no specification of inter-Reader coordination for Device localization in Rel-20 - This objective will start at RAN1#124

Which D2R signal(s)/channel(s) applicable to positioning

Work plan for RAN1#124 meeting (R1-2509614)

- Positioning - Which D2R signal(s)/channel(s) applicable to positioning - Measurement aspects (e.g., metric) for positioning

Based on the SID, the discussion in this sub-clause focuses on “which” D2R signal/channel is applicable to positioning.

Summary

Views on which D2R signal(s)/channel(s) is/are applicable to A-IoT positioning

- D2R preamble: FUTUREWEI, Ericsson, Apple,

- D2R preamble and midamble(s): Huawei, Ericsson, vivo, DCM,

- D2R preamble and/or midamble(s): OPPO, Xiaomi, ZTE,

- D2R signals (sequence embedded in the PDRCH) and D2R preamble: CATT

- D2R preamble and/or midamble(s), and PDRCH: Qualcomm,

- [H] Proposal 3.6.1a-1

On the D2R signal(s)/channel(s) applicable to A-IoT device localization, capture the following observations in TR 38.769.

D2R signal(s)/channel(s) which is/are feasible and applicable for A-IoT device localization is/are studied. AsFor the D2R signal(s)/channel(s) which is/are feasible and applicable for A-IoT device localization, sources (FUTUREWEI, Ericsson, Apple, vivo) mention D2R preamble.

- [FUTUREWEI] For D2R RSRP-like measurement, the reader can use the D2R preamble for the reference signal and measure the power between the end of R2D transmission and start of the D2R transmission to estimate the noise and interference.

- [Apple] D2R preamble is the applicable signal for RSRP-like measurement. D2R RSRP-like measurement can be performed on existing D2R preamble without requiring dedicated positioning signals, enabling proximity determination for outdoor device localization.

- [vivo] D2R preamble can be used for the positioning purpose. And no need to define D2R RSRP-like metric.

Sources (Huawei, Ericsson, vivo, DCM) mention D2R preamble and midamble(s) for the D2R signal(s)/channel(s) which is/are feasible and applicable for A-IoT device localization.

- [Huawei] D2R preamble and midamble signals are used for D2R RSRP-like measurement for device localization.

- [Ericsson] D2R-amble-RSRP is defined as the linear average over the power contributions (in [W]) of chips that carry the D2R preamble over PDRCH. A secondary measurement can be defined as the linear averaging over the power contributions of chips that carry D2R preamble and D2R midambles over PDRCH.

- [DCM] For positioning, at least D2R RSRP measurement on D2R preamble and D2R midamble when m sequence is used for midamble are feasible. In the previous meetings, it was agreed to consider m sequence and Gold sequence for D2R preamble, and it was agreed to consider m sequence, all 1 sequence and Hadamard sequence for D2R midamble. From our perspective, m sequence and Gold sequence can be used for RSRP measurement at reader thanks to good auto-correlation property, and RSRP measurement based on m sequence and Gold sequence has been widely used in NR and should be feasible.

Sources (OPPO, Xiaomi, ZTE) indicate D2R preamble and/or midamble(s).

- [OPPO] D2R preamble and/or mid-amble, which is applicable for all of D2R transmissions even for sequence-based Msg 1 without associated PDRCH, can be considered for D2R RSRP measurement.

- [Xiaomi] The current D2R signal structure including a preamble, midamble(s), and PDRCH, where preamble and midamble(s) are sequence based on the current agreements. Therefore, for RSRP-like measurement, it is feasible for Reader to measure the D2R preamble or midamble(s).

- [ZTE] Based on introduction of open loop or closed loop power control for D2R, reader is feasible to use D2R preamble or midamble to obtain more accurate localization information.

Source (CATT) states D2R signals (sequence embedded in the PDRCH) and D2R preamble, and source (Qualcomm) states D2R preamble and/or midamble(s), and PDRCH. Following observations were provided during the study.

- [CATT] A sequence with length 127, which is designed for the A-IoT device positioning, was embedded in the PDRCH for the D2R RSRP-like measurements. For the study of sequence length and type, we conducted the following length-31, length-63 and length-127 of M-sequence and Gold sequence with evaluation results. From the evaluation results, it can be seen length-127 M-sequence or Gold-sequence show a lower residual SFO error comparing to that of length-31 M-sequences or Gold-sequence and length-63 M-sequences or Gold-sequence. Since SFO is one of the main factors to measurement accuracy of RSRP, gold sequences or m-sequences with a length of 127 can be used as candidates.

- [Qualcomm] For D2R RSRP-measurement based positioning and proximity determination, PDRCH and D2R preamble/midamble with sufficient time duration and stable transmit power can be considered for D2R measurement. Using multiple measurements or extending D2R transmission increases accuracy. For D2R measurement, unicast D2R transmission can be used: Alt1: D2R Preamble/Midamble, Alt2: PDRCH. For Alt.1, the preamble and/or midamble can be used for RSRP measurement, provided they have sufficient transmission time and combined appropriately. For Alt.2, the PDRCH carries D2R control or data payload and repetitions for reliability can be used to improve power measurement accuracy.

D2R RSRP-like measurement and its feasibility

Work plan for RAN1#124 meeting (R1-2509614)

- Positioning - Which D2R signal(s)/channel(s) applicable to positioning - Measurement aspects (e.g., metric) for positioning

Based on the SID, the D2R RSRP-like measurement which is feasible for Device 2b/Device C for more accurate outdoor Device localization than based on Reader-ID is discussed in this sub-clause.

Summary

Measurement aspects (e.g., metric) for positioning

- Definition: FUTUREWEI, Huawei, Ericsson, CATT, Xiaomi,

- Noise and interference estimation: FUTUREWEI

- Additional measurements (AOA measurement): CATT

- [H] Proposal 3.6.2a-1

On the measurement aspects (e.g., metric) for A-IoT device localization, capture the following observations in TR 38.769.

Regarding the D2R RSRP-like measurement for A-IoT device localization, following observations were provided during the study.

- [FUTUREWEI] Use NR measurements (SL PRS-RSRP and UL-SRS-RSRP) as a reference. For D2R RSRP-like measurement, the reader can use the D2R preamble for the reference signal and measure the power between the end of R2D transmission and start of the D2R transmission to estimate the noise and interference. To estimate proximity, a reader can also consider the effects of propagation delay based on when the D2R transmission is received.

- [Huawei] D2R RSRP-like measurement is defined as the time domain linear average over the power contributions (in [W]) of preamble and midamble signals within the signal transmission bandwidth during the D2R transmission.

- [Ericsson] D2R-amble-RSRP is defined as the linear average over the power contributions (in [W]) of chips that carry the D2R preamble over PDRCH. A secondary measurement can be defined as the linear averaging over the power contributions of chips that carry D2R preamble and D2R midambles over PDRCH.

- [CATT] The definition of RSRP-like and AOA measurement quantity can refer to TR 38.215(NR; Physical layer measurements).

- [Xiaomi] The RSRP-like definition in A-IoT is the linear average of the received power in occupied chips and over the frequency resources that carry the measured signal. For example, if D2R is applied with OOK modulation, referring to the definition of LP-RSRP where measured signal is also an OOK waveform, RSRP-like definition in A-IoT is the linear average of the received power in OOK ON chips over the frequency resources that carry the measured signal.

- [vivo] Measurement can be up to network implementation based on D2R preamble, no need to define D2R RSRP-like metric. And nothing needs to be specified also.

- [Qualcomm] At reader side, how to measure the RSRP may be up to reader implementation. However, it may require the averaging of the D2R measurements based on D2R signals/channels with sufficient time duration and stable transmit power from device side.

- [NEC] D2R RSRP based measurement and positioning is feasible. R20 AIoT should support D2R RSRP based measurement.

Other aspects

This sub-clause is to discuss other aspects related to A-IoT device localization, such as positioning methods, evaluations results, etc.

Summary

On the positioning methods, following observations were provided:

[FUTUREWEI] A reader can combine timing and power measurements for proximity determination. A reader can estimate the distance to the device based on the delay measurement and accounting for the SFO.

[Huawei] D2R RSRP measurement based on preamble and midamble, and fingerprint-based positioning scheme is adopted. The positioning algorithm is Weighted K-Nearest Neighbor (WKNN).

[Ericsson] Assuming Tx power of the device is known by the reader and LOS between A-IoT and the reader, one approach for having a more granular outdoor device localization than the reader-based is that the reader can compare the RSRP of D2R signal with different RSRP threshold values, and estimate the zone that the device is located in.

[CATT] Joint estimation of distance from RSRP-like measurements and AOA estimation with reader-based beam sweeping in the implementation is proposed.

[Xiaomi] Reader determines whether A-IoT device is far from or close to itself by comparing the measured RSRP value with an RSRP threshold. Reader determines a corresponding distance range when the measured RSRP value falls in an RSRP range.

[NEC] D2R RSRP is a feasible approach at the current stage. Base on D2R RSRP value, at least device could be grouped into near group or far group roughly.

[Apple] Proximity determination based on D2R RSRP measurement is sufficient for outdoor use-cases requiring several tens of meters accuracy, without needing dedicated positioning architecture. Further consider more finer granularity of proximity including, very near, near and far depending up on the measurement strength.

[Qualcomm] Multi-reader positioning based on RSRP measurement is evaluated using weighted-centroid algorithm. The position estimate is given by a weighted average of BS reader locations, wherein the weights are a function of the RSRP measurement at each of the BS/readers and the same D2R transmission using a 63-length m-sequence is measured per reader.

[Fraunhofer] Introduce an explicit mechanism to set a context for positioning-related signaling. Even when high-accuracy positioning is not required, establishing a coarse spatial relationship such as a coverage zone is beneficial. To enable this, it is important that measurements and signaling exchanges are interpreted within a clear operational context. In particular, devices should be able to distinguish whether a reception instance is intended for regular inventory or command operation, or for positioning-related measurement purposes. Such contextual awareness will prevent unnecessary activity or energy consumption at the device.

- Proposal 3.6.3a

On A-IoT device localization methods, capture the following observations in TR 38.769.

On A-IoT device localization for Device 2b/Device C for more accurate outdoor Device localization than based on Reader-ID, following observations on device localization methods were reported.

- [FUTUREWEI] A reader can combine timing and power measurements for proximity determination. A reader can estimate the distance to the device based on the delay measurement and accounting for the SFO.

- [Huawei] D2R RSRP measurement based on preamble and midamble, and fingerprint-based positioning scheme is adopted. The positioning algorithm is Weighted K-Nearest Neighbor (WKNN).

- [Ericsson] Assuming Tx power of the device is known by the reader and LOS between A-IoT and the reader, one approach for having a more granular outdoor device localization than the reader-based is that the reader can compare the RSRP of D2R signal with different RSRP threshold values, and estimate the zone that the device is located in.

- [CATT] Joint estimation of distance from RSRP-like measurements and AOA estimation with reader-based beam sweeping in the implementation is proposed.

- [Xiaomi] Reader determines whether A-IoT device is far from or close to itself by comparing the measured RSRP value with an RSRP threshold. Reader determines a corresponding distance range when the measured RSRP value falls in an RSRP range.

- [NEC] D2R RSRP is a feasible approach at the current stage. Base on D2R RSRP value, at least device could be grouped into near group or far group roughly.

- [Apple] Proximity determination based on D2R RSRP measurement is sufficient for outdoor use-cases requiring several tens of meters accuracy, without needing dedicated positioning architecture. Further consider more finer granularity of proximity including, very near, near and far depending up on the measurement strength.

- [Fraunhofer] Introduce an explicit mechanism to set a context for positioning-related signaling. Even when high-accuracy positioning is not required, establishing a coarse spatial relationship such as a coverage zone is beneficial. To enable this, it is important that measurements and signaling exchanges are interpreted within a clear operational context. In particular, devices should be able to distinguish whether a reception instance is intended for regular inventory or command operation, or for positioning-related measurement purposes. Such contextual awareness will prevent unnecessary activity or energy consumption at the device.

- [ZTE] In Rel-20 Ambient IoT, based on D2R preamble or midamble, it is possible for reader to use these ambles to do the RSRP-like measurement and obtain the localization information if the power control including open loop or closed loop power control is introduced.

- [Qualcomm] Multi-reader positioning based on RSRP measurement is evaluated using weighted-centroid algorithm. The position estimate is given by a weighted average of BS reader locations, wherein the weights are a function of the RSRP measurement at each of the BS/readers and the same D2R transmission using a 63-length m-sequence is measured per reader.

Evaluation results were provided by 3 companies:

[Huawei] D2R RSRP-like measurement is feasible for Device C to achieve more accurate outdoor Device localization than based on Reader-ID. D2R RSRP measurement based on preamble and midamble, and fingerprint-based positioning scheme is adopted. The positioning algorithm is Weighted K-Nearest Neighbor (WKNN). Achieved localization performance, i.e., 82m@90%.

[CATT] After SFO/CFO calibration, the positioning accuracy of RSRP+AOA method is about 17m (90% CDF) for Device 2b and is about 13m (90% CDF) for Device C.

[Qualcomm] Multi-reader positioning based on RSRP measurement is evaluated using weighted-centroid algorithm. The position estimate is given by a weighted average of BS reader locations, wherein the weights are a function of the RSRP measurement at each of the BS/readers and the same D2R transmission using a 63-length m-sequence is measured per reader. For outdoor positioning, Outdoor BS/reader can achieve position estimation about 72~114% of cell radius at 80~90% percentage error for Device 2b in UMi layout, and about 84~110% of cell radius estimation at 80~90% percentage error for Device C in UMa layout. For proximity determination to identify a unique device relative to a reader, in Outdoor scenario, UE/reader can reach 95~98% success rate with 2 to 6 nearby devices; BS reader can achieve 48~80%. In Indoor scenario, UE/reader can reach 77~92% success rate with 2 to 6 nearby devices; BS/reader can achieve 56~83%. BS/readers generally perform worse, primarily because fixed BS/reader cannot approach the devices as flexible as UE/reader.

- Proposal 3.6.3b

On A-IoT device localization methods, capture the following observations in TR 38.769.

On A-IoT device localization for Device 2b/Device C for more accurate outdoor Device localization than based on Reader-ID, sources (Huawei, CATT, Qualcomm) provided evaluation results.

- [Huawei] D2R RSRP-like measurement is feasible for Device C to achieve more accurate outdoor Device localization than based on Reader-ID. D2R RSRP measurement based on preamble and midamble, and fingerprint-based positioning scheme is adopted. The positioning algorithm is Weighted K-Nearest Neighbor (WKNN). Achieved localization performance, i.e., 82m@90%.

- [CATT] After SFO/CFO calibration, the positioning accuracy of RSRP+AOA method is about 17m (90% CDF) for Device 2b and is about 13m (90% CDF) for Device C.

- [Qualcomm] Multi-reader positioning based on RSRP measurement is evaluated using weighted-centroid algorithm. The position estimate is given by a weighted average of BS reader locations, wherein the weights are a function of the RSRP measurement at each of the BS/readers and the same D2R transmission using a 63-length m-sequence is measured per reader. For outdoor positioning, Outdoor BS/reader can achieve position estimation about 72~114% of cell radius at 80~90% percentage error for Device 2b in UMi layout, and about 84~110% of cell radius estimation at 80~90% percentage error for Device C in UMa layout. For proximity determination to identify a unique device relative to a reader, in Outdoor scenario, UE/reader can reach 95~98% success rate with 2 to 6 nearby devices; BS reader can achieve 48~80%. In Indoor scenario, UE/reader can reach 77~92% success rate with 2 to 6 nearby devices; BS/reader can achieve 56~83%. BS/readers generally perform worse, primarily because fixed BS/reader cannot approach the devices as flexible as UE/reader.

[Initial frequency/broadcast information acquisition]

Broadcast information

Previous agreements

Agreement (RAN#122bis)

For R2D, study at least the following aspects for the study of broadcast information:

- Information included in the broadcast information

- Whether to use PRDCH to carry the broadcast information

- Transmission of broadcast information:

  - Option 1: periodic

  - Option 2: aperiodic

- Time and frequency resources for transmission of the broadcast information, e.g. in relation to the synchronization signal.

Agreement (RAN#123)

- The functionality of broadcast information is to provide the device with at least necessary information for subsequent transmission and reception

- On how A-IoT devices acquire the broadcast information, study the following two cases:

  - Case B1: Single-step acquisition of the broadcast information

  - Case B2: Two-step acquisition of the broadcast information (e.g., MIB-like and then SIB1-like broadcast information)

- On whether to use PRDCH to carry the broadcast information,

  - For Case B1, PRDCH is used to carry the broadcast information.

  - For Case B2

    - PRDCH is used to carry the SIB1-like broadcast information.

    - A separate PRDCH or a new physical channel dedicated to MIB-like is used to carry the MIB-like broadcast information

Agreement (RAN#123)

Periodic transmission of broadcast information to provide at least necessary information for subsequent transmission and reception is necessary for Device 2b and for Device C.

- FFS (in SI or WI): whether the transmission of broadcast information is strictly periodic at reader

- FFS (in SI or WI): periodicity of the transmission of broadcast information

Work plan for RAN1#124 meeting (R1-2509614)

- Broadcast information - Purposes of the broadcast information and the necessary information per each purpose - Views on Case B1 and Case B2 for how A-IoT devices acquire the broadcast information - For Case B2, views on whether to use a separate PRDCH or a new physical channel to carry the MIB-like broadcast information - Time and freq. resource for the broadcast information, e.g. in relation to the synchronization signal To be discussed in AI 10.3.2.1

Summary

Information included in the broadcast information

- Reader ID related information: Ericsson, CMCC, Xiaomi, ZTE, vivo, LGE, NEC, ETRI, DCM, Qualcomm, Spreadtrum

- D2R power control related information: Ericsson, Nokia, vivo, LGE, Samsung, ETRI, DCM, Qualcomm

- Information related to resource allocation for the first D2R transmission for DO-A: OPPO, Ericsson, CMCC, Xiaomi, ZTE, vivo, LGE, NEC, China Telecom, Samsung, ETRI, DCM, Qualcomm, Spreadtrum

- Parameters for subsequent R2D reception (e.g., for L1 R2D control information, paging monitoring/reception): OPPO, Ericsson, Nokia, CMCC, Xiaomi, ZTE, vivo, Samsung, DCM, Qualcomm

- Timing information: CMCC, Xiaomi, ZTE, vivo, LGE, China Telecom, ETRI

- Reader selection/reselection related information: Ericsson, vivo

- SIB1-like scheduling information (for Case B2): vivo

- Other information deemed necessary by RAN2: Ericsson, vivo

- [H] Proposal 3.1.2a-1-Alt.2

On the potential [necessary] information to be included in the broadcast information, capture the following observations in TR 38.769:

On the potential information included in the broadcast information, following observations were provided during the study.

- [OPPO] Broadcast information can contain time/frequency resource for L1 R2D control monitoring, time and frequency resources and related configurations for the initial DO-A transmission, etc.

- [Ericsson] Time offset (and possibly frequency resource) of the random-access occasions (RO) for the first D2R transmission (Msg1) for all traffic types (DO-A, DO-DTT, and DT). Cell-specific power control parameters. Time offset (and possibly frequency resource) of the paging occasions for Msg0 transmission of DT and DO-DTT traffic. Reader/cell selection information including certain RSRP level for reader selection/re-selection. D2R transmission configurations/parameters, e.g., size of preambles/midambles. Reader/network related information. Other information deemed necessary by RAN2.

- [CATT] It’s not necessary to include reader identification information, SFN, DO-A resource configuration, power control related parameters, device (un)availability related parameters and transmission parameters for L1 control information in broadcast information.

- [Nokia] Broadcast information, if supported, includes power control parameters such as target received power, allowed increment of transmission power, one or more chip duration values for a PRDCH for L1 control

- [CMCC] Study the following information to be included in broadcast information. Timing information, e.g. SFN-like info. Periodic random access resource configuration, e.g. periodicity, offset, available resource in each periodicity, and frequency resources for FDM transmission. DRX configuration for paging monitoring. Configurations of cell/reader-common L1 R2D control information. Cell ID related information.

- [Xiaomi] The number of Cell/Reader ID can be larger than 4, e.g., 1008, which may consider acquisition method based on synchronization signal sequence(s) in addition to MIB-like and/or SIB-like information. Timing information can be contained in broadcast information. Whether to use SFN or timing offset indication can be study by avoiding over-designing. Broadcast information containing L1 R2D control related information can provide configuration/updating flexibility.

- [ZTE] The following information can be considered included in the broadcast information: Paging-related information, timing information, e.g., SFN, frequency information for DL FDM, UL reference frequency position, DO-A resource related information, i.e., time and frequency resource information, parameters of R2D L1 control, e.g., chip duration, repetition number, code rate, cell/Reader/Transaction ID, SIP length information.

- [vivo] Study Case B2: Two-step acquisition of the broadcast information including both MIB-like and SIB1-like: The information can be carried by MIB-like: SFN information, AIoT SIB scheduling information. The information can be carried by SIB1-like: DOA resource configuration, L1 control related information configuration(s), R2D preamble configuration, The frequency information of AIOT carrier, DRX configuration for device power saving, Criteria for cell-(re)selection, Reader ID related information, Power control related information, Periodicity of sync signal (and the payload i.e., MIB). Note: other information is subject to RAN2 study.

- [LGE] Regarding the purpose and necessary information of broadcast information, it is observed that following information is needed: Cell/reader ID‑related information to enable reader identification. Resource‑allocation information for DO‑A transmissions. Timing information indicating the allocation timing of the D2R resources. D2R power‑control parameters for device transmit‑power control.

- [NEC] At least the following information can be carried in the A-IoT broadcast information. Scheduling Info for the first DO-A D2R transmission; Reader ID-related information; FFS: other information to be carried in A-IoT broadcast information.

- [China Telecom] In Rel-20, study the necessity and feasibility of the following scheduling information for device 2b/C: T/F resource configuration for SIB-like message and/or MSG1, SFN value, repetition index or period index of synchronization signal, time unit index (e.g., half-frame index), frequency offset between the synchronization signal and the SIB-like message or MSG1.

- [Samsung] Broadcast information is transmitted for the following purposes: Purpose #1: Triggering the inventory procedure (i.e., paging-like message for device discovery/presence checking), Purpose #2: Providing DO-A resources so that a device can initiate RA by itself, and Purpose #3: Providing reference information for open-loop power control. Broadcast information can carry at least one of the following information sets, depending on the purpose: Information for the inventory procedure trigger, DO-A resource information (e.g., periodicity, access occasion configuration, resource pool information, etc.) and Reference information related to R2D Tx power for power control.

- [ETRI] The broadcast information should include at least the following information elements: Cell/reader ID-related information, power control-related information, periodic D2R resource configuration for DO-A, and timing information (e.g., SFN-like information).

- [DOCOMO] For broadcast information, following information should be considered. Information for reader differentiation/identification such as reader ID. Parameters for first D2R transmission for DO-A such as D2R resource, physical layer design related parameters such as chip duration, FEC, repetition, etc., and power control information. Parameters for subsequent R2D reception such as potential R2D reception resources of R2D sync signal, L1 R2D control, A-IoT paging, R2D command and corresponding physical layer design related parameters such as M value, FEC, repetition, etc.

- [Qualcomm] Periodic system information (SIB) can provide necessary information including at least: Reader ID-related information (e.g., higher layer reader ID), R2D-related configuration (frequency location(s) for R2D carriers, configuration of periodic R2D monitoring occasions for SS and paging, L1 control information per coverage level), D2R-related configuration (frequency location(s) for D2R carriers, configuration of periodic DO-A D2R resources, power control per coverage level).

- [Lenovo] states a two-step method for acquiring broadcast information. In the first step, the broadcast provides a list of available frequencies for device access. In the second step, further time–frequency resources are supplied for device transmission/reception, along with possible reader ID information.

- [Huawei] For the broadcast information, the MIB-like transmission contains the reader identifier, SFN and the time and frequency resource allocation information for the SIB-like transmission. The SIB-like broadcast transmission contains the time and frequency resource allocation of DO-A, power control parameters, ARFCN-like frequency location information, RSRP threshold/level for the R2D measurement and other necessary parameters for DO-A and the active device.

- [H] Proposal 3.1.2b-1

On Case B1 and Case B2 for how A-IoT devices acquire the broadcast information, capture the following in TR 38.769:

On how A-IoT devices acquire the broadcast information, sources (Spreadtrum, Ericsson, CATTOPPO, Xiaomi, ZTE, Panasonic, Samsung, Qualcomm, Quectel, Fraunhofer) state that Case 1 (Single-step acquisition of the broadcast information) should be supported. Following observations on the Case 1 were provided during the study.

- [FUTUREWEI] Potential for high system overhead with frequent broadcast information transmission. Possible increase of device unavailability as a device may need to perform blind decoding of the L1 R2D control information before obtaining broadcast information.

- [Spreadtrum] For broadcast information, single-step acquisition of the broadcast information is supported. PRDCH is used to carry the broadcast information.

- [Ericsson] It is simpler and more efficient to transmit all the necessary information in a single step using a single PRDCH. Case B1 may also be better from reader and A-IoT device energy consumption perspectives.

- [CATTOPPO] Due to the limited battery capacity of A-IoT devices, it is desirable to provide all necessary system information to A-IoT devices at one time rather than introducing extra dedicated message for system information delivering (e.g., SIB1 like message), unless the payload size of the broadcast information identified at the end may result in an unacceptable A-IoT SSB length.

- [Xiaomi] If the payload size of broadcast information is small and fixed, there is no need to split it into two parts.

- [ZTE] For Case B1-1, the broadcast information includes L1 control, broadcast information scheduling is more flexible. For Case B1-2, the broadcast information does not include L1 control, the design is simpler and makes it easier for the device to detect the broadcast information.

- [vivo] If the periodicity of single broadcast information is same as that of periodic sync signal, larger resource overhead will be caused since some of the information is only used after initial reader search and don’t need to be transmission as frequent as periodic sync signal. If the periodicity of single broadcast information is larger than that of periodic sync signal, during initial search, a device needs to do more blind detection to find the potential broadcast information to obtain the essential information for completing the initial search e.g., SFN for timing alignment. This will increase the initial search complexity and latency. If the single broadcast information is SIB1-like information, the starting time location of SIB1-like broadcast information has to be fixed, which is less flexible for reader resource allocation.

- [LGE] The single‑step acquisition allows simple and fast acquisition at the device side but incurs signaling overhead and may introduce reader‑identification timing delay.

- [Panasonic] A single-step acquisition of broadcast signal using PRDCH should be considered.

- [Samsung] A single-step approach (i.e., Case B1) should be used for broadcast information, and introducing a two-step structure (i.e., Case B2) is not strongly motivated from an information-size perspective.

- [Sharp] For Case B1, the broadcast PRDCH should always be preceded by a synchronizing signal to facilitate detection and acquisition.

- [Qualcomm] For Case B1, devices may perform SIB monitoring with a predefined periodicity and detect the SIB directly. This approach is more straightforward and potentially more efficient.

- [Quectel] For the acquisition method of broadcast information, study single-step acquisition, i.e., Case B1 as baseline. Single-step acquisition of broadcast is simpler and costs less standardization efforts than two-step acquisition method. MIB-like in Ambient IoT is not necessary since ambient IoT devices do not require precise synchronization like SFN information and carrier offset. Another reason is the message size of broadcast information may be fixed and is not as large as SIB information.

- [Fraunhofer] For broadcast information acquisition prioritize Case B1. Single‑step acquisition minimizes the number of decoding tasks, and therefore, reducing the device activation time and avoiding the need to coordinate multiple acquisition stages. On the contrary, Case B2 introduces additional operational steps, requiring devices to decode two separate layers of system information, potentially from different physical channels. While this structure allows for modularity and independent periodicities for MIB‑like and SIB1‑like information, it also increases decoding complexity and extends the time during which devices must remain active. Therefore, Case B1 should be prioritized given the energy constrains of A-IoT devices.

Sources (FUTUREWEI, Huawei, CMCC, vivo, Lenovo) state that Case 2 (Two-step acquisition of the broadcast information) should be supported. Following observations were provided during the study.

- [FUTUREWEI] Allows the PRDCH containing broadcast information to be similar to other PRDCH by providing parameters to decode L1 R2D control information. Allows a device to know whether to process PRDCH containing broadcast information and whether the PRDCH containing broadcast information is scheduled. Allows a reader to coordinate device availability of all devices it serves.

- [Huawei] Advantages of two-step acquisition of the broadcast information (Case B2): The two-step split acquisition of the broadcast information is to facilitate an efficient method to provide the required information to the device at the required time - the system information carried by the MIB-like transmission is required by the device more frequently than the DO-A resource allocation information carried by the SIB-like broadcast information since it does not need to change frequently. The overhead of the resource allocation information for DO-A is larger than the other system information, and transmitting this in the SIB-like transmission with a longer periodic interval as compared to the MIB-like broadcast information can avoid a degradation of transmission efficiency. The impact of overhead is reduced when compared with Case B1 where a single-step acquisition of the broadcast information is used. The MIB-like transmission contains the reader identifier, SFN and the time and frequency resource allocation information for the SIB-like transmission. The SIB-like broadcast transmission contains the time and frequency resource allocation of DO-A, power control parameters, ARFCN-like frequency location information, RSRP threshold/level for the R2D measurement and other necessary parameters for DO-A and the active device.

- [ZTE] For Case B2, the design is more complex compared with Case B1, and incurs higher signaling overhead, which increases downlink blockage probability and consequently degrades inventory efficiency.

- [vivo] MIB-like is regarded as the payload of periodic sync signal to carry the minimum required information for device to complete the initial reader search, which has the same periodicity as that of periodic sync signal. Thus, it is less complex for a device to finish the initial reader search within the periodic sync signal cycle. With two-step acquisition of the broadcast information, the periodicity of SIB1-like can be more flexible e.g., larger than that of MIB-like to avoid unnecessary resource overhead. MIB-like can flexibly indicate the scheduling information of SIB1 e.g., the start time location of SIB1, which is benefit for reader resource allocation.

- [LGE] The two‑step acquisition reduces signaling overhead and allows early reader identification but may cause acquisition delay and may require additional specification work for a new physical channel.

- [Sharp] For Case B2, the MIB-like channel should be designed to be as simple as possible, in order to minimize device complexity.

- [Lenovo] Support two-step acquisition of the broadcast information (i.e., Case B2). In the first step, broadcast information transmitted on initial frequency provides a list of available frequencies for devices to access. In the second step, broadcast information transmitted on the available frequencies provides further resource occasions for devices to access.

- [Qualcomm] For Case B2, devices may perform MIB monitoring with a predefined periodicity. Devices first detect the MIB to obtain the SIB monitoring configuration and then acquire the SIB.

- [Quectel] For case B2, i.e., two step acquisition of the broadcast information results in more complexity comparing with single step acquisition.

Sources (NEC, Sharp, ETRI, DOCOMO) state that the decision can be made after the content/size of the broadcast information becomes clear. Following observations were provided during the study.

- [NEC] RAN1 can further decide whether single-step or two-step acquisition is required for broadcast information in Rel-20 A-IoT after all the information carried by broadcast information and corresponding size have been clearly identified.

- [Sharp] The down-selection between Case B1 and Case B2 may depend on how much broadcast information needs to be carried, as well as the resulting impact on device complexity, coverage, and power consumption.

- [ETRI] The introduction of an MIB/SIB-based structure for broadcast information should be decided after the contents of the broadcast information are agreed.

- [DOCOMO] Whether the broadcast information acquisition is single-step or two-step depends on the payload size of whole broadcast information, i.e., two-step is necessary if it would be expected to be large, otherwise, single-step is sufficient.

Views on whether to use a separate PRDCH or a new physical channel to carry the MIB-like broadcast information for Case B2

- PRDCH: Huawei, Spreadtrum, OPPO, Ericsson, CATT, CMCC, Xiaomi, vivo, LGE, China Telecom, Fraunhofer

- New PHY channel: CMCC

  - Definition of the “new” physical channel: For MIB-like part, the R2D parameters such as TBS, payload size, FEC or repetition number can be predefined.

- [H] Proposal 3.1.2c-1

On whether to use a separate PRDCH or a new physical channel to carry the MIB-like broadcast information for Case B2, capture the following in TR 38.769:

For Case 2, sources (Huawei, Spreadtrum, OPPO, Ericsson, CATT, CMCC, Xiaomi, vivo, LGE, China Telecom, Fraunhofer) state that PRDCH should be used to carry the MIB-like broadcast information, while sources (CMCC, FUTUREWEI) states that are also open to use a new physical channel dedicated to MIB-like information should be used. During the study, following observations were provided.

- [Huawei] Firstly, we do not see the PRDCH is used to carry the MIB-like broadcast information as there is no additional motivation to define a separate or new channel like NR SSB, in which 8 information bits are carried in the physical layer payload. Secondly, carrying the MIB-like broadcast information on an additional channel would increase the workload in terms of specification work for the WI.

- [SRPD] For broadcast information, single-step acquisition of the broadcast information is supported and PRDCH is used to carry the broadcast information.

- [China Telecom] If case B2 is selected for the broadcast information transmission scheme, support to reuse a separate PRDCH for the MIB-like broadcast information. While using PRDCH may require a new R2D message format for MIB-like information, defining an entirely new broadcast channel would be significantly a larger workload. Given the limited TU before the SID is completed, reusing the PRDCH to carry MIB-like information would be more feasible and simpler to achieve.

- [vivo] Use PRDCH as the transmission channel for both MIB-like and SIB1-like broadcast information. Do not introduce a new channel to carry MIB-like broadcast information.

while sSources (CMCC, FUTUREWEI) states that are also open to use a new physical channel dedicated to MIB-like information should be used. During the study, following observations were provided.

- [CMCC] MIB-like broadcast information can be carried by a separate PRDCH or a new physical channel dedicated to MIB-like information. For MIB-like part, the R2D parameters such as TBS, payload size, FEC or repetition number can be predefined.

- [FUTUREWEI] Whether the MIB-like information is carried in its own channel can follow the concept of the PBCH, which is a separate channel for the MIB. This separate channel can simplify the specifications.

Not limited to Case 2, following sources state that a new physical channel other than PRDCH should not be used to carry broadcast information.

- [OPPO] It is not necessary to introduce a new physical channel other than PRDCH to convey the broadcast information. There is no need to introduce a new physical channel other than PRDCH because the PRDCH was already used to transmit A-IoT paging message in Rel-19 A-IoT and the A-IoT paging message is also a kind of broadcast information especially when Paging ID Presence Indication field indicates Paging ID field is absent.

- [Ericsson] Using PRDCH to carry broadcast information could reduce the design complexity and specification impact. The broadcast information should be transmitted periodically in PRDCH with fixed parameters and the periodicity could be different from the periodic sync signal.

- [CATT] If the transmission parameters for L1 control information are indicated by broadcast information, a new physical channel other than PRDCH must be defined to carry the broadcast information including the configurations for L1 control information. Otherwise, if the broadcast information carried by PRDCH schedules a L1 control information, there will be a chicken-and-egg problem in the scheduling relationship between L1 control information and PRDCH. The introduction of a new channel will increase the complexity of R2D reception and relative standardization work.

- [Xiaomi] PRDCH in case B1 is capable to contain the single part of broadcast information.

- [LGE] If the introduction of a new physical channel for the partial broadcast information is considered, the specification work may significantly increase.

- [NEC] PRDCH is used to carry the broadcast information, no need to introduce new channel(s) for R2D. For the channel to carry this broadcast information, we prefer to reuse PRDCH since the coding/repetition schemes for broadcast information may not be quite different, and no specific RS will be designed for the broadcast information. Furthermore, keep only one channel for each of R2D or D2R would maintain the simplest design for Rel-20 A-IoT.

- [Fraunhofer] PRDCH is used to carry the broadcast information. From a device perspective, supporting multiple channels undermines the simplicity benefits targeted for A-IoT. Furthermore, the separation of MIB-like and SIB1-like information does not provide a clear functional advantage for the devices, given the limited system information needs.

- [CMCC] MIB-like broadcast information can be carried by a separate PRDCH or a new physical channel dedicated to MIB-like information. For MIB-like part, the R2D parameters such as TBS, payload size, FEC or repetition number can be predefined.

Proposals for Wednesday morning online session

[A-IoT positioning]

Work plan for RAN1#124 meeting (R1-2509614)

- Positioning - Which D2R signal(s)/channel(s) applicable to positioning - Measurement aspects (e.g., metric) for positioning

According to Vice Chair’s guidance, the aspects related to A-IoT positioning will also be handled in this sub-agenda.

The related SID objective on A-IoT positioning is shown below.

Objectives … - Identify D2R RSRP-like measurement, and the involved A-IoT signal(s)/channel(s), which are feasible for Device 2b/Device C for more accurate outdoor Device localization than based on Reader-ID [RAN1]. - RAN1 is not expected to conclude on the positioning accuracy as part of the identification of feasible D2R RSRP-like measurement - Note: Device localization based on Reader ID as specified in Rel-19 can be supported for active device(s) in Rel-20 with no additional work. - The design of D2R signal(s)/channel(s) for active devices is not impacted by the identification of the D2R RSRP-like measurement - This objective does not include the study of dedicated Device procedures for Device localization - The objective assumes no dedicated positioning architecture will be specified in Rel-20 - The objective assumes no specification of inter-Reader coordination for Device localization in Rel-20 - This objective will start at RAN1#124

Which D2R signal(s)/channel(s) applicable to positioning

Work plan for RAN1#124 meeting (R1-2509614)

- Positioning - Which D2R signal(s)/channel(s) applicable to positioning - Measurement aspects (e.g., metric) for positioning

Based on the SID, the discussion in this sub-clause focuses on “which” D2R signal/channel is applicable to positioning.

Summary

Views on which D2R signal(s)/channel(s) is/are applicable to A-IoT positioning

- D2R preamble: FUTUREWEI, Ericsson, Apple,

- D2R preamble and midamble(s): Huawei, Ericsson, vivo, DCM,

- D2R preamble and/or midamble(s): OPPO, Xiaomi, ZTE,

- D2R signals (sequence embedded in the PDRCH) and D2R preamble: CATT

- D2R preamble and/or midamble(s), and PDRCH: Qualcomm,

- [H] Proposal 3.6.1a-1

On the D2R signal(s)/channel(s) applicable to A-IoT device localization, capture the following observations in TR 38.769.

D2R signal(s)/channel(s) which is/are feasible and applicable for A-IoT device localization is/are studied. AsFor the D2R signal(s)/channel(s) which is/are feasible and applicable for A-IoT device localization, sources (FUTUREWEI, Ericsson, Apple, vivo) mention D2R preamble.

- [FUTUREWEI] For D2R RSRP-like measurement, the reader can use the D2R preamble for the reference signal and measure the power between the end of R2D transmission and start of the D2R transmission to estimate the noise and interference.

- [Apple] D2R preamble is the applicable signal for RSRP-like measurement. D2R RSRP-like measurement can be performed on existing D2R preamble without requiring dedicated positioning signals, enabling proximity determination for outdoor device localization.

- [vivo] D2R preamble can be used for the positioning purpose. And no need to define D2R RSRP-like metric.

Sources (Huawei, Ericsson, vivo, DCM) mention D2R preamble and midamble(s) for the D2R signal(s)/channel(s) which is/are feasible and applicable for A-IoT device localization.

- [Huawei] D2R preamble and midamble signals are used for D2R RSRP-like measurement for device localization.

- [Ericsson] D2R-amble-RSRP is defined as the linear average over the power contributions (in [W]) of chips that carry the D2R preamble over PDRCH. A secondary measurement can be defined as the linear averaging over the power contributions of chips that carry D2R preamble and D2R midambles over PDRCH.

- [DCM] For positioning, at least D2R RSRP measurement on D2R preamble and D2R midamble when m sequence is used for midamble are feasible. In the previous meetings, it was agreed to consider m sequence and Gold sequence for D2R preamble, and it was agreed to consider m sequence, all 1 sequence and Hadamard sequence for D2R midamble. From our perspective, m sequence and Gold sequence can be used for RSRP measurement at reader thanks to good auto-correlation property, and RSRP measurement based on m sequence and Gold sequence has been widely used in NR and should be feasible.

Sources (OPPO, Xiaomi, ZTE) indicate D2R preamble and/or midamble(s).

- [OPPO] D2R preamble and/or mid-amble, which is applicable for all of D2R transmissions even for sequence-based Msg 1 without associated PDRCH, can be considered for D2R RSRP measurement.

- [Xiaomi] The current D2R signal structure including a preamble, midamble(s), and PDRCH, where preamble and midamble(s) are sequence based on the current agreements. Therefore, for RSRP-like measurement, it is feasible for Reader to measure the D2R preamble or midamble(s).

- [ZTE] Based on introduction of open loop or closed loop power control for D2R, reader is feasible to use D2R preamble or midamble to obtain more accurate localization information.

Source (CATT) states D2R signals (sequence embedded in the PDRCH) and D2R preamble, and source (Qualcomm) states D2R preamble and/or midamble(s), and PDRCH. Following observations were provided during the study.

- [CATT] A sequence with length 127, which is designed for the A-IoT device positioning, was embedded in the PDRCH for the D2R RSRP-like measurements. For the study of sequence length and type, we conducted the following length-31, length-63 and length-127 of M-sequence and Gold sequence with evaluation results. From the evaluation results, it can be seen length-127 M-sequence or Gold-sequence show a lower residual SFO error comparing to that of length-31 M-sequences or Gold-sequence and length-63 M-sequences or Gold-sequence. Since SFO is one of the main factors to measurement accuracy of RSRP, gold sequences or m-sequences with a length of 127 can be used as candidates.

- [Qualcomm] For D2R RSRP-measurement based positioning and proximity determination, PDRCH and D2R preamble/midamble with sufficient time duration and stable transmit power can be considered for D2R measurement. Using multiple measurements or extending D2R transmission increases accuracy. For D2R measurement, unicast D2R transmission can be used: Alt1: D2R Preamble/Midamble, Alt2: PDRCH. For Alt.1, the preamble and/or midamble can be used for RSRP measurement, provided they have sufficient transmission time and combined appropriately. For Alt.2, the PDRCH carries D2R control or data payload and repetitions for reliability can be used to improve power measurement accuracy.

D2R RSRP-like measurement and its feasibility

Work plan for RAN1#124 meeting (R1-2509614)

- Positioning - Which D2R signal(s)/channel(s) applicable to positioning - Measurement aspects (e.g., metric) for positioning

Based on the SID, the D2R RSRP-like measurement which is feasible for Device 2b/Device C for more accurate outdoor Device localization than based on Reader-ID is discussed in this sub-clause.

Summary

Measurement aspects (e.g., metric) for positioning

- Definition: FUTUREWEI, Huawei, Ericsson, CATT, Xiaomi,

- Noise and interference estimation: FUTUREWEI

- Additional measurements (AOA measurement): CATT

- [H] Proposal 3.6.2a-1

On the measurement aspects (e.g., metric) for A-IoT device localization, capture the following observations in TR 38.769.

Regarding the D2R RSRP-like measurement for A-IoT device localization, following observations were provided during the study.

- [FUTUREWEI] Use NR measurements (SL PRS-RSRP and UL-SRS-RSRP) as a reference. For D2R RSRP-like measurement, the reader can use the D2R preamble for the reference signal and measure the power between the end of R2D transmission and start of the D2R transmission to estimate the noise and interference. To estimate proximity, a reader can also consider the effects of propagation delay based on when the D2R transmission is received.

- [Huawei] D2R RSRP-like measurement is defined as the time domain linear average over the power contributions (in [W]) of preamble and midamble signals within the signal transmission bandwidth during the D2R transmission.

- [Ericsson] D2R-amble-RSRP is defined as the linear average over the power contributions (in [W]) of chips that carry the D2R preamble over PDRCH. A secondary measurement can be defined as the linear averaging over the power contributions of chips that carry D2R preamble and D2R midambles over PDRCH.

- [CATT] The definition of RSRP-like and AOA measurement quantity can refer to TR 38.215(NR; Physical layer measurements).

- [Xiaomi] The RSRP-like definition in A-IoT is the linear average of the received power in occupied chips and over the frequency resources that carry the measured signal. For example, if D2R is applied with OOK modulation, referring to the definition of LP-RSRP where measured signal is also an OOK waveform, RSRP-like definition in A-IoT is the linear average of the received power in OOK ON chips over the frequency resources that carry the measured signal.

- [vivo] Measurement can be up to network implementation based on D2R preamble, no need to define D2R RSRP-like metric. And nothing needs to be specified also.

- [Qualcomm] At reader side, how to measure the RSRP may be up to reader implementation. However, it may require the averaging of the D2R measurements based on D2R signals/channels with sufficient time duration and stable transmit power from device side.

- [NEC] D2R RSRP based measurement and positioning is feasible. R20 AIoT should support D2R RSRP based measurement.

Other aspects

This sub-clause is to discuss other aspects related to A-IoT device localization, such as positioning methods, evaluations results, etc.

Summary

On the positioning methods, following observations were provided:

[FUTUREWEI] A reader can combine timing and power measurements for proximity determination. A reader can estimate the distance to the device based on the delay measurement and accounting for the SFO.

[Huawei] D2R RSRP measurement based on preamble and midamble, and fingerprint-based positioning scheme is adopted. The positioning algorithm is Weighted K-Nearest Neighbor (WKNN).

[Ericsson] Assuming Tx power of the device is known by the reader and LOS between A-IoT and the reader, one approach for having a more granular outdoor device localization than the reader-based is that the reader can compare the RSRP of D2R signal with different RSRP threshold values, and estimate the zone that the device is located in.

[CATT] Joint estimation of distance from RSRP-like measurements and AOA estimation with reader-based beam sweeping in the implementation is proposed.

[Xiaomi] Reader determines whether A-IoT device is far from or close to itself by comparing the measured RSRP value with an RSRP threshold. Reader determines a corresponding distance range when the measured RSRP value falls in an RSRP range.

[NEC] D2R RSRP is a feasible approach at the current stage. Base on D2R RSRP value, at least device could be grouped into near group or far group roughly.

[Apple] Proximity determination based on D2R RSRP measurement is sufficient for outdoor use-cases requiring several tens of meters accuracy, without needing dedicated positioning architecture. Further consider more finer granularity of proximity including, very near, near and far depending up on the measurement strength.

[Qualcomm] Multi-reader positioning based on RSRP measurement is evaluated using weighted-centroid algorithm. The position estimate is given by a weighted average of BS reader locations, wherein the weights are a function of the RSRP measurement at each of the BS/readers and the same D2R transmission using a 63-length m-sequence is measured per reader.

[Fraunhofer] Introduce an explicit mechanism to set a context for positioning-related signaling. Even when high-accuracy positioning is not required, establishing a coarse spatial relationship such as a coverage zone is beneficial. To enable this, it is important that measurements and signaling exchanges are interpreted within a clear operational context. In particular, devices should be able to distinguish whether a reception instance is intended for regular inventory or command operation, or for positioning-related measurement purposes. Such contextual awareness will prevent unnecessary activity or energy consumption at the device.

- Proposal 3.6.3a

On A-IoT device localization methods, capture the following observations in TR 38.769.

On A-IoT device localization for Device 2b/Device C for more accurate outdoor Device localization than based on Reader-ID, following observations on device localization methods were reported.

- [FUTUREWEI] A reader can combine timing and power measurements for proximity determination. A reader can estimate the distance to the device based on the delay measurement and accounting for the SFO.

- [Huawei] D2R RSRP measurement based on preamble and midamble, and fingerprint-based positioning scheme is adopted. The positioning algorithm is Weighted K-Nearest Neighbor (WKNN).

- [Ericsson] Assuming Tx power of the device is known by the reader and LOS between A-IoT and the reader, one approach for having a more granular outdoor device localization than the reader-based is that the reader can compare the RSRP of D2R signal with different RSRP threshold values, and estimate the zone that the device is located in.

- [CATT] Joint estimation of distance from RSRP-like measurements and AOA estimation with reader-based beam sweeping in the implementation is proposed.

- [Xiaomi] Reader determines whether A-IoT device is far from or close to itself by comparing the measured RSRP value with an RSRP threshold. Reader determines a corresponding distance range when the measured RSRP value falls in an RSRP range.

- [NEC] D2R RSRP is a feasible approach at the current stage. Base on D2R RSRP value, at least device could be grouped into near group or far group roughly.

- [Apple] Proximity determination based on D2R RSRP measurement is sufficient for outdoor use-cases requiring several tens of meters accuracy, without needing dedicated positioning architecture. Further consider more finer granularity of proximity including, very near, near and far depending up on the measurement strength.

- [Fraunhofer] Introduce an explicit mechanism to set a context for positioning-related signaling. Even when high-accuracy positioning is not required, establishing a coarse spatial relationship such as a coverage zone is beneficial. To enable this, it is important that measurements and signaling exchanges are interpreted within a clear operational context. In particular, devices should be able to distinguish whether a reception instance is intended for regular inventory or command operation, or for positioning-related measurement purposes. Such contextual awareness will prevent unnecessary activity or energy consumption at the device.

- [ZTE] In Rel-20 Ambient IoT, based on D2R preamble or midamble, it is possible for reader to use these ambles to do the RSRP-like measurement and obtain the localization information if the power control including open loop or closed loop power control is introduced.

- [Qualcomm] Multi-reader positioning based on RSRP measurement is evaluated using weighted-centroid algorithm. The position estimate is given by a weighted average of BS reader locations, wherein the weights are a function of the RSRP measurement at each of the BS/readers and the same D2R transmission using a 63-length m-sequence is measured per reader.

Evaluation results were provided by 3 companies:

[Huawei] D2R RSRP-like measurement is feasible for Device C to achieve more accurate outdoor Device localization than based on Reader-ID. D2R RSRP measurement based on preamble and midamble, and fingerprint-based positioning scheme is adopted. The positioning algorithm is Weighted K-Nearest Neighbor (WKNN). Achieved localization performance, i.e., 82m@90%.

[CATT] After SFO/CFO calibration, the positioning accuracy of RSRP+AOA method is about 17m (90% CDF) for Device 2b and is about 13m (90% CDF) for Device C.

[Qualcomm] Multi-reader positioning based on RSRP measurement is evaluated using weighted-centroid algorithm. The position estimate is given by a weighted average of BS reader locations, wherein the weights are a function of the RSRP measurement at each of the BS/readers and the same D2R transmission using a 63-length m-sequence is measured per reader. For outdoor positioning, Outdoor BS/reader can achieve position estimation about 72~114% of cell radius at 80~90% percentage error for Device 2b in UMi layout, and about 84~110% of cell radius estimation at 80~90% percentage error for Device C in UMa layout. For proximity determination to identify a unique device relative to a reader, in Outdoor scenario, UE/reader can reach 95~98% success rate with 2 to 6 nearby devices; BS reader can achieve 48~80%. In Indoor scenario, UE/reader can reach 77~92% success rate with 2 to 6 nearby devices; BS/reader can achieve 56~83%. BS/readers generally perform worse, primarily because fixed BS/reader cannot approach the devices as flexible as UE/reader.

- Proposal 3.6.3b

On A-IoT device localization methods, capture the following observations in TR 38.769.

On A-IoT device localization for Device 2b/Device C for more accurate outdoor Device localization than based on Reader-ID, sources (Huawei, CATT, Qualcomm) provided evaluation results.

- [Huawei] D2R RSRP-like measurement is feasible for Device C to achieve more accurate outdoor Device localization than based on Reader-ID. D2R RSRP measurement based on preamble and midamble, and fingerprint-based positioning scheme is adopted. The positioning algorithm is Weighted K-Nearest Neighbor (WKNN). Achieved localization performance, i.e., 82m@90%.

- [CATT] After SFO/CFO calibration, the positioning accuracy of RSRP+AOA method is about 17m (90% CDF) for Device 2b and is about 13m (90% CDF) for Device C.

- [Qualcomm] Multi-reader positioning based on RSRP measurement is evaluated using weighted-centroid algorithm. The position estimate is given by a weighted average of BS reader locations, wherein the weights are a function of the RSRP measurement at each of the BS/readers and the same D2R transmission using a 63-length m-sequence is measured per reader. For outdoor positioning, Outdoor BS/reader can achieve position estimation about 72~114% of cell radius at 80~90% percentage error for Device 2b in UMi layout, and about 84~110% of cell radius estimation at 80~90% percentage error for Device C in UMa layout. For proximity determination to identify a unique device relative to a reader, in Outdoor scenario, UE/reader can reach 95~98% success rate with 2 to 6 nearby devices; BS reader can achieve 48~80%. In Indoor scenario, UE/reader can reach 77~92% success rate with 2 to 6 nearby devices; BS/reader can achieve 56~83%. BS/readers generally perform worse, primarily because fixed BS/reader cannot approach the devices as flexible as UE/reader.

[Initial frequency/broadcast information acquisition]

Reader identification/differentiation

Previous agreements

Agreement (RAN#123)

Reader identification/differentiation is necessary for Device 2b and for Device C, at least in outdoor scenario to differentiate R2D among different readers.

Agreement (RAN#123)

For reader identification/differentiation at least in outdoor scenario to differentiate R2D among different readers, a device acquires the reader ID-related information by at least one of the following options:

- Option 1: Reader ID-related information is contained in the broadcast information.

- Option 2: Reader ID-related information is derived from the periodic sync signal sequence(s).

- Option 3: Reader ID-related information is carried by L1 R2D control information.

- Option 4: Reader ID-related information is contained in the higher-layer information in the paging-like message.

Work plan for RAN1#124 meeting (R1-2509614)

- Reader identification/differentiation - Views on Option 1/2/3/4 for how a device acquires the reader ID-related information

Options 1/2/3/4 for how a device acquires the reader ID-related information were already captured.

This sub-clause is to further capture views on Option 1/2/3/4 for how a device acquires the reader ID-related information in 6A.3.x (L1 procedures) of TR 38.769.

Summary

Preference for Option 1/2/3/4 on how a device acquires the reader ID-related information:

- Option 1

  - Positive: Huawei, Spreadtrum, CMCC, ZTE, NEC, China Telecom, Sharp, ETRI, DCM, Qualcomm, Fraunhofer

  - Negative: CATT (infeasible)

- Option 2

  - Positive: CATT, CMCC, LGE, Panasonic, Fraunhofer

  - Negative: Huawei, Spreadtrum, NEC, China Telecom, ETRI

- Option 3:

  - Positive: CATT, Panasonic, Samsung, DCM, Qualcomm

  - Negative: vivo, NEC, ETRI

- Option 4:

  - Positive: CATT

  - Negative: vivo (infeasible), NEC, ETRI

- Combination of Option 1 and 2: CMCC, vivo, LGE, China Telecom

- Combination of Option 1 and 3: FUTUREWEI, Panasonic

- Combination of Option 1 and 2/3/4: Sharp

- Combination of Option 1 and 4: LGE

- [H] Proposal 3.1.1a-1

On the options for how a device acquires the reader ID-related information, capture the following in TR 38.769:

For Option 1, Sources (Huawei, Spreadtrum, CMCC, ZTE, NEC, China Telecom, Sharp, ETRI, DOCOMO, Qualcomm, Fraunhofer) state that Option 1 should be supported.

- [Huawei] The device can receive reader ID-related information in the initial part of the access procedure, enabling it to be able to use the reader ID for both reader differentiation and for the initial D2R transmission for DO-A. In the case of 2-step broadcast information acquisition, the reader ID can be included in the first step itself to ensure that the device receives the reader ID-related information in time to perform reader differentiation and DO-A transmissions, while not impacting the overhead significantly.

- [Spreadtrum] [Considering reducing the complexity of sequence detection for device, only one sequence of R2D periodic synchronization signal is supported. It’s not suitable to carry reader-ID using sync signal, so option 1 should be supported that Reader ID-related information is contained in the broadcast information.]

- [CMCC] This relaxes synchronization signal detection complexity, but since the device cannot identify the cell before decoding the channel for broadcast, cell-specific parameters cannot be applied to physical channel carrying broadcast information, so inter-Reader interference randomization cannot be used.

- [ZTE] Considering outdoor networking requirements, additional information such as the Cell or reader ID and Transaction ID also needs to be included in the broadcast information to help the device identify the serving cell or reader.

- [NEC] Due to the poor anti-interference capability of OOK and Manchester coding, it is difficult to enable different readers to be deployed on the same frequency point. Then, if in a certain area, there can only be one reader at a specific frequency point, i.e., there will be no adjacent readers deployed on the same frequency point, reader ID is unnecessary to be indicated along with each of the R2D transmission, it is enough to only indicate it during the initial frequency acquisition procedure, i.e., indicated by broadcast information.

- [China Telecom] Placing the reader ID outside the broadcast information would affect measurement and camp operations, which means measurement is only performed after synchronization procedure and waiting for the related signaling. Therefore, the reader ID should be included in the broadcast information.

- [Sharp] Option 1 (broadcast-based reader ID) should be adopted as a baseline mechanism for reader identification, ensuring that all devices can obtain basic reader identification information with minimal complexity. Options 2, 3, and 4 may be additionally supported and used by devices to further identify and differentiate the corresponding reader, especially in outdoor scenarios where multiple readers coexist and enhanced differentiation is required.

- [ETRI] [Sharp] Option 1 (broadcast-based reader ID) should be adopted as a baseline mechanism for reader identification, ensuring that all devices can obtain basic reader identification information with minimal complexity. Options 2, 3, and 4 may be additionally supported and used by devices to further identify and differentiate the corresponding reader, especially in outdoor scenarios where multiple readers coexist and enhanced differentiation is required.

- [DOCOMO] For initial broadcast information acquisition, reader ID-related information is contained in the broadcast information or carried by L1 R2D control information, i.e., option 1 or option 3 in the agreement should be considered.

- [Qualcomm] Periodic system information (SIB) can provide necessary information including at least: Reader ID-related information, e.g., higher layer reader ID, R2D-related configuration, D2R-related configuration, etc.

- [Fraunhofer] Mechanisms that provide reader‑ID‑related information during the discovery phase are more suitable than those relying on later‑stage control or higher‑layer signaling. Options 1 and 2, which convey or derive reader identification from broadcast information or periodic synchronization signals, respectively, align naturally with this requirement. Embedding or deriving reader‑ID‑related information from these early signals therefore minimizes delay, reduces energy consumption, and supports rapid reader selection in multi‑reader and mobility scenarios.

On the other hand, source (CATT) states that Option 1 should not be supported.

- [CATT] Reader identification information is best stored in non-volatile memory to ensure A-IoT device can use the information in all the subsequent transmissions after the first reception of reader identification information. The only option is NOT feasible is including in the broadcast information in Option 1, which requires to store the reader identification.

Following observations were also provided for Option 1.

- [FUTUREWEI] A device obtains reader ID-related information in a higher layer message without physical layer impact at the rate of broadcast information transmission.

- [Xiaomi] According to the discussion on broadcast acquisition (i.e., single-step or two-step), MIB-like information and/or SIB-like information can be the potential broadcast information that contains reader ID. The complexity of option 1 is low since device can obtain the reader ID directly by decoding broadcast information without any blind detection on the sequence. If the reader ID is contained only by MIB-like information, a device can simply get it by one-step decoding. However, when the total number of reader IDs is large by referring to PCID (1008), the payload size of MIB-like information will be large. Considering about the limited payload size for MIB-like information, the reader ID can be divided into two parts. One part of reader ID-related information is contained by MIB-like information, and the other part is contained by SIB-like information (if defined). By referring to the legacy design, MIB along with PSS/SSS and SIB are separately transmitted. The transmission gap between MIB-like information and SIB-like information can be large, and their transmission periodicities may also be different. A device can obtain the whole reader ID until it receives a MIB and a SIB-like information, which leads to higher latency and processing time compared to single-step.

- [LGE] Reader identification via broadcast information requires no additional signal design but forces the device to decode broadcast information even for undesired readers.

- [Samsung] Options 1, 2, and 4 are not necessarily transmitted with every R2D transmission. Therefore, they cannot provide reader identification/differentiation for every R2D transmission. To support reader differentiation, for every R2D transmission, it should be considered to either assume that Option 3 is always included in all R2D transmissions, apply an association rule when reader information is obtained via Options 1/2/4, or leverage the always-present R2D signal (e.g., SIP or CAP) to carry a reader-specific signature for differentiation.

- [Ericsson R1-2600210] If the Reader ID were not too long, “Option 2: Reader ID-related information is derived from the periodic sync signal sequence(s)” can be used aiming at performing an earlier reader identification. Otherwise, “Option 1: Reader ID-related information is contained in the broadcast information” can be considered.

- [H] Proposal 3.1.1b-1

On the options for how a device acquires the reader ID-related information, capture the following in TR 38.769:

For Option 2, Sources (CATT, CMCC, LGE, Panasonic, Fraunhofer) state that Option 2 should be supported.

- [CATT] To facilitate the device in identifying readers and selecting the one with the strongest signal for access, the reader identification information, which indicates the identifier of the reader, should be carried by R2D synchronization signal.

- [CMCC] This is similar as NR. But detecting different sequences during initial cell search increases device energy consumption. So, the number of IDs should be limited for this option.

- [LGE] Reader identification via a periodic sync signal sequence helps avoiding unnecessary broadcast decoding but increases detection complexity as the number of reader IDs grows.

- [Fraunhofer] Mechanisms that provide reader‑ID‑related information during the discovery phase are more suitable than those relying on later‑stage control or higher‑layer signaling. Options 1 and 2, which convey or derive reader identification from broadcast information or periodic synchronization signals, respectively, align naturally with this requirement. Embedding or deriving reader‑ID‑related information from these early signals therefore minimizes delay, reduces energy consumption, and supports rapid reader selection in multi‑reader and mobility scenarios.

On the other hand, sources (Huawei, Spreadtrum, NEC, China Telecom, ETRI) state that Option 2 should not be supported.

- [Huawei] Option 2 would increase the device complexity and power consumption since the device would have to blindly detect multiple sequences when performing the initial frequency acquisition procedure to determine the frequency location and reader ID-related information simultaneously.

- [NEC] For reader identification/differentiation, Option 2 is precluded because the number of sequences of R2D sync signal should be limited.

- [China Telecom] If only the reader ID is carried in the sync signal without accompanying broadcast information containing the SFN or time‑unit index, the device would be unable to report measurements accurately, and the reader could not perform proper camp control. This would make measurements become meaningless. Consequently, it is preferable to include reader ID within the broadcast information.

Following observations were also provided for Option 2.

- [FUTUREWEI] A device may have to perform detection of the periodic sync signal sequence to derive the reader ID-related information. It is unclear how a device can determine the better-quality sync signal sequence when sequences from multiple readers are received. This reader ID-related information is provided at the rate of periodic sync signal transmission.

- [Xiaomi] This is similar to NR PCID acquisition method based on PSS and SSS detection. Option 2 helps a device to get reader ID at the very beginning step before following steps such as system-like information acquisition and PRDCH reception. As it is agreed, R2D periodic synchronization signal is a binary sequence in time domain. If the number of binary sequence(s) is larger than 1 and no larger than 4, the sequence(s) can be used for potential reader identification/differentiation. However, 4 binary sequence(s) are too limited to identify different readers by considering that outdoor scenario may exist even larger number of cells/readers. In order to support large number of reader ID, two-level indication can be considered by combining Option 2 and Option 1. A device needs to derive the first-level of reader ID by detecting the sequence of periodic synchronization signal and obtain the second-level of reader ID from MIB-like or SIB-like information. Compared to Option 2 only, this combination method requires the device to process two steps to get the reader ID.

- [Samsung] Options 1, 2, and 4 are not necessarily transmitted with every R2D transmission. Therefore, they cannot provide reader identification/differentiation for every R2D transmission. To support reader differentiation, for every R2D transmission, it should be considered to either assume that Option 3 is always included in all R2D transmissions, apply an association rule when reader information is obtained via Options 1/2/4, or leverage the always-present R2D signal (e.g., SIP or CAP) to carry a reader-specific signature for differentiation.

- [Sharp] Option 1 (broadcast-based reader ID) should be adopted as a baseline mechanism for reader identification, ensuring that all devices can obtain basic reader identification information with minimal complexity. Options 2, 3, and 4 may be additionally supported and used by devices to further identify and differentiate the corresponding reader, especially in outdoor scenarios where multiple readers coexist and enhanced differentiation is required.

- [DOCOMO] For initial broadcast information acquisition, reader identification/differentiation based on periodic sync signal which device needs to blindly search on multiple candidate frequencies increases device complexity considering correlation-based detection. After broadcast information acquisition, reader ID-related information is derived from periodic sync signal or carried by L1 R2D control information.

- [Ericsson R1-2600210] If the Reader ID were not too long, “Option 2: Reader ID-related information is derived from the periodic sync signal sequence(s)” can be used aiming at performing an earlier reader identification. Otherwise, “Option 1: Reader ID-related information is contained in the broadcast information” can be considered.

- [H] Proposal 3.1.1c-1

On the options for how a device acquires the reader ID-related information, capture the following in TR 38.769:

For Option 3, Sources (CATT, Panasonic, Samsung, DOCOMO, Qualcomm) state that Option 3 should be supported.

- [Samsung] To support reader differentiation, the study on reader identification should proceed by selecting one of the following directions. Direction 1: Assume that Option 3 (L1 R2D control information) is always included in every R2D transmission. Direction 2: Acquire reader-related information via at least one of Options 1/2/4, and handle R2D transmissions without reader ID-related information based on an association rule (i.e., assume they originate from the same reader based on prior information).

- [DOCOMO] For initial broadcast information acquisition, reader ID-related information is contained in the broadcast information or carried by L1 R2D control information, i.e., option 1 or option 3 in the agreement should be considered. After broadcast information acquisition, reader ID-related information is derived from periodic sync signal or carried by L1 R2D control information.

- [Qualcomm] For flexible and reliable broadcast transmission, SIB-L1 control can be used to indicate the physical reader ID-related information, types of broadcast information, TBS, coding/repetitions for different coverage and device types, etc.

- [CATT] Since the reader identification information is only used in the scenario of the A-IoT receiving the PRDCH information from more than one reader, the retrieval of the reader identification is not critical whether is included in the periodic synchronization signal of Option 2, the L1-control information of Option 3, or the paging-like message of Option 4.

On the other hand, sources (vivo, NEC, ETRI, Huawei) state that Option 3 should not be supported.

- [vivo] At least for initial reader search, a device anyway needs to acquire all the information to finish the reader search and camp on it, so no big different to carry reader ID information via L1 control information or payload part. It will also increase the payload of L1 control information and result in the loss of L1 control detection performance.

- [NEC] For reader identification/differentiation, there is no need to further indicating the Reader ID-related information after initial frequency acquisition procedure because there cannot be multiple adjacent readers in a same frequency point. Therefore, Option 3 is precluded.

- [Huawei] The device would receive the reader ID only during an R2D transmission, which would mean that the device would not be able to use the reader ID for reader differentiation, nor for the D2R transmissions for DO-A. It would also increase the overhead of the control information as it is not necessary to be transmitted along with every R2D transmission.

Following observations were also provided for Option 3.

- [FUTUREWEI] A device obtains reader ID-related information in the L1 R2D control information, either as a specific field or as a mask applied to the CRC. The mask in combination of option 1, option 2, or option 4 can be used to accept / reject the PRDCH.

- [Xiaomi] Potential R2D signals with L1 R2D control information are SIB-like information, paging-like information, Msg2, R2D command (if these signals are supported). It is simple for a device to obtain the reader ID from L1 R2D control. When a device gets the reader ID, the device can determine whether to decode the corresponding data in PRDCH that is scheduled by this L1 R2D control. To support Option 3, a pre-condition is that the periodic synchronization signal and/or MIB-like information should also contain the reader ID. For example, a device should be synchronized to a reader and acquired its MIB-like information. If the reader ID in L1 R2D control of the following up R2D transmission(s) is not matched/same with the reader ID of the synchronized reader, the device can determine not to decode the corresponding PRDCH data part scheduled by this L1 R2D control.

- [LGE] Conveying the reader ID in L1 R2D control information enables explicit identification but introduces signaling overhead when applied to every PRDCH transmission. For Option 3, including the cell/reader ID in the L1 R2D control information is beneficial only when the broadcast information is transmitted on PRDCH and the L1 R2D control information is transmitted prior to the PRDCH. However, transmitting the cell/reader ID in the L1 R2D control information for every PRDCH used for inventory/command procedures is not desirable due to signaling overhead.

- [Sharp] Option 1 (broadcast-based reader ID) should be adopted as a baseline mechanism for reader identification, ensuring that all devices can obtain basic reader identification information with minimal complexity. Options 2, 3, and 4 may be additionally supported and used by devices to further identify and differentiate the corresponding reader, especially in outdoor scenarios where multiple readers coexist and enhanced differentiation is required.

- [Fraunhofer] Options 3 and 4 inherently assume that the device has already achieved synchronization and is actively monitoring reader‑specific control or paging occasions. As a result, reader identification becomes available only after the device has already committed resources to a particular reader context, which increases both latency and energy expenditure.

- [H] Proposal 3.1.1d-1

On the options for how a device acquires the reader ID-related information, capture the following in TR 38.769:

For Option 4, Source (CATT) states that Option 4 should be supported.

- [CATT] Since the reader identification information is only used in the scenario of the A-IoT receiving the PRDCH information from more than one reader, the retrieval of the reader identification is not critical whether is included in the periodic synchronization signal of Option 2, the L1-control information of Option 3, or the paging-like message of Option 4.

On the other hand, sources (vivo, NEC, ETRI, Huawei) state that Option 4 should not be supported.

- [vivo] It is infeasible since it cannot provide reader ID-related information during initial reader search. At least Option4 cannot operate as a standalone solution.

- [NEC] For reader identification/differentiation, there is no need to further indicating the Reader ID-related information after initial frequency acquisition procedure because there cannot be multiple adjacent readers in a same frequency point. Therefore, Option 4 is precluded.

- [Huawei] The device would not be able to use the reader ID for reader differentiation, nor for the D2R transmissions for DO-A. It will result in devices that want to transmit D2R DO-A traffic missing out on receiving the reader-ID related information and not being able to transmit the first D2R transmission for DO-A.

Following observations were also provided for Option 4.

- [FUTUREWEI] A device obtains reader ID-related information in a higher layer message without physical layer impact at the rate of paging-like messages.

- [Xiaomi] If paging-like message is supported, all of paging related information should be contained by higher-layer (e.g., MAC CE) and mapping to the data part of PRDCH in PHY-layer. Not like Option 3 that device can get the reader ID in L1 R2D control before decoding data part, Option 4 requires the device to decode both L1 R2D control and data part, and then the device can get the reader ID. Furthermore, Option 4 also prerequisites that the reader ID should be contained in periodic synchronization signal and/or MIB-like information.

- [LGE] Reader identification via higher‑layer information simplifies physical-layer operations but prevents early identification prior to acquiring higher‑layer signaling.

- [Samsung] Options 1, 2, and 4 are not necessarily transmitted with every R2D transmission. Therefore, they cannot provide reader identification/differentiation for every R2D transmission. To support reader differentiation, for every R2D transmission, it should be considered to either assume that Option 3 is always included in all R2D transmissions, apply an association rule when reader information is obtained via Options 1/2/4, or leverage the always-present R2D signal (e.g., SIP or CAP) to carry a reader-specific signature for differentiation.

- [Sharp] Option 1 (broadcast-based reader ID) should be adopted as a baseline mechanism for reader identification, ensuring that all devices can obtain basic reader identification information with minimal complexity. Options 2, 3, and 4 may be additionally supported and used by devices to further identify and differentiate the corresponding reader, especially in outdoor scenarios where multiple readers coexist and enhanced differentiation is required.

- [Fraunhofer] Options 3 and 4 inherently assume that the device has already achieved synchronization and is actively monitoring reader‑specific control or paging occasions. As a result, reader identification becomes available only after the device has already committed resources to a particular reader context, which increases both latency and energy expenditure.

- [H] Proposal 3.1.1e-1

On the options for how a device acquires the reader ID-related information, capture the following in TR 38.769:

Sources (CMCC, vivo, LGE, China Telecom) provided observations that combination of Option 1 and 2 should be supported.

- [CMCC] This option offers a trade-off between synchronization signal detection complexity and subsequent broadcast decoding performance. Finial determination may depend on the number of Reader IDs and sequence design of synchronization signal. If only a few IDs can be carried by sync signal sequence(s), the remaining can be contained in broadcast information.

- [vivo] For Option1+Option2, i.e., partial reader identification information is derived from the periodic sync signal sequence (3~4 candidate sequences) and the remaining information is carried in SIB1 broadcast information, the following benefits are observed. Compared to adopting option 1 only, after the initial search, a device can differentiate readers only via synchronization signal without decoding the following broadcast information, thereby reducing power consumption. And different periodic sync signal sequences can also reduce interference from different readers. Compared to adopting option 2 only, due to the limited number (e.g., 3~4) of candidate sequences of periodic synchronization signal, device blind detection complexity and design complexity can be reduced, and also the detection performance loss caused by large number of candidate sync sequences can be avoided. Based on these observations, it is proposed to support Option1+Option2 i.e., partial reader identification information is derived from the periodic sync signal sequence (3~4 candidate sequences) and the remaining information is carried by SIB1 broadcast information.

- [LGE] When determining the sequence length of the periodic sync signal, the number of cell/reader IDs that the device is required to distinguish needs to be considered. However, if the number of cell/reader IDs increases, either the number of periodic sync signal sequences or the sequence length increases, which results in higher detection complexity at the device side. Therefore, it may be necessary to consider Option 1 or Option 4 together with Option 2. In this case, the periodic sync signal may be used to distinguish cell/reader ID groups, and the actual cell/reader ID within the group may then be identified through broadcast information and/or higher-layer information.

- [China Telecom] If the synchronization signal and broadcast information are transmitted together, the reader ID could potentially be derived from the periodic synchronization signal sequences.

Sources (FUTUREWEI, Panasonic) provided observations that combination of Option1 and 3 should be supported.

- [FUTUREWEI] If the goal is to identify PRDCH from intended readers, then option 3 plus option 1 provides a check on each message using information provided in a broadcast while having limited impact on processing complexity.

- [Panasonic] To reduce the device’s monitoring time, the device needs to be synchronized with the serving reader(s). For this purpose, the periodic sync signal can carry reader ID-related information (e.g., Option 2). The sync signal does not need to carry the actual reader ID. For instance, 3 or 4 different sync signal sequences could be considered, while each sequence is used by one reader. The sequence can be assigned by NW to the reader. Once the device is synchronized with the serving reader, it will follow the reader for the PRDCH transmissions. Since the subcarrier might be used by other reader(s), the PRDCH should also carry the reader ID-related information. Although such information can be carried by the L1 control information or higher-layer information, using the L1 control information enables the device to only decode the control part to determine whether the message is from the serving reader. The L1 control information can carry the reader ID or a shorter ID dedicated to the reader assigned by the NW.

Source (Sharp) provided observations that combination of Option1 and 2/3/4 should be supported.

- [Sharp] Option 1 (broadcast-based reader ID) should be adopted as a baseline mechanism for reader identification, ensuring that all devices can obtain basic reader identification information with minimal complexity. Options 2, 3, and 4 may be additionally supported and used by devices to further identify and differentiate the corresponding reader, especially in outdoor scenarios where multiple readers coexist and enhanced differentiation is required.

Source (LGE) provided observations that combination of Option1 and 4 should be supported.

- [LGE] When determining the sequence length of the periodic sync signal, the number of cell/reader IDs that the device is required to distinguish needs to be considered. However, if the number of cell/reader IDs increases, either the number of periodic sync signal sequences or the sequence length increases, which results in higher detection complexity at the device side. Therefore, it may be necessary to consider Option 1 or Option 4 together with Option 2. In this case, the periodic sync signal may be used to distinguish cell/reader ID groups, and the actual cell/reader ID within the group may then be identified through broadcast information and/or higher-layer information.

Broadcast information

Previous agreements

Agreement (RAN#122bis)

For R2D, study at least the following aspects for the study of broadcast information:

- Information included in the broadcast information

- Whether to use PRDCH to carry the broadcast information

- Transmission of broadcast information:

  - Option 1: periodic

  - Option 2: aperiodic

- Time and frequency resources for transmission of the broadcast information, e.g. in relation to the synchronization signal.

Agreement (RAN#123)

- The functionality of broadcast information is to provide the device with at least necessary information for subsequent transmission and reception

- On how A-IoT devices acquire the broadcast information, study the following two cases:

  - Case B1: Single-step acquisition of the broadcast information

  - Case B2: Two-step acquisition of the broadcast information (e.g., MIB-like and then SIB1-like broadcast information)

- On whether to use PRDCH to carry the broadcast information,

  - For Case B1, PRDCH is used to carry the broadcast information.

  - For Case B2

    - PRDCH is used to carry the SIB1-like broadcast information.

    - A separate PRDCH or a new physical channel dedicated to MIB-like is used to carry the MIB-like broadcast information

Agreement (RAN#123)

Periodic transmission of broadcast information to provide at least necessary information for subsequent transmission and reception is necessary for Device 2b and for Device C.

- FFS (in SI or WI): whether the transmission of broadcast information is strictly periodic at reader

- FFS (in SI or WI): periodicity of the transmission of broadcast information

Work plan for RAN1#124 meeting (R1-2509614)

- Broadcast information - Purposes of the broadcast information and the necessary information per each purpose - Views on Case B1 and Case B2 for how A-IoT devices acquire the broadcast information - For Case B2, views on whether to use a separate PRDCH or a new physical channel to carry the MIB-like broadcast information - Time and freq. resource for the broadcast information, e.g. in relation to the synchronization signal To be discussed in AI 10.3.2.1

Summary

Information included in the broadcast information

- Reader ID related information: Ericsson, CMCC, Xiaomi, ZTE, vivo, LGE, NEC, ETRI, DOCOMO, Qualcomm, Spreadtrum

- D2R power control related information: Ericsson, Nokia, vivo, LGE, Samsung, ETRI, DOCOMO, Qualcomm

- Information related to resource allocation for the first D2R transmission for DO-A: OPPO, Ericsson, CMCC, Xiaomi, ZTE, vivo, LGE, NEC, China Telecom, Samsung, ETRI, DOCOMO, Qualcomm, Spreadtrum

- Parameters for subsequent R2D reception (e.g., for L1 R2D control information, paging monitoring/reception): OPPO, Ericsson, Nokia, CMCC, Xiaomi, ZTE, vivo, Samsung, DOCOMO, Qualcomm

- Timing information: CMCC, Xiaomi, ZTE, vivo, LGE, China Telecom, ETRI

- Reader selection/reselection related information: Ericsson, vivo

- SIB1-like scheduling information (for Case B2): vivo

- Other information deemed necessary by RAN2: Ericsson, vivo

Preference for Case B1 and Case B2 for how A-IoT devices acquire the broadcast information

- Case B1: Spreadtrum, Ericsson, CATT, Xiaomi, ZTE, Panasonic, Samsung, Qualcomm, Quectel, Fraunhofer

- Case B2: FUTUREWEI, Huawei, CMCC, vivo, Lenovo

- Other views: NEC, Sharp, ETRI, DOCOMO

Views on whether to use a separate PRDCH or a new physical channel to carry the MIB-like broadcast information for Case B2

- PRDCH: Huawei, Spreadtrum, OPPO, Ericsson, CATT, CMCC, Xiaomi, vivo, LGE, China Telecom, Fraunhofer

- New PHY channel: CMCC

  - Definition of the “new” physical channel: For MIB-like part, the R2D parameters such as TBS, payload size, FEC or repetition number can be predefined.

- [H] Proposal 3.1.2a-1-Alt.2

On the potential [necessary] information to be included in the broadcast information, capture the following observations in TR 38.769:

On the potential information included in the broadcast information, following observations were provided during the study.

- [OPPO] Broadcast information can contain time/frequency resource for L1 R2D control monitoring, time and frequency resources and related configurations for the initial DO-A transmission, etc.

- [Ericsson] Time offset (and possibly frequency resource) of the random-access occasions (RO) for the first D2R transmission (Msg1) for all traffic types (DO-A, DO-DTT, and DT). Cell-specific power control parameters. Time offset (and possibly frequency resource) of the paging occasions for Msg0 transmission of DT and DO-DTT traffic. Reader/cell selection information including certain RSRP level for reader selection/re-selection. D2R transmission configurations/parameters, e.g., size of preambles/midambles. Reader/network related information. Other information deemed necessary by RAN2.

- [CATT] It’s not necessary to include reader identification information, SFN, DO-A resource configuration, power control related parameters, device (un)availability related parameters and transmission parameters for L1 control information in broadcast information.

- [Nokia] Broadcast information, if supported, includes power control parameters such as target received power, allowed increment of transmission power, one or more chip duration values for a PRDCH for L1 control

- [CMCC] Study the following information to be included in broadcast information. Timing information, e.g. SFN-like info. Periodic random access resource configuration, e.g. periodicity, offset, available resource in each periodicity, and frequency resources for FDM transmission. DRX configuration for paging monitoring. Configurations of cell/reader-common L1 R2D control information. Cell ID related information.

- [Xiaomi] The number of Cell/Reader ID can be larger than 4, e.g., 1008, which may consider acquisition method based on synchronization signal sequence(s) in addition to MIB-like and/or SIB-like information. Timing information can be contained in broadcast information. Whether to use SFN or timing offset indication can be study by avoiding over-designing. Broadcast information containing L1 R2D control related information can provide configuration/updating flexibility.

- [ZTE] The following information can be considered included in the broadcast information: Paging-related information, timing information, e.g., SFN, frequency information for DL FDM, UL reference frequency position, DO-A resource related information, i.e., time and frequency resource information, parameters of R2D L1 control, e.g., chip duration, repetition number, code rate, cell/Reader/Transaction ID, SIP length information.

- [vivo] Study Case B2: Two-step acquisition of the broadcast information including both MIB-like and SIB1-like: The information can be carried by MIB-like: SFN information, AIoT SIB scheduling information. The information can be carried by SIB1-like: DOA resource configuration, L1 control related information configuration(s), R2D preamble configuration, The frequency information of AIOT carrier, DRX configuration for device power saving, Criteria for cell-(re)selection, Reader ID related information, Power control related information, Periodicity of sync signal (and the payload i.e., MIB). Note: other information is subject to RAN2 study.

- [LGE] Regarding the purpose and necessary information of broadcast information, it is observed that following information is needed: Cell/reader ID‑related information to enable reader identification. Resource‑allocation information for DO‑A transmissions. Timing information indicating the allocation timing of the D2R resources. D2R power‑control parameters for device transmit‑power control.

- [NEC] At least the following information can be carried in the A-IoT broadcast information. Scheduling Info for the first DO-A D2R transmission; Reader ID-related information; FFS: other information to be carried in A-IoT broadcast information.

- [China Telecom] In Rel-20, study the necessity and feasibility of the following scheduling information for device 2b/C: T/F resource configuration for SIB-like message and/or MSG1, SFN value, repetition index or period index of synchronization signal, time unit index (e.g., half-frame index), frequency offset between the synchronization signal and the SIB-like message or MSG1.

- [Samsung] Broadcast information is transmitted for the following purposes: Purpose #1: Triggering the inventory procedure (i.e., paging-like message for device discovery/presence checking), Purpose #2: Providing DO-A resources so that a device can initiate RA by itself, and Purpose #3: Providing reference information for open-loop power control. Broadcast information can carry at least one of the following information sets, depending on the purpose: Information for the inventory procedure trigger, DO-A resource information (e.g., periodicity, access occasion configuration, resource pool information, etc.) and Reference information related to R2D Tx power for power control.

- [ETRI] The broadcast information should include at least the following information elements: Cell/reader ID-related information, power control-related information, periodic D2R resource configuration for DO-A, and timing information (e.g., SFN-like information).

- [DOCOMO] For broadcast information, following information should be considered. Information for reader differentiation/identification such as reader ID. Parameters for first D2R transmission for DO-A such as D2R resource, physical layer design related parameters such as chip duration, FEC, repetition, etc., and power control information. Parameters for subsequent R2D reception such as potential R2D reception resources of R2D sync signal, L1 R2D control, A-IoT paging, R2D command and corresponding physical layer design related parameters such as M value, FEC, repetition, etc.

- [Qualcomm] Periodic system information (SIB) can provide necessary information including at least: Reader ID-related information (e.g., higher layer reader ID), R2D-related configuration (frequency location(s) for R2D carriers, configuration of periodic R2D monitoring occasions for SS and paging, L1 control information per coverage level), D2R-related configuration (frequency location(s) for D2R carriers, configuration of periodic DO-A D2R resources, power control per coverage level).

- [Lenovo] states a two-step method for acquiring broadcast information. In the first step, the broadcast provides a list of available frequencies for device access. In the second step, further time–frequency resources are supplied for device transmission/reception, along with possible reader ID information.

- [Huawei] For the broadcast information, the MIB-like transmission contains the reader identifier, SFN and the time and frequency resource allocation information for the SIB-like transmission. The SIB-like broadcast transmission contains the time and frequency resource allocation of DO-A, power control parameters, ARFCN-like frequency location information, RSRP threshold/level for the R2D measurement and other necessary parameters for DO-A and the active device.

- [H] Proposal 3.1.2b-1 (needs update to include CMCC observation 4. Use Case B1/B2)  Done!

On Case B1 and Case B2 for how A-IoT devices acquire the broadcast information, capture the following in TR 38.769:

On how A-IoT devices acquire the broadcast information, sources (Spreadtrum, Ericsson, CATTOPPO, Xiaomi, ZTE, Panasonic, Samsung, Qualcomm, Quectel, Fraunhofer) state that Case B1 (Single-step acquisition of the broadcast information) should be supported. Following observations on the Case B1 were provided during the study.

- [FUTUREWEI] Potential for high system overhead with frequent broadcast information transmission. Possible increase of device unavailability as a device may need to perform blind decoding of the L1 R2D control information before obtaining broadcast information.

- [Spreadtrum] For broadcast information, single-step acquisition of the broadcast information is supported. PRDCH is used to carry the broadcast information.

- [Ericsson] It is simpler and more efficient to transmit all the necessary information in a single step using a single PRDCH. Case B1 may also be better from reader and A-IoT device energy consumption perspectives.

- [CATTOPPO] Due to the limited battery capacity of A-IoT devices, it is desirable to provide all necessary system information to A-IoT devices at one time rather than introducing extra dedicated message for system information delivering (e.g., SIB1 like message), unless the payload size of the broadcast information identified at the end may result in an unacceptable A-IoT SSB length.

- [Xiaomi] If the payload size of broadcast information is small and fixed, there is no need to split it into two parts.

- [ZTE] For Case B1-1, the broadcast information includes L1 control, broadcast information scheduling is more flexible. For Case B1-2, the broadcast information does not include L1 control, the design is simpler and makes it easier for the device to detect the broadcast information.

- [vivo] If the periodicity of single broadcast information is same as that of periodic sync signal, larger resource overhead will be caused since some of the information is only used after initial reader search and don’t need to be transmission as frequent as periodic sync signal. If the periodicity of single broadcast information is larger than that of periodic sync signal, during initial search, a device needs to do more blind detection to find the potential broadcast information to obtain the essential information for completing the initial search e.g., SFN for timing alignment. This will increase the initial search complexity and latency. If the single broadcast information is SIB1-like information, the starting time location of SIB1-like broadcast information has to be fixed, which is less flexible for reader resource allocation.

- [LGE] The single‑step acquisition allows simple and fast acquisition at the device side but incurs signaling overhead and may introduce reader‑identification timing delay.

- [Panasonic] A single-step acquisition of broadcast signal using PRDCH should be considered.

- [Samsung] A single-step approach (i.e., Case B1) should be used for broadcast information, and introducing a two-step structure (i.e., Case B2) is not strongly motivated from an information-size perspective.

- [Sharp] For Case B1, the broadcast PRDCH should always be preceded by a synchronizing signal to facilitate detection and acquisition.

- [Qualcomm] For Case B1, devices may perform SIB monitoring with a predefined periodicity and detect the SIB directly. This approach is more straightforward and potentially more efficient.

- [Quectel] For the acquisition method of broadcast information, study single-step acquisition, i.e., Case B1 as baseline. Single-step acquisition of broadcast is simpler and costs less standardization efforts than two-step acquisition method. MIB-like in Ambient IoT is not necessary since ambient IoT devices do not require precise synchronization like SFN information and carrier offset. Another reason is the message size of broadcast information may be fixed and is not as large as SIB information.

- [Fraunhofer] For broadcast information acquisition prioritize Case B1. Single‑step acquisition minimizes the number of decoding tasks, and therefore, reducing the device activation time and avoiding the need to coordinate multiple acquisition stages. On the contrary, Case B2 introduces additional operational steps, requiring devices to decode two separate layers of system information, potentially from different physical channels. While this structure allows for modularity and independent periodicities for MIB‑like and SIB1‑like information, it also increases decoding complexity and extends the time during which devices must remain active. Therefore, Case B1 should be prioritized given the energy constrains of A-IoT devices.

Sources (FUTUREWEI, Huawei, CMCC, vivo, Lenovo) state that Case B2 (Two-step acquisition of the broadcast information) should be supported. Following observations were provided during the study.

- [FUTUREWEI] Allows the PRDCH containing broadcast information to be similar to other PRDCH by providing parameters to decode L1 R2D control information. Allows a device to know whether to process PRDCH containing broadcast information and whether the PRDCH containing broadcast information is scheduled. Allows a reader to coordinate device availability of all devices it serves.

- [Huawei] Advantages of two-step acquisition of the broadcast information (Case B2): The two-step split acquisition of the broadcast information is to facilitate an efficient method to provide the required information to the device at the required time - the system information carried by the MIB-like transmission is required by the device more frequently than the DO-A resource allocation information carried by the SIB-like broadcast information since it does not need to change frequently. The overhead of the resource allocation information for DO-A is larger than the other system information, and transmitting this in the SIB-like transmission with a longer periodic interval as compared to the MIB-like broadcast information can avoid a degradation of transmission efficiency. The impact of overhead is reduced when compared with Case B1 where a single-step acquisition of the broadcast information is used. The MIB-like transmission contains the reader identifier, SFN and the time and frequency resource allocation information for the SIB-like transmission. The SIB-like broadcast transmission contains the time and frequency resource allocation of DO-A, power control parameters, ARFCN-like frequency location information, RSRP threshold/level for the R2D measurement and other necessary parameters for DO-A and the active device.

- [CMCC] The payload to be carried in the broadcast information is expected to be large, it is more reasonable to split the broadcast information into two parts, i.e., a MIB-like part and a SIB1-like part.

- [ZTE] For Case B2, the design is more complex compared with Case B1, and incurs higher signaling overhead, which increases downlink blockage probability and consequently degrades inventory efficiency.

- [vivo] MIB-like is regarded as the payload of periodic sync signal to carry the minimum required information for device to complete the initial reader search, which has the same periodicity as that of periodic sync signal. Thus, it is less complex for a device to finish the initial reader search within the periodic sync signal cycle. With two-step acquisition of the broadcast information, the periodicity of SIB1-like can be more flexible e.g., larger than that of MIB-like to avoid unnecessary resource overhead. MIB-like can flexibly indicate the scheduling information of SIB1 e.g., the start time location of SIB1, which is benefit for reader resource allocation.

- [LGE] The two‑step acquisition reduces signaling overhead and allows early reader identification but may cause acquisition delay and may require additional specification work for a new physical channel.

- [Sharp] For Case B2, the MIB-like channel should be designed to be as simple as possible, in order to minimize device complexity.

- [Lenovo] Support two-step acquisition of the broadcast information (i.e., Case B2). In the first step, broadcast information transmitted on initial frequency provides a list of available frequencies for devices to access. In the second step, broadcast information transmitted on the available frequencies provides further resource occasions for devices to access.

- [Qualcomm] For Case B2, devices may perform MIB monitoring with a predefined periodicity. Devices first detect the MIB to obtain the SIB monitoring configuration and then acquire the SIB.

- [Quectel] For case B2, i.e., two step acquisition of the broadcast information results in more complexity comparing with single step acquisition.

Sources (NEC, Sharp, ETRI, DCM) state that the decision can be made after the content/size of the broadcast information becomes clear. Following observations were provided during the study.

- [NEC] RAN1 can further decide whether single-step or two-step acquisition is required for broadcast information in Rel-20 A-IoT after all the information carried by broadcast information and corresponding size have been clearly identified.

- [Sharp] The down-selection between Case B1 and Case B2 may depend on how much broadcast information needs to be carried, as well as the resulting impact on device complexity, coverage, and power consumption.

- [ETRI] The introduction of an MIB/SIB-based structure for broadcast information should be decided after the contents of the broadcast information are agreed.

- [DCM] Whether the broadcast information acquisition is single-step or two-step depends on the payload size of whole broadcast information, i.e., two-step is necessary if it would be expected to be large, otherwise, single-step is sufficient.

- [H] Proposal 3.1.2c-1 (Use Case B1/B2)  Done!

On whether to use a separate PRDCH or a new physical channel to carry the MIB-like broadcast information for Case B2, capture the following in TR 38.769:

For Case 2, sources (Huawei, vivo, China Telecom, LGE) state that PRDCH should be used to carry the MIB-like broadcast information.

- [Huawei] PRDCH is used to carry the MIB-like broadcast information as there is no additional motivation to define a separate or new channel like NR SSB, in which 8 information bits are carried in the physical layer payload. Secondly, carrying the MIB-like broadcast information on an additional channel would increase the workload in terms of specification work for the WI.

- [China Telecom] If case B2 is selected for the broadcast information transmission scheme, support to reuse a separate PRDCH for the MIB-like broadcast information. While using PRDCH may require a new R2D message format for MIB-like information, defining an entirely new broadcast channel would be significantly a larger workload. Given the limited TU before the SID is completed, reusing the PRDCH to carry MIB-like information would be more feasible and simpler to achieve.

- [vivo] Use PRDCH as the transmission channel for both MIB-like and SIB1-like broadcast information. Do not introduce a new channel to carry MIB-like broadcast information.

- [LGE] If the introduction of a new physical channel for the partial broadcast information is considered, the specification work may significantly increase.

Sources (CMCC, FUTUREWEI) are also open to use a new physical channel dedicated to MIB-like information.

- [CMCC] MIB-like broadcast information can be carried by a separate PRDCH or a new physical channel dedicated to MIB-like information. For MIB-like part, the R2D parameters such as TBS, payload size, FEC or repetition number can be predefined.

- [FUTUREWEI] Whether the MIB-like information is carried in its own channel can follow the concept of the PBCH, which is a separate channel for the MIB. This separate channel can simplify the specifications.

Proposals for Wednesday afternoon online session

9.3.2.3 Other procedures

Initial frequency/broadcast information acquisition

Previous agreements

Agreement (RAN1#122bis)

For R2D, study at least the following aspects for the study of broadcast information:

- Information included in the broadcast information

- Whether to use PRDCH to carry the broadcast information

- Transmission of broadcast information:

  - Option 1: periodic

  - Option 2: aperiodic

- Time and frequency resources for transmission of the broadcast information, e.g. in relation to the synchronization signal.

Agreement (RAN1#123)

Initial frequency acquisition procedure is necessary for Device 2b and for Device C, at least for flexible deployment of A-IoT frequency location(s) for outdoor scenario.

Agreement (RAN#123)

Reader identification/differentiation is necessary for Device 2b and for Device C, at least in outdoor scenario to differentiate R2D among different readers.

Agreement (RAN#123)

For initial frequency acquisition for Device 2b and for Device C, a device is expected to determine a frequency point from predefined sync raster points for reception of the periodic synchronization signal.

Agreement (RAN#123)

- The functionality of broadcast information is to provide the device with at least necessary information for subsequent transmission and reception

- On how A-IoT devices acquire the broadcast information, study the following two cases:

  - Case B1: Single-step acquisition of the broadcast information

  - Case B2: Two-step acquisition of the broadcast information (e.g., MIB-like and then SIB1-like broadcast information)

- On whether to use PRDCH to carry the broadcast information,

  - For Case B1, PRDCH is used to carry the broadcast information.

  - For Case B2

    - PRDCH is used to carry the SIB1-like broadcast information.

    - A separate PRDCH or a new physical channel dedicated to MIB-like is used to carry the MIB-like broadcast information

Agreement (RAN#123)

Periodic transmission of broadcast information to provide at least necessary information for subsequent transmission and reception is necessary for Device 2b and for Device C.

- FFS (in SI or WI): whether the transmission of broadcast information is strictly periodic at reader

- FFS (in SI or WI): periodicity of the transmission of broadcast information

Agreement (RAN#123)

For reader identification/differentiation at least in outdoor scenario to differentiate R2D among different readers, a device acquires the reader ID-related information by at least one of the following options:

- Option 1: Reader ID-related information is contained in the broadcast information.

- Option 2: Reader ID-related information is derived from the periodic sync signal sequence(s).

- Option 3: Reader ID-related information is carried by L1 R2D control information.

- Option 4: Reader ID-related information is contained in the higher-layer information in the paging-like message.

Reader identification/differentiation

Previous agreements

Agreement (RAN#123)

Reader identification/differentiation is necessary for Device 2b and for Device C, at least in outdoor scenario to differentiate R2D among different readers.

Agreement (RAN#123)

For reader identification/differentiation at least in outdoor scenario to differentiate R2D among different readers, a device acquires the reader ID-related information by at least one of the following options:

- Option 1: Reader ID-related information is contained in the broadcast information.

- Option 2: Reader ID-related information is derived from the periodic sync signal sequence(s).

- Option 3: Reader ID-related information is carried by L1 R2D control information.

- Option 4: Reader ID-related information is contained in the higher-layer information in the paging-like message.

Work plan for RAN1#124 meeting (R1-2509614)

- Reader identification/differentiation - Views on Option 1/2/3/4 for how a device acquires the reader ID-related information

Options 1/2/3/4 for how a device acquires the reader ID-related information were already captured.

This sub-clause is to further capture views on Option 1/2/3/4 for how a device acquires the reader ID-related information in 6A.3.x (L1 procedures) of TR 38.769.

Summary

Preference for Option 1/2/3/4 on how a device acquires the reader ID-related information:

- Option 1

  - Positive: Huawei, Spreadtrum, CMCC, ZTE, NEC, China Telecom, Sharp, ETRI, DCM, Qualcomm, Fraunhofer

  - Negative: CATT (infeasible)

- Option 2

  - Positive: CATT, CMCC, LGE, Panasonic, Fraunhofer

  - Negative: Huawei, Spreadtrum, NEC, China Telecom, ETRI

- Option 3:

  - Positive: CATT, Panasonic, Samsung, DCM, Qualcomm

  - Negative: vivo, NEC, ETRI

- Option 4:

  - Positive: CATT

  - Negative: vivo (infeasible), NEC, ETRI

- Combination of Option 1 and 2: CMCC, vivo, LGE, China Telecom

- Combination of Option 1 and 3: FUTUREWEI, Panasonic

- Combination of Option 1 and 2/3/4: Sharp

- Combination of Option 1 and 4: LGE

- [H] Proposal 3.1.1a-1

On the options for how a device acquires the reader ID-related information, capture the following in TR 38.769:

For Option 1, Sources (Huawei, Spreadtrum, CMCC, ZTE, NEC, China Telecom, Sharp, ETRI, DOCOMO, Qualcomm, Fraunhofer) state that Option 1 should be supported.

- [Huawei] The device can receive reader ID-related information in the initial part of the access procedure, enabling it to be able to use the reader ID for both reader differentiation and for the initial D2R transmission for DO-A. In the case of 2-step broadcast information acquisition, the reader ID can be included in the first step itself to ensure that the device receives the reader ID-related information in time to perform reader differentiation and DO-A transmissions, while not impacting the overhead significantly.

- [Spreadtrum] [Considering reducing the complexity of sequence detection for device, only one sequence of R2D periodic synchronization signal is supported. It’s not suitable to carry reader-ID using sync signal, so option 1 should be supported that Reader ID-related information is contained in the broadcast information.]

- [CMCC] This relaxes synchronization signal detection complexity, but since the device cannot identify the cell before decoding the channel for broadcast, cell-specific parameters cannot be applied to physical channel carrying broadcast information, so inter-Reader interference randomization cannot be used.

- [ZTE] Considering outdoor networking requirements, additional information such as the Cell or reader ID and Transaction ID also needs to be included in the broadcast information to help the device identify the serving cell or reader.

- [NEC] Due to the poor anti-interference capability of OOK and Manchester coding, it is difficult to enable different readers to be deployed on the same frequency point. Then, if in a certain area, there can only be one reader at a specific frequency point, i.e., there will be no adjacent readers deployed on the same frequency point, reader ID is unnecessary to be indicated along with each of the R2D transmission, it is enough to only indicate it during the initial frequency acquisition procedure, i.e., indicated by broadcast information.

- [China Telecom] Placing the reader ID outside the broadcast information would affect measurement and camp operations, which means measurement is only performed after synchronization procedure and waiting for the related signaling. Therefore, the reader ID should be included in the broadcast information.

- [Sharp] Option 1 (broadcast-based reader ID) should be adopted as a baseline mechanism for reader identification, ensuring that all devices can obtain basic reader identification information with minimal complexity. Options 2, 3, and 4 may be additionally supported and used by devices to further identify and differentiate the corresponding reader, especially in outdoor scenarios where multiple readers coexist and enhanced differentiation is required.

- [ETRI] [Sharp] Option 1 (broadcast-based reader ID) should be adopted as a baseline mechanism for reader identification, ensuring that all devices can obtain basic reader identification information with minimal complexity. Options 2, 3, and 4 may be additionally supported and used by devices to further identify and differentiate the corresponding reader, especially in outdoor scenarios where multiple readers coexist and enhanced differentiation is required.

- [DOCOMO] For initial broadcast information acquisition, reader ID-related information is contained in the broadcast information or carried by L1 R2D control information, i.e., option 1 or option 3 in the agreement should be considered.

- [Qualcomm] Periodic system information (SIB) can provide necessary information including at least: Reader ID-related information, e.g., higher layer reader ID, R2D-related configuration, D2R-related configuration, etc.

- [Fraunhofer] Mechanisms that provide reader‑ID‑related information during the discovery phase are more suitable than those relying on later‑stage control or higher‑layer signaling. Options 1 and 2, which convey or derive reader identification from broadcast information or periodic synchronization signals, respectively, align naturally with this requirement. Embedding or deriving reader‑ID‑related information from these early signals therefore minimizes delay, reduces energy consumption, and supports rapid reader selection in multi‑reader and mobility scenarios.

On the other hand, source (CATT) states that Option 1 should not be supported.

- [CATT] Reader identification information is best stored in non-volatile memory to ensure A-IoT device can use the information in all the subsequent transmissions after the first reception of reader identification information. The only option is NOT feasible is including in the broadcast information in Option 1, which requires to store the reader identification.

Following observations were also provided for Option 1.

- [FUTUREWEI] A device obtains reader ID-related information in a higher layer message without physical layer impact at the rate of broadcast information transmission.

- [Xiaomi] According to the discussion on broadcast acquisition (i.e., single-step or two-step), MIB-like information and/or SIB-like information can be the potential broadcast information that contains reader ID. The complexity of option 1 is low since device can obtain the reader ID directly by decoding broadcast information without any blind detection on the sequence. If the reader ID is contained only by MIB-like information, a device can simply get it by one-step decoding. However, when the total number of reader IDs is large by referring to PCID (1008), the payload size of MIB-like information will be large. Considering about the limited payload size for MIB-like information, the reader ID can be divided into two parts. One part of reader ID-related information is contained by MIB-like information, and the other part is contained by SIB-like information (if defined). By referring to the legacy design, MIB along with PSS/SSS and SIB are separately transmitted. The transmission gap between MIB-like information and SIB-like information can be large, and their transmission periodicities may also be different. A device can obtain the whole reader ID until it receives a MIB and a SIB-like information, which leads to higher latency and processing time compared to single-step.

- [LGE] Reader identification via broadcast information requires no additional signal design but forces the device to decode broadcast information even for undesired readers.

- [Samsung] Options 1, 2, and 4 are not necessarily transmitted with every R2D transmission. Therefore, they cannot provide reader identification/differentiation for every R2D transmission. To support reader differentiation, for every R2D transmission, it should be considered to either assume that Option 3 is always included in all R2D transmissions, apply an association rule when reader information is obtained via Options 1/2/4, or leverage the always-present R2D signal (e.g., SIP or CAP) to carry a reader-specific signature for differentiation.

- [Ericsson R1-2600210] If the Reader ID were not too long, “Option 2: Reader ID-related information is derived from the periodic sync signal sequence(s)” can be used aiming at performing an earlier reader identification. Otherwise, “Option 1: Reader ID-related information is contained in the broadcast information” can be considered.

- [H] Proposal 3.1.1b-1

On the options for how a device acquires the reader ID-related information, capture the following in TR 38.769:

For Option 2, Sources (CATT, CMCC, LGE, Panasonic, Fraunhofer) state that Option 2 should be supported.

- [CATT] To facilitate the device in identifying readers and selecting the one with the strongest signal for access, the reader identification information, which indicates the identifier of the reader, should be carried by R2D synchronization signal.

- [CMCC] This is similar as NR. But detecting different sequences during initial cell search increases device energy consumption. So, the number of IDs should be limited for this option.

- [LGE] Reader identification via a periodic sync signal sequence helps avoiding unnecessary broadcast decoding but increases detection complexity as the number of reader IDs grows.

- [Fraunhofer] Mechanisms that provide reader‑ID‑related information during the discovery phase are more suitable than those relying on later‑stage control or higher‑layer signaling. Options 1 and 2, which convey or derive reader identification from broadcast information or periodic synchronization signals, respectively, align naturally with this requirement. Embedding or deriving reader‑ID‑related information from these early signals therefore minimizes delay, reduces energy consumption, and supports rapid reader selection in multi‑reader and mobility scenarios.

On the other hand, sources (Huawei, Spreadtrum, NEC, China Telecom, ETRI) state that Option 2 should not be supported.

- [Huawei] Option 2 would increase the device complexity and power consumption since the device would have to blindly detect multiple sequences when performing the initial frequency acquisition procedure to determine the frequency location and reader ID-related information simultaneously.

- [NEC] For reader identification/differentiation, Option 2 is precluded because the number of sequences of R2D sync signal should be limited.

- [China Telecom] If only the reader ID is carried in the sync signal without accompanying broadcast information containing the SFN or time‑unit index, the device would be unable to report measurements accurately, and the reader could not perform proper camp control. This would make measurements become meaningless. Consequently, it is preferable to include reader ID within the broadcast information.

Following observations were also provided for Option 2.

- [FUTUREWEI] A device may have to perform detection of the periodic sync signal sequence to derive the reader ID-related information. It is unclear how a device can determine the better-quality sync signal sequence when sequences from multiple readers are received. This reader ID-related information is provided at the rate of periodic sync signal transmission.

- [Xiaomi] This is similar to NR PCID acquisition method based on PSS and SSS detection. Option 2 helps a device to get reader ID at the very beginning step before following steps such as system-like information acquisition and PRDCH reception. As it is agreed, R2D periodic synchronization signal is a binary sequence in time domain. If the number of binary sequence(s) is larger than 1 and no larger than 4, the sequence(s) can be used for potential reader identification/differentiation. However, 4 binary sequence(s) are too limited to identify different readers by considering that outdoor scenario may exist even larger number of cells/readers. In order to support large number of reader ID, two-level indication can be considered by combining Option 2 and Option 1. A device needs to derive the first-level of reader ID by detecting the sequence of periodic synchronization signal and obtain the second-level of reader ID from MIB-like or SIB-like information. Compared to Option 2 only, this combination method requires the device to process two steps to get the reader ID.

- [Samsung] Options 1, 2, and 4 are not necessarily transmitted with every R2D transmission. Therefore, they cannot provide reader identification/differentiation for every R2D transmission. To support reader differentiation, for every R2D transmission, it should be considered to either assume that Option 3 is always included in all R2D transmissions, apply an association rule when reader information is obtained via Options 1/2/4, or leverage the always-present R2D signal (e.g., SIP or CAP) to carry a reader-specific signature for differentiation.

- [Sharp] Option 1 (broadcast-based reader ID) should be adopted as a baseline mechanism for reader identification, ensuring that all devices can obtain basic reader identification information with minimal complexity. Options 2, 3, and 4 may be additionally supported and used by devices to further identify and differentiate the corresponding reader, especially in outdoor scenarios where multiple readers coexist and enhanced differentiation is required.

- [DOCOMO] For initial broadcast information acquisition, reader identification/differentiation based on periodic sync signal which device needs to blindly search on multiple candidate frequencies increases device complexity considering correlation-based detection. After broadcast information acquisition, reader ID-related information is derived from periodic sync signal or carried by L1 R2D control information.

- [Ericsson R1-2600210] If the Reader ID were not too long, “Option 2: Reader ID-related information is derived from the periodic sync signal sequence(s)” can be used aiming at performing an earlier reader identification. Otherwise, “Option 1: Reader ID-related information is contained in the broadcast information” can be considered.

- [H] Proposal 3.1.1c-1

On the options for how a device acquires the reader ID-related information, capture the following in TR 38.769:

For Option 3, Sources (CATT, Panasonic, Samsung, DOCOMO, Qualcomm) state that Option 3 should be supported.

- [Samsung] To support reader differentiation, the study on reader identification should proceed by selecting one of the following directions. Direction 1: Assume that Option 3 (L1 R2D control information) is always included in every R2D transmission. Direction 2: Acquire reader-related information via at least one of Options 1/2/4, and handle R2D transmissions without reader ID-related information based on an association rule (i.e., assume they originate from the same reader based on prior information).

- [DOCOMO] For initial broadcast information acquisition, reader ID-related information is contained in the broadcast information or carried by L1 R2D control information, i.e., option 1 or option 3 in the agreement should be considered. After broadcast information acquisition, reader ID-related information is derived from periodic sync signal or carried by L1 R2D control information.

- [Qualcomm] For flexible and reliable broadcast transmission, SIB-L1 control can be used to indicate the physical reader ID-related information, types of broadcast information, TBS, coding/repetitions for different coverage and device types, etc.

- [CATT] Since the reader identification information is only used in the scenario of the A-IoT receiving the PRDCH information from more than one reader, the retrieval of the reader identification is not critical whether is included in the periodic synchronization signal of Option 2, the L1-control information of Option 3, or the paging-like message of Option 4.

On the other hand, sources (vivo, NEC, ETRI, Huawei) state that Option 3 should not be supported.

- [vivo] At least for initial reader search, a device anyway needs to acquire all the information to finish the reader search and camp on it, so no big different to carry reader ID information via L1 control information or payload part. It will also increase the payload of L1 control information and result in the loss of L1 control detection performance.

- [NEC] For reader identification/differentiation, there is no need to further indicating the Reader ID-related information after initial frequency acquisition procedure because there cannot be multiple adjacent readers in a same frequency point. Therefore, Option 3 is precluded.

- [Huawei] The device would receive the reader ID only during an R2D transmission, which would mean that the device would not be able to use the reader ID for reader differentiation, nor for the D2R transmissions for DO-A. It would also increase the overhead of the control information as it is not necessary to be transmitted along with every R2D transmission.

Following observations were also provided for Option 3.

- [FUTUREWEI] A device obtains reader ID-related information in the L1 R2D control information, either as a specific field or as a mask applied to the CRC. The mask in combination of option 1, option 2, or option 4 can be used to accept / reject the PRDCH.

- [Xiaomi] Potential R2D signals with L1 R2D control information are SIB-like information, paging-like information, Msg2, R2D command (if these signals are supported). It is simple for a device to obtain the reader ID from L1 R2D control. When a device gets the reader ID, the device can determine whether to decode the corresponding data in PRDCH that is scheduled by this L1 R2D control. To support Option 3, a pre-condition is that the periodic synchronization signal and/or MIB-like information should also contain the reader ID. For example, a device should be synchronized to a reader and acquired its MIB-like information. If the reader ID in L1 R2D control of the following up R2D transmission(s) is not matched/same with the reader ID of the synchronized reader, the device can determine not to decode the corresponding PRDCH data part scheduled by this L1 R2D control.

- [LGE] Conveying the reader ID in L1 R2D control information enables explicit identification but introduces signaling overhead when applied to every PRDCH transmission. For Option 3, including the cell/reader ID in the L1 R2D control information is beneficial only when the broadcast information is transmitted on PRDCH and the L1 R2D control information is transmitted prior to the PRDCH. However, transmitting the cell/reader ID in the L1 R2D control information for every PRDCH used for inventory/command procedures is not desirable due to signaling overhead.

- [Sharp] Option 1 (broadcast-based reader ID) should be adopted as a baseline mechanism for reader identification, ensuring that all devices can obtain basic reader identification information with minimal complexity. Options 2, 3, and 4 may be additionally supported and used by devices to further identify and differentiate the corresponding reader, especially in outdoor scenarios where multiple readers coexist and enhanced differentiation is required.

- [Fraunhofer] Options 3 and 4 inherently assume that the device has already achieved synchronization and is actively monitoring reader‑specific control or paging occasions. As a result, reader identification becomes available only after the device has already committed resources to a particular reader context, which increases both latency and energy expenditure.

- [H] Proposal 3.1.1d-1

On the options for how a device acquires the reader ID-related information, capture the following in TR 38.769:

For Option 4, Source (CATT) states that Option 4 should be supported.

- [CATT] Since the reader identification information is only used in the scenario of the A-IoT receiving the PRDCH information from more than one reader, the retrieval of the reader identification is not critical whether is included in the periodic synchronization signal of Option 2, the L1-control information of Option 3, or the paging-like message of Option 4.

On the other hand, sources (vivo, NEC, ETRI, Huawei) state that Option 4 should not be supported.

- [vivo] It is infeasible since it cannot provide reader ID-related information during initial reader search. At least Option4 cannot operate as a standalone solution.

- [NEC] For reader identification/differentiation, there is no need to further indicating the Reader ID-related information after initial frequency acquisition procedure because there cannot be multiple adjacent readers in a same frequency point. Therefore, Option 4 is precluded.

- [Huawei] The device would not be able to use the reader ID for reader differentiation, nor for the D2R transmissions for DO-A. It will result in devices that want to transmit D2R DO-A traffic missing out on receiving the reader-ID related information and not being able to transmit the first D2R transmission for DO-A.

Following observations were also provided for Option 4.

- [FUTUREWEI] A device obtains reader ID-related information in a higher layer message without physical layer impact at the rate of paging-like messages.

- [Xiaomi] If paging-like message is supported, all of paging related information should be contained by higher-layer (e.g., MAC CE) and mapping to the data part of PRDCH in PHY-layer. Not like Option 3 that device can get the reader ID in L1 R2D control before decoding data part, Option 4 requires the device to decode both L1 R2D control and data part, and then the device can get the reader ID. Furthermore, Option 4 also prerequisites that the reader ID should be contained in periodic synchronization signal and/or MIB-like information.

- [LGE] Reader identification via higher‑layer information simplifies physical-layer operations but prevents early identification prior to acquiring higher‑layer signaling.

- [Samsung] Options 1, 2, and 4 are not necessarily transmitted with every R2D transmission. Therefore, they cannot provide reader identification/differentiation for every R2D transmission. To support reader differentiation, for every R2D transmission, it should be considered to either assume that Option 3 is always included in all R2D transmissions, apply an association rule when reader information is obtained via Options 1/2/4, or leverage the always-present R2D signal (e.g., SIP or CAP) to carry a reader-specific signature for differentiation.

- [Sharp] Option 1 (broadcast-based reader ID) should be adopted as a baseline mechanism for reader identification, ensuring that all devices can obtain basic reader identification information with minimal complexity. Options 2, 3, and 4 may be additionally supported and used by devices to further identify and differentiate the corresponding reader, especially in outdoor scenarios where multiple readers coexist and enhanced differentiation is required.

- [Fraunhofer] Options 3 and 4 inherently assume that the device has already achieved synchronization and is actively monitoring reader‑specific control or paging occasions. As a result, reader identification becomes available only after the device has already committed resources to a particular reader context, which increases both latency and energy expenditure.

- [H] Proposal 3.1.1e-1

On the options for how a device acquires the reader ID-related information, capture the following in TR 38.769:

Sources (CMCC, vivo, LGE, China Telecom) provided observations that combination of Option 1 and 2 should be supported.

- [CMCC] This option offers a trade-off between synchronization signal detection complexity and subsequent broadcast decoding performance. Finial determination may depend on the number of Reader IDs and sequence design of synchronization signal. If only a few IDs can be carried by sync signal sequence(s), the remaining can be contained in broadcast information.

- [vivo] For Option1+Option2, i.e., partial reader identification information is derived from the periodic sync signal sequence (3~4 candidate sequences) and the remaining information is carried in SIB1 broadcast information, the following benefits are observed. Compared to adopting option 1 only, after the initial search, a device can differentiate readers only via synchronization signal without decoding the following broadcast information, thereby reducing power consumption. And different periodic sync signal sequences can also reduce interference from different readers. Compared to adopting option 2 only, due to the limited number (e.g., 3~4) of candidate sequences of periodic synchronization signal, device blind detection complexity and design complexity can be reduced, and also the detection performance loss caused by large number of candidate sync sequences can be avoided. Based on these observations, it is proposed to support Option1+Option2 i.e., partial reader identification information is derived from the periodic sync signal sequence (3~4 candidate sequences) and the remaining information is carried by SIB1 broadcast information.

- [LGE] When determining the sequence length of the periodic sync signal, the number of cell/reader IDs that the device is required to distinguish needs to be considered. However, if the number of cell/reader IDs increases, either the number of periodic sync signal sequences or the sequence length increases, which results in higher detection complexity at the device side. Therefore, it may be necessary to consider Option 1 or Option 4 together with Option 2. In this case, the periodic sync signal may be used to distinguish cell/reader ID groups, and the actual cell/reader ID within the group may then be identified through broadcast information and/or higher-layer information.

- [China Telecom] If the synchronization signal and broadcast information are transmitted together, the reader ID could potentially be derived from the periodic synchronization signal sequences.

Sources (FUTUREWEI, Panasonic) provided observations that combination of Option1 and 3 should be supported.

- [FUTUREWEI] If the goal is to identify PRDCH from intended readers, then option 3 plus option 1 provides a check on each message using information provided in a broadcast while having limited impact on processing complexity.

- [Panasonic] To reduce the device’s monitoring time, the device needs to be synchronized with the serving reader(s). For this purpose, the periodic sync signal can carry reader ID-related information (e.g., Option 2). The sync signal does not need to carry the actual reader ID. For instance, 3 or 4 different sync signal sequences could be considered, while each sequence is used by one reader. The sequence can be assigned by NW to the reader. Once the device is synchronized with the serving reader, it will follow the reader for the PRDCH transmissions. Since the subcarrier might be used by other reader(s), the PRDCH should also carry the reader ID-related information. Although such information can be carried by the L1 control information or higher-layer information, using the L1 control information enables the device to only decode the control part to determine whether the message is from the serving reader. The L1 control information can carry the reader ID or a shorter ID dedicated to the reader assigned by the NW.

Source (Sharp) provided observations that combination of Option1 and 2/3/4 should be supported.

- [Sharp] Option 1 (broadcast-based reader ID) should be adopted as a baseline mechanism for reader identification, ensuring that all devices can obtain basic reader identification information with minimal complexity. Options 2, 3, and 4 may be additionally supported and used by devices to further identify and differentiate the corresponding reader, especially in outdoor scenarios where multiple readers coexist and enhanced differentiation is required.

Source (LGE) provided observations that combination of Option1 and 4 should be supported.

- [LGE] When determining the sequence length of the periodic sync signal, the number of cell/reader IDs that the device is required to distinguish needs to be considered. However, if the number of cell/reader IDs increases, either the number of periodic sync signal sequences or the sequence length increases, which results in higher detection complexity at the device side. Therefore, it may be necessary to consider Option 1 or Option 4 together with Option 2. In this case, the periodic sync signal may be used to distinguish cell/reader ID groups, and the actual cell/reader ID within the group may then be identified through broadcast information and/or higher-layer information.

| Company | Y/N | Comments |
| --- | --- | --- |
| Samsung |  | The current options may not provide reader identification for every R2D transmission. If reader identification is not supported for all R2D transmissions, we should at least agree on what assumption applies to R2D transmissions that do not include any of the above options; otherwise, there may be R2D transmissions for which reader identification is not possible. Also, we would like to capture our observation in the main bullet of each option or summarized as a separate main bullet. - [Samsung] Options 1, 2, and 4 are not necessarily transmitted with every R2D transmission. Therefore, they cannot provide reader identification/differentiation for every R2D transmission. To support reader differentiation for every R2D transmissio, it should be considered to either assume that Option 3 is always included in all R2D transmissions, apply an association rule when reader information is obtained via Options 1/2/4, or leverage the always-present R2D signal (e.g., SIP or CAP) to carry a reader-specific signature for differentiation. |
| LG | Y |  |
| Qualcomm |  | We think all Options could be needed and not contradictory to each other. For Option 1 and Option 4, the reader-related information, i.e., higher-layer reader ID with long length used to uniquely identify a reader, could be included in the MAC IE. For Option 2 and Option 3, the reader-related information, i.e., PHY reader ID with short length used to differentiate the readers for RAN-level network management, could be included in the PHY signal/channel. Partial information can be carried by periodic SS (Option 2) and remaining information can be included in L1 R2D control per PRDCH. |
| ETRI |  | On our view for Option 2/3/4, the expression of “these options should not be supported” is stronger than what we have in our contribution. Our opinion is Option 1 is the most suitable option to proceed, so it is better to remove our company name from the negative view as follows. For Option 2, Sources (CATT, CMCC, LGE, Panasonic, Fraunhofer) state that Option 2 should be supported while sources (Huawei, Spreadtrum, NEC, China Telecom) state that Option 2 should not be supported. Following sources provided observations on Option 2. For Option 3, Sources (CATT, Panasonic, Samsung, DCM, Qualcomm) state that Option 3 should be supported while sources (vivo, NEC) state that Option 3 should not be supported. Following sources provided observations on Option 3. For Option 4, Source (CATT) states that Option 4 should be supported while sources (vivo, NEC) state that Option 4 should not be supported. Following sources provided observations on Option 4. |
| Huawei, HiSilicon |  | Given that the proposal is quite long, one option to optimize the capturing of company observations is to split it into multiple proposals for each of the options, and under each option, it would be more meaningful to split them into pros and cons observations. |
| CATT | Y | OK with the TP. |
| China Telecom | Y | Fine. |
| Ericsson | See comment | Our input is missing, in part because this topic is also related with the “synchronization signal” design under Agenda Item (AI) 9.3.2.1 for which we wrote “Confirm that the number of binary sequence(s) is less than or equal to 4 for potential reader identification/differentiation”, which in principle refers to “Option 1” in this proposal. We can consider “Option 2” if reader ID size is too long. Having explained the above, please capture the following input from our side: “Option 2: Reader ID-related information is derived from the periodic sync signal sequence(s)” aiming at performing an earlier reader identification, unless the Reader ID were too long then “Option 1: Reader ID-related information is contained in the broadcast information” can be considered. |

Broadcast information

Previous agreements

Agreement (RAN#122bis)

For R2D, study at least the following aspects for the study of broadcast information:

- Information included in the broadcast information

- Whether to use PRDCH to carry the broadcast information

- Transmission of broadcast information:

  - Option 1: periodic

  - Option 2: aperiodic

- Time and frequency resources for transmission of the broadcast information, e.g. in relation to the synchronization signal.

Agreement (RAN#123)

- The functionality of broadcast information is to provide the device with at least necessary information for subsequent transmission and reception

- On how A-IoT devices acquire the broadcast information, study the following two cases:

  - Case B1: Single-step acquisition of the broadcast information

  - Case B2: Two-step acquisition of the broadcast information (e.g., MIB-like and then SIB1-like broadcast information)

- On whether to use PRDCH to carry the broadcast information,

  - For Case B1, PRDCH is used to carry the broadcast information.

  - For Case B2

    - PRDCH is used to carry the SIB1-like broadcast information.

    - A separate PRDCH or a new physical channel dedicated to MIB-like is used to carry the MIB-like broadcast information

Agreement (RAN#123)

Periodic transmission of broadcast information to provide at least necessary information for subsequent transmission and reception is necessary for Device 2b and for Device C.

- FFS (in SI or WI): whether the transmission of broadcast information is strictly periodic at reader

- FFS (in SI or WI): periodicity of the transmission of broadcast information

Work plan for RAN1#124 meeting (R1-2509614)

- Broadcast information - Purposes of the broadcast information and the necessary information per each purpose - Views on Case B1 and Case B2 for how A-IoT devices acquire the broadcast information - For Case B2, views on whether to use a separate PRDCH or a new physical channel to carry the MIB-like broadcast information - Time and freq. resource for the broadcast information, e.g. in relation to the synchronization signal To be discussed in AI 10.3.2.1

Summary

Information included in the broadcast information

- Reader ID related information: Ericsson, CMCC, Xiaomi, ZTE, vivo, LGE, NEC, ETRI, DOCOMO, Qualcomm, Spreadtrum

- D2R power control related information: Ericsson, Nokia, vivo, LGE, Samsung, ETRI, DOCOMO, Qualcomm

- Information related to resource allocation for the first D2R transmission for DO-A: OPPO, Ericsson, CMCC, Xiaomi, ZTE, vivo, LGE, NEC, China Telecom, Samsung, ETRI, DOCOMO, Qualcomm, Spreadtrum

- Parameters for subsequent R2D reception (e.g., for L1 R2D control information, paging monitoring/reception): OPPO, Ericsson, Nokia, CMCC, Xiaomi, ZTE, vivo, Samsung, DOCOMO, Qualcomm

- Timing information: CMCC, Xiaomi, ZTE, vivo, LGE, China Telecom, ETRI

- Reader selection/reselection related information: Ericsson, vivo

- SIB1-like scheduling information (for Case B2): vivo

- Other information deemed necessary by RAN2: Ericsson, vivo

Preference for Case B1 and Case B2 for how A-IoT devices acquire the broadcast information

- Case B1: Spreadtrum, Ericsson, CATT, Xiaomi, ZTE, Panasonic, Samsung, Qualcomm, Quectel, Fraunhofer

- Case B2: FUTUREWEI, Huawei, CMCC, vivo, Lenovo

- Other views: NEC, Sharp, ETRI, DOCOMO

Views on whether to use a separate PRDCH or a new physical channel to carry the MIB-like broadcast information for Case B2

- PRDCH: Huawei, Spreadtrum, OPPO, Ericsson, CATT, CMCC, Xiaomi, vivo, LGE, China Telecom, Fraunhofer

- New PHY channel: CMCC

  - Definition of the “new” physical channel: For MIB-like part, the R2D parameters such as TBS, payload size, FEC or repetition number can be predefined.

On the potential necessary information to be included in the broadcast information, capture the following observations in TR 38.769:

On the potential information included in the broadcast information, at least reader ID related information for reader identification/differentiation (Ericsson, CMCC, Xiaomi, ZTE, vivo, LGE, NEC, ETRI, DOCOMO, Qualcomm, Huawei), D2R power control related information (Ericsson, Nokia, vivo, LGE, Samsung, ETRI, DOCOMO, Qualcomm, Huawei), the information related to resource allocation for the first D2R transmission for DO-A and sources (OPPO, Ericsson, CMCC, Xiaomi, ZTE, vivo, LGE, NEC, China Telecom, Samsung, ETRI, DOCOMO, Qualcomm, Huawei) and timing information (CMCC, Xiaomi, ZTE, vivo, LGE, China Telecom, ETRI, Huawei) are studied. Sources (OPPO, Ericsson, Nokia, CMCC, Xiaomi, ZTE, vivo, Samsung, DOCOMO, Qualcomm) also state that parameters for subsequence R2D reception should be included in the broadcast information. Reader selection/reselection related information (Ericsson, vivo), SIB1-like scheduling information (for Case B2) (vivo, Huawei), and other information deemed necessary by RAN2 (Ericsson, vivo) are also studied. Following observations were provided during the study.

- [H] Proposal 3.1.2a-1-Alt.1

On the potential [necessary] information to be included in the broadcast information, capture the following observations in TR 38.769:

On the potential information included in the broadcast information, the following is a summary of information to be included in the broadcast information:

- Reader ID related information for reader identification/differentiation (Ericsson, CMCC, Xiaomi, ZTE, vivo, LGE, NEC, ETRI, DOCOMO, Qualcomm, Spreadtrum, Lenovo, Huawei, [China Telecom])

- D2R power control related information (Ericsson, Nokia, vivo, LGE, Samsung, ETRI, DOCOMO, Qualcomm, Huawei)

- The information related to resource allocation for the first D2R transmission for DO-A and sources (OPPO, Ericsson, CMCC, Xiaomi, ZTE, vivo, LGE, NEC, China Telecom, Samsung, ETRI, DOCOMO, Qualcomm, Spreadtrum, Huawei, Lenovo)

- Timing information (CMCC, Xiaomi, ZTE, vivo, LGE, China Telecom, ETRI, Huawei)

- Information related to subsequent R2D reception (OPPO, Ericsson, Nokia, CMCC, Xiaomi, ZTE, vivo, Samsung, DOCOMO, Qualcomm, Lenovo, Huawei, NEC(AI 9.3.2.1))

- Reader selection/reselection related information (Ericsson, vivo)

- SIB1-like scheduling information (for Case B2) (vivo, Huawei)

- A list of available frequencies for device access (Lenovo)

- Time–frequency resources for device transmission/reception (Lenovo)

- Other information deemed necessary by RAN2 (Ericsson, vivo)

Following observations were provided during the study.

- [OPPO] Broadcast information can contain time/frequency resource for L1 R2D control monitoring, time and frequency resources and related configurations for the initial DO-A transmission, etc.

- [Ericsson] Time offset (and possibly frequency resource) of the random-access occasions (RO) for the first D2R transmission (Msg1) for all traffic types (DO-A, DO-DTT, and DT). Cell-specific power control parameters. Time offset (and possibly frequency resource) of the paging occasions for Msg0 transmission of DT and DO-DTT traffic. Reader/cell selection information including certain RSRP level for reader selection/re-selection. D2R transmission configurations/parameters, e.g., size of preambles/midambles. Reader/network related information. Other information deemed necessary by RAN2.

- [CATT] It’s not necessary to include reader identification information, SFN, DO-A resource configuration, power control related parameters, device (un)availability related parameters and transmission parameters for L1 control information in broadcast information.

- [Nokia] Broadcast information, if supported, includes power control parameters such as target received power, allowed increment of transmission power, one or more chip duration values for a PRDCH for L1 control

- [CMCC] Study the following information to be included in broadcast information. Timing information, e.g. SFN-like info. Periodic random access resource configuration, e.g. periodicity, offset, available resource in each periodicity, and frequency resources for FDM transmission. DRX configuration for paging monitoring. Configurations of cell/reader-common L1 R2D control information. Cell ID related information.

- [Xiaomi] The number of Cell/Reader ID can be larger than 4, e.g., 1008, which may consider acquisition method based on synchronization signal sequence(s) in addition to MIB-like and/or SIB-like information. Timing information can be contained in broadcast information. Whether to use SFN or timing offset indication can be study by avoiding over-designing. Broadcast information containing L1 R2D control related information can provide configuration/updating flexibility.

- [ZTE] The following information can be considered included in the broadcast information: Paging-related information, timing information, e.g., SFN, frequency information for DL FDM, UL reference frequency position, DO-A resource related information, i.e., time and frequency resource information, parameters of R2D L1 control, e.g., chip duration, repetition number, code rate, cell/Reader/Transaction ID, SIP length information.

- [vivo] Study Case B2: Two-step acquisition of the broadcast information including both MIB-like and SIB1-like: The information can be carried by MIB-like: SFN information, AIoT SIB scheduling information. The information can be carried by SIB1-like: DOA resource configuration, L1 control related information configuration(s), R2D preamble configuration, The frequency information of AIOT carrier, DRX configuration for device power saving, Criteria for cell-(re)selection, Reader ID related information, Power control related information, Periodicity of sync signal (and the payload i.e., MIB). Note: other information is subject to RAN2 study.

- [LGE] Regarding the purpose and necessary information of broadcast information, it is observed that following information is needed: Cell/reader ID‑related information to enable reader identification. Resource‑allocation information for DO‑A transmissions. Timing information indicating the allocation timing of the D2R resources. D2R power‑control parameters for device transmit‑power control.

- [NEC] At least the following information can be carried in the A-IoT broadcast information. Scheduling Info for the first DO-A D2R transmission; Reader ID-related information; FFS: other information to be carried in A-IoT broadcast information.

- [China Telecom] In Rel-20, study the necessity and feasibility of the following scheduling information for device 2b/C: T/F resource configuration for SIB-like message and/or MSG1, SFN value, repetition index or period index of synchronization signal, time unit index (e.g., half-frame index), frequency offset between the synchronization signal and the SIB-like message or MSG1.

- [Samsung] Broadcast information is transmitted for the following purposes: Purpose #1: Triggering the inventory procedure (i.e., paging-like message for device discovery/presence checking), Purpose #2: Providing DO-A resources so that a device can initiate RA by itself, and Purpose #3: Providing reference information for open-loop power control. Broadcast information can carry at least one of the following information sets, depending on the purpose: Information for the inventory procedure trigger, DO-A resource information (e.g., periodicity, access occasion configuration, resource pool information, etc.) and Reference information related to R2D Tx power for power control.

- [ETRI] The broadcast information should include at least the following information elements: Cell/reader ID-related information, power control-related information, periodic D2R resource configuration for DO-A, and timing information (e.g., SFN-like information).

- [DOCOMO] For broadcast information, following information should be considered. Information for reader differentiation/identification such as reader ID. Parameters for first D2R transmission for DO-A such as D2R resource, physical layer design related parameters such as chip duration, FEC, repetition, etc., and power control information. Parameters for subsequent R2D reception such as potential R2D reception resources of R2D sync signal, L1 R2D control, A-IoT paging, R2D command and corresponding physical layer design related parameters such as M value, FEC, repetition, etc.

- [Qualcomm] Periodic system information (SIB) can provide necessary information including at least: Reader ID-related information (e.g., higher layer reader ID), R2D-related configuration (frequency location(s) for R2D carriers, configuration of periodic R2D monitoring occasions for SS and paging, L1 control information per coverage level), D2R-related configuration (frequency location(s) for D2R carriers, configuration of periodic DO-A D2R resources, power control per coverage level).

- [Lenovo] states a two-step method for acquiring broadcast information. In the first step, the broadcast provides a list of available frequencies for device access. In the second step, further time–frequency resources are supplied for device transmission/reception, along with possible reader ID information.

- [Huawei] For the broadcast information, the MIB-like transmission contains the reader identifier, SFN and the time and frequency resource allocation information for the SIB-like transmission. The SIB-like broadcast transmission contains the time and frequency resource allocation of DO-A, power control parameters, ARFCN-like frequency location information, RSRP threshold/level for the R2D measurement and other necessary parameters for DO-A and the active device.

- [H] Proposal 3.1.2a-1-Alt.2 (Use Case B1/B2)

On the potential [necessary] information to be included in the broadcast information, capture the following observations in TR 38.769:

On the potential information included in the broadcast information, following observations were provided during the study.

- [OPPO] Broadcast information can contain time/frequency resource for L1 R2D control monitoring, time and frequency resources and related configurations for the initial DO-A transmission, etc.

- [Ericsson] Time offset (and possibly frequency resource) of the random-access occasions (RO) for the first D2R transmission (Msg1) for all traffic types (DO-A, DO-DTT, and DT). Cell-specific power control parameters. Time offset (and possibly frequency resource) of the paging occasions for Msg0 transmission of DT and DO-DTT traffic. Reader/cell selection information including certain RSRP level for reader selection/re-selection. D2R transmission configurations/parameters, e.g., size of preambles/midambles. Reader/network related information. Other information deemed necessary by RAN2.

- [CATT] It’s not necessary to include reader identification information, SFN, DO-A resource configuration, power control related parameters, device (un)availability related parameters and transmission parameters for L1 control information in broadcast information.

- [Nokia] Broadcast information, if supported, includes power control parameters such as target received power, allowed increment of transmission power, one or more chip duration values for a PRDCH for L1 control

- [CMCC] Study the following information to be included in broadcast information. Timing information, e.g. SFN-like info. Periodic random access resource configuration, e.g. periodicity, offset, available resource in each periodicity, and frequency resources for FDM transmission. DRX configuration for paging monitoring. Configurations of cell/reader-common L1 R2D control information. Cell ID related information.

- [Xiaomi] The number of Cell/Reader ID can be larger than 4, e.g., 1008, which may consider acquisition method based on synchronization signal sequence(s) in addition to MIB-like and/or SIB-like information. Timing information can be contained in broadcast information. Whether to use SFN or timing offset indication can be study by avoiding over-designing. Broadcast information containing L1 R2D control related information can provide configuration/updating flexibility.

- [ZTE] The following information can be considered included in the broadcast information: Paging-related information, timing information, e.g., SFN, frequency information for DL FDM, UL reference frequency position, DO-A resource related information, i.e., time and frequency resource information, parameters of R2D L1 control, e.g., chip duration, repetition number, code rate, cell/Reader/Transaction ID, SIP length information.

- [vivo] Study Case B2: Two-step acquisition of the broadcast information including both MIB-like and SIB1-like: The information can be carried by MIB-like: SFN information, AIoT SIB scheduling information. The information can be carried by SIB1-like: DOA resource configuration, L1 control related information configuration(s), R2D preamble configuration, The frequency information of AIOT carrier, DRX configuration for device power saving, Criteria for cell-(re)selection, Reader ID related information, Power control related information, Periodicity of sync signal (and the payload i.e., MIB). Note: other information is subject to RAN2 study.

- [LGE] Regarding the purpose and necessary information of broadcast information, it is observed that following information is needed: Cell/reader ID‑related information to enable reader identification. Resource‑allocation information for DO‑A transmissions. Timing information indicating the allocation timing of the D2R resources. D2R power‑control parameters for device transmit‑power control.

- [NEC] At least the following information can be carried in the A-IoT broadcast information. Scheduling Info for the first DO-A D2R transmission; Reader ID-related information; FFS: other information to be carried in A-IoT broadcast information.

- [China Telecom] In Rel-20, study the necessity and feasibility of the following scheduling information for device 2b/C: T/F resource configuration for SIB-like message and/or MSG1, SFN value, repetition index or period index of synchronization signal, time unit index (e.g., half-frame index), frequency offset between the synchronization signal and the SIB-like message or MSG1.

- [Samsung] Broadcast information is transmitted for the following purposes: Purpose #1: Triggering the inventory procedure (i.e., paging-like message for device discovery/presence checking), Purpose #2: Providing DO-A resources so that a device can initiate RA by itself, and Purpose #3: Providing reference information for open-loop power control. Broadcast information can carry at least one of the following information sets, depending on the purpose: Information for the inventory procedure trigger, DO-A resource information (e.g., periodicity, access occasion configuration, resource pool information, etc.) and Reference information related to R2D Tx power for power control.

- [ETRI] The broadcast information should include at least the following information elements: Cell/reader ID-related information, power control-related information, periodic D2R resource configuration for DO-A, and timing information (e.g., SFN-like information).

- [DOCOMO] For broadcast information, following information should be considered. Information for reader differentiation/identification such as reader ID. Parameters for first D2R transmission for DO-A such as D2R resource, physical layer design related parameters such as chip duration, FEC, repetition, etc., and power control information. Parameters for subsequent R2D reception such as potential R2D reception resources of R2D sync signal, L1 R2D control, A-IoT paging, R2D command and corresponding physical layer design related parameters such as M value, FEC, repetition, etc.

- [Qualcomm] Periodic system information (SIB) can provide necessary information including at least: Reader ID-related information (e.g., higher layer reader ID), R2D-related configuration (frequency location(s) for R2D carriers, configuration of periodic R2D monitoring occasions for SS and paging, L1 control information per coverage level), D2R-related configuration (frequency location(s) for D2R carriers, configuration of periodic DO-A D2R resources, power control per coverage level).

- [Lenovo] states a two-step method for acquiring broadcast information. In the first step, the broadcast provides a list of available frequencies for device access. In the second step, further time–frequency resources are supplied for device transmission/reception, along with possible reader ID information.

- [Huawei] For the broadcast information, the MIB-like transmission contains the reader identifier, SFN and the time and frequency resource allocation information for the SIB-like transmission. The SIB-like broadcast transmission contains the time and frequency resource allocation of DO-A, power control parameters, ARFCN-like frequency location information, RSRP threshold/level for the R2D measurement and other necessary parameters for DO-A and the active device.

| Company | Y/N | Comments |
| --- | --- | --- |
| Spreadtrum |  | We also support reader ID related information and the information related to resource allocation for the first D2R transmission for DO-A included in the broadcast information. On the information included in the broadcast information, reader ID related information for reader identification/differentiation (Ericsson, CMCC, Xiaomi, ZTE, vivo, LGE, NEC, ETRI, DCM, Qualcomm, Spreadtrum), D2R power control related information (Ericsson, Nokia, vivo, LGE, Samsung, ETRI, DCM, Qualcomm), the information related to resource allocation for the first D2R transmission for DO-A and sources (OPPO, Ericsson, CMCC, Xiaomi, ZTE, vivo, LGE, NEC, China Telecom, Samsung, ETRI, DCM, Qualcomm, Spreadtrum) |
| LG | Y |  |
| Lenovo |  | Please add our view as below: Source [Lenovo] states a two-step method for acquiring broadcast information. In the first step, the broadcast provides a list of available frequencies for device access. In the second step, further time–frequency resources are supplied for device transmission/reception, along with possible reader ID information. |
| Huawei, HiSilicon |  | Our observations from our TDoc have not been included in the proposal. Added them and our company name to the relevant supporting set of companies. On the potential necessary information to be included in the broadcast information, capture the following observations in TR 38.769: On the potential information included in the broadcast information, at least reader ID related information for reader identification/differentiation (Ericsson, CMCC, Xiaomi, ZTE, vivo, LGE, NEC, ETRI, DCM, Qualcomm, Huawei), D2R power control related information (Ericsson, Nokia, vivo, LGE, Samsung, ETRI, DCM, Qualcomm, Huawei), the information related to resource allocation for the first D2R transmission for DO-A and sources (OPPO, Ericsson, CMCC, Xiaomi, ZTE, vivo, LGE, NEC, China Telecom, Samsung, ETRI, DCM, Qualcomm, Huawei) and timing information (CMCC, Xiaomi, ZTE, vivo, LGE, China Telecom, ETRI, Huawei) are studied. Sources (OPPO, Ericsson, Nokia, CMCC, Xiaomi, ZTE, vivo, Samsung, DCM, Qualcomm) also state that parameters for subsequence R2D reception should be included in the broadcast information. Reader selection/reselection related information (Ericsson, vivo), SIB1-like scheduling information (for Case B2) (vivo, Huawei), and other information deemed necessary by RAN2 (Ericsson, vivo) are also studied. Following observations were provided during the study. - [Huawei] For the broadcast information, the MIB-like transmission contains the reader identifier, SFN and the time and frequency resource allocation information for the SIB-like transmission. The SIB-like broadcast transmission contains the time and frequency resource allocation of DO-A, power control parameters, ARFCN-like frequency location information, RSRP threshold/level for the R2D measurement and other necessary parameters for DO-A and the active device. To optimize this proposal, it would be better to capture the content and the respective supporting companies, similar to how the content of the L1 R2D control information is captured in the TR in Clause “6A.1.x.1 Content”. Also, is this list exhaustive, or are these the parameters that are at least potentially going to be included in the broadcast information? The word “potential” and “at least “has been added in the initial line, assuming this is not an exhaustive list. |
| CATT | Y |  |
| China Telecom |  | On the information included in the broadcast information, reader ID related information for reader identification/differentiation (Ericsson, CMCC, Xiaomi, ZTE, vivo, LGE, NEC, ETRI, DCM, Qualcomm, China Telecom), D2R power control related information (Ericsson, Nokia, vivo, LGE, Samsung, ETRI, DCM, Qualcomm), the information related to resource allocation for the first D2R transmission for DO-A and sources (OPPO, Ericsson, CMCC, Xiaomi, ZTE, vivo, LGE, NEC, China Telecom, Samsung, ETRI, DCM, Qualcomm) and timing information (CMCC, Xiaomi, ZTE, vivo, LGE, China Telecom, ETRI) are studied. Sources (OPPO, Ericsson, Nokia, CMCC, Xiaomi, ZTE, vivo, Samsung, DCM, Qualcomm) also state that parameters for subsequence R2D reception should be included in the broadcast information. Reader selection/reselection related information (Ericsson, vivo), SIB1-like scheduling information (for Case B2) (vivo), and other information deemed necessary by RAN2 (Ericsson, vivo) are also studied. Following observations were provided during the study. We propose to add us on the corresponding observations. In the previous FL proposal, our opinion is that reader-ID related information shall be included in the broadcast information, hence we propose to add our company. |

- [H] Proposal 3.1.2b-1 (needs update to include CMCC observation 4. Use Case B1/B2)  Done!

On Case B1 and Case B2 for how A-IoT devices acquire the broadcast information, capture the following in TR 38.769:

On how A-IoT devices acquire the broadcast information, sources (Spreadtrum, Ericsson, CATTOPPO, Xiaomi, ZTE, Panasonic, Samsung, Qualcomm, Quectel, Fraunhofer) state that Case B1 (Single-step acquisition of the broadcast information) should be supported. Following observations on the Case B1 were provided during the study.

- [FUTUREWEI] Potential for high system overhead with frequent broadcast information transmission. Possible increase of device unavailability as a device may need to perform blind decoding of the L1 R2D control information before obtaining broadcast information.

- [Spreadtrum] For broadcast information, single-step acquisition of the broadcast information is supported. PRDCH is used to carry the broadcast information.

- [Ericsson] It is simpler and more efficient to transmit all the necessary information in a single step using a single PRDCH. Case B1 may also be better from reader and A-IoT device energy consumption perspectives.

- [CATTOPPO] Due to the limited battery capacity of A-IoT devices, it is desirable to provide all necessary system information to A-IoT devices at one time rather than introducing extra dedicated message for system information delivering (e.g., SIB1 like message), unless the payload size of the broadcast information identified at the end may result in an unacceptable A-IoT SSB length.

- [Xiaomi] If the payload size of broadcast information is small and fixed, there is no need to split it into two parts.

- [ZTE] For Case B1-1, the broadcast information includes L1 control, broadcast information scheduling is more flexible. For Case B1-2, the broadcast information does not include L1 control, the design is simpler and makes it easier for the device to detect the broadcast information.

- [vivo] If the periodicity of single broadcast information is same as that of periodic sync signal, larger resource overhead will be caused since some of the information is only used after initial reader search and don’t need to be transmission as frequent as periodic sync signal. If the periodicity of single broadcast information is larger than that of periodic sync signal, during initial search, a device needs to do more blind detection to find the potential broadcast information to obtain the essential information for completing the initial search e.g., SFN for timing alignment. This will increase the initial search complexity and latency. If the single broadcast information is SIB1-like information, the starting time location of SIB1-like broadcast information has to be fixed, which is less flexible for reader resource allocation.

- [LGE] The single‑step acquisition allows simple and fast acquisition at the device side but incurs signaling overhead and may introduce reader‑identification timing delay.

- [Panasonic] A single-step acquisition of broadcast signal using PRDCH should be considered.

- [Samsung] A single-step approach (i.e., Case B1) should be used for broadcast information, and introducing a two-step structure (i.e., Case B2) is not strongly motivated from an information-size perspective.

- [Sharp] For Case B1, the broadcast PRDCH should always be preceded by a synchronizing signal to facilitate detection and acquisition.

- [Qualcomm] For Case B1, devices may perform SIB monitoring with a predefined periodicity and detect the SIB directly. This approach is more straightforward and potentially more efficient.

- [Quectel] For the acquisition method of broadcast information, study single-step acquisition, i.e., Case B1 as baseline. Single-step acquisition of broadcast is simpler and costs less standardization efforts than two-step acquisition method. MIB-like in Ambient IoT is not necessary since ambient IoT devices do not require precise synchronization like SFN information and carrier offset. Another reason is the message size of broadcast information may be fixed and is not as large as SIB information.

- [Fraunhofer] For broadcast information acquisition prioritize Case B1. Single‑step acquisition minimizes the number of decoding tasks, and therefore, reducing the device activation time and avoiding the need to coordinate multiple acquisition stages. On the contrary, Case B2 introduces additional operational steps, requiring devices to decode two separate layers of system information, potentially from different physical channels. While this structure allows for modularity and independent periodicities for MIB‑like and SIB1‑like information, it also increases decoding complexity and extends the time during which devices must remain active. Therefore, Case B1 should be prioritized given the energy constrains of A-IoT devices.

Sources (FUTUREWEI, Huawei, CMCC, vivo, Lenovo) state that Case B2 (Two-step acquisition of the broadcast information) should be supported. Following observations were provided during the study.

- [FUTUREWEI] Allows the PRDCH containing broadcast information to be similar to other PRDCH by providing parameters to decode L1 R2D control information. Allows a device to know whether to process PRDCH containing broadcast information and whether the PRDCH containing broadcast information is scheduled. Allows a reader to coordinate device availability of all devices it serves.

- [Huawei] Advantages of two-step acquisition of the broadcast information (Case B2): The two-step split acquisition of the broadcast information is to facilitate an efficient method to provide the required information to the device at the required time - the system information carried by the MIB-like transmission is required by the device more frequently than the DO-A resource allocation information carried by the SIB-like broadcast information since it does not need to change frequently. The overhead of the resource allocation information for DO-A is larger than the other system information, and transmitting this in the SIB-like transmission with a longer periodic interval as compared to the MIB-like broadcast information can avoid a degradation of transmission efficiency. The impact of overhead is reduced when compared with Case B1 where a single-step acquisition of the broadcast information is used. The MIB-like transmission contains the reader identifier, SFN and the time and frequency resource allocation information for the SIB-like transmission. The SIB-like broadcast transmission contains the time and frequency resource allocation of DO-A, power control parameters, ARFCN-like frequency location information, RSRP threshold/level for the R2D measurement and other necessary parameters for DO-A and the active device.

- [CMCC] The payload to be carried in the broadcast information is expected to be large, it is more reasonable to split the broadcast information into two parts, i.e., a MIB-like part and a SIB1-like part.

- [ZTE] For Case B2, the design is more complex compared with Case B1, and incurs higher signaling overhead, which increases downlink blockage probability and consequently degrades inventory efficiency.

- [vivo] MIB-like is regarded as the payload of periodic sync signal to carry the minimum required information for device to complete the initial reader search, which has the same periodicity as that of periodic sync signal. Thus, it is less complex for a device to finish the initial reader search within the periodic sync signal cycle. With two-step acquisition of the broadcast information, the periodicity of SIB1-like can be more flexible e.g., larger than that of MIB-like to avoid unnecessary resource overhead. MIB-like can flexibly indicate the scheduling information of SIB1 e.g., the start time location of SIB1, which is benefit for reader resource allocation.

- [LGE] The two‑step acquisition reduces signaling overhead and allows early reader identification but may cause acquisition delay and may require additional specification work for a new physical channel.

- [Sharp] For Case B2, the MIB-like channel should be designed to be as simple as possible, in order to minimize device complexity.

- [Lenovo] Support two-step acquisition of the broadcast information (i.e., Case B2). In the first step, broadcast information transmitted on initial frequency provides a list of available frequencies for devices to access. In the second step, broadcast information transmitted on the available frequencies provides further resource occasions for devices to access.

- [Qualcomm] For Case B2, devices may perform MIB monitoring with a predefined periodicity. Devices first detect the MIB to obtain the SIB monitoring configuration and then acquire the SIB.

- [Quectel] For case B2, i.e., two step acquisition of the broadcast information results in more complexity comparing with single step acquisition.

Sources (NEC, Sharp, ETRI, DCM) state that the decision can be made after the content/size of the broadcast information becomes clear. Following observations were provided during the study.

- [NEC] RAN1 can further decide whether single-step or two-step acquisition is required for broadcast information in Rel-20 A-IoT after all the information carried by broadcast information and corresponding size have been clearly identified.

- [Sharp] The down-selection between Case B1 and Case B2 may depend on how much broadcast information needs to be carried, as well as the resulting impact on device complexity, coverage, and power consumption.

- [ETRI] The introduction of an MIB/SIB-based structure for broadcast information should be decided after the contents of the broadcast information are agreed.

- [DCM] Whether the broadcast information acquisition is single-step or two-step depends on the payload size of whole broadcast information, i.e., two-step is necessary if it would be expected to be large, otherwise, single-step is sufficient.

| Company | Y/N | Comments |
| --- | --- | --- |
| OPPO | Comment | As proposed in our Tdoc, we also support Case B1(i.e., Single-step acquisition of the broadcast information). Due to the limited battery capacity of A-IoT devices, it is desirable to provide all necessary system information to A-IoT devices at one time rather than introducing extra dedicated message for system information delivering (e.g., SIB1 like message), unless the payload size of the broadcast information identified at the end may result in an unacceptable length. |
| LG | Y |  |
| Futurewei | Y | Editorial: Case 1 should be Case B1, Case 2 should be Case B2 for consistency On how A-IoT devices acquire the broadcast information, sources (Spreadtrum, Ericsson, CATT, Xiaomi, ZTE, Panasonic, Samsung, Qualcomm, Quectel, Fraunhofer) state that Case 1 (Single-step acquisition of the broadcast information) should be supported. Following observations on the Case 1 were provided during the study. Sources (FUTUREWEI, Huawei, CMCC, vivo, Lenovo) state that Case 2 (Two-step acquisition of the broadcast information) should be supported. Following observations were provided during the study. |
| Huawei, HiSilicon |  | For our observations that have been captured here, it also includes text describing the parameters to be included in the broadcast information. Since this has been described in the previous proposal, we prefer to move it there instead. - [Huawei] Advantages of two-step acquisition of the broadcast information (Case B2): The two-step split acquisition of the broadcast information is to facilitate an efficient method to provide the required information to the device at the required time - the system information carried by the MIB-like transmission is required by the device more frequently than the DO-A resource allocation information carried by the SIB-like broadcast information since it does not need to change frequently. The overhead of the resource allocation information for DO-A is larger than the other system information, and transmitting this in the SIB-like transmission with a longer periodic interval as compared to the MIB-like broadcast information can avoid a degradation of transmission efficiency. The impact of overhead is reduced when compared with Case B1 where a single-step acquisition of the broadcast information is used. The MIB-like transmission contains the reader identifier, SFN and the time and frequency resource allocation information for the SIB-like transmission. The SIB-like broadcast transmission contains the time and frequency resource allocation of DO-A, power control parameters, ARFCN-like frequency location information, RSRP threshold/level for the R2D measurement and other necessary parameters for DO-A and the active device. |
| ZTE, Sanechips |  | The following update(red text) is proposed: On Case B1 and Case B2 for how A-IoT devices acquire the broadcast information, capture the following in TR 38.769: On how A-IoT devices acquire the broadcast information, sources (Spreadtrum, Ericsson, CATT, Xiaomi, ZTE, Panasonic, Samsung, Qualcomm, Quectel, Fraunhofer) state that Case 1 (Single-step acquisition of the broadcast information) should be supported. Following observations on the Case 1 were provided during the study. - [FUTUREWEI] Potential for high system overhead with frequent broadcast information transmission. Possible increase of device unavailability as a device may need to perform blind decoding of the L1 R2D control information before obtaining broadcast information. - [Spreadtrum] For broadcast information, single-step acquisition of the broadcast information is supported. PRDCH is used to carry the broadcast information. - [Ericsson] It is simpler and more efficient to transmit all the necessary information in a single step using a single PRDCH. Case B1 may also be better from reader and A-IoT device energy consumption perspectives. - [CATT] Due to the limited battery capacity of A-IoT devices, it is desirable to provide all necessary system information to A-IoT devices at one time rather than introducing extra dedicated message for system information delivering (e.g., SIB1 like message), unless the payload size of the broadcast information identified at the end may result in an unacceptable A-IoT SSB length. - [Xiaomi] If the payload size of broadcast information is small and fixed, there is no need to split it into two parts. - [ZTE] For Case B1-1, the broadcast information includes L1 control, broadcast information scheduling is more flexible. For Case B1-2, the broadcast information does not include L1 control, the design is simpler and makes it easier for the device to detect the broadcast information. |
| NEC | Y |  |
| CATT | N | We don’t have the description that “Due to the limited battery capacity of A-IoT devices, it is desirable to provide all necessary system information to A-IoT devices at one time rather than introducing extra dedicated message for system information delivering (e.g., SIB1 like message), unless the payload size of the broadcast information identified at the end may result in an unacceptable A-IoT SSB length”. Please remove the bullet. |

- [H] Proposal 3.1.2c-1 (Use Case B1/B2)  Done!

On whether to use a separate PRDCH or a new physical channel to carry the MIB-like broadcast information for Case B2, capture the following in TR 38.769:

For Case 2, sources (Huawei, Spreadtrum, OPPO, Ericsson, CATT, CMCC, Xiaomi, vivo, LGE, China Telecom, Fraunhofer) state that PRDCH should be used to carry the MIB-like broadcast information, while sources (CMCC, FUTUREWEI) states that are also open to use a new physical channel dedicated to MIB-like information should be used. During the study, following observations were provided.

- [Huawei] Firstly, we do not see the PRDCH is used to carry the MIB-like broadcast information as there is no additional motivation to define a separate or new channel like NR SSB, in which 8 information bits are carried in the physical layer payload. Secondly, carrying the MIB-like broadcast information on an additional channel would increase the workload in terms of specification work for the WI.

- [SRPD] For broadcast information, single-step acquisition of the broadcast information is supported and PRDCH is used to carry the broadcast information.

- [China Telecom] If case B2 is selected for the broadcast information transmission scheme, support to reuse a separate PRDCH for the MIB-like broadcast information. While using PRDCH may require a new R2D message format for MIB-like information, defining an entirely new broadcast channel would be significantly a larger workload. Given the limited TU before the SID is completed, reusing the PRDCH to carry MIB-like information would be more feasible and simpler to achieve.

- [vivo] Use PRDCH as the transmission channel for both MIB-like and SIB1-like broadcast information. Do not introduce a new channel to carry MIB-like broadcast information.

- [LGE] If the introduction of a new physical channel for the partial broadcast information is considered, the specification work may significantly increase.

while sSources (CMCC, FUTUREWEI) states that are also open to use a new physical channel dedicated to MIB-like information should be used. During the study, following observations were provided.

- [CMCC] MIB-like broadcast information can be carried by a separate PRDCH or a new physical channel dedicated to MIB-like information. For MIB-like part, the R2D parameters such as TBS, payload size, FEC or repetition number can be predefined.

- [FUTUREWEI] Whether the MIB-like information is carried in its own channel can follow the concept of the PBCH, which is a separate channel for the MIB. This separate channel can simplify the specifications.

Not limited to Case 2, following sources state that a new physical channel other than PRDCH should not be used to carry broadcast information.

- [OPPO] It is not necessary to introduce a new physical channel other than PRDCH to convey the broadcast information. There is no need to introduce a new physical channel other than PRDCH because the PRDCH was already used to transmit A-IoT paging message in Rel-19 A-IoT and the A-IoT paging message is also a kind of broadcast information especially when Paging ID Presence Indication field indicates Paging ID field is absent.

- [Ericsson] Using PRDCH to carry broadcast information could reduce the design complexity and specification impact. The broadcast information should be transmitted periodically in PRDCH with fixed parameters and the periodicity could be different from the periodic sync signal.

- [CATT] If the transmission parameters for L1 control information are indicated by broadcast information, a new physical channel other than PRDCH must be defined to carry the broadcast information including the configurations for L1 control information. Otherwise, if the broadcast information carried by PRDCH schedules a L1 control information, there will be a chicken-and-egg problem in the scheduling relationship between L1 control information and PRDCH. The introduction of a new channel will increase the complexity of R2D reception and relative standardization work.

- [Xiaomi] PRDCH in case B1 is capable to contain the single part of broadcast information.

- [NEC] PRDCH is used to carry the broadcast information, no need to introduce new channel(s) for R2D. For the channel to carry this broadcast information, we prefer to reuse PRDCH since the coding/repetition schemes for broadcast information may not be quite different, and no specific RS will be designed for the broadcast information. Furthermore, keep only one channel for each of R2D or D2R would maintain the simplest design for Rel-20 A-IoT.

- [Fraunhofer] PRDCH is used to carry the broadcast information. From a device perspective, supporting multiple channels undermines the simplicity benefits targeted for A-IoT. Furthermore, the separation of MIB-like and SIB1-like information does not provide a clear functional advantage for the devices, given the limited system information needs.

| Company | Y/N | Comments |
| --- | --- | --- |
| LG | Y |  |
| CMCC |  | As the FL copied our proposal above, our proposal is MIB-like broadcast information can be carried by a separate PRDCH or a new physical channel dedicated to MIB-like information. Both are OK, so the observation needs to be modified, “For Case 2, sources (Huawei, Spreadtrum, OPPO, Ericsson, CATT, CMCC, Xiaomi, vivo, LGE, China Telecom, Fraunhofer) state that PRDCH should be used to carry the MIB-like broadcast information, while source (CMCC) states that are also open to use a new physical channel dedicated to MIB-like information should be used. During the study, following observations were provided. ......” |
| Futurewei |  | Our document contained “Whether the MIB-like information is carried in its own channel can follow the concept of the PBCH, which is a separate channel for the MIB. This separate channel can simplify the specifications.” - Please add Futurewei to the sources in the text provided by CMCC - Please change case 2 to case B2 |
| Huawei, HiSilicon |  | For our observation, we prefer to reuse the same words as what we had proposed in our TDoc, and have made the correction below: [Huawei] Firstly, we do not see the PRDCH is used to carry the MIB-like broadcast information as there is no additional motivation to define a separate or new channel like NR SSB, in which 8 information bits are carried in the physical layer payload. Secondly, carrying the MIB-like broadcast information on an additional channel would increase the workload in terms of specification work for the WI. |
| CATT | N | We don’t support Case2. Please remove CATT from the proposal. |
| China Telecom | Y | Fine. |

DO-A

Previous agreements

Agreement (RAN1#122)

Study the following aspects to support DO-A traffic (periodic and event-triggered) for Device 2b and for Device C in Rel-20.

- Timing and frequency synchronization for DO-A transmission

- Resource allocation/determination method for DO-A transmission

- Configuration/signaling of necessary information

Agreement (RAN1#122bis)

For resource allocation method for the first D2R transmission for DO-A for Device 2b and for Device C, study following options considering feasibility:

- Option 1: Periodic D2R resource allocation

- Option 2: Aperiodic D2R resource allocation

Agreement (RAN1#122bis)

For providing necessary information for the first D2R transmission for DO-A for Device 2b and for Device C, study at least following options considering feasibility:

- Option 1: necessary information provided by broadcast information

- Option 2: necessary information provided by a paging-like R2D message

- Option 3: necessary information derived from a synchronization signal and provided by broadcast information

Agreement (RAN1#122bis)

Study whether necessary information for the first D2R transmission for DO-A for Device 2b and for Device C is provided periodically and/or aperiodically by Reader.

Agreement (RAN1#122bis)

For the content of the first D2R transmission for DO-A for Device 2b and for Device C, study the following options:

- Option 1: Msg1-like content

- Option 2: DO-A data payload

  - FFS: with or without device ID

- Option 3: DO-A scheduling request

Agreement (RAN1#123)

For the 3 options for the content of the first D2R transmission for DO-A for Device 2b and for Device C, study the following aspects:

- Recommended scenarios (event-triggered or periodic)

- Subsequent transmission(s)/reception(s)/procedure(s) if any after the first D2R transmission for DO-A

- Other aspects

Agreement (RAN1#123)

For providing necessary information for the first D2R transmission for DO-A for Device 2b and for Device C, following options are considered feasible.

- Option 1: necessary information provided by broadcast information

- Option 2: necessary information provided by a paging-like R2D message

- Option 3: necessary information derived from a synchronization signal and provided by broadcast information

For providing necessary information for the first D2R transmission for DO-A for Device 2b and for Device C, study following option considering feasibility and applicability

- Option 4: necessary information provided by device-specific R2D transmission

Views on Option 1/2/3 for the content of the first D2R transmission for DO-A

Previous agreements

Agreement (RAN1#122bis)

For the content of the first D2R transmission for DO-A for Device 2b and for Device C, study the following options:

- Option 1: Msg1-like content

- Option 2: DO-A data payload

  - FFS: with or without device ID

- Option 3: DO-A scheduling request

Agreement (RAN1#123)

For the 3 options for the content of the first D2R transmission for DO-A for Device 2b and for Device C, study the following aspects:

- Recommended scenarios (event-triggered or periodic)

- Subsequent transmission(s)/reception(s)/procedure(s) if any after the first D2R transmission for DO-A

- Other aspects

Work plan for RAN1#124 meeting (R1-2509614)

- DO-A - For Option 1/2/3 for the content of the first D2R transmission for DO-A - recommended scenarios (event-triggered or periodic) - subsequent transmission(s)/reception(s)/procedure(s) if any after the first D2R transmission for DO-A - other aspects - Timing and frequency synchronization for DO-A - Resource allocation/determination for DO-A - Necessary information and its provisioning (Option 1/2/3)

Summary

Preference for Option 1/2/3 for the content of the first D2R transmission for DO-A

- Option 1: OPPO, Ericsson, CATT, CMCC, NEC, China Telecom, InterDigital, Quectel, IIT

- Option 2: Spreadtrum, Apple, InterDigital, Sony, TCL (for periodic)

- Option 3: NEC, Panasonic, Sharp, TCL (for event-triggered), IIT

- [H] Proposal 3.2.1a-1 (change Content type 1/2/3  option 1/2/3 (Done!), vivo’s preference to be included, QC’s preference to be captured from the Table in Proposal 3.2.1b-1, soften the wording)

On the support for the content of the first D2R transmission for DO-A traffic, capture the following in TR 38.769:

For the content of the first D2R transmission for DO-A traffic, following observations were provided. To summarize, Option 1 receives support from sources (OPPO, Ericsson, CATT, CMCC, NEC, China Telecom, InterDigital, Quectel, IIT, Samsung, Huawei, Xiaomi, Qualcomm, Lenovo), Option 2 is supported by sources (Spreadtrum, Apple, InterDigital, Sony, TCL, Samsung, Xiaomi, Lenovo), and Option 3 is supported by sources (NEC, Panasonic, Sharp, TCL (for event-triggered), IIT, ZTE, Xiaomi, Qualcomm, CATT, Lenovo0).

- [Spreadtrum] For the content of the first D2R transmission for DO-A for Device 2b and for Device C, DO-A data payload should be supported. Options 1 and option 3 mean that device needs to perform random access first and then send DO-A data payload according to the reader's schedule. For some urgent DO-A traffic, option 1 and option 3 have relatively high latency, so option 2 is more suitable.

- [OPPO] Msg1-like content should be considered as the baseline for the content of the first D2R transmission in DO-A traffic. Option 1 can strike a good balance between resource efficiency and collision resolution performance.

- [Ericsson] For the content of the first D2R transmission for DO-A for Device 2b and for Device C, consider only Option 1 (Msg1-like content) in Rel-20. For the content of the first D2R transmission for DO-A for Device 2b and for Device C, Option 2 (DO-A data payload) and Option 3 (DO-A scheduling request) may necessitate introduction of new procedures and is unnecessarily over-optimizing resource allocation to cater to specific use cases with DO-A traffic.

- [CATT] Msg1-like content for DO-A SR should be used for the content of the first D2R transmission for DO-A. Based on polling mechanism, the content of the first D2R transmission of DO-A for Device 2b/C is the response information to polling information, which is used to indicate whether the device needs to transmit DO-A services.

- [CMCC] Support Option 1:Msg1-like content as the first D2R transmission for DO-A for Device 2b and for Device C. Option 1:Msg1-like content as the first D2R transmission for DO-A for Device 2b and for Device C, is more efficient, due to the following aspects: 1) It is part of the procedure of option3, and a pre-procedure for option2, which means it has to be supported. 2) The overhead of ROs can be adjust by Reader based on traffic load and number of devices. The collision handling is simple. 3) It applies to both periodic and event-triggered service. And for high event-triggered rate, the overhead can be similar or smaller than option 3, which require frequent paging transmission. For low event-triggered rate, the RO resources can be sparse to reduce overhead.

- [NEC] RAN1 can further consider Option 1 and Option 3 for the first D2R transmission of DO-A during WI phase. The two options do not mutually exclusive, e.g., Option 1 is applied for periodic DO-A traffic and Option 3 is applied for event-trigger DO-A traffic.

- [Panasonic] The first DO-A transmissions could carry Msg1-like content or a scheduling request. Among these two, the scheduling request could provide more robust detection using a common sequence for all devices served by a reader.

- [Apple] Support DO-A data payload (Option 2) as the baseline for first D2R transmission content. For short messages typical in ambient IoT (sensor readings, event notifications), Option 2 (direct data payload) minimizes latency and signaling overhead. Including device ID enables acknowledgment if needed. For sporadic event-triggered traffic with low device density, collision probability remains manageable.

- [Sharp] The main problem of this approach (Option 1) is the need for excessive D2R resources configured for “Msg1”, and the lack of flexibility in supporting a mix of DO-A traffic and DO-DTT traffic, or both periodic and even-triggered DO-A traffic. Another approach to support DO-A traffic is to allow devices to trigger A-IoT paging (which can in turn trigger CBRA by mostly reusing the design in Rel-19) via an autonomously triggered “first D2R transmission” such that collision of the “first D2R transmission” from different devices dose not matter as the “first D2R transmission” does not serve any purpose other than triggering A-IoT paging; timing for the “first D2R transmission” can thus be quite flexible, e.g. it can be any time within a time window (e.g. following an R2D transmission).

- [Sony] Many use cases only require the A-IoT device to transmit a small amount of data per DO-A occasion and do not benefit from larger DO-A message sizes. For limited DO-A message sizes, option 2, data payload in the initial DO-A message has advantage over the other approaches since it reduces signalling overhead and system complexity.

- [Quectel] For the content in the first D2R transmission for DO-A traffic for device 2b/C, support the content of the first D2R transmission for DO-A is Msg1-like content, i.e., Option1.

- [TCL] For periodic DO-A traffic, Option 1(Msg1-like content) might incur additional costs and is less efficient than Option 2 (DO-A data payload). For non-periodic DO-A traffic, consider SR-like signaling to requests subsequent resources for the DO-A operation.

- [IIT] The first D2R transmission for DO-A for Device 2b and for Device C contains either Msg1-like content, a DO-A scheduling request, or a combination of these two options.

- [Samsung] For event-triggered DO-A transmission, the device randomly selects one resource from the configured periodic resource and initiates DO-A transmission via RA. For periodic DO-A transmission, the device transmits DO-A data directly using the configured periodic resource, without RA.

- [Xiaomi] states that 3 options can work based on different definitions of DO-A procedures, and the whole procedure of DO-A should be defined first, and the content of the first D2R transmission for DO-A can be determined correspondingly.

- [Huawei] Supports the first D2R transmission contain Msg1-like content (option 1) because by using such a first D2R transmission, it can follow the same procedure as in Rel-19, where on receiving the Msg1-like D2R transmission, the reader can transmit the random ID response message and continue with the same CBRA procedure defined in Rel-19. This would also allow for the contention resolution mechanism defined in Rel-19 to be used, where the random ID response message echoes the RN16 transmitted in Msg1.

- [ZTE] For Option 3, the network only needs to reserve a single DO-A resource per period, minimizing reserved resource overhead. Multiple devices can transmit the same access request sequence simultaneously in the same DO-A resource without resource-collision impact. If no access request is detected, the reader does not transmit a DO-A-specific paging message, thereby reducing both power consumption and unnecessary resource usage.

| Company | Y/N | Comments |
| --- | --- | --- |
| Samsung |  | In our contribution, we highlighted that the study scope for DO-A should not be limited to the first D2R transmission only, but should also include subsequent D2R transmissions. Therefore, for the first D2R transmission in particular, our views are as follows, and we would like them to be captured in the TR: [Samsung] stated that both Option 1 and Option 2 should be considered. Option 1 can be used for device access to the network, where a device performs a Msg1-like transmission using DO-A traffic to achieve network registration. Option 2 can be used by a device connected to the network to perform periodic or event-triggered DO-A reporting using DO-A traffic. |
| LG | Y |  |
| Xiaomi |  | In our contribution, based on analysis, we observed that “For the content of the first D2R transmission for DO-A, 3 options can work based on different definitions of DO-A procedures”. We propose that the whole procedure of DO-A should be defined first, and the content of the first D2R transmission for DO-A can be determined correspondingly. We would like that Xiaomi’s views can be captured in the TR as: [Xiaomi] states that 3 options can work based on different definitions of DO-A procedures, and the whole procedure of DO-A should be defined first, and the content of the first D2R transmission for DO-A can be determined correspondingly |
| Huawei, HiSilicon |  | Our observations from our TDoc have not been included in the proposal. Added them and our company name to the relevant supporting set of companies. On the support for the content of the first D2R transmission for DO-A traffic, capture the following in TR 38.769: For the content of the first D2R transmission for DO-A traffic, following observations were provided. To summarize, Content type 1 receives support from sources (OPPO, Ericsson, CATT, CMCC, NEC, China Telecom, InterDigital, Quectel, IIT, Huawei), Content type 2 is supported by sources (Spreadtrum, Apple, InterDigital, Sony, TCL), and Content type 3 is supported by sources (NEC, Panasonic, Sharp, TCL (for event-triggered), IIT). [Huawei] Supports the first D2R transmission contain Msg1-like content (option 1) because by using such a first D2R transmission, it can follow the same procedure as in Rel-19, where on receiving the Msg1-like D2R transmission, the reader can transmit the random ID response message and continue with the same CBRA procedure defined in Rel-19. This would also allow for the contention resolution mechanism defined in Rel-19 to be used, where the random ID response message echoes the RN16 transmitted in Msg1. |
| ZTE, Sanechips |  | We also have preference on Option 3 and based on our contribution R1-2600453, we have the following observation: For Option 3(corresponding to option 3b in Table 1 in our Tdoc), the network only needs to reserve a single DO-A resource per period, minimizing reserved resource overhead. Multiple devices can transmit the same access request sequence simultaneously in the same DO-A resource without resource-collision impact. If no access request is detected, the reader does not transmit a DO-A-specific paging message, thereby reducing both power consumption and unnecessary resource usage. |
| CATT | Y | OK with the proposal. |
| TCL | Y |  |
| China Telecom | Y | Fine |

- [H] Proposal 3.2.1b-1

On the content of the first D2R transmission for DO-A traffic, capture the following in TR 38.769:

Study results in terms of recommended scenarios, i.e. event-triggered or periodic, and the subsequent transmissions/receptions/procedures (if any), and other aspects are summarized in Table 6A.4-x.

Table 6A.4-x: Study of the content of the first D2R transmission for DO-A traffic

|  | Recommended scenarios (event-triggered or periodic) | Subsequent transmission(s)/reception(s)/procedure(s) | Other aspects |
| --- | --- | --- | --- |
| Option 1 Msg1-like content | Both event-triggered and periodic: FUTUREWEI, Huawei, Ericsson, Nokia, CMCC, Xiaomi, ZTE, vivo, Qualcomm Event-triggered: LGE, ETRI, DCM, Samsung Periodic: Lenovo | [FUTUREWEI] Procedure after first transmission: The reader transmits a Msg2-like transmission in response to successful reception of the first transmission. The Msg2-like transmission can provide scheduling parameters (e.g., size of payload). [Huawei] The subsequent procedure follows the CBRA procedure defined in Rel-19, where a random ID response message transmitted by the reader echoes the RN16 for contention resolution. [Spreadtrum] Device needs to perform random access first and then send DO-A data payload according to the reader's schedule. [OPPO] The reader transmits a response to each of identified the Msg1-like transmission and allocates subsequent D2R transmission resource to convey DO-A data and/or further identification information of the A-IoT device accordingly. The A-IoT device transmits the DO-A data and/or further identification information on the allocated D2R transmission resource. Reader transmits an echoed identification to the corresponding A-IoT device for acknowledgement. [Ericsson] Option 1 is essentially reusing the random access (RA) messages specified in Rel-19 for DT and DO-DTT also for DO-A, but without being triggered by a preceding R2D transmission (paging message/Msg0), i.e., the device accesses the reader through Msg1 triggered by DO-A traffic. [CATT] The reader will respond to the detected Msg1-like transmission (containing DO-A SR) and send out the resource allocation on PRDCH for DO-A traffic transmission. The A-IoT device transmits the DO-A traffic according to the resource allocation from reader. [Nokia] Contention-based random access procedure, can be based on the Rel-19 procedure. [CMCC] Random access procedure, e.g.Msg.2/3/4, and DOA data transmission [Xiaomi] A random access procedure is needed before the transmission of exact sensing result(s). [ZTE] CBRA defined in Rel-19 [vivo] (1) msg2 response to msg1 transmission, (2) msg3 transmission (3) msg4 for contention resolution. [LGE] 1) Devices transmit Msg1 like content in a contention-based manner. 2) Successful devices are granted D2R resources and transmit DO-A payload [NEC] Devices monitor Msg2 message for resource allocation to transmit D2R Upper Layer Data which contains DO-A traffic. [China Telecom] The device acquire ACK message contained in the MSG2 as well as other necessary information, such as the scheduling information for next D2R transmission. [Apple] Reuses random access procedure, device transmits preamble to request resources. [Sharp] Rel-19 CBRA [ETRI] The data payload is transmitted in the form of DO-DTT after the random access procedure. [Lenovo] CBRA-like procedure. Applying CDM to Msg1 transmission increases access capacity by allowing more devices to transmit simultaneously. [DCM] 1) Msg.2 in response to Msg.1. 2) Msg.3 with DO-A data, and ID for contention resolution (if sequence-based Msg.1 is applied). 3) contention resolution (if sequence-based Msg.1 is applied). [Qualcomm] To receive Msg2 for the following Msg3, Msg4 similar as CBRA [Quectel] The same procedure for random access in Rel-19 can be adopted, that device receives Msg2 and transmits Msg3 carrying DO-A data payload. [Samsung] RA-based access/registration procedure | [FUTUREWEI] - Other aspects: Mimicking contention-based access can simplify the procedure after first transmission. [Huawei] - Reusing the random-access procedure defined in Rel-19 without triggering message reduces the standardization effort. - If the resource allocation information is also periodic, the device can determine the required resources to transmit Msg1 irrespective of the transmission being periodic or event triggered without any delay or increased latency. - This periodicity can also be configurable based on the use cases to avoid unnecessary wastage of resources. [CMCC] - Pre-step: Periodic random access resource is configured by broadcast information. This option is anyway needed either for option3 after paging message or for option2 before periodic D2R resource is configured. [vivo] - The first D2R transmission resources is indicated by broadcast information for Msg 1 transmission. - A CBRA based procedure by using sequence-based msg1 as the first D2R transmission content. - Compared to R19 CBRA based random access procedure with additional contention resolution msg4, less specification effort will be. [LGE] - Inventory follows a slotted ALOHA based procedure - Higher latency due to inventory before DO-A - No need for prior inventory [NEC] - For Option 1, DO-A and DO-DTT can use unified procedure, i.e., starting with CBRA and then performing D2R transmission for the traffic. It is possible to share resources for the two types of traffic. [Lenovo] - The collision between Msg1 transmissions can be resolved by echoing Msg1 content within Msg2. - Applying CDM to Msg1 transmission increases access capacity by allowing more devices to transmit simultaneously. [Qualcomm] - The reader does not know which device to access and can configure a set of Msg1-like resources for random access to support large capacity. - The CBRA supported in Rel19 can be used as starting point with enhanced collision handling. - However, for event-triggered DO-A, which may have tighter latency requirements than periodic DO-A, the reader must allocate resources with shorter periodicity. This approach may lead to significant overhead and inefficient resource utilization, particularly when events occur infrequently. [Samsung] - The device initiates the procedure using DO-A traffic. - Contention-based resources are assumed. [vivo] - For procedure 1 as provided in the column of Subsequent transmission(s)/reception(s)/procedure(s) for option1, it is a reliable way for a device to access to reader and then perform device-specific transmission. And compared with procedure 1, procedure 4 (i.e., SR as the first D2R content to trigger R2D command and the R2D command schedules msg1-like resource for CBRA) which has additional two steps (scheduling request (SR) and R2D command) is unnecessary and overdesigned. [DCM] - It requires the highest R2D signaling overhead. |
| Option 2 DO-A data payload | Both event-triggered and periodic: FUTUREWEI, Xiaomi, Samsung, Xiaomi Periodic: CMCC, ZTE, vivo, LGE, ETRI, Lenovo, DCM, Qualcomm None: Huawei | [FUTUREWEI] Procedure after first transmission: The reader transmits a Msg2-like transmission in response to successful reception of the first transmission. Instead of the random number, the CRC of the first transmission can be used. The Msg2-like transmission can provide scheduling parameters (e.g., size of additional payload resources). [Huawei] CFA-like procedure [CATT] The content of subsequent transmission/reception after the first D2R transmission is to indicate the desired resource payload of DO-A data. [Nokia] None [CMCC] Possible feedback and re-transmission [Xiaomi] None [ZTE] CFA defined in Rel-19 [vivo] None [LGE] 1) Reader allocates device specific D2R resource. 2) Device directly transmits DO-A data payload (contention free). [ETRI] A collision resolution mechanism may be considered [Lenovo] CFA-like procedure. Acknowledgment mechanism to mitigate collisions among DO-A data payloads and ensure reliability. [DCM] Contention resolution (ID for contention resolution is provided in the first D2R transmission for DO-A.) [Qualcomm] If Option 2 is for device-specific resources, to receive the acknowledgement for D2R reception, similar as for paging with CFA indication. If Option 2 is for non-device-specific resources, to receive the acknowledgement for D2R reception, similar as for paging with CFA indication. | [FUTUREWEI] - Other aspects: Procedure can mix physical layer and upper layer elements are the reader and device. Large payload sizes can lead to unused resources when no device has DO-A content to transmit while small payload sizes can lead to unequal error detection if multiple segments are needed. [Huawei] - Low resource utilization and resource wastage due to the reader having to allocate a large number of resources in the absence of a scheduling request, to attempt to minimize the collision probability and factor in the large DO-A payload sizes. - System performance deteriorates in the absence of a collision resolution mechanism when there are more devices wanting to transmit than available resources. [Nokia] - Option 2 is suitable and can be very efficient if the following specific conditions are met: - 1. Payload size is small and has an upper bound that is known beforehand (e.g. fixed size for a sensor measurement report). - 2. Predictable message rate (e.g. periodic traffic generated by a known number of devices) - If the conditions are not met, then this is likely to be extremely inefficient since the network would have to vastly over-dimension the resources for the first D2R transmission. [CMCC] - Pre-step: Periodic D2R resource is configured to device. - If device common D2R resource is configured, collision may happen for DOA reporting. - If device dedicated D2R resource is configured, the resource overhead is large. - For this case, the device has to keep a connection state to maintain D2R synchronization, which means high power consumption for long interval, e.g. 5~15 minutes traffic interval in TR22.840. [vivo] - The first D2R transmission resources is indicated by broadcast information for CG resource for DOA. - A CFRA based procedure by using DoA data as the first D2R transmission content. - For whether device ID should be transmitted together with DoA data, for a reader to distinguish the DoA data from multiple devices, device ID needs to be transmitted together with DoA data. [LGE] - Suitable for low latency or urgent DO-A delivery - Requires prior inventory [Lenovo] - To enhance access capacity, the number of resources indicated by the R2D paging message must be increased. - Applying acknowledgment mechanism to mitigate collisions among DO-A data payloads and ensure reliability. [Sony] - Supporting a grant-free channel access for DO-A traffic for active devices in Rel-20 reduces overhead and latency. In our view, a grant-free D2R, where the device is allowed to initiate its transmission on resources it chooses itself is a viable choice. [Qualcomm] If Option 2 is for device-specific resources: - The reader already knows the device and can use device-specific R2D transmission to trigger DO-A data reporting. - CFA with fewer steps reduces latency and prevents collisions, but it also results in large overhead and low capacity. - The CFA supported in Rel19 can be used as starting point. If Option 2 is for non-device-specific resources: - The reader does not know the device and configure a set of MsgA-like resources for random access to support large capacity. - Two-step CBRA reduces latency but has higher probability of collisions than Msg1-like random access. - Two-step CBRA, not supported in Rel19 AIoT, may have specification impact. [Samsung] - Depending on whether the DO-A traffic is periodic data transmission or event-triggered transmission, the resources may be contention-free or contention-based. - For periodic reporting, DO-A traffic is transmitted over configured (dedicated) D2R resources. For event-triggered reporting, DO-A traffic is transmitted using contention-based resources. [vivo] - For procedure 2 as provided in the column of Subsequent transmission(s)/reception(s)/procedure(s) for option 2, the target scenario is latency-insensitive periodic DoA traffic with sparse transmission period e.g., in minute level. Due to the low data collision probability, using broadcast information to directly schedule CG-like resource for data transmissions will be efficient. - Regarding device ID under option 2, for a reader to distinguish the DoA data from multiple devices, device ID needs to be transmitted together with DoA data. [Xiaomi] - None For Option 2, DO-A data payload is transmitted by the first D2R. It is simple that device can directly use the configured resource(s) for sensing result transmission whenever it has, and it simplifies the whole procedure by eliminating random access from DO-A. [DCM] - It requires the lowest R2D signal overhead and shortest latency of DO-A procedure. On the other hand, it requires the largest size of pre-allocated periodic D2R resources and may lead to waste of the pre-allocated resources if device does not have frequent DO-A traffic to transmit considering event-triggered DO-A traffic. |
| Option 3 DO-A scheduling request | Both event-triggered and periodic: Huawei, Nokia, CMCC, Xiaomi, ZTE, Sharp Event-triggered: FUTUREWEI, vivo, LGE, DCM, TCL, Qualcomm Periodic: Lenovo | [FUTUREWEI] Procedure after first transmission: Procedure after first transmission: The reader transmits a paging-like transmission for inventory in response to determining the first transmission contains a scheduling request. Instead of the random number, the CRC of the first transmission can be used. [Huawei] An additional D2R message, i.e., DO-A scheduling request, and a corresponding additional R2D paging message, i.e., DO-A specific paging message carrying the resource allocation information, are required, following which CBRA procedure can be used. [Spreadtrum] Device needs to perform random access first and then send DO-A data payload according to the reader's schedule. [Nokia] It depends on exact usage of the resource for the first D2R transmission. If that resource is e.g., dedicated to a device then no contention resolution is needed, the reader provides a resource grant for a subsequent D2R transmission. Otherwise, contention resolution is required (e.g. reader transmits a paging message which indicates paging only for devices with DO-A to transmit, followed by the legacy CBRA). [CMCC] Paging-like R2D transmission to allocate random access resource, random access procedure, e.g., Msg.2/3/4, and DOA data transmission. [Xiaomi] It requires additional procedures and extending the latency to transmit sensing result. [ZTE] DO-A specific Paging + Msg1 + CBRA defined in Rel-19 [vivo] (1) An R2D command to trigger CG-like D2R resource for DOA data transmission, then, (2) following DoA data transmission. [LGE] 1) Reader allocates common D2R resource for SR. 2) Upon SR reception, reader grants D2R resource. 3) Device transmits DO-A data payload. [NEC] Devices monitor A-IOT paging message for Msg1 resource allocation and then follow the Rel-19 CBRA procedure to transmit D2R Upper Layer Data which contains DO-A traffic. [Panasonic] Once the reader detects the SR, it can initiate the random-access procedure, and data would be transmitted like the DO-DTT procedure. [Sharp] A-IoT paging triggers CBRA by mostly reusing the design in Rel-19. Collision of the “first D2R transmission” from different devices does not matter as the “first D2R transmission” does not serve any purpose other than triggering A-IoT paging. [Lenovo] The R2D paging message triggers the subsequent D2R transmission for scheduling requests. Following this, the R2D response message is used to indicate the resources allocated for the DO-A data payload. [DCM] 1) R2D schedules contention-based D2R data Tx, 2) DO-A data and ID for contention resolution. 3) Contention resolution. [TCL] After the reader receives the SR-like, the DO-A procedure falls back to the inventory procedure. [Qualcomm] To receive paging for the following Msg1, Msg2, Msg3, Msg4 similar as CBRA | [FUTUREWEI] - Other aspects: Procedure can mix physical layer and upper layer elements are the reader and device. Large payload sizes can lead to unused resources when no device has DO-A content to transmit while small payload sizes can lead to unequal error detection if multiple segments are needed. A small number of devices can improve resource efficiency for contention. [Huawei] - Increase in signalling overhead due to the additional DO-A scheduling request and the corresponding DO-A specific paging message required to be provided by the reader to each device that requested resources, as compared to using Msg1. [CMCC] - Pre-step: Periodic scheduling request resource is configured by broadcast information. - If SR resource is device common, it is hard for Reader to determine how many RO resource to be allocated by paging. - If the scheduling request rate is high for devices, following paging overhead will be large, the overhead reduction benefit compared to option1 will be reduced. - The delay is larger than option1. [vivo] - The SR resources is indicated by broadcast information to avoid the CG-like resource overhead for directly data transmission. [LGE] - Only small D2R resource needed for SR - Multiple SRs may cause DO-A collisions - Randomized SR resource or post grant inventory may mitigate collisions [NEC] - Multiple devices can share resource for the scheduling request transmission, which can improve the resource efficiency for DO-A first D2R transmission and more applicable to the event-trigger type of traffic. [Lenovo] - Study the design of R2D paging message, D2R for scheduling request, and R2D response message. - Frequency separation between periodic and aperiodic transmissions helps reduce both the interval for periodic transmissions and their associated transmission latency [vivo] - To avoid the CG-like resource overhead of procedure 2 as provided in the column of Subsequent transmission(s)/reception(s)/procedure(s) for option 2, procedure 3 as provided in the column of Subsequent transmission(s)/reception(s)/procedure(s) for option 3 with broadcast information to schedule SR resource instead of CG-like resource could be useful. Once SR is detected by reader, it can schedule the CG-like resource for data transmission by using R2D command. [Qualcomm] - The reader does not know which device to access and can use shared common resources for different devices to send same SR without random access. - Only if the reader detects the SR, it will send paging to configure additional resources for any devices who transmits the DO-A SR, where the CBRA supported in Rel19 can be used as starting point after receiving the SR. - For event-triggered DO-A, it can save the overhead than Msg1-like D2R (Option 1) when events occur infrequently. [DCM] - It requires the smallest size of pre-allocated periodic D2R resources. |

| Company | Y/N | Comments |
| --- | --- | --- |
| Samsung |  | Based on our contribution, we would like to have the following table captured in the TR: Recommended scenarios (event-triggered or periodic); Subsequent transmission(s)/reception(s)/procedure(s); Other aspects. Option 1 Msg1-like content; Event-triggered; RA-based access/registration procedure; - The device initiates the procedure using DO-A traffic. - Contention-based resources are assumed.. Option 2 DO-A data payload; Periodic and event-triggered; For periodic reporting, DO-A traffic is transmitted over configured (dedicated) D2R resources. For event-triggered reporting, DO-A traffic is transmitted using contention-based resources.; Depending on whether the DO-A traffic is periodic data transmission or event-triggered transmission, the resources may be contention-free or contention-based.. |
| vivo |  | Supplement our observations below into the other aspect column of the above table for each recommended procedure corresponding to each option. Just for better understanding and reading without changing the original observations, we further add the explanation in red. Proposal 14: Consider the following three potential procedures by using different contents of the first D2R transmission of DoA traffic: <<Add the following views into other aspects of option1.>> Observation 10: For procedure 1 as provided in the column of Subsequent transmission(s)/reception(s)/procedure(s) for option1, it is a reliable way for a device to access to reader and then perform device-specific transmission. And compared with procedure 1, procedure 4 (i.e., SR as the first D2R content to trigger R2D command and the R2D command schedules msg1-like resource for CBRA ) which has additional two steps (scheduling request (SR) and R2D command) is unnecessary and overdesigned. <<Add the following views into other aspects of option2.>> Observation 11: For procedure 2 as provided in the column of Subsequent transmission(s)/reception(s)/procedure(s) for option 2, the target scenario is latency-insensitive periodic DoA traffic with sparse transmission period e.g., in minute level. Due to the low data collision probability, using broadcast information to directly schedule CG-like resource for data transmissions will be efficient. Observation 13: Regarding device ID under option 2, for a reader to distinguish the DoA data from multiple devices, device ID needs to be transmitted together with DoA data. <<Add the following views into other aspects of option3.>> Observation 12: To avoid the CG-like resource overhead of procedure 2 as provided in the column of Subsequent transmission(s)/reception(s)/procedure(s) for option 2, procedure 3 as provided in the column of Subsequent transmission(s)/reception(s)/procedure(s) for option 3 with broadcast information to schedule SR resource instead of CG-like resource could be useful. Once SR is detected by reader, it can schedule the CG-like resource for data transmission by using R2D command. |
| LG | Y |  |
| Qualcomm |  | For Option 3, could you also add our input as in our tdoc R1-2601262 Option 3 DO-A scheduling request; [Qualcomm] Event-triggered; [Qualcomm] To receive paging for the following Msg1, Msg2, Msg3, Msg4 similar as CBRA; [Qualcomm] - The reader does not know which device to access and can use shared common resources for different devices to send same SR without random access. - Only if the reader detects the SR, it will send paging to configure additional resources for any devices who transmits the DO-A SR, where the CBRA supported in Rel19 can be used as starting point after receiving the SR. - For event-triggered DO-A, it can save the overhead than Msg1-like D2R (Option 1) when events occur infrequently.. |
| Xiaomi |  | For option 2 as discussed in our contribution, we would like to ask to add our views in the table. Recommended scenarios (event-triggered or periodic); Subsequent transmission(s)/reception(s)/procedure(s); Other aspects. Option 2 DO-A data payload; Periodic and event-triggered; [Xiaomi] None For Option 2, DO-A data payload is transmitted by the first D2R. It is simple that device can directly use the configured resource(s) for sensing result transmission whenever it has, and it simplifies the whole procedure by eliminating random access from DO-A.. |
| Huawei, HiSilicon | Y |  |
| ZTE, Sanechips | Y |  |
| CATT | Y | OK with the proposal. |
| TCL | Y |  |
| DOCOMO |  | We would like to add following of our observation to the “other aspect” column. For option 1: - [DCM] it requires the highest R2D signaling overhead. For option 2 - [DCM] it requires the lowest R2D signal overhead and shortest latency of DO-A procedure. On the other hand, it requires the largest size of pre-allocated periodic D2R resources and may lead to waste of the pre-allocated resources if device does not have frequent DO-A traffic to transmit considering event-triggered DO-A traffic. For option 3 [DCM] it requires the smallest size of pre-allocated periodic D2R resources. |

Timing and frequency synchronization for DO-A

Previous agreements

Agreement (RAN1#122)

Study the following aspects to support DO-A traffic (periodic and event-triggered) for Device 2b and for Device C in Rel-20.

- Timing and frequency synchronization for DO-A transmission

- Resource allocation/determination method for DO-A transmission

- Configuration/signaling of necessary information

Work plan for RAN1#124 meeting (R1-2509614)

- DO-A - For Option 1/2/3 for the content of the first D2R transmission for DO-A - recommended scenarios (event-triggered or periodic) - subsequent transmission(s)/reception(s)/procedure(s) if any after the first D2R transmission for DO-A - other aspects - Timing and frequency synchronization for DO-A - Resource allocation/determination for DO-A - Necessary information and its provisioning (Option 1/2/3)

Summary

Timing and frequency synchronization for DO-A is achieved by

- R2D periodic synchronization signal (Xiaomi, Sharp, ETRI, IIT)

- R2D periodic synchronization signal and/or CFO calibration signal (LGE, TCL)

- R2D periodic synchronization signal and CFO calibration signal (vivo)

- PRDCH transmission preceding the DO-A resources (FUTUREWEI)

- Periodic or semi-periodic PRDCH transmission (Samsung)

- [H] Proposal 3.2.2a

On timing and frequency synchronization for DO-A transmission, capture the following in TR 38.769:

Fo the time and frequency synchronization required for the device to transmit DO-A traffic, sources (Xiaomi, Sharp, ETRI, IIT, LGE, TCL, vivo) report that the time and frequency synchronization can be achieved by at least R2D periodic synchronization signal. Sources (LGE, TCL, vivo) further state that in addition to the R2D periodic synchronization signal, CFO calibration signal can also be used. Source (FUTUREWEI) reports that the time and frequency synchronization can be achieved by PRDCH transmission preceding the DO-A resources, while source (Samsung) states that it can be achieved by periodic or semi-periodic PRDCH transmission. Following observations were provided during the study.

- [FUTUREWEI] A PRDCH transmission precedes the DO-A resources providing timing and frequency synchronization for the first D2R transmission and can be used to mark the beginning of DO-A resources.

- [Xiaomi] To decrease the residual SFO, R2D periodic synchronization signal can be studied. For example, the R2D periodic synchronization signal can be inserted between adjacent DO-A indication messages or Msg0s.

- [vivo] The timing and frequency synchronization for DoA traffic is provided by periodic sync signal and CFO calibration signal.

- [LGE] A CFO calibration signal may need to be transmitted close to the DO‑A transmission occasion depending on whether the periodic sync signal includes a CFO calibration signal and on the period of the periodic sync signal.

- [Samsung] A PRDCH is periodically transmitted, which provides a basis for synchronization for DO-A transmission. This periodic PRDCH provides synchronization for DO-A transmission, system-information-like information for DO-A transmission, etc.

- [Sharp] For Device 2b and Device C with periodic DO-A traffic, a periodic synchronization signal is expected to precede the DO-A transmission.

- [ETRI] The device acquires time and frequency synchronization for DO-A by using the periodic synchronization signal.

- [TCL] Considering periodic synchronization signal/calibration signal to provide R2D synchronization and timing for DO-A traffic.

- [IIT] A periodic synchronization signal should be used to provide timing and frequency synchronization for DO-A traffic for device 2b and device C.

| Company | Y/N | Comments |
| --- | --- | --- |
| Samsung |  | We consider that synchronization for DO-A can be obtained using at least one of the elements included in a PRDCH transmission, such as a preamble, a synchronization signal, or broadcast information. |
| LG | Y |  |
| Qualcomm |  | Pls add our views as well: Fo the time and frequency synchronization required for the device to transmit DO-A traffic, sources (Xiaomi, Sharp, ETRI, IIT, LGE, TCL, vivo, Qualcomm) report that the time and frequency synchronization can be achieved by at least R2D periodic synchronization signal. |
| Lenovo |  | Please add our views as well: Fo the time and frequency synchronization required for the device to transmit DO-A traffic, sources (Xiaomi, Sharp, ETRI, IIT, LGE, TCL, vivo, Qualcomm, Lenovo) report that the time and frequency synchronization can be achieved by at least R2D periodic synchronization signal. |
| Huawei, HiSilicon |  | We would like to have our views captured here as well, based on our TDoc from the previous meeting: On timing and frequency synchronization for DO-A transmission, capture the following in TR 38.769: For the time and frequency synchronization required for the device to transmit DO-A traffic, sources (Xiaomi, Sharp, ETRI, IIT, LGE, TCL, vivo, Huawei) report that the time and frequency synchronization can be achieved by at least R2D periodic synchronization signal. Sources (LGE, TCL, vivo, Huawei) further state that in addition to the R2D periodic synchronization signal, CFO calibration signal can also be used. [Huawei] For initial frequency acquisition, the device uses a set of pre-defined sync rasters to blindly detect the periodic synchronization signal. On detecting the synchronization signal, it obtains coarse frequency synchronization and calibrates its clock accuracy, and on decoding the following MIB-like broadcast information carried on the PRDCH, it obtains the reader ID and SFN information. The device then detects the following CFO calibration signal to obtain fine frequency synchronization and calibrates the CFO/SFO to the residual level, e.g., 10 ppm, hence acquiring time and frequency synchronization. |
| Panasonic | Y | We mentioned in our tdoc that the period DO-A resources could be indicated by proceeding with PRDCH. Please add our view as: Source (FUTUREWEI, Panasonic) reports that the time and frequency synchronization can be achieved by PRDCH transmission preceding the DO-A resources |
| TCL | Y |  |
| China Telecom |  | We mentioned in our tdoc that using one of the three R2D messages as the broadcast information content, and indicate the common MSG1 resource that allow all types of devices to occupy and sent the first D2R transmission for DO-A, and we also mentioned that the broadcast information is a periodic transmission, hence we propose to add our company on the observation: On timing and frequency synchronization for DO-A transmission, capture the following in TR 38.769: Fo the time and frequency synchronization required for the device to transmit DO-A traffic, sources (Xiaomi, Sharp, ETRI, IIT, LGE, TCL, vivo) report that the time and frequency synchronization can be achieved by at least R2D periodic synchronization signal. Sources (LGE, TCL, vivo) further state that in addition to the R2D periodic synchronization signal, CFO calibration signal can also be used. Source (FUTUREWEI) reports that the time and frequency synchronization can be achieved by PRDCH transmission preceding the DO-A resources, while source (Samsung, China Telecom) states that it can be achieved by periodic or semi-periodic PRDCH transmission. Following observations were provided during the study. - [FUTUREWEI] A PRDCH transmission precedes the DO-A resources providing timing and frequency synchronization for the first D2R transmission and can be used to mark the beginning of DO-A resources. - [Xiaomi] To decrease the residual SFO, R2D periodic synchronization signal can be studied. For example, the R2D periodic synchronization signal can be inserted between adjacent DO-A indication messages or Msg0s. - [vivo] The timing and frequency synchronization for DoA traffic is provided by periodic sync signal and CFO calibration signal. - [LGE] A CFO calibration signal may need to be transmitted close to the DO‑A transmission occasion depending on whether the periodic sync signal includes a CFO calibration signal and on the period of the periodic sync signal. - [Samsung] A PRDCH is periodically transmitted, which provides a basis for synchronization for DO-A transmission. This periodic PRDCH provides synchronization for DO-A transmission, system-information-like information for DO-A transmission, etc. - [Sharp] For Device 2b and Device C with periodic DO-A traffic, a periodic synchronization signal is expected to precede the DO-A transmission. - [ETRI] The device acquires time and frequency synchronization for DO-A by using the periodic synchronization signal. - [TCL] Considering periodic synchronization signal/calibration signal to provide R2D synchronization and timing for DO-A traffic. - [IIT] A periodic synchronization signal should be used to provide timing and frequency synchronization for DO-A traffic for device 2b and device C. [China Telecom] A period PRDCH transmission that contains broadcast information will indicate the common MSG1 resource that allow all types of devices to occupy and sent the first D2R transmission for DO-A. |

Resource allocation/determination for the first D2R transmission for DO-A

Previous agreements

Agreement (RAN1#122)

Study the following aspects to support DO-A traffic (periodic and event-triggered) for Device 2b and for Device C in Rel-20.

- Timing and frequency synchronization for DO-A transmission

- Resource allocation/determination method for DO-A transmission

- Configuration/signaling of necessary information

Agreement (RAN1#122bis)

For resource allocation method for the first D2R transmission for DO-A for Device 2b and for Device C, study following options considering feasibility:

- Option 1: Periodic D2R resource allocation

- Option 2: Aperiodic D2R resource allocation

Work plan for RAN1#124 meeting (R1-2509614)

- DO-A - For Option 1/2/3 for the content of the first D2R transmission for DO-A - recommended scenarios (event-triggered or periodic) - subsequent transmission(s)/reception(s)/procedure(s) if any after the first D2R transmission for DO-A - other aspects - Timing and frequency synchronization for DO-A - Resource allocation/determination for DO-A - Necessary information and its provisioning (Option 1/2/3)

Summary

Views on Option 1 and Option 2

- Option 1: Periodic D2R resource allocation

- Option 2: Aperiodic D2R resource allocation

Preference for Option 1 and Option 2 for resource allocation method for the first D2R transmission for DO-A

- Option 1: Huawei, OPPO, Ericsson, CMCC, vivo, NEC, Panasonic, Samsung, Lenovo, DCM, Quectel, IIT, Fraunhofer,

- Option 2: CATT, Xiaomi,

- Option 1 and 2: ZTE, Apple, InterDigital, Qualcomm,

- No preference but observations: [LGE], China Telecom,

Views on resource determination

- [Spreadtrum] For determining the time domain of resource for DO-A, the end of periodic sync signal could be the reference point.

- [OPPO] Consider the ending of the A-IoT SSB and the single-tone sine wave in the A-IoT SSB as the reference point in time and frequency domain respectively for candidate resources allocation of DO-A traffic.

- [NEC] For resource allocation/determination method for DO-A transmission, the reference of the scheduling can be the end of the corresponding R2D sync signal or the PRDCH carrying broadcast information.

- [InterDigital] Periodic resource allocation with reference to an R2D periodic sync signal.

- [H] Proposal 3.2.3a

For resource allocation method for the first D2R transmission for DO-A for Device 2b and for Device C, capture the following in TR 38.769:

On whether the resource for the first D2R transmission for DO-A traffic should be periodic or aperiodic, following observations were provided. In summary, sources (Huawei, OPPO, Ericsson, CMCC, vivo, NEC, Panasonic, Samsung, Lenovo, DCM, Quectel, IIT, Fraunhofer) report that it should be periodic, while sources (CATT, Xiaomi) state that it should be aperiodic. Sources (ZTE, Apple, InterDigital, Qualcomm) state that periodic and aperiodic resource allocation should be supported. Sources (LGE, China Telecom) submitted observations without mentioning their preferences.

- [Huawei] The resource allocation information provided in the SIB-like broadcast information is periodic for both periodic and event-triggered D2R transmissions for DO-A. If the resource allocation information is provided aperiodically, it is possible that the number of resources indicated is less than the actual number of devices wanting to transmit D2R transmissions for DO-A, and is not aware of when the next SIB-like transmission containing the resource allocation information can be expected, resulting in a decreased access capability due to the limited resources.

- [OPPO] Because whether to transmit DO-A data is fully up to device, which is different from DT or DO-DTT where the device should transmit data whenever triggered by R2D. Thus, aperiodic D2R resource allocation for DO-A cannot timely adapt to the real DO-A transmission demand and will exacerbate the issue that the traffic of DO-A is available but lack of allocated D2R candidate resources.

- [Ericsson] For resource allocation method for Option 1 (Msg1-like content) for DO-A for Device 2b and for Device C, consider only periodic Msg1 resource allocation.

- [CATT] The polling-based mechanism for aperiodic resource allocation is an on-demand resource allocation and would allocate the resource efficiently by the reader polling the A-IoT device for query of any DO-A traffic arrival. Compared to aperiodic resource allocation, the periodic resource allocation for DO-A transmission would have a lot of resources not used and wasteful. Most of periodically configured resources for DO-A are not used when the device has no data to transmit.

- [CMCC] For resource allocation method for the first D2R transmission for DO-A for Device 2b and for Device C, support Option 1: Periodic D2R resource allocation. For Option 1, as long as device acquires cell synchronization and reads broadcast information, it can get the access resource allocation and initiate random access any time it has D2R traffic to report, this option is simple and more straightforward. Option 2: Aperiodic D2R resource allocation for the first D2R transmission for DO-A requires more standard effort and consumes more energy before device can send DOA traffic.

- [Xiaomi] For DO-A resources determination in Rel-20, resource pool-based method and Msg3-associated method can be studied. For resource pool-based method, R2D message carrying DO-A indication information can be sent periodically. Between two adjacent R2D messages, several DO-A occasions can be pre-allocated by the adjacent R2D message. For the device who has DO-A data to transmit, it can choose one of the DO-A occasions to send DO-A transmission. For Msg3-associated method, Msg0 can be sent periodically, and each DO-A procedure following an inventory procedure. After the inventory procedure, the R2D message carrying DO-A indication can be sent to allocate several DO-A occasions. For the device who has DO-A data to transmit, it can choose one of the DO-A occasions to send DO-A transmission.

- [ZTE] Option 2 (necessary information provided by a paging-like R2D message) and Option 3 (necessary information derived from a synchronization signal and provided by broadcast information) for providing necessary information for the first D2R transmission for DO-A support periodic and aperiodic DO-A resource allocation.

- [vivo] Pros of Option 1 (Periodic D2R resource allocation): 1) For periodic DoA traffic, periodic D2R resource is naturally needed. Otherwise, by using R2D message to trigger every D2R resource will cause heavy signaling overhead. 2) For event trigger DoA traffic type, since reader cannot be aware of when the urgent event will happen, providing a periodic D2R resource occasions will be more guaranteed. Cons of Option 2 (Aperiodic D2R resource allocation): 1) Aperiodic D2R resource cannot support periodic DoA traffic type. 2) A reader cannot know when the need for event-triggered DoA traffic will happen, so using aperiodic D2R resource will cause either traffic transmission latency or resource overhead.

- [LGE] For the resource allocation method for the first D2R transmission for DO-A, periodic allocation provides predictable resource availability but may waste resources when device activity is low, whereas polling-based aperiodic allocation improves efficiency by assigning resources only when DO-A traffic is detected.

- [NEC] For resource allocation/determination method for DO-A transmission, at least a subset of periodic signals can have associated PRDCH to carry the scheduling information for the first D2R transmission.

- [China Telecom] Support a SR scheduling mechanism for the device to report its data actively in the DO-A service. Support a DO-A data payload D2R transmission mechanism for the device to report its data actively in the DO-A service. Support a SR mechanism without access step (SR without MSG1) for the device to report its data actively in the DO-A service.

- [Panasonic] Support periodic resources for DO-A transmission within a valid time period, indicated by the reader or preconfigured for the device.

- [Samsung] For resource allocation method for the D2R transmission(s) for DO-A for Device 2b and for Device C, periodic D2R resource allocation is feasible. For event-triggered DO-A transmission, the device randomly selects one resource from the configured periodic resource and initiates DO-A transmission via RA. For periodic DO-A transmission, the device transmits DO-A data directly using the configured periodic resource, without RA.

- [Apple] Support both periodic and aperiodic resource allocation for DO-A. Periodic allocation for devices with regular reporting patterns and aperiodic allocation using contention-based access for event-triggered traffic.

- [Lenovo] Periodic resource allocation is recommended. In aperiodic resource allocation, resources are assigned only when a specific predefined event occurs - for example, when a device has DO-A data ready for transmission. Such allocation must be initiated through prior signaling, meaning it cannot be applied until after the first D2R transmission within the DO-A process. Consequently, this approach is not suitable for the very first D2R transmission of DO-A. Dedicated band or broadcasting a set of resource can be further considered for resource allocation in supporting DO-A traffic.

- [InterDigital] For resource allocation method for the first D2R transmission for DO-A for Device 2b and for Device C, both periodic D2R resource allocation and aperiodic D2R resource allocation are supported with further study on: 1) Periodic resource allocation with reference to an R2D periodic sync signal. 2) Aperiodic resource allocation multiplexed with other D2R transmissions. Periodic resource allocation may be useful to accommodate periodic traffic while aperiodic resource allocation may be useful for transmission of event-triggered traffic.

- [Sony] Traffic type dedicated DO-A resources simplify conflict avoidance but may lead to reduced spectrum efficiency as those resources cannot be used by e.g. DO-DTT traffic. Different readers may have different traffic characteristics, and reader-specific DO-A resource pools may be beneficial. Introducing a listening period prior to a DO-A occasion allows the scheduling entity to invalidate or reclaim configured DO-A resources, improving resource utilization.

- [DCM] For resource allocation method for the first D2R transmission for DO-A, periodic D2R resource allocation has an advantage of reducing R2D signalling overhead.

- [Qualcomm] Periodic resources are needed at least for the first D2R transmission for DO-A. Periodic Access occasion (AO) resources for device random access to support periodic DO-A traffic. Periodic Access request (AR) resources for device common request to support for event-triggered DO-A traffic. Aperiodic resources can be additionally configured by on-demand paging for the following DO-A transmissions. Consider resource sharing for DO-A and DO-DTT/DT to improve resource utilization.

- [IIT] The resources required for transmitting the first DO-A message can be scheduled after the periodic synchronization signal. These resources do not require explicit scheduling information; instead, they can be implicitly indicated through a defined timing and frequency relationship with the periodic synchronization signal.

- [Fraunhofer] From the device’s perspective, an aperiodic resource allocation may impose a significant burden, as devices would need to continuously monitor R2D signals and wait for dynamically indicated transmission opportunities. In our view, a periodic resource allocation is preferred, as it provides predictable transmission opportunities that both the reader and devices can rely on.

| Company | Y/N | Comments |
| --- | --- | --- |
| Spreadtrum |  | We support that resource allocation method for the first D2R transmission for DO-A should be periodic. In summary, sources (Huawei, OPPO, Ericsson, CMCC, vivo, NEC, Panasonic, Samsung, Lenovo, DCM, Quectel, IIT, Fraunhofer, Spreadtrum) report that it should be periodic, while sources (CATT, Xiaomi) state that it should be aperiodic. |
| LG | Y |  |
| Xiaomi |  | We support that resource allocation method for the first D2R transmission for DO-A should also be periodic and aperiodic. In summary, sources (Huawei, OPPO, Ericsson, CMCC, vivo, NEC, Panasonic, Samsung, Lenovo, DCM, Quectel, IIT, Fraunhofer) report that it should be periodic, while sources (CATT, Xiaomi) state that it should be aperiodic. Sources (ZTE, Apple, InterDigital, Qualcomm, Xiaomi) state that periodic and aperiodic resource allocation should be supported. Sources (LGE, China Telecom) submitted observations without mentioning their preferences |
| Huawei, HiSilicon |  | It would be better if the observations are categorized based on their support for periodic or aperiodic resource allocation method. |
| ZTE, Sanechips |  | The following observation can be captured base on our contribution R1-2600453: Periodic resource allocation (e.g. option 1a/1b discussed in our Tdoc) provides predictable and reliable access opportunities with bounded access latency. Aperiodic resource allocation (e.g. option 2 discussed in our Tdoc) can dynamically configure DO-A resources to enable more flexible scheduling and reducing both signaling and resource overhead. Through network coordination, Option 2a/2b can effectively avoid resource conflicts between DO-A traffic and DT traffic, and mitigate network congestion. |
| China Telecom | Y | OK. |

- [H] Proposal 3.2.3b-1

For resource determination method for the first D2R transmission for DO-A for Device 2b and for Device C, capture the following in TR 38.769:

On resource determination, following observations were provided.

- [Spreadtrum, China Telecom] For determining the time domain of resource for DO-A, the end of periodic sync signal could be the reference point.

- [OPPO] Consider the ending of the A-IoT SSB and the single-tone sine wave in the A-IoT SSB as the reference point in time and frequency domain respectively for candidate resources allocation of DO-A traffic.

- [NEC] For resource allocation/determination method for DO-A transmission, the reference of the scheduling can be the end of the corresponding R2D sync signal or the PRDCH carrying broadcast information.

- [InterDigital] Periodic resource allocation with reference to an R2D periodic sync signal.

- [CMCC] Broadcast information should include timing information that help devices determining frame number/slot number, based on which devices can determine the RACH occasions.

- [ZTE] Using the end of the SIB-like information as the reference point for DO-A resource determination provides high flexibility in periodic allocation. However, the DO-A resource may be located far from the reference point, which results in a relatively large timing error due to the residual SFO of Device 2b/C. In contrast, using the end of the periodic synchronization signal as the reference point constrains the DO-A period to the sync signal periodicity but places the DO-A resource adjacent to the reference point, thereby minimizing timing error.

- [CATT] The polling message in PRDCH contains resource allocation for scheduling request for DO-A traffic. The resource allocation for scheduling request can be similar to that for Msg1 scheduled by Msg0 in Rel-19 random access procedure.

| Company | Y/N | Comments |
| --- | --- | --- |
| CMCC |  | For resource determination method for the first D2R transmission for DO-A for Device 2b and for Device C, Proposal 3.1.2a has provided views that broadcast information can provide the information related to resource allocation for the first D2R transmission for DO-A and sources. And if period resource allocation are used as discussed in Proposal 3.2.3a, absolute slot index or frame index may be needed for determining the resource. Single reference point may not be enough. They are both related to the resource determination. This proposal is related to specific reference timing determination? |
| ZTE, Sanechips |  | The following observation can be captured base on our contribution R1-2600453: Using the end of the SIB-like information as the reference point (corresponding to Alt 1 in section 3.1.1 in our Tdoc) for DO-A resource determination provides high flexibility in periodic allocation. However, the DO-A resource may be located far from the reference point, which results in a relatively large timing error due to the residual SFO of Device 2b/C. In contrast, using the end of the periodic synchronization signal as the reference point (corresponding to Alt 2 in section 3.1.1 in our Tdoc) constrains the DO-A period to the sync signal periodicity but places the DO-A resource adjacent to the reference point, thereby minimizing timing error. |
| CATT |  | Please add CATT’s view into the proposal: The polling message in PRDCH contains resource allocation for scheduling request for DO-A traffic. The resource allocation for scheduling request can be similar to that for Msg1 scheduled by Msg0 in Rel-19 random access procedure. |
| China Telecom |  | We mentioned in our Tdoc that using the end of synchronization signal reception occasion as the reference point for both case B1 and case B2, hence we propose to add our company in the observation: - [Spreadtrum, China Telecom] For determining the time domain of resource for DO-A, the end of periodic sync signal could be the reference point. |

Necessary information and its provisioning for the first D2R transmission for DO-A

Previous agreements

Agreement (RAN1#122)

Study the following aspects to support DO-A traffic (periodic and event-triggered) for Device 2b and for Device C in Rel-20.

- Timing and frequency synchronization for DO-A transmission

- Resource allocation/determination method for DO-A transmission

- Configuration/signaling of necessary information

Agreement (RAN1#122bis)

For providing necessary information for the first D2R transmission for DO-A for Device 2b and for Device C, study at least following options considering feasibility:

- Option 1: necessary information provided by broadcast information

- Option 2: necessary information provided by a paging-like R2D message

- Option 3: necessary information derived from a synchronization signal and provided by broadcast information

Agreement (RAN1#122bis)

Study whether necessary information for the first D2R transmission for DO-A for Device 2b and for Device C is provided periodically and/or aperiodically by Reader.

Agreement (RAN1#123)

For providing necessary information for the first D2R transmission for DO-A for Device 2b and for Device C, following options are considered feasible.

- Option 1: necessary information provided by broadcast information

- Option 2: necessary information provided by a paging-like R2D message

- Option 3: necessary information derived from a synchronization signal and provided by broadcast information

For providing necessary information for the first D2R transmission for DO-A for Device 2b and for Device C, study following option considering feasibility and applicability

- Option 4: necessary information provided by device-specific R2D transmission

Work plan for RAN1#124 meeting (R1-2509614)

- DO-A - For Option 1/2/3 for the content of the first D2R transmission for DO-A - recommended scenarios (event-triggered or periodic) - subsequent transmission(s)/reception(s)/procedure(s) if any after the first D2R transmission for DO-A - other aspects - Timing and frequency synchronization for DO-A - Resource allocation/determination for DO-A - Necessary information and its provisioning (Option 1/2/3)

Views on each of the Options including the feasibility in the case of Option 4, and whether the necessary information is provided periodically and/or aperiodically by a reader need to be captured in the TR.

Summary

Preference for Option 1/2/3 and 4 for providing necessary information for the first D2R transmission for DO-A

Option 1: Huawei, Spreadtrum, OPPO, Ericsson, CMCC, ETRI, DCM, Qualcomm, Quectel, Fraunhofer

Option 2: CATT, Xiaomi, InterDigital, Qualcomm, Quectel, ASUSTeK

Option 3: NEC, InterDigital, Sony, Qualcomm, Fraunhofer

Option 4: ASUSTeK

Views on the feasibility and applicability of Option 4

Feasible and applicable: LGE, Samsung

Not sure if it is feasible: CATT, Lenovo

Infeasible: vivo

Views on whether necessary information for the first D2R transmission for DO-A is provided periodically and/or aperiodically by Reader.

- Regardless of which options:

  - Periodically by Reader: Huawei, Spreadtrum, OPPO,

  - Aperiodically by Reader:

- Option 1:

  - Periodically by Reader: ZTE, vivo, LGE, DCM, Qualcomm,

  - Aperiodically by Reader:

- Option 2:

  - Periodically by Reader: Xiaomi, ZTE,

  - Aperiodically by Reader: ZTE, vivo, LGE, Qualcomm,

- Option 3:

  - Periodically by Reader: ZTE, vivo, LGE, Qualcomm,

  - Aperiodically by Reader: ZTE,

- Option 4:

  - Periodically by Reader: ZTE,

  - Aperiodically by Reader:

Necessary information for the first D2R transmission for DO-A

[FUTUREWEI]

[Spreadtrum]

[CATT]

[Tejas]

[ZTE]

[NEC]

[TCL]

[Fraunhofer]

- [H] Proposal 3.2.4a

On Option 1/2/3/4 for providing necessary information for the first D2R transmission for DO-A, capture the following in TR 38.769:

Sources (LGE, Samsung) state that Option 4 is feasible and applicable after inventory procedure/registration, while sources (CATT, Lenovo, vivo) state that it is or may be infeasible. For the options on how the necessary information for the first D2R transmission for DO-A traffic is provided to the device, it is reported that sources (Huawei, Spreadtrum, OPPO, Ericsson, CMCC, ETRI, DCM, Qualcomm, Quectel, Fraunhofer) support Option 1. Following observations were provided for Option 1.

- [FUTUREWEI] Broadcast information contains at least one resource allocation. After a device obtains an ID for a DO-A group, the device can select the resource allocation associated with the group. This option offers the reader control to set the allocation globally.

- [Huawei] (Pos) Broadcast information should be used to provide the necessary information for the first D2R transmission for DO-A because it was agreed to be periodically transmitted, thereby ensuring that the device can receive this information regardless of the availability status of the device and the type of DO-A transmission (event-triggered or periodic) and guarantees the device to have the opportunity to transmit the first D2R transmission for DO-A. In the case of a two-step acquisition procedure, the necessary information is included in the SIB-like broadcast information, and the device can use the last received broadcast information to determine the necessary information.

- [OPPO] (Pos) Candidate D2R resources for the first D2R transmission of DO-A traffic should be indicated by the broadcast information in the A-IoT SSB rather than paging like aperiodic R2D signal (i.e., Option 2). Because whether to transmit DO-A data is fully up to device, which is different from DT or DO-DTT where the device should transmit data whenever triggered by R2D. Thus, aperiodic D2R resource allocation for DO-A cannot timely adapt to the real DO-A transmission demand and will exacerbate the issue that the traffic of DO-A is available but lack of allocated D2R candidate resources.

- [Ericsson] (Pos) For providing necessary information for the first D2R transmission for DO-A for Device 2b and for Device C, the necessary information is provided by broadcast information.

- [CATT] (Neg) For Options 1 and 3, the resource overhead and energy consumption are high with periodic transmission of the broadcast information. The A-IoT device also needs to store the information in non-volatile memory and count the time interval with the decoded broadcast information.

- [CMCC] (Pos) For Option 1, as long as device acquires cell synchronization and reads broadcast information, it can get the access resource allocation and initiate random access any time it has D2R traffic to report, this option is simple and more straightforward.

- [ZTE] Option 1 is a periodic DO-A resource allocation scheme that provides predictable and reliable access opportunities with bounded access latency.

- [vivo] (Pos) As agreed in RAN1, broadcast information is necessary for subsequent transmission and reception by all devices, so it is more signaling efficiency for periodic broadcast information to provide information of D2R transmission for both periodic DoA and event-triggered DoA traffics, compared to option2. For event-triggered DoA traffics, Option1 provide more guaranteed and stable DoA traffic service compared to option2 aperiodic paging-like triggering. Option 1 will not impact on the detection performance of periodic sync signal, compared to option3 that since necessary information derived from a synchronization signal e.g., by increase the number of sync signal sequences.

- [LGE] Delivering the necessary information through broadcast information is simple and straightforward, but it increases signaling overhead when the broadcast information is transmitted together with a short‑period sync signal.

- [ETRI] (Pos) From an initial DO-A transmission perspective, it is desirable for a device to obtain the required resource allocation and related parameters in a simple and deterministic manner, without relying on device-specific signaling or additional interaction with the reader. In this context, broadcast information naturally serves as a suitable mechanism for conveying such information. Providing the necessary information via broadcast information enables the device to acquire all required parameters after synchronization, without waiting for a paging-like message or performing additional decoding procedures. This approach is particularly beneficial for periodic DO-A transmission, where predictable transmission behavior and low latency are desired.

- [DCM] (Pos) Necessary information for the first D2R transmission for DO-A provided by periodic broadcast information should be considered.

- [Qualcomm] For providing necessary information for the first D2R transmission for DO-A, combining Option 1/3 with Option 2 is feasible, where Option 1/3 can be considered for periodic D2R resources and Option 2 can be used to adjust the D2R resources.

- [Quectel] The resource allocation information of the first D2R transmission for DO-A is included in paging-like message or broadcast information.

- [Fraunhofer] Select between Option 1 and Option 3 to provide necessary information for the first D2R transmission for DO-A. Among the options under study for determining the initial set of resources for DO-A, Option 1 and 3 should be prioritized. These approaches not only minimize signaling overhead but also provides greater flexibility.

Sources (CATT, Xiaomi, InterDigital, Qualcomm, Quectel, ASUSTeK) support Option 2 according to the following observations provided for Option 2.

- [FUTUREWEI] Paging-like information also contains at least one resource allocation. After a device obtains an ID for a DO-A group, the device can select the resource allocation associated with the group. This option offers the reader control to set the allocation globally.

- [Huawei] (Neg) If the necessary information is provided by a paging-like R2D message (Option 2), if this message is used for polling, there would be an increase in the transmission overhead due to the additional DO-A scheduling request and another paging message to provide resource allocation information for the first D2R transmission. If the message is intended to be triggered by the device, it is unclear how the device can do so without any resource allocation information for such a trigger to be transmitted.

- For Option 2‌, a paging-like indicator can be used as a polling message to query the A-IoT devices on the need of resource for first D2R transmission. This mechanism triggers device response to indicate whether DO-A message transmission is required.

- [CMCC] (Neg) “Option2: necessary information provided by a paging-like R2D message” means more energy consumption before DOA traffic report than option 1 and has a risk of not enough energy for traffic report.

- [Xiaomi] R2D message carrying DO-A indication information can be sent periodically. Between two adjacent R2D messages, several DO-A occasions can be pre-allocated by the adjacent R2D message. For the device who has DO-A data to transmit, it can choose one of the DO-A occasions to send DO-A transmission. For the DO-A occasion, it can be interpreted as a direct transmission occasion, or a transmission occasion for transmission request. For example, the R2D message can be paging, and the occasion is used for Msg1 transmission.

- [ZTE] Option 2 supports periodic and aperiodic DO-A resource allocation. For the aperiodic allocation, the network can dynamically configure DO-A resources to enable more flexible scheduling and reducing both signaling and resource overhead. Through network coordination, Option 2 can effectively avoid resource conflicts between DO-A traffic and DT traffic and mitigate network congestion.

- [vivo] (Neg) Since broadcast information is necessary for subsequent transmission and reception of all devices, if the necessary information is provided by the paging-like R2D message, additional signaling overhead and more power consumption will be caused for a device to get the first D2R resource, compared to option1. For periodic DoA traffic, using option2 aperiodic paging-like R2D message to trigger D2R resource of every periodicity will obviously cause signaling waste. For event-triggered DoA traffic, a reader cannot know when to do the first D2R resource scheduling via paging-like R2D message, thereby resulting in D2R transmission latency and maybe discontinuity of services.

- [LGE] Delivering the necessary information through a paging‑like R2D message reduces signaling overhead at the reader side, but it introduces transmission delay because DO‑A transmission can only occur after the device receives the paging message.

- [ETRI] (Neg) Paging-like R2D messages introduce additional signaling steps and may not be well suited for devices that have not yet established any access context.

- [InterDigital] (Pos) Since AIoT devices may have significant SFO, over long intervals, timing synchronization would be degraded. Therefore, to locate the resources accurately, a reference point such as a synchronization signal or a paging-like R2D message would be beneficial. A paging-like R2D message may incur higher overhead depending on the signal design but it would provide higher flexibility in resource allocation.

- [Qualcomm] For providing necessary information for the first D2R transmission for DO-A, combining Option 1/3 with Option 2 is feasible, where Option 1/3 can be considered for periodic D2R resources and Option 2 can be used to adjust the D2R resources. For Option 2: periodic broadcast information to indicate periodic monitoring occasions for paging message(s) and the paging message can indicate one-time resources for first D2R.

- [Quectel] The resource allocation information of the first D2R transmission for DO-A is included in paging-like message or broadcast information.

- [ASUSTeK] For providing necessary information for the first D2R transmission for DO-A for Device 2b and for Device C, at least support Option 2 and Option 4. For the first D2R transmission autonomously triggered by active device, reader implements CBRA triggering which targets devices with pending DO-A data.

Sources (NEC, InterDigital, Sony, Qualcomm, Fraunhofer) support Option 3 based on the following observations provided for Option 3.

- [FUTUREWEI] This option can use MIB-like information and broadcast information. It combines control as well as allocation.

- [Huawei] (Neg) If the necessary information is derived from a synchronization signal and provided by broadcast information (Option 3), it would require multiple sequences to be used to design the synchronization signal and the device to blindly detect these multiple sequences when performing the initial frequency acquisition procedure, which will add to the complexity and power consumption of the device.

- [CATT] (Neg) For Options 1 and 3, the resource overhead and energy consumption are high with periodic transmission of the broadcast information. The A-IoT device also needs to store the information in non-volatile memory and count the time interval with the decoded broadcast information.

- [CMCC] Option 3 is still based on option 1, and details can be further studied.

- [ZTE] Option 3 supports periodic and aperiodic DO-A resource allocation.

- [vivo] (Neg) If the necessary information is derived by sync signal e.g., by increasing the number of sync signal sequences, it will have impact on the detection performance of sync signal. What necessary information for the first D2R transmission for DO-A should be carried by periodic synchronization signal and also the corresponding benefit are unclear.

- [LGE] Delivering the resource information through synchronization signals is less desirable because blind detection of synchronization signals is required, which increases device complexity.

- [LGE] For providing necessary information for the first D2R transmission for DO-A for Device 2b and for Device C, Option 3 should be adopted as following: TR->D for time resource determination is pre-defined and the end of the corresponding R2D sync signal or the PRDCH carrying broadcast information is used for the device to determine the time resource for the first D2R transmission.

- [ETRI] (Neg) Deriving information from synchronization signals may impose constraints on signal design and limit flexibility in conveying resource configuration details.

- [InterDigital] (Pos) Since AIoT devices may have significant SFO, over long intervals, timing synchronization would be degraded. Therefore, to locate the resources accurately, a reference point such as a synchronization signal or a paging-like R2D message would be beneficial.

- [Sony] (Pos) Option 3 has advantage over the others. When DO-A resources are conveyed via broadcast information, waiting for the next broadcast to detect changes may introduce unnecessary delays. Frequent monitoring of broadcast information to detect updates is inefficient for both devices and the system. Therefore, indicating updates to broadcast information via a field associated with a periodic synchronization signal can reduce unnecessary monitoring.

- [Qualcomm] For providing necessary information for the first D2R transmission for DO-A, combining Option 1/3 with Option 2 is feasible, where Option 1/3 can be considered for periodic D2R resources and Option 2 can be used to adjust the D2R resources.

- [Fraunhofer] Select between Option 1 and Option 3 to provide necessary information for the first D2R transmission for DO-A. Among the options under study for determining the initial set of resources for DO-A, Option 1 and 3 should be prioritized. These approaches not only minimize signaling overhead but also provides greater flexibility.

Lastly, source (ASUSTeK) supports Option 4.

- [Huawei] (Neg) If the necessary information is contained in a device-specific R2D transmission (Option 4), it is necessary for the device to transmit an additional scheduling request which also increases the transmission overhead.

- [CATT] (Neg) Reader cannot know which device would require resource for DO-A transmission. Thus, it is very unlikely that Reader can provide the necessary information by device-specific R2D transmission.

- [CMCC] (Neg) Option 4 is based on RACH procedure ahead, and is not the first D2R transmission for DOA.

- [ZTE] The DO-A traffic/resource periodicity is indicated to a connected device via the dedicated device-specific R2D transmission. The device then transmits its DO-A data payload directly based on the period. This option is appropriate for connected devices that have periodic DO-A reporting requirements. For example, when a connected device is configured to periodically report sensor data or status updates.

- [vivo] For option4, if the first D2R transmission for DoA is provided by device-specific R2D transmission, it means that DoA traffic can only be served for a device after it is identified/inventoried by a reader, which is one kind of Do-DTT operation in fact. That is, option 4 is infeasible to support DoA traffic to some extent. For devices only oriented to DoA traffic, performing this device identification before the first D2R transmission of DoA traffic is also unnecessary, since it confers no concrete benefit and instead incurs unnecessary resource overhead and power consumption.

- [LGE] Option 4 is feasible and considered applicable when the device completes the inventory procedure prior to the DO‑A data transmission.

- [Samsung] Option 4 is considered feasible and applicable for DO-A reporting after registration.

- [Lenovo] Not support Option 4. In event-triggered resource allocation, resources are assigned only when a specific predefined event occurs - for example, when a device has DO-A data ready for transmission. Such allocation must be initiated through prior signaling, meaning it cannot be applied until after the first D2R transmission within the DO-A process. Consequently, this approach is not suitable for the very first D2R transmission of DO-A. Moreover, device-specific R2D transmissions are unavailable at this stage, so they cannot supply the necessary information for the first DO A D2R transmission.

- [Qualcomm] For Option 4: the first D2R transmission may be interpreted differently but the resource allocation for the first D2R transmission can be based on paging message in Option 2. If a device needs to go through inventory procedure before DO-A data reporting, the paging message with CBRA indication (as in Option 2) is used to trigger the inventory procedure (Msg1 is the first D2R), and after inventory procedure the device-specific R2D message can trigger DO-A reporting. If a device is known by the reader and does not need to go through inventory procedure, the paging message with CFA indication (as in Option 2) can be used to trigger the DO-A data reporting (DO-A data is the first DO-A).

- [ASUSTeK] For providing necessary information for the first D2R transmission for DO-A for Device 2b and for Device C, at least support Option 2 and Option 4. For the first D2R transmission autonomously triggered by active device, reader implements CBRA triggering which targets devices with pending DO-A data.

| Company | Y/N | Comments |
| --- | --- | --- |
| Samsung |  | We consider Option 4 feasible and even necessary. SA2 has already discussed modes where a device autonomously triggers registration (i.e., transmits a Msg1-like signal) and data transmission. Accordingly, it is feasible—and also necessary—for a device to obtain device-specific resources for data transmission via an R2D transmission. In addition, please revise our observation as follows: [Samsung] states that, the study should include cases where a device autonomously triggers registration and/or data transmission. In practical deployments, a reader generally collects sensing data only from devices already associated/paired/registered with it, and requiring random access for each transmission would severely impact device energy consumption. Hence, enabling autonomous transmissions via device-specific resource configuration after registration (Option 4) is feasible and necessary. |
| vivo |  | According to our observation for these four options, we exactly support option1, so please add our name into the description “For the options on how the necessary information for the first D2R transmission for DO-A traffic is provided to the device, it is reported that sources (Huawei, Spreadtrum, OPPO, Ericsson, CMCC, ETRI, DCM, Qualcomm, Quectel, Fraunhofer, vivo) support Option 1.” Besides, it seems clearer to move “Sources (LGE, Samsung) state that Option 4 is feasible and applicable after inventory procedure/registration, while sources (CATT, Lenovo, vivo) state that it is or may be infeasible.” in the front of “Lastly, source (ASUSTeK) supports Option 4.” |
| LG | Y |  |
| Xiaomi |  | We also support option1. Broadcast-type R2D can be used to convey the resource allocation information for the first D2R transmission for DO-A. So, we make the following revision: For the options on how the necessary information for the first D2R transmission for DO-A traffic is provided to the device, it is reported that sources (Huawei, Spreadtrum, OPPO, Ericsson, CMCC, ETRI, DCM, Qualcomm, Quectel, Fraunhofer, Xiaomi) support Option 1. Following observations were provided for Option 1. |
| Huawei, HiSilicon | Y | Apart from capturing the observations, we would need to decide on whether option 4 is feasible or not, for which an agreement either way is required. In our view, it would not be possible for the reader to provide the necessary information using a device-specific transmission because it would mean that the reader has to be aware of the device ID, for which the device would have to provide this information beforehand in another prior D2R transmission. Hence, the prior D2R transmission would be the first D2R transmission for DO-A, which would mean that the information provided in this device-specific R2D transmission can be used only for a subsequent D2R transmission and not the first one. It would also be better if the observations for each of the options were categorized into positive and negative observations. |
| Panasonic | Y | We also think the paging-like signal could be used to indicate the DO-A resources. Please add our view as: Sources (CATT, Xiaomi, InterDigital, Qualcomm, Quectel, ASUSTeK, Panasonic) support Option 2 according to the following observations provided for Option 2. |
| NEC |  | Please add our observations for supporting Option 3: [NEC] A subset of sync signal can have associated broadcast information carried by PRDCH which contains the D2R Scheduling Info for the first D2R transmission for scheduling request or Msg1 transmission. |
| CATT | Y | OK with the proposal |
| China Telecom |  | We mentioned in our Tdoc that we prefer to use a paging-like message as the broadcast information. Hence, we propose to add our company to the corresponding observations: Sources (LGE, Samsung) state that Option 4 is feasible and applicable after inventory procedure/registration, while sources (CATT, Lenovo, vivo) state that it is or may be infeasible. For the options on how the necessary information for the first D2R transmission for DO-A traffic is provided to the device, it is reported that sources (Huawei, Spreadtrum, OPPO, Ericsson, CMCC, ETRI, DCM, Qualcomm, Quectel, Fraunhofer, China Telecom) support Option 1. Following observations were provided for Option 1. Sources (CATT, Xiaomi, InterDigital, Qualcomm, Quectel, ASUSTeK, China Telecom) support Option 2 according to the following observations provided for Option 2. [China Telecom]: A new paging-like message may be necessary to broadcast the necessary information effectively. |

- [H] Proposal 3.2.4b-1

On the necessary information and its provisioning for the first D2R transmission for DO-A, capture the following in TR 38.769:

On whether the reader provides the necessary information for the first D2R transmission for DO-A traffic periodically and/or aperiodically, sources (Huawei, Spreadtrum, OPPO) state that it is provided periodically without mentioning specific option. Sources (ZTE, vivo, LGE, DCM, Qualcomm, China Telecom) state that it is periodic for Option 1. For Option 2, sources (Xiaomi, ZTE, China Telecom) state that it is periodic, while sources (ZTE, vivo, LGE, Qualcomm) state that it is aperiodic. For Option 3, sources (ZTE, vivo, LGE, Qualcomm) state that it is periodic, while source (ZTE) states that it is aperiodic. For Option 4, source (ZTE) states that it is provided periodically. Following options were provided during the study.

- [Xiaomi] R2D message carrying DO-A indication information can be sent periodically. For example, the R2D message can be paging, and the occasion is used for Msg1 transmission.

- [ZTE] Option 1 is a periodic DO-A resource allocation scheme that provides predictable and reliable access opportunities with bounded access latency. Option 2 supports periodic and aperiodic DO-A resource allocation. For the aperiodic allocation, the network can dynamically configure DO-A resources to enable more flexible scheduling and reducing both signaling and resource overhead. Through network coordination, Option 2 can effectively avoid resource conflicts between DO-A traffic and DT traffic and mitigate network congestion. Option 3 supports periodic and aperiodic DO-A resource allocation. For Option 4, the DO-A traffic/resource periodicity is indicated to a connected device via the dedicated device-specific R2D transmission. The device then transmits its DO-A data payload directly based on the period.

- [vivo] By using option 1 or option3, necessary information is periodically provided by broadcast information. And for option2, a paging-like R2D message is more like an aperiodic transmitted polling message.

- [Qualcomm] For providing necessary information for the first D2R transmission for DO-A, combining Option 1/3 with Option 2 is feasible, where Option 1/3 can be considered for periodic D2R resources and Option 2 can be used to adjust the D2R resources. For Option 1/3: periodic broadcast information to indicate periodic resources for first D2R. For Option 2: periodic broadcast information to indicate periodic monitoring occasions for paging message(s) and the paging message can indicate one-time resources for first D2R.

- [China Telecom] A strictly periodic broadcast information transmission scheme is supported at the reader side, and a paging-like message is used as the broadcast information.

| Company | Y/N | Comments |
| --- | --- | --- |
| LG | Y |  |
| Huawei, HiSilicon |  | We have an agreement from the last meeting stating that periodic broadcast information is necessary for subsequent transmissions and receptions, which would essentially include the necessary information for the first D2R transmission for DO-A. While there is an FFS on whether it has to be strictly periodic, the agreement rules out the broadcast information being aperiodic. Hence we do not need this proposal. |
| China Telecom |  | We mentioned in our Tdoc that a paging-like message as the broadcast information and the broadcast information is periodic. Hence, we propose add our company to the corresponding observations: On whether the reader provides the necessary information for the first D2R transmission for DO-A traffic periodically and/or aperiodically, sources (Huawei, Spreadtrum, OPPO) state that it is provided periodically without mentioning specific option. Sources (ZTE, vivo, LGE, DCM, Qualcomm, China Telecom) state that it is periodic for Option 1. For Option 2, sources (Xiaomi, ZTE, China Telecom) state that it is periodic, while sources (ZTE, vivo, LGE, Qualcomm) state that it is aperiodic. For Option 3, sources (ZTE, vivo, LGE, Qualcomm) state that it is periodic, while source (ZTE) states that it is aperiodic. For Option 4, source (ZTE) states that it is provided periodically. Following options were provided during the study. - [Xiaomi] R2D message carrying DO-A indication information can be sent periodically. For example, the R2D message can be paging, and the occasion is used for Msg1 transmission. - [ZTE] Option 1 is a periodic DO-A resource allocation scheme that provides predictable and reliable access opportunities with bounded access latency. Option 2 supports periodic and aperiodic DO-A resource allocation. For the aperiodic allocation, the network can dynamically configure DO-A resources to enable more flexible scheduling and reducing both signaling and resource overhead. Through network coordination, Option 2 can effectively avoid resource conflicts between DO-A traffic and DT traffic and mitigate network congestion. Option 3 supports periodic and aperiodic DO-A resource allocation. For Option 4, the DO-A traffic/resource periodicity is indicated to a connected device via the dedicated device-specific R2D transmission. The device then transmits its DO-A data payload directly based on the period. - [vivo] By using option 1 or option3, necessary information is periodically provided by broadcast information. And for option2, a paging-like R2D message is more like an aperiodic transmitted polling message. - [Qualcomm] For providing necessary information for the first D2R transmission for DO-A, combining Option 1/3 with Option 2 is feasible, where Option 1/3 can be considered for periodic D2R resources and Option 2 can be used to adjust the D2R resources. For Option 1/3: periodic broadcast information to indicate periodic resources for first D2R. For Option 2: periodic broadcast information to indicate periodic monitoring occasions for paging message(s) and the paging message can indicate one-time resources for first D2R. - [China Telecom] a strictly periodic broadcast information transmission scheme is supported at the reader side, and a paging-like message is used as the broadcast information |

D2R power control

Previous agreements

Agreement (RAN1#122)

Study necessity and feasibility of D2R Tx power control for Device 2b and for Device C, including the following aspects

- Open loop and/or closed loop power control

- Required measurements (e.g., signal strength)

- Required information at the device

- Required information at reader side

- Signaling overhead and power consumption

Agreement (RAN1#122bis)

Study the necessity of D2R Tx power control for Device 2b and for Device C for outdoor scenarios

- Target to agree on observations on pros and cons of D2R Tx power control vs. no Tx power control at RAN1#123

Continue studying feasibility of the following open loop power control methods considering device complexity:

- Methods of open loop power control

  - Option 1: Device calculates Tx power based on measured pathloss, a predefined formula, and some configured parameters from reader

    - Required information at the device side at least includes reader Tx power, received target power at the reader, etc.

  - Option 2: Device measures RSRP, and uses a corresponding Tx power when the RSRP falls in an RSRP range.

    - The RSRP range is based on RSRP threshold(s) configured by reader or predefined

    - Required information at the device side at least includes RSRP threshold(s), corresponding D2R Tx power related information per each RSRP range defined by the RSRP threshold(s)

  - FFS details (e.g., required measurements, required information)

Continue studying the necessity and feasibility of closed loop power control for D2R.

Agreement (RAN1#123)

On D2R Tx power control for Device 2b and for Device C, capture the following in TR 38.769.

The necessity of D2R Tx power control for Device 2b and Device C for outdoor scenarios were studied and the following observations were provided:

- 22 sources (Futurewei, LGE, Spreadtrum, vivo, Nokia, Semtech, CMCC, CEWiT, BOSCH, NEC, InterDigital, TCL, Apple, China Telecom, ZTE, OPPO, Huawei, Docomo, Ericsson, Xiaomi, Samsung, Qualcomm) reported that the D2R Tx power control for Device 2b and Device C is necessary for outdoor scenarios.

  - 20 sources (Futurewei, LGE, Spreadtrum, vivo, Nokia, Semtech, CMCC, CEWiT, BOSCH, Quectel, InterDigital, TCL, Apple, OPPO, Huawei, Docomo, Ericsson, Xiaomi, Samsung, ZTE) state that it can reduce the interference between different devices (i.e., the Near-Far problem).

  - 14 sources (Futurewei, LGE, Spreadtrum, vivo, Semtech, CMCC, NEC, InterDigital, TCL, Apple, OPPO, Xiaomi, Samsung, ZTE) reported that it helps devices remain available longer.

  - 4 sources (Futurewei, Semtech, Apple, Docomo) reported that the interference due to a transmission from a device to a reader can be reduced at an unintended reader

  - 1 source (Qualcomm) added that depending on device’s peak power consumption and PA support, the D2R Tx power control may not be feasible.

- 1 source (CATT) reported that the D2R Tx power control for Device 2b and Device C is not necessary mentioning in its contribution that the inter-channel interference from different devices is negligible, D2R coverage is limited, and the complexity and the power consumption required at the device side is high.

Necessity of D2R Tx power control (Pros and cons of Tx power control vs. no Tx power control for D2R)

Previous agreements

Agreement (RAN1#123)

On D2R Tx power control for Device 2b and for Device C, capture the following in TR 38.769.

The necessity of D2R Tx power control for Device 2b and Device C for outdoor scenarios were studied and the following observations were provided:

- 22 sources (Futurewei, LGE, Spreadtrum, vivo, Nokia, Semtech, CMCC, CEWiT, BOSCH, NEC, InterDigital, TCL, Apple, China Telecom, ZTE, OPPO, Huawei, Docomo, Ericsson, Xiaomi, Samsung, Qualcomm) reported that the D2R Tx power control for Device 2b and Device C is necessary for outdoor scenarios.

  - 20 sources (Futurewei, LGE, Spreadtrum, vivo, Nokia, Semtech, CMCC, CEWiT, BOSCH, Quectel, InterDigital, TCL, Apple, OPPO, Huawei, Docomo, Ericsson, Xiaomi, Samsung, ZTE) state that it can reduce the interference between different devices (i.e., the Near-Far problem).

  - 14 sources (Futurewei, LGE, Spreadtrum, vivo, Semtech, CMCC, NEC, InterDigital, TCL, Apple, OPPO, Xiaomi, Samsung, ZTE) reported that it helps devices remain available longer.

  - 4 sources (Futurewei, Semtech, Apple, Docomo) reported that the interference due to a transmission from a device to a reader can be reduced at an unintended reader

  - 1 source (Qualcomm) added that depending on device’s peak power consumption and PA support, the D2R Tx power control may not be feasible.

- 1 source (CATT) reported that the D2R Tx power control for Device 2b and Device C is not necessary mentioning in its contribution that the inter-channel interference from different devices is negligible, D2R coverage is limited, and the complexity and the power consumption required at the device side is high.

Work plan for RAN1#124 meeting (R1-2509614)

- D2R Tx power control - Necessity of D2R Tx power control (Pros and cons of Tx power control vs. no Tx power control for D2R) - Feasibility study of the two open-loop Tx power control methods for D2R - Necessity and feasibility of closed-loop Tx power control for D2R - Necessary information for D2R Tx power control and its provisioning

Necessity of D2R Tx power control (Pros and cons of Tx power control vs. no Tx power control for D2R) was already captured! So, this topic will be deprioritized.

Feasibility study of the two open-loop Tx power control methods for D2R

Previous agreements

Agreement (RAN1#122bis)

Study the necessity of D2R Tx power control for Device 2b and for Device C for outdoor scenarios

- Target to agree on observations on pros and cons of D2R Tx power control vs. no Tx power control at RAN1#123

Continue studying feasibility of the following open loop power control methods considering device complexity:

- Methods of open loop power control

  - Option 1: Device calculates Tx power based on measured pathloss, a predefined formula, and some configured parameters from reader

    - Required information at the device side at least includes reader Tx power, received target power at the reader, etc.

  - Option 2: Device measures RSRP, and uses a corresponding Tx power when the RSRP falls in an RSRP range.

    - The RSRP range is based on RSRP threshold(s) configured by reader or predefined

    - Required information at the device side at least includes RSRP threshold(s), corresponding D2R Tx power related information per each RSRP range defined by the RSRP threshold(s)

  - FFS details (e.g., required measurements, required information)

Continue studying the necessity and feasibility of closed loop power control for D2R.

Work plan for RAN1#124 meeting (R1-2509614)

- D2R Tx power control - Necessity of D2R Tx power control (Pros and cons of Tx power control vs. no Tx power control for D2R) - Feasibility study of the two open-loop Tx power control methods for D2R - Necessity and feasibility of closed-loop Tx power control for D2R - Necessary information for D2R Tx power control and its provisioning

Summary

Views on feasibility of each option

- Option 1 is

  - feasible: FUTUREWEI, Xiaomi, LGE, DOCOMO, TCL, vivo

  - not feasible:

- Option 2 is

  - feasible: FUTUREWEI, Xiaomi, LGE, DOCOMO, TCL, vivo

  - not feasible: Samsung

Preference for Option 1 and Option 2 for methods of open loop power control

- Prefer Option 1: Xiaomi, ZTE, LGE, ETRI, ASUSTeK, Quectel, TCL, OPPO,

  - Not prefer Option 2: Samsung, vivo, OPPO, NEC

- Prefer Option 2: Panasonic, Apple, Qualcomm, KT, Fraunhofer

- [Closed][H] Proposal 3.3.2a-1

On the two open‑loop Tx power control methods for D2R, capture the following in TR 38.769.

For Option 1, sources (FUTUREWEI, Xiaomi, LGE, DOCOMO, TCL, vivo) report that Option 1 is feasible.

- [FUTUREWEI] mentions that Option 1 is feasible, indicating that a device can measure the received R2D power at RF and derive its D2R transmit power based on a pathloss estimate obtained from reader‑provided transmit power and target power level.

- [Xiaomi] considers Option 1 feasible and prefers to support it due to its higher adjustment accuracy based on actual pathloss.

- [LGE] notes that Option 1 provides higher‑accuracy and more adaptive open‑loop power control based on pathloss, and is therefore the preferred approach.

- [DOCOMO] states that Option 1 offers finer‑granularity and more accurate D2R transmit‑power control, giving it an advantage for open‑loop operation.

- [TCL] states that Option 1 provides precise pathloss‑based D2R transmit‑power control that improves reliability, reduces interference, and enables energy‑efficient operation.

For Option 2, sources (FUTUREWEI, Xiaomi, LGE, DOCOMO, TCL, vivo) report that Option 2 is feasible, while source (Samsung) reports that Option 2 is not feasible.

- [FUTUREWEI] mentions that Option 2 is feasible, indicating that a device can perform an RSRP‑like baseband measurement that includes AGC effects and determine its D2R transmit power using a lookup table provided by the reader.

- [Xiaomi] notes that Option 2 is feasible due to its low complexity.

- [LGE] notes that Option 2 offers lower complexity through RSRP‑based power selection but provides coarser and less adaptive power control.

- [DOCOMO] states that Option 2 avoids device‑side formula computation, though the impact of such complexity remains unclear.

- [TCL] states that Option 2 offers simple RSRP‑range‑based selection but may cause under‑ or over‑powering due to coarse power levels.

Between Option 1 and Option 2, sources (Xiaomi, ZTE, LGE, ETRI, ASUSTeK, Quectel, TCL, vivo, [OPPO]) report that they prefer Option 1 over Option 2, while sources (Panasonic, Apple, Qualcomm, KT, Fraunhofer) report that they prefer Option 2 over Option 1. For Option 1, following observations were also provided during the study.

- [OPPO] mentions that in Option 1 the device needs to estimate the pathloss using the reader‑indicated Tx power and an R2D measurement, and that limited measurement accuracy and power‑adjustment capability may cause the received power at the reader to deviate from the configured P0, which can affect CDM‑based Msg1 transmission.

- [Ericsson] mentions that under an open‑loop power‑control mechanism, a simplified NB‑IoT–like approach can be used as a baseline for Rel‑20 A‑IoT.

- [Nokia] notes that Option 1 should cover cases where the device cannot use the required or indicated transmit power.

- [ZTE] mentions that Option 1 provides smooth and adaptive power control based on real‑time pathloss and a predefined formula, is therefore the preferred approach.

- [vivo] notes that Option 1 achieves better power‑control accuracy because each device derives its own transmit power using pathloss calculated from RSRP.

- [NEC] notes that Option 1 for open‑loop power control follows a traditional pathloss‑based approach and can be studied for Rel‑20.

- [Panasonic] notes that Option 1 can provide more precise transmit‑power control but increases the computational complexity of Device 2b/C.

- [Samsung] notes that Option 1 enables open‑loop D2R power control using pathloss estimation based on comparing the average ON‑state power of known R2D signals with the configured ON‑state power.

- [Apple] notes that Option 1 enables continuous Tx power adjustment and can be applied for device C.

- [ETRI] states that Option 1 provides higher‑accuracy D2R transmit power adjustment based on pathloss and is therefore more suitable for A‑IoT operation.

- [Qualcomm] states that Option 1 provides more precise pathloss‑based power control but may not be essential for ultra‑low‑power devices given their large transmit‑power tolerance and higher complexity.

- [Quectel] states that Option 1 provides higher‑precision D2R transmit‑power control through pathloss‑based calculation, and the additional complexity is acceptable for device 2b/C.

- [ASUSTeK] states that Option 1 allows pathloss‑based D2R transmit‑power calculation using reader‑configured parameters and is slightly preferred due to lower specification complexity.

- [KT] states that Option 1 provides finer‑grained D2R transmit‑power control through pathloss‑based calculation.

- [Fraunhofer] states that Option 1 performs pathloss‑based open‑loop power control but requires explicit pathloss calculation and higher device complexity.

For Option 2, following observations were also provided during the study.

- [OPPO] mentions that in Option 2 the device maps a measured RSRP to a D2R transmit power, but frequent updates of the mapping between RSRP ranges and D2R transmit powers are required when the reader’s Tx power changes, resulting in increased R2D signaling overhead.

- [Ericsson] mentions that within the open‑loop power‑control framework, Option 2 can be considered alongside Option 1.

- [Nokia] notes that Option 2 assumes that the corresponding D2R transmit power includes zero power.

- [ZTE] notes that Option 2 uses an RSRP‑based mapping with complexity comparable to Option 1.

- [vivo] notes that Option 2 provides worse power‑control accuracy, as devices rely only on RSRP ranges and may select identical transmit power within the same range, by which the power control is unachievable within each RSRP range. And suitable RSRP thresholds are difficult for the reader to configure. [For example, the reader cannot be aware of the certain range of RSRP for devices. To reach all the devices, reader has to consider a very wide total RSRP range, leading to the inventory or service latency.]

- [NEC] notes that Option 2 can also be studied, but it relies on the underlying assumption that the reader’s transmit power remains constant.

- [Panasonic] notes that Option 2 offers a simpler RSRP‑based implementation and is therefore the preferred approach for Device 2b/C.

- [Samsung] notes that Option 2 relies on an RSRP‑to‑power lookup table with coarse or dense quantization and is therefore not considered feasible for low‑complexity device operation.

- [Apple] notes that Option 2 offers a simpler implementation suitable for device 2b, making it a more practical choice.

- [ETRI] states that Option 2 reduces complexity but offers only coarse power control due to its RSRP‑range‑based design.

- [Qualcomm] states that Option 2 offers a simpler RSRP‑range‑based approach that aligns well with the practicality of ultra‑low‑power devices and is generally more suitable for such deployments.

- [Quectel] states that Option 2 avoids pathloss calculation but offers only lower‑precision power control despite having similar measurement requirements.

- [ASUSTeK] states that Option 2 uses RSRP‑range‑based power control with lower calculation complexity but involves a trade‑off between required information overhead and power‑control performance.

- [KT] states that Option 2 offers a simpler RSRP‑range‑based method that aligns well with low‑complexity A‑IoT devices and can be enhanced through refined RSRP range design.

- [Fraunhofer] states that Option 2 provides a simpler RSRP‑range‑based open‑loop power control method with configurable power‑ramping behavior, making it suitable for A‑IoT devices with intermittent availability.

| Company | Y/N | Comments |
| --- | --- | --- |
| OPPO | Comment | As for the above Option 1 and Option 2, we have more preference on Option 1. |
| vivo |  | As providing in our contribution“There are two potential options for open loop power control, both of which are feasible.”, we suggest to add our name into the description “For Option 1, sources (FUTUREWEI, Xiaomi, LGE, DOCOMO, TCL, vivo) report that Option 1 is feasible.” and “For Option 2, sources (FUTUREWEI, Xiaomi, LGE, DOCOMO, TCL, vivo) report that Option 2 is feasible”. Besides, according to our observations for these two options, we suggest to add our name into the description “Between Option 1 and Option 2, sources (Xiaomi, ZTE, LGE, ETRI, ASUSTeK, Quectel, TCL, vivo) report that they prefer Option 1 over Option 2”. For option 2, according to our provided observations, we want to supplement as follows: [vivo] notes that Option 2 provides worse power‑control accuracy, as devices rely only on RSRP ranges and may select identical transmit power within the same range, by which the power control is unachievable within each RSRP range. And suitable RSRP thresholds are difficult for the reader to configure. For example, the reader cannot be aware of the certain range of RSRP for devices. To reach all the devices, reader has to consider a very wide total RSRP range, leading to the inventory or service latency. |
| LG | Y |  |
| TCL | Y |  |

Necessity and feasibility of closed-loop Tx power control for D2R

Previous agreements

Agreement (RAN1#122bis)

Study the necessity of D2R Tx power control for Device 2b and for Device C for outdoor scenarios

- Target to agree on observations on pros and cons of D2R Tx power control vs. no Tx power control at RAN1#123

Continue studying feasibility of the following open loop power control methods considering device complexity:

- Methods of open loop power control

  - Option 1: Device calculates Tx power based on measured pathloss, a predefined formula, and some configured parameters from reader

    - Required information at the device side at least includes reader Tx power, received target power at the reader, etc.

  - Option 2: Device measures RSRP, and uses a corresponding Tx power when the RSRP falls in an RSRP range.

    - The RSRP range is based on RSRP threshold(s) configured by reader or predefined

    - Required information at the device side at least includes RSRP threshold(s), corresponding D2R Tx power related information per each RSRP range defined by the RSRP threshold(s)

  - FFS details (e.g., required measurements, required information)

Continue studying the necessity and feasibility of closed loop power control for D2R.

Work plan for RAN1#124 meeting (R1-2509614)

- D2R Tx power control - Necessity of D2R Tx power control (Pros and cons of Tx power control vs. no Tx power control for D2R) - Feasibility study of the two open-loop Tx power control methods for D2R - Necessity and feasibility of closed-loop Tx power control for D2R - Necessary information for D2R Tx power control and its provisioning

Summary

Views on necessity and feasibility of closed-loop Tx power control for D2R

- Both feasible and necessary: DOCOMO

- Necessary: Spreadtrum, OPPO, Nokia, ZTE, Samsung, KT

- [Feasible and] can be beneficial: Qualcomm

- Feasible but not necessary: LGE

- Not feasible: FUTUREWEI, Xiaomi

- Not necessary: vivo, ETRI

- [Closed][H] Proposal 3.3.3a-1

On the necessity and feasibility of closed‑loop Tx power control for D2R, capture the following in TR 38.769.

Necessity and feasibility of closed‑loop Tx power control for D2R are studied. Sources (DOCOMO, Spreadtrum, OPPO, Nokia, ZTE, Samsung, KT) indicate that closed‑loop Tx power control is necessary and source (DOCOMO) further states that it is feasible and necessary.

- [DOCOMO] states that closed‑loop power control can effectively mitigate interference in multi‑reader scenarios, with only minimal additional R2D overhead.

- [Spreadtrum] mentions that because device‑side measurement is difficult, at least closed‑loop D2R power control should be supported, with the reader measuring the D2R signal, determining the appropriate transmit power, and indicating it to the device for power control.

- [OPPO] mentions that closed‑loop power control is suitable for Msg 3 and other D2R data transmissions, where the reader measures the preceding D2R transmission and indicates a power‑control command to the device through the D2R scheduling message.

- [Nokia] observes that in a stationary environment, closed‑loop power control can achieve significantly higher accuracy than open‑loop power control due to device measurement errors, device power‑setting tolerance, and FDD uplink/downlink pathloss differences.

- [ZTE] notes that closed‑loop power control enables devices to transmit at a configured power level through R2D control signaling.

- [Samsung] notes that closed‑loop D2R power control should be studied, as reader feedback can refine device transmit power beyond OLPC accuracy.

- [KT] states that closed‑loop power control should be studied for D2R transmission, as it enables real‑time power adjustment based on reader feedback.

Source (LGE) states that closed‑loop Tx power control is feasible but not necessary.

- [LGE] notes that closed‑loop power control is feasible but not essential compared to open‑loop power control.

Source (Qualcomm) states that closed‑loop Tx power control [is feasible and] can be beneficial.

- [Qualcomm] states that the closed-loop power control [is feasible and] can be beneficial for interference management of device-specific D2R transmission at the price of higher overhead and complexity.

Sources (FUTUREWEI, Xiaomi) state that closed‑loop Tx power control is not feasible.

- [FUTUREWEI] mentions that closed‑loop power control is not feasible, noting that it introduces additional signaling overhead for power adjustment, is limited by device availability, and still requires support for open‑loop power control for the initial D2R transmission.

- [Xiaomi] notes that closed‑loop power control increases device power consumption due to continuous monitoring and is therefore not feasible for A‑IoT devices with limited power and low complexity.

Sources (vivo, ETRI) state that closed‑loop Tx power control is not necessary.

- [vivo] notes that closed‑loop power control is unnecessary for R20 A‑IoT devices if open‑loop power control is introduced, given the added design complexity and unclear HARQ‑ACK feedback mechanism.

- [ETRI] states that closed‑loop power control is not considered for Rel‑20 A‑IoT due to its signaling and energy overhead.

Source (Panasonic) states that the closed-loop power control would increase the signaling overhead and the device complexity, but a device/group-specific correction parameter could be considered along with the open-loop power control for better power control flexibility.

| Company | Y/N | Comments |
| --- | --- | --- |
| LG | Y |  |
| Qualcomm |  | Pls also capture our observations: [Qualcomm] The closed-loop power control is feasible and beneficial for interference management of device-specific D2R transmission at the price of higher overhead and complexity. |
| Xiaomi | N | Our proposal is not captured completely, and we would appreciate it if the following description is adopted. [Xiaomi] notes that closed‑loop power control increases device power consumption due to continuous monitoring and is therefore not feasible for A‑IoT devices with limited power and low complexity. |
| Panasonic |  | Please capture our observation: [Panasonic] The closed-loop power control would increase the signaling overhead and the device complexity. However, a device/group-specific correction parameter could be considered along with the open-loop power control for better power control flexibility. |
| Ericsson |  | Please add the following Ericsson’s view, which can be find in out T-doc in section 3 observation 6, which is specially import to consider for a close-loop power control due to e.g., the additional complexity and signalling overhead: [Ericsson] The benefit of having in place a power control mechanism for A-IoT devices must be pondered with respect to any measurements, power consumption and signalling required for it to work, which are aspects that may go against the necessity of it |

Necessary information for D2R Tx power control and its provisioning

Previous agreements

Agreement (RAN1#122)

Study necessity and feasibility of D2R Tx power control for Device 2b and for Device C, including the following aspects

- Open loop and/or closed loop power control

- Required measurements (e.g., signal strength)

- Required information at the device

- Required information at reader side

- Signaling overhead and power consumption

Work plan for RAN1#124 meeting (R1-2509614)

- D2R Tx power control - Necessity of D2R Tx power control (Pros and cons of Tx power control vs. no Tx power control for D2R) - Feasibility study of the two open-loop Tx power control methods for D2R - Necessity and feasibility of closed-loop Tx power control for D2R - Necessary information for D2R Tx power control and its provisioning

Summary

Views on necessary information for D2R Tx power control and its provisioning

- Required information at the device

  - Tx power of the reader: OPPO, Samsung, Qualcomm (Option 1)

  - Coefficient of pathloss: DOCOMO (Option 1)

  - Target received power at reader: Qualcomm (Option 1)

  - RSRP thresholds: Qualcomm (Option 2)

  - Tx power mapping per RSRP range: Qualcomm (Option 2)

- Required information at reader side

  - Available power of A-IoT device: Nokia

  - Device energy level: InterDigital

- [Closed][H] Proposal 3.3.4a-1

On the necessary information for D2R Tx power control and its provisioning, capture the following in TR 38.769.

On the required information for D2R Tx power control at the reader and device, sources (OPPO, Samsung, Qualcomm, DOCOMO) state that Tx‑power–related parameters are necessary.

- Sources (OPPO, Samsung, Qualcomm) state that the Tx power of the reader can be set by the reader.

- Source (DOCOMO) states that coefficient of pathloss can be beneficial for balancing between inter-cell interference and performance at cell edge for Option 1.

- Source (Qualcomm, Panasonic) states that the target received power at the reader can be configured by the reader for Option 1.

- Source (Qualcomm, Panasonic) states that the RSRP thresholds and the Tx‑power mapping per RSRP range can be configured by the reader for Option 2.

Sources (Nokia, InterDigital) state that device power/energy‑related information is needed [for closed-loop Tx power control for D2R].

- Source (Nokia) states that at least the available power of an A‑IoT device can be reported by the device.

- Source (InterDigital) states that D2R power control depends on the device energy level.

| Company | Y/N | Comments |
| --- | --- | --- |
| LG | Y |  |
| Panasonic |  | Please capture our view: - Source (Qualcomm , Panasonic) states that the target received power at the reader can be configured by the reader for Option 1. - Source (Qualcomm, Panasonic) states that the RSRP thresholds and the Tx‑power mapping per RSRP range can be configured by the reader for Option 2. |

Random access

Previous agreements

Agreement (RAN1#122)

At least for CBRA for DT and DO-DTT, study necessity and feasibility of enhancements of D2R TDM(A) for Device 2b and for Device C based on the following:

- X1 > 2 for Msg1 (X1: number of time domain resources for Msg1 transmission indicated by the corresponding R2D message)

- X3 > 1 for Msg3 (X3: number of time domain resources for Msg3 transmission indicated by the corresponding R2D message)

Agreement (RAN1#122bis)

Study necessity and feasibility of D2R CDM(A) for Device 2b and for Device C with sequence-based Msg1 transmission and potential impact to random access procedures.

- FFS: other cases, except for data transmission

Necessity and feasibility of X1>2 for Msg1 and X3>1 for Msg3

Previous agreements

Agreement (RAN1#122)

At least for CBRA for DT and DO-DTT, study necessity and feasibility of enhancements of D2R TDM(A) for Device 2b and for Device C based on the following:

- X1 > 2 for Msg1 (X1: number of time domain resources for Msg1 transmission indicated by the corresponding R2D message)

- X3 > 1 for Msg3 (X3: number of time domain resources for Msg3 transmission indicated by the corresponding R2D message)

Work plan for RAN1#124 meeting (R1-2509614)

- Enhancements on random access from procedural aspects - Necessity and feasibility of X1>2 for Msg1 and X3>1 for Msg3 - Potential impact of D2R CDM(A) on random access procedures - Potential impact of R2D FDM on random access procedures

Summary

Views on necessity and feasibility of X1>2 for Msg1 and X3>1 for Msg3

- X1>2 for Msg1 is

  - both feasible and necessary: FUTUREWEI, ETRI, LGE, DOCOMO, ASUSTeK

  - feasible: InterDigital, Qualcomm, Samsung

  - feasible but not necessary: vivo, ZTE

  - not necessary: Huawei, Xiaomi

- X3>1 for Msg3 is

  - both feasible and necessary: FUTUREWEI, ETRI, LGE, DOCOMO, ASUSTeK

  - feasible: Samsung, InterDigital, Qualcomm

  - feasible but not necessary: vivo

  - not necessary: Huawei

  - not feasible: Xiaomi

  - neither feasible nor necessary: ZTE

- [H] Proposal 3.4.1a-1 [Further grouping needed]

On enhancements of D2R TDM(A) for Device 2b and for Device C for CBRA for DT and DO-DTT, capture the following in TR 38.769.

On enhancements to increase the number of time domain resources indicated by the corresponding R2D message for the Access Random ID message (Msg1) to greater than 2, sources (FUTUREWEI, LGE, ETRI, DOCOMO, ASUSTeK, InterDigital, Qualcomm, Samsung, vivo, ZTE, CMCC, , Panasonic, NEC, [Sharp]) state that it is feasible. Sources (FUTUREWEI, ETRI, LGE, DOCOMO, ASUSTeK, [Sharp]) further state that it is feasible and necessary, and sources (vivo, ZTE) state that it is feasible but not necessary. Sources (Huawei, Xiaomi) report that it is not necessary. Following observations were provided during the study.

- [FUTUREWEI] mentions that increasing X1 for Msg1 transmission is necessary, as the larger PRDCH overhead can be mitigated by allocating more time‑domain resources.

- [LGE] notes that the smaller residual SFO of Device 2b/C may enable the use of more than two TDM resources for Msg1. (e.g., X1 > 2).

- [ETRI] states that supporting more than two Msg1 time‑domain access occasions (X1 > 2) is feasible and necessary for device 2b and device C.

- [DOCOMO] states that supporting X1 > 2 for Msg1 can enhance multiplexing capacity and shorten the overall random‑access procedure.

- [ASUSTeK] states that supporting X1 = {1, 2, 4, 8} for Msg1 is reasonable given the larger coverage and improved SFO capability of active devices.

- [Samsung] notes that with the improved timing accuracy of Device 2b/C, X1 for Msg1 transmission can be increased (e.g., 4) though it must remain limited due to A‑IoT device constraints.

- [InterDigital] states that supporting X1 > 2 for Msg1 is feasible in Rel‑20 because active devices have higher capability and more accurate clocks.

- [Qualcomm] states that supporting X1 > 2 for Msg1 is feasible because device 2b/C can use more accurate clocks and calibration to reduce the required time gap between TDMed D2Rs.

- [ZTE] notes that increasing X1 is feasible due to smaller SFO and reduced guard time for Device 2b/C, but its necessity remains unclear because a larger X1 introduces multiple drawbacks and therefore requires further evaluation.

- [vivo] notes that although X1 > 2 for Msg1 is feasible due to the reduced SFO in Rel‑20, its necessity is low because the resulting latency‑reduction gain is very limited.

- [Huawei] mentions that larger values of X1 for Msg1 are unnecessary because the performance gain is limited and further decreases as X1 increase.

- [Xiaomi] notes that it is not necessary to support X1 > 2 for Msg1, as the 16 Msg1 transmission resources enabled by 8 FDM + 2 TDM D2R in Rel‑19 are already sufficient.

- [CATT] notes that the maximum number of time domain resources for Msg1 transmission should be limited to 11 for Device 2b/C, as larger values provide limited performance gain.

- [CMCC] notes that supporting X1 > 2 for Msg1 can be studied.

- [Panasonic] notes that supporting X1 > 2 for Msg1 transmissions could be considered due to the lower SFO of Device 2b/C.

- [Apple] states that configurable X1 values such as 2, 4, and 8 help reduce Msg1 collision probability in high‑density environments.

- [Ericsson] mentions that it should be determined whether X1 = 2 is sufficient for outdoor capacity and latency requirements, and that any need for enhancement beyond X1 = 2 for Msg1 should be justified with supporting evaluations.

- [Panasonic] states that the Device types 2b and C have more accurate oscillators with lower SFO. To reduce the signaling overhead for Msg1 trigger signal and Msg2, larger transmission occasions could be considered, i.e., X1>2 and X3>1.

- [NEC] states that X1 values such as 4 and 8 to provide more efficient access.

On enhancements to increase the number of time domain resources indicated by the corresponding R2D message for the D2R Upper Layer Data Transfer message (Msg3) to greater than 1, sources (FUTUREWEI, LGE, ETRI, DOCOMO, ASUSTeK, Samsung, InterDigital, Qualcomm, vivo, CMCC, Panasonic, NEC) state that it is feasible. Sources (FUTUREWEI, LGE, ETRI, DOCOMO, ASUSTeK) further state that it is feasible and necessary, and source (vivo) states that it is feasible but not necessary. Source (Huawei) reports that it is not necessary, source (Xiaomi) states that it is not feasible, and source (ZTE) states that is neither feasible nor necessary. Following observations were provided during the study.

- [FUTUREWEI] mentions that supporting X3 > 1 is feasible due to the higher clock accuracy of Device 2b/C, although increasing X3 beyond 2 is unlikely because of Msg3 length and clock drift.

- [LGE] notes that the smaller residual SFO of Device 2b/C may enable the use of multiple TDM resources for Msg3 (e.g., X3 > 1).

- [ETRI] states that supporting more than one Msg3 time‑domain access occasion (X3 > 1) is feasible and necessary for device 2b and device C.

- [DOCOMO] states that supporting X3 > 1 for Msg3 can improve random‑access efficiency and reduce total access time.

- [ASUSTeK] states that enhancements on X3 > 1 should be limited to X3 = 2, since supporting higher X3 adds design and signaling complexity without clear benefit.

- [Samsung] notes that the X3 capacity for Msg3 transmission should match that of Msg1 and can likewise be increased (e.g., 4) rather than being restricted to half as in Rel‑19.

- [InterDigital] states that supporting X3 > 1 for Msg3 is feasible in Rel‑20 with shorter gaps between time resources compared to Rel‑19.

- [Qualcomm] states that supporting X3 > 1 for Msg3 is feasible since improved clock accuracy of device 2b/C makes the time‑gap overhead marginal even for longer Msg3 durations.

- [vivo] notes that although X3 > 1 for Msg3 is feasible, its necessity is low given the similarly limited latency‑reduction benefit.

- [Huawei] mentions that larger values of X3 for Msg3 are unnecessary because the performance gain is limited and further decreases as X3 increase.

- [Xiaomi] notes that it is not feasible to support X3 > 1 for Msg3, as the device ID can be several hundred bits long.

- [ZTE] notes that supporting X3 > 1 is technically feasible but brings significant performance and scheduling penalties, so its necessity is weak and requires further study.

- [CATT] notes that TDMA support for Msg3 can reduce transmission latency, though the latency‑reduction benefit diminishes as the TDMA size increases, and at least X3 = 2 should be supported for Device 2b/C.

- [CMCC] notes that supporting X3 > 1 for Msg3 can be studied.

- [Panasonic] notes that supporting X3 > 1 for Msg3 transmissions could also be considered given the improved oscillator accuracy of Device 2b/C.

- [Apple] states that configurable X3 values greater than 1 allow parallel Msg3 processing and improve random‑access performance.

- [Ericsson] mentions that from the device perspective, X3 should be 1.

- [Panasonic] states that the Device types 2b and C have more accurate oscillators with lower SFO. To reduce the signaling overhead for Msg1 trigger signal and Msg2, larger transmission occasions could be considered, i.e., X1>2 and X3>1.

- [NEC] states that X3 values such as 2 and 4 to provide more efficient access.

| Company | Y/N | Comments |
| --- | --- | --- |
| LG | Y |  |
| CMCC |  | Our contribution provide the following text and proposal, and we think it is feasible when R19 random access procedure is reused, but whether R19 random access procedure is reused or not considering the more accurate timing and specification aspects for active devices can be another discussion. ① Msg1 transmission Thanks to better clock accuracy, the time gap between adjacent Msg1 occasions caused by SFO is now only ~0.01 % level of the Msg1 duration (100 ppm). This gap is far smaller than the Msg1 duration, so the value of X1 for Msg1 (X1: number of time domain resources for Msg1 transmission indicated by the corresponding R2D message) can be increased to reduce Access trigger commands and reduce inventory time. Then Toffset for the Msg1 occasions corresponding to X1>1 should be defined. Proposal 13: If R19 random access procedure is reused for R20 active devices, the following modification can be studied, - Support X1 > 2 for Msg1 (X1: number of time domain resources for Msg1 transmission indicated by the corresponding R2D message), and the related Toffset values for occasions with X1>2. - Increase the maximum echo random IDs in Msg2, and the frequency resource indication in Msg2 for Msg3 scheduling needs update. - Support X3 > 1 for Msg3 (X3: number of time domain resources for Msg3 transmission indicated by the corresponding R2D message), and the related Toffset values for occasions with X3>1 - Support Msg3 retransmission. So we propose to add CMCC to the sources state it is feasible. “On enhancements to increase the number of time domain resources indicated by the corresponding R2D message for the Access Random ID message (Msg1) to greater than 2, sources (FUTUREWEI, LGE, ETRI, DOCOMO, ASUSTeK, InterDigital, Qualcomm, Samsung, vivo, ZTE, CMCC) state that it is feasible” “On enhancements to increase the number of time domain resources indicated by the corresponding R2D message for the D2R Upper Layer Data Transfer message (Msg3) to greater than 1, sources (FUTUREWEI, LGE, ETRI, DOCOMO, ASUSTeK, Samsung, InterDigital, Qualcomm, vivo, CMCC) state that it is feasible. ” |
| Sharp | Y | Please add Sharp to the support list, thanks, On enhancements to increase the number of time domain resources indicated by the corresponding R2D message for the Access Random ID message (Msg1) to greater than 2, sources (FUTUREWEI, LGE, ETRI, DOCOMO, ASUSTeK, InterDigital, Qualcomm, Samsung, vivo, ZTE, Sharp) state that it is feasible. Sources (FUTUREWEI, ETRI, LGE, DOCOMO, ASUSTeK,Sharp) further state that it is feasible and necessary, and sources (vivo, ZTE) state that it is feasible but not necessary. Sources (Huawei, Xiaomi) report that it is not necessary. Following observations were provided during the study. |
| Panasonic |  | Please capture our view: So we propose to add CMCC to the sources state it is feasible. “On enhancements to increase the number of time domain resources indicated by the corresponding R2D message for the Access Random ID message (Msg1) to greater than 2, sources (FUTUREWEI, LGE, ETRI, DOCOMO, ASUSTeK, InterDigital, Qualcomm, Samsung, vivo, ZTE, CMCC, Panasonic) state that it is feasible” “On enhancements to increase the number of time domain resources indicated by the corresponding R2D message for the D2R Upper Layer Data Transfer message (Msg3) to greater than 1, sources (FUTUREWEI, LGE, ETRI, DOCOMO, ASUSTeK, Samsung, InterDigital, Qualcomm, vivo, CMCC, Panasonic) state that it is feasible. ” |
| NEC |  | We put analysis of X1 and X3 in AI 9.3.2.2 (R1-2600660) with the following proposal: Proposal 7: At least for CBRA for DT and DO-DTT, X1= {4, 8} for Msg1 and X3= {2, 4} for Msg3 can be starting point as the enhancements of D2R TDM(A) for Device 2b and for Device C. - Additional bits in D2R Scheduling Info are needed which is up to RAN2 discussion. Please add us to supporting company for both X1 and X3. Suggestion to changes are: - [NEC] states that X1 values such as 4 and 8 to provide more efficient access. - [NEC] states that X3 values such as 2 and 4 to provide more efficient access. |
| CATT | Y | OK with the proposal |

Potential impact of D2R CDM(A) on random access procedures

Previous agreements

Agreement (RAN1#122bis)

Study necessity and feasibility of D2R CDM(A) for Device 2b and for Device C with sequence-based Msg1 transmission and potential impact to random access procedures.

- FFS: other cases, except for data transmission

Work plan for RAN1#124 meeting (R1-2509614)

- Enhancements on random access from procedural aspects - Necessity and feasibility of X1>2 for Msg1 and X3>1 for Msg3 - Potential impact of D2R CDM(A) on random access procedures - Potential impact of R2D FDM on random access procedures

Necessity and feasibility of D2R CDM(A) for Device 2b and for Device C with sequence-based Msg1 transmission will be discussed in sub-agenda 10.3.2.2.

Summary

Views on the potential impact of D2R CDM(A) on random access procedures

- If supported, random access procedure would need updates: FUTUREWEI, Huawei, LGE, DOCOMO, Qualcomm

- [Closed][H] Proposal 3.4.2a-1

On potential impact of D2R CDM(A) on random access procedure for Device 2b and for Device C, capture the following in TR 38.769.

Potential impact of D2R CDM(A) on random access procedure for Device 2b and for Device C is studied and the following observations were provided during the study.

- [FUTUREWEI] mentions that although CDMA can increase the number of supported Msg1 transmissions in the contention‑based access procedure, it would modify the handling of Msg2 and the scheduling of Msg3, while the dominant bottleneck remains the much larger Msg3 transmission.

- [Huawei] mentions that if sequence‑based Msg1 is introduced, the RA procedure becomes very similar to the legacy NR 4‑step RACH procedure and can be handled by RAN2 during the WI phase.

- [LGE] notes that the necessity of Msg1 CDMA remains unclear given existing Msg1 TDMA/FDMA designs, but if required, binary sequence–based Msg1 and related procedural enhancements—including sequence indication via Msg2 and predefined D2R modulation—should be considered.

- [NEC] notes that D2R CDM(A) for sequence‑based Msg1 is challenging for Device 2b due to its residual SFO/CFO and may also impact the random‑access procedure, requiring consideration of a four‑step RA process.

- [DOCOMO] states that collision handling for sequence‑based Msg1 should be studied and can be resolved with minimal enhancements in the RAN2 random‑access procedure.

- [Qualcomm] states that explicit or implicit contention resolution using Msg3 and Msg4 should be considered when sequence‑based CDMA is used for Msg1 in the CBRA procedure.

- [vivo] states that if msg1 is sequence-based, instead of RN16, contention resolution i.e., msg4 is needed for random access procedure, aiming to deal with the msg1 collision.

- [CMCC] states that contention resolution using Msg3 and Msg4 should be considered when sequence‑based CDMA is used for Msg1 in the CBRA procedure.

- [[CATT] states that sequence-based Msg1 could be supported. The length of sequences should not be shorter than 31. The NR preamble number could be as starting point for Device 2b/C in outdoor scenario.]

| Company | Y/N | Comments |
| --- | --- | --- |
| vivo |  | According to our observation below, we suggest to supplement it into proposal 3.4.2a. Observation 2: If msg1 is sequence-based, instead of RN16, contention resolution i.e., msg4 is needed for random access procedure, aiming to deal with the msg1 collision. |
| LG | Y |  |

| CMCC |  | To our understanding, current R19 random access will lead to high collision probability, so we need to support sequence based Msg1 and support CDM. And then contention is resolved via Msg3 and Msg4. Observation 5: Reusing the R19 Msg1+Msg2 collision resolution procedure for outdoor active device random access leads to high collision probability and excessive D2R resource consumption. Proposal 8: Study for R20 a random access procedure update, including 1)sequence-based Msg1 is used and carries no Random ID, 2) contention is resolved via Msg3 and Msg4. Proposal 9: Study CDM for Msg1 to reduce random access collision. We propose to capture our observation as following, - [CMCC] states that contention resolution using Msg3 and Msg4 should be considered when sequence‑based CDMA is used for Msg1 in the CBRA procedure. |
| --- | --- | --- |
| ZTE, Sanechips |  | As CDM(A) for sequence-based Msg1 is also discussed in Section 9.3.2.2, Proposal 3.4.2a shall avoid to capture duplicate content here. |
| CATT |  | Please include following descriptions into the proposal, as described in our contribution: The sequence-based Msg1 could be supported. The length of sequences should not be shorter than 31. The NR preamble number could be as starting point for Device 2b/C in outdoor scenario. |

Potential impact of R2D FDM on random access procedures

Work plan for RAN1#124 meeting (R1-2509614)

- Enhancements on random access from procedural aspects - Necessity and feasibility of X1>2 for Msg1 and X3>1 for Msg3 - Potential impact of D2R CDM(A) on random access procedures - Potential impact of R2D FDM on random access procedures

Necessity and feasibility will be discussed in sub-agenda 10.3.2.1.

Summary

Other than the necessity and feasibility of R2D FDM which will be discussed in sub-agenda 10.3.2.1, no specific impact of R2D FDM on random access procedures were identified from the contributions in this meeting.

| Company | Y/N | Comments |
| --- | --- | --- |

Tx timing

Previous agreements

Agreement (RAN1#122)

At least for DT and DO-DTT, study necessity and feasibility of changes to existing timing offsets for Device 2b and for Device C, e.g.,

- timing offset value between R2D and corresponding subsequent D2R (e.g., Toffset1/2/3/4)

- timing offset value between D2R and corresponding subsequent R2D (e.g., T_D2R_min)

  - Note: RAN1 only discusses potential reasons that may impact the value of T_D2R_min, and if any provides those reasons to RAN4

- timing offset between R2D and corresponding subsequent D2R provided by a reader

- timing offset between D2R and corresponding subsequent R2D provided by a reader

At least for DT and DO-DTT, study necessity and feasibility of introduction of new timing offsets for Device 2b and for Device C, e.g.,

- timing offset between two consecutive R2D or D2R transmissions for the same Device, if such scenario exists

Necessity and feasibility of changes to existing timing offsets (Toffset1/2/3/4, T_D2R_min)

Previous agreements

Agreement (RAN1#122)

At least for DT and DO-DTT, study necessity and feasibility of changes to existing timing offsets for Device 2b and for Device C, e.g.,

- timing offset value between R2D and corresponding subsequent D2R (e.g., Toffset1/2/3/4)

- timing offset value between D2R and corresponding subsequent R2D (e.g., T_D2R_min)

  - Note: RAN1 only discusses potential reasons that may impact the value of T_D2R_min, and if any provides those reasons to RAN4

- timing offset between R2D and corresponding subsequent D2R provided by a reader

- timing offset between D2R and corresponding subsequent R2D provided by a reader

At least for DT and DO-DTT, study necessity and feasibility of introduction of new timing offsets for Device 2b and for Device C, e.g.,

- timing offset between two consecutive R2D or D2R transmissions for the same Device, if such scenario exists

Work plan for RAN1#124 meeting (R1-2509614)

- Timing offsets - Necessity and feasibility of changes to existing timing offsets (Toffset1/2/3/4, T_D2R_min) - Necessity and feasibility of dynamic indication of the existing timing offsets - Necessity and feasibility of introduction of new timing offsets (T_R2D_R2D_min, T_D2R_D2R_min)

Summary

Views on necessity and feasibility of changes to Toffset1/2/3/4

- Changes to existing timing offsets (Toffset1/2/3/4) is necessary: FUTUREWEI, Xiaomi, ZTE, CATT, CMCC, NEC, ETRI, LGE, TCL, DOCOMO

Views on necessity and feasibility of changes to T_D2R_min

- T_D2R_min can be changed (e.g., due to RF switching, R2D FDM, R2D FEC/repetition, Tx-Rx switching): FUTUREWEI, OPPO, vivo, TCL

- No need to change: ZTE, DOCOMO

- [Closed][H] Proposal 3.5.1a-1

At least for CBRA for DT and DO-DTT, regarding the changes to existing timing offset value b/w R2D and corresponding subsequent D2R (e.g., Toffset1/2/3/4) for Device 2b and for Device C, capture the following observations in TR 38.769:

Sources (FUTUREWEI, CATT, CMCC, Xiaomi, ZTE, LGE, NEC, ETRI, TCL, DOCOMO, Huawei, vivo, OPPO, Qualcomm) state that changes to existing timing offset values Toffset1/2/3/4 are necessary. Following observations were provided.

- [FUTUREWEI] mentions that the values of Toffset1, Toffset3, and Toffset4 may vary depending on the PRDCH and PDRCH payload lengths, while Toffset2 is expected to be smaller due to improved SFO but may still reflect propagation‑delay effects.

- [CATT] observes that the timing‑offset configuration should be updated in Rel‑20 to reflect the revised maximum numbers of time‑domain resources for Msg1/Msg3, the improved SFO accuracy of Device 2b/C, and the FEC scheme for R2D transmission.

- [CMCC] notes that the corresponding Toffset values need to be updated for occasions with X1 > 2 and X3 > 1.

- [Xiaomi] notes that the existing timing‑offset values (e.g., Toffset1/2) need to be updated for Device 2b/C, given their smaller SFO values and the potential additional R2D decoding time when R2D FEC is considered.

- [ZTE] notes that new values of Toffset1/2/3/4 need to be considered for Device 2b/C, as their R2D/D2R processing characteristics and SFO values differ from those of Device 1.

- [LGE] notes that supporting multiple TDM resources may require increasing TR→D for reliable reception and synchronization, and that introducing R2D FEC may further increase the minimum TR→D due to device‑side decoding time and the minimum TD→R due to reader‑side PRDCH encoding time.

- [NEC] notes that timing coefficients and related parameters should be refined in Rel‑20 due to improved SFO for Device 2b, enhanced processing capability of active devices, the use of multiple Msg1/Msg3 transmissions, and the additional delay introduced by R2D FEC decoding.

- [ETRI] states that Toffset2 as well as Toffset1, Toffset3, and Toffset4 should be revised in Rel‑20 A‑IoT to reflect improved device SFO performance and updated R2D/D2R air‑interface assumptions.

- [TCL] states that the timing offset TR→D​ for MSG1, MSG3, and upper‑layer data should be updated for device 2b/C due to different processing time compared with device 1.

- [DOCOMO] states that timing offsets between R2D and the corresponding D2R should be revisited in Rel‑20 considering improved device capabilities, potential new processing requirements, updated chip duration, and RTT/SFO impacts, and that using reader‑provided timing offsets can offer better scheduling flexibility.

- [Huawei] mentions that if the number of time‑domain resources for Msg1 or Msg3 exceeds that in Rel‑19, additional timing offsets would be required, making the use of predefined timing offsets overly complex and therefore not supportable.

- [vivo] notes that active A‑IoT devices should operate in half‑duplex mode, and the minimum timing offset between R2D and the subsequent D2R (e.g., Toffset1/2/3/4) for Device 2b/C should account for Rx–Tx switching delay—around 1 ms as referenced from NB‑IoT—and any R2D FEC processing delay if supported.

- [OPPO] states that if the same self-calculated approach is reused for device 2b/C, at least Toffset1/2/3/4 can be redefined to optimize both the latency and capacity. However, when the X number of time domain resources is increased (e.g., X=4) for Msg1 or Msg3 transmission for device 2b/C, additional timing offset would be needed for the 3rd and 4th time domain resources.

| Company | Y/N | Comments |
| --- | --- | --- |
| OPPO | Comment | As described in our Tdoc, we also think it is necessary to update the existing timing offset value Toffset1/2/3/4. “As such, if the same self-calculated approach is reused for device 2b/C, at least Toffset1/2/3/4 can be redefined to optimize both the latency and capacity. However, when the X number of time domain resources is increased (e.g., X=4) for Msg1 or Msg3 transmission for device 2b/C, additional timing offset would be needed for the 3rd and 4th time domain resources.” |
| LG | Y |  |
| Qualcomm |  | Pls also capture our views: Sources (FUTUREWEI, CATT, CMCC, Xiaomi, ZTE, LGE, NEC, ETRI, TCL, DOCOMO, Qualcomm) state that changes to existing timing offset values Toffset1/2/3/4 are necessary. |
| CATT | Y | OK with the proposal |
| TCL | Y |  |

- [Closed][H] Proposal 3.5.1b-1

At least for CBRA for DT and DO-DTT, regarding the changes to existing timing offset value b/w D2R and corresponding subsequent R2D (e.g., T_D2R_min) for Device 2b and for Device C, capture the following observations in TR 38.769:

Sources (FUTUREWEI, OPPO, vivo, TCL) state that changes to existing timing offset T_D2R_min are necessary.

- [FUTUREWEI] mentions that TD2Rmin is expected to change due to the RF switch occurring.

- [OPPO] provides to RAN4 that TD2R_min may be impacted by increased D2R resource usage, the introduction of FDM for R2D, higher R2D‑generation complexity (e.g., FEC/repetition), and device frequency switching between UL and DL.

- [vivo] notes that the Tx–Rx switching delay imposed by half‑duplex operation should be an additional factor affecting TD2R_min for Device 2b/C.

- [TCL] states that the minimum timing offset TD2R_min​ should also be changed for device 2b/C relative to device 1.

while On the other hand, sources (ZTE, DOCOMO) state that there is no need to change at least from RAN1 perspective. Following observations were provided.

- [ZTE] notes that the value of TD2R_min for Device 2b/C is determined by RAN4, as no additional RAN1‑side factors affecting this parameter are introduced in Rel‑20.

- [DOCOMO] states that the value of T_D2R_min for device 2b/C does not need to change from Rel‑19.

| Company | Y/N | Comments |
| --- | --- | --- |
| LG | Y |  |
| TCL | Y |  |

Necessity and feasibility of dynamic indication of the existing timing offsets

Previous agreements

Agreement (RAN1#122)

At least for DT and DO-DTT, study necessity and feasibility of changes to existing timing offsets for Device 2b and for Device C, e.g.,

- timing offset value between R2D and corresponding subsequent D2R (e.g., Toffset1/2/3/4)

- timing offset value between D2R and corresponding subsequent R2D (e.g., T_D2R_min)

  - Note: RAN1 only discusses potential reasons that may impact the value of T_D2R_min, and if any provides those reasons to RAN4

- timing offset between R2D and corresponding subsequent D2R provided by a reader

- timing offset between D2R and corresponding subsequent R2D provided by a reader

At least for DT and DO-DTT, study necessity and feasibility of introduction of new timing offsets for Device 2b and for Device C, e.g.,

- timing offset between two consecutive R2D or D2R transmissions for the same Device, if such scenario exists

Work plan for RAN1#124 meeting (R1-2509614)

- Timing offsets - Necessity and feasibility of changes to existing timing offsets (Toffset1/2/3/4, T_D2R_min) - Necessity and feasibility of dynamic indication of the existing timing offsets - Necessity and feasibility of introduction of new timing offsets (T_R2D_R2D_min, T_D2R_D2R_min)

Summary

Views on necessity and feasibility of dynamic indication of the existing timing offsets

- Timing offset provided by a reader is both feasible and necessary: FUTUREWEI, Huawei, vivo, ETRI, Qualcomm

- Timing offset provided by a reader is not necessary: Spreadtrum, CATT, LGE

- [Closed][H] Proposal 3.5.2a

At least for DT and DO-DTT, regarding the necessity and feasibility of timing offset provided by a reader for Device 2b and for Device C, capture the following observations in TR 38.769:

Sources (FUTUREWEI, Huawei, vivo, ETRI, Qualcomm) state that the timing offset provided by a reader is both feasible and necessary.

- [FUTUREWEI] mentions that determining the timing values is better handled at the reader side due to their dependence on PRDCH and PDRCH payload lengths, and that the reader‑computed value(s) can be included in the PRDCH payload portion and stored for contention‑based procedures.

- [Huawei] mentions that indicating the time offset between R2D and the corresponding D2R at the reader side is feasible, beneficial, and should be supported, whereas providing the timing offset between a D2R transmission and the subsequent R2D transmission is unnecessary since the device is expected to continuously receive the corresponding R2D message after transmitting the D2R message subject to processing‑delay requirements.

- [vivo] notes that a reader‑indicated timing offset between R2D and the subsequent D2R transmission is both feasible, given achievable timing alignment in Rel‑20 A‑IoT, and necessary to accommodate new R2D transmission types (e.g., periodic sync signal) and improve scheduling flexibility.

- [ETRI] states that reader‑indicated dynamic timing offsets between R2D and the corresponding D2R transmission are feasible in Rel‑20 A‑IoT.

- [Qualcomm] states that a configurable time offset between R2D and the corresponding D2R, and between D2R and the corresponding R2D, should be considered, as delayed scheduling with such an offset is noted as feasible and beneficial for supporting active devices.

Sources (Spreadtrum, CATT, LGE) state that the timing offset provided by a reader is not necessary.

- [Spreadtrum] mentions not supporting the indication of existing timing offsets between R2D and the corresponding D2R, or between D2R and the corresponding R2D, for Device 2b/C.

- [CATT] considers that dynamic signaling for indicating the timing offset for R2D and D2R transmission is unnecessary in Rel‑20.

- [LGE] notes that providing the timing offset between R2D and the subsequent D2R (or vice versa) helps the device anticipate the next event for timely CFO calibration and synchronization, but a long interval may introduce frequency drift and degrade the accuracy of the subsequent transmission or reception.

On the time offset provided by a reader, following observations were also provided during the study.

- [Ericsson] mentions that the reader can indicate Toffset1 through broadcast information and can also indicate Toffset3 and Toffset4 via higher layer or L1 control information in the corresponding PRDCH.

- [CMCC] states that the Rel‑20 timing design between R2D and the subsequent D2R transmission should support time‑offset indication in the R2D message.

- [Samsung] notes that indicating TR2D/TD2R timing relationships, including the timing of each access occasion in TDMA‑based access, in the preceding PRDCH should be studied to provide flexibility beyond the fixed and overly pessimistic timing defined in Rel‑19.

- [DOCOMO] states that device 2b/C can save energy if the R2D reception timing is indicated by the reader.

| Company | Y/N | Comments |
| --- | --- | --- |
| LG | Y |  |
| CATT | Y | OK with the proposal |

Necessity and feasibility of introduction of new timing offsets (T_R2D_R2D_min, T_D2R_D2R_min)

Previous agreements

Agreement (RAN1#122)

At least for DT and DO-DTT, study necessity and feasibility of changes to existing timing offsets for Device 2b and for Device C, e.g.,

- timing offset value between R2D and corresponding subsequent D2R (e.g., Toffset1/2/3/4)

- timing offset value between D2R and corresponding subsequent R2D (e.g., T_D2R_min)

  - Note: RAN1 only discusses potential reasons that may impact the value of T_D2R_min, and if any provides those reasons to RAN4

- timing offset between R2D and corresponding subsequent D2R provided by a reader

- timing offset between D2R and corresponding subsequent R2D provided by a reader

At least for DT and DO-DTT, study necessity and feasibility of introduction of new timing offsets for Device 2b and for Device C, e.g.,

- timing offset between two consecutive R2D or D2R transmissions for the same Device, if such scenario exists

Work plan for RAN1#124 meeting (R1-2509614)

- Timing offsets - Necessity and feasibility of changes to existing timing offsets (Toffset1/2/3/4, T_D2R_min) - Necessity and feasibility of dynamic indication of the existing timing offsets - Necessity and feasibility of introduction of new timing offsets (T_R2D_R2D_min, T_D2R_D2R_min)

Summary

Views on necessity and feasibility of introduction of new timing offsets (T_R2D_R2D_min, T_D2R_D2R_min)

- Introduction of new timing offsets (TR2D_R2D_min) is

  - both feasible and necessary: FUTUREWEI, vivo, LGE

  - not necessary: ETRI

- Introduction of new timing offsets (TD2R_D2R_min) is

  - both feasible and necessary:

  - not necessary: FUTUREWEI, ETRI

- [Closed][H] Proposal 3.5.3a-1

At least for DT and DO-DTT, regarding the necessity and feasibility of introduction of new timing offsets for Device 2b and for Device C, capture the following observations in TR 38.769:

On the timing offset between two consecutive R2D transmissions for the same device (TR2D_R2D_min), sources (FUTUREWEI, vivo, LGE, OPPO, ZTE, DOCOMO) state that introduction of the new timing offset (TR2D_R2D_min) is feasible and necessary, while source (ETRI) states that it is not necessary. Following observations on the new timing offset (TR2D_R2D_min) were provided during the study.

- [FUTUREWEI] mentions that TR2D_R2D_min is needed when consecutive R2D transmissions for the same device are supported, as the device requires time to process the first R2D transmission before receiving the next, and that the corresponding value functions as a scheduling constraint at the reader side that is not provided to the device.

- [vivo] notes that introducing a minimum timing offset TR2D_R2D_min between two consecutive R2D transmissions for the same device is both feasible and necessary, given that devices cannot decode the latter R2D transmission if the interval is shorter than the required processing time.

- [LGE] notes that the timing offset between two consecutive R2D transmissions (TR2D_R2D) should be studied to support scenarios where multiple Msg2 PRDCHs are transmitted in succession.

- [OPPO] mentions that a minimum time gap between L1 R2D control information and the corresponding R2D data should be defined if they are not always consecutive.

- [ZTE] notes that a time interval TR2D_R2D between two consecutive R2D transmissions, along with the associated transmission details, should be considered in Rel‑20 A‑IoT.

- [DOCOMO] states that the timing offset between two consecutive R2Ds should be studied for cases where R2D control information is encoded with a separate CRC from R2D data.

- [Ericsson] mentions that the timing relationship between the R2D sync signal and the R2D broadcast information can be hardcoded in the specification or derived from a predefined rule, and that the start of the paging/Msg0 monitoring window can be defined relative to the R2D sync signal and indicated to devices through the broadcast information.

- [ETRI] states that defining a new minimum timing offset T_R2D_R2D_min is not considered because no use cases require consecutive R2D transmissions.

On the timing offset between two consecutive D2R transmissions for the same device (TD2R_D2R_min), sources (FUTUREWEI, ETRI) state that introduction of the new timing offset (TD2R_D2R_min) is not necessary. Following observations on the new timing offset (TD2R_D2R_min) were provided during the study.

- [OPPO] mentions that the time gap for consecutive D2R transmissions may need to be studied when an A‑IoT device can transmit multiple Msg1 triggered by DO‑A traffic, and that the interval between such transmissions should be larger than a minimum time gap

- [FUTUREWEI] mentions that TD2R_D2R_min is unnecessary because a device may transmit two PDRCHs successively depending on its availability.

- [ETRI] states that defining a new minimum timing offset T_D2R_D2R_min is not considered because no use cases require consecutive D2R transmissions.

- [DOCOMO] states that while consecutive D2R transmissions may occur for DO‑A traffic, valid scenarios for DT and DO‑DTT are unclear.

- [H] Proposal 3.5.3b-1

At least for DT and DO-DTT, regarding the necessity and feasibility of introduction of new timing offsets for Device 2b and for Device C, capture the following observations in TR 38.769:

On the timing offset between two consecutive D2R transmissions for the same device (TD2R_D2R_min), source (OPPO) states that the introduction of the new timing offset (TD2R_D2R_min) may be necessary, while sources (FUTUREWEI, ETRI, DOCOMO) state that introduction of the new timing offset (TD2R_D2R_min) is not necessary. Following observations on the new timing offset (TD2R_D2R_min) were provided during the study.

- [OPPO] mentions that the time gap for consecutive D2R transmissions may need to be studied when an A‑IoT device can transmit multiple Msg1 triggered by DO‑A traffic, and that the interval between such transmissions should be larger than a minimum time gap

- [FUTUREWEI] mentions that TD2R_D2R_min is unnecessary because a device may transmit two PDRCHs successively depending on its availability.

- [ETRI] states that defining a new minimum timing offset T_D2R_D2R_min is not considered because no use cases require consecutive D2R transmissions.

- [DOCOMO] states that while consecutive D2R transmissions may occur for DO‑A traffic, valid scenarios for DT and DO‑DTT are unclear.

| Company | Y/N | Comments |
| --- | --- | --- |
| LG | Y |  |

A-IoT device localization

Work plan for RAN1#124 meeting (R1-2509614)

- Positioning - Which D2R signal(s)/channel(s) applicable to positioning - Measurement aspects (e.g., metric) for positioning

According to Vice Chair’s guidance, the aspects related to A-IoT positioning will also be handled in this sub-agenda.

The related SID objective on A-IoT positioning is shown below.

Objectives … - Identify D2R RSRP-like measurement, and the involved A-IoT signal(s)/channel(s), which are feasible for Device 2b/Device C for more accurate outdoor Device localization than based on Reader-ID [RAN1]. - RAN1 is not expected to conclude on the positioning accuracy as part of the identification of feasible D2R RSRP-like measurement - Note: Device localization based on Reader ID as specified in Rel-19 can be supported for active device(s) in Rel-20 with no additional work. - The design of D2R signal(s)/channel(s) for active devices is not impacted by the identification of the D2R RSRP-like measurement - This objective does not include the study of dedicated Device procedures for Device localization - The objective assumes no dedicated positioning architecture will be specified in Rel-20 - The objective assumes no specification of inter-Reader coordination for Device localization in Rel-20 - This objective will start at RAN1#124

Which D2R signal(s)/channel(s) applicable to positioning

Work plan for RAN1#124 meeting (R1-2509614)

- Positioning - Which D2R signal(s)/channel(s) applicable to positioning - Measurement aspects (e.g., metric) for positioning

Based on the SID, the discussion in this sub-clause focuses on “which” D2R signal/channel is applicable to positioning.

Summary

Views on which D2R signal(s)/channel(s) is/are applicable to A-IoT positioning

- D2R preamble: FUTUREWEI, Ericsson, Apple,

- D2R preamble and midamble(s): Huawei, Ericsson, vivo, DCM,

- D2R preamble and/or midamble(s): OPPO, Xiaomi, ZTE,

- D2R signals (sequence embedded in the PDRCH) and D2R preamble: CATT

- D2R preamble and/or midamble(s), and PDRCH: Qualcomm,

- [H] Proposal 3.6.1a-1

On the D2R signal(s)/channel(s) applicable to A-IoT positioning, capture the following observations in TR 38.769.

D2R signal(s)/channel(s) which is/are feasible and applicable for A-IoT positioning is/are studied. AsFor the D2R signal(s)/channel(s) which is/are feasible and applicable for A-IoT positioning, sources (FUTUREWEI, Ericsson, Apple, vivo) mention D2R preamble.

- [FUTUREWEI] For D2R RSRP-like measurement, the reader can use the D2R preamble for the reference signal and measure the power between the end of R2D transmission and start of the D2R transmission to estimate the noise and interference.

- [Apple] D2R preamble is the applicable signal for RSRP-like measurement. D2R RSRP-like measurement can be performed on existing D2R preamble without requiring dedicated positioning signals, enabling proximity determination for outdoor device localization.

- [vivo] D2R preamble can be used for the positioning purpose. And no need to define D2R RSRP-like metric.

Sources (Huawei, Ericsson, vivo, DCM) mention D2R preamble and midamble(s) for the D2R signal(s)/channel(s) which is/are feasible and applicable for A-IoT positioning.

- [Huawei] D2R preamble and midamble signals are used for D2R RSRP-like measurement for device localization.

- [Ericsson] D2R-amble-RSRP is defined as the linear average over the power contributions (in [W]) of chips that carry the D2R preamble over PDRCH. A secondary measurement can be defined as the linear averaging over the power contributions of chips that carry D2R preamble and D2R midambles over PDRCH.

- [DCM] For positioning, at least D2R RSRP measurement on D2R preamble and D2R midamble when m sequence is used for midamble are feasible. In the previous meetings, it was agreed to consider m sequence and Gold sequence for D2R preamble, and it was agreed to consider m sequence, all 1 sequence and Hadamard sequence for D2R midamble. From our perspective, m sequence and Gold sequence can be used for RSRP measurement at reader thanks to good auto-correlation property, and RSRP measurement based on m sequence and Gold sequence has been widely used in NR and should be feasible.

Sources (OPPO, Xiaomi, ZTE) indicate D2R preamble and/or midamble(s).

- [OPPO] D2R preamble and/or mid-amble, which is applicable for all of D2R transmissions even for sequence-based Msg 1 without associated PDRCH, can be considered for D2R RSRP measurement.

- [Xiaomi] The current D2R signal structure including a preamble, midamble(s), and PDRCH, where preamble and midamble(s) are sequence based on the current agreements. Therefore, for RSRP-like measurement, it is feasible for Reader to measure the D2R preamble or midamble(s).

- [ZTE] Based on introduction of open loop or closed loop power control for D2R, reader is feasible to use D2R preamble or midamble to obtain more accurate localization information.

Source (CATT) states D2R signals (sequence embedded in the PDRCH) and D2R preamble, and source (Qualcomm) states D2R preamble and/or midamble(s), and PDRCH. Following observations were provided during the study.

- [CATT] A sequence with length 127, which is designed for the A-IoT device positioning, was embedded in the PDRCH for the D2R RSRP-like measurements. For the study of sequence length and type, we conducted the following length-31, length-63 and length-127 of M-sequence and Gold sequence with evaluation results. From the evaluation results, it can be seen length-127 M-sequence or Gold-sequence show a lower residual SFO error comparing to that of length-31 M-sequences or Gold-sequence and length-63 M-sequences or Gold-sequence. Since SFO is one of the main factors to measurement accuracy of RSRP, gold sequences or m-sequences with a length of 127 can be used as candidates.

- [Qualcomm] For D2R RSRP-measurement based positioning and proximity determination, PDRCH and D2R preamble/midamble with sufficient time duration and stable transmit power can be considered for D2R measurement. Using multiple measurements or extending D2R transmission increases accuracy. For D2R measurement, unicast D2R transmission can be used: Alt1: D2R Preamble/Midamble, Alt2: PDRCH. For Alt.1, the preamble and/or midamble can be used for RSRP measurement, provided they have sufficient transmission time and combined appropriately. For Alt.2, the PDRCH carries D2R control or data payload and repetitions for reliability can be used to improve power measurement accuracy.

| Company | Y/N | Comments |
| --- | --- | --- |
| vivo |  | We want to clarify our position on which D2R signal(s)/channel(s) is/are applicable to A-IoT positioning according to our following observation. And suggest to change the description in red. “sources (FUTUREWEI, Ericsson, Apple, vivo) mention D2R preamble, sources (Huawei, Ericsson, vivo, DCM) mention D2R preamble and midamble(s), and sources (OPPO, Xiaomi, ZTE) indicte D2R preamble and/or midamble(s).” Observation 15: D2R preamble can be used for the positioning purpose. And no need to define D2R RSRP-like metric. |

D2R RSRP-like measurement and its feasibility

Work plan for RAN1#124 meeting (R1-2509614)

- Positioning - Which D2R signal(s)/channel(s) applicable to positioning - Measurement aspects (e.g., metric) for positioning

Based on the SID, the D2R RSRP-like measurement which is feasible for Device 2b/Device C for more accurate outdoor Device localization than based on Reader-ID is discussed in this sub-clause.

Summary

Measurement aspects (e.g., metric) for positioning

- Definition: FUTUREWEI, Huawei, Ericsson, CATT, Xiaomi,

- Noise and interference estimation: FUTUREWEI

- Additional measurements (AOA measurement): CATT

- [H] Proposal 3.6.2a-1

On the measurement aspects (e.g., metric) for A-IoT positioning, capture the following observations in TR 38.769.

Regarding the D2R RSRP-like measurement for A-IoT positioning, following observations were provided during the study.

- [FUTUREWEI] Use NR measurements (SL PRS-RSRP and UL-SRS-RSRP) as a reference. For D2R RSRP-like measurement, the reader can use the D2R preamble for the reference signal and measure the power between the end of R2D transmission and start of the D2R transmission to estimate the noise and interference. To estimate proximity, a reader can also consider the effects of propagation delay based on when the D2R transmission is received.

- [Huawei] D2R RSRP-like measurement is defined as the time domain linear average over the power contributions (in [W]) of preamble and midamble signals within the signal transmission bandwidth during the D2R transmission.

- [Ericsson] D2R-amble-RSRP is defined as the linear average over the power contributions (in [W]) of chips that carry the D2R preamble over PDRCH. A secondary measurement can be defined as the linear averaging over the power contributions of chips that carry D2R preamble and D2R midambles over PDRCH.

- [CATT] The definition of RSRP-like and AOA measurement quantity can refer to TR 38.215(NR; Physical layer measurements).

- [Xiaomi] The RSRP-like definition in A-IoT is the linear average of the received power in occupied chips and over the frequency resources that carry the measured signal. For example, if D2R is applied with OOK modulation, referring to the definition of LP-RSRP where measured signal is also an OOK waveform, RSRP-like definition in A-IoT is the linear average of the received power in OOK ON chips over the frequency resources that carry the measured signal.

- [vivo] Measurement can be up to network implementation based on D2R preamble, no need to define D2R RSRP-like metric. And nothing needs to be specified also.

- [Qualcomm] At reader side, how to measure the RSRP may be up to reader implementation. However, it may require the averaging of the D2R measurements based on D2R signals/channels with sufficient time duration and stable transmit power from device side.

- [NEC] D2R RSRP based measurement and positioning is feasible. R20 AIoT should support D2R RSRP based measurement.

| Company | Y/N | Comments |
| --- | --- | --- |
| NEC |  | Our proposal is listed below Proposal 29: D2R RSRP based measurement and positioning is feasible. R20 AIoT should support D2R RSRP based measurement. How to determine the position of device based on measurement results at reader is up to implementation. Please add our observation to the supporting company: [NEC] D2R RSRP based measurement and positioning is feasible. R20 AIoT should support D2R RSRP based measurement. |

Other aspects

This sub-clause is to discuss other aspects related to A-IoT positioning, such as positioning methods, evaluations results, etc.

Summary

On the positioning methods, following observations were provided:

[FUTUREWEI] A reader can combine timing and power measurements for proximity determination. A reader can estimate the distance to the device based on the delay measurement and accounting for the SFO.

[Huawei] D2R RSRP measurement based on preamble and midamble, and fingerprint-based positioning scheme is adopted. The positioning algorithm is Weighted K-Nearest Neighbor (WKNN).

[Ericsson] Assuming Tx power of the device is known by the reader and LOS between A-IoT and the reader, one approach for having a more granular outdoor device localization than the reader-based is that the reader can compare the RSRP of D2R signal with different RSRP threshold values, and estimate the zone that the device is located in.

[CATT] Joint estimation of distance from RSRP-like measurements and AOA estimation with reader-based beam sweeping in the implementation is proposed.

[Xiaomi] Reader determines whether A-IoT device is far from or close to itself by comparing the measured RSRP value with an RSRP threshold. Reader determines a corresponding distance range when the measured RSRP value falls in an RSRP range.

[NEC] D2R RSRP is a feasible approach at the current stage. Base on D2R RSRP value, at least device could be grouped into near group or far group roughly.

[Apple] Proximity determination based on D2R RSRP measurement is sufficient for outdoor use-cases requiring several tens of meters accuracy, without needing dedicated positioning architecture. Further consider more finer granularity of proximity including, very near, near and far depending up on the measurement strength.

[Qualcomm] Multi-reader positioning based on RSRP measurement is evaluated using weighted-centroid algorithm. The position estimate is given by a weighted average of BS reader locations, wherein the weights are a function of the RSRP measurement at each of the BS/readers and the same D2R transmission using a 63-length m-sequence is measured per reader.

[Fraunhofer] Introduce an explicit mechanism to set a context for positioning-related signaling. Even when high-accuracy positioning is not required, establishing a coarse spatial relationship such as a coverage zone is beneficial. To enable this, it is important that measurements and signaling exchanges are interpreted within a clear operational context. In particular, devices should be able to distinguish whether a reception instance is intended for regular inventory or command operation, or for positioning-related measurement purposes. Such contextual awareness will prevent unnecessary activity or energy consumption at the device.

Evaluation results were provided by 3 companies:

[Huawei] D2R RSRP-like measurement is feasible for Device C to achieve more accurate outdoor Device localization than based on Reader-ID. D2R RSRP measurement based on preamble and midamble, and fingerprint-based positioning scheme is adopted. The positioning algorithm is Weighted K-Nearest Neighbor (WKNN). Achieved localization performance, i.e., 82m@90%.

[CATT] After SFO/CFO calibration, the positioning accuracy of RSRP+AOA method is about 17m (90% CDF) for Device 2b and is about 13m (90% CDF) for Device C.

[Qualcomm] Multi-reader positioning based on RSRP measurement is evaluated using weighted-centroid algorithm. The position estimate is given by a weighted average of BS reader locations, wherein the weights are a function of the RSRP measurement at each of the BS/readers and the same D2R transmission using a 63-length m-sequence is measured per reader. For outdoor positioning, Outdoor BS/reader can achieve position estimation about 72~114% of cell radius at 80~90% percentage error for Device 2b in UMi layout, and about 84~110% of cell radius estimation at 80~90% percentage error for Device C in UMa layout. For proximity determination to identify a unique device relative to a reader, in Outdoor scenario, UE/reader can reach 95~98% success rate with 2 to 6 nearby devices; BS reader can achieve 48~80%. In Indoor scenario, UE/reader can reach 77~92% success rate with 2 to 6 nearby devices; BS/reader can achieve 56~83%. BS/readers generally perform worse, primarily because fixed BS/reader cannot approach the devices as flexible as UE/reader.

- Proposal 3.6.3a

On A-IoT positioning methods, capture the following observations in TR 38.769.

On A-IoT positioning for Device 2b/Device C for more accurate outdoor Device localization than based on Reader-ID, following observations on positioning methods were reported.

- [FUTUREWEI] A reader can combine timing and power measurements for proximity determination. A reader can estimate the distance to the device based on the delay measurement and accounting for the SFO.

- [Huawei] D2R RSRP measurement based on preamble and midamble, and fingerprint-based positioning scheme is adopted. The positioning algorithm is Weighted K-Nearest Neighbor (WKNN).

- [Ericsson] Assuming Tx power of the device is known by the reader and LOS between A-IoT and the reader, one approach for having a more granular outdoor device localization than the reader-based is that the reader can compare the RSRP of D2R signal with different RSRP threshold values, and estimate the zone that the device is located in.

- [CATT] Joint estimation of distance from RSRP-like measurements and AOA estimation with reader-based beam sweeping in the implementation is proposed.

- [Xiaomi] Reader determines whether A-IoT device is far from or close to itself by comparing the measured RSRP value with an RSRP threshold. Reader determines a corresponding distance range when the measured RSRP value falls in an RSRP range.

- [NEC] D2R RSRP is a feasible approach at the current stage. Base on D2R RSRP value, at least device could be grouped into near group or far group roughly.

- [Apple] Proximity determination based on D2R RSRP measurement is sufficient for outdoor use-cases requiring several tens of meters accuracy, without needing dedicated positioning architecture. Further consider more finer granularity of proximity including, very near, near and far depending up on the measurement strength.

- [Qualcomm] Multi-reader positioning based on RSRP measurement is evaluated using weighted-centroid algorithm. The position estimate is given by a weighted average of BS reader locations, wherein the weights are a function of the RSRP measurement at each of the BS/readers and the same D2R transmission using a 63-length m-sequence is measured per reader.

- [Fraunhofer] Introduce an explicit mechanism to set a context for positioning-related signaling. Even when high-accuracy positioning is not required, establishing a coarse spatial relationship such as a coverage zone is beneficial. To enable this, it is important that measurements and signaling exchanges are interpreted within a clear operational context. In particular, devices should be able to distinguish whether a reception instance is intended for regular inventory or command operation, or for positioning-related measurement purposes. Such contextual awareness will prevent unnecessary activity or energy consumption at the device.

| Company | Y/N | Comments |
| --- | --- | --- |

- Proposal 3.6.3b

On A-IoT positioning methods, capture the following observations in TR 38.769.

On A-IoT positioning for Device 2b/Device C for more accurate outdoor Device localization than based on Reader-ID, sources (Huawei, CATT, Qualcomm) provided evaluation results.

- [Huawei] D2R RSRP-like measurement is feasible for Device C to achieve more accurate outdoor Device localization than based on Reader-ID. D2R RSRP measurement based on preamble and midamble, and fingerprint-based positioning scheme is adopted. The positioning algorithm is Weighted K-Nearest Neighbor (WKNN). Achieved localization performance, i.e., 82m@90%.

- [CATT] After SFO/CFO calibration, the positioning accuracy of RSRP+AOA method is about 17m (90% CDF) for Device 2b and is about 13m (90% CDF) for Device C.

- [Qualcomm] Multi-reader positioning based on RSRP measurement is evaluated using weighted-centroid algorithm. The position estimate is given by a weighted average of BS reader locations, wherein the weights are a function of the RSRP measurement at each of the BS/readers and the same D2R transmission using a 63-length m-sequence is measured per reader. For outdoor positioning, Outdoor BS/reader can achieve position estimation about 72~114% of cell radius at 80~90% percentage error for Device 2b in UMi layout, and about 84~110% of cell radius estimation at 80~90% percentage error for Device C in UMa layout. For proximity determination to identify a unique device relative to a reader, in Outdoor scenario, UE/reader can reach 95~98% success rate with 2 to 6 nearby devices; BS reader can achieve 48~80%. In Indoor scenario, UE/reader can reach 77~92% success rate with 2 to 6 nearby devices; BS/reader can achieve 56~83%. BS/readers generally perform worse, primarily because fixed BS/reader cannot approach the devices as flexible as UE/reader.

| Company | Y/N | Comments |
| --- | --- | --- |

References

- RP-253394 	Revised SID: Study on enhancements for solutions for Ambient IoT (Internet of Things) in NR outdoor for active devices, RAN#110, Baltimore, USA, December 2025

- TR 38.769 	Study on solutions for ambient IoT (Internet of Things)

- R1-2600071 	Other Air Interface Procedures			FUTUREWEI

- R1-2600090 	Study on other procedures for A-IoT			Huawei, HiSilicon

- R1-2600103	Discussion on other procedure for Ambient IoT		Spreadtrum, UNISOC

- R1-2600181	Discussion on other procedures for Rel-20 A-IoT	OPPO

- R1-2600212	Other procedures of A-IoT				Ericsson

- R1-2600329	Study of general procedures of A-IoT enhancement for …	CATT

- R1-2600362	Discussion on other procedures of AIoT		Tejas Network Limited

- R1-2600370	Other procedures for AIoT				Nokia

- R1-2600380	Discussion on other procedures			CMCC

- R1-2600419	Discussion on other procedures for Device 2b/C		Xiaomi

- R1-2600453	Discussion on other procedures for active Ambient IoT …	ZTE Corporation, Sanechips

- R1-2600493	Discussion on Other Procedures for R20 AIoT		vivo

- R1-2600547	Other procedures for Device 2b/C			LG Electronics

- R1-2600661	Study on other procedures for Device 2b and C		NEC

- R1-2600683	Discussion on other procedures for Ambient IoT		China Telecom

- R1-2600713	Discussion on A-IoT other procedures			Panasonic

- R1-2600745	Study on other procedures of air interface for Device …	Samsung

- R1-2600817	On other procedures for device 2b/C			Apple

- R1-2600967	Discussion on other procedures			Sharp

- R1-2600993	Discussion on other procedures for A-IoT		ETRI

- R1-2601022	Discussion on other procedures for R20 Ambient IoT	Lenovo

- R1-2601076	Procedures for Active AIoT Devices			InterDigital, Inc.

- R1-2601124	System information and DO-A aspects for Ambient …	Sony

- R1-2601169	Study on other procedures for Ambient IoT outdoor for …	NTT DOCOMO, INC.

- R1-2601262	Study of other procedures for Device 2b/C		Qualcomm Incorporated

- R1-2601293	Discussion on other procedures for Device 2b/C		Quectel

- R1-2601301	Discussion on procedures for A-IoT active device	ASUSTeK

- R1-2601341	Discussion on other procedures for Ambient IoT		TCL

- R1-2601347	Discussion on other procedures for Device 2b/C		KT Corp.

- R1-2601442	Discussion on other aspects of Rel-20 AIoT		IIT Kanpur

- R1-2601450	Discussion on other procedures for R20 A-IoT		Fraunhofer HHI, Fraunhofer IIS

Annex A : RAN1 agreements

RAN1#122 (Bengaluru)

Agreement

For R2D, study necessity of addition/removal of M values from Rel-19 M value set for Device 2b and for Device C for at least the following M values:

- Addition of M=1

- Addition of M=32

- Removal of M=24

Agreement

Study necessity and feasibility of FEC for R2D for Device 2b and for Device C including the following aspects:

- Coding schemes: convolutional code (e.g TBCC), RM code

  - Note: focus on coding schemes already specified in LTE or NR

- FEC code rates: at least 1/2, 1/3

- Interleaver or LTE bit collection scheme for TBCC

Agreement

Study necessity and feasibility of R2D repetitions for R2D for Device 2b and for Device C considering the following repetition types:

- Block-level repetitions

- Bit-level Type-2 repetitions

- Chip-level repetitions (in case of M=1 with FEC if applied)

Agreement

For R2D signal(s), study at least the following functionalities (if needed), and study whether to reuse/enhance existing R2D signal(s) or to introduce new R2D signal(s) for the following functionalities (if needed):

- R2D chip duration determination

- indication of the start of R2D

- indication of the end of PRDCH

- frequency synchronization (including frequency acquisition)

- CFO estimation / LO calibration

- SFO calibration

- timing synchronization/tracking

- measurement (e.g., signal strength)

- reader identification/differentiation

Agreement

For D2R signal(s), study at least the following functionalities (if needed), and study whether to reuse/enhance existing D2R signal(s) or to introduce new D2R signal(s) for the following functionalities (if needed):

- CFO estimation

- SFO estimation

- timing acquisition

- channel estimation

- measurement (e.g., signal strength, interference estimation)

- Device differentiation e.g. for collision resolution, for interference randomization

Agreement

Study necessity and feasibility of reuse, modifications or enhancements of existing Rel-19 D2R modulations, and the potential introduction of new modulations schemes, for Device 2b and for Device C, including at least the following aspects:

- introduction of new modulation schemes (e.g., MSK or DBPSK)

- down-selection of existing modulation schemes

- design considerations related to spectrum shaping and D2R signals

Agreement

Study the following aspects to support DO-A traffic (periodic and event-triggered) for Device 2b and for Device C in Rel-20.

- Timing and frequency synchronization for DO-A transmission

- Resource allocation/determination method for DO-A transmission

- Configuration/signaling of necessary information

Agreement

Study necessity and feasibility of enhancements on existing Rel-19 D2R FEC for Device 2b and for Device C including the following aspects:

- additional FEC code rate(s) (e.g., 1/2, 1/4)

- interleaver, or LTE bit collection scheme for TBCC

- Note: no change to Rel-19 TBCC polynomial

Agreement

For PDRCH, study necessity and feasibility of larger (>2) number of repetitions for block-level repetitions for Device 2b and for Device C.

Agreement

For PDRCH, study necessity of scrambling for Device 2b and for Device C.

Agreement

At least for CBRA for DT and DO-DTT, study necessity and feasibility of enhancements of D2R TDM(A) for Device 2b and for Device C based on the following:

- X1 > 2 for Msg1 (X1: number of time domain resources for Msg1 transmission indicated by the corresponding R2D message)

- X3 > 1 for Msg3 (X3: number of time domain resources for Msg3 transmission indicated by the corresponding R2D message)

Agreement

Study necessity and feasibility of enhancements of D2R for Device 2b and for Device C, considering the following aspects or potential enhancements:

- Considering that Device 2b and Device C can adjust the LO towards a desired frequency, whether to also support baseband small FS for Device 2b and Device C

- D2R frequency resources for FDM(A) (e.g., considering D2R transmission bandwidth, guard-band size between FDMed D2R transmissions, CFO, SFO, etc.)

- How a device determines the frequency resource for D2R transmission

- FDM(A) for D2R transmissions other than Msg1 and Msg3

Agreement

Study necessity and feasibility of D2R Tx power control for Device 2b and for Device C, including the following aspects

- Open loop and/or closed loop power control

- Required measurements (e.g., signal strength)

- Required information at the device

- Required information at reader side

- Signaling overhead and power consumption

Agreement

At least for DT and DO-DTT, study necessity and feasibility of changes to existing timing offsets for Device 2b and for Device C, e.g.,

- timing offset value between R2D and corresponding subsequent D2R (e.g., Toffset1/2/3/4)

- timing offset value between D2R and corresponding subsequent R2D (e.g., T_D2R_min)

  - Note: RAN1 only discusses potential reasons that may impact the value of T_D2R_min, and if any provides those reasons to RAN4

- timing offset between R2D and corresponding subsequent D2R provided by a reader

- timing offset between D2R and corresponding subsequent R2D provided by a reader

At least for DT and DO-DTT, study necessity and feasibility of introduction of new timing offsets for Device 2b and for Device C, e.g.,

- timing offset between two consecutive R2D or D2R transmissions for the same Device, if such scenario exists

RAN1#122bis (Prague)

Agreement

Study L1 R2D control information with separate CRC for Device 2b and for Device C considering the following aspects

- Necessary information to be included in the L1 R2D control

- Details on how the L1 R2D control information is transmitted

Agreement

Study necessity and feasibility of R2D FDM from the same reader for Device 2b and for Device C considering at least the following aspects:

- complexity of device to support FDM

- how a device determines the frequency resource for receiving one of the FDMed R2D transmissions

Agreement

Study necessity and feasibility of D2R CDM(A) for Device 2b and for Device C with sequence-based Msg1 transmission and potential impact to random access procedures.

- FFS: other cases, except for data transmission

Agreement

For the functionality of indicating the start of the R2D transmission that contains PRDCH, study at least the following options if the functionality relies on SIP:

- Option 1:  binary sequence-based SIP (e.g M-sequence or Golay sequence).

- Option 2: Reuse the Rel-19 SIP.

Agreement

For the functionality of indicating the end of the PRDCH, study the following option:

- Option 1: Use the L1 R2D control information to indicate the TBS.

[H] Proposal 4.2a-r1

For PRDCH, study necessity and feasibility of scrambling for Device 2b and for Device C to achieve the following:

- Interference randomization

- To reduce continuous 1s or 0s if needed

Agreement

For the functionality of chip duration determination of the data payload in the PRDCH, study the following options:

- Option 1: same as the chip duration of the corresponding L1 R2D control information

- Option 2: indicated by the corresponding L1 R2D control information

Agreement

For the functionality of chip duration determination of the L1 R2D control information, study at least the following options:

- Option 1: Fixed chip duration. No use of CAP for chip duration determination.

- Option 2: Blind detection from a predefined set of the chip durations of L1 R2D control information. No use of CAP for chip duration determination.

- Option 3: Use CAP

  - Alt.3-1 Rel-19 CAP pattern

  - Alt.3-2 Enhanced CAP

    - Alt.3-2a: Rel-19 CAP pattern with repetition

    - Alt.3-2b: CAP pattern and the association between chip durations for CAP and chip durations of L1 R2D control information

- Option 4: Use a set of different binary sequences with fixed or variable length

  - Alt.4-1: using binary sequences for SIP. No use of CAP for chip duration determination.

  - Alt.4-2: using binary sequences for CAP

- Option 5: Use broadcast information (if any) to indicate the chip duration

Agreement

For the functionality of CFO estimation/LO calibration at the device side, study CFO calibration signal with the following aspects:

- Whether the signal is

  - Option a: unmodulated sinusoid single tone

  - Option b: unmodulated multiple sinusoid single tones

  - Option c: modulated multiple tones

    - Whether the CFO calibration signal is the same or different from the synchronization signal (if supported)

- Whether the CFO calibration signal is transmitted periodically and/or as needed (aperiodically)

- The time location(s) of CFO calibration signal.

  - Option 1: in relation to the L1 R2D control information

  - Option 2: in relation to the data payload of the PRDCH

  - Option 3: in relation to the PDRCH

  - Option 4: in relation to the synchronization signal (if supported)

  - Option 5: in relation to broadcast information (if supported)

  - Combination of Options is not precluded

- Other details (e.g., time duration, frequency location(s) of the CFO calibration signal)

Proposal 4.4.g

For R2D signals, for the functionality of timing synchronization/tracking, study the following options:

- Option 1: New periodic synchronization signals

- Option 2: R2D midamble(s)

- Option 3: Manchester encoding

Agreement

Study R2D synchronization signal at least for the functionality of frequency synchronization (including frequency acquisition) and for the functionality of timing synchronization/tracking.

- Alt1: periodic synchronization signal

- Alt2: aperiodic synchronization signal

Agreement

For resource allocation method for the first D2R transmission for DO-A for Device 2b and for Device C, study following options considering feasibility:

- Option 1: Periodic D2R resource allocation

- Option 2: Aperiodic D2R resource allocation

Agreement

For providing necessary information for the first D2R transmission for DO-A for Device 2b and for Device C, study at least following options considering feasibility:

- Option 1: necessary information provided by broadcast information

- Option 2: necessary information provided by a paging-like R2D message

- Option 3: necessary information derived from a synchronization signal and provided by broadcast information

Agreement

Study whether necessary information for the first D2R transmission for DO-A for Device 2b and for Device C is provided periodically and/or aperiodically by Reader.

Agreement

For the content of the first D2R transmission for DO-A for Device 2b and for Device C, study the following options:

- Option 1: Msg1-like content

- Option 2: DO-A data payload

  - FFS: with or without device ID

- Option 3: DO-A scheduling request

Agreement

The study assumes single-carrier waveform for D2R.

- FFS whether to use 2SB or 1SB for D2R

Agreement

Regarding D2R preamble for PDRCH, study the following aspects of the D2R preamble.

- Preamble length (e.g., longer than Rel-19 D2R preamble)

- Sequence type

- Number of sequences using the same length (e.g., for reader/device differentiation)

- Considering other factors affecting preamble design (e.g., D2R modulation and detection schemes)

Agreement

Regarding D2R midamble for PDRCH, study the following aspects of the D2R midamble.

- Midamble pattern(s) including the length(s) and interval(s) within (and possibly at the end of) a PDRCH

- Sequence type, and whether the same or different sequence type is used for D2R preamble and D2R midamble

- Considering other factors affecting midamble design (e.g., D2R modulation and detection schemes)

Proposal 4.1a

For R2D, it is beneficial to add M=1 to the set of M values from Rel-19 in order to support larger coverage distances.

R1-2508129	FL summary #7 for 10.3.2 “Study of air interface for Device 2b/C”	Moderator (LG Electronics)

Agreement

Study the necessity of D2R Tx power control for Device 2b and for Device C for outdoor scenarios

- Target to agree on observations on pros and cons of D2R Tx power control vs. no Tx power control at RAN1#123

Continue studying feasibility of the following open loop power control methods considering device complexity:

- Methods of open loop power control

  - Option 1: Device calculates Tx power based on measured pathloss, a predefined formula, and some configured parameters from reader

    - Required information at the device side at least includes reader Tx power, received target power at the reader, etc.

  - Option 2: Device measures RSRP, and uses a corresponding Tx power when the RSRP falls in an RSRP range.

    - The RSRP range is based on RSRP threshold(s) configured by reader or predefined

    - Required information at the device side at least includes RSRP threshold(s), corresponding D2R Tx power related information per each RSRP range defined by the RSRP threshold(s)

  - FFS details (e.g., required measurements, required information)

Continue studying the necessity and feasibility of closed loop power control for D2R.

Agreement

Study interleaving for PDRCH with FEC, including the necessity and feasibility of the following:

- Alt 1: reuse both LTE sub-block interleaver and LTE bit collection scheme

- Alt 2: reuse LTE bit collection scheme without LTE sub-block interleaver

- Alt 3: based on LTE bit collection scheme with some update (e.g., for memory reduction) without LTE sub-block interleaver

- Combination of alternatives can be studied

Agreement

Study interleaving for PRDCH with FEC, including the necessity and feasibility of the following:

- Alt 1: reuse both LTE sub-block interleaver and LTE bit collection scheme

- Alt 2: reuse LTE bit collection scheme without LTE sub-block interleaver

- Alt 3: based on LTE bit collection scheme with some update (e.g., for memory reduction) without LTE sub-block interleaver

- Combination of alternatives can be studied

Agreement

For PDRCH, further study the necessity of scrambling with following details:

- Scrambling applied after block-level repetition (if applied) and channel coding (if applied, including interleaving if supported)

- Scrambling sequence type and generation method

Agreement

Regarding D2R modulation for device 2b and device C, further study the following:

- Option 1: BPSK

  - FFS the corresponding preamble and midamble design for coherent detection at reader side

- Option 2: OOK

  - FFS the corresponding preamble and midamble design for coherent or non-coherent detection at reader side

- Option 3: DBPSK

  - FFS the corresponding preamble and midamble design for non-coherent detection at reader side

- Option 4: MSK

  - FFS the corresponding preamble and midamble design for coherent or non-coherent detection at reader side

Agreement

The minimum R2D transmission bandwidth of 1 PRB is the baseline for Rel-20.

Agreement

For R2D signals, study the necessity for the functionality of SFO calibration, and study the feasibility of the following options:

- Option 1: Reuse CAP and/or Manchester encoding

- Option 2: sequence-based SIP with or without R2D midamble(s)

- Option 3: using the CFO calibration signal

R1-2508130	FL summary #8 for 10.3.2 “Study of air interface for Device 2b/C”	Moderator (LG Electronics)

Agreement

For the study of necessity and feasibility of FEC for R2D for Device 2b and for Device C, further study the following aspects:

- Coding schemes

  - LTE TBCC

  - Tailed convolutional code with the same polynomial as LTE TBCC

  - RM code

- FEC code rates: 1/2, 1/3, 1/4

Proposal

For the study of necessity and feasibility of enhancements on existing Rel-19 D2R FEC for Device 2b and for Device C, further study the following codes:

- Code rate = 1/2 obtained by Rel-19 TBCC polynomial followed by puncturing

- Code rate = 1/4 obtained by Rel-19 TBCC polynomial followed by repetition and then puncturing

- Code rate = 1/4 obtained by adding a new polynomial for the fourth branch

Agreement

For R2D and D2R, study the order of channel coding (including interleaver if supported) and block-level repetition (if supported) for complexity reduction, considering necessity and feasibility.

Proposal

For the other factors affecting preamble and midamble design in the two agreements below, consider only the following:

- D2R modulation (BPSK, OOK, DBPSK, MSK)

- Detection schemes at the reader side (coherent or non-coherent detection)

Proposal

For R2D, for the study of addition/removal of M values from Rel-19 M value set for Device 2b and for Device C, assume a constant power within each OFDM symbol containing at least one OOK ON chip.

Agreement

For the study of R2D binary sequence-based SIP, study the following aspects:

- Sequence type

  - Option 1: m-sequence

  - Option 2: Golay sequence

- Length of sequence

- Number of sequence(s)

- FFS: whether to apply Manchester coding to the sequence

- Note: the same aspects can be studied for R2D midamble if applied with a sequence-based SIP

Agreement

For the study of binary sequence-based Msg1 for the study of necessity and feasibility of D2R CDM(A) for Device 2b and for Device C, study at least the following aspects:

- Sequence type (e.g. Gold sequence, m-sequence)

- Length of the sequence

- Number of sequences

Agreement

For R2D, study at least the following aspects for the study of broadcast information:

- Information included in the broadcast information

- Whether to use PRDCH to carry the broadcast information

- Transmission of broadcast information:

  - Option 1: periodic

  - Option 2: aperiodic

- Time and frequency resources for transmission of the broadcast information, e.g. in relation to the synchronization signal.

Agreement

For the L1 R2D control information preceding and scheduling the data payload of PRDCH, study the following aspects:

- Time location of L1 R2D control information:

  - Option 1: L1 R2D control information and data payload of PRDCH are contiguous in time.

  - Option 2: L1 R2D control information and data payload of PRDCH are not contiguous in time.

- Study frequency resource of L1 R2D control information

- Whether L1 R2D control information is included in the PRDCH that carries that data payload, or in another PRDCH, or in another channel

RAN1#123 (Dallas)

Agreement

Initial frequency acquisition procedure is necessary for Device 2b and for Device C, at least for flexible deployment of A-IoT frequency location(s) for outdoor scenario.

Agreement

Reader identification/differentiation is necessary for Device 2b and for Device C, at least in outdoor scenario to differentiate R2D among different readers.

Agreement

For initial frequency acquisition for Device 2b and for Device C, a device is expected to determine a frequency point from predefined sync raster points for reception of the periodic synchronization signal.

- Details on the sync raster design is handled by RAN4.

Agreement

- The functionality of broadcast information is to provide the device with at least necessary information for subsequent transmission and reception

- On how A-IoT devices acquire the broadcast information, study the following two cases:

  - Case B1: Single-step acquisition of the broadcast information

  - Case B2: Two-step acquisition of the broadcast information (e.g., MIB-like and then SIB1-like broadcast information)

- On whether to use PRDCH to carry the broadcast information,

  - For Case B1, PRDCH is used to carry the broadcast information.

  - For Case B2

    - PRDCH is used to carry the SIB1-like broadcast information.

    - A separate PRDCH or a new physical channel dedicated to MIB-like is used to carry the MIB-like broadcast information

Agreement

Periodic transmission of broadcast information to provide at least necessary information for subsequent transmission and reception is necessary for Device 2b and for Device C.

- FFS (in SI or WI): whether the transmission of broadcast information is strictly periodic at reader

- FFS (in SI or WI): periodicity of the transmission of broadcast information

Agreement

For the 3 options for the content of the first D2R transmission for DO-A for Device 2b and for Device C, study the following aspects:

- Recommended scenarios (event-triggered or periodic)

- Subsequent transmission(s)/reception(s)/procedure(s) if any after the first D2R transmission for DO-A

- Other aspects

Agreement

On D2R Tx power control for Device 2b and for Device C, capture the following in TR 38.769.

The necessity of D2R Tx power control for Device 2b and Device C for outdoor scenarios were studied and the following observations were provided:

- 22 sources (Futurewei, LGE, Spreadtrum, vivo, Nokia, Semtech, CMCC, CEWiT, BOSCH, NEC, InterDigital, TCL, Apple, China Telecom, ZTE, OPPO, Huawei, Docomo, Ericsson, Xiaomi, Samsung, Qualcomm) reported that the D2R Tx power control for Device 2b and Device C is necessary for outdoor scenarios.

  - 20 sources (Futurewei, LGE, Spreadtrum, vivo, Nokia, Semtech, CMCC, CEWiT, BOSCH, Quectel, InterDigital, TCL, Apple, OPPO, Huawei, Docomo, Ericsson, Xiaomi, Samsung, ZTE) state that it can reduce the interference between different devices (i.e., the Near-Far problem).

  - 14 sources (Futurewei, LGE, Spreadtrum, vivo, Semtech, CMCC, NEC, InterDigital, TCL, Apple, OPPO, Xiaomi, Samsung, ZTE) reported that it helps devices remain available longer.

  - 4 sources (Futurewei, Semtech, Apple, Docomo) reported that the interference due to a transmission from a device to a reader can be reduced at an unintended reader

  - 1 source (Qualcomm) added that depending on device’s peak power consumption and PA support, the D2R Tx power control may not be feasible.

- 1 source (CATT) reported that the D2R Tx power control for Device 2b and Device C is not necessary mentioning in its contribution that the inter-channel interference from different devices is negligible, D2R coverage is limited, and the complexity and the power consumption required at the device side is high.

Agreement

For reader identification/differentiation at least in outdoor scenario to differentiate R2D among different readers, a device acquires the reader ID-related information by at least one of the following options:

- Option 1: Reader ID-related information is contained in the broadcast information.

- Option 2: Reader ID-related information is derived from the periodic sync signal sequence(s).

- Option 3: Reader ID-related information is carried by L1 R2D control information.

- Option 4: Reader ID-related information is contained in the higher-layer information in the paging-like message.

Agreement

For providing necessary information for the first D2R transmission for DO-A for Device 2b and for Device C, following options are considered feasible.

- Option 1: necessary information provided by broadcast information

- Option 2: necessary information provided by a paging-like R2D message

- Option 3: necessary information derived from a synchronization signal and provided by broadcast information

For providing necessary information for the first D2R transmission for DO-A for Device 2b and for Device C, study following option considering feasibility and applicability

- Option 4: necessary information provided by device-specific R2D transmission

Appendix B: Work plan for the next meeting (RAN1#124)

In the next meeting (RAN1#124), at least following topics will be discussed.

- Initial frequency/broadcast information acquisition

  - Reader identification/differentiation

    - Views on Option 1/2/3/4 for how a device acquires the reader ID-related information

  - Broadcast information

    - Purposes of the broadcast information and the necessary information per each purpose

    - Views on Case B1 and Case B2 for how A-IoT devices acquire the broadcast information

      - For Base B2, views on whether to use a separate PRDCH or a new physical channel to carry the MIB-like broadcast information

    - Time and freq. resource for the broadcast information, e.g. in relation to the synchronization signal  To be discussed in AI 10.3.2.1

- DO-A

  - For Option 1/2/3 for the content of the first D2R transmission for DO-A

    - recommended scenarios (event-triggered or periodic)

    - subsequent transmission(s)/reception(s)/procedure(s) if any after the first D2R transmission for DO-A

    - other aspects

  - Timing and frequency synchronization for DO-A

  - Resource allocation/determination for DO-A

  - Necessary information and its provisioning (Option 1/2/3)

- D2R Tx power control

  - Necessity of D2R Tx power control (Pros and cons of Tx power control vs. no Tx power control for D2R)

  - Feasibility study of the two open-loop Tx power control methods for D2R

  - Necessity and feasibility of closed-loop Tx power control for D2R

- Enhancements on random access from procedural aspects

  - Necessity and feasibility of X1>2 for Msg1 and X3>1 for Msg3

  - Potential impact of D2R CDM(A) on random access procedures

  - Potential impact of R2D FDM on random access procedures

- Timing offsets

  - Necessity and feasibility of changes to existing timing offsets (Toffset1/2/3/4, T_D2R_min)

  - Necessity and feasibility of dynamic indication of the existing timing offsets

  - Necessity and feasibility of introduction of new timing offsets (T_R2D_R2D_min, T_D2R_D2R_min)

According to Vice Chair’s guidance, the aspects related to A-IoT positioning will also be handled in this sub-agenda.

The related SID objective on A-IoT positioning is shown below.

- Identify D2R RSRP-like measurement, and the involved A-IoT signal(s)/channel(s), which are feasible for Device 2b/Device C for more accurate outdoor Device localization than based on Reader-ID [RAN1]. - RAN1 is not expected to conclude on the positioning accuracy as part of the identification of feasible D2R RSRP-like measurement - Note: Device localization based on Reader ID as specified in Rel-19 can be supported for active device(s) in Rel-20 with no additional work. - The design of D2R signal(s)/channel(s) for active devices is not impacted by the identification of the D2R RSRP-like measurement - This objective does not include the study of dedicated Device procedures for Device localization - The objective assumes no dedicated positioning architecture will be specified in Rel-20 - The objective assumes no specification of inter-Reader coordination for Device localization in Rel-20 - This objective will start at RAN1#124

- Positioning

  - D2R signal(s)/channel(s) applicable to positioning

  - Measurement aspects (e.g., metric) for positioning

Overall, for easier process in preparation for and during the next meeting, companies are encouraged to provide their views and observations per each topic clearly and concisely, so that your views can be collected easily and observations can be captured in the TR.