3GPP TSG RAN WG1 #123	                                   					       												                                    R1-2509438

Dallas, USA, Nov 17th – 21st, 2025

Source:	Ad-Hoc Chair (AT&T)

Title:	Session Notes of AI 9.3

Agenda Item:	9.3

Document for: 	Endorsement

### 9.3 UE features Batch C

Note: Maximum one contribution per company/organization/university for NR_AIML_air, NR_MIMO_Ph5, Netw_Energy_NR_enh, NR_Mob_Ph4, LTE_terr_bcast_Ph2 and TEI19 with other than [Common_PDCCH_rep_TN]

Note:For efficient review, please use the following sections in your contribution corresponding to the maintenance issues, if any:

- NR_AIML_air

- NR_MIMO_Ph5

- Netw_Energy_NR_enh

- NR_Mob_Ph4

- LTE_terr_bcast_Ph2

- TEI19 with other than [Common_PDCCH_rep_TN]

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 58. NR_AIML_air | 58-1-2 | UE-side beam prediction for BM Case1 for inference | 1. Support of beam prediction with reporting of predicted beam index for BM-Case1 for inference with UE-side model 3. Maximum number of inference report(s) configured for BM-Case1 per BWP 3a. Maximum number of inference report(s) configured for BM-Case1 across all CCs 6. Support of SSB as RS type for Set B 6a. Support of CSI-RS as RS type for Set B 6b. Support of SSB as RS type for Set A 6c. Support of CSI-RS as RS type for Set A 7a: Supported maximum number of resources for Set B 7b: Supported maximum number of resources for Set A 8. Supported CSI-RS resource types for Set B 9. Supported inference report types 11. Supported BM-Case 1 sub-usecase(s) 12. Supported maximum number of predicted beams in each reporting instance 13. Supported number of occupied CPU 14. Supported number of occupied CPU,2/CPU,3 15. Supported value of di for the relaxation of Z3 timeline, where i is the index of SCS, i=1,2,3,4,5,6 corresponding to 15,30,60,120,480,960 kHz SCS 16. Supported value of d’i for the relaxation of Z’3 timeline, where i is the index of SCS, i=1,2,3,4,5,6 corresponding to 15,30,60,120,480,960 kHz SCS 17. Occupied resource pool between CPU,2 and CPU,3 | 2-35 | yes | n/a | UE-sided beam prediction for BM Case 1 for inference is not supported | Per band | n/a | n/a | n/a | Component 3 candidate values: - Periodic reporting: {1, 2, 3, 4} - Aperiodic reporting: {1, 2, 3, 4} - Semi-persistent reporting: {1, 2, 3, 4} Component 3a candidate values: {1, 2, 3, 4, 8, 10, 12, 16} Component 7a candidate values: {4, 8, 16} Component 7b candidate values: {8, 16, 32, 64} Component 8 candidate values: {Periodic CSI-RS, Semi-persistent CSI-RS, Aperiodic CSI-RS} Component 9 candidate values: {Periodic CSI report, Aperiodic CSI report, semi-persistent CSI report} Component 11 candidate values: {setB-subset-of-setA, setB-different-from-setA, both} Component 12 candidate values: {1, 2, 3, 4} Component 13 candidate values: {0, 1, 2, …, 8} Component 14 candidate values: {0, 1, 2, …, 8} Note: The values of component 13 and 14 are not allowed to be 0 simultaneously Component 15 candidate values: d1 is FFS {2, 7, 8, 14, 21, 22, 28, 35, 42, 44, 56} d2 is FFS {4, 14, 16, 28, 33, 42, 56, 66, 70, 84, 112} d3 is FFS {8, 28, 32, 44, 56, 84, 88, 112, 140, 168, 224} d4 is FFS {16, 56, 97, 112, 168, 194, 224, 280, 336, 448} d5 is FFS {32, 112, 224, 388, 448, 672, 776, 896, 1120, 1344, 1792} d6 is FFS {64, 224, 448, 776, 896, 1344, 1552, 1792, 2240, 2688, 3584} Component 16 candidate values: d1 is FFS {2, 7, 8, 14, 21, 22, 28, 35, 42, 44, 56} d2 is FFS {4, 14, 16, 28, 33, 42, 56, 66, 70, 84, 112} d3 is FFS {8, 28, 32, 44, 56, 84, 88, 112, 140, 168, 224} d4 is FFS {16, 56, 97, 112, 168, 194, 224, 280, 336, 448} d5 is FFS {32, 112, 224, 388, 448, 672, 776, 896, 1120, 1344, 1792} d6 is FFS {64, 224, 448, 776, 896, 1344, 1552, 1792, 2240, 2688, 3584} Component 17 candidate values: {1, 2} representing the first CPU pool (i.e., CPU,2) and the second CPU pool (i.e., CPU,3), respectively | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 58. NR_AIML_air | 58-1-4 | UE-side beam prediction for BM Case2 for inference | 1. Support of beam prediction with reporting of predicted beam index for BM-Case2 for inference with UE-side model 3. Maximum number of inference report(s) configured for BM-Case2 per BWP 3a. Maximum number of inference report(s) configured for BM-Case2 across all CCs 6. Support of SSB as RS type for Set B 6a. Support of CSI-RS as RS type for Set B 6b. Support of SSB as RS type for Set A 6c. Support of CSI-RS as RS type for Set A 7a: Supported maximum number of resources for Set B 7b: Supported maximum number of resources for Set A 7c: Supported minimum number of received transmission occasions for Set B 8. Supported CSI-RS resource types for Set B 9. Supported inference report types 11. Supported maximum number of predicted beams in each predicted time instance 12. Supported maximum number of predicted time instances 13. Supported maximum total number of reported predicted beams for predicted time instances in one report 15. Supported value(s) of time gap between predicted time instances and between reference time to the first future time instance 20. Supported BM-Case 2 sub use case(s) 21. supported number of occupied CPU 22. supported number of occupied CPU,2/CPU,3 23. supported value of di for the relaxation of Z3 timeline, where i is the index of SCS, i=1,2,3,4,5,6 corresponding to 15,30,60,120,480,960 kHz SCS 24. supported value of d’i for the relaxation of Z’3 timeline, where i is the index of SCS, i=1,2,3,4,5,6 corresponding to 15,30,60,120,480,960 kHz SCS 25. Occupied resource pool between CPU,2 and CPU,3 | 2-35 | yes | n/a | UE-side beam prediction for BM-Case2 for inference is not supported | Per band | n/a | n/a | n/a | Component 3 candidate values: - Periodic reporting: {1, 2, 3, 4} - Aperiodic reporting: {1, 2, 3, 4} - Semi-persistent reporting: {1, 2, 3, 4} Component 3a candidate values: {1, 2, 3, 4, 8, 10, 12, 16} Component 7a candidate values: {4, 8, 16, 32, 64} Component 7b candidate values: {4, 8, 16, 32, 64} Component 7c candidate values: {2, 4, 6, 8, 10} Component 8 candidate values: {Periodic CSI-RS, Semi-persistent CSI-RS} Component 9 candidate values: {Periodic CSI report, Aperiodic CSI report, semi-persistent CSI report} Component 11 candidate values: {1, 2, 3, 4} Component 12 candidate values: {1, 2, 4, 8} Component 13 candidate values: {1, 2, 4, 6, 8, 12, 16, 32} Component 15 candidate values: {10ms, 20ms, 40ms, 80ms, 160ms} Component 20 candidate values: { setB-equals-to-setA, setB-subset-of-setA, setB-different-from-setA, merged versions} Component 21 candidate values: {0, 1, 2, … 8} Component 22 candidate values: {0, 1, 2, … 8} Note: The values of component 21 and 22 are not allowed to be 0 simultaneously. Component 23 candidate values: d1 is FFS {2, 8, 14, 28, 33, 42, 56, 66, 70, 84, 98112} d2 is FFS {4, 16, 28, 50, 56, 84, 100, 112, 140, 168, 196, 224} d3 is FFS {8, 32, 56, 66, 112, 132, 168, 224, 280, 336, 392} d4 is FFS {16, 56, 112, 146, 224, 291, 336, 448, 560, 672, 784} d5 is FFS {32, 112, 224, 448, 582, 896, 1164, 1344, 1792, 2240, 2688, 3136} d6 is FFS {64, 224, 448, 896, 1164, 1792, 2328, 2688, 3584, 4480, 5376, 6272} Component 24 candidate values: d1 is FFS {2, 8, 14, 28, 33, 42, 56, 66, 70, 84, 98112} d2 is FFS {4, 16, 28, 50, 56, 84, 100, 112, 140, 168, 196, 224} d3 is FFS {8, 32, 56, 66, 112, 132, 168, 224, 280, 336, 392} d4 is FFS {16, 56, 112, 146, 224, 291, 336, 448, 560, 672, 784} d5 is FFS {32, 112, 224, 448, 582, 896, 1164, 1344, 1792, 2240, 2688, 3136} d6 is FFS {64, 224, 448, 896, 1164, 1792, 2328, 2688, 3584, 4480, 5376, 6272} Component 25 candidate values: {1, 2} representing the first CPU pool (i.e., CPU,2) and the second CPU pool (i.e., CPU,3), respectively Note: UE should not report non-zero value for Component 22 if FG 58-0-1 is not signalled | Optional with capability signalling |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 58. NR_AIML_Air | 58-3-1 | CSI prediction for UE-sided inference when N4=1 | 1. Support of CSI prediction for UE-sided inference when N4=1 2. Support for reporting predicted PMI with N4=1 3. A list of supported combinations, each combination is { Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously 4. Support of Rel-16 eType-II regular codebook refinement for predicted PMI with PMI subband R=1 5. Support parameter combinations with L=2,4 6. Support for rank = 1,2 7. Support for the size of DD-basis, N4=1 8. Support X=1 CQI based on the first/earliest slot of the CSI reporting window and the first/earliest predicted PMI (TDCQI=’1-1’) 11. Scaling factor for active resource counting Kp 12. supported value of ti for the relaxation of Z and Z’ timeline, where i is the index of SCS, i=1,2,3,4,5,6 corresponding to 15,30,60,120,480,960 kHz SCS 13. supported number of occupied CPU 14. supported number of occupied CPU,2/CPU,3 15. Occupied resource pool between CPU,2 and CPU,3 | 2-35 | yes | n/a | CSI prediction for N4=1 for inference is not supported | Per band and Per BC | n/a | n/a | n/a | Component 3 candidate values: a. {4,8,12,16,24,32} b. {2,3,4 … 64} c. {4, …, 256} Component 11 candidate values: {1,2,4} Component 12 candidate values: FFS t1 is {0, 1, 2, 4, 7, 8, 14, 28, 56, 112} t2 is {0, 2, 4, 8, 14, 28, 56, 112, 224} t3 is {0, 4, 8, 14, 16, 28, 56, 112, 224, 448} t4 is {0, 8, 14, 16, 28, 32, 56, 112, 224, 448, 896} t5 is {0, 16, 32, 56, 64, 112, 224, 448, 896, 1792, 3684} t6 is {0, 32, 64, 112, 128, 224, 448, 896, 1792, 3684, 7368} Component 13 candidate values: {0, 1, 2, … 8} Component 14 candidate values: {0, 1, 2, … 8} Component 15 candidate values: {1, 2} representing the first CPU pool (i.e., CPU,2) and the second CPU pool (i.e., CPU,3), respectively. Note: UE should not report non-zero value for Component 14 if FG 58-0-1 is not signaled | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

- Alt. 1:

| 58. NR_AIML_air | 58-1-5 | UE-side beam prediction for BM-Case2 with predicted RSRP for inference | 1. Support of beam prediction, reporting of predicted beams and predicted RSRP, for BM-Case2 (domain beam prediction) for inference 2. Supported maximum number of predicted beams in each predicted time instance 3. Supported maximum number of predicted time instances 4. Supported maximum total number of reported predicted beams for predicted time instances in one report 5. Supported value(s) of time gap between predicted time instances and between reference time to the first future time instance | 58-1-4 | yes | n/a | UE-side beam prediction for BM-Case2 for inference is not supported | Per band | n/a | n/a | n/a | Component 2 candidate values: {1, 2, 3, 4} Component 3 candidate values: {1, 2, 4, 8} Component 4 candidate values: {1, 2, 3, 4, 6, 8, 12, 16, 24, 32} Component 5 candidate values: {10ms, 20ms, 40ms, 80ms, 160ms} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- Alt. 2:

| 58. NR_AIML_air | 58-1-4 | UE-side beam prediction for BM Case2 for inference | 1. Support of beam prediction with reporting of predicted beam index for BM-Case2 for inference with UE-side model 3. Maximum number of inference report(s) configured for BM-Case2 per BWP 3a. Maximum number of inference report(s) configured for BM-Case2 across all CCs 6. Support of SSB as RS type for Set B 6a. Support of CSI-RS as RS type for Set B 6b. Support of SSB as RS type for Set A 6c. Support of CSI-RS as RS type for Set A 7a: Supported maximum number of resources for Set B 7b: Supported maximum number of resources for Set A 8. Supported CSI-RS resource types for Set B 9. Supported inference report types 11. Supported maximum number of predicted beams in each predicted time instance 12. Supported maximum number of predicted time instances 13. Supported maximum total number of reported predicted beams for predicted time instances in one report 15. Supported value(s) of time gap between predicted time instances and between reference time to the first future time instance 21. supported number of occupied CPU 22. supported number of occupied CPU,2/CPU,3 23. supported value of d for the relaxation of Z3 timeline, where i is the index of SCS, i=1,2,3,4,5,6 corresponding to 15,30,60,120,480,960 kHz SCS 24. supported value of d’ for the relaxation of Z’3 timeline, where i is the index of SCS, i=1,2,3,4,5,6 corresponding to 15,30,60,120,480,960 kHz SCS 25. Occupied resource pool between CPU,2 and CPU,3 26. Supported value(s) of time gap between predicted time instances and between reference time to the first future time instance | 2-35 | yes | n/a | UE-side beam prediction for BM-Case2 for inference is not supported | Per band | n/a | n/a | n/a | Component 3 candidate values: - Periodic reporting: {1, 2, 3, 4} - Aperiodic reporting: {1, 2, 3, 4} - Semi-persistent reporting: {1, 2, 3, 4} Component 3a candidate values: {1, 2, 3, 4, 8, 10, 12, 16} Component 7a candidate values: {4, 8, 16, 32, 64} Component 7b candidate values: {4, 8, 16, 32, 64} Component 8 candidate values: {Periodic CSI-RS, Semi-persistent CSI-RS} Component 9 candidate values: {Periodic CSI report, Aperiodic CSI report, semi-persistent CSI report} Component 11 candidate values: {1, 2, 3, 4} Component 12 candidate values: {1, 2, 4, 8} Component 13 candidate values: {1, 2, 4, 6, 8, 12, 16, 32} Component 15 candidate values: {10ms, 20ms, 40ms, 80ms, 160ms} Component 21 candidate values: {0, 1, 2, … 8} Component 22 candidate values: {0, 1, 2, … 8} Note: The values of component 21 and 22 are not allowed to be 0 simultaneously. Component 23 candidate values: d1 is FFS d2 is FFS d3 is FFS d4 is FFS d5 is FFS d6 is FFS Component 24 candidate values: d'1 is FFS d’2 is FFS d’3 is FFS d’4 is FFS d’5 is FFS d’6 is FFS Component 25 candidate values: {1, 2} representing the first CPU pool (i.e., CPU,2) and the second CPU pool (i.e., CPU,3), respectively Component 26 candidate values: {10ms, 20ms, 40ms, 80ms, 160ms} Note: UE should not report non-zero value for Component 22 if FG 58-0-1 is not signalled | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 58. NR_AIML_air | 58-1-5 | UE-side beam prediction for BM-Case2 with predicted RSRP for inference | 1. Support of beam prediction, reporting of predicted beams and predicted RSRP, for BM-Case2 (domain beam prediction) for inference 2. Supported maximum number of predicted beams in each predicted time instance 3. Supported maximum number of predicted time instances 4. Supported maximum total number of reported predicted beams for predicted time instances in one report | 58-1-4 | yes | n/a | UE-side beam prediction for BM-Case2 for inference is not supported | Per band | n/a | n/a | n/a | Component 2 candidate values: {1, 2, 3, 4} Component 3 candidate values: {1, 2, 4, 8} Component 4 candidate values: {1, 2, 3, 4, 6, 8, 12, 16, 24, 32} | Optional with capability signalling |

Proposal: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| 58. NR_AIML_Air | 58-2-22 | Support of UE-based positioning Case 1 in RRC_INACTIVE state | UE-based positioning Case 1 in RRC_INACTIVE state is supported | 58-2-3 | N/A | N/A | UE-based positioning Case 1 in RRC_INACTIVE state is not supported | Per Band | FFS | FFS | FFS | Support of UE-based positioning Case 1 in RRC_INACTIVE state at least for data collection | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| 58. NR_AIML_Air | 58-2-23 | Support of Data Collection aided by PRU information for UE-based positioning Case 1 | UE-based positioning Case 1 supports PRU information for data collection | 58.2.1 | N/A | N/A | Data collection aided by PRU in UE-based positioning Case 1 is not supported | Per Band | FFS | FFS | FFS | Note: Data collection when PRU information is involved may include Implicit information (Associated ID associated to the PRU) | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| 58. NR_AIML_Air | 58-2-12 | Support of UE-based positioning Case 1 in RRC_INACTIVE state | UE-based positioning Case 1 in RRC_INACTIVE state is supported | 58-2-3 and 58-2-13 | N/A | N/A | UE-based positioning Case 1 in RRC_INACTIVE state is not supported | Per Band | N/A | N/A | N/A | Support of UE-based positioning Case 1 in RRC_INACTIVE state does not imply that LMF is aware of or controlling UE RRC state | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 58. NR_AIML_Air | 58-2-13 | DL PRS processing capabilities in RRC inactive state for UE-based positioning Case 1 | 1. DL PRS buffering capability a) Type 1 – sub-slot/symbol level buffering b) Type 2 – slot level buffering 2. Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE 3. Max number of DL PRS resources that UE can process in a slot | 58-2-4 | N/A | N/A |  | Per band | N/A | N/A | N/A | Component 1 candidate values: {Type 1, Type 2} Component 2 candidate values: T: {8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms Component 3 candidate values: FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz Need for location server to know if the feature is supported Note: Having the PRS processing capabilities in RRC_INACTIVE state does not imply that LMF is aware of or controlling UE RRC state Note: if UE supports same values for one or more components as in FG 27-6, then the UE can skip indicating these components in this FG and the values in corresponding FG 27-6 components indicate supported PRS processing capabilities in RRC inactive state for Case 1 | Optional with capability signalling |
| 58. NR_AIML_Air | 58-2-14 | Support of UE-based positioning Case 1 in RRC_IDLE | Support of UE-based positioning Case 1 in RRC_IDLE | 58-2-4, 58-2-12, 58-2-13 | N/A | N/A | UE-based positioning Case 1 in RRC_IDLE is not supported | Per Band | N/A | N/A | N/A | Need for location server to know if the feature is supported | Optional with capability signalling |
| 58. NR_AIML_Air | 58-2-15b | DL PRS processing capabilities for aggregated PRS processing of 2 PFLs in intra-band contiguous for RRC_IDLE and RRC_INACTIVE - UE-based positioning Case 1 | 1. Maximum aggregated DL PRS bandwidth in MHz, which is supported and reported by UE 2. Maximum DL PRS bandwidth in MHz, per PFL 3. DL PRS buffering capability 4. Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum aggregated DL PRS bandwidth in MHz, which is supported and reported by UE. 5. Maximum number of aggregated DL PRS resources across aggregated PFLs that UE can process in a slot | 58-2-13 | N/A | N/A |  | Per Band | N/A | N/A | N/A | Component 1 candidate values: a) FR1 bands: {10, 20, 40, 50, 80, 100, 160, 200} b) FR2 bands: {100, 200, 400, 800} Component 2 candidate values:a) FR1 bands: {5, 10, 20, 40, 50, 80, 100} b) FR2 bands: {50, 100, 200, 400} Note: Component 3 in FG 58-2-15b (this FG) follows buffering capability type reported in FG 58-2-4 Component 4 candidate values: a) T: {8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms b) N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms Note: this value N should be equal or smaller than the value N reported by FG 58-2-13 or this value T should be equal or larger than the value T reported by FG 58-2-13 Component 5 candidate values: a. FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz b. FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz Note: each two linked PRS resources are counted as 1 resource Note: this value should be equal or smaller than the value reported by FG 58-2-13 Note: if UE supports same values for one or more components as in FG 41-4-1b, then the UE can skip indicating these components in this FG and the values in corresponding FG 41-4-1b components indicate supported for aggregated PRS processing of 2 PFLs in intra-band contiguous for RRC_IDLE and RRC_INACTIVE capabilities for Case 1 | Optional with capability signalling |
| 58. NR_AIML_Air | 58-2-15c | DL PRS processing capabilities for aggregated PRS processing of 3 PFLs in intra-band contiguous for RRC_IDLE and RRC_INACTIVE - UE-based positioning Case 1 | 1. Maximum aggregated DL PRS bandwidth in MHz, which is supported and reported by UE 2. Maximum DL PRS bandwidth in MHz, per PFL 3. DL PRS buffering capability 4. Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum aggregated DL PRS bandwidth in MHz, which is supported and reported by UE. 5. Max number of aggregated DL PRS resources across aggregated PFLs that UE can process in a slot under it | 58-2-15b | N/A | N/A |  | Per Band | N/A | N/A | N/A | Component 1 candidate values: a) FR1 bands: {15, 20, 30, 40, 50, 60, 80, 100, 120, 140, 150, 160, 180, 200, 240, 300} b) FR2 bands: {150, 200, 300, 400, 600, 800, 1000, 1200} Component 2 candidate values: a) FR1 bands: {5, 10, 20, 40, 50, 80, 100} b) FR2 bands: {50, 100, 200, 400} Note: Component 3 in FG 58-2-15c (this FG) follows buffering capability type reported in FG 58-2-4 Component 4 candidate values: a) T: {8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms b) N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms Note: this value N should be equal or smaller than the value N reported by FG 58-2-13 or this value T should be equal or larger than the value T reported by FG 58-2-13 Component 5 candidate values: a. FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz b. FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz Note: each three linked PRS resources are counted as 1 resource Note: this value should be equal or smaller than the value reported by FG 58-2-13 Note: if UE supports same values for one or more components as in FG 41-4-1c, then the UE can skip indicating these components in this FG and the values in corresponding FG 41-4-1c components indicate supported aggregated PRS processing of 3 PFLs in intra-band contiguous for RRC_IDLE and RRC_INACTIVE for Case 1 | Optional with capability signalling |
| 58. NR_AIML_Air | 58-2-18 | PRS bandwidth aggregation in RRC_ INACTIVE UE-based positioning Case 1 | Support of PRS bandwidth aggregation in RRC_ INACTIVE for UE-based positioning Case 1 | 58-2-12, 58-2-15b | N/A | N/A | PRS bandwidth aggregation in RRC_ INACTIVE for UE-based positioning Case 1 is not supported | Per Band | N/A | N/A | N/A | Need for location server to know if the feature is supported. | Optional with capability signalling |
| 58. NR_AIML_Air | 58-2-19 | PRS bandwidth aggregation in RRC_IDLE - UE-based positioning Case 1 | Support of PRS bandwidth aggregation in RRC_IDLE for UE-based positioning Case 1 | 58-2-14, 58-2-15b | N/A | N/A | PRS bandwidth aggregation in RRC_IDLE for UE-based positioning Case 1 is not supported | Per Band | N/A | N/A | N/A | Need for location server to know if the feature is supported. | Optional with capability signalling |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 58. NR_AIML_air | 58-0-1 | CSI report framework for UE-side inference | 1. Number of CPU,x pools N 2. Maximum number of CPU,x in each CPU,x pool of UE-sided inference for CSI report(s) simultaneously in a CC 3. Maximum number of CPU,x in each CPU,x pool of UE-sided inference for CSI report(s) simultaneously across all CCs | 2-35 | yes | n/a | Maximum number of CPU(s),x per each CPU pool and CPU pool(s) for UE-sided inference is unknown to the network | Per band and per BC | n/a | n/a | n/a | Component 1 candidate values: {1,2} Component 2candidate values: {1…8} Component 3 candidate values: {1…32} Note: Component 2 and 3 candidate values are signalled separately for each pool A UE that does not support this FG should not report non-zero occupied CPU,2 or CPU,3 values in any dependency FG | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 58. NR_AIML_air | 58-1-1 | Increased number of reported RSs for beam management | 1. Support of reporting format for L1-RSRP measurements not including CRI/SSBRI other than one for the largest measured L1-RSRP in a reporting instance, if the number of reported L1-RSRPs is equal to the size of the measurement resource set. 2. Support of reporting format for L1-RSRPs and corresponding beam information of Top M beam(s) with largest M measured value(s) of L1-RSRP(s) of a measurement resource set, where M is configured by gNB, if the number of reported L1-RSRPs is smaller than the size of the measurement resource set 3. Maximum number of M reported RSs, M>4 | {2-21, 2-22} or {2-23, 2-23a} or {2-29 or 2-24}2-24 or 2-29 | yes | n/a | Increased number of reported beams for beam management is not supported | Per band | n/a | n/a | n/a | Component 3 candidate values: {6,8} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 63. NR_Mob_Ph4 | 63-8 | Inclusion of current SpCell in the L1 measurement report based on CSI-RS (s) | 1. Support of always including the current SpCell in the L1 measurement report based on CSI-RS (s) | 63-1, or 63-2, 63-1a, or 63-2a | Yes | No | UE does not always include measurement report for SpCell in the L1 measurement report based on CSI-RS (s) | Per BC | n/a | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 63. NR_Mob_Ph4 | 63-10 | CSI-RS measurement and CSI reporting without CSI-IM reception | 1. Support of CSI-RS measurement and CSI reporting for candidate cells without CSI-IM resource configuration | 63-6 or 63-6a or 63-7 or 63-7a or 63-7c or 63-7d | Yes | No | CSI -RS measurement and CSI reporting without CSI-IM reception is not supported | Per band | n/a | n/a | n/a |  | Optional with capability signaling |

Proposal: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| 59. NR_MIMO_Ph5 | 59-4-4c | Support two TAs enhancement for sDCI based intra-cell Multi-TRP operation | Support of two TAs without the restriction of multi-DCI based multi-TRP operation for sDCI based intra-cell Multi-TRP operation | 40-1-1 | yes | n/a | Two TAs without the restriction of multi-DCI based multi-TRP operation sDCI based intra-cell Multi-TRP operation is not supported | Per FS | No | No | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 59. NR_MIMO_Ph5 | 59-2-1-5b | Extended Rel-18 eType-II Doppler codebook for 128 Tx ports | 1. Support of extended Rel-18 Type-II Doppler codebook for 128 Tx ports by aggregating multiple NZP CSI-RS resource groups within 1 slot 2. Support X=1 CQI based on the first/earliest slot of the CSI reporting window and the first/earliest predicted PMI (TDCQI=’1-1’) 3. Support of PMI subband R=1 for extended Rel-18 eType II Doppler codebook 4. Support parameter combinations with L=2,4 5. Support for rank = 1,2 6. Support 64 ports 7. A list of supported combinations, each combination is { Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously 8. Supported processing capability 9. Value of Y for CPU occupation (OCPU = Y.N4), when P/SP-CSI-RS is configured for CMR 10. Value of Y for CPU occupation (OCPU = Y. KDOPP), when A-CSI-RS is configured for CMR 11. Support for the size of DD-basis, N4=1 12. Scaling factor for active resource counting Kp 13. Support 4 CSI-RS resources in a resource group for aperiodic CSI-RS resource set or in a resource set for periodic CSI-RS resource set 14. A list of supported combinations, each combination is {Max # of resources and total # of Tx ports} per CC simultaneously | 59-2-1-5 | yes | n/a | Extended Rel-18 Type-II Doppler codebook is not supported for 128 Tx ports, aggregated CSI-RS resources within one slot | Per band and per BC | n/a | n/a | n/a | Component 7 candidate values a. {1, …, 64, 128, 256} b. {64, …, 256, 512, 768, 1024} Component 8 candidate value {Capability 1, Capability 2} Component 9 candidate values: {1, 2, 3} Component 10 candidate values: {1, 2, 3} Component 12 candidate values: {1, 2, 4} Component 14 candidate values a. {1, …, 64, 128, 256} b. {64, …, 256, 512, 768, 1024} Note: For component of processing capability Capability 1: Legacy timeline OCPU = Y x N4 x ceil(P/32) ), when P/SP-CSI-RS is configured for CMR OCPU = Y x KDOPP x ceil(P/32)), when A-CSI-RS is configured for CMR Capability 2: Scale the legacy timeline by ceil(P/32) where P is the total number of ports across all the K aggregated CSI-RS resources OCPU = Y x N4, when P/SP-CSI-RS is configured for CMR OCPU = Y x KDOPP, when A-CSI-RS is configured for CMR Note: maximum OCPU is 8 Note: KDOPP is the number of CSI-RS resource groups configured for channel measurement, and each CSI-RS resource groups contain K CSI-RS resources for aggregating up to 128 ports | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 59. NR_MIMO_Ph5 | 59-1-1 | UE-initiated/event-driven beam management for Event-2 based measurement and report for Mode A | 1. Support of UE-initiated/event-driven beam report based on one event instance 2. Support of Event-2 based measurement and report 3. Support of Mode A UE-initiated/event-driven beam report 4. Maximum number of the configured RS(s) for new beam in the RS resource set 5. Support of current beam measurement by using QCL RS in the indicated TCI state and the corresponding QCL SSB for Scheme-1 and Scheme-2, respectively 6. Support the first PUCCH and second PUSCH from the same PUCCH group | 2-22, 2-24 | yes | n/a | UEI/ED beam report is not supported for Event-2 and Mode A | Per band | n/a | n/a | n/a | Component 4 candidate values: {1, 2, …, 64} Note For Component 4 and Component 5, an SSB can be associated with the serving cell PCI or a PCI other than the serving cell PCI Note: Regarding Event-2, QCL RS(s) in indicated TCI state(s) and resources configured for component 4 are also counted in FG 16-1g, and 16-1g-1 | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 59. NR_MIMO_Ph5 | 59-2-1-7 | Group-specific 3-bit scaling factors for up to 128 ports | Support of group-specific 3-bit scaling factors | One or more of {59-2-1-1, 59-2-1-1c} | yes | n/a | Group-specific 3-bit scaling factors is not supported | Per band and per BC | n/a | n/a | n/a | Candidate values: {’rank-1’, ‘rank-1 and rank-2’} Note: ‘rank-1’is the lower capability Note: 3-bit scaling applies only to the Type-I SP codebook | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 59. NR_MIMO_Ph5 | 59-2-3-1 | CJTC Dd report | 1. Configured minimum quantization range for CJTC Dd reporting 2. Configured maximum resolution (number of steps) for the quantization alphabet for CJTC Dd reporting 3. Supported value of scaling factor X for OCPU calculation | 2-35 | yes | n/a | CJTC Dd report is not supported | Per band and per BC | n/a | n/a | n/a | Component 1 candidate values: {half cyclic prefix, full cyclic prefix} Note: For Component 1, ‘half cyclic prefix’ is the lower capability Component 2 candidate values: {32, 64, 128, 256} Component 3 candidate values: {1, 2} Note：OCPU =X.NTRP | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 59. NR_MIMO_Ph5 | 59-2-3-2 | CJTC FO report | 1. Configured minimum quantization range for CJTC FO reporting 2. Configured maximum resolution (number of steps) for the quantization alphabet for CJTC FO reporting 3. Supported value of scaling factor X for OCPU calculation | 2-35 | yes | n/a | CJTC FO report is not supported | Per band and per BC | n/a | n/a | n/a | Component 1 candidate values: {0.1ppm, 0.2ppm} Note: For Component 1 ‘0.1ppm’ is the lower capability Component 2 candidate values: {16, 32, 256} Component 3 candidate values: {1, 2} Note：OCPU =X.NTRP Note: parts per million (ppm) of the carrier frequency | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-2-3-5 | CJTC Dd+FO report | 1. Configured minimum quantization range for CJTC Dd reporting 2. Configured maximum resolution (number of steps) for the quantization alphabet for CJTC Dd reporting 3. Configured minimum quantization range for CJTC FO reporting 4. Configured maximum resolution (number of steps) for the quantization alphabet for CJTC FO reporting 5. Supported value of scaling factor X for OCPU calculation | 2-35, 59-2-3-1, 59-2-3-2 | yes | n/a | CJTC Dd+FO report is not supported | Per band and Per BC | n/a | n/a | n/a | Component 1 candidate values: {half cyclic prefix, full cyclic prefix} Note: For Component 1, ‘half cyclic prefix’ is the lower capability Component 2 candidate values: {32, 64, 128, 256} Component 3 candidate values: {0.1ppm, 0.2ppm} Note: For Component 1 ‘0.1ppm’ is the lower capability Component 4 candidate values: {16, 32, 256} Component 5 candidate values: {1, 2} Note: OCPU =2X.NTRP Note: parts per million (ppm) of the carrier frequency | Optional with capability signalling |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 59. NR_MIMO_Ph5 | 59-2-1-5l | Processing timeline for CSI reference slot for extended Rel-18 Type-II Doppler codebook for up to 128 ports | 1. Aperiodic CSI report timing relaxation, w, extended Rel-18 Type-II Doppler codebook for up to 128 ports 2. Aperiodic CSI report timing relaxation, type, for extended Rel-18 Type-II Doppler codebook for up to 128 ports | 59-2-1-5 | Yes | n/a |  | Per band and Per BC | n/a | n/a | n/a | Component 1 candidate values: UE reports candidate value, w, independently for each SCS in unit of symbols: {14*(KP–1)*d, 14*KP*d} Note: Kp is according to Component 12 of FG59-2-5 Note: d=4 (minimum periodicity of periodic CSI-RS) Component 2 candidate values: {CAP1, CAP2} Note: For Component 2 ‘CAP2’ is the lower capability For N4 = 1 1) For AP CSI-RS: (Z,Z’) = (Z2 + 14*( KDOPP –1)*m, Z'2) 2) For P/SP CSI-RS: (Z,Z’) = (Z2 + w, Z'2) For N4 > 1 and CAP1 in component 2 1) For AP CSI-RS: (Z,Z’) = (Z2 + 14*( KDOPP –1)*m, Z'2) 2) For P/SP CSI-RS: (Z,Z’) = (Z2 + w, Z'2) For N4 > 1 and CAP2 in component 2 1) For AP CSI-RS: (Z,Z’) = (Z2 + 14*( KDOPP –1)*m + Z'2, 2Z'2) 2) For P/SP CSI-RS: (Z,Z’) = (Z2 + w + Z'2, 2Z'2) Z2/Z'2 are defined in Table 5.4-2 in TS38.214 KDOPP is the number of CSI-RS resource groups configured for channel measurement, and each CSI-RS resource groups contain K CSI-RS resources for aggregating up to 128 ports M = {1,2}, is the offset between two adjacent AP CSI-RS resource groups for the CMR in slots This FG is not applicable to FR 2-2 | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal: Add the following note “Note: Capability 1 is a higher capability than capability 2” for FGs 59-2-1-1, 59-2-1-1a, 59-2-1-1b, 59-2-1-1c, 59-2-1-1d, 59-2-1-1e, 59-2-1-2, 59-2-1-2a, 59-2-1-2b, 59-2-1-3, 59-2-1-3a, 59-2-1-3b, 59-2-1-4, 59-2-1-4a, 59-2-1-5, 59-2-1-5a, 59-2-1-5b, and 59-2-1-9

Proposed Conclusion:

- When UE indicates both per band and per BC capability, if the capability/component is not counted across CCs and CA is not configured, then

  - gNB only considers per Band capability regardless of reported per BC capability (i.e., per BC capability can be ignored in this case)

- When UE indicates both per band and per BC capability, if the capability/component is counted across CCs and CA is not configured, then

  - Alt. 1-1: gNB only considers per Band capability regardless of reported per BC capability (i.e., per BC capability can be ignored in this case)

  - Alt. 1-2: take the minimum capability between per BC capability and per band capability for the single band

- When UE indicates both per band and per BC capability, if the capability/component is counted across CCs and CA is configured, then

  - Alt. 2-1: assume that the per BC indication is more restrictive than the per band indication in any band combination

  - Alt. 2-1: a per BC capability only applies in the case when the CA operation is configured for the BC. Further, the per BC capability limits the capability/component counted across all CCs in the bands of the BC and for each single band of each CC, take the minimum capability between per BC capability and per band capability.

R1-2508355	UE features Batch C (AIML, MIMO, MobEnh)	Nokia

R1-2508417	Remaining issues of UE features for AI/ML for NR Air Interface	vivo

R1-2508484	Remaining issues for Rel-19 WIs in UE features Batch C	Huawei, HiSilicon

R1-2508532	Discussion on UE features Batch C	ZTE Corporation, Sanechips

R1-2508578	Remianing issues on UE features for NR MIMO Phase 5	CATT

R1-2508635	Summary of UE Features Batch C	Moderator (AT&T)

R1-2508669	UE features for Rel-19 AI based CSI prediction	Xiaomi

R1-2508699	UE features for Batch C	OPPO

R1-2508748	Discussion on UE features for AI/ML for NR Air Interface	LG Electronics

R1-2508787	UE features for batch C	Samsung

R1-2509091	Views on UE features Batch C	Apple

R1-2509180	On UE features Batch C	Ericsson

R1-2509212	UE features Batch C	Qualcomm Incorporated

R1-2509265	Discussion on UE features Batch C	NTT DOCOMO, INC.