3GPP TSG-RAN WG1 Meeting #124	     R1-2601415

Gothenburg, Sweden, February 9 – 13, 2026

Agenda Item:	10.1

Source:	Moderator (Huawei)

Title:	FLS#2 on evaluation assumptions for 6GR air interface

Document for:	Discussion and Decision

# Introduction

Evaluation assumptions for 6GR air interface (in AI 10.1 from chair notes)

Update of traffic models and evaluation models that can be commonly used for evaluating technology proposals.

[124-R20-6GR-Evaluation] Email discussion on Rel-20 6GR-Evaluation – Jinhuan (Huawei)

- To be used for sharing updates on online/offline schedule, details on what is to be discussed in online/offline sessions, tdoc number of the moderator summary for online session, etc

This meeting is supposed to be the last remaining meeting for this agenda. The critical issues are high prioritized, including

- UE antenna modelling

- Traffic models

- Refinement/update on the agreements mainly on some of SLS assumptions.

- Note: NTN related proposals are NOT summarized nor discussed in this agenda.

Kind Note for the discussions in this summary:

- Different rounds with proposals with label [FL1, FL2, …] of discussion may be created in this summary for proposal update based on the online/offline progress. It’s companies’ discretion to provide your comments to either round but just being reminded that the discussion is supposed to carry on the proposal in the latest round in principle.

- Please use the following convention for uploading your comments:

  - Filename_v001_Moderator

  - Filename_v002_Moderator_CompanyA

  - Filename_v003_CompanyA_CompanyB

  - Filename_v004_CompanyB_CompanyC

  - Etc

# Antenna modeling for TN

## (CLOSED)UE antenna modelling

### Companies’ views

| Company | Views/proposals |
| --- | --- |
| CATT | Proposal 1: For the evaluation assumption in 6GR, for the UE antenna elements, the suggested values are provided as follows: - Around 700MHz - UE: Up to 4Tx/Rx, including 1T1R, 1T2R, 1T4R, 2T4R and 4T4R - Around 4GHz - UE: Up to 8Tx/Rx (4 Tx/Rx for handheld device), including 1T2R, 1T4R, 2T4R, 4T4R, 4T8R and 8T8R - Around 7GHz - UE: Up to 8Tx/Rx (4 Tx/Rx for handheld device), including 1T2R, 1T4R, 2T4R, 4T4R, 4T8R and 8T8R. |
| CMCC | According to the current agreement in RAN plenary, we have the following considerations: - For Combination 0, it only applies to IoT type devices. - For Combination 1 and 2, they had already been widely implemented in 5G devices. - For Combination 4, it should be only applied for FWA, and whether 16T16R is feasible can be further discussed. - For Combination 3, at least 4T8R should be considered for handheld device like foldable smartphone, and whether 8T8R is feasible can be further discussed. |
| Ericsson | Proposal 4-1 - For UE antenna modelling, evaluations should consider realistic UE antenna modelling (e.g., non-uniform antenna positioning, use of directional antennas, power imbalance / insertion loss among antennas, and limited control of relative phase). UE/CPE models in TR38.901 can be considered as the starting point with further elaboration when needed. - Use Alt2, i.e., the UT device antenna model using candidate antenna locations as described in section 7.3 in TR38.901, as a starting point for 6G evaluations of handheld devices. For at least UL MIMO evaluations, extend the model as necessary to take into account variation among UE implementations, configurations, and bands, as discussed in - Use TR 38.901 assumptions for UE antenna element gain patterns and antenna polarization as baseline assumptions for 6G evaluations - For evaluations relevant to massive communications, 1Rx and 1Tx should be considered for at least 700MHz and 2GHz carrier frequencies. Table 2.4-1 – Detailed UE antenna assumptions Around 700MHz; Around 2GHz; Around 4GHz; Around 7GHz; Around 30GHz. Handheld UE for eMBB and Positioning; 1Tx and 2Rx (ITU allows up to 4 Tx/Rx); Up to 2 Tx and 4 Rx; Up to 4 Tx and 4 Rx; Up to 4 Tx and 8 Rx; Up to 8 Tx and 8 Rx. Non-Handheld UE (e.g., Computers) for eMBB and Positioning; Up to 8 Tx and 8 Rx; Up to 8 Tx and 8 Rx; Up to 32 Tx and 32 Rx. Handheld UE for High Reliability Low Latency Communications (HRLLC); Up to 4 Tx and 4 Rx; Up to 4 Tx and 8 Rx. Non-Handheld UE for HRLLC; Up to 4 Tx and 8 Rx; Up to 4 Tx and 8 Rx. CPE for Fixed Wireless Access (FWA) Service (see Note 1); Up to 2 Tx and 4 Rx; Up to 4 Tx and 8 Rx; Up to 8 Tx and 8 Rx; Up to 8 Tx and 16 Rx. Machine Type Communications (MTC) Service; 1 Tx/Rx antenna; 1 Tx/Rx antenna; 1 Tx and up to 2 Rx. Sensing-Capable UE for Integrated Sensing and Communication (ISAC) Service; Up to 8 Tx and 8 Rx; Up to 8 Tx and 8 Rx; Up to 32 Tx and 32 Rx. Note - Indoor and outdoor CPEs may have different antenna configurations.. |
| Google | Proposal6: Prioritize a single-panel UE configuration for evaluations in FR2/FR3. Proposal7: Support the following for the 6GR evaluations for UL antenna configurations: - 2Tx/4Rx for > 1GHz and TDD band and 1Tx/2Rx for <= 1GHz - 1Tx/1Rx for wearables - 2Tx/4Rx for NTN bands - 4Tx/6Rx/8Rx are optional. Proposal8: 16T16R (combination 4) is not a practical assumption for handheld UEs. |
| Huawei | Support combinations 0~4 as discussed in the last meeting. |
| Intel | - For UE Antenna, use (FL6) Proposal 2.2.2 rv4 as a starting point for discussion in RAN1#124 - For UE Antenna Combination 4, remove the 16T16R option, to align with recent RAN#110 agreement - For UE Antenna, Alt 2 is default - It is up to individual technology study topics to pick Alt 1 over Alt 2 if needed Proposal 4 - For UE Antenna in 30 GHz carrier frequency, - 8 AE: 2R: (1, 4, 2, 1, 1; 1, 1), 4R: (1, 4, 2, 1, 2; 1, 1), 8R: (1, 4, 2, 1, 1; 1, 4) - 24 AE: 6R: (1, 4, 2, 1, 3; 1, 1) - 32 AE: 2R: (4, 4, 2, 1, 1; 1, 1), 4R: (2, 4, 2, 1, 2; 1, 1), 8R: (1, 4, 2, 1, 4; 1, 1) |
| Interdigital | We strongly suggest adopting the UT antenna assumptions provided by the moderator with additional modification made by the Chair based on inputs from companies. Combination 0, 3, and 4 are not intended for handheld UE. |
| MediaTek | Proposal3: For key parameters of UE antenna model, prioritize the following value for the evaluation in 6G study: - Around 700 MHz: - Option 1: 1 Tx and 1 Rx - Option 2: 1 Tx and 2 Rx - Option 3: 2 Tx and 4 Rx - Around 4 GHz: - Option 1: 2 Tx and 4 Rx - Option 2: 4 Tx and 8 Rx - Around 7 GHz: - Option 1: 2 Tx and 4 Rx - Option 2: 4 Tx and 8 Rx |
| Nokia | - Proposal8: RAN1 to select UE candidate antenna locations (4, 8) as a baseline for Configuration 1 (1T2R), Alt 2 to better support antenna blockage spatial non-stationarity modelling at below 1 GHz. - Proposal9:RAN1 to select UE candidate antenna locations (1, 3, 5, 7) that were already calibrated in TR 38.901 as a baseline for Configuration 2 (4 antenna elements), Alt 2. - Proposal10: RAN1 to consider a UE antenna model configuration with 8 total number of antenna elements (4T8R and 8T8R) including 2 GHz carrier frequency. - Proposal11: RAN1 to consider a UE antenna model configuration with 16 total number of antenna elements (4T16R and 8T16R) intended only for FWA. - Proposal12: RAN1 should use new directive antenna model described in section 7.3 of Rel-19 version of TR 38.901 (Alt 2) for UE handheld devices by default for SLSs in the 6GR study. - Proposal13: RAN1 to clarify that if Alt 1 legacy UE antenna modelling is considred for handheld devices (M,N,P,Mg,Ng; Mp,Np) then isotropic antenna radiation pattern is assumed. - Proposal14: RAN1 to define the following antenna radiation patterns for the CPE devices with 4, 8, and 16 antenna elements: - Isotropic, - Directional with different half power beamwidth and maximum directional gains as described in above, - CPE can be equipped with 1 to 3 antenna panels, each following Alt 1 configuration, and the orientation of the device with a single antenna panel can be optimized, - Omnidirectional as described in above. - Proposal15: CPEs can be equipped with multiple antenna panels (as described in TR 38.802 in Table A.3.1-4) and CPE orientation can be optimized. |
| DOCOMO | Proposal 6 - For combination3 in UE antenna modeling, delete the following note to study handheld UE with 8 antennas. - This combination is not intended for handheld UE. Proposal 7 - For applicable frequency range for 30GHz, combination3 and 4 should be included. |
| Ofinno | Observation 9: The number of UE Tx and Rx antennas agreed in TR 38.914 for different deployment scenarios are relevant for 6GR evaluations in RAN1. Proposal 11: Consider the agreed number of Tx and Rx antenna elements in TR 38.914 for RAN1 evaluations for relevant frequencies as follows: - up to 16 Rx antenna elements for evaluations at carrier frequencies around 4 GHz and 7 GHz only for FWA; - up to 8 Tx and 8 Rx antenna elements for evaluations at carrier frequencies around 2 GHz, 4GHz, and 7 GHz for eMBB (handheld) and FWA; - up to 4 Tx and 4 Rx for evaluations at carrier frequencies of 700 MHz, 2 GHz, 4 GHz, and 7 GHz for IoT, eMBB (handheld), and FWA. |
| OPPO | Update based on the FL proposal discussed in the last meeting: - delete combination 4. - Delete 8T8R and add 2T8R for combination 3. |
| Qualcomm | Proposal1: The Rel-19 defined handheld UT and CPE antenna model should be used as the default in system-level simulations. Legacy omnidirectional antenna could be used for link level simulation. Proposal 2: Study extension of CPE antenna model for larger array and more than 8 antennas. Proposal 3: For 6GR evaluation, consider modelling the UE antenna configuration for frequency up to 7GHz as in Table - Table 1: UE antenna configurations for 6GR evaluation UE antenna modelling for RAN1 evaluations; Total number of antenna elements; Total number of TXRU; Applicable carrier frequency. Combination 0 Note: This combination is not for handheld UE; 1; 1T1R,; 700MHz, 2GHz 4GHz. Combination 1; 2; 1T2R; 700MHz, 2GHz, 4GHz. Combination 2; 4; 1T4R, 2T4R, 4T4R; 700MHz 2GHz, 4GHz, 7GHz. Combination 3; 6; 2T6R, 6T6R,; 4GHz, 7GHz. Combination 4; 8; 1T8R, 4T8R, 8T8R; 2GHz, 4GHz, 7GHz. Combination 5 Note: This combination is intended only for FWA; 16; 4T16R, 8T16R, 16T16R; 7GHz,. For around frequency 30GHz, we can reuse the existing NR assumption as shown in Table 2 for 6GR evaluation. Table 2: 30GHz UE antenna configurations for 6G evaluation UE antenna configuration; Values. # of antenna elements; Up to 32 elements, 8 elements per panel (M, N, P) = (2, 2, 2). # of panels; Config 1 can be 2 panels on front and back Config 2 can be 4 panels on 4 edges. # of TXRUs; 2T2R per panel. |
| Samsung | Proposal #3: Evaluation assumptions for UE antenna modeling should be differentiated based on the device type (e.g., Handheld or Advanced UE or low-capable UE) to verify the performance specific to each device's specific characteristics. Proposal #4: For the evaluation of handheld type devices and IoT devices, 1Tx should be assumed as the baseline, and the antenna modelling should support a maximum of up to 4 antenna ports as table 2.1-1. Table 2.1-1 Antenna configurations for Handheld and low-capable UE UE antenna modelling for RAN1 evaluations; Total number of antenna elements; Total number of TXRU; UT device antenna model using candidate antenna locations as described in section 7.3 in TR38.901; Applicable carrier frequency. Combination0 (This combination is for low-capable UE); 1; 1T1R; 1T and 1R; 700MHz, 2GHz. Combination1; 2; 1T2R; 2R: [(1, 5), or (4, 8)] as described in section 7.3 in TR 38.901.; 700MHz, 2GHz, 4GHz. Combination2; 4; 1T4R, 2T4R; 4R: [(2, 4, 6, 8), or (1, 3, 5, 7)] as described in section 7.3 in TR 38.901; 700MHz 2GHz, 4GHz, 7GHz, 15GHz. Proposal #5: For the evaluation of Advanced UE types, antenna modelling should support up to 8Tx and 16Rx as table 2.1-2, reflecting their larger form factors and specific design capabilities. Table 2.1-2 Antenna configurations for advanced UE UE antenna modelling for RAN1 evaluations; Total number of antenna elements; Total number of TXRU; (M,N,P,Mg,Ng; Mp,Np), (dH,dV), (dg,H,dg,V); Applicable carrier frequency. Combination2; 4; 1T4R, 2T4R, 2T4R, 4T4R; 4R: (M, N, P, Mg, Ng; Mp, Np)=(1, 2, 2, 1, 1; 1, 2) for dual polarization or (2, 2, 1, 1, 1; 2, 2) for single polarization, (dH,dV)= (0.5, 0.5)λ; 700MHz 2GHz, 4GHz, 7GHz, 15GHz. Combination3; 8; 1T8R, 2T8R, 4T8R, 8T8R; 8R: (M, N, P, Mg, Ng; Mp, Np)= (1, 4, 2, 1, 1; 1, 4) for dual polarization or (2, 4, 1, 1, 1; 2, 4) for single polarization , (dH,dV)= (0.5, 0.5)λ; 4GHz, 7GHz, 15GHz. Combination4; 16; 1T16R, 2T16R, 4T16R, 8T16R; 16R: (M, N, P, Mg, Ng; Mp, Np)= (2, 4, 2, 1, 1; 2, 4) , (dH,dV)= (0.5, 0.5)λ; 7GHz, 15GHz. |
| T-Mobile USA, Ericsson, MediaTek, Nokia, Verizon | - Proposal4: RAN1 agrees to using CPE antenna placement based on reference CPE antenna locations from section 7.3 in TR 38.901 as a starting point for where array elements are in a single panel and have different boresights and the panel can be assumed to point at a serving gNB. The UT antenna model can be used as a starting point for CPEs where a panel cannot be assumed to point at a serving gNB. Uniform linear or planar arrays are a starting point for horizontally omnidirectional elements in CPEs. Other antenna / (multi-) panel configurations are not precluded. |
| vivo | Propsoal1: For UE antenna modeling in 6G evaluation, support adding the following clarifying notes: - Combination 3 is not intended for handheld UEs. - Combination 4 is only intended for FWA. - All the combinations are intended for evaluation purpose. It does not impact future discussions on UE capability. |
| Xiaomi | Proposal 2: For UE antenna modelling for RAN1 evaluation - Support combination 0,1,2,3,4 with total UE antenna number of 1, 2, 4, 8, 16, respectively - Include 2T8R for combination 3 with total UE antenna number of 8 - Do not include 16T16X for combination 4 with total UE antenna number of 16 - Clarify that 4T4R for combination 2 is not for handheld UEs in carrier frequency around 4GHz or below - Clarify that combination 3 with total UE antenna number of 8 is not for handheld UEs in carrier frequency around 4GHz or below - Clarify that combination 4 with total UE antenna number of 16 is not for handheld UEs Proposal 3: Consider the legacy UE antenna model, represented by a 5-element tuple (M, N, P, Mg, Ng), in the evaluation of 6GR. |

### Discussions

Background

The UE antenna modelling for the carrier frequency under 30GHz was discussed in the last meeting but was not agreed. The controversy in the last meeting was mainly about which combination should be clarified for which type of UEs. For instance, whether 8R is intended for hand-held UE was very controversial even among UE vendors.

The RAN plenary further discussed and agreed on several maximum values for UE antenna numbers as follows:

For the purpose of further study in RAN and RAN WGs for frequency up to 7GHz, evaluate the following UE RX/TX antennas: Up to 4 Tx and Rx antenna elements Up to 8 Tx and Rx antenna elements Up to 16 Rx antenna elements (intended only for FWA) Note: Above does not impact future discussions on UE capability

Key issues discussed in the contributions for this meeting:

- Companies continued discussing the number of UE antenna for different types of UEs, e.g., Ericsson, Google, Samsung.

- Some companies suggested deleting 16R, e.g., CATT, MediaTek, OPPO; Some companies view that 8R is not applicable for 4GHz and below but the other companies view the opposite.

- Whether 8R is for handheld-UE is still controversial

  - Support 8R for handheld-UE: CMCC, Ericsson, Huawei, DOCOMO, Nokia, Ofinno, Qualcomm

  - Not support 8R for handheld-UE: CATT, Interdigital, Samsung, vivo, Xiaomi (not for <=4GHz)

- Some companies further discussed which alternative should be assumed for the UE antenna locations:

  - Support Alt1: Xiaomi

  - Support Alt2: Ericsson, Intel, Nokia, Qualcomm

- Some companies discussed the UE antenna for 30GHz carrier frequency: Intel, Qualcomm, DOCOMO, Ericsson

- Some companies discussed the specific consideration for the FWA/CPE antenna modelling, including:

  - Antenna radiation pattern:

    - The isotropic antenna radiation pattern should be assumed for Alt1 for handheld devices: Nokia

    - The antenna radiation patterns for the CPE devices to consider: Isotropic, Directional with specific values of both horizontal and vertical directions and element max gain, Omnidirectional in horizontal plane: T-Mobile USA, Ericsson, MediaTek, Nokia, Verizon

  - Antenna locations:

    - Extending the CPE antenna model to support more than 9 locations for Alt2: Qualcomm.

    - CPEs can be equipped with multiple antenna panels following Alt 1 configuration for each: Nokia

Plan for the discussions in this meeting:

Take the moderator’s proposal from the last meeting as the starting point and address the FFS and issues discussed in the contributions. In particular,

- The yellow highlighted parts will be further discussed/explained based on the discussions from the contributions, e.g., discuss whether take Alt2 as the default for handheld UE, whether add 2T8R into the table for combination3.

- Discuss the newly proposed UE antenna modelling for 30GHz carrier frequency.

- Discuss/Clarify the antenna radiation pattern assumed.

Round-1 discussions:

(FL1) Proposal 2.1.2-1

For 6GR evaluation, RAN1 to model the UE antenna as follows for below 30GHz carrier frequency,

- Note: Each of other topics could further decide to use which combination(s) for the evaluations. Other combinations are not precluded for evaluations, e.g., 2T6R, 3T6R, 6T6R, 6T8R.

- Note: The antenna locations in Alt1 and Alt 2 in the following table are considered as examples and used for performance calibration. Any antenna array structures and/or antenna locations in section 7.3 in TR38.901 is possible for evaluations and up to companies to report.

- Note: The antenna locations in Alt 2 not included in section 7.3 in TR38.901 are up to companies to report.

- FFS: Alt1 or Alt2 is used for each of the combination.

- Note: The radiation power pattern of a single antenna element in Table 7.3-2 TR38.901 is assumed for handheld UT. FFS for CPE.

- Note: The antenna element/location of T is a subset of the element/locations for R.

- Note: The mapping between the combination and the device types might be separately discussed.

| UE antenna modelling for RAN1 evaluations | Total number of antenna elements | Total number of TXRU | Alt 1: (M,N,P,Mg,Ng; Mp,Np), (dH,dV), (dg,H,dg,V) if any, or Alt 2: UT device antenna model using candidate antenna locations as described in section 7.3 in TR38.901 | Applicable carrier frequency |
| --- | --- | --- | --- | --- |
| Combination0 NOTE1 | 1 | 1T1R, | Alt 1: 1T: (M, N, P, Mg, Ng; Mp, Np)=(1, 1, 1, 1, 1; 1, 1) 1R: (M, N, P, Mg, Ng; Mp, Np)=(1, 1, 1, 1, 1; 1, 1) Alt 2: - 1T - 1R | 700MHz, 2GHz |
| Combination1 | 2 | 1T2R, | Alt 1: - 2R: (M, N, P, Mg, Ng; Mp, Np)=(1, 2, 1, 1, 1; 1, 2) for single polarization or (1, 1, 2, 1, 1; 1, 1) for dual polarization, (dH,dV)= (0.5, 0.5)λ Alt 2: - 2R: [(1, 5), or (4, 8)] as described in section 7.3 in TR 38.901. | 700MHz, 2GHz, 4GHz |
| Combination2 | 4 | 1T4R, 2T4R, 4T4R | Alt 1: - 4R: (M, N, P, Mg, Ng; Mp, Np)=(1, 2, 2, 1, 1; 1, 2) for dual polarization or (2, 2, 1, 1, 1; 2, 2) for single polarization, (dH,dV)= (0.5, 0.5)λ Alt 2: - 4R: [(2, 4, 6, 8), or (1, 3, 5, 7)] as described in section 7.3 in TR 38.901 | 700MHz, 2GHz, 4GHz, 7GHz, 15GHz |
| Combination3 | 8 | 1T8R, 2T8R, 4T8R, 8T8R | Alt 1: (M, N, P, Mg, Ng; Mp, Np)= (1, 4, 2, 1, 1; 1, 4) for dual polarization or (2, 4, 1, 1, 1; 2, 4) for single polarization , (dH,dV)= (0.5, 0.5)λ Alt 2: (1, 2, 3, 4, 5, 6, 7, 8) as described in section 7.3 in TR38.901 | 2GHz, 4GHz, 7GHz, 15GHz NOTE3 |
| Combination4 NOTE2 | 16 | 4T16R 8T16R, | Alt 1: - 16R: (M, N, P, Mg, Ng; Mp, Np)= (2, 4, 2, 1, 1; x, y) , (dH,dV)= (0.5, 0.5)λ Alt 2: - 16R: (1, 2, 3, 4, 5, 6, 7, 8) as described in section 7.3 in TR38.901, dual polarization | 7GHz, 15GHz |
| NOTE1: This combination is for IoT UE only. NOTE2: This combination is for CPE UE only. NOTE3: If number of TXRU and frequency combination is applicable. |  |  |  |  |

Any comments/suggestions, please leave them here:

| Company | Comments |
| --- | --- |
| ZTE | We have following comments: - Since Combination 4 is for CPE UE only, Alt 2 should be removed. - For combination 3, add Config. (M, N, P, Mg, Ng; Mp, Np)= (2, 2, 2, 1, 1; 2, 2) for dual polarization, as it can be used for CPE. |
| vivo | Considering the practical UE implementation, and the objective of introducing Alt2 is to align with real antenna deployments, we propose adding the following contents: - Note: All the combinations are intended for evaluation purpose. It does not impact future discussions on UE capability. - Note: the antenna element-wise power variation at the UE in TR 38.901 section 7.6.14.2 is assumed for Alt2 for handheld devices. - Alt2: antenna indices in each combination in the table are used for performance calibration purpose. - Remove Alt2 for combination 4. |
| Xiaomi | For combination 2, from our perspective, 4TX cannot be feasible for handheld UEs when the carrier frequency is 4GHz or below. We suggest to add another note for combination 2 to make it clearer, i.e., 4T4R for combination 2 is not for handheld UEs in carrier frequency around 4GHz or below. For combination 3, deploying 8Rx at 7 GHz is already quite challenging for handheld UE. From our viewpoint, achieving 8Rx for handheld terminals at 4 GHz and below is practically impossible. The note is needed for combination 3, i.e., Combination 3 is not for handheld UEs in carrier frequency around 4GHz or below For Alt 1 and 2, we think alt 1 to be more appropriate. In practice, the actual antenna design will most likely not strictly follow the placement positions specified in alt2, the evaluation based on alt2 has limited reference value. |
| Nokia | It will be good to clarify for the Alt.1 that the Isotropic radiation pattern is assumed, if it is common understanding of course. Regarding the CPEs, in general, and for Combination4 in particular, we can also consider CPE form factor and candidate antenna locations defined in TS 38.901. Can we add this in NOTE4? |
| OPPO | For combination 3, whether it is possible to apply it to handheld UE needs further study. We agreed with xiaomi’s note for combination 3. We also suggest to remove Alt2 from combination 4. |
| Samsung | We suggest to divide two different tables for handheld + low capable UE and advanced UE (such as CPE/FWA). For handheld, Alt 2 based antenna location is more realistic assumption. So, we suggest to support only Alt 2 for handheld and low-capable UE. In combination2 for handheld and low-capable UE, it is challenging to productize 4Tx based handheld devices. Therefore, for handheld and low-capable UE, we suggest to delete 4T4R. And for advanced UE, we can consider 1T and 2T also for evaluation. In addition, for design of new UL precoder considering various UE antenna structures, we suggest not to preclude other option for Alt2. Therefore, we should support below tables for handheld + low capable UE and advanced UE respectively. For handheld and low-capable UE: UE antenna modelling for RAN1 evaluations; Total number of antenna elements; Total number of TXRU; UT device antenna model using candidate antenna locations as described in section 7.3 in TR38.901; Applicable carrier frequency. Combination0 NOTE; 1; 1T1R,; - 1T - 1R; 700MHz, 2GHz. Combination1; 2; 1T2R,; 2R: [(1, 5), or (4, 8)] as described in section 7.3 in TR 38.901.; 700MHz, 2GHz, 4GHz. Combination2; 4; 1T4R, 2T4R, 4T4R; - 4R: [(2, 4, 6, 8), or (1, 3, 5, 7)] as described in section 7.3 in TR 38.901; 700MHz, 2GHz, 4GHz, 7GHz, 15GHz. NOTE: This combination is for IoT UE only. Other candidate antenna locations for each combination is not precluded.. For advanced UE: UE antenna modelling for RAN1 evaluations; Total number of antenna elements; Total number of TXRU; Alt 1: (M,N,P,Mg,Ng; Mp,Np), (dH,dV), (dg,H,dg,V) if any, or Alt 2: UT device antenna model using candidate antenna locations as described in section 7.3 in TR38.901; Applicable carrier frequency. Combination2; 4; 1T4R, 2T4R, 4T4R; Alt 1: - 4R: (M, N, P, Mg, Ng; Mp, Np)=(1, 2, 2, 1, 1; 1, 2) for dual polarization or (2, 2, 1, 1, 1; 2, 2) for single polarization, (dH,dV)= (0.5, 0.5)λ Alt 2: - 4R: [(2, 4, 6, 8), or (1, 3, 5, 7)] as described in section 7.3 in TR 38.901; 700MHz, 2GHz, 4GHz, 7GHz, 15GHz. Combination3; 8; 1T8R, 2T8R, 4T8R, 8T8R; Alt 1: (M, N, P, Mg, Ng; Mp, Np)= (1, 4, 2, 1, 1; 1, 4) for dual polarization or (2, 4, 1, 1, 1; 2, 4) for single polarization , (dH,dV)= (0.5, 0.5)λ Alt 2: (1, 2, 3, 4, 5, 6, 7, 8) as described in section 7.3 in TR38.901; 2GHz, 4GHz, 7GHz, 15GHz NOTE. Combination4; 16; 1T16R, 2T16R, 4T16R, 8T16R; Alt 1: - 16R: (M, N, P, Mg, Ng; Mp, Np)= (2, 4, 2, 1, 1; x, y) , (dH,dV)= (0.5, 0.5)λ Alt 2: - 16R: (1, 2, 3, 4, 5, 6, 7, 8) as described in section 7.3 in TR38.901, dual polarization; 7GHz, 15GHz. NOTE: If number of TXRU and frequency combination is applicable. Other candidate antenna locations for each combination is not precluded.. |
| Google | We support that 6GR evaluations adopt a baseline of 2Tx/4Rx for frequencies > 1 GHz and TDD bands, and 1Tx/2Rx for frequencies ≤ 1 GHz. For wearable devices, a 1Tx/1Rx (1T1R) configuration should be assumed. In the case of NTN bands, we support a 2Tx/4Rx configuration. Higher configurations, such as 4Tx, 6Rx, or 8Rx, should remain optional for evaluation. We emphasize a preference for a single-panel UE configuration as the baseline for FR2/FR3 evaluations. Additionally, we support that 16T16R (Combination 4) is intended only for CPE/FWA and is not a practical assumption for handheld UE Regarding implementation feasibility, we support Xiaomi and believe that 4Tx (4T4R) is not feasible for handheld UEs at carrier frequencies of 4 GHz or below. Also, 8Rx (Combination 3) should not be for handheld UEs in carrier frequencies around 4 GHz or below, as achieving this density is very challenging. |

(FL2) Proposal 2.1.2-1-rv1

For 6GR evaluation, RAN1 to model the UE antenna as follows for below 30GHz carrier frequency,

- Note: Each of other topics could further decide to use which combination(s) for the evaluations. Other combinations are not precluded for evaluations, e.g., 2T6R, 3T6R, 6T6R, 6T8R.

- Note: The antenna locations in Alt1 and Alt 2 in the following table are considered as examples and used for performance calibration. Any antenna array structures and/or antenna locations in section 7.3 in TR38.901 is possible for evaluations and up to companies to report.

- Note: The antenna locations in Alt 2 not included in section 7.3 in TR38.901 are up to companies to report.

- Note: The antenna element-wise power variation at the UE in TR 38.901 section 7.6.14.2 is can be optionally considered for Alt2 for handheld devices.

- Note: The radiation power pattern of a single antenna element in Table 7.3-2 TR38.901 is assumed for Alt2. The isotropic radiation power pattern is assumed for Alt1.

- Note: Whether only Alt2 is used for the evaluations could be further decided.

- Note: The antenna element/location of T is a subset of the element/locations for R.

- Note: The mapping between the combination and the device types might be separately discussed.

- Note: Both Combination2 and Combination3 do not imply that all the handheld UE are required to support all antenna configurations across all the listed frequency bands.

| UE antenna modelling for RAN1 evaluations | Total number of antenna elements | Total number of TXRU | Alt 1: (M,N,P,Mg,Ng; Mp,Np), (dH,dV), (dg,H,dg,V) if any, or Alt 2: UT device antenna model using candidate antenna locations as described in section 7.3 in TR38.901 | Applicable carrier frequency |
| --- | --- | --- | --- | --- |
| Combination0 NOTE1 | 1 | 1T1R, | Alt 1: 1T: (M, N, P, Mg, Ng; Mp, Np)=(1, 1, 1, 1, 1; 1, 1) 1R: (M, N, P, Mg, Ng; Mp, Np)=(1, 1, 1, 1, 1; 1, 1) Alt 2: - 1T - 1R | 700MHz, 2GHz |
| Combination1 | 2 | 1T2R, | Alt 1: - 2R: (M, N, P, Mg, Ng; Mp, Np)=(1, 2, 1, 1, 1; 1, 2) for single polarization or (1, 1, 2, 1, 1; 1, 1) for dual polarization, (dH,dV)= (0.5, 0.5)λ Alt 2: - 2R: [(1, 5), or (4, 8)] as described in section 7.3 in TR 38.901. | 700MHz, 2GHz, 4GHz |
| Combination2 | 4 | 1T4R, 2T4R, 4T4R | Alt 1: - 4R: (M, N, P, Mg, Ng; Mp, Np)=(1, 2, 2, 1, 1; 1, 2) for dual polarization or (2, 2, 1, 1, 1; 2, 2) for single polarization, (dH,dV)= (0.5, 0.5)λ Alt 2: - 4R: [(2, 4, 6, 8), or (1, 3, 5, 7)] as described in section 7.3 in TR 38.901 | 700MHz, 2GHz, 4GHz, 7GHz, 15GHz NOTE3 |
| Combination3 | 8 | 1T8R, 2T8R, 4T8R, 8T8R | Alt 1: (M, N, P, Mg, Ng; Mp, Np)= (1, 4, 2, 1, 1; 1, 4), or (2, 2, 2, 1, 1; 2, 2) for dual polarization or (2, 4, 1, 1, 1; 2, 4) for single polarization , (dH,dV)= (0.5, 0.5)λ Alt 2: (1, 2, 3, 4, 5, 6, 7, 8) as described in section 7.3 in TR38.901 | 2GHz, 4GHz, 7GHz, 15GHz NOTE3 |
| Combination4 NOTE2 | 16 | 4T16R 8T16R, | Alt 1: - 16R: (M, N, P, Mg, Ng; Mp, Np)= (2, 4, 2, 1, 1; x, y) , (dH,dV)= (0.5, 0.5)λ | 7GHz, 15GHz |
| NOTE1: This combination is for IoT UE only. NOTE2: This combination is for CPE UE only. NOTE3: If number of TXRU and frequency combination is applicable. |  |  |  |  |

Any comments/suggestions, please leave them here:

| Company | Comments |
| --- | --- |
| Qualcomm | We suggest removing the last note since it may imply that non-handheld devices are required to support all antenna configurations across all the listed frequency bands. But this is a UE capability discussion and should not be part of evaluation assumption. According to the main bullet, the proposed antenna combinations are only for evaluation purposes and not for implementation feasibility. Therefore, we think the last note is not needed. For combination 4, we suggest keeping Alt. 2. Otherwise it may be misunderstood that only Alt. 1 is supported for CPE. But this is not aligned with the FL proposal 2.1.2-3-rv1 where directional antenna (candidate 2 &3) can be assumed for the CPE. Regarding antenna placement for Alt. 2, if there are concerns on candidate antenna locations defined in 38.901 then probably companies could report the values. Therefore, we suggest the following update. Combination 4: Alt. 2: Company to report the antenna placement, directional pattern orientation of the CPE panel. |
| ZTE | We suggest deleting the note “Whether only Alt 2 is used for the evaluations could be further decided,” since Alt 1 can also be used for CPE. There is no need to emphasize that only Alt 2 can be used. |
| Ericsson | Regarding the following, our understanding is that CPEs may use directional elements in a panel that is set to point to a serving base station, so we prefer to limit to handheld UEs for Alt 1 for now: •Note: The radiation power pattern of a single antenna element in Table 7.3-2 TR38.901 is assumed for Alt2. The isotropic radiation power pattern is assumed for Alt1 at least for handheld devices. |

(FL1) Proposal 2.1.2-2

For 6GR evaluation, RAN1 to model the UE antenna as follows for around 30GHz carrier frequency,

- UE antenna configuration follows Table 1 below.

- UE antenna radiation pattern follows Table 2 below.

Table 1: UE antenna configuration for around 30GHz

| UE antenna configuration | Values |
| --- | --- |
| # of antenna elements per panel | 8 elements per panel (M, N, P) = (2, 2, 2) |
| # of panels | Config 1: 2 panels on front and back; Config 2: 4 panels on 4 edges. |
| # of TXRUs | 2T2R per panel - The antenna elements of the same polarization of the same panel is virtualized into one TXRU |

Table 2: UE antenna radiation pattern for around 30GHz [see Table A.2.1-8 TR38.802]

| Parameter | Values |
| --- | --- |
| Antenna element radiation pattern in θ'' dim (dB) |  |
| Antenna element radiation pattern in φ'' dim (dB) |  |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 5dBi |

Any comments/suggestions, please leave them here:

| Company | Comments |
| --- | --- |
| ZTE | We agree with this proposal. For FR2, to reflect the characteristics of analog beamforming at the UE side, a multi-panel configuration should be considered, where each panel can dynamically update its transmit/receive beams by using analog beam-forming. This not only aligns with practical product requirements, but is also essential for the evaluation of FR2 schemes, such as beam management and simultaneous transmission across multi-panel (STxMP). |
| Samsung | We suggest to support UE antenna modeling for around 30GHz: Combination5; 8; 2T2R; Alt 1: (M, N, P, Mg, Ng; Mp, Np) = (4, 1, 2, 1, 1; 1, 1) for handheld, (dH,dV)= (0.5, 0.5)λ; 30GHz. Combination6; 32; 2T2R; Alt 1: (M, N, P, Mg, Ng; Mp, Np) = (4, 4, 2, 1, 1; 1, 1) for non-handheld, (dH,dV)= (0.5, 0.5)λ; 30GHz. |
| NTT DOCOMO | Support. We are also open for discussion on other UE antenna configuration |

(FL2) Proposal 2.1.2-2-rv1

For 6GR evaluation, RAN1 to model the UE antenna as follows for around 30GHz carrier frequency,

- UE antenna configuration follows Table 1 below.

- UE antenna radiation pattern follows Table 2 below.

- Other antenna configuration can be considered and up to companies to report.

Table 1: UE antenna configuration for around 30GHz

| UE antenna configuration | Values |
| --- | --- |
| # of antenna elements per panel | 8 elements per panel (M, N, P) = (2, 2, 2) for Config 1 and Config 2. (M, N, P, Mg, Ng; Mp, Np) = (4, 1, 2, 1, 1; 1, 1) for Config 0. (M, N, P, Mg, Ng; Mp, Np) = (4, 4, 2, 1, 1; 1, 1) for CPE only for Config 0. |
| # of panels | Config 0: 1 panel. Config 1: 2 panels on front and back; Config 2: 4 panels on 4 edges. |
| # of TXRUs | 2T2R per panel - The antenna elements of the same polarization of the same panel is virtualized into one TXRU |

Table 2: UE antenna radiation pattern for around 30GHz [see Table A.2.1-8 TR38.802]

| Parameter | Values |
| --- | --- |
| Antenna element radiation pattern in θ'' dim (dB) |  |
| Antenna element radiation pattern in φ'' dim (dB) |  |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 5dBi |

Any comments/suggestions, please leave them here:

| Company | Comments |
| --- | --- |

(FL1) Proposal 2.1.2-3

For 6GR evaluation, RAN1 to model the radiation pattern of a single antenna element for FWA/CPE including,

- Candidate1: Isotropic,

- Candidate2: Directional with different half power beamwidth and maximum directional gains as described in Table 1 below,

  - CPE can be equipped with 1 to 3 antenna panels, each following Alt 1 ((M,N,P,Mg,Ng; Mp,Np), (dH,dV)) configuration, and the orientation of the device with a single antenna panel can be optimized.

- Candidate3: Omnidirectional as described in Table 2 below.

Table 1: Directional radiation power pattern of a single antenna element for CPE

| Parameter | Values |
| --- | --- |
| Vertical cut of the radiation power pattern (dB) | AdB″θ″, ϕ″=0°=-min12θ″-90°θ3dB2,SLAV with SLAV=22.5 dB and θ″∈0°, 180° |
| Horizontal cut of the radiation power pattern (dB) | AdB″θ″=90°, ϕ″=-min12ϕ″ϕ3dB2,Amax with , Amax=22.5 dB and ϕ″∈-180°, 180° |
| 3D radiation power pattern (dB) | AdB″θ″,ϕ″=-min-AdB″θ″, ϕ″=0°+AdB″θ″=90°, ϕ″,Amax |
| Maximum directional gain of an antenna element | GE,max dBi |
| θ3dB, ϕ3dB, GE_max=32°, 32°, 14, 65°, 50°, 9, 90°, 90°, 5 |  |

Table 2: Omnidirectional radiation power pattern of a single antenna element for CPE

| Parameter | Values |
| --- | --- |
| Vertical cut of the radiation power pattern (dB) | AV″θ″=-min12θ″-90°θ3dB2,SLAV with θ3dB=22.5°,SLAV=22.5dB and θ″∈0°, 180° |
| 3D radiation power pattern (dB) | AdB″θ″,ϕ″= AV″θ″ for any ϕ″∈-180°, 180° |
| Maximum directional gain of an antenna element, GE,max | 5 dBi |

Question from FL: which radiation pattern should be assumed for 30GHz CPE considering the above two proposals together??

Any comments/suggestions, please leave them here:

| Company | Comments |
| --- | --- |
| ZTE | - We prefer to use omnidirectional model for CPE, and during simulation, the orientation of CPE panel should be optimized to ensure a good coverage quality. - For Candidate2, it is recommended to be considered as an optional configuration, and the θ3dB, ϕ3dB can be reported by the companies. Moreover, the maximum directional gain can be determined by the 3D radiation power pattern (θ3dB, ϕ3dB), so we don’t need to specify GE,max. |
| NTT DOCOMO | Support in principle |

(FL2) Proposal 2.1.2-3-rv1

For 6GR evaluation, RAN1 to model the radiation pattern of a single antenna element for FWA/CPE including,

- Candidate1: Isotropic,

- Candidate2: Directional with different half power beamwidth and maximum directional gains as described in Table 1 below,

  - CPE can be equipped with 1 to 3 antenna panels, each following Alt 1 ((M,N,P,Mg,Ng; Mp,Np), (dH,dV)) configuration.

- Candidate3: Omnidirectional as described in Table 2 below.

- Note: The orientation of the CPE panel can be optimized.

Table 1: Directional radiation power pattern of a single antenna element for CPE

| Parameter | Values |
| --- | --- |
| Vertical cut of the radiation power pattern (dB) | AdB″θ″, ϕ″=0°=-min12θ″-90°θ3dB2,SLAV with SLAV=22.5 dB and θ″∈0°, 180° |
| Horizontal cut of the radiation power pattern (dB) | AdB″θ″=90°, ϕ″=-min12ϕ″ϕ3dB2,Amax with , Amax=22.5 dB and ϕ″∈-180°, 180° |
| 3D radiation power pattern (dB) | AdB″θ″,ϕ″=-min-AdB″θ″, ϕ″=0°+AdB″θ″=90°, ϕ″,Amax |
| Maximum directional gain of an antenna element | GE,max dBi |
| Company reports, e.g., θ3dB, ϕ3dB, GE_max=32°, 32°, 14, 65°, 50°, 9, 90°, 90°, 5 |  |

Table 2: Omnidirectional radiation power pattern of a single antenna element for CPE

| Parameter | Values |
| --- | --- |
| Vertical cut of the radiation power pattern (dB) | AV″θ″=-min12θ″-90°θ3dB2,SLAV with θ3dB=22.5°,SLAV=22.5dB and θ″∈0°, 180° |
| 3D radiation power pattern (dB) | AdB″θ″,ϕ″= AV″θ″ for any ϕ″∈-180°, 180° |
| Maximum directional gain of an antenna element, GE,max | 5 dBi |

Any comments/suggestions, please leave them here:

| Company | Comments |
| --- | --- |
| Ericsson | Support |

## (CLOSED)gNB antenna modeling

| Company | Views/proposals |
| --- | --- |
| Ericsson | Proposal 2 - Define a 4 TXRU outdoor BS antenna configuration for 4GHz as described above. |
| OPPO | Proposal1: For 7GHz, update the gNB antenna configuration as highlighted: BS antenna modelling; Total number of antenna elements; Total number of TXRU; (M, N, P, Mg, Ng; Mp, Np); (dH,dV). Indoor. Combination 1; 64; 32; (4, 8, 2, 1, 1; 2, 8); (0.5, 0.5)λ. Combination 2; 256; 64; (16, 8, 2, 1, 1; 4, 8); (0.5, 0.5)λ. Combination 3; 512; 128; (16, 16, 2, 1, 1; 8, 8); (0.5, 0.5)λ. Outdoor. Combination 1; 768; 128; (24, 16, 2, 1, 1; 4, 16); (0.5, 0.8)λ. Combination 2; 1024; 256; (32, 16, 2, 1, 1; 8, 16); (0.5, 0.58)λ. Combination 3; 1536; 256; (48, 16 ,2, 1, 1; 8, 16); (0.5, 0.58)λ. Combination 4; 2048; 256; (32, 32, 2, 1, 1, 8, 16); (0.5, 0.5)λ. Combination 5; 2048; 512; (64, 16, 2, 1, 1; 16, 16); (0.5, 0.5)λ. Note1: A single TXRU is mapped per panel per subarray per polarization as mandatory option. Companies can provide results optionally, assuming fully connected TXRU mapping within a panel per polarization.· Note2: Other combinations used in the simulation results are up to company to report.. |

### Discussions

Two proposals from Ericsson and OPPO, respectively, discuss gNB antenna modelling.

Reasoning from Ericsson for adding one more combination of 4 TXRU for 4GHz:

‘Classical’ gNB antenna configurations with 4 TXRUs have been defined for 2 GHz but such a configuration is missing for 4 GHz: the smallest number of TXRUs is 64 for outdoor base stations.  This implies that 2GHz gNB antenna arrays can be assumed to be AASs while 4GHz gNB antenna arrays cannot, which is incorrect. Since 4GHz will likely be a commonly used band in simulations, being able to compare performance of features with a classical array at gNB vs. an AAS will be more straightforward at 4 GHz vs. comparing the feature on 2GHz with a classical array to when the feature is used at 4 GHz with an AAS.  Therefore, a similar antenna configuration to the one defined for 2 GHz 4 TXRU should be defined as an option for 4 GHz.

Reasoning from OPPO for changing the vertical antenna spacing for outdoor combinations 2 and 3:

There are some issues on the antenna configuration for outdoor. For antenna spacing, smaller spacing is usually applied to larger antenna array to avoid over-huge antenna size. For 7GHz outdoor, 0.5λ is used for 32 and 64 elements in vertical dimension for combination 4 and 5, while 0.8λ is used for 32 and 48 elements for combination 2 and 3. The methodology to determine these configurations are illogical, e.g. 48 elements apply a larger spacing than 32 elements. Furthermore, based on the aperture size, the height of the antenna array in Combination 3 is 1.64m, which is even larger than the maximal aperture size assumed for near field channel modeling as below. Neither the aperture size NOR the array shape is practical in Combination 3. The evaluation based on this configuration may not be reasonable reference for realistic performance.

| BS antenna modelling | Total number of antenna elements | Total number of TXRU | (M, N, P, Mg, Ng; Mp, Np) | (dH,dV) | aperture size of antenna array |
| --- | --- | --- | --- | --- | --- |
| Indoor |  |  |  |  |  |
| Combination 1 | 64 | 32 | (4, 8, 2, 1, 1; 2, 8) | (0.5, 0.5)λ | (0.17m, 0.08m) |
| Combination 2 | 256 | 64 | (16, 8, 2, 1, 1; 4, 8) | (0.5, 0.5)λ | (0.17m, 0.34m) |
| Combination 3 | 512 | 128 | (16, 16, 2, 1, 1; 8, 8) | (0.5, 0.5)λ | (0.34m, 0.34m) |
| Outdoor |  |  |  |  |  |
| Combination 1 | 768 | 128 | (24, 16, 2, 1, 1; 4, 16) | (0.5, 0.8)λ | (0.34m, 0.82m) |
| Combination 2 | 1024 | 256 | (32, 16, 2, 1, 1; 8, 16) | (0.5, 0.8)λ | (0.34m, 1.1m) |
| Combination 3 | 1536 | 256 | (48, 16 ,2, 1, 1; 8, 16) | (0.5, 0.8)λ | (0.34m, 1.64m) |
| Combination 4 | 2048 | 256 | (32, 32, 2, 1, 1, 8, 16) | (0.5, 0.5)λ | (0.68m, 0.68m) |
| Combination 5 | 2048 | 512 | (64, 16, 2, 1, 1; 16, 16) | (0.5, 0.5)λ | (0.34m, 1.37m) |
| Note1: A single TXRU is mapped per panel per subarray per polarization as mandatory option. Companies can provide results optionally, assuming fully connected TXRU mapping within a panel per polarization.· Note2: Other combinations used in the simulation results are up to company to report. |  |  |  |  |  |

(FL2) Question

Whether the following two proposals are needed?? Please provide your views into the table.

Proposal a:

Define a 4 TXRU outdoor BS antenna configuration for about 4GHz carrier frequency as below.

| Outdoor |  |  |  |  |
| --- | --- | --- | --- | --- |
| BS antenna modelling | Total number of antenna elements | Total number of TXRU | (M, N, P, Mg , Ng; Mp, Np) | (dH,dV) |
| Combination [0] (Optional) | 32 | 4 | (8, 2, 2, 1, 1; 1, 2) | (0.5, 0.8)λ |

Any comments/suggestions, please leave them here:

| Company | Comments |
| --- | --- |
| FL | This proposal was discussed offline. Based on Ericsson’s explanation for the intension. No other different views were heard offline. Please continue reviewing and sharing your views. |
| Ericsson | To reiterate the online discussion here: not only AASs will be used at midbands, including 4 GHz, so it is inconsistent to define 4T4R for 2 GHz but not for 4 GHz. It is important to design 6G to support these ‘classical’ radio systems as well as AASs. Aspects such as antenna calibration for the downlink and degrees of freedom for uplink MIMO, etc, need to be addressed and these will be incomplete without addressing ‘classical’ radios. |

Proposal b:

For around 7GHz carrier frequency, update the agreed gNB antenna configuration as highlighted in yellow:

| BS antenna modelling | Total number of antenna elements | Total number of TXRU | (M, N, P, Mg, Ng; Mp, Np) | (dH,dV) |
| --- | --- | --- | --- | --- |
| Indoor |  |  |  |  |
| Combination 1 | 64 | 32 | (4, 8, 2, 1, 1; 2, 8) | (0.5, 0.5)λ |
| Combination 2 | 256 | 64 | (16, 8, 2, 1, 1; 4, 8) | (0.5, 0.5)λ |
| Combination 3 | 512 | 128 | (16, 16, 2, 1, 1; 8, 8) | (0.5, 0.5)λ |
| Outdoor |  |  |  |  |
| Combination 1 | 768 | 128 | (24, 16, 2, 1, 1; 4, 16) | (0.5, 0.8)λ |
| Combination 2 | 1024 | 256 | (32, 16, 2, 1, 1; 8, 16) | (0.5, 0.58)λ |
| Combination 3 | 1536 | 256 | (48, 16 ,2, 1, 1; 8, 16) | (0.5, 0.58)λ |
| Combination 4 | 2048 | 256 | (32, 32, 2, 1, 1, 8, 16) | (0.5, 0.5)λ |
| Combination 5 | 2048 | 512 | (64, 16, 2, 1, 1; 16, 16) | (0.5, 0.5)λ |
| Note1: A single TXRU is mapped per panel per subarray per polarization as mandatory option. Companies can provide results optionally, assuming fully connected TXRU mapping within a panel per polarization. Note2: Other combinations used in the simulation results are up to company to report. |  |  |  |  |

Please share your views here about the two proposals:

| Company |  | Comments |  |
| --- | --- | --- | --- |
| Qualcomm |  | Currently there are already 3 antenna combinations for outdoor and 2 combinations for indoor deployment. Adding another 4TxRU combination for outdoor is not necessary. 4GHz is a TDD band and using small number of TXRUs (i.e. 4 TxRUs) is not typical. For proposal b, we support to update dv to 0.5λ for outdoor combination 3. But for combination 2, the aperture size is still within the valid range even with dv equal to 0.8λ. Therefore, we don’t see a strong need to update combination 2. |  |
| ZTE | For Proposal b, we think an antenna array of about 1 m height is practical, and a larger aperture helps to achieve higher beamforming gain. So we suggest keeping Combination 2 as is, and modifying the dV of Combination 3/4 as follows. Combination 2; 1024; 256; (32, 16, 2, 1, 1; 8, 16); (0.5, 0.8)λ; (0.34m, 1.1m). Combination 3; 1536; 256; (48, 16 ,2, 1, 1; 8, 16); (0.5, 0.8 0.5)λ; (0.34m, 1 m). Combination 4; 2048; 256; (32, 32, 2, 1, 1, 8, 16); (0.5, 0.5 0.8)λ; (0.68m, 1.1m). |
| Xiaomi |  | For proposal b: Reducing the antenna spacing in the vertical dimension will degrade the angular resolution of the array in the vertical domain, weakening the interference suppression capability among multiple UEs for MU-MIMO, especially when UEs are widely distributed in the vertical dimension, such as indoor UEs in UMa scenarios. Therefore, we prefer to keep the original values. |  |
| OPPO |  | We are fine to only modify the antenna spacing of combination 3 in proposal b. |  |
| Samsung |  | For proposal b, we don’t support the change. We already agreed possible combinations for 7GHz. And if we consider some of current products which can have long form-factor in vertical domain, we don’t need to update the agreement |  |
| NTT DOCOMO |  | Open to discuss |  |
| FL |  | There are different views whether to make the update based on the comments received above. Companies can continue the discussion offline. If no consensus, this proposal will be dropped. |  |

# SLS assumptions for TN

## (CLOSED)BS/UE transmission power

### Companies’ views

| Company | Views/proposals |
| --- | --- |
| Nokia | - Proposal16: For CPE modelling, RAN1 supports power classes with maximum transmit power of 35 dBm, and peak EIRP not exceeding 43 dBm. - Proposal17: RAN1 to adjust the second note on BS Tx power and leave only the part related to Option 1, i.e., The values defined in option1 refer to the Report ITU-R M. [IMT-2030. EVAL]. The values defined in option2 is calculated based on the proportional scaling with simulation bandwidth under the limitation of the maximum BS Tx power of 56dBm. System BW; Ratio w.r.t. 20 MHz; Power Scaling (in dB); BW upscaled Tx power. 60 MHz; 60/20 = 3; 10log10(3) = 4.8 dB; 49 dBm -> 53.8 dBm 46 dBm -> 50.8 dBm 44 dBm -> 48.8 dBm 43 dBm -> 47.8 dBm. 100 MHz; 100/20 = 5; 10log10(3) = 7.0 dB; 49 dBm -> 56 dBm 46 dBm -> 53 dBm 44 dBm -> 51 dBm 43 dBm -> 50 dBm. 200 MHz; 200/20 = 10; 10log10(10) = 10.0 dB; 49 dBm -> 56 dBm 46 dBm -> 56 dBm 44 dBm -> 54 dBm 43 dBm -> 50 dBm. 300 MHz; 300/20 = 15; 10log10(15) = 11.8 dB; 49 dBm -> 56 dBm 46 dBm -> 56 dBm 44 dBm -> 55.8 dBm 43 dBm -> 54.8 dBm. 400 MHz; 400/20 = 20; 10log10(20) = 13.0 dB; 49 dBm -> 56 dBm 46 dBm -> 56 dBm 44 dBm -> 56 dBm 43 dBm -> 56 dBm. - Proposal18: RAN1 either to remove BS Tx power options reflecting only to BW scaling or at least to indicate Option 1 as default. |
| DOCOMO | In the agreement, multiple values for each scenario/carrier frequency were specified. However, to ensure appropriate values are applied in various 6GR system-level simulations, the values of UE power class for each scenario/carrier frequency should be simplified. Therefore, no new values such as the values in FFS of the agreement should be introduced beyond the current table. Proposal 5 Regarding the parameter of UE power class, no new values should be introduced beyond the current table. |
| Ofinno | Observation 10: Higher output power such as 29 dBm is achieved means of transmit diversity or MIMO. Observation 11: Supporting UE power class of 29 dBm with 1 Tx will be an unrealistic implementation for a UE. Observation 12: The maximum EIRP of 55 dBm is relevant for FWA deployment. Proposal 12: Do not consider UE power class of 29 dBm for 1 Tx in 6GR evaluations. Proposal 13: Consider a power class comprising minimum EIRP=35 dBm and maximum EIRP=55 dBm for 6GR evaluations around 30 GHz. |
| Samsung | Proposal #7: The 29 dBm power class should be defined as optional, considering that it typically requires TxD (2Tx or more) which is not a baseline feature for all handheld devices. Proposal #8: For FR2, additionally consider 43 dBm as minimal peak EIRP value and 55 dBm as maximal EIRP for FWA/CPE. It is up to company to report the simulated power but both of the minimal peak EIRP and maximal EIRP limit should be satisfied. |
| Xiaomi | Proposal 4: Transmit power of 29dBm for 1Tx can be studied for high-end handheld UEs. |

### Discussions

Background

The last meeting agreed on assumptions for gNB transmission power and the following for the UE transmission power:

Agreement For 6GR evaluation, the UE power class for system-level simulation is assumed as follows: - FFS: 29dBm for 1 Tx. - FFS: 31dBm, or 35 dBm or 43dBm with EIRP <55 dBm UE power class; Indoor Hotspot; Dense Urban; Rural; Urban Macro; Sub-urban macro. Around 700MHz; NA; 23dBm; 23dBm; 23dBm; 23dBm. Around 2GHz; 23dBm, 26dBm, 29dBm; 23dBm, 26dBm, 29dBm; 23dBm, 26dBm, 29dBm; 23dBm, 26dBm, 29dBm; 23dBm, 26dBm, 29dBm. Around 4GHz; 23dBm, 26dBm, 29dBm; 23dBm, 26dBm, 29dBm; 23dBm, 26dBm, 29dBm; 23dBm, 26dBm, 29dBm; 23dBm, 26dBm, 29dBm. Around 7GHz; 23dBm, 26dBm, 29dBm; 23dBm, 26dBm, 29dBm; 23dBm, 26dBm, 29dBm; 23dBm, 26dBm, 29dBm; 23dBm, 26dBm, 29dBm. Around 15GHz; 23dB, 26dBm, 29dBm; 23dB, 26dBm, 29dBm; NA; 23dB, 26dBm, 29dBm; 23dBm, 26dBm,29dBm. Around 30GHz; Minimum peak EIRP: 23dB, [26dBm, 29dBm] Note: EIRP should not exceed 43 dBm; Minimum peak EIRP: 23dB, [26dBm, 29dBm] Note: EIRP should not exceed 43 dBm; NA; Minimum peak EIRP: 23dB, [26dBm, 29dBm] Note: EIRP should not exceed 43 dBm; Minimum peak EIRP: 23dBm, [26dBm, 29dBm] EIRP should not exceed 43 dBm.

Observations from contributions:

  - Controversy on 29dBm for UE transmission power

    - Not for 1Tx: Ofinno

    - can be studied for 1Tx: Xiaomi

    - optional for handheld: Samsung

  - Update the values for 30GHz for UE transmission power

    - 35dBm<=peak EIRP<=55dBm: Ofinno

  - Clarify the values for FWA/CPE

    - Max transmission power 35dBm and peak EIRP <=43dBm: Nokia

    - Maximum EIRP of 55 dBm: Ofinno

    - 43dBm<=peak EIRP<=55dBm for FWA/CPE: Samsung

    - It is up to company to report the simulated power but both of the minimal peak EIRP and maximal EIRP limit should be satisfied: Samsung

  - Nokia mentioned some discrepancies (Option2 for 700MHz, Option 2 for 4GHz) of the agreement regarding gNB transmission power assumptions.

    - In the existing agreement on BS Tx power, the second note regarding the values under Option 2 may be confusing because this option does not always correspond to proportional scaling with simulation bandwidth from Option 1 (e.g., in Dense urban deployment scenario). There is no need to list all possible scaled values because they can be derived from Option 1 assuming maximum Tx power of 56 dBm.

    - Suggested update to the agreement: RAN1 to adjust the second note on BS Tx power and leave only the part related to Option 1, i.e., The values defined in option1 refer to the Report ITU-R M. [IMT-2030. EVAL]. The values defined in option2 is calculated based on the proportional scaling with simulation bandwidth under the limitation of the maximum BS Tx power of 56dBm.

Discussion plan for this meeting:

Companies can discuss whether it is needed to clarify UE transmission power of 29dBm is not applied to 1Tx. This remained an FFS from the last meeting. It is better to address this issue despite the answer.

The UE transmission power for 30GHz could be revisited or updated after further discussions. At least, the proposed values by companies for this meeting are quite different from the values agreed.

Need to further discuss whether to clarify the transmission power values for FWA/CPE specifically depending on whether the values will be different from that for non-CPE UEs.

Regarding gNB transmission power, the suggestion from Nokia for updating the second note of the agreement makes sense.

Round-1 discussions:

(FL1) Question:

Whether it is needed to clarify UE transmission power of 29dBm is not applied to 1Tx?

Any comments/suggestions, please leave them here:

| Company | Comments |
| --- | --- |
| Qualcomm | We don’t think the need to clarify 29dB Tx power not applied to 1Tx. It is in the RAN4 scope. Currently RAN4 has ongoing R20 WI to define whether 1Tx 29dBm for TDD band with one PA is supported for FWA. RAN1 can just follow the RAN4 decision and there is no need to add additional restriction for 29dBm |
| vivo | We propose not to clarify whether 29dBm is applied to 1Tx, and this can be left to future discussion in RAN4. |
| Nokia | Sorry for putting the comment here but seems there is no corresponding Issue created: We also need to consider hier transmit powers for CPEs, i.e., 35dBm max transmit power and peak EIRP not exceeding 43 dBm. |
| Samsung | 1Tx based 29dBm is not feasible. We can consider 29 dBm only based on TxD (more than 2Tx). In our view, 1Tx based evaluation could be baseline and more than 1Tx should be optional. Therefore, 29dBm with >1Tx could be optional feature for evaluation purpose. |
| DOCOMO | We think there is no need to clarify UE transmission power of 29dBm is not applied to 1Tx. |

(FL1) Proposal 3.1.2-1

Regarding the agreement on UE transmission power assumptions in system-level simulation, the further update is highlighted as follows:

| UE power class | Indoor Hotspot | Dense Urban | Rural | Urban Macro | Sub-urban macro |
| --- | --- | --- | --- | --- | --- |
| Around 30GHz | Minimum peak EIRP: 23dB, [26dBm, 29dBm] Note: EIRP should not exceed 43 dBm 35dBm<=peak EIRP<=55dBm | Minimum peak EIRP: 23dB, [26dBm, 29dBm] Note: EIRP should not exceed 43 dBm 35dBm<=peak EIRP<=55dBm | NA | Minimum peak EIRP: 23dB, [26dBm, 29dBm] Note: EIRP should not exceed 43 dBm 35dBm<=peak EIRP<=55dBm | Minimum peak EIRP: 23dBm, [26dBm, 29dBm] EIRP should not exceed 43 dBm 35dBm<=peak EIRP<=55dBm |

- Note: It is up to company to report the simulated transmission power confined within the defined peak EIRP range below.

Any comments/suggestions, please leave them here:

| Company | Comments |
| --- | --- |
| Qualcomm | Following the last meeting agreement, the minimum of peak EIRP is 23dBm and the maximum is 43dBm. We don’t understand why the minimum and maximum values are changed. For the note, we are okay. |
| Samsung | We are fine with added minimum peak EIRP requirement and maximum peak EIRP limitation for CPE/FWA but we should not delete agreed one (minimum peak EIRP: 23dBm, maximum peak EIRP: 43 dBm). The agreed EIRP requirement and limitation should be captured for handheld device. |
| DOCOMO | We think it is reasonable for the UE Tx power to be determined based on the max/min peak EIRP. For the specific values of max/min peak EIRP, clear justification is needed. |

(FL2) Proposal 3.1.2-1-rv1

Regarding the agreement on UE transmission power assumptions in system-level simulation, the further update is highlighted as follows:

- Transmission power of 35dBm is assumed for CPE only for below 30GHz.

| UE power class | Indoor Hotspot | Dense Urban | Rural | Urban Macro | Sub-urban macro |
| --- | --- | --- | --- | --- | --- |
| Around 30GHz | Minimum peak EIRP: 23dBm, [26dBm, 29dBm] Note: EIRP should not exceed 43dBm For CPE only (NOTE1): 35dBm<=peak EIRP<=55dBm | Minimum peak EIRP: 23dBm, [26dBm, 29dBm] Note: EIRP should not exceed 43dBm For CPE only (NOTE1): 35dBm<=peak EIRP<=55dBm | NA | Minimum peak EIRP: 23dBm, [26dBm, 29dBm] Note: EIRP should not exceed 43dBm For CPE only (NOTE1): 35dBm<=peak EIRP<=55dBm | Minimum peak EIRP: 23dBm, [26dBm, 29dBm] EIRP should not exceed 43dBm For CPE only (NOTE1): 35dBm<=peak EIRP<=55dBm |
|  | - NOTE1: It is up to company to report the simulated transmission power confined within the defined peak EIRP range. |  |  |  |  |

Any comments/suggestions, please leave them here:

| Company | Comments |
| --- | --- |

(FL2) Proposal 3.1.2-2

- Regarding the gNB transmission power assumptions in the evaluations, update the second note of the agreement as follows:

Note: For evaluation purpose, BS Tx power scales up with bandwidth proportionally under the limitation of the maximum BS Tx power is 56dBm for outdoor and 33dBm for indoor for the above carrier frequencies. - Note: The values defined in option1 refer to the Report ITU-R M. [IMT-2030. EVAL]. The values defined in option2 is calculated based on the proportional scaling with simulation bandwidth under the limitation of the maximum BS Tx power of 56dBm.

Any comments/suggestions, please leave them here:

| Company | Comments |
| --- | --- |
| Qualcomm | OK |
| Nokia | Shall we keep all alternatives to Option 1? What is the understanding of the companies, why these additional values are needed? |
| DOCOMO | Support. |

## (CLOSED)UE height/distribution/noise figure

### Companies’ views

| Company | Views/proposals |
| --- | --- |
| Ericsson | Proposal 1 - For assumptions agreed in RAN1#123 on UE speed/UE distribution, clarify that outdoor UEs are assumed to be ‘in cars’ for DU, Rural and Uma scenarios. |
| Nokia | - Proposal19: Main 6G deployment scenarios should include FWA specific assumptions, such as outdoor CPE antenna heights 1m above rooftop, specific CPE distributions (indoor, outdoor and mixed) and enable prequalification techniques. - Proposal20: RAN1 to consider 30 and up to 50 UEs per TRxP to evaluate the gains of some 6GR features such as MU-MIMO. - Proposal21: RAN1 to consider the UE noise figure equal to 7 dB around 7 GHz and below for 6GR evaluations that is aligned with the ITU IMR-2030 requirements. |
| Samsung | - Proposal #6: The number of UEs per TRxP should be defined separately for macro and micro layers to account for the disparity in their coverage areas. We propose to update the simulation assumptions as follows: Parameters; Indoor Hotspot; Dense Urban; Rural; Urban Macro; Suburban Macro. UE distribution and UE speed; 10 users per TRxP. 100% Indoor, 3km/h; Single layer: Uniform/macro TRxP Two layers: Uniform/macro TRxP + Clustered/micro TRxP UE number per macro TRxP is 30, 50. UE number per micro TRxP is 10. Opt1: 80% indoor (3km/h); 20% outdoor(30km/h). Opt2: 40% indoor (3km/h) 40% outdoor (3km/h) 20% outdoor (30km/h); Single layer: Uniform/macro TRxP UE number per TRxP is [10, 30, 50]. Opt1: 50% indoor (3km/h); 50% outdoor(120km/h). Opt2: 20% indoor (3km/h) 40% outdoor (60km/h) 40% outdoor (120km/h); Single layer: Uniform/macro TRxP Two layers: Uniform/macro TRxP + Clustered/micro TRxP UE number per macro TRxP is 30, 50. UE number per micro TRxP is 10. Opt1: 80% indoor (3km/h); 20% outdoor(30km/h). Opt2: 40% indoor (3km/h) 40% outdoor (3km/h) 20% outdoor (30km/h); Single layer: Uniform/macro TRxP UE number per TRxP is [10, 30, 50]. Opt1: 10% Outdoor pedestrian: 3km/h; 10% Outdoor in cars: 40km/h; 80% Indoor in houses: 3km/h. Opt2: 20% outdoor in cars: 40km/h 80% indoor in houses: 3km/h. FFS: Applicability for FWA. |
| DOCOMO | The agreement above has one FFS regarding applicability for FWA. In our understandings, following directions can be considered for the applicability for FWA. - Alt. 1: Rural with Option 2 can be used as a scenario which includes both handheld UE and FWA. - Alt. 2: New option should be included at least for Rural to consider FWA, e.g., - 5% indoor (0km/h), 15% indoor (3km/h), 40% outdoor (60km/h), 40% outdoor (120km/h) - 20% indoor (3km/h), 20% outdoor (0km), 30% outdoor (60km/h), 30% outdoor (120km/h) The difference between handheld UE (3 km) and FWA (0 km) is small, and the number of FWA may be very small compared to the number of other UEs. On the other hand, the impacts/necessities of FWA on UE distribution/speed can be further discussed. Observation 4 - Regarding the applicability of FWA for UE distribution and UE speed, the necessity of considering FWA and its impact on SLS can be further discussed. Proposal 4 Regarding the parameters of layout, following parameters and their values should be specified: Table 1: Minimum distance between UE/Macro TRP/Micro TRP. Parameter; Value. Minimum distance between Micro TRPs; 40 m (Number of the micro TRPs per macro TRP = 3), 32 m (Number of the micro TRPs per macro TRP = 6), 25 m (Number of the micro TRPs per macro TRP = 9) [specific value(s) can be further discussed]. Minimum distance between Micro TRP and Macro TRP; 70 m [specific value(s) can be further discussed]. Minimum distance between Macro TRP and UE; 10 m. Minimum distance between Micro TRP and UE; 10 m. |
| Xiaomi | Proposal 5: For a carrier frequency around 7GHz and below, consider a typical noise figure of 9dB for UE receivers. |
| T-Mobile USA, Ericsson, MediaTek, Nokia, Verizon | - Proposal3: RAN1 shall consider additional options for UE distribution and UE speed (e.g., 0 km/h or ~0.3km/h) inclusive of FWA deployments (indoor/outdoor CPE) for system level simulation assumptions. |

### Discussions

Issues discussed in the contributions

- UE distribution assumptions

  - Ericsson suggested to add ‘in car’ for the options with higher velocity.

  - Number of UEs per TRxP

    - Nokia suggested specific numbers for MU-MIMO

    - Samsung suggested the number of UEs per TRxP should be defined separately for macro and micro layers.

  - FWA/CPE assumptions for antenna height and UE speed should be clarified: Nokia, DOCOMO, T-Mobile USA, Ericsson, MediaTek, Nokia, Verizon

- UE noise figure for <=7GHz carrier frequency

  - Support 7dB: Nokia (aligned with IMT-2030)

  - Support 9dB: Xiaomi

- Layout for distance between UE/Macro TRP/Micro TRP: DOCOMO

Discussion plan for this meeting:

  - The most critical issue might be the assumptions for FWA/CPE, which remained FFS from the last meeting.

  - Ericsson suggested ‘in car’ can be updated into the table along with the changes made for CPE.

  - Number of UEs per TRxP can be further clarified in the evaluations when applied.

  - UE noise figure can try 7dB as Nokia suggested to align with IMT-2030.

  - DOCOMO’s suggestion on layout for distance between UE/Macro TRP/Micro TRP can be further clarified in the evaluations when applied.

Round-1 discussions:

(FL2) Proposal 3.2.2-1-rv1

The agreed table for UE distribution and UE speed for system-level simulation, the further update is highlighted in cyan as follows:

| Parameters | Indoor Hotspot | Dense Urban | Rural | Urban Macro | Suburban Macro |
| --- | --- | --- | --- | --- | --- |
| UE distribution and UE speed | 10 users per TRxP. Opt1: 100% Indoor, 3km/h | Single layer: Uniform/macro TRxP Two layers: Uniform/macro TRxP + Clustered/micro TRxP UE number per TRxP is [10, 30, 50]. Opt1: 80% indoor (3km/h); 20% outdoor in cars (30km/h). Opt2: 40% indoor (3km/h) 40% outdoor (3km/h) 20% outdoor in cars (30km/h). | Single layer: Uniform/macro TRxP UE number per TRxP is [10, 30, 50]. Opt1: 50% indoor (3km/h); 50% outdoor in cars (120km/h). Opt2: 20% indoor (3km/h) 40% outdoor (60km/h) in cars 40% outdoor in cars (120km/h). | Single layer: Uniform/macro TRxP Two layers: Uniform/macro TRxP + Clustered/micro TRxP UE number per TRxP is [10, 30, 50]. Opt1: 80% indoor (3km/h); 20% outdoor in cars (30km/h). Opt2: 40% indoor (3km/h) 40% outdoor (3km/h) 20% outdoor in cars (30km/h). | Single layer: Uniform/macro TRxP UE number per TRxP is [10, 30, 50]. Opt1: 10% Outdoor pedestrian: 3km/h; 10% Outdoor in cars: 40km/h; 80% Indoor in houses: 3km/h. Opt2: 20% outdoor in cars: 40km/h 80% indoor in houses: 3km/h |
| FFS: Applicability for FWA Note: Regarding the number of UEs per TRxP, a smaller or the same number of UEs is assumed for each micro TRxPs compared to each macro TRxPs. |  |  |  |  |  |

Any comments/suggestions, please leave them here:

| Company | Comments |
| --- | --- |
| Qualcomm | We support the update. |
| ZTE | We don’t see a strong need to explicitly state that the UE is in a car. For example, an electric bicycle can achieve a similar speed. If penetration loss needs to be considered, it would be more appropriate to handle it in the channel model. |
| Nokia | OK with the clarification |
| OPPO | We share similar view as ZTE. |
| Samsung | We prefer to clarify different # of UE per TRxP for each macro TRxP and micro TRxP. But considering FL’s comment, we are fine with adding following note: “note: Regarding the number of UEs per TRxP, a smaller number of UEs is assumed for micro TRxPs compared to macro TRxPs” |
| DOCOMO | Support. |

(FL1) Proposal 3.2.2-2

RAN1 to assume the UE antenna height and UE distribution for CPE only for 6GR evaluations as follows:

| Parameters | Rural | Urban Macro | Suburban Macro |
| --- | --- | --- | --- |
| Antenna height for CPE only | Indoor CPEs: follow the heights in 38.901 for RMa. Outdoor CPEs: 1m above rooftop. Building heights modeled as 3m or 6m, equally likely. | Indoor CPEs: follow the heights in 38.901 for UMa. Outdoor CPEs: 1m above building height in 38.901 for UMa | Indoor CPEs: follow the heights in 38.901 for SMa. Outdoor CPEs: 1m above building height in 38.901 for SMa |
| UE distribution and UE speed for CPE only | Profile 1 (mixed deployment): 80% Indoor CPE: 3 km/h; 20% Outdoor rooftop mounted CPE: 0~0.3km/h. Profile 2 (Indoor CPE only): 100% Indoor: 0~0.3km/h. Profile 3 (Outdoor mounted CPE only): Rooftop mounted; 100% Outdoor: 0~0.3 km/h. |  |  |

Any comments/suggestions, please leave them here:

| Company | Comments |
| --- | --- |
| ZTE | We should be cautious about setting the CPE speed to 0, as this would result in a static channel under the current channel model. To better reflect the semi-static characteristics of the wireless channel, a non-zero speed should be supported. For the CPE distribution, we suggest adding a note to ensure adequate coverage quality for the CPE. Note: The CPE location/orientation should be optimized to guarantee a sufficiently good coverage quality. |
| Nokia | We also need to add a note,e.g., as in TR 38.802: UE orientation for customer premise equipment (CPE) can be optimized. |
| MTK | For UE distribution and UE speed for CPE only - For Profile 1 (mixed deployment): - Indoor CPE speed can also be 0 ~ 0.3 km/h to model slow environmental variations - Mixes of indoor and outdoor CPEs should not be chosen randomly; outdoor CPEs should be installed depends on some criteria. Pre-selection criterion and mechanism are needed for the mixed deployment based on the network planning. We suggest adding a FFS for indoor and outdoor CPE pre-select criterion or mechanism. - For Profile 3 (Outdoor mounted CPE only): - Non rooftop mounted outdoor CPE should not be precluded. 100% Outdoor CPE placed near an outside wall at the same height as for indoor eMBB UEs should not be precluded. |
| DOCOMO | Generally fine. For Profile 3 in UE distribution and UE speed for CPE only, we suggest the following update: Profile 3 (Outdoor mounted CPE only): Rooftop mounted; 100% Outdoor: 0~0.3 km/h.. |
| T-Mobile | This proposal assumes 100% of the UEs are for FWA, which does not reflect the real deployment scenario. It is not always possible to have a dedicated carrier bandwidth for CPEs. In some cases, a serving cell might have both CPEs and mobile devices. We propose to add some percentage of other types of UEs to the UE distributions for performance evaluations. |

(FL2) Proposal 3.2.2-2-rv1

RAN1 to assume the UE antenna height and UE distribution for CPE for 6GR evaluations as follows:

- Note: Indoor and outdoor CPE pre-selection criterion or mechanism could be further discussed in the evaluation phase.

| Parameters | Rural | Urban Macro | Suburban Macro |
| --- | --- | --- | --- |
| Antenna height for CPE only | Indoor CPEs: follow the heights in 38.901 for RMa. Outdoor CPEs: 1m above rooftop. Building heights modeled as 3m or 6m, equally likely. | Indoor CPEs: follow the heights in 38.901 for UMa. Outdoor CPEs: 1m above building height in 38.901 for UMa | Indoor CPEs: follow the heights in 38.901 for SMa. Outdoor CPEs: 1m above building height in 38.901 for SMa |
| UE distribution and UE speed for CPE only | Profile 1 (mixed deployment): 80% Indoor CPE: (0, 0.3] km/h; 20% Outdoor rooftop mounted CPE: (0, 0.3]km/h. Profile 2 (Indoor CPE only): 100% Indoor: (0, 0.3]km/h. Profile 3 (Outdoor mounted CPE only): Rooftop mounted; 100% Outdoor: (0, 0.3] km/h. |  |  |

Any comments/suggestions, please leave them here:

| Company | Comments |
| --- | --- |
| ZTE | We suggest replacing (0~0.3] with (0, 0.3]. |

(FL1) Proposal 3.2.2-3

RAN1 to assume the UE noise figure equal to 7 dB around 7 GHz carrier frequency and below for 6GR evaluations.

- Note: It is aligned with the ITU IMT-2030 requirements.

Any comments/suggestions, please leave them here:

| Company | Comments |
| --- | --- |
| Qualcomm | We don’t support the proposal. 9dB noise figure has been widely used before in NR FR1 evaluation and should be the baseline also for 6G evaluation. There is no need to align with the ITU IMT-2030 evaluation. Secondly, we cannot expect smaller NF values for higher frequency. It is contracted with practical implementation. If we assume 9dB for FR1 below 6GHz then we cannot assume 7dB for around 7GHz. If we cannot agree a common value then it can be company to report the exact value for evalulation |
| vivo | Fine with this proposal. |
| Xiaomi | Not support. Selecting a smaller value of UE noise figure can help to achieve the requirement of performance, e.g. coverage, SE, etc., however, the UE cost and design difficulty will be increased. Therefore, although a noise figure of 7dB is recommended by ITU, noise figure of 9dB shall still be used in RAN1 evaluation |
| DOCOMO | A single value (7dB or 9dB) should be adopted to ensure appropriate values are applied in various 6GR system-level simulations. There is no strong view on whether the value should be 7dB or 9dB. |
| FL | This proposal was debated offline and no consensus to do down-selection in this agenda. This proposal is dropped. |

## (CLOSED)Other Scenarios

### Companies’ views

| Company | Views/proposals |
| --- | --- |
| vivo | For Urban grid Scenario: Layout: The road configuration and building size are agreed and, with the agreed ISD and the 3D buildings modeling, BS can be located at the top of buildings at the sideways or at one corner. Refer to Table 7.9.6.1-3 and Table 7.9.6.3-2 of TR 38.901 [12], the example BS layout (with ISD 500m and ISD 250m) are shown in the following Figure 5-1. - Channel model TR 38.901 v19.1.0 UMa, considering the impact of building walls on the LOS probability refer to the option 1 in Clause 7.9.5.2 of TR38.901 [12]. |
| ZTE | Proposed evaluation assumptions for the following additional scenrios: - High speed train - Extreme long distance coverage in low density area - Air-to-Ground Scenario - Non-Terrestrial Network - Multi-layer heterogeneous network with assisting node |

### Discussions

RAN plenary has progressed substantially on all other scenarios and the related assumptions. RAN1 does not seem to need more discussions and it makes sense to presume those scenarios captured in TR38.914 but not discussed in RAN1 are also candidate scenarios for RAN1 evaluations for 6GR.

To avoid confusion, maybe it’s good to agree on the following conclusion.

(FL2) Conclusion

The scenarios captured in TR38.914 but are not discussed in RAN1 are also candidate scenarios for RAN1 evaluations for 6GR. The other detailed assumptions could be further discussed in the evaluation phase.

Any comments/suggestions, please leave them here:

| Company | Comments |
| --- | --- |
| vivo | There are some detailed parameters for urban grid scenario in TR 38.914 are yet to be finalized, which will affect future evaluation. We propose to further discuss the following assumptions in RAN1: - BS layout. - UE distribution and mobility. - Channel model. |
| DOCOMO | Fine. |

# Traffic models

## (CLOSED)New model 1- AI/ML services

### Companies’ views

| Company | Views/proposals |
| --- | --- |
| Ericsson | Proposal 6-3 - For the study on traffic models for generative AI services, RAN1 wait for further input from RAN2 and SA4 before continuing discussions due to RAN#110 decision. - For any intermediate evaluations while waiting for input from other groups, for video related applications, the video traffic modeling in XR TR 38.838 can be used, and for other applications the extensions being studied for FTP Model 1/FTP Model 3 extension can be used. |
| Huawei | Views are discussed in RAN2’s paper R2-2600418. |
| Intel | Proposal 7 - Reuse the XR traffic model as the baseline framework for AI/ML service evaluations, with minimal extensions to support - (i) optional burst-based packet arrivals for interactive AI sessions, - (ii) extended packet size ranges where needed, and - (iii) reuse of XR PDU set based packet importance handling, while continuing to use packet-level delay budget and success rate as PHY evaluation metrics and avoiding introduction of token-level KPIs. - Revise, if necessary, when SA4 updates are received |
| Ofinno | Observation 1: Based on SA4 response LS, SA4 cannot provide any feedback on the RAN1 options related to AI/ML traffic characteristics before progressing their (SA4) studies. Observation 2: Based on RAN plenary agreement, RAN2 will lead the RAN related aspects of the AI/ML traffic characteristics. Proposal 1: RAN1 evaluations on 6GR involving AI/ML related traffic are put on hold until received necessary input from RAN2 on the AI/ML traffic characteristics. |
| OPPO | Observation 1: When defining a traffic model for AI services, a general framework that takes into account the impacts brought by different traffic types and various AI services could be considered. Observation 2: RAN1 could align its study and timeline with the research progress and outcomes from RAN2 (as well as SA2 and SA4) regarding AI services and their characteristics before defining specific traffic models. |
| Qualcomm | Proposal 11: On studying AI/ML traffic model, no further RAN1 work is needed before RAN2 completes studies on AI traffic characteristics. |
| ZTE | Proposal 4-2-1: Regarding AI traffic model discussion, in RAN1#124, RAN1 related discussion should be postponed till having clearly consensus/agreements from RAN2. Proposal 4-2-2: When discussing packet size, it is necessary to clarify the input data type (e.g., text, image). Proposal 4-2-3: Whether to model the generative AI service as a burst of packets depends on the existence of packet importance. If packet importance is not considered, packets can be assumed to arrive independently. Proposal 4-2-4: The existence and modeling of packet importance for generative AI services should not be determined by RAN1. RAN1 should wait for the study results from other working groups (e.g., SA4 and RAN2). Proposal 4-2-5: Adopt the extended FTP model or XR model for generative AI service traffic modeling. Proposal 4-2-6: Study the traffic model(s) for AI/ML model training/inference in 6GR evaluation. - The traffic model consists of the following 3 service types: training data collection, inference result transmission, and model download. Each service type adopts FTP model 3 for simulation, which is characterized by packet size, arrival rate, latency requirement, and reliability requirement. - The parameter values for each service type are fixed and defined in Table 4-2-1. - FFS: Whether to select a single service type for simulation, or a combination of the 3 services. - FFS: If a combination of services is selected, whether each UE is assigned with mixed services. |

### Discussions

Background

The traffic model for AI/ML services (a representative AI/ML service is the generative AI) was discussed in previous RAN1 meetings and some preliminary agreements were reached. RAN plenary further clarified that RAN2 will lead the related traffic characteristics/model discussions. Therefore, the discussion on this model in RAN1 is put on hold.

(FL1) Conclusion

No discussion on this model for this meeting.

Any comments/suggestions, please leave them here:

| Company | Comments |
| --- | --- |
| Samsung | Ok with conclusion |
| NTT DOCOMO | Support |
| Google | OK. Yes we agree with the conclusion |

## New model 2-Immersive comm.

### Companies’ views

| Company | Views/proposals |
| --- | --- |
| CATT | Proposal 3: For 6G immersive communication services, RAN1 should wait for inputs from SA4 before going into details on the potential models. |
| China Telecom | Proposal 1: 6G shall support the requirement of jitter management to ensure deterministic latency for immersive communication. Observation 1: Different media services have different levels of jitter requirements, which are required to be addressed to ensure deterministic latency for immersive communication. Proposal 2: For Model-2 of eXR model with Haptics, support the jitter parameter as a mandatory. - Reuse the description in clause 5.1.1.2 in TR 38.838 as a starting point. - Further considering whether it needs to differentiate typical types of Haptics for evaluation purposes. |
| Interdigital | Proposal 4: Support the traffic model haptics - Haptics traffic is defined as XR traffic packet generation with co-generated haptics packets. - Generation of haptics packets are determined by the following pseudo-code. - Haptics packets has packet delay budget (PDB) of either 12 msec or 30 msec, which can be selected as a traffic model parameter. - Haptics traffic parameters for different uses cases and different FPS of XR traffic in Table 2, 3, and 4 of R1-2600526. At a XR packet generation event, - If previously silent state - Transition to haptic state with probability P2 - If silent state, generate a haptics silent packet - If transitioned to haptics state, - Determine number of channels (exponential distribution with min and max values) - Packet size is determined to be ‘haptics unit size X number of channels’ - Generate haptics active packet - If previously haptics state - Transition to silent state with probability P1 - If haptics state, generate another haptic active packet with same size as previous haptics active packet. - If transitioned to silent state, generate a haptics silent packet. |
| Nokia | - Proposal6: Further discussion of eXR model with haptics, and especially a-synchronicity between different QoS flows, should continue in coordination in RAN2. |
| Ofinno | Observation 3: Based on SA4 response LS, among parametric and pulse code modulation (PCM), the parametric haptics are used in most recent services. Observation 4: Packet delay budget (PDB) values of 12 ms and 30 ms for the haptic packets are aligned with the tolerable haptic delay thresholds in table 7.4.2.2-1 of TR 26.925. Proposal 2: Prioritize the parametric coded media format for RAN1 work on Model-2 on eXR with haptics. Proposal 3: Consider average bitrate per channel corresponding to the use case of 6 to 32 channels for parametric format in table 5.7-1 of TR 26.925 as the basis for deriving the necessary parameters (e.g., packet arrival rate and packet size) for Model-2 on eXR with haptics. Proposal 4: Consider typical values of the target jitter (e.g., 2ms) and packet loss rate/success rate (e.g., 10%/90%) in section 10.3 of TR 26.854 for deriving corresponding traffic parameter values for Model-2 on eXR with haptics. |
| Samsung | Proposal #12: Recommend to wait for further information from SA4 to continue the further discussion on such eXR traffic models. |
| Qualcomm | Proposal 10: On eXR model with and without haptics, RAN1 should wait for the response from SA4 before confirming the working assumption and further studying and identifying the FFS. |
| vivo | Propsoal2: For the modeling of DL haptics traffic, the statistical parameters in Table 3-1 above are supported. Table 3-1: Statistical parameters for DL haptic traffic model Parameters; Unit; Value (Parametric media format). Packet size; bit; Pareto distribution with α=7, range = [504, 1736] bits CDF of pareto distribution: Fx=1-xmxα, xm is the minimum value in x.. Packet inter-arrival time (T); ms; Generated according to an exponential distribution with λ=0.015, min=M, and subsequently quantized to the multiple of M using a rounding function (e.g., T=roundXM∙M), such as M =32ms. CDF of exponential distribution: Fx=1-e-λx x≥0. Jitter; ms; Optional, follows the description in clause 5.1.1.2 in TR 38.838. PDB; ms; 30. Packet Success rate; %; 90. Proposal3: For the modeling of UL haptics traffic, the UL control/pose traffic model defined in section 5.2 of TR 38.838 is reused. Proposal4: For multimodal immersive services, support the independent generation of haptics, video and audio flows as baseline; correlation is optional. |
| ZTE | Proposal 4-3-1: For 6G immersive communication services, reuse the existing XR traffic model structure with updated parameter values, which should be determined in accordance with the results from SA4. Observation 4-3-1: Video stream and haptic stream is one of typical multi-modality in the use case of XR/metaverse/multi-modality service. Proposal 4-3-2: Haptics traffic model can be studied for 6G immersive communication, the following traffic model can be the starting point for haptic traffic modeling: - Periodicity: Aperiodic (e.g. Pareto distribution) / periodic (e.g., per slot arrival) - Packet size: fixed up to 96 Bytes (for a single haptics sensor) / variable up to 2K Bytes (for multiplex haptic sensors) - Packet success rate/Reliability: 99.9%~99.999% - Packet delay budget: 1~5ms - Synchronization (with video): 15~50ms |

.

### Discussions

Background

The last meeting discussed two eXR models (with and without haptics) and reached the following working assumptions along with the LS sent to SA4:

Working Assumption For 6GR evaluations related to immersive communications services, the following two amended XR models based on the existing XR traffic model (in TR 38.838) can be considered: - Model-1: eXR model without Haptics - …… - Model-2: eXR model with Haptics - Haptics traffic is defined as XR traffic packet generation with co-generated haptics packets. - FFS on how to generate the multi-channel haptics packet including how to handle silent periods of haptics and the haptics packet sizes. - FFS on how to co-generate haptics packets and the XR traffic packets. - Haptics packets has packet delay budget (PDB) of either 12 msec or 30 msec, which can be selected as a traffic model parameter. - Send LS to SA4 to inform about the above agreement and check if SA4 has related inputs for the model. Note: whether the working assumption can be confirmed relies on SA4’s response

Observations from the submitted paper for this meeting

About 8 companies (CATT, China Telecom, Interdigital, Nokia, Ofinno, Samsung, Qualcomm, vivo, ZTE) discussed on the two eXR traffic models. In particular,

  - 4 companies (CATT, Nokia, Samsung, Qualcomm) view that RAN1 should wait for other WGs’ study first and closely coordinate with them before RAN1 pursuing further discussion.

  - The other 5 companies (China Telecom, Interdigital, Ofinno, vivo, ZTE) discussed and proposed something to further progress on the eXR model with haptics based on the current study outcome (TR26.854, TR26.925) from SA4 so far or TR22.856 and TR 22.847 from SA1 further. In details, including

    - Addressing FFS on how to co-generate haptics packets and the XR traffic packets;

    - Addressing FFS on how to generate the multi-channel haptics packet including how to handle silent periods of haptics and the haptics packet sizes

    - The key parameters and the values encompass jitter, PDB, packet success rate, etc.

  - However, the views among the proponents are divergent, e.g., how to model the generation of the haptics packets.

Excerpted tables from SA4 TR26.854 and TR26.925 or TR22.847

Table 5.7-1: summary of typical haptics media traffic characteristic. [TR26.925]

| Use case | Nb of channels | Media Format | Density | Average bitrate per channel |
| --- | --- | --- | --- | --- |
| Haptic enhanced media distribution (clause 5.2 of [48]) | 1 to 32 | Parametric | Light Medium High | 0.25 to 0.75 kbps 0. 5 to 1.5 kbps 1 to 5 kbps |
|  | 1 to 32 | Time sampled | Light Medium High | From 6 to 64kbps depending on the density and the quality of the desired signal. 8-16 kbps for good quality at medium and high density. 32-64 kbps for very high quality at medium and high density. |
| Haptic enhanced communication (clause 5.3 of [48])) | 1 to 4 | Parametric | Light Medium | 0.25 to 0.75 kbps 0.5 to 2 kbps |
| Immersive Entertainment (clause 5.4 of [48])) | 1 to 32 | Parametric | Light Medium High | 0.25 to 0.75 kbps 0. 5 to 1.5 kbps 1 to 5 kbps |
|  | 1 to 32 | Time sampled | Light Medium High | From 6 to 64kbps depending on the density and the quality of the desired signal. 8-16 kbps for good quality at medium and high density 32-64 kbps for very high quality at medium and high density |
| Immersive multi-modal XR and metaverse (clause 5.5 of [48])) | 6 to 32 | Parametric | Light Medium High | 0.25 to 0.75 kbps 0. 5 to 1.5 kbps 1 to 5 kbps |
|  | 6 to 32 | Time sampled | Light Medium High | From 6 to 64kbps depending on the density and the quality of the desired signal. 8-16 kbps for good quality at medium and high density 32-64 kbps for very high quality at medium and high density |

Table 7.4.2.2-1: Tolerable asynchronicity thresholds per use-cases [TR26.925]

| Use case | Media | Tolerable asynchronicity threshold (note 1) |  |
| --- | --- | --- | --- |
| Haptic enhanced media distribution | audio-haptics | audio delay: 100ms | haptic delay: 50ms |
|  | visual-haptics | visual delay: 80ms | Haptic delay: 60ms |
| Haptic enhanced communication | audio-haptics | audio delay: 3 frames (25ms) | haptic delay: 1 frame (12ms) |
|  | Visual-haptics | Visual delay: 20ms | Haptic delay: 30ms |
| Immersive games and Immersive multimodal XR and metaverse | audio-haptics | audio delay: 50 ms | haptic delay: 25 ms 1 frame for gaming |
|  | visual-haptics | visual delay: 15 ms | Haptic delay: 50 ms |
| Immersive entertainment | audio-haptics | audio delay: 25 ms | haptic delay: 12 ms |
|  | visual-haptics | visual delay: 20 ms | Haptic delay: 30ms |
| NOTE: For each media component, “delay” refers to the case where that media component is delayed compared to the other. |  |  |  |

Table 10.3-1 Typical QoS requirements for haptic media enhanced services [TR26.854]

|  | Haptics | Video | Audio |
| --- | --- | --- | --- |
| Jitter (ms) | ≤ 2 | ≤ 30 | ≤ 30 |
| Delay (ms) | ≤ 50 | ≤ 400 | ≤ 150 |
| Packet loss (%) | ≤ 10 | ≤ 1 | ≤ 1 |
| Update rate (Hz) | ≥ 2000 | ≥ 24 | ≥ 20k |
| Packet size (bytes) per channel | 60-350 compressed parametric 50-1500 time sampled | ≤ MTU | 160-320 |
| Throughput (kbit/s) per channel | n*(fe*16)1 bits/s time sampled 16-32kbps for compressed parametric2 | 2500 - 40000 | 64-128 |

NOTE 1:	 where n is the number of channels and fe the sampling frequency synthetized in 16 bits.

NOTE 2:	 throughput for compressed parametric vary based on density and placement of keyframe.

Table 4-3-1 Typical use cases including haptic stream [ZTE]

| Use case of metaverse and immersive communication | Reliability | Latency |
| --- | --- | --- |
| 1. XR enabled collaborative and concurrent engineering in product design | Audio/video: 99.9% Haptic(with compression): 99.999% Haptic(without compression): 99.9% | Video/Audio: 10 ms Haptic: 5 ms |
| 2. Remote control robot | Haptic: 99.99% Sensor: 99.999% Video/Audio: 99.9% | Haptic: 1~100 ms Video/Audio/Sensor: 5 ms |
| 3. Immersive multi-modal VR application with multiple 5G UEs directly connected | Video/Audio: 99.9% Haptic(with compression): 99.999% Haptic(without compression): 99.9% | Video/Audio:10 ms Haptic: 5 ms |

Two illustrations for the haptics packet generation from contributions:

  - The attributes of the traffic characteristics of haptic media services are packet size per channel, jitter, packet delay budget, packet loss rate, and throughput per channel.

  - Need to study not only its periodic characteristic as modelled in existing XR traffic model in TR 38.838, but also its aperiodic traffic characteristic.

  - Many are relatively low data rates that are paired with high data rate streams. Haptics traffic has much more stringent latency requirements as it is intended to be a feedback system. Synchronicity with paired data stream is an important feature of haptics.

Atl1: Haptics packets genaration

Alt2: Haptics packets genaration

Planning for this meeting:

Given SA4 has output some study outcome, referring to the excerpted tables above, we can discuss whether those suffice to derive a model on haptics that could be used for RAN1 techniques study. However, components’ views are not aligned even though mostly based on SA4 study. Offline discussion is needed before getting to a concrete proposal for the model.

The harmonized proposal from moderator for the key components of the model can be used as the starting point.

Round-1 Discussions:

#### (FL1) Proposal 4.2.2

For the traffic model on eXR with haptics,

- Haptics traffic is defined as XR traffic packet generation with co-generated haptics packets, following the pseudo-code

At a XR packet generation event, - If previously silent state - Transition to haptic state with probability P2 - If silent state, generate a haptics silent packet - If transitioned to haptics state, - Determine number of channels - Opt1: exponential distribution with min and max values - Opt2: fixed values. - Packet size is - Opt1: determined to be ‘haptics unit size X number of channels’ - Opt2: following Pareto distribution with α=7, range = [504, 1736] bits. - Generate haptics active packet - If previously haptics state - Transition to silent state with probability P1 - If haptics state, generate another haptic active packet with same size as previous haptics active packet. - If transitioned to silent state, generate a haptics silent packet

- The jitter is modelled as a random variable added on top of the haptics packet’s arrival. The jitter follows truncated Gaussian distribution. FFS on Mean/STD/Truncation range.

- Haptics packets have packet delay budget (PDB) of either 12 msec or 30 msec, which can be selected as a traffic model parameter

- Haptics packets success rate 99.9%~99.999%.

- Haptics packets synchronization tolerance: 15~50ms.

Any comments/suggestions, please leave them here:

| Company | Comments |
| --- | --- |
| Qualcomm | It is better to wait for the response from SA4 before RAN1 continue discussion the remaining model details |
| ZTE | We are generally fine with the proposal. However, the wording “silent packet” sounds a bit odd. In our view, once the UE transitions to a silent state, it is reasonable that no packets are generated. |
| vivo | In our understanding, the above pseudo-code applies only to haptics traffic. For the packet size in Opt2, we propose to revise it as follows: Opt2: following Pareto distribution, e.g., α=7, with range = [504, 1736] bits. Regarding the co-generation of haptics packets and audio/video packets, the following two approaches can be considered: - Approach 1: Independent multi-flows generation Since video, audio, and haptic sensors operate independently, multiple flows can be generated separately. - Approach 2: Correlated packets generation Randomly associate one haptic packet with adjacent video/audio packets. |
| Nokia | We have an opinion that complex relation in between the traffic flows and switching in between the states should be defined in the other WGs first, e.g., SA4 and RAN2, before One additional comment: packet delay budget cannot be a parameter of the traffic model. Model is responsible only for the generation of the traffic, not for dropping packets. Other NW mechanisms defines if and when packets can be dropped. |

## New model 3- Multiple packet size

### Companies’ views

| Company | Views/proposals |
| --- | --- |
| CATT | Proposal 2: For FTP model 3 extension 1, support Alt2: Y=X. |
| CMCC | Proposal 2: For mixed-service traffic model, support the followings: - The number of packet size X is 3, and the specific values in ETSI TS 103 786 (i.e., 0.28KB, 30.5KB, 665KB) can be set as baseline. - The packet sizes Y simulated for each UE can be same as X. - The baseline value of T_i can be 14ms, which is same as the value of WT1 in ETSI TS 103 786, and different values of S_i can related to one same T_i for sake of simplicity. |
| Ericsson | Proposal 6-1-1 - For FTP3-extension with X=2, packet sizes of {4kB,400kB} can be used - Corresponding fractions of traffic {small, large} per packet size can be [{2%,98%} or {12%,88%}] Proposal 6-1-2 - Include additional modelling of multiple packets per session for FTP Model extensions, - Session sizes of {4kB,400kB} and packet sizes of {0.2kB,20kB} can be used - Corresponding fractions of traffic per packet size are the same as for the single packet per session case. Proposal 6-1-3 - Define eFTP1-extension as a special case of FTP3-extension, where a single packet arrives in the case of eFTP1-option1 and a session of packets arrives in the case of eFTP1-option2, while the remaining parameters of eFTP1-extension are the same as those of FTP3-extension. Table 2.6.1.2-1 – Example parameters for FTP3-extension (Alt 1) Parameters; FTP Model 3 (used in LTE/NR); FTP3-extension (Alt1) (for 6GR). Packet Size 1 (small); Packet Size 2 (large). Session Size (S); 4kB; 400kB. Packet Size (P); 500kB; 200bytes; 20kB. Mean inter-arrival time (T) between sessions arriving according to Poisson distribution; 200ms; 83.33ms; 166.66ms. Traffic fraction (set 1); NA; 2%; 98%. Traffic fraction (set 2); NA; 12%; 88%. Inter-arrival time between packets within a session (T_p); NA; 5ms; 1.5ms. PDB; FFS; FFS; FFS. Notes - Offered load and traffic fraction calculations are the same as for eFTP3-option1, since the session sizes are the same. - The Inter-arrival time between packets within a session (Tp) can set to fixed value or modelled as random variable - PDB details are FFS but such a bound can be defined for at least some of the UE classes (e.g., for small packets). |
| Google | Proposal1: Adopt Alt 1 (Y=1) as the baseline for FTP3 extension evaluations, where each UE is assigned exactly one traffic flow size from the agreed set of X sizes per simulation drop Proposal2: Prioritise a set of PDB values ( e.g. 10 ms, 20 ms, 50 ms, and 100 ms) to evaluate the performance of delay-sensitive MBB applications in 6GR system-level simulations |
| Huawei | Proposal 2: Regarding the study on extending FTP- 3 to consider the mixed packet sizes, the following parameters are proposed: - The number of packet size X = 2. - Y = X packet sizes and Z = X packet arrival rates/mean inter-arrival time are simulated for each UE, and up to Y packets may arrive at the same time. The inter-relation for values of S_i and T_i depends on the service QoS characteristics. - The other parameters refer to the following table. Packet type; Packet size; Mean inter-arrival time. Candidate 1; Small packet; S_1 = 2 Kbytes; T_1 = 30 ms or other value, which is used to achieve different RUs. Large packet; S_2 = 1 Mbytes; T_2 = 200 ms or other value, which is used to achieve different RUs. Candidate 2; Small packet; S_1 = 40bytes; T_1 = 30 ms or other value, which is used to achieve different RUs. Large packet; S_2 = 0.1Mbytes; T_2 = 200 ms or other value, which is used to achieve different RUs. Note 1: S_2 > S_1, and T_2 > T_1. Note 2: PDB, reliability and/or other parameter may be additionally configured based on services.. |
| Intel | Proposal 6 - For the FTP Mode 3 Extension 1, - X = 2, Y = 1, - Parameter pairs {S1, T1} and {S2, T2} are up to evaluation assumptions for particular technology study - Baseline: A UE is assigned to group i with random uniform probability U∼U(0,1): S=S1,U≤R1S2,U>R1 - R1 is selected from 0.5 and 0.8 - Optional: A UE is assigned to group i based on its radio conditions |
| Interdigital | Proposal 5: For FTP3 extension, support Y=1 packet sizes for each UE (Alt 1) with X = 3. - Companies to provide information on the set of {arrival rates (λi) & packet sizes (Si)} tuple. - Companies to provide detailed information on how the arrival rate & packet sizes are selected for each user. For example, - random assignment of {arrival rates (λi) & packet sizes (Si)} for each user, or - {arrival rates (λi) & packet sizes (Si)} tuple assignment based on UE geometry, assuming S3∙λ3>S2∙λ2>S1∙λ1tuple is ordered from lowest to highest, highest load, and the expected throughput per user is sorted such that Tn'≥Tn'-1, n'=1,⋯,N, where N is the toal number of users in the system, then we may assign offer load for each user based on relative population threshold χ2 and χ1 where if user n' is n'>χ2, then it is assign with {S3,λ3} packet size and arrival rate, if user n' is χ2≥n'>χ1, then it is assign with S2,λ2, and if user n' is χ1≥n', then it is assign with S1,λ1. The population threshold is defined as χ2=S3∙λ3+2S2∙λ23-S1∙λ1S3∙λ3-S1∙λ1 and χ2=2S2∙λ2+S1∙λ13-S1∙λ1S3∙λ3-S1∙λ1. Tn is the estimate throughput per user, Tn=BNlog21+SNRn, SNRn is the geometry of the user, B is the system bandwidth, and N is the number of users in the cell. |
| MediaTek | Proposal2: Down-select X=2 as baseline for FTP3 extension with multiple packet sizes. |
| Nokia | - Proposal1: RAN1 to define FTP3 extension with multiple file sizes according to the following principles: - At most two file sizes (S_1 and S_2) are assumed in one simulation, i.e. X=2. - For each UE traffic is generated with both files sizes, i.e. X=Y=2. - A new parameter K is defined to express the ratio between arrival rates of the files sizes, i.e. λ_1= K·λ_2, with K≥1, assuming S_1>S_2. - The value of K can be FFS and may depend on the values selected for S_1 and S_2. - The arrival rate λ_1 is selected to achieve a certain target cell load level (e.g. Low, Medium, High, each corresponding to a certain RB utilization %), while still maintaining the ratio K between arrival rates. - There is no timing relation between the arrivals of the files of different sizes, i.e. the packet of each size is generated following the independent Poisson Process. - Files sizes and arrival rates can be selected from more than two candidates, also considering DL and UL directions. - Proposal2: Consider the following steps for performing evaluations with the extended FTP3 model: - First, values for S_1 and S_2 are selected (for X=2). - Next, a reasonable value of K is selected, e.g. K=S_1/S_2 means that the load generated by each packet size is the same. - Simulations are run with different arrival rates to identify the arrival rate that is needed to meet a certain load. The X=2 Poisson processes are run independently while always maintaining the agreed ratio K. For example, Low, Medium and High loads can be defined based on a certain RB utilization target as per Error! Reference source not found.. - The identified arrival rates (one per load per link direction) are used for generating and reporting the performance results. |
| DOCOMO | Proposal 1 - For extension to FTP3, support the following number of packet sizes: - The total number of packet sizes: 2 - The number of packet sizes per UE: 1 Proposal 2 - For packet ratio for different packet sizes and mean inter-arrival rate for each packet size, candidate combinations should be predefined. |
| NVIDIA | Proposal 3: Consider “FTP model 3” as the de-facto non-full buffer traffic model for performance evaluation during 6GR study with the following enhancements – - File size (Si): - Variable file size = log-normal (µi, σi) distributions to model each range of file sizes, from Si,small (<10 MB) to Si,medium (~10-100 MB) and Si,large (>100 MB), i.e., number of file sizes (X) = 3. - Inter-arrival time (Ti): - Poisson distribution (λi) governing next packet arrival time (Ti), where λi = 1/ Ti. - Ti is defined in two ways – 1) correlated with Si such that smaller size files arrive more frequently (Ti ∝ Si), and 2) uncorrelated with Si and independently defined based on specific use case scenario. - Number of file sizes per user (Y) - For each user, Y file sizes are simulated, where Y=1 or Y=X, depending on the evaluation scenario. - When Y = 1 (i.e., a single file size simulated per UE), the number of UEs corresponding to each value of X in each drop will depend on the specific use case scenario under evaluation. - When Y=X (i.e. multiple file sizes simulated per UE), the timing relationship between the arrivals of different file sizes per UE will depend on the specific use case scenario under evaluation. - Packet delay budget (PDB): - Service-dependent PDB parameters (i.e., different PDB parameters for different traffic flows). - Candidate PDB values: {10ms, 20ms, 30ms, 40ms, 50ms, 100ms, 200ms, 300ms, 1000ms, 2000ms, 10000ms}, where the extremely large value of 10000ms effectively indicates that there is no PDB constraint. |
| Ofinno | Observation 5: Typically, the file download/upload session for the same user involves multiple files of different sized Observation 6: Different packet sizes in the extended FTP model 3 can be used for modelling different types of traffic/service associated with the same user. Observation 7: Different packet sizes in the extended FTP model 3 can be used for modelling different types of traffic/service associated with the same user. Observation 8: The packet delay budget (PDB) value depends on the type of traffic, i.e., target QoS associated with the service. Proposal 5: Consider alternative 2 with X=2 and Y=2, i.e., for each user generate two packets of different sizes. Proposal 6: Consider small packet size (S1) = 0.3 or 30 Kbyte and larger packet size (S2) = 0.5 Mbyte. Proposal 7: Two packets are generated by independent Poisson processes using independent mean arrival rates of 1 and 2. Proposal 8: Each packet in both FTP model 1 and FTP model 3 with a packet delay budget (PDB), which depends on the packet size. Proposal 9: A suitable value of PDB is used for the packet (S1 or S2) based on the traffic type associated with that packet. Proposal 10: Consider using “file size” instead of “packet size” in the extended FTP model 3. |
| OPPO | Proposal5: For extension of FTP model 3 with multiple packet sizes, support single packet size per UE (Alt1), with different UE distribution for different packet sizes. Proposal6: For extension of FTP model 3 with multiple packet sizes, support at most X=3 packet sizes. Proposal7: For extension of FTP model 3 with multiple packet sizes, support different scaling factors for Si and Ti, to reflect different contribution of packet sizes to throughput. |
| Qualcomm | Proposal 6: On extension of FTP model 3, consider Alt 2 (Y=X) for modeling the number of packet sizes per UE Proposal 7: On extension of FTP model 3, the total number of packet sizes X can be set to 2 Proposal 8: On extension of FTP model 3, the timing relationship for different packet sizes if Y=X is not specified, i.e., following the corresponding packet arrival process Proposal 9: On extension of FTP model 3, the packet size S_i and mean inter-arrival time T_i can be further discussed and decided based on the use case for evaluation. |
| Samsung | Proposal #10: For extended FTP traffic with variable packet sizes adopt as working assumption the following parameters: - Number of packet sizes X = 3 - Packet sizes (S1, S2, S3) = (0.3, 20, 150) kB or (1, 60, 450) kB for comparability of the packet size with the legacy FTP traffic model - Normalized inter-arrival rates (r1, r2, r3) = (4, 8, 1) Proposal #11: For extended FTP traffic with variable packet sizes: - For extended FTP traffic model 1, adopt Y = 1, i.e., each UE can receive packet of only one size - For extended FTP traffic model 3, adopt Y = X, i.e., each UE can receive packets of different sizes |
| vivo | Proposal5: Support the extension of the FTP Model 3 with multiple packet sizes, adopting the parameters in Table 3-3 and the following supplementary notes: - Smaller packets arrive more frequently with stricter PDB requirements. - For a given UE, a maximum of 2 different packet sizes are supported for evaluation. - Further clarify the scenario and purpose for supporting the evaluation of different packet sizes across distinct UEs. |
| ZTE | Proposal 4-1-1: To reflect realistic network traffic conditions, we support Alt1, i.e., Y=1, for extended FTP model. Proposal 4-1-2: For the extended FTP model with Y=1: - Fix the ratio of UEs involved in small packet to those in large packet traffic and randomly assign the traffic type to each UE based on the fixed ratio. - Adopt the model parameters as shown in Table 4-1-1. Table 4-1-1 Model parameters for extended FTP model small packet service; large packet service. packet size; 10KB; 500KB. arrival rate; 5*λ; λ. UE number; 9*N; N. |

### Discussions

Background

The traffic model on FTP3 extension with multiple packet sizes has been discussed for several meetings and the last meeting agreed on the following:

Agreement - For FTP3 extension with multiple packet sizes (the number of packet size X =FFS: 2 or 3), FTP 3-extension 1 - For each packet size S_i, the packets arrive according to Poisson distribution (as FTP 3) with mean inter-arrival time T_i (or arrival rate λ_i where T_i = 1/ λ_i) - Y packet sizes are simulated for each UE - Down-select one from following - Alt1: Y=1; X=e.g., 2 or 3 - Alt2: Y=X; X=e.g., 2 or 3 - Alt3: Either Alt1 or Alt2 can be used depending on the evaluation purpose - FFS: values of S_i and T_i, and their inter-relation (if any) - FFS: change “packet size” to “File size” (terminology) - FFS timing relationship for different packet sizes if Y=X. - FFS the number of UEs for each of X different sizes in a drop if Y=1. - Note: PDB can be considered separately if needed - Note: modeling sessions with multiple packets in each session can be discussed separately if needed. - Down-selection between X=2 and 3.

Observations from the submitted paper for this meeting

The motivation for this model seems to be quite clear already, i.e., extend FTP traffic model with variable/mixed packet sizes, to reflect the traffic observed from the real deployment approximately that the small size packets arrive more frequently than the large size packets.

The discussion points are also clear to follow up the agreement made in the last meeting.

- Whether the packet sizes are generated per cell or per UE.

  - Support Alt1: Ericsson, Google, Intel, Interdigital, DOCOMO, OPPO, ZTE

  - Support Alt2: CATT, CMCC, Huawei, Nokia, Ofinno, Qualcomm, Samsung, vivo

  - Ok with both Alts: NVIDIA

  - Support FTP-1 extension: Ericsson, Samsung

- The number of the packet sizes.

  - X=2: Ericsson, Huawei, Intel, Nokia, DOCOMO, Ofinno, Qualcomm, vivo, ZTE

  - X=3: CMCC, Interdigital, NVIDIA, OPPO, Samsung

Two companies provided simulation results

- Ericsson’s simulation results showing:

  - In the case of the FTP3 model, resource utilization increases steadily with the served traffic and the system is able to serve higher traffic loads. In comparison, in the case of the FTP3-extended model, as the served traffic increases the resource utilization grows unproportionally, highlighting the inability of the system to cater to such traffic. Note that the total offered traffic in both the models is the same. The differences in performance can be driven by a variety of effects, including restrictions on spatially multiplexing UEs, and design or algorithm choices optimized for FTP1 and FTP3 like traffic. By using a more realistic FTP3-extended traffic model, more informed design and algorithm choices that also perform well in real deployments can be made.

- Huawei’s simulation results showing:

  - UPT performance of Alt1 is severely impacted by the different ratios of the two packet sizes, but Alt2 exhibits similar mean UPT performance for the two configurations. Therefore, the Alt2 is more robust for the assumed packet sizes for the two traffic flows.

- The two companies’ logic seems different. Offline discussion is needed to clarify.

Planning for this meeting:

One of the factors concerned for down-selection between Alt1 and Alt2 is about simulation complexity. However, some companies view Alt1 is simpler but other companies view Alt2 should be simpler. Defining two Alternatives needs more efforts but is doable if needed.

At least the point that the small size packets arriving more frequently than the large size packets needs to be defined for either Alternative. The packet size and the packet arrive rate need to be defined properly as well.

Round-1 discussions can start from clarifying the details for implementing each of Alt as follows.

Round-1 Discussions:

#### (FL1) Proposal 4.3.2

- Regarding FTP3 extension with multiple packet sizes:

- The number of packet size X = 2;

- For each packet size S_i, the packets arrive according to Poisson distribution (as FTP 3) with mean inter-arrival time T_i  (or arrival rate λ_i where T_i = 1/ λ_i);

- For Alt1: Y=1 packet size is simulated for each UE

  - For FTP3-extension with X=2, packet sizes of {4kB,400kB} can be used.

  - Corresponding fractions of traffic {small, large} per packet size can be [{2%,98%} or {12%,88%}].

- For Alt2: Y=X=2 packet sizes are simulated for each UE

  - A new parameter K is defined to express the ratio between arrival rates of the packet sizes, i.e. λ_1= K·λ_2, with K≥1, assuming S_1>S_2.

  - The value of K can be FFS and may depend on the values selected for S_1 and S_2.

  - The arrival rate λ_1 is selected to achieve a certain target cell load level (e.g. Low, Medium, High, each corresponding to a certain RB utilization %), while still maintaining the ratio K between arrival rates.

  - There is no timing relation between the arrivals of the packets of different sizes, i.e. the packet of each size is generated following the independent Poisson Process.

  - Packets sizes and arrival rates can be selected from more than two candidates, also considering DL and UL directions.

Any comments/suggestions, please leave them here:

| Company | Comments |
| --- | --- |
| Qualcomm | For Alt 1, the packet size {4kB, 400kB} and the distribution ratio can be examples, and exact values can be discussed in individual evaluations. We support Alt 2: Y=X=2 for each UE. Details can be further discussed in the individual evaluation. |
| ZTE | We believe that the traffic load for small and large packets should be comparable, and the fraction such as {25%, 75%} or {50%, 50%} should be considered. With current config., i.e., {4kB, 400kB} and {2%, 98%}, the resulting number of small and large packets is almost the same, which does not reflect the realistic traffic characteristics. |
| vivo | Overall, we prefer Alt2. We would like to add the following description to clarify the understanding on PDB: The packet delay budget (PDB) agreed for FTP3 can also be additionally considered for FTP3 extension. |
| Nokia | We are generally OK to support options with Y=1 and Y=2, with slightly more preference for Alt.2 because it does not require the definition of UEs ratios with different file sizes. Moreover, our preference is to have as much similarity between Alt1 and Alt2 as possible when it comes to packet sizes, arrival rates, etc. For instance, we think the bulletpoint ‘• A new parameter K is defined to express the ratio between arrival rates of the packet sizes’ can be common to both options. For Alt1, it would be good to clarify (or have an FFS) on how many UEs of each traffic type should be deployed in each BS (and whether the distribution should be kept at BS level or network level). |
| OPPO | We prefer Alt.1 but can live with Alt.2 (not both). For fractions of traffic {small, large} per packet size can be [{98%,2%} or {88%,12%}] where small packet size should have larger ratio. |
| Samsung | OK to accept X = 2, but X = 3 would be better choice for evaluations. The proposal itself should be improved: - Alt 2 is not complete since value K and packet sizes S_i are not specified. - For Alt 1 it is better to define the fraction of UEs with small and large packet sizes and their ratio should be aligned with normalized average packet arrival rates (r1, r2) in Alt 2. For both alternatives our preference (in case X = 2 is accepted) we prefer - (S1,S2) = (60, 450) kB - Normalized inter-arrival rates (r1, r2) = (8, 1) or fraction of UEs with small to large packets are 8:1. |
| NTT DOCOMO | We support Alt1 since we don’t think there is large impact even if multiple packet size per UE is introduced. |
| Google | We support Alt 1 (Y=1) as the baseline for the FTP Model 3 extension. And each UE is assigned exactly one traffic flow size per simulation drop in order to maintain implementation simplicity. We support that a prioritized set of PDB values, e.g. 10 ms, 20 ms, 50 ms, and 100 ms are used to evaluate delay-sensitive MBB applications. |

## (CLOSED)New model 4-bidirectional traffic

### Companies’ views

| Company | Views/proposals |
| --- | --- |
| CMCC | Proposal 3: For traffic model on bidirectional traffic flow, RAN1 further discuss and clarify the following issues: - The delay modeling on core/transport/internet network, i.e., whether this fixed delay as 5~10ms is proper. - The impact on slow start, i.e., how much impact will be reflected on slow start by TCP ACK considering that majority transmission is small packet and large initial congestion window size in current TCP protocol. |
| Ericsson | Proposal 6-2 - 6G system simulation methodology should include realistic modelling of bidirectional traffic flows by considering impact of TCP slow start and TCP ACK latency on throughput. - In DL system simulations the UL TCP ACK delay can be modeled by combining the two components below - Component 1: Fixed delay (e.g., 5-10ms) to reflect Core/transport/internet network delays - Component 2: SR+ UL grant+UL transmission delay. - Suitable values can be chosen based on HARQ RTT, SR availability and TTI length for the corresponding evaluation - Ideal transmission of UL TCP ACK can be assumed instead of explicitly simulating UL. |
| Google | Proposal3: RAN1 should focus on Layer 1/2 aspects within its scope and avoid modeling transport-layer protocols such as TCP/IP, as their complexity and evolving nature are more appropriate for study in other working groups. Proposal4: If agreed necessary, adopt a simplified modelling for UL TCP ACK latency consisting of a fixed 5–10 ms Core Network delay (Component 1) and an explicitly modeled RAN-side transmission delay (Component 2). |
| Huawei | Proposal 3: Modelling the impact of bidirectional traffic flows caused by the TCP ACK needs to be well justified first. |
| Nokia | - Proposal3: RAN1 to include at least one closed-loop (bi-directional) traffic model into the 6GR study, e.g., to consider a scenario when the existing XR model is modelled with realistic TCP ACK feedback. - Proposal4: Default assumptions for simulation with TCP functionality shall be agreed for cases where this is enabled. Our suggestion is to rely on TCP CUBIC. - Proposal5: For simulations with TCP, we suggest that the transmission of the TCP-ACK over the RAN is simulated in UL explicitly, and possible CN latency is added to that. - For cases where CN delay plays a role, bi-direction model can assume a fixed one-way CN delay of e.g. 5 ms, representing the latency between the application (traffic source/sink) and the gNB. - Such parameters as L1/L2 processing latencies, HARQ processing, preparing and decoding of transmission, assumption on TTI size, etc. should not be a part of the traffic model definition, but simulation assumptions. |
| Samsung | Proposal #13: RAN1 does not consider to model the impact of TCP/IP protocol for 6GR evaluations in RAN1. |
| ZTE | Proposal 4-4-1: Whether to model the TCP slow start mechanism in traffic models still requires sufficient justification. |

### Discussions

Background

This bidirectional traffic has been summarized for each of meeting. However, the comments/views collected in the past meetings are unchanged, i.e., some companies have concern to support it or not convinced yet.

Rationale about the modelling (inherited from the last meeting)

The bidirectional traffic discussed the last meeting was mainly talking about the TCP protocol from transport layer and its impact on the wireless network performance. The TCP protocol has a slow start mechanism and is used to prevent network congestion.

Specifically, as explained in a couple of companies’ contributions (ZTE, Ericsson) as well, when the TCP connection starts, the congestion window (cwnd) is set to a small initial value, typically measured in units of maximum segment size (MSS). For each ACK received, the congestion window increases by one MSS, which implies cwnd size increases exponentially. During the slow start process, the performance is latency limited. Only when the cwnd size stabilizes can the UE's data rate reach its maximum. This means that a packet transported using TCP consists of a sequence of data chunks, where the separation between packets is determined by the latency of the acknowledgment in the opposite link direction.

Since DL throughput depends on packet size divided by the total transmission time, the DL throughput will be affected by the UL latency. With the high data rates to be supported, the intervals between data chunks will be comparable to the time it takes to transmit the bits. This is the motivation from the proponent to consider the bidirectional traffic flow impact on wireless network performance.

Illustration of slow start mechanism of TCP protocol

Companies’ view on whether to consider this bidirectional traffic flow impact:

Whether to define a bidirectional traffic flow modelling as follows: - In DL system simulations the UL TCP ACK delay can be modeled by combining the two components below - Component 1: Fixed delay (e.g., 5-10ms) to reflect Core/transport/internet network delays - Component 2: SR+ UL grant+UL transmission delay. - Suitable values can be chosen based on HARQ RTT, SR availability and TTI length for the corresponding evaluation - Ideal transmission of UL TCP ACK can be assumed instead of explicitly simulating UL.

-     Most companies (Futurewei, CMCC, ZTE, Huawei, Apple, DOCOMO) view that it should be well justified or clarified first and be prudent in assuming a higher layer protocol to be considered for evaluating wireless network performance.

-    Specific concerns mentioned in companies’ contributions:

- Transport layer protocol is out of 3GPP and has been evolving as well in another standard origination. For instance, as mentioned that the TCP protocol nowadays allows a larger initial congestion window size, which can mitigate significantly the performance bottleneck caused by latency-limited slow start phases. (ZTE, Huawei)

- It is unclear and should be clarified how to consider the latency on RAN MAC/RRC/core network layer/etc., and how to model the impact on ACK/NAK for TCP layer. (CMCC)

Plan for this meeting

Given the situation has never been changed in the past meetings and this meeting will be the last meeting for this agenda, moderator suggests this bidirectional traffic discussion is not to be pursued.

(FL1) Conclusion

No discussion on this model for this meeting.

Any comments/suggestions, please leave them here:

| Company | Comments |
| --- | --- |
| Samsung | Ok with conclusion |
| NTT DOCOMO | We are OK with the conclusion. |

## Traffic model for IoT

### Companies’ views

| Company | Views/proposals |
| --- | --- |
| Ericsson | Proposal 6.4-1 - For 6GR evaluations related to Massive Communication (IoT), the following traffic types can be assumed: - Triggered/polled reporting - Autonomous reporting (event-driven or periodic) - Remote actuation - Firmware/software upgrade Proposal 6.4-2 - Adopt the traffic models in tables 2.6.4-1 to 2.6.4-4 for 6G Massive Communication (IoT) use cases. |
| Google | Adopt the IMT-2020 mMTC traffic model (TR 37.910) as the baseline for 6GR massive communication evaluations to maintain comparability with 5G connection density benchmarks |
| Huawei | Proposal 4: mMTC traffic model from IMT-2020 (TR 37.910) shall be reused to 6G IoT evaluation. |
| Intel | Proposal 5 - RAN1 to workout two additional models covering 6G MTC/IoT use cases - Bulk download, as captured by Firmware/software upgrade example - Correlated message storm/arrival, as captured by Autonomous Reporting example table |
| Nokia | - At this stage, we do not observe a strong need to introduce new traffic models for IoT connection density evaluation beyond what has been agreed. The first three models under FFS consideration (triggered/polled reporting, autonomous reporting, remote actuation) were previously used in IoT analyses. However, these models are similar in nature, and not all three are required. If more models need to be introduced, our preference is on autonomous reporting. The firmware/software upgrade scenario has low priority due to its infrequent occurrence. Additionally, instant messaging is not that relevant use case for massive IoT connection density evaluation, as it has not been studied in this context. - Proposal7: Should there be a need to introduce more traffic models for IoT/massive communication, we prefer to focus solely on Autonomous reporting. |
| DOCOMO | Proposal 3 - For traffic model for massive communication, support to model new traffic types by existing models (e.g., mMTC traffic model, FTP3) with new parameter sets. |
| Sony | Proposal 1: The following new IoT traffic model(s) for 6GR evaluation in RAN1 are necessary and realistic: - Triggered/polled reporting - Autonomous reporting (event-driven or periodic) - Remote actuation - Firmware/software upgrade |

### Discussions

Background

The last meeting agreed to further study whether new traffic model is needed for 6GR evaluation in RAN1:

Agreement For 6GR evaluations related to Massive Communication (IoT), - For comparability with 5G results and verify that 6G can meet the IMT-2030 connection density requirements, the mMTC traffic model from IMT-2020 (TR 37.910) may be used as a starting point. This traffic model can be applied in UL or DL. - FFS: necessity of new traffic model(s) for 6GR evaluation in RAN1, e.g., for the following traffic types. - Triggered/polled reporting - Autonomous reporting (event-driven or periodic) - Remote actuation - Firmware/software upgrade

The proponent’s rationale about the modelling (inherited from the last meeting)

What traffic models were used in the past

-   In TR 37.910 (“Study on self-evaluation towards IMT-2020 submission”), mMTC uses a traffic model with layer 2 PDU (Protocol Data Unit) message size of 32 bytes and 1 message/day/device or 1 message/2 hours/device, where the packet arrival follows Poisson arrival process for non-full buffer system-level simulation.

-    In TR 36.888 (“Machine-Type Communications (MTC) User Equipments (UEs) based on LTE”), the following traffic types have been considered for mMTC (Annex A and Annex A.1):

- Triggered reporting (command-response traffic)

- Autonomous reporting (exception/event-driven reports or periodic reports)

-   In TR 45.820 (“Cellular system support for ultra-low complexity and low throughput Internet of Things”), the following traffic types have been considered (Annex E.2):

- Autonomous reporting (exception reports or periodic reports)

- Network command

- Software update

Why a new model is needed?

-    Although the traffic types described in the above TRs may still be relevant for 6G massive communication, the traffic characteristics (e.g., packet size, inter-arrival time) may not be representative of what is expected for 6G massive communication or what has been observed in real deployments of legacy massive IoT solutions.

What the new aspect needs to be considered for the new traffic model for IOT

The following traffic models can be considered as representative of the applications expected for 6G massive communications for evaluation purposes:

- Network triggered/polled reporting

  - Application layer in the network triggering an UL application payload from the device, e.g., for sensor reading.

- Device autonomous reporting (event-driven or periodic)

  - Event-driven: An UL application payload triggered by an event in the device delivered (within a certain latency target) and a DL application ACK, e.g., for outage notifications from sensors.

  - Periodic: Periodic UL reporting from a device and a DL application ACK, e.g., for regular sensor reading.

- Remote actuation

  - An application server generates an application layer command to the device to perform an action (in the physical world) with an UL application ACK received (within a certain latency target), e.g., for disconnecting devices or triggering an emergency shutoff.

- Firmware/software update

  - All 6G massive communication devices are expected to require occasional firmware and/or software updates (e.g., every few months). Although updates are expected rather occasionally, file sizes are expected to be relatively large (e.g., for new version release) and certain updates (e.g., security patches) may need to be delivered to selected UE groups within a certain update campaign timeframe (e.g. within a few days or weeks). Based on the above discussion, we have the following proposals.

Observations from the submitted paper for this meeting

  - The proponent company Ericsson proposed the same traffic models (i.e., 4 tables as below) as in the last meeting.

  - Sony sympathised with Ericsson’s proposals and in particular views that,

    - For Autonomous reporting, this traffic model allows autonomous transmissions from the UE. In the 6G era, we expect an increase in devices autonomously reporting, for example personal AI agents sending machine-generated data. The requirement to consider messaging storms allows the modelling high PRACH usage and overload.

    - For Remote actuation, in the 6G era, it is time to consider these network-originated IoT traffic models. We expect an increase of personal AI agents generating and consuming traffic, where the remote actuation traffic model models this data consumption.

    - For Firmware/software upgrade, this traffic model allows the data rate performance of 6G IoT to be evaluated. It would also potentially allow multicast or broadcast proposals to be evaluated. The traffic model offers a data rate of close to 5Mbps, testing the ability of 6G IoT to support higher data rates.

  - The other companies (Huawei, Intel, Nokia, DOCOMO) shared concern to introduce too many traffic models for IoT.

Handling plan for this meeting

The new models have not been discussed yet though summarized for each meeting. Moreover, there are still not many views from companies shared on these new models.

Moderator suggests companies can review the use cases as exemplified by Sony for the proposed models first (as summarized above) and check whether the proposed models from the proponent are convincing.

#### (FL1) Proposal 4.5.2

For the new traffic model(s) for 6GR evaluation related to Massive Communication (IoT), the following models can be considered:

Table 1 – Network triggered reporting

| Parameter | Characterization |
| --- | --- |
| Packet size | DL trigger: [150] bytes UL payload: [1000] bytes |
| Inter-arrival time | [1] report/hour/UE Packet arrival process follows uniform distribution in a 10-minute time window per hour |
| Number of UEs per cell | [50,000] |
| Mobility pattern | 70% stationary, 30% non-stationary |

Table 2 – Autonomous reporting

| Parameter | Characterization |
| --- | --- |
| Packet size | UL payload: [1000] bytes DL ACK: [100] bytes |
| Inter-arrival time | Periodic reporting: - [1] report/hour/UE - Packet arrival process follows uniform distribution in a 10-minute time window per hour Event-driven reporting: - Reports from [5%] of the UEs within a 1-minute window (message storm in case of an outage) |
| Number of UEs per cell | [50,000] |
| Mobility pattern | 70% stationary, 30% non-stationary |

Table 3 – Remote actuation

| Parameter | Characterization |
| --- | --- |
| Packet size | DL payload: [500] bytes UL ACK: [100] bytes |
| Inter-arrival time | [2] commands/day/UE Packet arrival process follows Poisson distribution |
| Number of UEs per cell | [50,000] |
| Mobility pattern | 70% stationary, 30% non-stationary |

Table 4 – Firmware/software upgrade

| Parameter | Characterization |
| --- | --- |
| Packet size | DL payload: [2] Mbytes, transmitted using FTP3 traffic model with [0.1 Mbyte] segment size and [200 ms] interarrival time between segments UL ACK: [100] kbytes in total for all segments |
| Inter-arrival time | [4] upgrades/year/UE Upgrade [10%] of the UEs within a 12-hour (night-time) window |
| Number of UEs per cell | [50,000] |
| Mobility pattern | 100% stationary |

Any comments/suggestions, please leave them here:

| Company | Comments |
| --- | --- |
| ZTE | We don’t see a strong need to introduce a new model for mMTC, since the KPIs in 6GR are the same as those in 5G. To keep the evaluation results aligned, the 5G model should be reused. |
| Nokia | We think that there is a strong need to introduces more models, and models have similar properties. If we even need to define more models, then will be good to select a single more model to add. In this case our preference is Autonomous reporting. |
| NTT DOCOMO | We are open to study the models, but how to model should be clarified. (e.g., whether to model new model or make it based on the existing model) |
| SONY | We support these IoT traffic models. The IoT traffic models in this proposal are not targeted at the connection density requirement. This traffic model agreed in RAN1#123 Dallas is used for the connection density requirement: “the IMT-2030 connection density requirements, the mMTC traffic model from IMT-2020 (TR 37.910) may be used as a starting point. This traffic model can be applied in UL or DL”. The traffic models in this proposal allow features / techniques / proposals in the 6GR design to be evaluated. The traffic models allow the following to be evaluated: Network triggered reporting: paging, DRX, UE power consumption Autonomous reporting: PRACH capacity, signalling overhead, small data transmissions, PRACH overload Remote actuation: paging, DRX, UE power consumption (we note that this evaluates similar features to those evaluated by network triggered reporting) Firmware update: data rate supported by IoT devices, multicast / broadcast We think that it is important that the 6G study considers the small / bursty traffic models characteristic of IoT as well as the high data rate traffic models of eMBB. If 6GR just specified a system that was good at streaming 10Gbps files, but couldn’t efficiently handle initial access, paging, DRX, signalling overhead, small data transmission etc., we think that 6GR would be incomplete. Hence, we support the traffic models in this proposal. |
| Ericsson | We support introducing these IoT traffic models. As pointed out by Sony, these models are not for the IMT-2030 connection density requirement related evaluations. Note that the proposed models are also intended to capture (foreseen and unforeseen) IoT access storms, where many IoT UEs need to access the network within a relatively short time window. |

## Other models

### Companies’ views

| Company | Views/proposals |
| --- | --- |
| NVIDIA | Proposal 1: Consider “Full buffer traffic model” for performance evaluation during 6GR study for– - Initial simulation results calibration, - Peak performance benchmarking. Proposal 2: Consider “FTP model 1” as the optional non-full buffer traffic model for performance evaluation during 6GR study with the following enhancements – - File size: at least 3 different file sizes- - flarge (~100s of MB), - fmedium (~10s of MB), - fsmall (~1MB). - User arrival rate distribution (λ): at least 3 different ranges of λ (users/s) – - λlarge-100MB = [.002, .005, .007, .01, .012], - λmedium-10MB = [.025, .05, .075, .1, .12], - λsmall-1MB = [0.25, 0.5, 0.75, 1.0, 1.25]. Proposal 4: Consider existing VoIP model as the baseline for performance evaluation during 6GR study for use case scenarios with ‘voice-only’ traffic. Proposal 5: Consider the following enhancements to the existing VoIP model for performance evaluation during 6GR study for use case scenarios with mixed traffic (voice + data) – - Variable packet size, data rate and inter-arrival time: - Packet size modelling, depending on traffic pattern (voice-only vs. mixed). - Data rate modelling, depending on the heterogeneity of traffic patterns and volumes. - Burst/silence pattern modelling, depending on traffic types and QoS requirements. - Flexible codec and QoS parameters: - Adaptive codec instead of static voice codec, catering to different traffic types and service-based requirements. - Variable latency, jitter, and packet loss requirements to cater to mixed traffic. Proposal 6: Deprioritize instant messaging model and use enhanced FTP model 1/3 (as described in sections 2.2 and 2.3) with appropriate parameterization for performance evaluation during 6GR study for use case scenarios with traffic pattern typical of messaging apps. Proposal 7: Study traffic models for performance evaluation during 6GR study taking into consideration the unique characteristics of UL-heavy traffic. |

# Other assumptions

## Companies’ views

| Company | Views/proposals |
| --- | --- |
| Ericsson | Proposal 7 - 6G coverage evaluation studies for FR1 should use a 1Rx UE antenna and UE bandwidth corresponding to lowest-capability device as baseline assumptions, at least for DL physical signals/channels relevant for Idle/Inactive mode and initial access. Proposal 8 - UE PA output power backoff related studies in RAN1 should consistently use RF simulations/RF requirements. Proposal 9 - NR as deployed by the operators should be used as baseline for evaluations comparing 6GR performance with NR. |
| Interdigital | Support a common link level assumption parameter that can be used for link level evaluation assumptions for various topics. For each topic, adjust and update the common link level assumption parameters as needed. |
| OPPO | Proposal4: Considering that the coverage performance of 5G mid-band (~3.5 GHz) and around 7 GHz would be evaluated, the calculation of (27) Penetration margin (dB) for 5G mid-band (~3.5 GHz) and around 7 GHz needs to clarify. |
| Tejas | Proposal 1: Study Isolated Macro cell with cluster UE drop as one of the deployment scenarios for Rural in 6G. Proposal 2: For 6GR evaluation, study 600 MHz and 7 GHz as carrier frequencies for handheld UE and FWA respectively, for the rural isolated macro cell deployment. Consider Table 2.1-1 and Table 2.1-2 as the evaluation parameters for rural isolated macro cell deployments for large coverage. Proposal 3: The link budget template should specify the target data rate to obtain required SNR. |
| ZTE | Proposal 3-1: For link budget template, - Candidate 1: Reusing the link budget template from TR38.830 with followings: - Reuse the definition of MCL row. - Delete the “(30) Maximum range (based on (29) and according to the system configuration section of the link budget) (m)” row - Candidate 2: Reusing the Template as Table 7.10.1-1 from TR38.913 without any update. Proposal 3-2: Alignment on parameters listed in Table 3-1 and other channel specific assumption can be further discussed in other agendas along with analysis on link budget. |
| QUALCOMM | Proposal 5: For rML receiver modeling into system level simulation, consider the following LLR-based modeling: - For each tone and OFDM symbol, given the corresponding channel matrix H, run the soft-output rML receiver to obtain LLRs l0,…,lM-1 for all M bits of 2M-QAM modulation. - Let bi denote the hard decision for li and bi* be the true value of this bit, collect all li and convert them into mutual information I using the following equation - I=1-E-log2a1+a ,a=eli⋅-1bi*⊕bi - Obtain the effective SNR as eSNR=CM-1(I), where CM denotes BICM capacity for 2M-QAM Use link-to-system BLER curves in order to obtain the BLER |
| ZTE | Proposal 2-3-1: Regarding system level evaluation on R-ML receiver, the emulated UE receiver performance can be derived according to the lower bound and upper bound of R-ML receiver performance, i.e., MMSE-IRC receiver and ideal SIC receiver. - FFS: Determination of the fitting parameter(s). - FFS: Whether the SIC detection order affects system-level simulation results. Proposal 2-3-2: For system-level simulations of the R-ML receiver, joint demodulation of the target UE and co-scheduled UE(s) should be considered. |

## Discussions

Link budget Template and coverage related:

The issues brought up were discussed in the last meeting and there was no consensus for any update then. Coverage related discussion will be handled in another agenda separately and moderator presumes any assumptions related should be discussed there including the explanation of each row of the template.

Other scenarios not included in TR38.914

Including carrier frequency of 600MHz and isolated macro cell, moderator presumes such issues should be discussed and decided in the RAN plenary first.

PA issue

RAN4 is studying on it and should be remained to RAN4.

Common link level simulation assumptions

Interdigital proposed a fairly common set of link level simulation assumption. However, not sure whether it could be agreeable in one last meeting with discussion time reduced a lot for this agenda. It’s better not to open this discussion and left to other agenda to discuss presuming the agreed system level simulation assumptions will be considered anyway when applied.

R-ML Receiver as optional (FFS modelling)

It was agreed that R-ML receiver could be assumed as well as optional but FFS modeling. Qualcomm and ZTE further discussed the modelling for R-ML in this meeting. However, the two proponents have different views for the modelling. Given this meeting is supposed to be the last meeting, moderator suggests the r-ML receiver modelling could be further discussed in evaluation phase in other agendas.

Overall, no other issues need to be discussed in this meeting. Please indicate it otherwise.

Any comments/suggestions, please leave them here:

| Company | Comments |
| --- | --- |

# Proposals for offline/online

## Online proposals on Tue

(FL2) Proposal 2.1.2-1-rv2

For 6GR evaluation, RAN1 to model the UE antenna as follows for below 30GHz carrier frequency,

- Note: Each of other topics could further decide to use which combination(s) for the evaluations. Other combinations are not precluded for evaluations, e.g., 2T6R, 3T6R, 6T6R, 6T8R.

- Note: The antenna locations in Alt1 and Alt 2 in the following table are considered as examples and used for performance calibration. Any antenna array structures and/or antenna locations in section 7.3 in TR38.901 is possible for evaluations and up to companies to report.

- Note: The antenna locations in Alt 2 not included in section 7.3 in TR38.901 are up to companies to report.

- Note: The antenna element-wise power variation at the UE in TR 38.901 section 7.6.14.2 can be optionally considered for Alt2 for handheld devices.

- Note: The radiation power pattern of a single antenna element in Table 7.3-2 TR38.901 is assumed for Alt2. The isotropic radiation power pattern is assumed for Alt1 at least for handheld devices.

- Note: Whether only Alt2 is used for the evaluations could be further decided.

- Note: The antenna element/location of T is a subset of the element/locations for R.

- Note: The mapping between the combination and the device types might be separately discussed.

- Note: Both Combination2 and Combination3 do not imply that all the handheld UE are required to support all antenna configurations across all the listed frequency bands.

| UE antenna modelling for RAN1 evaluations | Total number of antenna elements | Total number of TXRU | Alt 1: (M,N,P,Mg,Ng; Mp,Np), (dH,dV), (dg,H,dg,V) if any, or Alt 2: UT device antenna model using candidate antenna locations as described in section 7.3 in TR38.901 | Applicable carrier frequency |
| --- | --- | --- | --- | --- |
| Combination0 NOTE1 | 1 | 1T1R, | Alt 1: 1T: (M, N, P, Mg, Ng; Mp, Np)=(1, 1, 1, 1, 1; 1, 1) 1R: (M, N, P, Mg, Ng; Mp, Np)=(1, 1, 1, 1, 1; 1, 1) Alt 2: - 1T - 1R | 700MHz, 2GHz |
| Combination1 | 2 | 1T2R, | Alt 1: - 2R: (M, N, P, Mg, Ng; Mp, Np)=(1, 2, 1, 1, 1; 1, 2) for single polarization or (1, 1, 2, 1, 1; 1, 1) for dual polarization, (dH,dV)= (0.5, 0.5)λ Alt 2: - 2R: [(1, 5), or (4, 8)] as described in section 7.3 in TR 38.901. | 700MHz, 2GHz, 4GHz |
| Combination2 | 4 | 1T4R, 2T4R, 4T4R | Alt 1: - 4R: (M, N, P, Mg, Ng; Mp, Np)=(1, 2, 2, 1, 1; 1, 2) for dual polarization or (2, 2, 1, 1, 1; 2, 2) for single polarization, (dH,dV)= (0.5, 0.5)λ Alt 2: - 4R: [(2, 4, 6, 8), or (1, 3, 5, 7)] as described in section 7.3 in TR 38.901 | 700MHz, 2GHz, 4GHz, 7GHz, 15GHz NOTE3 |
| Combination3 | 8 | 1T8R, 2T8R, 4T8R, 8T8R | Alt 1: (M, N, P, Mg, Ng; Mp, Np)= (1, 4, 2, 1, 1; 1, 4), or (2, 2, 2, 1, 1; 2, 2) for dual polarization or (2, 4, 1, 1, 1; 2, 4) for single polarization , (dH,dV)= (0.5, 0.5)λ Alt 2: (1, 2, 3, 4, 5, 6, 7, 8) as described in section 7.3 in TR38.901 | 2GHz, 4GHz, 7GHz, 15GHz NOTE3 |
| Combination4 NOTE2 | 16 | 4T16R 8T16R, | Alt 1: - 16R: (M, N, P, Mg, Ng; Mp, Np)= (2, 4, 2, 1, 1; x, y) , (dH,dV)= (0.5, 0.5)λ Alt2: Company to report the antenna placement, directional pattern orientation of the CPE panel. | 7GHz, 15GHz |
| NOTE1: This combination is for IoT UE only. NOTE2: This combination is for CPE UE only. NOTE3: If number of TXRU and frequency combination is applicable. |  |  |  |  |

(FL2) Proposal 2.1.2-2-rv1

For 6GR evaluation, RAN1 to model the UE antenna as follows for around 30GHz carrier frequency,

- UE antenna configuration follows Table 1 below.

- UE antenna radiation pattern follows Table 2 below.

- Other antenna configuration can be considered and up to companies to report.

Table 1: UE antenna configuration for around 30GHz

| UE antenna configuration | Values |
| --- | --- |
| # of antenna elements per panel | 8 elements per panel (M, N, P) = (2, 2, 2) for Config 1 and Config 2. (M, N, P, Mg, Ng; Mp, Np) = (4, 1, 2, 1, 1; 1, 1) for Config 0. (M, N, P, Mg, Ng; Mp, Np) = (4, 4, 2, 1, 1; 1, 1) for CPE only for Config 0. |
| # of panels | Config 0: 1 panel. Config 1: 2 panels on front and back; Config 2: 4 panels on 4 edges. |
| # of TXRUs | 2T2R per panel - The antenna elements of the same polarization of the same panel is virtualized into one TXRU |

Table 2: UE antenna radiation pattern for around 30GHz [see Table A.2.1-8 TR38.802]

| Parameter | Values |
| --- | --- |
| Antenna element radiation pattern in θ'' dim (dB) |  |
| Antenna element radiation pattern in φ'' dim (dB) |  |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 5dBi |

(FL2) Proposal 2.1.2-3-rv1

For 6GR evaluation, RAN1 to model the radiation pattern of a single antenna element for FWA/CPE including,

- Candidate1: Isotropic,

- Candidate2: Directional with different half power beamwidth and maximum directional gains as described in Table 1 below,

  - CPE can be equipped with 1 to 3 antenna panels, each following Alt 1 ((M,N,P,Mg,Ng; Mp,Np), (dH,dV)) configuration.

- Candidate3: Omnidirectional as described in Table 2 below.

- Note: The orientation of the CPE panel can be optimized.

Table 1: Directional radiation power pattern of a single antenna element for CPE

| Parameter | Values |
| --- | --- |
| Vertical cut of the radiation power pattern (dB) | AdB″θ″, ϕ″=0°=-min12θ″-90°θ3dB2,SLAV with SLAV=22.5 dB and θ″∈0°, 180° |
| Horizontal cut of the radiation power pattern (dB) | AdB″θ″=90°, ϕ″=-min12ϕ″ϕ3dB2,Amax with , Amax=22.5 dB and ϕ″∈-180°, 180° |
| 3D radiation power pattern (dB) | AdB″θ″,ϕ″=-min-AdB″θ″, ϕ″=0°+AdB″θ″=90°, ϕ″,Amax |
| Maximum directional gain of an antenna element | GE,max dBi |
| Company reports, e.g., θ3dB, ϕ3dB, GE_max=32°, 32°, 14, 65°, 50°, 9, 90°, 90°, 5 |  |

Table 2: Omnidirectional radiation power pattern of a single antenna element for CPE

| Parameter | Values |
| --- | --- |
| Vertical cut of the radiation power pattern (dB) | AV″θ″=-min12θ″-90°θ3dB2,SLAV with θ3dB=22.5°,SLAV=22.5dB and θ″∈0°, 180° |
| 3D radiation power pattern (dB) | AdB″θ″,ϕ″= AV″θ″ for any ϕ″∈-180°, 180° |
| Maximum directional gain of an antenna element, GE,max | 5 dBi |

Proposal a:

Define a 4 TXRU outdoor BS antenna configuration for about 4GHz carrier frequency as below.

| Outdoor |  |  |  |  |
| --- | --- | --- | --- | --- |
| BS antenna modelling | Total number of antenna elements | Total number of TXRU | (M, N, P, Mg , Ng; Mp, Np) | (dH,dV) |
| Combination [0] (Optional) | 32 | 4 | (8, 2, 2, 1, 1; 1, 2) | (0.5, 0.8)λ |

(FL2) Proposal 3.1.2-1-rv1

Regarding the agreement on UE transmission power assumptions in system-level simulation, the further update is highlighted as follows in red:

- Transmission power of 35dBm is assumed for CPE only for below 30GHz.

| UE power class | Indoor Hotspot | Dense Urban | Rural | Urban Macro | Sub-urban macro |
| --- | --- | --- | --- | --- | --- |
| Around 30GHz | Minimum peak EIRP: 23dBm. EIRP should not exceed 43dBm. For CPE only (NOTE1): 35dBm<=peak EIRP<=55dBm. | Minimum peak EIRP: 23dBm. EIRP should not exceed 43dBm. For CPE only (NOTE1): 35dBm<=peak EIRP<=55dBm. | NA | Minimum peak EIRP: 23dBm. EIRP should not exceed 43dBm. For CPE only (NOTE1): 35dBm<=peak EIRP<=55dBm. | Minimum peak EIRP: 23dBm. EIRP should not exceed 43dBm. For CPE only (NOTE1): 35dBm<=peak EIRP<=55dBm. |
|  | - NOTE1: It is up to company to report the simulated transmission power confined within the defined peak EIRP range. |  |  |  |  |

(FL2) Proposal 3.1.2-2

- Regarding the gNB transmission power assumptions in the evaluations, update the second note of the agreement as follows:

Note: For evaluation purpose, BS Tx power scales up with bandwidth proportionally under the limitation of the maximum BS Tx power is 56dBm for outdoor and 33dBm for indoor for the above carrier frequencies. - Note: The values defined in option1 refer to the Report ITU-R M. [IMT-2030. EVAL]. The values defined in option2 is calculated based on the proportional scaling with simulation bandwidth under the limitation of the maximum BS Tx power of 56dBm.

(FL2) Proposal 3.2.2-1-rv1

The agreed table for UE distribution and UE speed for system-level simulation, the further update is highlighted in cyan as follows:

| Parameters | Indoor Hotspot | Dense Urban | Rural | Urban Macro | Suburban Macro |
| --- | --- | --- | --- | --- | --- |
| UE distribution and UE speed | 10 users per TRxP. Opt1: 100% Indoor, 3km/h | Single layer: Uniform/macro TRxP Two layers: Uniform/macro TRxP + Clustered/micro TRxP UE number per TRxP is [10, 30, 50] NOTE1. Opt1: 80% indoor (3km/h); 20% outdoor in cars (30km/h). Opt2: 40% indoor (3km/h) 40% outdoor (3km/h) 20% outdoor in cars (30km/h). | Single layer: Uniform/macro TRxP UE number per TRxP is [10, 30, 50]. Opt1: 50% indoor (3km/h); 50% outdoor in cars (120km/h). Opt2: 20% indoor (3km/h) 40% outdoor (60km/h) in cars 40% outdoor in cars (120km/h). | Single layer: Uniform/macro TRxP Two layers: Uniform/macro TRxP + Clustered/micro TRxP UE number per TRxP is [10, 30, 50] NOTE1. Opt1: 80% indoor (3km/h); 20% outdoor in cars (30km/h). Opt2: 40% indoor (3km/h) 40% outdoor (3km/h) 20% outdoor in cars (30km/h). | Single layer: Uniform/macro TRxP UE number per TRxP is [10, 30, 50]. Opt1: 10% Outdoor pedestrian: 3km/h; 10% Outdoor in cars: 40km/h; 80% Indoor in houses: 3km/h. Opt2: 20% outdoor in cars: 40km/h 80% indoor in houses: 3km/h |
| FFS: Applicability for FWA NOTE1: Regarding the number of UEs per TRxP, a smaller or the same number of UEs is assumed for each micro TRxPs compared to each macro TRxPs. |  |  |  |  |  |

(FL2) Proposal 3.2.2-2-rv1

RAN1 to assume the UE antenna height and UE distribution for CPE for 6GR evaluations as follows:

- Note: Indoor and outdoor CPE pre-selection criterion or mechanism could be further discussed in the evaluation phase.

| Parameters | Rural | Urban Macro | Suburban Macro |
| --- | --- | --- | --- |
| Antenna height for CPE only | Indoor CPEs: follow the heights in 38.901 for RMa. Outdoor CPEs: 1m above rooftop. Building heights modeled as 3m or 6m, equally likely. | Indoor CPEs: follow the heights in 38.901 for UMa. Outdoor CPEs: 1m above building height in 38.901 for UMa | Indoor CPEs: follow the heights in 38.901 for SMa. Outdoor CPEs: 1m above building height in 38.901 for SMa |
| UE distribution and UE speed for CPE only | Profile 1 (mixed deployment): 80% Indoor CPE: (0, 0.3] km/h; 20% Outdoor rooftop mounted CPE: (0, 0.3]km/h. Profile 2 (Indoor CPE only): 100% Indoor: (0, 0.3]km/h. Profile 3 (Outdoor mounted CPE only): Rooftop mounted; 100% Outdoor: (0, 0.3] km/h. |  |  |

(FL2) Conclusion

The scenarios captured in TR38.914 but are not discussed in RAN1 are also candidate scenarios for RAN1 evaluations for 6GR. The other detailed assumptions could be further discussed in the evaluation phase.

# References

R1-2600026	On remaining aspects of 6GR evaluations	Nokia

R1-2600137	Evaluation assumptions for 6GR air interface	Huawei, HiSilicon

R1-2600187	Evaluation assumption for 6GR air interface	OPPO

R1-2600227	Discussion on evaluation assumptions for 6GR air interface	ZTE Corporation, Sanechips

R1-2600293	Further discussion on evaluation assumptions for 6GR air interface	CATT

R1-2600383	Discussion on evaluation assumptions for 6GR air interface	CMCC

R1-2600423	Discussion on evaluation assumptions for 6GR air interface	Xiaomi

R1-2600498	Remaining issues on evaluation assumptions and traffic models for 6GR	vivo

R1-2600591	Traffic modelling for 6GR air interface	NVIDIA

R1-2600691	Discussion on evaluation assumptions for haptic traffic models	China Telecom

R1-2600750	Evaluation assumptions for 6GR	Samsung

R1-2600790	Fixed Wireless Access Scenarios	T-Mobile USA, Ericsson, MediaTek, Nokia, Verizon

R1-2600822	On evaluation assumptions for 6GR air interface	Apple

R1-2600901	Evaluation assumptions for 6GR air interface	MediaTek Inc.

R1-2600913	Evaluation assumptions for 6GR	Sharp

R1-2600933	Evaluation assumptions for 6GR	Intel Corporation

R1-2600998	Discussion on evaluation assumptions for 6GR air interface	ETRI

R1-2601045	Evaluation assumptions for 6GR	Ericsson

R1-2601091	Discussion on Evaluation assumptions for 6GR air interface	Ofinno

R1-2601175	Discussion on Evaluation assumptions for 6GR air interface	NTT DOCOMO, INC.

R1-2601267	Evaluation assumptions for 6GR air interface	Qualcomm Incorporated

R1-2601356	Evaluation Assumptions for 6GR Air Interface	Tejas Network Limited, CEWiT, IIT Madras

R1-2601369	On Evaluation Assumptions for the 6GR air interface	Google Korea LLC

R1-2601436	Evaluation assumptions for 6GR air interface	Sony

R1-2601531	Evaluation assumptions for 6GR air interface	InterDigital, Inc.

# Appendix – Existing traffic models

- FTP Model 1, FTP Model 2, and VOIP (in TR 36.814)

#### A.2.1.3 Traffic models Traffic models for system performance evaluations are given in Table A.2.1.3-1. System throughput studies shall be assessed using full-buffer traffic model capturing continuous traffic and non-varying interference. Additionally, evaluations with time-varying interference shall be carried out using bursty traffic models. Table A.2.1.3-1 proposes FTP traffic models to exercise system performance studies in bursty traffic. Table A.2.1.3-1. Traffic Models Traffic Models; Model Applies to. Full buffer; DL and UL. Continuous traffic.. Non-full buffer FTP models; DL and UL. Bursty traffic.. VoIP; DL and UL Real time services. ##### A.2.1.3.1 FTP traffic models Two FTP traffic models are considered as non-full buffer traffic models. Tables A.2.1.3-2 and A.2.1.3-3 show the parameters for FTP traffic model 1 and model 2, respectively. Figure A.2.1.3.1-1 and A.2.1.3.1-2 illustrate the user arrival of traffic model 1 and 2, respectively. Baseline model is Model 1 with file size of 2 Mbytes, however Model 1 with file size of 0.5 Mbytes and Model 2 with file size of 0.5 Mbytes can be also evaluated. Table A.2.1.3.1-1. FTP Traffic Model 1 Parameter; Statistical Characterization. File size, S; 2 Mbytes (0.5 Mbytes optional) (one user downloads a single file). User arrival rate λ; Poisson distributed with arrival rate λ. - Small file size of 0.5 Mbytes can be chosen to speed-up the simulation. - Simulations are run for various λ to find performance metrics covering at least the range of HM-NCT (See A.2.1.3.2) that leads to [10%, 50%] of RU (See A.2.1.3.2) in non-CoMP SU-MIMO. - Possible range of λ: [0.5, 1, 1.5, 2, 2.5] for 0.5 Mbytes, [0.12, 0.25, 0.37, 0.5, 0.625] for 2 Mbytes (See A.2.1.3.4 for more details). Range of λ can further be adjusted. - The same traffic should be simulated for CoMP and non-CoMP schemes. The above range of λ will cover RU from 10% to 50% for non-CoMP SU-MIMO Figure A.2.1.3.1-1: Traffic generation of FTP Model 1 Table A.2.1.3.1-2. FTP Traffic Model 2 Parameter; Statistical Characterization. File Size, S; 0.5 Mbytes. Reading Time, D; Exponential Distribution, Mean= 5 seconds PDF: λ = 0.2. Number of users, K; Fixed. - Simulations are run for various K to find performance metrics covering at least the range of HM-NCT that leads to [10%, 50%] of RU in non-CoMP SU-MIMO. - Possible range of K: [2, 5, 8, 10, 14] (See A.2.1.3.4 for more details). Range of K can further be adjusted. - The reading time D is the time interval between end of download of previous file and the user request for the next file. - The same traffic should be simulated for evaluating CoMP and non-CoMP schemes. The above range of K will cover RU from 10% to 50% for non-CoMP SU-MIMO. Figure A.2.1.3.1-2: Traffic generation of FTP Model 2

- FTP Model 3 (in TR 36.872)

FTP Model 3: based on FTP model 2 with the exception that packets for the same UE arrive according to a Poisson process and the transmission time of a packet is counted from the time instance it arrives in the queue 0.5Mbytes file size. The offered traffic is generated per macro cell geographical area when FTP model 1 is used.

- XR Traffic models (in TR 38.838)

# 5 Traffic models In this clause, we provide the DL and UL traffic models for VR, CG, and AR applications. Since DL/UL traffic models for these applications share similar characteristics, we first define a generic and parameterized DL / UL traffic model, which could be later used in defining VR, CG, AR applications. The traffic model defined in this clause is statistical traffic model, where packet size and packet arrival process are characterized by certain random variables. The described model is based on the input XR traffic study from SA4 [7][3][4]. ## 5.1 Generic DL traffic model ### 5.1.1 Single stream DL traffic model This clause provides a parameterized generic single stream DL traffic model. In this model, as shown in Figure 5.1-1, the XR DL traffic is modelled as a sequence of video frames arriving at gNB according to the considered video frame rates and random jitter. The size of each frame is also random according to a certain distribution. Figure 5.1.1-1: Single stream DL traffic model #### 5.1.1.1 Packet Size In this model, a packet models the set of IP packets belong to the same video frame. The video frame includes both left and right eye frame sharing the same buffer, which is referred to as 'single stream for dual eye buffer' or 'single eye buffer' throughout this document. The size of a packet is determined by the given data rates and frame rates, which is modelled as a random variable following truncated Gaussian distribution with following statistical parameters. Table 5.1.1.1-1: Statistical parameters for packet size following truncated Gaussian distribution Parameter; unit; Baseline values for evaluation; Optional values for evaluation for single eye buffer. Mean: M; byte; R×1e6 / F / 8; R×1e6 / F / 8. STD; byte; 10.5% of M; 3 % of M. Max; byte; 150% of M; 109% of M. Min; byte; 50% of M; 91% of M. R: data rate of the flow in Mbps. F: frame generation rate of the flow in fps. Note that the mean and STD apply before truncation applies. Note that the value of R, F depend on application.. Exploration to other distributions for packet size are left up to each company and could be reported with the modelling details. #### 5.1.1.2 Packet arrival In this model, the packet arrival rate is determined by the frame generation rate, e.g., 60fps. Accordingly, the average packet arrival periodicity is given by the inverse of the frame rate, e.g., 16.6667ms = 1/60fps. The periodic arrival without jitter gives the arrival time at gNB for packet with index k (=1,2,3….) as k/F*1000 [ms], where F is the given frame generation rates (per second). Note that this periodic packet arrival implicitly assumes fixed delay contributed from network side including fixed video encoding time, fixed network transfer delay, etc. However, in a real system, the varying frame encoding delay and network transfer time introduces jitter in packet arrival time at gNB which. In this model, the jitter is modelled as a random variable added on top of periodic arrivals. The jitter follows truncated Gaussian distribution with following statistical parameters shown in Table 5.1-2. Table 5.1.1.2-1: Statistical parameters for jitter Parameter; unit; Baseline value for evaluation; Optional value for evaluation. Mean; ms; 0. STD; ms; 2. Truncation range; ms; [-4, 4]; [-5, 5]. Note that the given parameter values and considered frame generation rates (60 or 120 in this model) ensure that packet arrivals are in order (i.e., arrival time of a next packet is always larger than that of the previous packet). Thus, the periodic arrival with jitter gives the arrival time for packet with index k (=1,2,3….) as offset + k/F*1000 + J [ms], where F is the given frame generation rates (per second) and J is a random variable capturing jitter. Note that actual traffic arrival timing of traffic for each UE could be shifted by the UE specific arbitrary offset. #### 5.1.1.3 Packet delay budget The latency requirement of XR traffic in RAN side (i.e., air interface) is modelled as packet delay budget (PDB). The PDB is a limited time budget for a packet to be transmitted over the air from a gNB to a UE. For a given packet, the delay of the packet incurred in air interface is measured from the time that the packet arrives at the gNB to the time that it is successfully transferred to the UE. If the delay is larger than a given PDB for the packet, then, the packet is said to violate PDB, otherwise the packet is said to be successfully delivered. The value of PDB may vary for different applications and traffic types. #### 5.1.1.4 Packet success rate requirement The performance requirement in terms of packet success rate is given as X (%). If packet delivery delay exceed a given PDB, then, the packet is counted as failure. Following values for packet success rate X are considered. Table 5.1.1.4-1: Packet Success Rate Requirement Parameter; unit; Baseline values for evaluation; Optional values for evaluation. Packet success rate requirement X for DL single stream; %; 99; 95, 99.99, etc.. Note that the Packet error rate (PER) in percentage is given as PER = 100 – X. #### 5.1.1.5 Dual eye buffer model This clause describes optional modification of packet size and frame rates for separate packet arrival for dual-eye buffer. In single eye buffer model, the frame for both eyes arrive at the same time as a single packet. Thus, mean packet size M is given as R×1e6 / F, where R is frame generation rate in Mbps and F is frame generation rate. Whereas, in dual eye buffer model of data rate R, the left and right eye frame arrive separately with a time offset, which makes the arrival process effectively equivalent to have two times of frame rates and half mean packet size of that of single eye buffer model. Accordingly, we have mean packet size M of dual eye buffer model is given as R×1e6 / (2×F) for dual eye buffer model. Table 5.1.1.5-1: Statistical parameter values for dual eye buffer packet size Parameter; unit; values for evaluation; Optional values for evaluation. Mean: M; byte; R×1e6 / (2×F) /8; R×1e6 / (2×F) / 8. STD; byte; 10.5% of M; 4% of M. Max; byte; 150% of M; 112% of M. Min; byte; 50% of M; 88% of M. R: data rate of the flow in Mbps F: frame generation rate of the flow in fps. ### 5.1.2 Multi-streams DL traffic model This clause provides optional multi-streams model for XR DL traffic. - Option 1: I-frame + P-frame - Option 1A: slice-based traffic model - Option 1B: Group-Of-Picture (GOP) based traffic model - Option 2: video + audio/data - Option 3: FOV + omnidirectional stream #### 5.1.2.1 Option 1 (I+P) For Option 1, two streams (I-stream and P-stream) are modelled according to Table 5.1-5. - Stream 1: I stream - Stream 2: P stream Depending on the video encoding scheme, two additional sub models – slice based, and Group of Picture (GOP)-based models are defined. - Slice-based: In this encoding scheme, a single video frame is divided into N slices. Out of N, one slice is I slice and remaining N-1 slices are P slices. N packets (one I and N-1 P) packets corresponds to one video frame arriving at the same time. - GOP-based: In this encoding scheme, a single video frame is either I frame or P frame. I frame is transmitted every K frames, where K is the GOP size, i.e., every group of picture. One video frame arrives at a time as a packet. Table 5.1.2.1-1: Statistical parameters for Option 1 multi streams DL traffic model Two data streams; Option 1A: slice-based; Option 1B: GOP-based. I-stream; P-stream; I-stream; P-stream. Packet modelling; Slice-level; Frame-level. Traffic pattern; Both streams are periodic at 60 fps with the same jitter model as for single stream.; Follow the GOP structure, where GOP size K = 8 with the same jitter model as for single stream.. Number of packets per stream at a time; 1; N-1; I-frame: 1 or 0 P-frame: 0 or 1 At each time instant, there is either only one I-stream packet or only one P-stream packet. N = 8: the number of slices per frame.. Average data rate per stream; RI=R*αN-1+α; RP=R*N-1N-1+α; RI=R*αK-1+α; RP=R*K-1K-1+α. - R: average data rate of a single stream video - α: average size ratio between one I-frame/slice and one P-frame/slice - α = 1.5, 2 (baseline) - α = 3 (optional). Packet size distribution; Truncated Gaussian distribution. Mean = RIFPS; Mean = RPFPS*(N-1); Mean = RI*KFPS; Mean = RP*KFPS*(K-1). - [STD, Max, Min]: [10.5, 150, 50]% of Mean packet size - FPS is the frame rate of the single stream video. Depends on application, see 6.3.1, 6.4.1, 6.5.1 for VR, CG, AR respectively.. PDB; Depends on application, see 6.3.1, 6.4.1, 6.5.1 for VR, CG, AR respectively.. #### 5.1.2.2 Option 2 (video + audio/data) For Option 2, two streams (video + audio/data) are modelled. - Stream 1: video - Stream 2: audio/data The stream 1 - video stream follows the generic single stream model given in clause 5.1.1. The stream 2 - audio/data a periodic traffic with following parameters. Table 5.1.2.2-1: Statistical parameter values for Option 2 multi streams model Parameters; unit; Baseline values for evaluation; Optional values for evaluation. Periodicity P; ms; 10. Data rate: R; Mbps; 0.756, 1.12. Packet size; byte; R×1e6 × P /1000 / 8. PDB; ms; 30; Other values can be optionally evaluated. Packet Success Rate; %; 99; 99.9. #### 5.1.2.3 Option 3 (FOV + omnidirectional view) For Option 3, following two streams are modelled. - Stream 1: FOV - Stream 2: omnidirectional view stream The detailed modelling of the two streams is left to company with the report of evaluation results. ## 5.2 Generic UL pose/control traffic In this clause, we provide the generic UL pose/control stream traffic model. A packet for UL pose/control arrives at UE periodically with following parameters. Table 5.2-1: Statistical parameters for the UL pose/control traffic Parameters; unit; Baseline values for evaluation; Optional value for evaluation. Periodicity; ms; 4; Other values can be optionally evaluated.. Jitter; ms; No jitter. Packet size; byte; 100. PDB; ms; 10. Packet Success Rate X; %; 99; 90, 95.

- Instant message (as in TR 38.840)

Traffic model used for the UE power saving scheme evaluation - Applications with the traffic model for the evaluation of the UE power saving scheme - FTP - FTP model 3 - Other bursty traffic arrival models can be considered - Web-browsing - Video streaming - Instant messaging - VoIP - Gaming - Background app sync For FTP, instant messaging, and VoIP application, the following traffic models and DRX configuration should be included for evaluation: FTP traffic; Instant messaging; VoIP. Model; FTP model 3; FTP model 3; As defined in R1-070674. Assume max two packets bundled.. Packet size; 0.5 Mbytes; 0.1 Mbytes. Mean inter-arrival time; 200 ms; 2 sec. DRX setting; Period = 160 ms Inactivity timer = 100 ms; Period = 320 ms Inactivity timer = 80 ms; Period = 40 ms Inactivity timer = 10 ms. Note: For ON duration setting, following reference DRX configurations as previously agreed. - For web-browsing, video streaming, and gaming applications, the traffic models and the delay requirements defined in R1-070674 can be used in the evaluation. The parameters (e.g. packet size) may be updated to be in line with EMBB traffic requirements. - For background app sync application, for power consumption evaluation purpose, it can be assumed that idle mode operations (inclusive of page detection, RRM, deep sleep and transition overhead) contributes to X% of the use case power. The remaining portion is contributed by intermittent RRC connections due to background activities (FFS: value of X) - Companies should report the assumptions made in the evaluation

# Appendix - Agreements

## Agreements from RAN1#122

Agreement

- The deployment scenarios in TR38.914 should be considered for evaluation assumption

- The common evaluation assumptions including the antenna modelling, general system-level simulation assumptions (including the carrier frequency, bandwidth and subcarrier spacing used for link-level simulation) for the deployment scenarios in TR38.914, link budget and traffic models will be discussed in AI 11.2

  - Other assumptions including for link-level simulation specific to each technical topic will be separately discussed under each individual agenda.

  - Note: Subcarrier spacing decision is up to AI 11.3.2.

Conclusion

- Template in R1-2506582 is to be used for collecting inputs from companies.

  - Additional NTN or TN assumptions, if any, or any necessary change of the parameters, are to be incorporated into the updated one of R1-2506582.

Agreement

- Study which of the following traffic models are to be used for 6G evaluations, e.g.,

  - Full buffer

  - FTP Model 1 (in TR 36.814)

  - FTP Model 2 (in TR 36.814)

  - FTP Model 3 (in TR 36.872)

  - XR Traffic models (in TR 38.838)

  - VoIP model (as in TR 36.814)

  - Instant message (as in TR 38.840)

- Study whether to introduce the following traffic models for 6G evaluations considering, e.g.,

  - FTP-3 variant with packet delay budget requirement

    - Details FFS

  - New traffic model considering a mixed/variable packet size and the associated time domain behaviors (e.g., time between adjacent packet arrivals, packet delay budget)

    - Details FFS

  - New traffic model(s) considering the new use cases or services, e.g., AI/ML services, immersive communication services, etc.

    - Details FFS

- Study whether to introduce new/additional approaches that can reflect the impact of bidirectional traffic flows on performance metrics (e.g., impact of UL TCP ACK latency on DL throughput/latency)

Note: Whether/how to consider the combination of traffic model and loading level will be studied under individual agendas.

## Agreements from RAN1#122bis

Agreement

For around 700MHz, for TXRU mapping at base station, it is adopted as mandatory option for simulation campaign that a single TXRU is mapped per panel per subarray per polarization.

Note: Companies can provide results optionally, assuming fully connected TXRU mapping within a panel per polarization.

Agreement

- For around 700MHz, 32 for total number of antenna element at base station, 4 for total number of TXRU at base station, (8, 2, 2, 1, 1; 1, 2) for (M,N,P,Mg,Ng; Mp, Np), and (0.5, 0.5)λ for (dH,dV) are assumed as the baseline combination.

- For around 700MHz, 64 for total number of antenna element at base station, 8 for total number of TXRU at base station, (8, 4, 2, 1, 1; x, y) for (M,N,P,Mg,Ng; Mp, Np), and (0.5, 0.5)λ for (dH,dV) are assumed as the optional combination.

Note: Other values/combinations are up to company to report

Agreement

For around 2GHz carrier frequency, for BS antenna modelling

| BS antenna modelling | Total number of antenna elements | Total number of TXRU | (M, N, P, Mg , Ng; Mp, Np) | (dH,dV) |
| --- | --- | --- | --- | --- |
| Indoor |  |  |  |  |
| Combination 1(Optional) | 8 | 4 | (2, 2, 2, 1, 1; 1, 2) | (0.5, 0.5)λ |
| Combination 2 (Baseline) | 32 | 8 | (4, 4, 2, 1, 1; 1, 4) | (0.5, 0.5)λ |
| Outdoor |  |  |  |  |
| Combination 1(Optional) | 32 | 4 |  | (0.5, 0.8)λ |
| Combination 2 (Baseline) | 192 | 64 | (12, 8, 2, 1, 1; 4, 8) | (0.5, 0.5)λ |
| Note1: A single TXRU is mapped per panel per subarray per polarization as mandatory option. Companies can provide results optionally, assuming fully connected TXRU mapping within a panel per polarization. Note2: Other combinations used in the simulation results are up to company to report. |  |  |  |  |

Conclusion

The following existing traffic models could be used for 6GR performance evaluations,

- Full buffer

- FTP Model 1 (in TR 36.814)

- FTP Model 3 (in TR 36.872)

- XR Traffic models (in TR 38.838)

- VoIP model (as in TR 36.814)

- Instant message (as in TR 38.840)

- Note that which model(s) will be used can be further decided when performing simulations in each individual topic.

Agreement

For the study traffic model(s) for 6GR AI/ML services:

  - A representative AI/ML service is the generative AI, e.g., as defined in TR22.870.

Send LS to SA4 (cc RAN2, SA1, SA2) requesting input if any on traffic characteristics for AI/ML services.

Note: RAN1 is discussing the following options for the model:

  - Option-1a: The model is parameterized by Token, e.g., Token size, Token arrival rate, and Token delay budget.

    - Token is the minimum unit of data generated in the application layer.

    - How to associate Tokens to PHY layer packets.

    - How to reflect the variable importance of tokens.

    - Whether other parameters are additionally needed when tokens are encapsulated together into a packet, e.g., packet arrival rate, packet success rate, and packet delay.

  - Option-1b: The model is characterized by the parameters of PHY layer packet, including e.g., packet size, arrival rates, latency requirement, reliability requirement, etc.

  - Option-1c: reusing or extending the FTP-3/XR traffic model.

  - FFS other models/options need to be defined for other AI/ML services.

Agreement

Study traffic modelling for evaluations related to immersive communication services including but not limited to advanced XR [e.g., TR22.870] and haptics services,

- XR traffic models (in TR 38.838) are considered as starting point.

  - FFS the detailed modifications on the parameters to the XR traffic model, e.g., higher packet size, higher packet arrival rate, higher packet size deviation, PDB, etc.

- FFS how many models need to be defined and the corresponding representative use cases.

- FFS how to incorporate haptics traffic (TR26.854).

Send LS to SA4 requesting input if any on the relevant traffic characteristics, RAN1 can continue the study before SA4 potential response.

Agreement

Study extensions to FTP Model 1/FTP Model 3 to incorporate the following:

- Multiple packet sizes and associated time-domain behaviors (e.g., inter arrival time)

  - FFS number of packet sizes (e.g., 2 or 3).

  - FFS whether to have fixed or variable packet size and packet arrival rate for a given UE.

  - FFS applicability of multiple packet sizes to only one or both of FTP Model 1/FTP Model 3.

  - FFS packet size and arrival rate characteristics.

- Packet delay budget (PDB) related parameters

  - FFS PDB applicability to packets (e.g., one PDB parameter for only one traffic flow or different PDB parameters for different traffic flows).

  - FFS how to consider the PDB, e.g., whether to drop packets when exceeding the budget, PDB aware metric.

- Note consider the following for PDB:

  - Applicability to the extension to FTP Model 1/ FTP Model 3 with one packet size.

  - Applicability or not to the extension to FTP Model 1/ FTP Model 3 with multiple packet sizes.

Agreement

The attached templates for NTN in R1-2507956 are endorsed in principle.

Agreement

The following configurations for system-level simulations could be used for 6GR evaluation:

|  | Indoor Hotspot | Dense Urban | Rural | Urban Macro | Sub-urban macro |
| --- | --- | --- | --- | --- | --- |
| Carrier frequency | Around 2 GHz Around 4 GHz Around 7 GHz Around 15 GHz Around 30 GHz | Around 700 MHz Around 2 GHz Around 4 GHz Around 7 GHz Around 15 GHz Around 30 GHz | Around 700 MHz Around 2 GHz Around 4 GHz Around 7 GHz | Around 700 MHz Around 2 GHz Around 4 GHz Around 7 GHz Around 15 GHz Around 30 GHz | Around 700 MHz Around 2 GHz Around 4 GHz Around 7 GHz Around 15 GHz Around 30 GHz |
| Aggregated BW | Follow system bandwidth per carrier frequency in TR 38.914 as 1) Around 700 MHz: Up to 60 MHz 2) Around 2GHz: Up to 200 MHz 3) Around 4GHz: Up to 300 MHz 4) Around 7GHz: Up to 400MHz 5) Around 15GHz: Up to 400MHz 6) Around 30GHz: Up to 1GHz |  |  |  |  |
| Simulation BW | Around 700 MHz: 20MHz, 60MHz |  |  |  |  |
|  | Around 2 GHz: 20MHz, 100MHz, 200MHz |  |  |  |  |
|  | Around 4 GHz: 20MHz, 100MHz, 200MHz, 300MHz |  |  |  |  |
|  | Around 7 GHz: 20MHz, 100MHz, 200MHz, 400MHz |  |  |  |  |
|  | Around 15 GHz: 20MHz, 100MHz, 200MHz, 400MHz |  |  |  |  |
|  | Around 30GHz: 100MHz, 400MHz, 800MHz |  |  |  |  |
|  | Note: other simulation BW could be considered. |  |  |  |  |
| Note: The layout for each scenario will be separately discussed, including the carrier frequency combination for single layer and/or two layers. |  |  |  |  |  |

Agreement

Draft LS R1-2508183 is endorsed in principle.

Agreement

Final LS R1-2508184 is endorsed.

Agreement

For link budget template, consider the following candidates:

- Candidate 1: Reusing the link budget template from TR38.830, i.e., the following table with notes as follows:

- The values of the parameters are TBD.

- MCL in row (22bis) is TBD.

- FFS: whether/how/why to update

| System configuration |  |
| --- | --- |
| Channel for evaluation |  |
| Scenarios and Carrier frequency (GHz) |  |
| BS antenna heights (m) |  |
| UT antenna heights (m) |  |
| Cell area reliability (%) |  |
| Lognormal shadow fading std deviation (dB) |  |
| Tx Diversity |  |
| Number of SSB |  |
| Transmitter |  |
| (1) Number of transmit antenna elements |  |
| (2) Number of transmit TxRUs Note: this row is void (left empty) for uplink |  |
| (2a) Number of transmit chains modelled in LLS |  |
| (3) Total transmit power (dBm) Note: total transmit power for system bandwidth |  |
| (3a) System bandwidth for downlink, or occupied bandwidth for uplink (Hz) |  |
| (3b) Power Spectrum Density = (3) - 10 log( (3a) / 1000000 ) (dBm/MHz) Note: no PSD constraint for uplink |  |
| (3c) Bandwidth used for the evaluated channel (Hz) Note: (3c) is identical to the number of PRBs assigned to the channel evaluated. For uplink, (3a) = (3c) |  |
| (3bis) Total transmit power for occupied bandwidth = (3b) + 10 log ((3c) /1000000) (dBm) |  |
| (4) Total antenna gain at antenna gain component 3 & antenna gain component 4 of transmitter = (4a) – (4b) (dB) |  |
| (4a) Antenna gain at antenna gain component 3 & antenna gain component 4 of transmitter = (4c) + 10 log ((1) / (2)) (dB) for downlink, and = (4c) + 10 log ((1) / (2a)) (dB) for uplink |  |
| (4b) Antenna gain correction factor at antenna gain component 3 & antenna gain component 4 of transmitter (dB) |  |
| (4c) Gain of antenna element (dBi) |  |
| (5) Total antenna gain at antenna gain component 2 of transmitter = (5a) - (5b) (dB) Note: zero for uplink |  |
| (5a) Antenna gain at antenna gain component 2 of transmitter = 10 log((2)/(2a)) (dB) Note: zero for uplink |  |
| (5b) Antenna gain correction factor at antenna gain component 2 of transmitter (dB) Note: zero for uplink |  |
| (8) Cable, connector, combiner, body losses, etc. (enumerate sources) (dB) (feeder loss must be included for and only for downlink) |  |
| (9) EIRP = (3bis) + (4) + (5) – (8) dBm |  |
| Receiver |  |
| (10) Number of receive antenna elements |  |
| (10a) Number of receive TxRUs Note: this row is void (empty) for downlink |  |
| (10b) Number of receive chains modelled in LLS |  |
| (11) Total antenna gain at antenna gain component 3 & antenna gain component 4 of receiver = (11a) - (11b) (dB) |  |
| (11a) Antenna gain at antenna gain component 3 & antenna gain component 4 of receiver = (11c) + 10 log ((10)/(10a)) (dB) for uplink = (11c) + 10 log ((10)/(10b)) (dB) for downlink |  |
| (11b) Antenna gain correction factor at antenna gain component 3 & antenna gain component 4 of receiver (dB) |  |
| (11c) Gain of antenna element (dBi) |  |
| (11bis) Total antenna gain at antenna gain component 2 of receiver = (11bis-a) - (11bis-b) (dB) Note: zero for downlink |  |
| (11bis-a) Antenna gain at antenna gain component 2 of receiver = 10 log((10a)/(10b)) (dB) Note: zero for downlink |  |
| (11bis-b) Antenna gain correction factor at antenna gain component 2 of receiver (dB) Note: zero for downlink |  |
| (12) Cable, connector, combiner, body losses, etc. (enumerate sources) (dB) (feeder loss must be included for and only for uplink) |  |
| (13) Receiver noise figure (dB) |  |
| (14) Thermal noise density (dBm/Hz) |  |
| (15) Receiver interference density (dBm/Hz) |  |
| (16) Total noise plus interference density = 10 log (10^(( (13) + (14))/10) + 10^((15)/10)) (dBm/Hz) |  |
| (18) Effective noise power = (16) + 10 log ((3c)) (dBm) |  |
| (19) Required SNR (dB) |  |
| (20) Receiver implementation margin (dB) |  |
| (21) H-ARQ gain (dB) Note: Only applicable if HARQ is not considered in LLS |  |
| (22) Receiver sensitivity = (18) + (19) + (20) – (21) (dBm) |  |
| (22bis) MCL = (3bis) – (22) + (5) + (11bis) (dB) |  |
| (23) Hardware link budget, a.k.a. MIL = (9) + (11) + (11bis) − (12) − (22) (dB) Note: MIL can also be derived by (22bis) + (4) – (8) + (11) − (12) |  |
| Calculation of available pathloss |  |
| (25) Shadow fading margin (function of the cell area reliability and lognormal shadow fading std deviation) (dB) |  |
| (26) BS selection/macro-diversity gain (dB) |  |
| (27) Penetration margin (dB) |  |
| (28) Other gains (dB) (if any please specify) |  |
| (29) Available path loss = (23) – (25) + (26) – (27) + (28) (dB) |  |
| Range/coverage efficiency calculation |  |
| FFS: (30) Maximum range (based on (29) and according to the system configuration section of the link budget) (m) |  |

- Candidate 2: Template as Table 7.10.1-1 from TR38.913.

  - FFS: whether/how/why to update.

| Item | Value |
| --- | --- |
| Transmitter |  |
| (1) Tx power (dBm) |  |
| Receiver |  |
| (2) Thermal noise density (dBm/Hz) |  |
| (3) Receiver noise figure (dB) |  |
| (4) Interference margin (dB) |  |
| (5) Occupied channel bandwidth (Hz) |  |
| (6) Effective noise power = (2) + (3) + (4) + 10 log(5) (dBm) |  |
| (7) Required SINR (dB) |  |
| (8) Receiver sensitivity = (6) + (7) (dBm) |  |
| (9) MaxCL = (1) - (8) (dB) |  |

Agreement

For around 4GHz carrier frequency:

| BS antenna modelling | Total number of antenna elements | Total number of TXRU | (M, N, P, Mg, Ng; Mp, Np) | (dH,dV) |
| --- | --- | --- | --- | --- |
| Indoor |  |  |  |  |
| Combination 1 | 32 | 32 | (4, 4, 2, 1, 1; 4, 4) | (0.5, 0.5)λ |
| Combination 2 | 128 | 32 | (8, 8, 2, 1, 1; 2, 8) | (0.5, 0.5)λ |
| Combination 3 | 256 | 64 | (16, 8, 2, 1, 1;4, 8) | (0.5, 0.5)λ |
| Outdoor |  |  |  |  |
| Combination 1 | 192 | 64 | (12, 8, 2, 1, 1; 4, 8) | (0.5, 0.8)λ |
| Combination 2 | 256 | 64 | (16, 8, 2, 1, 1; 4, 8) | (0.5, 0.8)λ |
| Combination 3 | 512 | 128 | (16, 16, 2, 1, 1; 4, 16) | (0.5, 0.5)λ |
| Note1: A single TXRU is mapped per panel per subarray per polarization as mandatory option. Companies can provide results optionally, assuming fully connected TXRU mapping within a panel per polarization. Note2: Other combinations used in the simulation results are up to company to report. |  |  |  |  |

For around 7GHz carrier frequency:

| BS antenna modelling | Total number of antenna elements | Total number of TXRU | (M, N, P, Mg, Ng; Mp, Np) | (dH,dV) |
| --- | --- | --- | --- | --- |
| Indoor |  |  |  |  |
| Combination 1 | 64 | 32 | (4, 8, 2, 1, 1; 2, 8) | (0.5, 0.5)λ |
| Combination 2 | 256 | 64 | (16, 8, 2, 1, 1; 4, 8) | (0.5, 0.5)λ |
| Combination 3 | 512 | 128 | (16, 16, 2, 1, 1; 8, 8) | (0.5, 0.5)λ |
| Outdoor |  |  |  |  |
| Combination 1 | 768 | 128 | TBD | (0.5, 0.8)λ |
| Combination 2 | 1024 | 256 | (32, 16, 2, 1, 1; 8, 16) | (0.5, 0.8)λ |
| Combination 3 | 1536 | 256 | TBD | (0.5, 0.8)λ |
| Combination 4 | 2048 | 256 | (32, 32, 2, 1, 1, 8, 16) | (0.5, 0.5)λ |
| Combination 5 | 2048 | 512 | (64, 16, 2, 1, 1; 16, 16) | (0.5, 0.5)λ |
| Note1: A single TXRU is mapped per panel per subarray per polarization as mandatory option. Companies can provide results optionally, assuming fully connected TXRU mapping within a panel per polarization. Note2: Other combinations used in the simulation results are up to company to report. |  |  |  |  |

For around 30GHz carrier frequency:

| BS antenna modelling | Total number of antenna elements | Total number of TXRU | (M, N, P, Mg, Ng; Mp, Np) | (dH,dV) |
| --- | --- | --- | --- | --- |
| Indoor |  |  |  |  |
| Combination 1 | 128 | 8 | (4, 4, 2, 2, 2; 1, 1) | (0.5, 0.5)λ |
| Combination 2 | 512 | 8 | (8, 8, 2, 2, 2; 1, 1) | (0.5, 0.5)λ |
| Combination 3 | 1024 | 8 | (16, 8, 2, 2, 2; 1, 1) | (0.5, 0.5)λ |
| Combination 4 | 768 | 2 | (24, 16, 2, 1, 1; 1, 1) | (0.5, 0.5)λ |
| Outdoor |  |  |  |  |
| Combination 1 | 2048 | 16 | (16, 8, 2, 4, 2; 1, 1) | (0.5, 0.5)λ |
| Combination 2 | 4096 | 32 | (16, 8, 2, 4, 4; 1, 1) | (0.5, 0.5)λ |
| Combination 3 | 1024 | 4 | (16, 16, 2, 2, 1; 1, 1) | (0.5, 0.5)λ |
| Note1: A single TXRU is mapped per panel per polarization as mandatory option. Companies can provide results optionally, assuming a single TXRU is mapped per panel per subarray per polarization as mandatory option. Note2: Other combinations used in the simulation results are up to company to report. |  |  |  |  |

Agreement

At least the following carrier frequencies could be considered (from RAN1 perspective) for 6GR NTN evaluations:

- L-band (i.e., 1.5GHz)

- S-band (i.e. 2 GHz)

- Ku-band (FFS detailed frequency range)

- Ka-band (i.e. 30 GHz for UL, 20GHz for DL)

## Agreements from RAN1#123

Agreement

Updating the BS antenna modelling agreed in the last meeting as follows:

- For around 700MHz carrier frequency, for BS antenna modelling,

- update the (8, 4, 2, 1, 1; x, y) to be (8, 4, 2, 1, 1; 1, 4).

- For around 2GHz carrier frequency, for BS antenna modelling,

- for outdoor combination 1 (i.e., 32AE/4TXRU), update the (M,N,P,Mg,Ng; Mp,Np) to be (8, 2, 2, 1, 1; 1, 2)

- For around 7GHz carrier frequency, for BS antenna modelling,

- for outdoor combination 1 (i.e., 768AE/128TXRU), update the (M,N,P,Mg,Ng; Mp,Np) to be (24, 16, 2, 1, 1; 4, 16).

- for outdoor combination 3 (i.e., 1536AE/256TXRU), update the (M,N,P,Mg,Ng; Mp,Np) to be (48, 16 ,2, 1, 1; 8, 16).

Agreement

For 6GR evaluation, the layout for system-level simulation is assumed as follows:

- Note: Single layer will be prioritized for the evaluations.

- Note: The carrier frequency for the corresponding layout for the two layers will be reported by companies for the evaluations.

- FFS the minimum distance for random drop in two layers.

- Note: for system-level simulation of MIMO schemes, specific assumptions could be discussed under MIMO discussion

| Parameters | Indoor Hotspot | Dense Urban | Rural | Urban Macro | Suburban Macro |
| --- | --- | --- | --- | --- | --- |
| Layout | Single layer - Indoor floor (Open office), (Room size: 120m x 50m) | Single layer: - Hex. Grid Two layers: - Macro layer: Hex. Grid - Micro layer: Random drop | Single layer: - Hex. Grid | Single layer: - Hex. Grid Two layers: - Macro layer: Hex. Grid - Micro layer: Random drop | Single layer: - Hex. Grid |

Agreement

For 6GR evaluation, the total transmit power per BS for system-level simulation is assumed as follows:

| Total transmit power per BS | Indoor Hotspot | Dense Urban | Rural | Urban Macro | Sub-urban macro |
| --- | --- | --- | --- | --- | --- |
| Around 700MHz | NA | Macro BS: - Option1: 44 dBm per 20 MHz - Option2: 49 dBm per 20 MHz Micro BS: 33 dBm per 20 MHz | Macro BS: 49 dBm per 20 MHz | Macro BS: 49 dBm per 20 MHz Micro BS: 33 dBm per 20 MHz | Macro BS: 49 dBm per 20 MHz |
| Around 2GHz | 24 dBm per 20 MHz | Macro BS: - Option1: 44 dBm per 20 MHz - Option2: 46 dBm per 20 MHz Micro BS: 33 dBm per 20 MHz | Macro BS: 49 dBm per 20 MHz | Macro BS: - Option1: 49 dBm per 20 MHz - Option2: 46 dBm per 20 MHz Micro BS: 33 dBm per 20 MHz | Macro BS: 49 dBm per 20 MHz |
| Around 4GHz | 24 dBm per 20 MHz | Macro BS: - Option1: 44 dBm per 20 MHz Micro BS: 33 dBm per 20 MHz | Macro BS: 49 dBm per 20 MHz | Macro BS: - Option1: 49 dBm per 20 MHz - Option2: 44 dBm per 20 MHz - Option3: 46 dBm per 20 MHz Micro BS: 33 dBm per 20 MHz | Macro BS: 49 dBm per 20 MHz |
| Around 7GHz | 24 dBm per 20 MHz | Macro BS: - Option1: 44 dBm per 20 MHz - Option2: 43 dBm per 20 MHz Micro BS: 33 dBm per 20 MHz | Macro BS: 49 dBm per 20 MHz | Macro BS: - Option1: 49 dBm per 20 MHz - Option2: 43 dBm per 20 MHz - Option3: 46 dBm per 20 MHz Micro BS: 33 dBm per 20 MHz | Macro BS: 49 dBm per 20 MHz |
| Around 15GHz | 23dBm per 20MHz | Macro BS: 43dBm per 20MHz Micro BS: 33 dBm per 20 MHz | NA | - Macro BS: 43dBm per 20MHz - Micro BS: 33 dBm per 20 MHz | Macro BS: 43dBm per 20MHz |
| Note: For evaluation purpose, BS Tx power scales up with bandwidth proportionally under the limitation of the maximum BS Tx power is 56dBm for outdoor and 33dBm for indoor for the above carrier frequencies. Note: The values defined in option1 refer to the Report ITU-R M. [IMT-2030. EVAL]. The values defined in option2 is calculated based on the proportional scaling with simulation bandwidth under the limitation of the maximum BS Tx power of 56dBm. |  |  |  |  |  |
| Around 30GHz | - Option1: 23 dBm per 20 MHz - Option2: 16dBm per 20MHz. | Micro BS: 33 dBm per 20 MHz | NA | Micro BS: 33 dBm per 20 MHz | Macro BS: 33 dBm per 20 MHz |
| Note: For evaluation purpose, for around 30GHz, BS Tx power scales up with bandwidth proportionally under the limitation of EIRP 75dBm. |  |  |  |  |  |

Agreement

For FTP Model 3, the packet delay budget (PDB) can be additionally considered,

- The latency characteristic of the traffic in RAN side (i.e., air interface) is modelled as packet delay budget (PDB). The PDB is a limited time budget for a packet to be transmitted over the air from a BS to a UE for DL, or from a UE to a BS for UL.

- For a given packet, the delay of the packet incurred in air interface is measured from the time that the packet arrives at the BS to the time that it is successfully transferred to the UE for DL, or from a UE to a BS for UL. If the delay is larger than a given PDB for the packet, the packet is said to violate PDB, otherwise the packet is said to be successfully delivered.

- Values for PDB, e.g., {10ms, 20ms, 30ms, 50ms, 100ms, 200ms, 300ms, 1000ms, 2000ms} can be considered.

- Which values will be used will consider the use case for the evaluations.

Agreement

For 6GR evaluations related to Massive Communication (IoT),

- For comparability with 5G results and verify that 6G can meet the IMT-2030 connection density requirements, the mMTC traffic model from IMT-2020 (TR 37.910) may be used as a starting point. This traffic model can be applied in UL or DL.

- FFS: necessity of new traffic model(s) for 6GR evaluation in RAN1, e.g., for the following traffic types.

  - Triggered/polled reporting

  - Autonomous reporting (event-driven or periodic)

  - Remote actuation

  - Firmware/software upgrade

Working Assumption

For 6GR evaluations related to immersive communications services, the following two amended XR models based on the existing XR traffic model (in TR 38.838) can be considered:

- Model-1: eXR model without Haptics

  - Regarding the statistical parameters for single stream CG traffic model defined in Table 5.4.1-1 TR 38.838, add values for immersive gaming regarding the data rate and the frame generation rate as in red:

| Parameters | unit | Baseline values for evaluation | Optional values for evaluation | Values for immersive gaming |
| --- | --- | --- | --- | --- |
| data rate: R | Mbps | 30, 8 | 45 | 100, 300, 500 |
| frame generation rate: F | fps or Hz | 60 |  | 90,120 |
| PDB | ms | 15 | 10, 30 | 15, or 10, 30 |

  - Regarding the statistical parameters for packet size following truncated Gaussian distribution in Table 5.1.1.1-1 TR 38.838, add values for immersive gaming regarding STD, Max, and Min values as in red:

| Parameter | unit | Baseline values for evaluation | Optional values for evaluation for single eye buffer | Values for immersive gaming |
| --- | --- | --- | --- | --- |
| Mean: M | byte | R×1e6 / F / 8 | R×1e6 / F / 8 | R×1e6 / F / 8 |
| STD | byte | 10.5% of M | 3 % of M | [25 %] of M |
| Max | byte | 150% of M | 109% of M | 300% of M |
| Min | byte | 50% of M | 91% of M | 25% of M |
| R: data rate of the flow in Mbps. F: frame generation rate of the flow in fps. Note that the mean and STD apply before truncation applies. Note that the value of R, F depend on application. |  |  |  |  |

  - Regarding the statistical parameters for AR UL Model 1 defined in Table 5.5.2.1-1 TR 38.838, add values for UL-heavy video uploading regarding packet size, generate rate, data rate, and PDB values as in red:

| Parameters | unit | value | Values for UL video uploading |
| --- | --- | --- | --- |
| Packet size | byte | Follows clause 5.1.1.1 (i.e., mean packet size = R×1e6 / F / 8, STD/Min/Max=10.5/50/150%) | 1st candidate: Follows clause 5.1.1.1 (i.e., mean packet size = R×1e6 / F / 8, STD/Min/Max=10.5/50/150%) 2nd candidate: Follows clause 5.1.1.1 (i.e., mean packet size = R×1e6 / F / 8, STD/Min/Max=[25]/25/300%) |
| packet generation rate: F | Hz | 60 | 15, 30 |
| Jitter | ms | Optional, follows the description in clause 5.1.1.2 | Optional, follows the description in clause 5.1.1.2 |
| Data rate: R | Mbps | 10 (baseline), 20 (optional) | 20, 60, 100 |
| PDB | ms | 30 (baseline), 10 or 15 or 60 (optional) | 10, 15 |

  - The jitter is modelled the same as XR traffic model.

- Model-2: eXR model with Haptics

  - Haptics traffic is defined as XR traffic packet generation with co-generated haptics packets.

    - FFS on how to generate the multi-channel haptics packet including how to handle silent periods of haptics and the haptics packet sizes.

    - FFS on how to co-generate haptics packets and the XR traffic packets.

  - Haptics packets has packet delay budget (PDB) of either 12 msec or 30 msec, which can be selected as a traffic model parameter.

- Send LS to SA4 to inform about the above agreement and check if SA4 has related inputs for the model.

Note: whether the working assumption can be confirmed relies on SA4’s response

Agreement

For 6GR evaluation, the following are assumed for system-level simulation:

| Parameters | Indoor Hotspot | Dense Urban | Rural | Urban Macro | Suburban Macro |
| --- | --- | --- | --- | --- | --- |
| ISD | 20m, equivalent to 12TRxPs per 120m x 50m | Macro layer: 200m | ISD 1: 1732m ISD 2: 5000m | Macro: 500m | ISD 1: 1299m ISD 2: 1732m |
| BS antenna height | 3m | 25m for macro cells and 10m for micro cells | 35 m | 25m for macro cells and 10m for micro cells | Alt 1: 35m(baseline) Alt 2: 25m(optional) |
| BS noise figure | Around 7GHz and below: 5dB Around 15GHz and above: 7dB |  |  |  |  |
| UE antenna height | TR38.901 Indoor-Office Table 7.2-2 | TR38.901 UMi/UMa Table 7.2-1 | TR38.901 RMa Table 7.2-3 | TR38.901 UMa Table 7.2-1 | TR38.901 SMa Table 7.2-5 |
| UE noise figure | Around 7GHz and below: [7, 9]dB Around 15GHz and above: 13dB, 10dB |  |  |  |  |
| UE Receiver | MMSE-IRC as the baseline R-ML Receiver as optional (FFS modelling) |  |  |  |  |
| UE Power control parameter for UL | Company report |  |  |  |  |
| Channel model | TR 38.901 v19.1.0 Indoor-Office | TR 38.901 v19.1.0 UMa/UMi | TR 38.901 v19.1.0 RMa | TR 38.901 v19.1.0 UMa | TR 38.901 v19.1.0 SMa, 0% vegetation. |
| Numerology | In line with AI 11.3 |  |  |  |  |
| Scheduling | Proportional fairness (PF) |  |  |  |  |
| Inter-cell interference model | Explicitly and realistically modelled |  |  |  |  |
| Inter-cell interference estimation model | Alt 1: Ideal, calculated by ground truth channel matrix Alt 2: Realistic model, Company report, e.g., Wishart distribution-based model; retain only diagonal elements of interference Cov. Matrix. |  |  |  |  |
| Channel estimation assumption | Alt 1: Ideal for benchmark Alt 2: Realistic, company report, e.g., direct/explicit RS estimation, apply gauss noise to real channel matrix, or random |  |  |  |  |
| Feedback assumption | Alt 1: Ideal Alt 2: Realistic, company report, e.g., consider feedback delay and overhead; codebook; |  |  |  |  |
| O2I penetration loss (X% high loss, Y% low loss) | NA | Two options are supported: Option 1: 80% low loss, 20% high loss; Option 2: 50% low loss, 50% high loss | 100% low loss | Two options are supported: Option 1: 80% low loss, 20% high loss; Option 2: 50% low loss, 50% high loss | Option 1: 100% Low-loss A Model as TR38.901. Option 2: 50% Low-loss A 50% Low-loss Model as TR38.901. |
| Mechanic tilt | 180° in GCS (pointing to the ground) as baseline | 90° in GCS (pointing to horizontal direction) as baseline. Company can report if not follow the baseline. | 90° in GCS (pointing to horizontal direction) as baseline. Company can report if not follow the baseline. | 90° in GCS (pointing to horizontal direction) as baseline. Company can report if not follow the baseline. | Baseline: 95° in GCS (pointing to horizontal direction) for ISD = 1299m; 92° in GCS (pointing to horizontal direction) for ISD = 1732m; Company can report if not follow the baseline. |
| Electrical tilt | Company can report other values for evaluations. | Company can report other values for evaluations. | Company can report other values for evaluations. | Company can report other values for evaluations. | Company can report other values for evaluations. |
| Handover margin (dB) | 1dB as baseline. 0dB and 3dB as optional configuration. | 1dB as baseline. 0dB and 3dB as optional configuration. | 1dB as baseline. 0dB and 3dB as optional configuration. | 1dB as baseline. 0dB and 3dB as optional configuration. | 1dB as baseline. 0dB and 3dB as optional configuration. |
| UE attachment | Based on RSRP from BS |  |  |  |  |
| Wrapping around method | No wrapping around | Geographical distance-based wrapping | Geographical distance-based wrapping | Geographical distance-based wrapping | Geographical distance-based wrapping |
| Multi-TRP operation, e.g., ideal or non-ideal backhaul/sync | Backhaul: ideal or non-ideal; sync: ideal or non-ideal; Company reports the assumptions of the non-ideal backhaul/non-ideal sync. |  |  |  |  |

Agreement

- For FTP3 extension with multiple packet sizes (the number of packet size X =FFS: 2 or 3), FTP 3-extension 1

  - For each packet size S_i, the packets arrive according to Poisson distribution (as FTP 3) with mean inter-arrival time T_i  (or arrival rate λ_i where T_i = 1/ λ_i)

  - Y packet sizes are simulated for each UE

    - Down-select one from following

      - Alt1: Y=1; X=e.g., 2 or 3

      - Alt2: Y=X; X=e.g., 2 or 3

      - Alt3: Either Alt1 or Alt2 can be used depending on the evaluation purpose

  - FFS: values of S_i and T_i, and their inter-relation (if any)

  - FFS: change “packet size” to “File size” (terminology)

  - FFS timing relationship for different packet sizes if Y=X.

  - FFS the number of UEs for each of X different sizes in a drop if Y=1.

- Note: PDB can be considered separately if needed

- Note: modeling sessions with multiple packets in each session can be discussed separately if needed.

- Down-selection between X=2 and 3.

Agreement

For traffic model(s) for AI/ML services, the following can be considered:

- Packet size:

  - How to model the packet size, a fixed one or multiple values, or modelled as a random variable.

- Packet arrival:

  - FFS the details to determine the packet arrival rate, e.g.,

    - N multiple packets arrive together as a burst. The burst interval time is modelled as a random variable.

      - Within the burst, the N packets arrive according to a statistical distribution.

    - Packets arrive separately.

  - FFS whether/how to model the Jitter and the relation with the packet arrival.

- FFS: Whether the packet importance is known. Whether/how to reflect the packet importance.

- Whether/How to consider the PDB, e.g., Packet delay budget: The latency characteristic of the traffic in RAN side (i.e., air interface) is modelled as packet delay budget (PDB). The PDB is a limited time budget for a packet to be transmitted over the air from a base station to a UE, or from a UE to a base station

- FFS Whether/how to consider the Packet success rate requirement: [xx%] and the relation with the PDB.

- FFS how to model different cases, e.g., image-based GenAI, video-based GenAI, and chatbot, etc.

- FFS: Whether/how other traffic models (e.g., XR, FTP1/3) can be used to reflect above characteristics.

Note: input from SA4 if any will be considered.

Agreement

For 6GR evaluations, RAN1 to consider BS antenna modelling for around 15GHz carrier frequency as follows:

| BS antenna modelling | Total number of antenna elements | Total number of TXRU | (M, N, P, Mg, Ng; Mp, Np) | (dH,dV) |
| --- | --- | --- | --- | --- |
| Indoor |  |  |  |  |
| Combination 1 | 512 | 128 | (16, 16, 2, 1, 1; 8, 8) | (0.5, 0.5)λ |
| Combination 2 | 128 | 8 | (4, 4, 2, 2, 2; 1, 1) | (0.5, 0.5)λ |
| Outdoor |  |  |  |  |
| Combination 1 | 2048 | 256 | (32, 32, 2, 1, 1; 4, 32) | (0.5, 0.5)λ |
| Combination 2 | 2048 | 128 | (32, 32, 2, 1, 1; 4, 16) | (0.5, 0.5)λ |
| Combination 3 | 2048 | 32 | (8, 8, 2, 4, 4; 1, 1) | (0.5, 0.5)λ |
| Note1: A single TXRU is mapped per panel per subarray per polarization for Indoor combination 1 and Outdoor combination 1 and combination 2. A single TXRU is mapped per panel per polarization for Indoor combination 2 and outdoor combination 3. Note2: Other combinations used in the simulation results are up to company to report. |  |  |  |  |

## Agreements from RAN1#124