3GPP TSG-RAN WG1 #124	<No TDoc Number>

Gothenburg, Sweden, 09 – 13 February, 2026

Source:	Moderator (Nokia)

Title:	Feature Lead summary #1 on 6G waveform

Document for:	Discussion

Agenda item:	10.2.1

Work Item:	FS_6G_Radio

# Introduction

37 Tdocs were submitted (including one submitted to the top-level agenda item 10.2 and one submitted a week after the submission deadline) with a total of 577 pages, of which consist of 266 observations and 239 proposals spanning 35 pages. This contribution acts as a platform to facilitate discussions on various topics raised in these conributions.

Tentative schedule for waveform sessions (v01):

- [Offline] Tuesday 11:00-11:40 (40min)

- [Online] Tuesday 12:00-13:00 (60min)

- [Online] Wednesday 9:30-10:30 (60min)

- [Offline] Wednesday 15:50-16:30 (40min)

- [Online] Thursday 14:30-15:30 (60min)

- [Offline] Thursday 17:40-18:30 (50min)

- [Online] Friday 9:30-10:30 (60min)

# References

|  | Tdoc# | Title | Source |
| --- | --- | --- | --- |
| [1] | R1-2600027 | On remaining aspects of waveform for 6GR | Nokia |
| [2] | R1-2600138 | Waveform for 6GR air interface | Huawei, HiSilicon |
| [3] | R1-2600188 | On waveform enhancements/additions for 6G Radio | OPPO |
| [4] | R1-2600239 | Discussion on waveform for 6GR | LG Electronics |
| [5] | R1-2600255 | Discussion on waveform for 6GR air interface | THALES |
| [6] | R1-2600261 | Views on the waveform for 6G | ZTE Corporation, Sanechips |
| [7] | R1-2600295 | Discussions on waveform for 6GR | CATT |
| [8] | R1-2600366 | Waveform design for 6G air interface | Tejas Network Limited |
| [9] | R1-2600384 | Discussion on the waveform design for 6G radio | CMCC |
| [10] | R1-2600424 | Further discussion on 6GR waveform | Xiaomi |
| [11] | R1-2600499 | Discussion on Waveform for 6GR air interface | vivo |
| [12] | R1-2600572 | Discussion on Waveform for 6GR Air Interface | IMU, Turkcell |
| [13] | R1-2600584 | Discussion on 6G Waveform | NEC |
| [14] | R1-2600612 | Waveform for 6GR Air Interface | Cohere Technologies |
| [15] | R1-2600627 | Waveform for 6GR Air Interface | Google |
| [16] | R1-2600716 | Discussions on 6G Waveforms | Lekha Wireless Solutions |
| [17] | R1-2600751 | Discussion on waveform for 6GR | Samsung |
| [18] | R1-2600801 | Waveform for 6GR air interface | InterDigital, Inc. |
| [19] | R1-2600823 | On Waveforms for 6GR air interface | Apple |
| [20] | R1-2600909 | Waveform for 6GR air interface | MediaTek Inc. |
| [21] | R1-2600914 | Study on waveform for 6GR | Sharp |
| [22] | R1-2600999 | Discussion on 6GR waveform | ETRI, University of Surrey |
| [23] | R1-2601019 | Discussion on New Waveform for 6GR Air Interface | Shanghai Jiao Tong University, NERC-DTV |
| [24] | R1-2601047 | Discussion on 6GR waveform design | Hanbat National University |
| [25] | R1-2601080 | Discussion on Waveform for 6GR | Lenovo |
| [26] | R1-2601092 | Discussion on waveform for 6GR air interface | Ofinno |
| [27] | R1-2601110 | New waveform for 6GR air interface | NICT |
| [28] | R1-2601113 | Discussion on waveform for 6GR air interface | Panasonic |
| [29] | R1-2601127 | Waveforms for 6GR | Sony |
| [30] | R1-2601156 | On waveform for 6GR | Ericsson |
| [31] | R1-2601517 | Discussion on Waveform | NTT DOCOMO, INC |
| [32] | R1-2601212 | Discussion on waveform for 6GR air interface | Pengcheng Laboratory |
| [33] | R1-2601268 | Waveforms for 6GR | Qualcomm Incorporated |
| [34] | R1-2601294 | Discussion on waveform for 6G air interface | Quectel |
| [35] | R1-2601354 | Discussion on waveform for 6GR air interface | KDDI Corporation |
| [36] | R1-2601366 | Enhancements for pi/2-BPSK DFT-s-OFDM: Overlapped Allocations | Wisig Networks, IITH |
| [37] | R1-2600786 | Feature Lead summary #1 on 6G waveform | Nokia |
| [38] | R1-2600787 | Feature Lead summary #2 on 6G waveform | Nokia |
| [39] | R1-2600788 | Feature Lead summary #3 on 6G waveform | Nokia |
| [40] | R1-2600789 | Feature Lead summary #4 on 6G waveform | Nokia |
| [41] | R1-2601539 | New Waveform for 6GR - OSDM | R1-2601539 |

# Requirements and Baseline waveform

| [1] | R1-2600027 | On remaining aspects of waveform for 6GR | Nokia |
| --- | --- | --- | --- |
|  | Baseline communication waveform - Proposal 1: CP-OFDM waveform as defined in 5G NR is supported for communications in 6G downlink. - Enhancements/modifications on CP-OFDM will be studied as potential additions - DFT-s-OFDM or any other OFDM-based waveform will be studied as a potential additional waveform for downlink - Proposal 2: CP-OFDM and DFT-s-OFDM waveforms as defined in 5G NR are supported for communications in 6G uplink. - Enhancements/modifications on CP-OFDM/DFT-s-OFDM will be studied as potential additions - Other OFDM based waveforms are not precluded as potential additions. - Clarify the role of each of CP-OFDM and DFT-s-OFDM |  |  |
| [3] | R1-2600188 | On waveform enhancements/additions for 6G Radio | OPPO |
|  | Proposal 1: The study on 6GR waveform enhancements focuses on the requirements of 6G TN communication (i.e., eMBB and 6G IoT). - Strive for reusing the 6G TN communication waveform for NTN. - Consider an NTN-specific waveform enhancement when a substantial performance gain is justified. - The NTN-specific waveform enhancement, if accepted, is not used for 6G TN communication. Proposal 2: Study on waveform enhancements or additional waveform for 6G Sensing in the Sensing agenda. - Strive for reusing the 6G TN communication waveform for 6G Sensing. - The Sensing-specific waveform enhancement or additional waveform can be considered if substantial gain over the TN communication waveform can be justified. - The Sensing-specific waveform enhancement or additional waveform, if accepted, is not used for 6G communication. Proposal 3: For studying the waveform enhancements for 6GR, besides the evaluation of spectrum efficiency and coverage, the following should also be considered: - NW and UE side complexity. - Compatibility and neutrality for proposals in other areas, i.e., no restriction to or bundling with specific proposals for 6G MIMO, modulation, channel coding, AI/ML enhancements, etc. - Support flexible frequency-domain (e.g., RB-level) and time-domain (e.g., symbol-level) resource allocation. - Support of efficient 5G/6G spectrum sharing. Proposal 4: For 6GR TN communication, support up to 2 waveforms in DL and up to 2 waveforms in UL, e.g., one optimized for spectrum efficiency, one optimized for coverage. - At least 1 waveform in DL and 1 waveform in UL are mandatorily supported for all device types, e.g., CP-OFDM in DL and DFT-s-OFDM in UL. Proposal 5: Only one DL waveform is supported for 6GR initial access procedure. |  |  |
| [5] | R1-2600255 | Discussion on waveform for 6GR air interface | THALES |
|  | Proposal 1: RAN1 to investigate the performance of candidate waveforms under varying carrier frequency and time offsets inherent to satellite links, specifically considering scenarios with significant uncertainty in the UE’s position. This study will focus on conditions relevant to GNSS-free physical layer operation, aiming to identify robust waveform solutions suitable for environments with no GNSS availability. Proposal 2: Identify the set of NTN scenarios/use cases for which is beneficial to use DFT-s-OFDM in DL. Proposal 3: RAN1 to study the performance of DFT-s-OFDM in the downlink for non-terrestrial network (NTN)-based 6G radio access. |  |  |
| [7] | R1-2600295 | Discussions on waveform for 6GR | CATT |
|  | Proposal 1: Lower PAPR schemes shall be studied due to following aspects in 6GR: - Larger FFT size (e.g. from 4096 to 8192 or 16384) - Larger transmission channel bandwidth - Increased downlink free space path loss in NTN due to large propagation distance. |  |  |
| [9] | R1-2600384 | Discussion on the waveform design for 6G radio | CMCC |
|  | Proposal 1. CP-OFDM and DFT-s-OFDM waveforms as defined in 5G NR are supported for 6GR uplink. - Enhancements/modifications on CP-OFDM/DFT-s-OFDM will be studied as potential additions Proposal 2. CP-OFDM waveform as defined in 5G NR is supported for 6GR downlink. - Enhancements/modifications on CP-OFDM will be studied as potential additions Proposal 6. The proponents of non-OFDM based waveform are recommended to provide comprehensive air-interface design related to the proposed waveform, with proper evaluations according to the key requirements of 6GR. |  |  |
| [12] | R1-2600572 | Discussion on Waveform for 6GR Air Interface | IMU, Turkcell |
|  | Proposal 1: Define PAPR evaluation for 6G RAN over the composite transmitted waveform, including mixed numerologies and non-payload components (reference signals, pilots, control, and optional sensing components), to correctly capture PA back-off requirements and realistic efficiency/coverage trade-offs. Proposal 2: Enable composite-aware PAPR reduction mechanisms that preferentially exploit non-payload degrees of freedom, including inter-numerology degrees of freedom, while maintaining payload transparency targets (for example, EVM and BLER neutrality) and compliance with spectral constraints. Proposal 3: For ISAC scenarios, allow the sensing component (or other auxiliary component) to be adjusted (for example, time-segmented complex scaling) to reduce composite PAPR while respecting auxiliary-function KPIs (range/Doppler resolution and sidelobe control) and OOBE constraints. Proposal 4: Assess transparent versus signaled operation for composite-aware PAPR reduction. If transparency is feasible (for example, via equalization absorption for bounded phase factors), prioritize it to avoid overhead. If signaling is required, constrain it to low-overhead indices and bounded parameterization, and evaluate the trade-off jointly with complexity and receiver robustness. Proposal 6: Study coexistence mechanisms beyond strict block-based orthogonalization. In particular, enable (i) overlapping time-frequency occupancy under controlled rules, and (ii) frame-spanning transmission for spread waveforms (e.g., OTFS/OTSM/OCDM) to preserve delay/Doppler resolution, with structured mapping in the logical domain to manage interference and scheduling constraints. Proposal 7: Define a coexistence evaluation methodology that jointly measures link performance (BER/EVM/BLER), sensing KPIs (e.g., range/velocity RMSE where relevant), OOBE/INI/ACI, control overhead, and receiver complexity, and quantifies the cost-benefit tradeoff of guards versus structured spreading/windowing (including adaptive windowing across the grid). For low-power coexistence, include receiver-burden metrics (sampling/FFT requirements, synchronization burden) and benchmark against LPWAN-like baselines where applicable. |  |  |
| [13] | R1-2600584 | Discussion on 6G Waveform | NEC |
|  | Proposal 3: 6GR strives for a unified waveform baseband generation and upconversion for all channels and signals including PRACH. |  |  |
| [18] | R1-2600801 | Waveform for 6GR air interface | InterDigital, Inc. |
|  | 6GR DL and UL baseline waveforms Proposal 1: CP-OFDM is the only downlink waveform for 6GR; do not support additional DL waveforms Proposal 2: CP-OFDM and DFT-s-OFDM are the only waveforms for uplink. Study enhancements for PAPR reduction for DFT-s-OFDM. Proposal 3: Waveform for sensing is not covered in Agenda Item 11.3.1 and shall be studied separately in Agenda Item 11.14. |  |  |
| [22] | R1-2600999 | Discussion on 6GR waveform | ETRI, University of Surrey |
|  | Proposal 1. RAN1 to explicitly capture pros and cons of potential new or enhanced waveform candidates as summarized in the FLS. - Simple re-use of NR CP-OFDM and DFT-s-OFDM without any official RAN1 observation is NOT recommended, given that a number of contributions have been submitted during three WG meetings with in-depth assessments on the waveform candidates. |  |  |
| [25] | R1-2601080 | Discussion on Waveform for 6GR | Lenovo |
|  | Proposal 3: The study and evaluation of waveform enhancements should focus on CM characteristic of the waveform Proposal 4: When evaluating CP-OFDM and DFT-s-OFDM waveform enhancements techniques, the added complexity for both UE and BS, spectral efficiency loss, and the impact on signal quality should be reported for each technique. |  |  |
| [28] | R1-2601113 | Discussion on waveform for 6GR air interface | Panasonic |
|  | Proposal 1: 6GR should allow certain time / frequency resources can be different waveform for forward compatibility perspective and to support MRSS. Proposal 2: For 6GR waveform design, time/frequency grid should be allowed to be aligned and orthogonal with NR boundary. Proposal 3: OFDM-based waveform should be supported for 6GR. - The definition of “OFDM-based” is to have subcarrier mapping and IFFT to generate time-domain signal. Proposal 4: Striving for OFDM-based waveforms across all the identified use cases can be sufficient at least for 6G Day 1. Proposal 5: Any enhancements to CP-OFDM or DFT-s-OFDM and/or any newly introduced waveform must demonstrate clear and justified advantages over 5G waveform. This is not limited to 6G Day 1 assuming such enhancement is introduced only for CONNECTED mode. |  |  |
| [32] | R1-2601212 | Discussion on waveform for 6GR air interface | Pengcheng Laboratory |
|  | Proposal 2: Incorporate the following metrics into the system-level simulation evaluation metrics for waveforms: peak spectral efficiency, user plane latency and reliability, area spectral efficiency and average spectral efficiency, the 5th percentile user spectral efficiency, sensing performance, and energy efficiency. Where comparisons should be conducted based on identical simulation conditions. Proposal 3: In line with the previous meeting’s emphasis on high-speed train and highway scenarios, we propose to mandate high-mobility evaluations in waveform studies. UE speeds of 200 km/h and 500km/h should be considered for all uplink low-PAPR proposals to evaluate Doppler robustness and channel estimation impact. 3GPP 6GR. Carrier frequency and scenario; 4GHz. Channel BW; At least 100MHz for Urban (4GHz). Occupied BW; To be discussed with detailed simulation assumptions. SCS; 30 kHz for 4GHz. Channel model; TDL-C 300ns for 4GHz. UE speed; 3km/h(pedestrian), 200km/h(highway), 500km/h(high-speed train). Number of Tx antennas for TDL channel; 1. Number of Rx antennas for TDL channel; 1 and 4 for 4GHz. Number of DMRS symbols/slot (location as defined in NR); 2. Number of PUSCH data symbols/slot; 12. HARQ configuration; No retransmissions. Frequency hopping; Disabled. Proposal 4: Waveform design for 6GR must explicitly account for ISCI in high-mobility scenarios. This includes evaluating candidate waveforms under time-frequency doubly-selective channels (e.g., CDL/TDL models with high Doppler) and reporting BER/SER floors and sensing accuracy degradation as key metrics. |

# PAPR reduction

## UL DFT-s-OFDM PAPR reduction

| [1] | R1-2600027 | On remaining aspects of waveform for 6GR | Nokia |
| --- | --- | --- | --- |
|  | Low PAPR for coverage extension Proposal 5: Proposed low PAPR methods are compared using net gains and assuming similar spectral efficiency and bandwidth for each compared method and used baseline. Proposal 6: RAN1 to discuss further the methodology for comparison of low PAPR methods. Proposal 7: Transparent filtering approach (receiver does not need to be aware of the used filter parameters) is assumed for FDSS and FDSS-SE in 6G Radio. - Proposal 8: Frequency Domain Spectrum shaping (FDSS) and FDSS with spectrum extension (FDSS-SE) are supported in 6G Radio. - Proposal 9: High power class should be the baseline for 6G due to significant enhancement in coverage. - Proposal 10: Power boosting features such as the ones specified in NR should be part of the baseline for 6G. Proposal 11: Dynamic waveform switching using DCI is introduced to 6G in the first release |  |  |
| [2] | R1-2600138 | Waveform for 6GR air interface | Huawei, HiSilicon |
|  | Proposal 1: Support I/Q-offset DFT-s-OFDM as a lower PAPR waveform candidate for further RAN4 evaluations. Proposal 2: RAN1 can start discuss the potential spec impact of low PAPR waveform enhancement schemes, including but not limited to - Candidate spectrum adjustment ratios - Determination of spectrum adjustment ratios - TB size calculation - Multi-user overlap Proposal 3: Adopt Table 11 to characterize I/Q-offset DFT-s-OFDM as a RAN1 observation. |  |  |
| [3] | R1-2600188 | On waveform enhancements/additions for 6G Radio | OPPO |
|  | Proposal 8: For Net Gain evaluation for uplink low-PAPR proposals, check and compare the results from different proponents, and identify whether more aligned parameter combinations need to be identified to draw the conclusion. Proposal 9: Prioritize the implementation-based schemes without specification impacts. - Study additional gain from schemes with specification impacts. |  |  |
| [4] | R1-2600239 | Discussion on waveform for 6GR | LG Electronics |
|  | - Proposal 3: Study and evaluate uplink DM‑RS sequence designs with lower PAPR that can better align with the PAPR reduction achieved for the shared channel under FDSS SE. |  |  |
| [6] | R1-2600261 | Views on the waveform for 6G | ZTE Corporation, Sanechips |
|  | Proposal 1: For uplink low-PAPR proposals, the existing sub-6 GHz PA model from RAN4 (R4-163314) can be reused as baseline. - Proposal 3: To maintain same spectral efficiency in evaluation for both DL and UL low-PAPR proposals, it’s recommended to keep the same TB size for both candidate and baseline waveforms with adjusting code rate based on extension/truncation ratio. |  |  |
|  | Proposal 5: Tone reservation can be considered in 6G as a low-complexity scheme to achieve Net Gain for UL with DFT-s-OFDM. |  |  |
|  | Proposal 6: The FDSS with additional operation can be considered for UL PAPR reduction. Proposal 7: I-modulation scheme for π/2-BPSK/QPSK/QAM should be considered to reduce PAPR for DFT-s-OFDM.. Proposal 8: To improve coverage, FDSS-TR should be considered in 6G waveform design. |  |  |
| [7] | R1-2600295 | Discussions on waveform for 6GR | CATT |
|  | Proposal 6: Both symmetric and asymmetric FDSS-SE for DFT-s-OFDM can be supported in 6GR‌. Proposal 7: For UL low-PAPR proposals with spectrum extension, the granularity of both A and B is assumed as RB level, and A is determined based on B as followings: - For asymmetry spectrum extension A=1-α*B*1212 RBs - For symmetry spectrum extension If B is even A=1-α*B*1224*2 % note: A is also even Else A=1-α*B*1224*2+1 . %note: A is also odd. Proposal 8: For multi-user case of UL low-PAPR, supporting UE to perform dynamic adjustment of spectrum extension with symmetric way, or asymmetric way, or no spectrum extension according to expected PAPR value. Proposal 9: Study and consider 32-QAM as a candidate modulation for 6GR, particularly for power-limited and medium-SNR operation. Proposal 10: RAN1 should continue the study of π/2 BPSK frequency-domain truncation for uplink DFT-s-OFDM, including (i) single-user evaluation of truncation rates, mapping/reconstruction rules, required signaling (if any), and the SE–PAPR trade-off, and (ii) multi-user evaluation of coexistence under practical scheduling/multiplexing, including avoidance/management of mapping conflicts and any additional coordination/signaling needed for reliable per-UE reconstruction. Proposal 15: For improving spectrum efficiency, the multiplexing between DMRS and DFT-S-OFDM PUSCH data on a symbol can be studied. |  |  |
| [9] | R1-2600384 | Discussion on the waveform design for 6G radio | CMCC |
|  | Proposal 3. The study focuses on PAPR reduction for DFT-s-OFDM waveform. Proposal 4. The proposed design of FDSS and tone reservation should clarify the difference and improvement comparing to the corresponding work already been done for NR. Proposal 5. The MCSs used in the link-level evaluations of PAPR reduction techniques should satisfied the required 5th percentile user spectral efficiency in IMT-2030 requirements. |  |  |
| [10] | R1-2600424 | Further discussion on 6GR waveform | Xiaomi |
|  | Proposal: RAN1 further studies different filters including Kaiser filter and 3-tap filter for FDSS-SE based UL low PAPR waveform based on both net gain and implementation burden in both single UE and multi UE cases including how to improve net gain using interference mitigation techniques in multi UE case. |  |  |
| [11] | R1-2600499 | Discussion on Waveform for 6GR air interface | vivo |
|  | Proposal 1: For PAPR reduction, taking CFR or CFR based optimization as basis for study. Proposal 2: The spectrum extension of CFR-SE can be shared by UEs which can improve network spectrum efficiency. Proposal 3: Study AI/ML based scheme for low PAPR waveform enhancement. |  |  |
| [13] | R1-2600584 | Discussion on 6G Waveform | NEC |
|  | Proposal 1: Technical discussions and any future evaluations regarding the support of DL DFT-s-OFDM or FDSS enhancements should prioritize the agreed 6GR baseline parameters, including the specific PRB allocations and the 4 GHz Urban deployment scenario. |  |  |
|  | Proposal 4: Study the use of Frequency Domain Spectrum Shaping (FDSS) for DFT-s-OFDM in the 6GR uplink to enhance coverage and power efficiency. Proposal 5: The evaluation of FDSS enhancements should incorporate a realistic PA model, following the parameters defined in Option 2 of the 6GR system-level configurations, and must be validated against the full suite of agreed RF metrics, including ACLR, EVM, and IBE Proposal 6: Study a non-transparent FDSS operation for 6GR, including the signaling of the applied shaping filter to the receiver. The technical merits of this scheme should be quantified using the agreed metric of Net gain @ 10% BLER to ensure a balanced assessment of coverage gains against potential spectral efficiency impacts. |  |  |
| [15] | R1-2600627 | Waveform for 6GR Air Interface | Google |
|  | Proposal 3: The further study for UL waveform should consider the pratical impacts on uplink transmission power, e.g., port-specific power backoff. |  |  |
| [16] | R1-2600716 | Discussions on 6G Waveforms | Lekha Wireless Solutions |
|  | Proposal 6: UE transmit power is limited; hence uplink need waveforms with low PAPR for efficient PA usage, robust to mobility, CFO, and Doppler and need spectral efficiency and coexistence with other UL transmissions. Enhancements to current waveforms can be done using windowing and filtering techniques, CP length variations, SC-FDMA variants, and DFT-s-OFDM. Proposal 7: Due to restricted power requirements in uplink, DFT-s-OFDM was chosen for LTE UL instead of plain OFDM because it already offers lower PAPR. Still, additional PAPR reduction techniques are desired. Some good PAPR reduction techniques include DFT precoding, sub-carrier mapping, tone reservation or injection, clipping and filtering, selective mapping, partial transmit sequence, and precoding-based techniques. Proposal 8: TR provides measurable PAPR reduction while causing minimal degradation to BER compared to the baseline DFT-s-OFDM. Spectrum truncation-based methods in conjunction with TR do not give much PAPR reduction without causing severe BER degradation. Zadoff–Chu precoding yields modest PAPR improvement with increased processing complexity, and its hybridization with tone reservation offers limited additional benefit. In contrast, π/2-BPSK modulation significantly reduces PAPR without affecting BER performance. The hybrid π/2-BPSK + TR scheme achieves the lowest PAPR among all evaluated DFT-s-OFDM techniques while preserving BER transparency. Proposal 9: UL DFT-s-OFDM with Rank > 1 offers a practical means to extend uplink throughput while preserving low PAPR and high PA efficiency. Practical deployments are recommended up to Rank = 2, balancing efficiency, complexity, and link performance relative to CP-OFDM with higher ranks. |  |  |
| [17] | R1-2600751 | Discussion on waveform for 6GR | Samsung |
|  | UL DFT-s-OFDM PAPR reduction with FDSS Observation 1: Frequency domain spectrum shaping with the half-sine pulse filter for π/2-BPSK transmission achieves very low PAPR of below 1 dB. Observation 2: Half-sine pulse filter can satisfy 3GPP in-band emission requirement, so the effective occupied bandwidth does not increase. Observation 3: PAPR gain translates into large coverage gain when FDSS is applied with filter knowledge at the receiver side. - Unless the filter is specified, the benefit from applying FDSS on DFT-s-OFDM is limited Proposal 1: Support non-spec-transparent frequency domain spectrum shaping (FDSS) on DFT-s-OFDM to reduce PAPR for 6GR UL coverage enhancement - Identify and study filter design candidates to maximize the resulting PAPR reduction |  |  |
| [17] | R1-2600751 | Discussion on waveform for 6GR | Samsung |
|  | AI-based waveform for PAPR reduction Observation 8: The key differentiator between UE-side and two-sided models is the NW-side complexity rather than inter-vendor collaboration, which can be addressed through reference model specification for both architectures. Observation 9: Two-sided models achieve ~4.3 dB PAPR gain over DFT-s-OFDM, while UE-side models achieve ~2.3 dB gain. Both maintain BLER similar to DFT-s-OFDM. Observation 10: UE-side models (Approach 2) offer advantages in terms of NW complexity and backward compatibility, as the NW can reuse existing DFT-based receiver without requiring dual waveform support or NN-based processing. Proposal 6: Study AI-based reduced PAPR waveform for 6GR, considering the trade-off between PAPR reduction gain and NW-side complexity. Reference model specification should be considered to ensure interoperability and simplify the implementation of both UE-side and two-sided architectures. |  |  |
| [18] | R1-2600801 | Waveform for 6GR air interface | InterDigital, Inc. |
|  | Net gain evaluation results for PAPR reduction techniques for UL DFT-s-OFDM Observation 3: For Rx=1 and B=8, 64, 128 and 256, the extension factor of 1/4 for DFT-s-OFDM with FDSS-SE consistently yields positive net gain and superior performance over DFT-s-OFDM with FDSS Observation 4: For Rx=1 and B=8, 64, 128 and 256, the extension factor of 3/8, 7/16 or 1/2 for DFT-s-OFDM with FDSS-SE consistently yields negative or no net gain and inferior performance over DFT-s-OFDM with FDSS Observation 5: For Rx=4 and B=8, 64, 128 and 256, the extension factor of 1/4 for DFT-s-OFDM with FDSS-SE consistently yields positive net gain and superior performance compared to DFT-s-OFDM with FDSS Observation 6: For Rx=4 and B=8, 64, 128 and 256, the extension factor of 1/2 for DFT-s-OFDM with FDSS-SE consistently yields inferior performance compared to DFT-s-OFDM with FDSS Proposal 5: Support FDSS with spectrum extension for uplink PAPR reduction for DFT-s-OFDM to achieve the coverage target for 6G |  |  |
| [19] | R1-2600823 | On Waveforms for 6GR air interface | Apple |
|  | Proposal 1: The following table details the motivations of a GMSK-Approximation based FDSS as a low PAPR waveform for 6G Table 1: Low PAPR Waveform Motivation Description. Name of the proposal; GMSK-Approximation based FDSS. Motivation of the proposal; Uplink Coverage Enhancement and Energy Efficiency for a Terrestrial Network, Mitigation of PA non-linearity. Applicable link direction; UL. Enhancement to CP-OFDM?; No. Enhancement to DFT-s-OFDM?; Yes. Additional OFDM-compatible waveform?; No. Target channel(s)/signal(s); PUSCH. Target modulation; pi/2-BPSK. Motivation / use case; Improved Coverage, Energy Efficiency. Key Metric / KPI; Net Gain, EVM, IBE, EVM Equalizer Spectrum Flatness. Key spec impact foreseen; RAN1: Details of Non-transparent GMSK approximation FDSS. MRSS compatibility; Yes. Multiplexing/coexistence with other waveforms; Same as Rel-18 FDSS. Multi-user multiplexing; Same as baseline DFT-s-OFDM. MIMO compatibility; Same as baseline DFT-s-OFDM. Proposal 2: Study UL π/2-BPSK DFT-s-OFDM with GMSK approximation filters to reach near constant envelope waveform Proposal 3: Consider non-Transparent schemes in which the FDSS is always known at the Rx. Proposal 4: Consider near constant envelope waveform and potential impact to RAN4 for example in terms of amount of power boost and associated RF requirement. - Adopt pi/2-BPSK and DFT-s-OFDM with GMSK approximation framework for uplink coverage enhancement - Assume PC2 UE with and without power boost Proposal 5: Based on both the performance gain and the EVM performance, use the non-Transparent scheme. |
| [20] | R1-2600909 | Waveform for 6GR air interface | MediaTek Inc. |
|  | Proposal 1: As O-QPSK modulated DFT-s-OFDM offers a significant net gain compared to π/2-BPSK modulated DFT-s-OFDM, 6G should support O-QPSK modulated DFT-s-OFDM for UL coverage enhancement. Proposal 2: Support subcarrier truncation for π/2-BPSK and O-QPSK modulated DFT-s-OFDM to achieve spectral efficiency larger than 1 bit/s/Hz. |  |  |
| [21] | R1-2600914 | Study on waveform for 6GR | Sharp |
|  | Proposal 2: For 6GR waveform, RAN1 should support UL DFT-s-OFDM with FDSS for pi/2 BPSK. |  |  |
| [25] | R1-2601080 | Discussion on Waveform for 6GR | Lenovo |
|  | Proposal 2: Study enhancing DFT-s-OFDM waveform by incorporating PAPR/CM reduction techniques such as FDSS, DFT precoder extension, etc. |  |  |
| [26] | R1-2601092 | Discussion on waveform for 6GR air interface | Ofinno |
|  | FDSS-SE: Proposal 1: Multiple values of spectrum extension for FDSS-SE to enable uplink PAPR reduction in different scenarios are supported. Proposal 2: Both symmetric and asymmetric FDSS-SE schemes are supported. Proposal 3: FDSS-SE is employed for DFT-s-OFDM with at least /2-BPSK and QPSK. Frequency domain spectrum truncation (FDST): Proposal 4: Multiple values of spectrum truncation for FDST to achieve target spectral efficiency in different scenarios are supported. Proposal 5: FDST is employed in conjunction with at least FDSS. Proposal 6: Study feasibility of using FDST with FDSS-SE to achieve reasonable spectral efficiency and uplink PAPR reduction. Proposal 7: FDST is employed for DFT-s-OFDM with at least /2-BPSK. Proposal 8: Study feasibility of using FDST for DFT-s-OFDM with other modulation schemes, e.g., QPSK. |  |  |
| [28] | R1-2601113 | Discussion on waveform for 6GR air interface | Panasonic |
|  | Proposal 6: RAN1 should assess the need to introduce MPR / PAPR reduction techniques, e.g., FDSS-SE targeting coverage enhancement for UL. Proposal 7: RAN1 should study GMSK approximation proposed in R1-2509110 as one of candidate UL waveforms for MPR / PAPR reduction. |  |  |
| [29] | R1-2601127 | Waveforms for 6GR | Sony |
|  | Proposal 2: RAN1 should study PAPR reduction for DFT-s-OFDM especially when used with higher-order modulation. Proposal 3: RAN1 should study constellation shaping for low PAPR for DFT-s-OFDM with higher-order modulation. Proposal 6: 3GPP should allow PAPR reduction methods which traditionally have static configurations to be configured for each transmit signal individually. Proposal 7: 3GPP should not require that the PAPR configuration is explicitly signalled, rather the receiver may infer it from the received signal Proposal 8: 3GPP should specify probabilities for spectral extension to be applied. Proposal 9: 3GPP should study 4D constellations for the UL for PAPR reduction Proposal 10: 3GPP should adopt partial FDSS schemes. |  |  |
| [31] | R1-2601517 | Discussion on Waveform | NTT DOCOMO, INC |
|  | Proposal 2-1: Support the low PAPR proposals based on DFT-s-OFDM with SE, including FDSS-CE with CS and FDSS-SE, to improve uplink coverage. Proposal 2-2: Support the low PAPR proposals based on DFT-s-OFDM with ST, including DFT-s-OFDM with truncated mapping and asymmetric DFT-s-OFDM, to improve uplink coverage. |  |  |
| [32] | R1-2601212 | Discussion on waveform for 6GR air interface | Pengcheng Laboratory |
|  | Proposal 1: For uplink waveform evaluation, add the following metrics for UL PAPR reduction to the existing agreement (made in RAN1#123) - PAPR (CCDF @ 1e-4) - BLER performance under varied channel conditions (e.g., high mobility, frequency selectivity) - Out-of-band emissions (OOBE) - Computational complexity and implementation impact |  |  |
| [32] | R1-2601212 | Discussion on waveform for 6GR air interface | Pengcheng Laboratory |
|  | Proposal 6: It is recommended that Frequency Domain Spectrum Shaping (FDSS) be considered a foundational component for the 6G uplink waveform design. Proposal 7: It is recommended to investigate enhanced FDSS techniques, with a focus on FDSS with Spectrum Extension (SE), for potential inclusion in the 6G specification. The study should also encompass other spectral processing methods, such as spectral truncation. Proposal 8: We propose that 6GR standardizes an adaptive waveform enhancement for DFT‑s‑OFDM with π/2‑BPSK, based on configurable spectrum truncation ratios and a dynamic selection mechanism. This enhancement is specifically designed to maximize the uplink Net Gain across diverse scenarios, with a focus on high-mobility conditions. To this end, we recommend initiating a study to define a normative set of truncation ratios and the associated UE/network adaptation procedures (including necessary signaling and criteria) within the 6G specifications. |  |  |
| [33] | R1-2601268 | Waveforms for 6GR | Qualcomm Incorporated |
|  | On low PAPR waveform design Proposal 3.1: For uplink, focus study of tone reservation to DFT-S-OFDM waveforms with reserved tones immediately adjacent to the allocation (sideband tone reservation). Proposal 3.2: For low-PAPR waveform design for 6G uplink, recommend focusing on DFT-S-OFDM waveforms with pi/2 BPSK and frequency-domain truncation for further consideration. On other enhancements to DFT-S-OFDM Proposal 5.1: For 6GR waveform study, for DFT-S-OFDM waveforms, decouple the size of allocation from the DFT size. Define any DFT size that is a product of powers of 2, 3 and 5 as a valid DFT size. Proposal 5.2: For 6GR waveform study, when considering DFT-S-OFDM waveforms, consider flexible frequency-domain mapping of the DFT output to the spectrum allocation, e.g., frequency-domain multiplexing of DMRS and data, non-contiguous mapping, etc. Proposal 5.3: For 6GR waveform study, consider multi-tx enhancements for DFT-S-OFDM where different transmit ports transmit over different frequency domain allocations. On spectrum utilization Proposal 6.1: For 6GR waveform study, consider feasibility to enhance spectrum utilization for small channel bandwidths using spectrum confinement techniques (e.g. WOLA) of reasonable complexity. |  |  |
| [34] | R1-2601294 | Discussion on waveform for 6G air interface | Quectel |
|  | Proposal 1: Spectrum extension has the capability to reduce PAPR. Building upon this foundation, the expanded scope of spectrum extension application can be considered for reducing PAPR. Proposal 2: Extending the waveform to enable controlled spectrum spreading should be considered, with the option of overlapping resources among multiple users. In addition, alternative spectrum extension schemes beyond the currently defined extension factors should be explored. Such approaches can enhance sensing resolution while maintaining low PAPR and preserving coverage performance. |  |  |
| [36] | R1-2601366 | Enhancements for pi/2-BPSK DFT-s-OFDM: Overlapped Allocations | Wisig Networks, IITH |
|  | Proposal-1: RAN1 should study the overlapped-allocation method as a candidate technique for uplink PAPR reduction and spectral-efficiency improvement, and determine the optimum overlap for system implementation. Proposal-2: For π/2-BPSK DFT-s-OFDM, RAN1 should evaluate and specify the maximum number of DMRS ports required to support SU-MIMO with multiple layers and to enable partial or full spectral overlap between adjacent users. |  |  |

## UL CP-OFDM PAPR reduction

| [7] | R1-2600295 | Discussions on waveform for 6GR | CATT |
| --- | --- | --- | --- |
|  | Proposal 2: If Selective Mapping (SLM) is adopted for CP-OFDM waveformin 6GR, the scheme on reducing the overhead and indicating complexity of side information shall be studied. Proposal 3: The SLM based PRB bundling phase rotation evolved DMRS and data is suggested, with considering following benefit: - The phase ration factor for each PRB bundling set is no need indicated by DCI; - About 2dB net gain can be obtained. Proposal 4: The SLM based adding redundancy information in TB (transport block) is suggested for CP-OFDM waveform. Proposal 5: If SLM is applied to CP-OFDM waveforms, the recommended target channels/signals can include Unicast PDSCH, SIBX PDSCH, Msg2/4 PDSCH, Paging PDSCH, PDCCH, PBCH. |  |  |
| [11] | R1-2600499 | Discussion on Waveform for 6GR air interface | vivo |
|  | Proposal 1: For PAPR reduction, taking CFR or CFR based optimization as basis for study. Proposal 2: The spectrum extension of CFR-SE can be shared by UEs which can improve network spectrum efficiency. Proposal 3: Study AI/ML based scheme for low PAPR waveform enhancement. |  |  |
| [15] | R1-2600627 | Waveform for 6GR Air Interface | Google |
|  | Proposal 3: The further study for UL waveform should consider the pratical impacts on uplink transmission power, e.g., port-specific power backoff. |  |  |
| [25] | R1-2601080 | Discussion on Waveform for 6GR | Lenovo |
|  | Proposal 1: Study and evaluate CP-OFDM waveform enhancement techniques including PAPR/CM reduction techniques such as Selected Mapping (SLM) and Tone Reservation (TR) for coverage enhancement and energy efficiency improvement and compared to implementation-based techniques in terms of complexity, signal distortion, and spectral efficiency. |  |  |

## DL CP-OFDM PAPR reduction

| [2] | R1-2600138 | Waveform for 6GR air interface | Huawei, HiSilicon |
| --- | --- | --- | --- |
|  | Proposal 4: Support pruning QAM as a lower PAPR modulation candidate for RAN4 further evaluations Proposal 5: Study pruning QAM under CP-OFDM waveform for ISAC. Proposal 6: Adopt Table 14 to characterize Pruning QAM as a RAN1 observation |  |  |
| [6] | R1-2600261 | Views on the waveform for 6G | ZTE Corporation, Sanechips |
|  | Proposal 2: For downlink low-PAPR proposals, the Net Gain can be used for evaluation with following updates: - Net Gain [dB] = PAPR gain relative to the reference – SNR degradation relative to the reference. - Note:For data and control channel, the SNR is associated with 10% BLER. Proposal 3: To maintain same spectral efficiency in evaluation for both DL and UL low-PAPR proposals, it’s recommended to keep the same TB size for both candidate and baseline waveforms with adjusting code rate based on extension/truncation ratio. Proposal 4: Tone reservation can be considered in 6G as a low-complexity scheme to achieve Net Gain along with compatibility for DL with CP-OFDM. |  |  |
| [7] | R1-2600295 | Discussions on waveform for 6GR | CATT |
|  | Proposal 2: If Selective Mapping (SLM) is adopted for CP-OFDM waveformin 6GR, the scheme on reducing the overhead and indicating complexity of side information shall be studied. Proposal 3: The SLM based PRB bundling phase rotation evolved DMRS and data is suggested, with considering following benefit: - The phase ration factor for each PRB bundling set is no need indicated by DCI; - About 2dB net gain can be obtained. Proposal 4: The SLM based adding redundancy information in TB (transport block) is suggested for CP-OFDM waveform. Proposal 5: If SLM is applied to CP-OFDM waveforms, the recommended target channels/signals can include Unicast PDSCH, SIBX PDSCH, Msg2/4 PDSCH, Paging PDSCH, PDCCH, PBCH. |  |  |
| [11] | R1-2600499 | Discussion on Waveform for 6GR air interface | vivo |
|  | Proposal 5: Transparent solutions are the baseline of DL low-PAPR waveform for coverage/NW energy saving motivation. |  |  |
| [16] | R1-2600716 | Discussions on 6G Waveforms | Lekha Wireless Solutions |
|  | Proposal 3: Plain OFDM works well in DL but has some limitations such as high PAPR, sensitivity to phase noise and Doppler, out-of-band emissions, and limited flexibility to mixed numerologies. Hence, enhancements and alternative OFDM-based schemes need to be explored including windowing and filtering, sub-band filtering, cyclic prefix-based enhancements, precoding and MIMO enhancements, and DFT-s-OFDM. Proposal 4: Downlink transmit power gain needs to be calculated under realistic PA constraints while ensuring compliance with ACLR and EVM limits. Evaluation is based on the Net Gain metric (Tx power gain – link loss at 10% BLER) using a realistic base station PA model, complemented by secondary metrics such as PA efficiency, ACLR, EVM, network energy savings, and occupied bandwidth. Proposal 5: By maintaining BER performance that is almost the same as the baseline and obtaining a moderate PAPR decrease, SLM offers an equitable trade-off. Conversely, TR retains excellent BER behavior while producing only a slight boost in PAPR. |  |  |
| [25] | R1-2601080 | Discussion on Waveform for 6GR | Lenovo |
|  | Proposal 1: Study and evaluate CP-OFDM waveform enhancement techniques including PAPR/CM reduction techniques such as Selected Mapping (SLM) and Tone Reservation (TR) for coverage enhancement and energy efficiency improvement and compared to implementation-based techniques in terms of complexity, signal distortion, and spectral efficiency. |  |  |
| [26] | R1-2601092 | Discussion on waveform for 6GR air interface | Ofinno |
|  | CP-OFDM waveform for downlink: Observation 15: Base station typically employs PA linearization techniques (e.g., digital pre-distortion, power backoff, etc.), which is up to the base station implementation. Proposal 13: PAPR related enhancement for CP-OFDM in the downlink should be left for base station implementation in 6GR. |  |  |
| [29] | R1-2601127 | Waveforms for 6GR | Sony |
|  | Proposal 1: RAN1 should study PAPR reduction for CP-OFDM that can be applied to the NTN DL. |  |  |

## DL DFT-s-OFDM

| [1] | R1-2600027 | On remaining aspects of waveform for 6GR | Nokia |
| --- | --- | --- | --- |
|  | Proposal 3: RAN1 to deprioritize DFT-s-OFDM study for DL for the following reasons: - PAPR of DFT-s-OFDM in DL is comparable to DL CP-OFDM with transparent PAPR reduction techniques. - No meaningful pure DFT-s-OFDM NES gain or DL coverage gain in current FRs of interest compared to DL CP-OFDM especially where DFT-s-OFDM system limitations may be tolerated (e.g., low load, etc.). - DL EIRP could be already at the maximum with CP-OFDM in these DFT-s-OFDM potential scenarios, and no DL coverage extension can be foreseen. - DL DFT-s-OFDM may rely more on time domain multiplexing reducing time domain NES gain potential. - Base station and UE baseband complexity with DL DFT-s-OFDM is higher (e.g., DFT, Rx processing, transparent PAPR techniques need to be maintained). - DFT-s-OFDM can have link performance degradation compared to CP-OFDM in different conditions in DL (e.g., low complexity UE Rx, impractical R-ML per RE with DFT-s-OFDM, limited number of UE Rx antennas, etc.), - DFT-s-OFDM consideration in DL may need to impose limitations for minimizing PAPR impact with DFT-s-OFDM to the following: - UE frequency domain resource allocation (e.g., contiguous) - UE frequency domain multiplexing - Multiplexing of different DL physical channels/signals and efficient spectrum use (e.g., no FDM of physical channels using CP-OFDM with channel/signal using DFT-s-OFDM, or no/limited number of FDMed channels using DFT-s-OFDM) - SU/MU-MIMO precoding (e.g., limited number of layers for all UEs per port, wideband/subband precoding) - Efficient multi-RAT/MRSS support in FR1 (e.g., avoid frequency multiplexing of 5G DL CP-OFDM with some 6G DL in same RU) - Efficient RU use and carrier configuration flexibility (e.g., number and possible active carriers, potential of non-contiguous intra-band, inter-band spectrum aggregation, fragmented DL support per RU, restriction for all carriers in RU BS RF BW to use same DL waveform at least in overlapping DL transmission period) |  |  |
| [2] | R1-2600138 | Waveform for 6GR air interface | Huawei, HiSilicon |
|  | Proposal 10: For DL DFT-s-OFDM additional synch. Signal/DL-WUS, the performance evaluation criterion from waveform perspective is net gain - Net Gain [dB] = Tx power gain relative to the reference – Required SNR degradation relative to the reference - For additional synch. signal and sequence based DL-WUS, the required SNR is for detection rate below 1% and false alarm rate below [1%] assuming same resource overhead - The requirements of RSRP accuracy based on additional synch. signal should be met, e.g. as in TS 38.133 Proposal 11: Take Table 17 as a start point for DL DFT-s-OFDM waveform evaluation for additional synch. Signal/DL-WUS. Proposal 12: Study DL DFT-s-OFDM for additional synch. Signal/DL-WUS for coverage enhancement, network energy saving and UE energy saving under related agendas, e.g., initial access, DL WUS agenda. |  |  |
| [3] | R1-2600188 | On waveform enhancements/additions for 6G Radio | OPPO |
|  | Proposal 6: For downlink low-PAPR proposals the evaluation criterion is the PAPR reduction and SINR degradation @10% BLER. Proposal 7: DFT-s-OFDM is not supported as additional DL waveform for 6GR TN communication, due to limited performance gain, restriction on multiuser scheduling and extra complexity on UE side. - DL DFT-s-OFDM for NTN can be further studied. |  |  |
| [5] | R1-2600255 | Discussion on waveform for 6GR air interface | THALES |
|  | Proposal 1: RAN1 to investigate the performance of candidate waveforms under varying carrier frequency and time offsets inherent to satellite links, specifically considering scenarios with significant uncertainty in the UE’s position. This study will focus on conditions relevant to GNSS-free physical layer operation, aiming to identify robust waveform solutions suitable for environments with no GNSS availability. Proposal 2: Identify the set of NTN scenarios/use cases for which is beneficial to use DFT-s-OFDM in DL. Proposal 3: RAN1 to study the performance of DFT-s-OFDM in the downlink for non-terrestrial network (NTN)-based 6G radio access. |  |  |
| [7] | R1-2600295 | Discussions on waveform for 6GR | CATT |
|  | Proposal 11: DFT-s-OFDM waveform for downlink is needed to increase the output power of the PA. Proposal 12: For downlink low-PAPR proposals the primary evaluation criterion may use the following criterion: - Net Gain [dB] = Tx power gain - link loss relative to the reference @ Target KPI (e.g., BLER or detection rate) of target channel/signal. - A realistic PA model should be used - When calculating the Tx power gain, the RAN4 metrics on the Tx power should be taken into account - For SNR degradation, fading channel and non-ideal channel estimation, including DMRS configuration, and equalization is encouraged - FFS: Other evaluation metrics - Note: Companies to report how to calculate the Tx power gain, modulation and coding. Proposal 13: DFT-s-OFDM waveform can be applied in NTN downlink with introducing little complexity on the UE side to achieve significant power efficiency improvement. Proposal 14: For lager bandwidth transmission and enable scheduling flexibility, two segments DFT-S-OFDM can be studied. |  |  |
| [11] | R1-2600499 | Discussion on Waveform for 6GR air interface | vivo |
|  | Proposal 5: Transparent solutions are the baseline of DL low-PAPR waveform for coverage/NW energy saving motivation. |  |  |
| [13] | R1-2600584 | Discussion on 6G Waveform | NEC |
|  | Proposal 7: Study the support of DL DFT-s-OFDM for 6GR, prioritizing the evaluation of coverage and power efficiency gains in the target scenarios of IoT, NTN, FR2/FR3, and Network Energy Saving. The study should also address the trade-offs between PAPR reduction and downlink scheduling flexibility. Proposal 8: Study a simplified framework for DL waveform support, where a default waveform is used for initial access, and UE-specific configuration for DFT-s-OFDM is performed semi-statically via RRC signaling. Proposal 9: Study the waveform design for PDCCH in deployments supporting DL DFT-s-OFDM, evaluating two approaches: - The use of CP-OFDM for PDCCH to ensure implementation simplicity and compatibility. - The feasibility of using DFT-s-OFDM for PDCCH to improve performance, including a detailed analysis of the required structural redesign, challenges in supporting multiple users, and the overall system impact. Proposal 10: Study multi-user scheduling techniques for downlink DFT-s-OFDM, including group-based or sub-band DFT, to balance multi-user throughput with low-PAPR properties. |  |  |
| [15] | R1-2600627 | Waveform for 6GR Air Interface | Google |
|  | Proposal 1: Support DFT-s-OFDM waveform as the DL waveform with regard to the following aspects: - To support the same coverage for FR1 and FR3 - To provide a good coverage for NTN - Compared to other coverage enhancement techniques, e.g., to increase the number of antennas or to transmit the DL signals by multiple repetitions, using DFT-s-OFDM waveform does not require additional complexity for complicated CSI calculation, does not require large delay for beam measurement or multi-repetitions-based DL signal reception, and does not require large overhead for complicated CSI report and more DL-RSs for beam measurement. |  |  |
| [16] | R1-2600716 | Discussions on 6G Waveforms | Lekha Wireless Solutions |
|  | Proposal 1: DFT-s-OFDM has benefits in terms of PAPR and receiver performance. Due to the multi-carrier capabilities of CP-OFDM, it is more preferred in DL. However, there are use-cases like coverage-limited cells, small-cell BSs etc., where DFT-s-OFDM can help in power efficient transmission. Proposal 2: Transmit power gain needs to be calculated under realistic PA constraints while ensuring compliance with ACLR and EVM limits. Complementary metrics—such as SNR degradation, effective occupied bandwidth, and PA efficiency can also be analysed to provide a comprehensive assessment of waveform linearity and spectral behaviour. |  |  |
| [17] | R1-2600751 | Discussion on waveform for 6GR | Samsung |
|  | DL DFT-s-OFDM Observation 6: For downlink, the PAPR reduction of DFT-s-OFDM over CP-OFDM does not translate to significant DL coverage gain at least because of the following factors: - F1) Non-contiguous DL FDRA and its BLER advantage - F2) Common use of carrier aggregation (CA) - F3) Higher-rating PA resulting in minimal/zero PA backoff for CP-OFDM (including multi-carrier PA that makes the SC property of DFT-s-OFDM inapplicable) - F4) Active antenna arrays employing beamforming - F5) Multi-layer MIMO transmission In addition, it suffers from the following drawbacks: - D1) Multiplexing efficiency loss leading to SE and ESG loss - D2) Real-time multi-waveform processing at the UE receiver - D3) Overly complex and/or sub-optimal MIMO receiver leading to SE loss and higher UE power consumption Observation 7: For DL NTN scenarios, there is significant performance loss (in terms of both average and cell-edge UPT) with DFT-s-OFDM waveform when compared with CP-OFDM waveform Proposal 4: Discontinue the study for the potential support of DFT-s-OFDM waveform for 6GR downlink - No evidence of any potential benefit in DL coverage over CP-OFDM in all relevant use cases such as NTN, IoT, FR3, energy efficiency, common signals/channels. In fact, in most cases, DL DFT-s-OFDM would likely result in spectral efficiency loss, increased energy consumption at base stations and UEs, and substantial specification/testing efforts - New waveform for sensing purposes, if needed, will be discussed as a part of 6GR ISAC study |  |  |
| [21] | R1-2600914 | Study on waveform for 6GR | Sharp |
|  | Proposal 1: For 6GR waveform, RAN1 should NOT study DL DFT-s-OFDM. |  |  |
| [24] | R1-2601047 | Discussion on 6GR waveform design | Hanbat National University |
|  | Proposal 1: Study downlink waveform enhancements including the use of DFT-s-OFDM with FDSS and SLM. Proposal 2: Use the Net Gain (dB) = Tx power gain – Link Loss to the reference @10% BLER as a primary downlink waveform evaluation metric Proposal 3: The "Net Gain" evaluation framework shall be used to evaluate AI/ML-based PAPR reduction schemes. Proposal 4: In addition to "Net Gain," AI/ML-based schemes should be evaluated on their computational complexity and feasibility. The following metrics can be considered: - Model Complexity (e.g., number of parameters, FLOPs/symbol). - Signaling Overhead (e.g., bits for side information, if any). - Complexity Type: A clear distinction between offline training requirements and the real-time, on-device inference complexity. |  |  |
| [26] | R1-2601092 | Discussion on waveform for 6GR air interface | Ofinno |
|  | DFT-s-OFDM waveform for downlink: Observation 16: A base station employing large antenna array size (e.g., around 7 GHz) to increase coverage will require high energy efficiency. Observation 17: From downlink coverage perspective and network energy saving standpoint, waveform with relatively smaller PAPR will be beneficial in both TN and NTN deployment scenarios. Observation 10: Multiplexing CP-OFDM and DFT-s-OFDM across different downlink time resources (e.g., symbols) can reduce base station implementation complexity. Proposal 14: Consider DFT-s-OFDM as a potential additional waveform for downlink. Proposal 15: Consider at least /2-BPSK and QPSK as modulation schemes for DFT-s-OFDM waveform in downlink. Proposal 16: Target channels/signals for DFT-s-OFDM can be unicast PDSCH, UE specific PDCCH, and relevant reference signals (e.g., DMRS, CSI-RS). |  |  |
| [30] | R1-2601156 | On waveform for 6GR | Ericsson |
|  | Proposal 1 RAN1 to deprioritize the study of DFT-s-OFDM for downlink due to lack of any significant advantage compared to CP-OFDM even for the potential use cases such as NES, NTN ISAC, and over frequency range around 7 GHz, and for the transmission of common channels/signals as well as to keep the 6GR system design to a reasonable complexity. |  |  |
| [32] | R1-2601212 | Discussion on waveform for 6GR air interface | Pengcheng Laboratory |
|  | - Proposal 10: It is recommended to adopt DFT-s-OFDM as a complementary waveform to CP-OFDM in the 6G downlink. |  |  |
| [34] | R1-2601294 | Discussion on waveform for 6G air interface | Quectel |
|  | Proposal 4: Joint DFT for MU-DL-DFT-s-OFDM PDSCH should be supported for its better PAPR performance. |  |  |

# Rank > 1 for UL DFT-s-OFDM

| [1] | R1-2600027 | On remaining aspects of waveform for 6GR | Nokia |
| --- | --- | --- | --- |
|  | UL Multi-layer DFT-s-OFDM Proposal 12: RAN1 needs to further clarify power class and power mode in the last RAN1 agreement, and to consider explicitly rel-16 full power mode(s) (e.g., full power mode 0, full power mode 1, etc.) for all cases. Proposal 13: Low PAPR waveform study for multi-layers UL transmission in 6G needs to consider at least fully coherent and partially coherent UEs. Proposal 14 : RAN1 to consider the following baseline reference scenario for multi-layer waveforms study: - 5G NR UL MIMO codebooks - CP-OFDM for all number of layers & DFT-s-OFDM for single layer - Fully coherent UE coherence capability - DWS rel-18 enabled - At least rel-16 Full power mode(s) (e.g., full power mode 1, etc.) Proposal 15: For multi-layer DFT-s-OFDM study and comparison with CP-OFDM, RAN1 to consider the following metrics: - Cell average throughput gain or loss relative to the 5G NR baseline reference - Cell edge throughput (5%-percentile) gain or loss relative to the 5G NR baseline reference Proposal 16: RAN1 to study DFT-s-OFDM and assess its potential from UL MU-MIMO system perspective while ensuring more flexible scheduling for paired UE and better co-existence with CP-OFDM UL waveform compared to 5G NR. Proposal 17: Support CP-OFDM in uplink for all number of layers and all UE coherence capabilities, at least for non-power limited UEs, where DFT-s-OFDM potential power gain cannot be exploited. Proposal 18: Support DFT-s-OFDM in uplink for single layer transmission due to its advantage for power limited UEs. The potential of UL DFT-s-OFDM for more than 1 layer in 6G needs further justification. |  |  |
| [2] | R1-2600138 | Waveform for 6GR air interface | Huawei, HiSilicon |
|  | Proposal 7: Take net gain as the link level simulation metrics for multi-layer DFT-s-OFDM compare to multi-layer CP-OFDM. Proposal 8: CDF-based throughput gain is used as the system level simulation metrics for evaluations of UL multi-layer DFT-s-OFDM/CP-OFDM. Proposal 9: With observed coverage net gains, at least both 2-layers uplink DFT-s-OFDM waveform and 2-layer uplink CP-OFDM waveform should be supported in 6GR. The maximum number of layers should be further studied in 6GR MIMO agenda, e.g., codebook design. |  |  |
| [6] | R1-2600261 | Views on the waveform for 6G | ZTE Corporation, Sanechips |
|  | Proposal 9: The link-level simulation to evaluate the PAPA/Tx power gain compared with the OFDM is considered to justify the performance of DFT-s-OFDM for UL with number of layers > 1. Proposal 10: DFT-s-OFDM with rank 2 for uplink transmission can be considered in 6G waveform study. |  |  |
| [8] | R1-2600366 | Waveform design for 6G air interface | Tejas Network Limited |
|  | Proposal 1: DFT-s-OFDM is a suitable uplink waveform for 6GR due to its lower PAPR compared to CP-OFDM, which enables more efficient power amplifier operation and is particularly beneficial in coverage-limited scenarios. Proposal 2: Adoption of 2-layer DFT-s-OFDM for multi-layer uplink transmission. |  |  |
| [9] | R1-2600384 | Discussion on the waveform design for 6G radio | CMCC |
|  | Proposal 2. The target use cases are recommended to be clarified and distinguished between multi-layer CP-OFDM and multi-layer DFT-s-OFDM before starting detailed design for multi-layer DFT-s-OFDM. |  |  |
| [11] | R1-2600499 | Discussion on Waveform for 6GR air interface | vivo |
|  | Proposal 4: Support rank 2 for DFT-s-OFDM waveform at least for non-coherent precoders. |  |  |
| [13] | R1-2600584 | Discussion on 6G Waveform | NEC |
|  | Proposal 11: Study the support for multi-layer (SU-MIMO) transmissions using DFT-s-OFDM for 6GR. The study should prioritize the evaluation of Rank-2 performance to determine the net gain in spectral efficiency and transmit power compared to the multi-layer CP-OFDM baseline. Proposal 12: Evaluations for multi-layer UL waveforms should utilize the agreed configurations, including the 4 GHz carrier frequency, 30 kHz subcarrier spacing, and the specified CDL/TDL channel models to ensure comparability of results. |  |  |
| [15] | R1-2600627 | Waveform for 6GR Air Interface | Google |
|  | Proposal 2: Support the DFT-s-OFDM waveform for multiple layers for UL transmission. |  |  |
| [16] | R1-2600716 | Discussions on 6G Waveforms | Lekha Wireless Solutions |
|  | Proposal 9: UL DFT-s-OFDM with Rank > 1 offers a practical means to extend uplink throughput while preserving low PAPR and high PA efficiency. Practical deployments are recommended up to Rank = 2, balancing efficiency, complexity, and link performance relative to CP-OFDM with higher ranks. |  |  |
| [17] | R1-2600751 | Discussion on waveform for 6GR | Samsung |
|  | Proposal 2: To assess whether multi-rank DFT-s-OFDM can offer significant spectral efficiency gain, focus the study on rank-2 (two-layer UL transmission on PUSCH) Proposal 3: To assess whether rank-2 DFT-s-OFDM can offer significant UL spectral efficiency gain, further investigate its performance in deployment scenarios with primarily line-of -sight channels |  |  |
| [18] | R1-2600801 | Waveform for 6GR air interface | InterDigital, Inc. |
|  | Proposal 4: Multi-rank UL DFT-s-OFDM is not supported for 6GR |  |  |
| [20] | R1-2600909 | Waveform for 6GR air interface | MediaTek Inc. |
|  | Proposal 3: Study whether multi-layer DFTs-OFDM transmission is supported in 6G. Proposal 4: Study PAPR reduction for UL multi-layer transmissions, aiming to develop a unified solution applicable to both DFT-s-OFDM and CP-OFDM. Impacts on computational complexity and processing time should be considered. |  |  |
| [26] | R1-2601092 | Discussion on waveform for 6GR air interface | Ofinno |
|  | DFT-s-OFDM with multilayer uplink transmission: Proposal 9: DFT-s-OFDM with at least 2 ranks in the uplink is supported in 6GR. |  |  |
| [29] | R1-2601127 | Waveforms for 6GR | Sony |
|  | Proposal 5: RAN1 should study multi-layer transmission with DFT-s-OFDM for both UL and DL. |  |  |
| [30] | R1-2601156 | On waveform for 6GR | Ericsson |
|  | Proposal 2 In the link-level evaluations of multi-layer UL waveform, RAN1 to consider user throughput vs. SNR as a metric, by accounting MPR based on the RB allocation (e.g., inner/outer/edge), RB size and modulation order, subject to maximum requirements according to the RAN4 specifications. Proposal 3 In the system-level evaluations of multi-layer UL waveform, RAN1 to consider metrics such as rank statistics as well as cell-edge (5th percentile) user throughput, median (50th percentile) user throughput, and mean user throughput based on the statistics of user throughput. Proposal 4 Support multi-layer DFT-s-OFDM and multi-layer CP-OFDM for uplink transmissions in 6GR. |  |  |
| [31] | R1-2601517 | Discussion on Waveform | NTT DOCOMO, INC |
|  | Proposal 3-1: Support multi-layer UL MIMO transmission based on DFT-s-OFDM. |  |  |
| [32] | R1-2601212 | Discussion on waveform for 6GR air interface | Pengcheng Laboratory |
|  | Proposal 9: The support of at least Rank 2 DFT-s-OFDM for uplink transmission is proposed for inclusion in the 6G waveform study. |  |  |
| [33] | R1-2601268 | Waveforms for 6GR | Qualcomm Incorporated |
|  | On multi-rank DFT-S-OFDM Proposal 4.1: For 6G Radio, support DFT-S-OFDM in addition to CP-OFDM for multi-layer transmissions in uplink. |  |  |
| [35] | R1-2601354 | Discussion on waveform for 6GR air interface | KDDI Corporation |
|  | Proposal 1: UPT (mean, median and 5%-tile) based on SLS is used as a metric to show performance benefit. Optionally, Net Gain can be used based on LLS. Proposal 2: Thorough evaluation of 4-layer DFT-s-OFDM should also be conducted. Proposal 3: Non-coherent precoding should be the baseline for multi-layer DFT-s-OFDM. Coherent precoding is also considered if the precoding gain justifies the loss in the transmit power. |  |  |

# Other waveforms

| [1] | R1-2600027 | On remaining aspects of waveform for 6GR | Nokia |
| --- | --- | --- | --- |
|  | Proposal 4: RAN1 to deprioritize studying Zak-OTFS for the following reasons: - CP-OFDM outperforms Zak-OTFS with realistic simulation assumptions and realistic channel estimation - Zak-OTFS is claimed to provide benefit mainly in propagation conditions that are not typical in real deployments - Zak-OTFS would be a major change for the current systems even if it may be able to be implemented on top of CP-OFDM waveform - Zak-OTFS would introduce additional complexity to both network and UE side |  |  |
| [6] | R1-2600261 | Views on the waveform for 6G | ZTE Corporation, Sanechips |
|  | Proposal 11: DFT-s-OFDM with enhanced time domain resource multiplexing in symbol-level (i.e., eDFT-s-OFDM waveform) can be considered as a candidate waveform technology for 6G waveform design to improve the performance at least for high-speed scenario. Proposal 12: GFB-OFDM should be considered in 6G waveform study as a scheme to improve the spectrum utilization. Proposal 14: The characterization of the proposed candidate schemes presented in Table 5-1 should be incorporated into 6G waveform study. |  |  |
| [9] | R1-2600384 | Discussion on the waveform design for 6G radio | CMCC |
|  | Proposal 6. The proponents of non-OFDM based waveform are recommended to provide comprehensive air-interface design related to the proposed waveform, with proper evaluations according to the key requirements of 6GR. |  |  |
| [12] | R1-2600572 | Discussion on Waveform for 6GR Air Interface | IMU, Turkcell |
|  | Proposal 5: Adopt a coexistence-first waveform evolution direction where new waveform options are realized as configurable PHY slices (logical lattice mapping, precoding/spreading, and windowing) over a common CP-OFDM/FFT-based pulse-shaping baseline, so that different options remain multiplexable on the same resource grid and reference-signal structure. |  |  |
| [14] | R1-2600612 | Waveform for 6GR Air Interface | Cohere Technologies |
|  | Proposal 1: Zak-OTFS and Zak-OTFS-over-OFDM will be included in the waveform study for 6G Proposal 2: Zak-OTFS proposed frame structure and numerology should be included in the study for 6G |  |  |
| [17] | R1-2600751 | Discussion on waveform for 6GR | Samsung |
|  | Other waveforms Proposal 5: Discontinue the study for the potential support of “other waveforms” in 6GR - Deviating from “single technology framework” goal of 6GR study - Increased risk of RAT divergence for different 6GR use cases - Not conducive to the agreed support for NR-6GR migration via MRSS |  |  |
| [22] | R1-2600999 | Discussion on 6GR waveform | ETRI, University of Surrey |
|  | Proposal 2. Capture the following performances of AFDM into the TR: - PAPR reduction: 3 dB (by turning AFDM modulation parameter) - BLER: 0.5 dB gain @ SNR= 5dB with Doppler frequency of 3000 Hz; 1.5 dB gain @ SNR= 5dB with Doppler frequency of 6000 Hz - Compatibility with FMCW Radar - Net Gain: - 1 dB and -1.1 dB over DFT-s-OFDM and CP-OFDM, respectively @ 10% BLER with UE speed of 3 km/h - 1.5 dB and 3.6 dB over DFT-s-OFDM and CP-OFDM, respectively @ 10% BLER with UE speed of 500 km/h - 4.1 dB and 6.3 dB dB over DFT-s-OFDM and CP-OFDM, respectively @ 10% BLER with UE speed of 1500 km/h Proposal 3. RAN1 to capture the following summary on AFDM waveform to the TR on 6GR: Description. Name of the proposal; AFDM. Motivation of the proposal; NTN, ISAC, etc. Applicable link direction; UL / DL. Enhancement to CP-OFDM?; Yes. Enhancement to DFT-s-OFDM?; Yes. Additional OFDM-compatible waveform?; No. Target channel(s); PDSCH, PUSCH, PRACH (for NTN). Target modulation; No restriction (applicable for all existing NR modulations). Motivation / use case; Coverage enhancement, support of high-speed mobility, ISAC. Key Metric / KPI; BLER, Sensing accuracy, Possible Net Gain and delay-Doppler robustness. Key spec impact foreseen; RAN1 waveform definition, time-frequency resource mapping, pilot design, scheduling extensions. MRSS compatibility; Highly compatible; supports FDM/TDM/Hybrid resource partitioning for NTN/ISAC slices within Multi-RAT Spectrum Sharing deployments.. Multiplexing/co-existence with other waveforms; Flexible; enables coexistence with CP-OFDM/DFT-s-OFDM via FDM/TDM. Multi-user multiplexing; Support of chirp-domain multiplexing. MIMO capability; AFDM based multi-layer Tx (MIMO) may be less critical in major use cases, including NTN and ISAC scenarios. Compatibility with SU-MIMO and MU-MIMO remains for further study.. |
| [23] | R1-2601019 | Discussion on New Waveform for 6GR Air Interface | Shanghai Jiao Tong University, NERC-DTV |
|  | Proposal 1: RAN1 to study additional waveform candidate (e.g., AFDM) for 6G that stays structurally compatible with OFDM, enabling reuse of 5GNR/6GR ecosystem components, while targeting enhanced performance in sensing, high mobility, and NTN scenarios. Proposal 2: RAN1 to investigate AFDM as a candidate waveform for 6G radio, with a specific focus on evaluating its robustness in high-mobility scenarios, high-efficiency sensing capabilities, low PAPR characteristics, and integration into an 6GR compatible system architecture. Proposal 3: RAN1 to study and specify the design of AFDM parameters (e.g., and ), low-complexity receiver algorithms for communication, sensing, and PAPR reduction, and their integration into a 6G-compatible system architecture, to enhance robustness against doubly-selective channels while targeting superior performance in sensing, high-mobility, and NTN scenarios. Proposal 4: RAN1 to investigate the implications of AFDM on MIMO channel estimation, receiver signal processing, etc., and to study low-complexity techniques to ensure the efficient integration of AFDM with existing multi-antenna systems. |  |  |
| [27] | R1-2601110 | New waveform for 6GR air interface | NICT |
|  | Proposal 1: RAN1 to consider the spectral precoding (SP) as one of candidate waveform techniques that has potential to improve spectral efficiency which is one of important criteria of 6GR design. |  |  |
| [41] | R1-2601539 | New Waveform for 6GR - OSDM | R1-2601539 |
|  | Proposal: To retain OSDM as 6GR’s candidate waveform. |  |  |

# Uncategorized proposals

| [2] | R1-2600138 | Waveform for 6GR air interface | Huawei, HiSilicon |
| --- | --- | --- | --- |
|  | Proposal 5: Study pruning QAM under CP-OFDM waveform for ISAC. Proposal 6: Adopt Table 14 to characterize Pruning QAM as a RAN1 observation |  |  |
| [3] | R1-2600188 | On waveform enhancements/additions for 6G Radio | OPPO |
|  | Proposal 10: Postpone the discussion on waveform switching/selection until the waveform decisions have stabilized. When the study starts, - Consider all potential solutions, including explicit indication as well as implicit mechanisms without DCI overhead. - Study on dynamic UL waveform switching during initial access is deprioritized. |  |  |
| [4] | R1-2600239 | Discussion on waveform for 6GR | LG Electronics |
|  | - Proposal 1: Discuss how the UE should be instructed to use a particular waveform for 6GR in uplink. - Option 1: Waveform selection based on transmission rank. - Option 2: Explicit waveform indication via cell-specific configuration, channel-specific, or BWP-specific configuration, including dynamic switching. - Option 3: Waveform selection based on frequency band or usage scenario. - Proposal 2: Study whether/how reference signal design should consider commonality across CP-OFDM and DFT-s-OFDM in both uplink and downlink. |  |  |
| [6] | R1-2600261 | Views on the waveform for 6G | ZTE Corporation, Sanechips |
|  | Proposal 13: Study pulse RS design using OFDM-based waveform for large sensing coverage. - The pulse is applicable for both mono-static and bi-static sensing. - Study the application for communication, e.g., RSRP measurement, time/frequency tracking. |  |  |
| [7] | R1-2600295 | Discussions on waveform for 6GR | CATT |
|  | Proposal 16: For supporting sensing, OFDM-based wave can be supported for both pulse wave (PW) and Continuous wave (CW): - OFDM-based PW with larger SCS (i.e. 960 kHz or 1920 kHz) than communication - OFDM-based CW with same SCS as communication. Proposal 17: For the enhancement of sensing waveform, the new waveform different to OFDM can be studied, such as LFM (Linear Frequency Modulation), AFDM (Affine Frequency Division Multiplexing) and OCDM (Orthogonal Chirp Division Multiplexing). |  |  |
| [8] | R1-2600366 | Waveform design for 6G air interface | Tejas Network Limited |
|  | Proposal 3: Dynamic waveform switching enables the waveform to be adjusted in real time to match changing uplink demands and operating conditions. |  |  |
| [12] | R1-2600572 | Discussion on Waveform for 6GR Air Interface | IMU, Turkcell |
|  | Proposal 10: For sensing, study additional waveforms (e.g., FMCW-like or other Doppler-robust sensing signals) that can coexist with OFDM-based communication grids without excessive receiver complexity. |  |  |
| [13] | R1-2600584 | Discussion on 6G Waveform | NEC |
|  | Proposal 2: CP-OFDM and DFT-s-OFDM in NR are baseline as 6GR uplink waveform. 6GR could study to support dynamic waveform switching during initial access. |  |  |
| [26] | R1-2601092 | Discussion on waveform for 6GR air interface | Ofinno |
|  | UE power boosting techniques: Proposal 10: Consider high UE power class (e.g., 26 dBm) as a mandatory feature in 6GR from Day 1. Proposal 11: Study the possibility of reducing MPR in 6GR. Dynamic waveform switching: Proposal 12: Support dynamic switching between DFT-s-OFDM and CP-OFDM in 6GR from Day 1. |  |  |
| [28] | R1-2601113 | Discussion on waveform for 6GR air interface | Panasonic |
|  | Proposal 8: The need to introduce flexible DMRS and data techniques (e.g., TDM between data and DMRS before DFT precoding such as OTFDM) could be investigated. |  |  |
| [29] | R1-2601127 | Waveforms for 6GR | Sony |
|  | Proposal 4: RAN1 should study multiplexing of CP-OFDM reference signals on the same component carrier with DFT-s-OFDM physical channels. |  |  |
| [32] | R1-2601212 | Discussion on waveform for 6GR air interface | Pengcheng Laboratory |
|  | Proposal 5: We recommend focused research and development on this enhanced, Doppler-resilient CP-OFDM precoding scheme. Furthermore, we advocate that future 6G systems be designed with the agility to support multiple waveform enhancement modes, dynamically tailored to user mobility. For instance, the system could seamlessly switch to this anti-Doppler precoding scheme in high-speed scenarios while retaining standard CP-OFDM for low- to medium-mobility users, thereby optimizing performance and efficiency across the network. Proposal 11: It is proposed to study efficient waveform multiplexing and switching mechanism for 6GR. |  |  |
| [33] | R1-2601268 | Waveforms for 6GR | Qualcomm Incorporated |
|  | On other enhancements to DFT-S-OFDM Proposal 5.3: For 6GR waveform study, consider multi-tx enhancements for DFT-S-OFDM where different transmit ports transmit over different frequency domain allocations. On spectrum utilization Proposal 6.1: For 6GR waveform study, consider feasibility to enhance spectrum utilization for small channel bandwidths using spectrum confinement techniques (e.g. WOLA) of reasonable complexity. |  |  |
| [34] | R1-2601294 | Discussion on waveform for 6G air interface | Quectel |
|  | Proposal 3: Multi-user coordination mechanisms, such as controlled resource overlapping, joint scheduling, or cooperative transmission, should be considered to compensate for the spectral efficiency loss introduced by spectrum extension, while preserving the benefits in PAPR reduction, coverage enhancement, and sensing performance. |  |  |
| [35] | R1-2601354 | Discussion on waveform for 6GR air interface | KDDI Corporation |
|  | Proposal 4: Dynamic switching between multi-layer DFT-s-OFDM and multi-layer CP-OFDM should be discussed after the maximum number of layers for DFT-s-OFDM in 6G has been determined. Proposal 5: Signalling overhead in uplink is also taken into account when the comparative evaluation between the proposed low-PAPR techniques is conducted. |  |  |
| [36] | R1-2601366 | Enhancements for pi/2-BPSK DFT-s-OFDM: Overlapped Allocations | Wisig Networks, IITH |
|  | Proposal-2: For π/2-BPSK DFT-s-OFDM, RAN1 should evaluate and specify the maximum number of DMRS ports required to support SU-MIMO with multiple layers and to enable partial or full spectral overlap between adjacent users. |  |  |

# Discussion: Waveform for UL MIMO

Background:

- LTE supporting DFT-s-OFDM for all supported LTE UL MIMO ranks

- NR supporting DFT-s-OFDM for single layer transmission and CP-OFDM for ranks 1 to 8

- So far, we only have agreed to DFT-s-OFDM and CP-OFDM as a bases for 6G uplink – based on the following agreement from RAN1#122:

Agreement CP-OFDM and DFT-s-OFDM waveforms as defined in 5G NR are supported as the basis for 6GR for uplink - Enhancements/modifications on CP-OFDM/DFT-s-OFDM will be studied as potential additions - Other OFDM based waveforms are not precluded.

- But we have not yet any association of the baseline UL waveforms to the related UL SU-MIMO transmission ranks

  - During RAN1#122bis, there has been a try to get some progress in the understanding how the interaction of higher rank DFT-s-OFDM UL would be in terms of UL MIMO operation

The moderator thinks that it would be still good to get some common understanding on the overall framework of combinations of UL SU-MIMO and the relation to CP-OFDM and DFT-s-OFDM.

## Single layer (i.e. rank=1) UL transmissions:

Question 8.1.1: For single layer (i.e. rank=1) UL transmission, how do you see the support of DFT-s-OFDM and/or CP-OFDM?

- Please indicate  your ‘support’ only to one of the 3 options.

- If having (further) comments, please provide  your input in the separate table below.

| Supported baseline UL WF (single layer, i.e. rank=1) | Companies position (list of companies) |
| --- | --- |
| Support CP-OFDM only |  |
| Support DFT-s-OFDM only | CATT |
| Support of both, DFT-s-OFDM & CP-OFDM | OPPO, Spreadtrum, Nokia, CMCC, Lekha, Apple, Sony, DOCOMO, NEC, Panasonic, IMU, Samsung, Shef |

| Company | Comments |
| --- | --- |
| OPPO | This question is also related to the “dynamic UL waveform switching” issue. But the current assumption is that only semi-static switching between DFT-s-OFDM and CP-OFDM in UL. In this case, support of single-layer CP-OFDM is necessary, so that a UE can dynamically switch between single layer and multiple layers without RRC reconfiguration. |
| CATT | Since the coverage requirement is more fundamental, the intitial access stage may only allow DFT-s-OFDM, and in the RRC-connection stage, CP-OFDM can be chosen to support more than rank 1 transmission. |
| Nokia | We support DCI-based dynamic waveform switching |
| CMCC | One related issue is whether to allow BS to support only one waveform in some scenario. For example, coverage and device power consumption may not be a problem for indoor industrial scenarios. Correspondingly, the indoor BS may only support e.g. CP-OFDM for simple implementation. Meanwhile, DFT-s-OFDM is important for meeting the target coverage in outdoor widearea deployment. Consequently, it is suggested to support both DFT-s-OFDM and CP-OFDM for single layer transmission. |
| Lekha | Currently, we advocate considering both. |
| Sony | We support both waveforms for UL |
| DOCOMO | 5G NR has proven the necessity to support both waveforms as specified by semi-static and dynamic waveform switching |
| NEC | CP-OFDM and DFT-s-OFDM should be considered baseline for 6GR uplink waveform. 6GR should also support dynamic waveform switching which enabled efficient link adaptation. |
| IMU | Wavefom adaptation depending on the link condition is more valuable than layer swithing |
| Samsung | In 5G NR, the motivation of supporting DFT-s-OFDM for single layer UL transmission was clearly UL coverage enhancement, which complements CP-OFDM targeting high spectral efficiency. |
| Shef | Both cover more use cases with dynamic WF switching flexibility. |

Question 8.1.2: If both DFT-s-OFDM and CP-OFDM is to be supported for single-layer UL transmission (i.e. rank=1) from specification point of view, how do you see the required UE support?

- Please indicate your ‘support’ only to one of the options.

- If having (further) comments, please provide your input in the separate table below.

| UE support for single-layer UL | Companies position (list of companies) |
| --- | --- |
| Alt. 1: CP-OFDM mandatory DFT-s-OFDM optional |  |
| Alt. 2: DFT-s-OFDM mandatory CP-OFDM optional | CATT, IMU |
| Alt. 3: Both (i.e. DFT-s-OFDM & CP-OFDM) mandatory | OPPO (for eMBB UE), Spreadtrum, Nokia, CMCC, Lekha, Apple. Sony, DOCOMO, NEC, Panasonic, Shef |
| Alt. 4: Up to UE capability indication (per band and/ band combination) |  |

| Company | Comments |
| --- | --- |
| OPPO | This question is related to “device type” discussion. Different device types can have different mandatory functionality sets. Our assumption is that this question is for eMBB UE, for which both CP-OFDM and DFT-s-OFDM should be mandatory. However, for 6G IoT device, only DFT-s-OFDM is mandatory, since only single layer is mandatory for 6G IoT device. In our option, CP-OFDM is not mandatory for 6G IoT devices. |
| CATT | For one default option, DFT-s-OFDM waveform can be applied for the initial access stage for coverage purpose. |
| CMCC | Each of the two waveforms shows advantage in different scenarios. BS can determine the more suitable one or both to be used. In other words, UE has to support both for efficient and flexible deployment. |
| Sony | Similar situation as NR |
| DOCOMO | Different UE capability would increase operational complexity. Since both waveforms were mandatory supported in NR, this should be kept for 6GR otherwise 6GR would have worse performance than NR. |
| NEC | To manage the 6GR requirements of extended coverage as well as high data rates, it is essential to support both DFT-s-OFDM (for coverage enablement) and CP-OFDM (to ensure high throughput by enabling higher MIMO ranks) |
| IMU | IoT should support DFT-s-OFDM (always), other devices can support both |
| Samsung | It’s early to discuss in the study stage. |

## UL transmissions with rank=2

Question 8.2.1: For UL transmissions with rank= 2, how do you see the support of DFT-s-OFDM and/or CP-OFDM?

- Please indicate  your ‘support / yes’ only to one of the 3 options in color (1., 2. or 3.).

- If you indicate “Yes” to 1. (CP-OFDM only) or 2. (CP-OFDM only), please indicate in 4. if you are open to consider &/ study the support of both waveforms or not (i.e. yes / no)

- If having (further) comments, please provide your input in the separate table below.

| Supported baseline UL WF for rank=2 | Considered options | Companies position (list of companies) |
| --- | --- | --- |
| 1. CP-OFDM only | Yes | OPPO, CATT, Nokia, CMCC, Samsung |
| 2. DFT-s-OFDM only | Yes |  |
| 3. DFT-s-OFDM & CP-OFDM | Yes | Spreadtrum, DOCOMO |
| 4. Open for studies / consider to support both WFs | Yes | OPPO, CMCC, Lekha, Apple, Sony, NEC, Panasonic, IMU, Shef |
|  | No |  |

| Company | Comments |
| --- | --- |
| OPPO | The LLS result shows no gain of 2-layer DFT-s-OFDM. But the SLS assumption was agreed in RAN1#123. We are open to further invest the gain based on the SLS results. |
| CATT | Concering the complexity and performance gain for DFT-s-OFDM in rank-2 transmission, only CP-OFDM waveform is used in UL transmission if rank-2 is needed. |
| Nokia | Further clarification on simulation assumptions is needed to assess the benefit, if any, of DFT-s-OFDM for rank=2. |
| CMCC | We are open to study the DFT-s-OFDM waveform for RANK-2 UL transmission. However, the corresponding use cases and benefits have to be clarified before introducing it for 6GR. |
| Sony | DFT-s-OFDM for rank > 1 should be studied. |
| DOCOMO | As provided by a number of companies, 2-layer DFT-s-OFDM shows net gain thanks to Tx power gain for non-coherent precoder, and hence, 2-layer DFT-s-OFDM should be supported at least for non-coherent precoder. For coherent precoder, DFT-s-OFDM woud lose its Tx power gain due to higher PAPR, while it is unclear at this stage which waveform can provide better performance due to lack of precoder design, which should be discussed under AI 10.5.2.3. |
| NEC | Even supporting rank-2 for DFT-s-OFDM does not necessarily ensure that rank-2 can be used in all scenarios (considering the restrictions on precoder matrix we may have for DFT-s-OFDM). CP-OFDM would still be required to obtain high UL data rates where rank-2 of DFT-s-OFDM is not feasible. So, CP-OFDM should be supported as baseline and DFT-s-OFDM should be further studied. |
| Panasonic | There are evaluation results showing the gain of 2-layer DFT-s-OFDM, while there are also the results showing no gain of 2-layer DFT-s-OFDM. Then, the gain should be further studied. |
| IMU | Further study is needed to see the potential gains and trade-offs |
| Samsung | Coverage-limited UEs are not expected to be assigned more than one layers – not only due to its respective channel condition, but also due to the significant PAPR increase associated with multi-layer transmission. Supporting multi-layer DFT-s-OFDM lacks clear motivation in relation to UL coverage enhancement. |
| Shef | New waveforms offer enhanced high rank performance and should be encouraged to future proof 6GR. |

Question 8.2.2: If both DFT-s-OFDM and CP-OFDM is to be supported for UL transmission with rank=2 from specification point of view, for a UE supporting UL MIMO rank=2 - how do you see the required UE support?

- Preferably, please indicate at your ‘support’ only to one of the options.

- If having (further) comments, please provide your input in the separate table below.

| UE support for UL rank=2 | Companies position (list of companies) |
| --- | --- |
| Alt. 1: CP-OFDM mandatory DFT-s-OFDM optional | OPPO (for eMBB UE), Spreadtrum, Nokia, NEC, IMU |
| Alt. 2: DFT-s-OFDM mandatory CP-OFDM optional |  |
| Alt. 3: Both (i.e. DFT-s-OFDM & CP-OFDM) mandatory | Sony, DOCOMO |
| Alt. 4: Up to UE capability indication (per band and/ band combination) | Lekha, Apple, Shef |

| Company | Comments |
| --- | --- |
| OPPO | It is desired CP-OFDM with all layers can over most of usage scenarios for eMBB UE. DFT-s-OFDM is only used in very coverage-limited scenario. This is enough for most of eMBB devices. Only high-capability eMBB UEs support 2-layer DFT-s-OFDM for a better DL throughput in coverage-limited scenario. But for 6G IoT devices, all UL multi-layer transmssions (including with CP-OFDM and DFT-s-OFDM) are optional. |
| Nokia | We don’t support rank=2 UL DFT-s-OFDM. We understand this question (and hence the reply) as a hypothetical one. |
| CMCC | If DFT-s-OFDM waveform is finally introduced for RANK-2 UL transmission, UE has to support both for efficient and flexible deployment in dfferent scenarios. |
| Sony | First we study and if DFT-s-OFDM can support rank > 1 efficiently, both should be mandatory. |
| DOCOMO | We are not sure whether there is any difficulty to support both waveforms for 2-layers, assuming both waveforms are mandatory for 1-layer, but open to hear companies’ view. |

## UL transmissions with ranks 3 & 4

Question 8.3.1: For UL transmissions with rank= 3 or 4, how do you see the support of DFT-s-OFDM and/or CP-OFDM?

- Please indicate  your ‘support / yes’ only to one of the 3 options in color (1., 2. or 3.).

- If you indicate “Yes” to 1. (CP-OFDM only) or 2. (CP-OFDM only), please indicate in 4. if you are open to consider &/ study the support of both waveforms or not (i.e. yes / no)

- If having (further) comments, please provide your input in the separate table below.

| Supported baseline UL WF for rank=3 or 4 | Considered options | Companies position (list of companies) |
| --- | --- | --- |
| 1. CP-OFDM only | Yes | OPPO, CATT, Nokia, CMCC, Lekha, Apple, DOCOMO, Samsung |
| 2. DFT-s-OFDM only | Yes |  |
| 3. DFT-s-OFDM & CP-OFDM | Yes |  |
| 4. Open for studies / consider to support both WFs | Yes | Spreadtrum, CMCC, Sony, DOCOMO, NEC, IMU, Shef |
|  | No | OPPO, Nokia |

| Company | Comments |
| --- | --- |
| OPPO | First identify the gain of 2-layer DFT-s-OFDM before discussing the >3-layer DFT-s-OFDM proposal. |
| CATT | We need firstly discuss rank-2 case. |
| Spreadtrum | Open for study, but the benefit of the rank>2 DFT-s-OFDM UL transmission should be identified first. |
| Nokia | We need first to discuss the rank=2 case |
| CMCC | Open to study DFT-s-OFDM wavefor for UL transmission with RANK>2. |
| Lekha | Beyond 2 layers, we don’t think DFT-s-OFDM will give any additional benefits |
| DOCOMO | We are not sure how much gain can be obtained for DFT-sOFDM w/ 3 or 4 layers, considering both non-coherent and coherent precoding performance for now, but open to further study. However, we assume coherent precoder design should be discussed under AI 10.5.2.3. |
| NEC | While for Rank-2 DFT-s-OFDM, evaluations results indicate that impact on PAPR is not significant, we need to study the detailed comparison of CP-OFDM and DFT-s-OFDM for Rank-4 in terms of PAPR loss and throughput differences. |
| Panasonic | We agree that to identify the necessity of 2-layer DFT-s-OFDM should be first. |
| IMU | We need to see how 2-layer DFT-s-OFDM perfoms. Discuss this later futher |
| Samsung | The potential benefit of DFT-s-OFDM, if any diminishes at a higher rank value. Especially when applying non-coherent CB, the PAPR gain is marginal compared to CP-OFDM. |
| Shef | New waveforms offer significant potential in higher rank channels. Considering 2 layers should not preclude >2 layers |

Question 8.3.2: If both DFT-s-OFDM and CP-OFDM is to be supported for UL transmission with rank= 3 or 4 from specification point of view, for a UE supporting UL MIMO rank= 3 or 4 - how do you see the required UE support?

- Preferably, please indicate at your ‘support’ only to one of the options.

- If having (further) comments, please provide your input in the separate table below.

| UE support for UL rank=3 or 4 | Companies position (list of companies) |
| --- | --- |
| Alt. 1: CP-OFDM mandatory DFT-s-OFDM optional | IMU |
| Alt. 2: DFT-s-OFDM mandatory CP-OFDM optional |  |
| Alt. 3: Both (i.e. DFT-s-OFDM & CP-OFDM) mandatory |  |
| Alt. 4: Up to UE capability indication (per band and/ band combination) |  |

| Company | Comments |
| --- | --- |
| DOCOMO | The best alternative depends on the performance gain, coherent precoder design, etc. This can not be decided only in this agenda, due to high dependency on other agendas, such as AI 10.5.2.3. |
| IMU | Need to wait until 2 layer results are clear |

## UL transmissions with ranks 5 to 8

Question 8.4.1: For UL transmissions with rank= 5 to 8, how do you see the support of DFT-s-OFDM and/or CP-OFDM?

- Please indicate  your ‘support / yes’ only to one of the 3 options in color (1., 2. or 3.).

- If you indicate “Yes” to 1. (CP-OFDM only) or 2. (CP-OFDM only), please indicate in 4. if you are open to consider &/ study the support of both waveforms or not (i.e. yes / no)

- If having (further) comments, please provide your input in the separate table below.

| Supported baseline UL WF for rank= 5 to 8 | Considered options | Companies position (list of companies) |
| --- | --- | --- |
| 1. CP-OFDM only | Yes | OPPO, Nokia, CMCC, Lekha, Apple, DOCOMO, Samsung |
| 2. DFT-s-OFDM only | Yes |  |
| 3. DFT-s-OFDM & CP-OFDM | Yes |  |
| 4. Open for studies / consider to support both WFs | Yes | Spreadtrum, Sony, Shef |
|  | No | OPPO, Nokia, CMCC, DOCOMO |

| Company | Comments |
| --- | --- |
| OPPO | First identify the gain of 2-layer DFT-s-OFDM before discussing the >3-layer DFT-s-OFDM proposal. |
| Nokia | same view as OPPO |
| CMCC | We are wondering whether the benefit of low-PAPR still exists for DFT-s-OFDM waveform in such high-RANK transmissions. |
| Lekha | Beyond 2 layers, we don’t think DFT-s-OFDM will give any additional benefits |
| DOCOMO | The performance gain for more than 4-layer DFT-s has not been shown sufficienty so far. As mentioned above, details need to be discussed under AI 10.5.2.3 |
| Panasonic | We agree that to identify the necessity of 2-layer DFT-s-OFDM should be first. |
| Shef | New waveforms offer significant potential in higher rank channels. Considering 2 layers should not preclude >2 layers |

Question 8.3.2: If both DFT-s-OFDM and CP-OFDM is to be supported for UL transmission with rank= 5 to 8 from specification point of view, for a UE supporting UL MIMO rank= 5 to 8 - how do you see the required UE support?

- Preferably, please indicate at your ‘support’ only to one of the options.

- If having (further) comments, please provide your input in the separate table below.

| UE support for UL rank= 5 to 8 | Companies position (list of companies) |
| --- | --- |
| Alt. 1: CP-OFDM mandatory DFT-s-OFDM optional |  |
| Alt. 2: DFT-s-OFDM mandatory CP-OFDM optional |  |
| Alt. 3: Both (i.e. DFT-s-OFDM & CP-OFDM) mandatory |  |
| Alt. 4: Up to UE capability indication (per band and/ band combination) |  |

| Company | Comments |
| --- | --- |
| DOCOMO | The best alternative depends on the performance gain, coherent precoder design, etc. This can not be decided only in this agenda, due to high dependency on other agendas, such as AI 10.5.2.3. |
| Shef | New waveforms offer opportunities to manage high channel correlation making it worth considering more (simple) antennas at the UE. |

## Higher rank DFT-s-OFDM Evaluation assumptions: Possible clarifications & amendments

For multi-layer UL DFT-s-OFDM study, following evaluation agreements have been made in RAN1 #123.

However, we noticed that different companies provided simulation results with different reference. For the sake of comparability, we believe it would be helpful to agree on simulation assumptions for the reference.

Question 8.5.1: What are the simulation assumptions for the reference for multi-layer UL DFT-s-OFDM? Particularly, with respect to following items:

- 1. Release 18 DWS enabled/disabled?

- 2. Release 16 full power mode (0 and/or 1) enabled/disabled?

- Precoding assumptions for coherent UE for CP-OFDM (3.) and DFT-s-OFDM (4.)?

- Delivering statistics for power limited UEs (5.), rank (6.) and MCS (7.)

| Question | Considered options | Companies position (list of companies) |
| --- | --- | --- |
| 1. Release 18 DWS enabled or disabled? | Enabled | OPPO, Nokia, CMCC, Sony, DOCOMO, Samsung |
|  | Disabled | OPPO, vivo, DOCOMO |
| 2. Release 16 full power mode (0 and/or 1) enabled/disabled? | Enabled | Nokia, CMCC, vivo, DOCOMO, Samsung |
|  | Disabled | OPPO |
| 3. Precoding assumptions for coherent UEs for CP-OFDM? | CP-OFDM: no CB restriction, wideband and subband precoding. Please justify. | OPPO, Nokia, CMCC, Lekha |
|  | CP-OFDM: only non-coherent CB and wideband precoding. Please justify. | Vivo, DOCOMO |
| 4. Precoding assumptions for coherent UEs for DFT-s-OFDM? | DFT-s-OFDM: only non-coherent CB with wideband precoding. Please justify. | Nokia, vivo, Apple, DOCOMO |
|  | DFT-s-OFDM: no CB restriction, wideband precoding. Please justify how to assess PAPR. | OPPO, CMCC, Lekha |
| 5. Companies to report statistics on power limited UEs? | Yes | Nokia, CMCC, vivo, Samsung |
|  | No | OPPO, DOCOMO |
| 6. Companies to report statistics on UL TX rank? | Yes | Nokia, CMCC, vivo, Samsung |
|  | No | OPPO, DOCOMO |
| 7. Companies to report statistics on used MCS? | Yes | Nokia, CMCC, vivo, Samsung |
|  | No | OPPO, DOCOMO |

| Company | Further comments |
| --- | --- |
| OPPO | DWS for 6G is still under study. For evaluation, both R18 DWS enabled and disabled scenarios can be considered. |
| CATT | Not so sure if this evaluation campaign should be conducted in MIMO agenda if following previous meeting discussion. |
| Nokia | We don’t support a reference without R18 DWS. The same is valid for R16 full power mode. We cannot base the reference on R15 where single layer has 3 dB less tx-power compared with 2-layers. |
| vivo | Firstly, R-18 DWS is not deployed. So, we cannot assume it is supported as baseline for 6GR. Instead, it should be competing solution compared with multi-layer DFT transmission. Besides, per ourevaluation, DFT waveform outperform CP-OFDM in link-level performance when number of Rx is equal to or larger than 16. So, it’s enough to evaluate performance based on RRC-configured CP-OFDM and DFT-s-OFDM. Secondly, for fair comparison of single layer transmission and rank 2, total maximum power should be aligned, so full-power mode 0 and full power mode1 should be considered according to the selected PA architecture. Considering the fact that MPR of 2-layers CP-OFDM transmission with full-coherent precoders should be evauated further because PPAR increases in this case. Non-coherent codebook subset can be utilized to evaluate performance of both CP-OFDM and DFT-s-OFDM rank2 transmission based on current Ran4 38.101-1 section 6.2D definition. |
| DOCOMO | For evaluation purposes, scenarios both with and without R18 DWS enabled should be considered, as DWS for 6G is still under study. Subband precoding is under study in other agendas, so it should depend on the discussion in agenda AI 10.5.2.3. In this agenda, the evaluation could focus on non-coherent precoders. Because we assume the coherent precoder design for DFT-s-OFDM should be further discussed in other agendas, such as AI 10.5.2.3. |

# Waveform proposal characterization

This section focuses on the waveform categorization based on the agreed table from RAN1#123

At RAN1#123, the following table had been agreed to provide further information on the individual waveform proposals for discussion:

Agreement Table is endorsed to characterize each proposal as a potential RAN1 observation. Characterization of each waveform proposal Description. Name of the proposal. Motivation of the proposal; E.g. TN, NTN, ISAC, etc…. Applicable link direction; DL/UL/both. Enhancement to CP-OFDM?; No/Yes. Enhancement to DFT-s-OFDM?; No/Yes. Additional OFDM-compatible waveform?; No/Yes. Target channel(s)/signal(s); PDCCH/PDSCH/PUCCH/PUSCH/xxx. Target modulation. Motivation / use case; Improved spectral efficiency, …. Key Metric / KPI; Spectral efficiency, …. Key spec impact foreseen. MRSS compatibility; Please explain. Multiplexing/coexistence with other waveforms; Please explain. Multi-user multiplexing; Please explain. MIMO compatibility; Please explain.

First of all, we think it would be especially for new waveform proposals to also capture the related impact on transmitter and receiver processing / complexity for potential inclusion to the TR later on (based on discussions with the RAN WG SI rapporteur). Therefore, the following addition is suggested.

Proposal 9.1: Extend the RAN1#123 endorsed table to characterize each (waveform) proposal as a potential RAN1 observation as follows to cover also impacts to transmitter and receiver processing operation:

|  | Description |
| --- | --- |
| Name of the proposal |  |
| Motivation of the proposal | E.g. TN, NTN, ISAC, etc… |
| Applicable link direction | DL/UL/both |
| Enhancement to CP-OFDM? | No/Yes |
| Enhancement to DFT-s-OFDM? | No/Yes |
| Additional OFDM-compatible waveform? | No/Yes |
| Target channel(s)/signal(s) | PDCCH/PDSCH/PUCCH/PUSCH/xxx |
| Target modulation |  |
| Motivation / use case | Improved spectral efficiency, … |
| Key Metric / KPI | Spectral efficiency, … |
| Key spec impact foreseen |  |
| MRSS compatibility | Please explain |
| Multiplexing/coexistence with other waveforms | Please explain |
| Multi-user multiplexing | Please explain |
| MIMO compatibility | Please explain |
| Impacts on transmitter processing | Please explain |
| Impacts on receiver processing | Please explain |

|  | List of companies |
| --- | --- |
| Yes / Support | OPPO, Nokia, CMCC,IMU, Lekha, Sony, DOCOMO, Panasonic, IMU, Shef |
| No |  |

| Company | Further comments |
| --- | --- |
| CATT | In the FL summary, we don’t say any discussion points for CP-OFDM PAPR reduction. We are wondering if the optization for CP-OFDM is within scope of 6GR or not？ Maybe FL can clarify a bit. |
| IMU | We need to make sure also the alternative waveform proposals are compatible with OFDM and DFT-s-OFDM frame work. Identifying the best use cases for different schemes and enabling adaptability and flexibility is essential in achieving the best performance. For DFT-s-OFDM enhancements this should include possible interleaving and flexible allocation, and additional precoding. Support of low power applications and IoT devices (WuS/WuR, ambient IoT) should be included in use cases/motivation. |
| DOCOMO | The table extension is very helpful, as it provides deeper insight into the associated impact on transmitter and receiver processing/complexity. |
| IMU | We need to make sure also the alternative waveform proposals are compatible with OFDM and DFT-s-OFDM frame work. Identifying the best use cases for different schemes and enabling adaptability and flexibility is essential in achieving the best performance. For DFT-s-OFDM enhancements this should include possible interleaving and flexible allocation, and additional precoding. Support of low power applications and IoT devices (WuS/WuR, ambient IoT) should be included in use cases/motivation. |
| Shef | Helpful to have clear statements on complexity and compatibility to maximise gains over CP-OFDM with minimal deviation from 5G-NR and its transition to 6GR. |

Based on discussions with the vice-chair (Hiroki-san), that it would be good to collect the characterization of the different waveform proposals of individual companies. This will gives us a better overview of the individual proposals and allows for further grouping (and more focused discussions, e.g. what is considering what is an (UL) low-PAPR enhancement (of an agreed baseline WF) when comparing the related proposals and what is “another” waveform).

Several companies provided in their TDocs their assessment of waveform characterization already. As there are plenty of proposals out there, the moderator collected the input given in an Excel sheet in the drafts folder. But of course this does not include all possible proposals, but only those for which companies provided their characterization already.

Requested Company inputs on waveform proposal characterization in the Excel sheet (by Tue, 10am CET – to have your input for the first offline discussion already):

- The Excel sheet and your related inputs are in this sub-folder: Waveform Characterization

- The Excel sheet in addition to the RAN1#123 agreed characterization aspects (in rows 5-19) also includes

  - In Row 20 & 21, already the two proposed additional aspects of Proposal 9.1.1 above (in rows 20 & 21) – still in yellow

  - In Row 24, the company name that provided the characterization (pre-filled based on companies TDocs already)

  - In Row 25, the list of companies interested in studying and committing to provide evaluation results to RAN1#124bis based on the agreed evaluations assumptions.

    - Moderator added companies that provided the assessment and also showed at the same time evaluation results based on the agreed evaluation assumptions already

- List of proposals & characterization (rows 5-24): Please check what is there already:

  - if your proposal is not listed yet, please add your waveform proposal characterization in a new column

  - if a proposal you are interested in is already there, and if you think something in the characterization is really missing (not just wording please), also add your name in a different color to row 24 (as company with characterization input) – and provide additional input to rows 5-21 using same color (see example in Columns K & L where more than one company provided their assessment in their input TDocs)

- Commitment to evaluations of proposals (row 25):

  - Consider if you commit yourself to provide evaluation results for a waveform proposal to RAN1#124bis based on (1) the agreed evaluation assumptions, (2) methodologies and providing the related (3) performance figures. If so, please add to your name to the respective column / proposal in row 25

# Evaluation assumption clarifications on UL low-PAPR proposals

This section focuses on further clarifications on evaluation assumptions for UL low-PAPR proposals.

In RAN#123, the following was agreed on evaluations for UL low-PAPR proposals for spectrum extension and truncation:

Agreement

For single user evaluation assumption for MCS and subcarriers UL low-PAPR proposals with spectrum extension

| No Spectrum Extension |  | With Spectrum Extension |  |  |
| --- | --- | --- | --- | --- |
| MCS | #subcarriers | #SCs before extension (A) | Occupied BW: #SCs after extension (B) | Spectrum extension Extension: α=B-AB |
| NR MCS | B | (1-α)B | B | α∈16, 14,27,13,38,25,716,12 |

For single user evaluation assumption for MCS and subcarriers UL low-PAPR proposals with spectrum truncation

| No Spectrum Truncation |  | With Spectrum Truncation |  |  |
| --- | --- | --- | --- | --- |
| MCS | #subcarriers | #SCs before truncation (A) | Occupied BW: #SCs after truncation (B) | Spectrum truncation factor Truncation: α=A-BA |
| NR MCS | B | A | 𝐵 | α∈110, 210,310,410 |

Note: other values for extension or truncation are not precluded.

Agreement

For UL PAPR reduction, values for occupied BW B:

- {2, 4, 8, 16, 24, 30, 32, 64, 128, 240, 256} PRBs.

- Other PRB allocations are not precluded.

- Edge, outer and inner PRB allocations as defined in TS 38.101 should be considered.

Problem Statement: Considering B and the spectrum extension or truncation factor α, deriving A for extension/truncation leads generally to a:

- non-integer number of subcarriers

- non-integer number of PRBs

- the number of PRBs not being an integer multiple of {2,3,5} for efficient DFT processing

So, the intention of what is discussed / proposed blow includes:

- Both A and B should be an integer number of RBs (as pointed out by CATT in R1-2600295)

  - Note: CATT provided the following example solution for spectrum extension to guarantee this:

Proposal 7: For UL low-PAPR proposals with spectrum extension, the granularity of both A and B is assumed as RB level, and A is determined based on B as followings: - For asymmetry spectrum extension A=1-α*B*1212 RBs - For symmetry spectrum extension If B is even A=1-α*B*1224*2 % note: A is also even Else A=1-α*B*1224*2+1 . %note: A is also odd.

- Moreover, as Nokia (in R1-2600027) and Qualcomm (in R1-2601268) pointed out, the allocation of A should result in a valid DFT size for efficient processing,

Question 10.1: Should A for spectrum extension and spectrum truncation for UL low-PAPR solutions be an integer multiple of RBs?

- Note: Please provide possible solutions on defining A for spectrum extension and spectrum truncation as a function of B and (the target value of) α in the comments table below.

|  | List of companies |
| --- | --- |
| Yes | OPPO, CATT, Nokia, CMCC, IMU, Panasonic |
| No |  |

| Company | Further comments |
| --- | --- |
| vivo | Non-interger extension has impact on TBS determination and DMRS sequence generation which cost additional effort to align at evaluation stage. Since it doesn’t have impact on waveform comparison, it should be deprioritized. |

Question 10.2: Should A for spectrum extension and spectrum truncation for UL low-PAPR solutions result in a valid DFT-size (i.e. the number of RBs of A being an integer multiple of 2, 3 & 5)?

- Note: Please provide possible solutions on defining A for spectrum extension and spectrum truncation as a function of B and (the target value of) α in the comments table below.

|  | List of companies |
| --- | --- |
| Yes | CATT, Nokia, CMCC |
| No | OPPO |

| Company | Further comments |
| --- | --- |
| OPPO | For evaluation, we can first apply the restriction as in Question 10.1. After the gain is generally identified, we can discuss the potential restriction as in Question 10.2. And if the PAPR reduction is only used for CP-OFDM, should the DFT-size restriction also apply? |
| CATT | To simplify DFT operation, it is necessary to keep the DFT size as an integer multiple of 2, 3 &5. |
| Nokia | It is necessary to assess the Net Gain with valid DFT size |
| vivo | For evaluation perpurse, combination of A and B resulting valid DFT size should be prioritized. |

Nokia in R1-2600027 further recognized (see the tables below), that the possible alignment of A discussed above to an integer number of RBs and a valid DFT size may result in (i) an effective α being far of the target value (incl. α=0 e.g. for B=2 for SE in general, for α=0.1, 0.2 & 0.3), (ii) resulting in the same A (and therefore the same effective α) for more than one target value of alpha (e.g. for SE with B=8 PRBs and α= 1/6, 1/4 and 2/7) and (iii) result in the same A for different Bs for a given target value of alpha (e.g. α= 1/6 or 1/4 for B= 30 and 32PRBs all resulting in the same A of 24 PRBs).

Table 3 in R1-2600027: Parameter combinations for extension

Table 4 in R1-2600027: Parameter combinations for truncation

Question 10.3: Assuming to have A as an integer multiple of {2,3,5} PRBs, how to treat combinations of (B, α) resulting in rather different effective values of α (incl. α=0) and the same effective value of α for different target α values for certain allocations of B?

| Company | Comments |
| --- | --- |
| CATT | Need to re-evaluat the potential gain if in order to keep the integer multiple of {2,3,5} and turn out to be another resource assignment. |
| Nokia | companies to report the truly simulated (A,B,alpha) and not the target one. |

In addition to the above, the moderator would find it further reasonable trying to clarifying the reference / baseline for the related evaluations. The plain-vanilla NR Rel-18 FDSS has been used as the baseline reference by most of the companies in their provided results already – but it would be maybe good to clarify this further:

Proposal 10.4: NR Rel-18 FDSS should be used as the baseline reference when evaluating the gains of UL low-PAPR proposals.

|  | List of companies |
| --- | --- |
| Yes | CATT, CMCC |
| No | OPPO |

| Company | Further comments |
| --- | --- |
| CMCC | At least, the conclusions are expected not to conflict with the Rel-18 FDSS work. It is also appreciated to clarify the additional optimizations for 6GR waveform comparing to the Rel-18 FDSS work. |
| vivo | Transparent CFR should also be baseline. CFR is the typical implementation solution which is widely utilized in cumercial devices. Besides, it is more robust than FDSS. Performance of FDSS is impacted by channel estimation algorithm. For instance, BLER performance will degrade if MMSE-based channel estimation is used for estimating the equivalent channel especially for low SNR range for PI/2 BPSK and QPSK. |
| Nokia | It isn’t enough to consider FDSS as baseline. We need to clarify the filter too. Should a single specific filter be considered as the baseline or should the best per case performing filter be considered as baseline. In any case, companies shall report which filter is used and considered as “baseline”. Furthermore, there has been no agreements so far to support FDSS in 6GR. Proposal 8: Frequency Domain Spectrum shaping (FDSS) and FDSS with spectrum extension (FDSS-SE) are supported in 6G Radio. |
| Shef | Companies should clarfy that their proposal does not degrade performance across the whole operating range (e.g., challenging channel conditions and high-order modulation) |

# xxxx